IEEE P802.11
Wireless LANs

| CIDs 45, 66, 299 and 350 Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

Resolution of misc REVmc letter ballor comments, CIDs 45, 66, 299 and 350.

The changes marked in this document are based on 802.11-2012 Specification.

Revision Notes:

Rev0: Initial resolution proposal

CID        Pg.        Sec.              Comment                                            Proposal

| 45 | 1669.00 | 20.1.1 | Re "An HT non-AP STA shall support all ...", 802.11 has three distinct concepts: mandatory rates, basic rates and supported rates (which is very confusing for us PHY guys). I think this text is echoing the language in 20.3.5 which explicitly refers to "mandatory", and obviously any specificaiton on Basic/Supported rates is outside the scope of the PHY - and indeed the MAC since it is an SME parameter | Change language using "supported" to language using "mandatory" |
| --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: On pg. 1669, modify the text as follows:

“An HT non-AP STA shall support all mandatory equal modulation (EQM) rates for one spatial stream (MCSs 0 to 7)

using 20 MHz channel width. An HT AP shall support all EQM rates for one and two spatial streams (MCSs

0 to 15) using 20 MHz channel width.”

| 66 | 1693.00 | 20.3.7 | Equation 20-60 and 20-61 refer to a variable N^TONE_HT-Duplicate which is alleged to be defined in table 20-8. Sadly, there is no such variable in that table. | Add an entry or a Note 4 which adequately defines N^Tone_Field for Non-HT modulations |
| --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: In Table 20-8 N^Tone_Field is defined which is a general notation for all fields. For HT-Duplicate mode Field = HT-Duplicate. This generalization is also assumed in Eq(20-4).

| 299 | 1614.22 | 18.3.10.6 | The statement "The CCA-ED shall not be required for license-exempt operation in any band." is incomplete, and fails to describe that CCA-ED may be required by regulation. | Change to "Unless required by regulation, CCA-ED shall not be required for license-exempt operation in any band." |
| --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: On pg. 1614, modify the text as follows:

“Unless required by regulation, The CCA-ED shall not be required for license-exempt operation in any band.”

| 355 | 1713.00 | 20.3.11.7.5 | rem() is undefined (within step (c)). | Define the function rem(). |
| --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: On pg. 1713, modify the text as follows:

“..first codewords rem(Nshrt,NCW) shortened 1 bit more than the remaining codewords, where rem(a,b) is the reminder of division a/b.