IEEE P802.11
Wireless LANs

| CIDs 45, 66, 299 and 355 Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-06 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

Resolution of misc REVmc letter ballor comments, CIDs 45, 66, 299 and 355.

The changes marked in this document are based on 802.11-2012 Specification.

Revision Notes:

Rev0: Initial resolution proposal

Rev1: Some CIDs were updated based on discussions

CID        Pg.        Sec.              Comment                                            Proposal

| 45 | 1669.00 | 20.1.1 | Re "An HT non-AP STA shall support all ...", 802.11 has three distinct concepts: mandatory rates, basic rates and supported rates (which is very confusing for us PHY guys). I think this text is echoing the language in 20.3.5 which explicitly refers to "mandatory", and obviously any specificaiton on Basic/Supported rates is outside the scope of the PHY - and indeed the MAC since it is an SME parameter | Change language using "supported" to language using "mandatory" |
| --- | --- | --- | --- | --- |

Proposed resolution: Revise.

Instruction to Editor: Please modify text on pg 1688 as follows:

An HT non-AP STA shall support all equal modulation (EQM) rates for one spatial stream (MCSs 0 to 7)

using 20 MHz channel width. An HT AP shall support all EQM rates for one and two spatial streams (MCSs

0 to 15) using 20 MHz channel width.MCS 0 to 15 are mandatory in 20 MHz with 800 ns GI at an AP. MCS 0 to 7 are mandatory in 20 MHz with800 ns GI at all STAs. All other MCSs and modes are optional, specifically including transmit and receive support of 400 ns GI, operation in 40 MHz, and support of MCSs with indices 16 to 76.

Instruction to Editor: Please remove the text on pg 1669 as follows:

An HT non-AP STA shall support all equal modulation (EQM) rates for one spatial stream (MCSs 0 to 7)

using 20 MHz channel width. An HT AP shall support all EQM rates for one and two spatial streams (MCSs

0 to 15) using 20 MHz channel width.

| 66 | 1693.00 | 20.3.7 | Equation 20-60 and 20-61 refer to a variable N^TONE_HT-Duplicate which is alleged to be defined in table 20-8. Sadly, there is no such variable in that table. | Add an entry or a Note 4 which adequately defines N^Tone_Field for Non-HT modulations |
| --- | --- | --- | --- | --- |

Proposed resolution: Revise.

Instruction to Editor: In Table 20-8 change the text as follows:

“              HT-Data DATA

MCS 32 HT-Duplicate, see NOTE 3

Non-HT Duplicate                                -                                              104”   (Note to Editor: this is a new row)

Instruction to Editor: On pg 1727 EQ(20-61) change the equation as follows:

Non-HT Duplicate

Instruction to Editor: On pg 1727 below EQ(20-61) starting with “where” change the text as follows:

Non-HT Duplicate

| 299 | 1614.22 | 18.3.10.6 | The statement "The CCA-ED shall not be required for license-exempt operation in any band." is incomplete, and fails to describe that CCA-ED may be required by regulation. | Change to "Unless required by regulation, CCA-ED shall not be required for license-exempt operation in any band." |
| --- | --- | --- | --- | --- |

Proposed resolution: Revise.

Instruction to Editor: On pg. 1614, modify the text as follows:

“Unless required by regulation, The CCA-ED shall not be required for license-exempt operation in any band.”

| 355 | 1713.00 | 20.3.11.7.5 | rem() is undefined (within step (c)). | Define the function rem(). |
| --- | --- | --- | --- | --- |

Proposed resolution: Revise.

Instruction to Editor: On pg. 1713, modify the text as follows:

“..first codewords remmod(Nshrt,NCW) shortened 1 bit more than the remaining codewords.