IEEE P802.11
Wireless LANs

| TGac D4.0 Comment Resolution Misc PHY CIDs |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Vinko Erceg | Broadcom | 16340 W. Bernardo Dr. San Diego, CA 92127 | +1 858 521 5885 | verceg@broadcom.com |

Abstract

This submission proposes resolutions to some of the PHY CIDs submitted in the Task Group LB 188. The following CIDs are addressed: CID 7105, 7186, 7060, 7059, 7106, 7267, 7268, 7269, 7270, 7271, 7272, 7273, 7314, 7191, 7176, 7266, 7190, and 7192.

The changes marked in this document are based on TGac Draft 4.0

Revision Notes:

Rev0: initial version.

CID        Sec.         Pg.            Ln.                   Comment                                  Proposal

| 7105 | 22.3.19.3 | 302 | 14 | As currently written, it is ok to build a VHT 20MHz receiver operating at, say, ch. 36 which passes the nonadjacent channel rejection requirement for blockers in ch. 44, but does not passes the requirement for blockers in ch. 48. | Clarify whether the nonadjacent blocker requirement applies only to 2*W MHz away blocker, or to any blockers 2*W MHz or more away. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: On page 302, line 14, please modify the text as follows:

“The center frequency of the nonadjacent channel shall be placed 2×W MHz or, in the case channelization does not support 2×W MHz placing, more away from the center frequency of the desired signal.”

| 7086 | 22.3.18 | Annex T defines the Time of Departure accuracy test which evaluates rms of TIME_OF_DEPARTURE against aTxPHYTxStartRMS and aTxPHYTxStartRMS against TIME_OF_DEPARTURE_ACC URACY_TEST_THRESH. Though, valuses of test parameters and TIME_OF_DEPARTURE_ACC URACY_TEST_THRESH are not defined when transmitting VHT PPDU. | Insert subclause 22.3.18.5 Time of Departure accuracy as following. The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPHYTxStartRMS and aTxPHYTxStartRMS against IME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters: -- MULTICHANNEL_SAMPLING_RATE is: 20e6 x (1+[(fH - fL)/20 MHz]) sample/s, for a CH_BANDWIDTH parameter equal to CBW20 40e6 x (1+[(fH - fL)/40 MHz]) sample/s, for a CH_BANDWIDTH parameter equal to CBW40 80e6 x (1+[(fH - fL)/80 MHz]) sample/s, for a CH_BANDWIDTH parameter equal to CBW80 160e6 x (1+[(fH - fL)/160 MHz]) sample/s, for a CH_BANDWIDTH parameter equal to CBW160 or CBW80+80 where fH is the nominal center frequency in Hz of the highest channel in the channel set fL is the nominal center frequency in Hz of the lowest channel in the channel set, the channel set is the set of channels upon which frames providing measurements are transmitted, the channel set comprises channels uniformly spaced across fH - fL <= 50 MHz [x] equals the smallest integer equal to or larger than x. -- FIRST_TRANSITION_FIELD is L-STF. -- SECOND_TRANSITION_FIELD is L-LTF. -- TRAINING_FIELD is L-LTF windowed in a manner which should approximate the windowing described in 18.3.2.5 (Mathematical conventions in the signal descriptions) with TTR = 100 ns. -- TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH is 80 ns. NOTE The indicated windowing applies to the time of departure accuracy test equipment, and not the transmitter or receiver. |
| --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7060 | 22.3.10.1 | 266 | 12 | Why can 22-52 for BCC just talk of Nsym and this apparently works for both SU and MU, but an LDPC user in a MU PPDU cannot just use Nsym ... | Basically, is there a simplification available? |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: In the LDPC case there is extra symbol ambiguity, see page 269:

“In addition, if  Nsym computed in Equation (20-41) in step (d) of 20.3.11.7.5 (LDPC PPDU encoding process)

is greater than Nsym_init, then the LDPC Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 1.

Otherwise, the LDPC Extra OFDM Symbol field of VHT-SIG-A2 shall be set to 0.”

| 7059 | 22.3.10.1 | 266 | 12 | What is this "In the case of MU LDPC encoding". Is this the case only if all users LDPC? Or if just 1 user is LDPC then this is triggered? | Be specific ... I think we want "in the case of an MU PPDU for an LDPC-encoded PSDU for a user ..." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7106 | 22.3.10.12 | 291 | 23 | With respect to "The L-STF, L-LTF, and L-SIG fields shall be transmitted in the same way as in the VHT transmission, with the exceptions for the Rate and Length fields listed in 22.3.8.1.4 (L-SIG defini- tion).", it isn't specified how the Rate and Length need to be set. | Please clarify |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: Rate and Length settings are defined in Sec 22.3.8.1.4:

“In a VHT PPDU, the RATE field shall be set to the value representing 6 Mb/s in the 20 MHz channel spacing

column of Table 18-6 (Contents of the SIGNAL field). In a non-HT duplicate PPDU, the RATE field is defined

in 18.3.4.2 (RATE field) using the L_DATARATE parameter in the TXVECTOR.

The LENGTH field shall be set to the value given by Equation (22-20).”

| 7267 | 22.3.10.1 | 265 | 45 | Pad bits should be per user | Replace "pad bits to append" with "pad bits to append for user u" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7268 | 22.3.10.6 | 271 | 44 | Confusing wording "S bits from the output of the first encoder are fed into all spatial streams" | Replace with "S bits from the output of the first encoder are divided among all spatial streams, s bits per stream" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7269 | 22.3.10.6 | 272 | 28 | The NOTE lists cases where N_CBPS is greater than N_block x N_ES x S. Currently it only contains 160 MHz cases. Since N_CBPS, N_ES and S are identical for 160 and 80+80 (See Tables 22-54 to 22-61), this list should also contain the 80+80 MHz cases. | Add equivalent 80+80 cases to the list |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree in Principle.

Instruction to Editor: on pg 272, ln 28, please modify the text as follows:

— 160 MHz and 80+80 MHz, NSS = 5, VHT-MCS = 5

— 160 MHz and 80+80 MHz, NSS = 5, VHT-MCS = 6

— 160 MHz and 80+80 MHz, NSS = 7, VHT-MCS = 5

— 160 MHz and 80+80 MHz, NSS = 7, VHT-MCS = 6

| 7270 | 22.3.10.9.2 | 281 | 29 | Range of index u is missing | Add u=0, ..., N_u -1 to equation (22-81) |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7271 | 22.3.10.9.3 | 282 | 20 | User index is missing | User index is missing from left-hand side and right-hand side of Equation (22-84) Same comment for Equations (22-85), (22-86) |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.Include user index u in Eqs (22-84, 22-85, 22-86).

| 7272 | 22.3.10.9.4 | 282 | 53 | User index missing in text | d should have an additonal index u Same comment for subsequent lines of same paragraph. Index u missing in several places |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.Add index u on pg 282, everywhere on this page, equations (see CID 7271) and Sec 22.3.10.9.4 Space-time block coding.

| 7273 | 22.3.10.9.5 | 282 | 65 | Replace "segment index i_Seg is omitted in Table 22-20" with "segment index i_Seg and user index u are omitted in Table 22-20" | As in comment |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7314 | 22.3.18 | 272 |  | Would the standard define the requirement to the effect of beamforming and DFS? |  |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Disagree.

Discussion: We don’t see a reason to add DFS requirements specifically for beamforming, Seems thatthist should not be a part of specification.

| 7191 | 22.3.10.9.3 | 282 | 19 | In \tilde{d}^(i_Seg), user index is missed, but in Equations (22-92) through (22-95), it does included the index. | Add the user index u to d^(i_Seg) and \tilde{d}^(i_Seg) throughout the D4.0. Change the sentence "Note that the segment index i_Seg is omitted..." to "Note that the segment index i_Seg and the user index u are omitted..." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. See resolutions to CIDs 7271, 7272, 7273.

| 7176 | 22.3.10.5.5 | 270 | 7 | "it is not necessary" is actually a vague "may", which unfottunately is inside an informative note. | Delete "NOTE-" and replace "Thus, it is not necessary to actually encode encode the data using LDPC at this stage." with "Thus, at this stage NSYM for each user may be calculated without actually encoding the data using LDPC." |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7266 | 22.3.9.2 | 265 | 21 | HT_MF PPDU is not defined | Replace :HT_MF PPDU" with "HT PPDU with FORMAT equal to HT_MF" |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification.

| 7190 | 22.3.10.9.3 | 282 | 18 | The variable of d'', which is defined in Equations (22-81) and (22-83), has user index u; however, in Equation (22-84), the index u is missed on d^(i_Seg) and d''. Ditto in Equations (22-85) and (22-86). | Add user index u to d''. Change the superscript of d^(i_Seg) to "(i_Seg,u)" as well as \tilde{D^(i_Seg,u)_{k,m,n,BW} }" in P285L51. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree. See resolutions to CIDs 7271, 7272, 7273.

| 7192 | 22.3.10.11.1 | 287 | 6 | This subclause describes the transmission in VHT format; therefore, "SU PPDUs" should be replaced with "VHT SU PPDUs". | As in comment. |
| --- | --- | --- | --- | --- | --- |

Proposed resolution: Agree.

Instruction to Editor: Please include commenter’s proposed text in the draft 11ac specification, on line 6, pg 287.