IEEE P802.11
Wireless LANs

| 802.11 TGac WG Letter Ballot LB190 Proposed resolutions to comments on clause 10.39 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Eric Wong | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1 408 922 6672 | ewong@broadcom.com |

Abstract

This submission contains proposed comment resolutions to comments received during WG letter ballot 190.

The MAC comments assigned to the author in Clause 10.39:

7145, 7147, 7229, 7230, 7389

The changes marked in this document are based on TGac Draft 4.0.

# Revision Notes

| R0 | Initial revision |
| --- | --- |

# Comments for Clause 10.39.1

## CID 7389

| 7389 | 185.07 | 10.39.1 | "The STA that is creating the BSS shall be able to receive and transmit at each of the <VHT-MCS, NSS> tuple values indicated by the BSSBasicVHTMCS_NSSSet and OperationalVHTMCS_NSSSet." -- no, a STA is not required to be able to transmit at a rate in the operational rate set, only receive | "The STA that is creating the BSS shall be able to receive and transmit at each of the <VHT-MCS, NSS> tuple values indicated by the BSSBasicVHTMCS_NSSSet and to receive at each of the <VHT-MCS, NSS> tuple values indicated by the OperationalVHTMCS_NSSSet." | MAC |
| --- | --- | --- | --- | --- | --- |

Discussion:

The description for BSSBasicVHTMCS_NSSSet states the set of MCSs for which the STA creating the BSS is able to receive and transmit. The description for OperationalVHTMCS_NSSSet, on the other hand, denotes the MCSs that a STA is able to receive. The commeter is correct in that there is no requirement for a STA to transmit at a rate defined in the OperationalVHTMCS_NSSSet.

.

Proposed resolution:

Revise. Please see changes in 11-12/1318r0 for CID 7389.

Instruction to Editor:

Please make the following changes in TGac D4.0 P185L6 under subclause 10.39.1.

The STA that is creating the BSS shall be able to receive and transmit at each of the <VHT-MCS, NSS> tuple values indicated by the BSSBasicVHTMCS_NSSSet , and shall be able to receive at each of the <VHT-MCS, NSS> tuple values indicated by the OperationalVHTMCS_NSSSet.

## CID 7229

| 7229 | 186.01 | 10.39.1 | A VHT STA can transmit 20MHz frames (e.g. Probe Request) to a STA which is not in the VHT BSS for BSS handover etc. | Change the paragraph to "A VHT STA that is a member of a VHT BSS shall not transmit a 20 MHz VHT PPDU to a STA in the same BSS on a channel other than the primary 20 MHz channel of the BSS , except for a 20 MHz VHT PPDU transmission on an offchannel TDLS direct link as constrained by 10.22.6.4.2 (Basic wideband functionality)." The similar changes should be done to the following 3 paragraphes. | MAC |
| --- | --- | --- | --- | --- | --- |

Discussion:

After a discussion with the commenter, it is believed that VHT STA is unlikely to send a VHT Probe Request for active scanning purpose and BSS handover. Therefore, commenter proposed that this comment be rejected.

Proposed resolution:

Reject. Since a VHT STA is unlikely to send a VHT Probe Request, the commenter has proposed to reject this comment.

# Comments for Clause 10.39.3

## CID 7145

| 7145 | 187.03 | 10.39.3 | A prepositional phrase should not be separated by a comma from the body of its sentence. | Delete the commas after "operation" on this line and line 6. | MAC |
| --- | --- | --- | --- | --- | --- |

Discussion:

Excerpts from P187L3 in subclause 10.39.3

During an individual scan within an OBSS scan operation, the minimum per-channel scan duration is

dot11OBSSScanPassiveDwell TU (for a passive scan) or dot11OBSSScanActiveDwell TU (for an active

scan). During an OBSS scan operation, each channel in the set is scanned at least once per

dot11BSSWidthTriggerScanInterval seconds, and the minimum total scan time (i.e., the sum of the scan durations)

per channel within a single OBSS scan operation is dot11OBSSScanPassiveTotalPerChannel TU (for

a passive scan) or dot11OBSSScanActiveTotalPerChannel TU (for an active scan).

Proposed resolution:

Reject. The comma seems reasonable since the sentence starts with a long introductory prepositional phrase.

## CID 7147

| 7147 | 187.13 | 10.39.3 | "it is necessary" is actually the statement of a requirement -- which unfortunately currently is inside an informative note. | Replace "For some combinations of parameter values, it is necessary to exceed the minimum values of some of the parameters in order to" with: "For some combinations of parameter values the minimums might be exceeded in order to". Or, if the permission actually is desired, then remove this sentence from the informative note and replace "might" with "may". | MAC |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree in principle.

Proposed resolution:

Revise. Please see changes in 11-12/1318r0 for CID 7147.

Instruction to Editor:

Please make the following changes in TGac D4.0 P187L11 under subclause 10.39.3.

NOTE—The values provided in the previous paragraph indicate the minimum requirements. For some combinations of parameter values the minimum values might be exceeded in order to meet the minimum value constraints of all parameters.

# Comments for Clause 10.39.5

## CID 7230

| 7230 | 189.45 | 10.39.5 | The sentense means that a VHT STA update the NAV even if the Duration value in the received frame creates a smaller NAV. This is not right per NAV update per subclause 9. | Harmonize the sentense with clause 9. | MAC |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with commeter.

Proposed resolution:

Revise. Please see changes in 11-12/1318r0 for CID 7230.

Instruction to Editor:

Please make the following changes in TGac D4.0 P189L45 under subclause 10.39.5.

A VHT STA shall update its NAV as described in 9.3.2.4 (Setting and resetting the NAV) using the Duration/ID field value in any frame that does not have an RA matching the STA’s MAC address and that was received in a 20 MHz PPDU in the primary 20 MHz channel or received in a 40 MHz PPDU in the primary 40 MHz channel or received in a 80 MHz PPDU in the primary 80 MHz channel or received in a 160 MHz or 80+80 MHz PPDU.