IEEE P802.11
Wireless LANs

| Comment resolution for CAQ related comment resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Zhou Lan | NICT | 3-4, Hikarino-oka, Yokosuka, Japan | +81-46-847-5097 | lan@nict.go.jp |
| Keiichi Mizutani | NICT |  |  | mizk@nict.go.jp |
| Gabriel Villardi | NICT |  |  | gpvillardi@nict.go.jp |
| Ha-Nguyen Tran | NICT |  |  | haguen@nict.go.jp |
| Hiroshi Harada | NICT |  |  | harada@nict.go.jp |

Abstract

Submission for comment resolution of LB 189 on CAQ.

Comments CIDs covered in this document are 254, 377, 953, and 954.

Editing instructions are based on modifying P802.11af_D2.0.

- Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE Std 802.11™–2012, as amended by

- Amendment 1 802.11ae-2012

- Amendment 2 802.11aa-2012

- Amendment 3 P802.11ad Draft 8.0

- Amendment 4 P802.11ac Draft 3.0

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 254 | 58.02 | 8.4.5.2 | Regarding Figure 8-431i, the Device Location Information fields are not actually repeated, I think this means there are multiple Device Location Information fields. Similar issue with Figure 8-431k and Figure 8-431l. | Change the note above the Device Location Information field within Figure 8-431i to read "There may be multiple Device Location Information fields" |

Propose Accepted for CIDs 254 per editing instructions in 802.11-12/1348r1.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 377 | 59.00 | 8.4.5.2 | a little confusing. suggest rewording. see proposed change for suggestions if my interpretation is correct. | The "Number of Device Location Information" subfield indicates the number of Device Location Information fields presented in the (RLQP?) Channel Availability Query element. When no device locations are present, the value of "Number of Device Location Information" subfield is set to 0. |

Discussions:

The suggested changes doesn’t seem improve the original text. The original text follows the traditional way of explaining the information element fields. Propose to reject the comment and make no change to the current text.

Propose Rejected for CIDs 377 per discussion and editing instructions in 802.11-12/1348r1.

| CID | Page | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- |
| 953 | 80.51 | 10.42.4 | "Any GDC STA .. sets its" unclearly sounds sort of like a requirement. Either make it a requirement or an informative statement. | If a requirement, then replace "Any" with "Every" and "sets" with "shall set". If informative, replace "Any" with "A". |
| 954 | 80.54 | 10.42.4 | "must" is deprecated in IEEE standards. | Replace "must" with "shall". |

Discussions:

Propose to accept comment CID 953. An fixed device doesn’t have to have geolocation capability, therefore we proposal to revise comment CID 954, changing “must” to “may”

Propose Accepted for CID 953 per discussion and editing instructions in 802.11-12/1348r1. Revised for CID 954 per discussion and editing instructions in 802.11-12/1348r1.

TGaf Editor: replace the note on the Device Location Information field with “There may be multiple Device Location Information fields” in Figure 8-431i. :

TGaf Editor: replace the note on the Device Location Information field with “There may be multiple Device Location Information fields, as determined by the Length field” in Figure 8-431l. :

10.42.4 Channel Availability Query (CAQ) procedures

TGaf Editor: modify the fifth paragraph starting with “Any GDC…” as follows:

Any A GDC STA that acquires its geolocation position with the accuracy required by the regulatory domain sets its dot11GeolocationCapabilityActivated attribute to true. Any STA that operates as a GDC enabling STA in TVWS band must may have its geolocation position before it initiates the CAQ procedure. Such a STA may transmit a Channel Availability Query request to another GDC enabling STA. The responding STA generates the CAQ response after communicating to the RLSS, if the CAQ request was received in an RLQP CAQ element.