IEEE P802.11
Wireless LANs

| Delete the PMD (Comment Resolution for TGmc) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 30 November 2012 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Eldad Perahia | Intel Corporation |  |  | eldad.perahia@intel.com |

Abstract

This document provides resolutions for CIDs: 35, 61, 65, 275, 276

Delete the PLCP/PMD interface.  Edits are referenced to P802.11REVmc D0.5

Reference: 12/1009r3, which deletes the PLCP/PMD interface from 802.11ac

Introduction

This submission deletes the PLCP/PMD interface.  The rationale is that this is a completely imaginary and useless interface and there are no devices that split their PHY in such a manner.  In fact, the MAC is perceived by many in two halves (“upper” and “lower”), but such an architectural split is not in the standard.

Terminology changes:

- Convert use of PLCP/PMD to PHY

  - In a few places I made a judgement call and changed PLCP to PPDU.  Specifically where PCLP was clearly being used as the name of the PHY frame.

- Convert definition of PPDU from PLCP PDU to PHY PDU

- Convert definition of PSDU from PLCP SDU to PHY SDU

MAC clause changes:

- 3: Definitions, acronyms, and abbreviations

- 4: General description

- 6: Layer management

- 7: PHY service specification

- 8: Frame formats

- 9: MAC sublayer functional description

  - Combine aRxRFDelay + aRxPLCPDelay into aRxPHYDelay

Annexes

- B: PICS

  - Delete PICS for PMD

- C: ASN.1 encoding of the MAC and PHY MIB

- T: Location and Time Difference accuracy test

- W: Mesh BSS operation

The approach taken to modifying the PHY clauses is to delete the PMD sublayer and any reference to the PMD, and convert uses of PLCP to PHY.  PHY clauses:

- 16: DSSS PHY specification for the 2.4 GHz band designated for ISM applications (11)

  - Added TXVECTOR and RXVECTOR, including TX_ANTENNA for antenna selection

  - added TXSTATUS

- 17: High Rate direct sequence spread spectrum (HR/DSSS) PHY specification (11b)

  - Added TX_ANTENNA for antenna selection to the TXVECTOR

- 18: Orthogonal frequency division multiplexing (OFDM) PHY specification (11a)

  - Delete 18.3.8.6 (TX RF delay)

- 19: Extended Rate PHY (ERP) specification (11g)

  - No ERP-PBCC

  - No DSS-OFDM

- 20: High Throughput (HT) PHY specification (11n)

FH, IR, ERP-PBCC, and DSSS-OFDM PHYs are skipped as they are being removed from D0.6.

Editing note: clause number references that include a title of an offending acronym are ignored as they will be automatically updated when the clause title is fixed.

Comments:

| CID | Page | Clause | Comment | Proposed Change | Resn Status | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 35 | 1504.00 | 16.1.2 | PLCP/PMD split is not a useful concept. | Merge PLCP/PMD into PHY in each PHY clause and update MAC to suit | V | Revised. Refer to resolution in 12/1431 |
| 61 |  |  | Delete PLCP/PMD interface from all the PHYs. | as in comment | V | Revised. Refer to resolution in 12/1431 |
| 65 | 1537.14 | 17.1.3.2 | Merge the PMD and PCLP sublayers in PHY clauses 17, 18, 19, and 20.Optionally in Clauses 14, 15, and 16. | Merge the PMDand PCLP into a single "PHY" for the Clause 17 to 20 PHYs (802.11b/a/g/n), as well as the later PHYs that are are coming (in a similar manner to what they have done). | V | Revised. Refer to resolution in 12/1431 |
| 275 |  |  | PMDs -- what are they and are they worth having? | Clarify and/or unite PLCPs and PMDs into one glorious United PHY of PLCP and PMD | V | Revised. Refer to resolution in 12/1431 |
| 276 |  |  | PMDs -- they seem to be lacking a noun (physical medium-dependent ... what?) | Add a noun or otherwise make the use of the term less grammatically revolting | V | Revised. Refer to resolution in 12/1431 |

MAC Modifications:

3. Definitions, acronyms, and abbreviations

3.1 Definitions

REVmc editor: modify REVmc D0.5 Clause 3.1 as follows:

aggregate medium access control (MAC) protocol data unit (A-MPDU): A structure containing multiple

MPDUs, transported as a single physical layer (PHY) service data unit (PSDU) by the PHY.

beamformee: A station (STA) that receives a physical layer (PHY) protocol data

unit (PPDU) that was transmitted using a beamforming steering matrix.

beamformer: A station (STA) that transmits a physical layer (PHY) protocol data

unit (PPDU) using a beamforming steering matrix.

null data packet (NDP): A physical layer (PHY) protocol data unit (PPDU) that carries no Data field.

sounding: The use of preamble training fields to measure the channel for purposes other than demodulation of the Data portion of the physical layer (PHY) protocol data unit (PPDU) containing the training fields.

NOTE—These uses include calculation of transmit steering, calculation of recommended MCS, and calculation of calibration

parameters.

3.2 Definitions specific to IEEE Std 802.11

REVmc editor: modify REVmc D0.5 Clause 3.2 as follows:

20 MHz mask physical layer (PHY) protocol data unit (PPDU):

20 MHz physical layer (PHY) protocol data unit (PPDU):

40 MHz mask physical layer (PHY) protocol data unit (PPDU):

40 MHz physical layer (PHY) protocol data unit (PPDU):

high-throughput-greenfield (HT-greenfield) format: A physical layer (PHY) protocol data unit (PPDU) format of the HT physical layer (PHY) using the HT-greenfield format preamble.

This format is represented at the PHY data service access point (SAP) by the TXVECTOR/RXVECTOR

FORMAT parameter being equal to HT_GF.

high-throughput-mixed (HT-mixed) format: A physical layer (PHY) protocol

data unit (PPDU) format of the HT physical layer (PHY) using the HT-mixed format preamble. This format is represented at the PHY data service access point (SAP) by the TXVECTOR/RXVECTOR FORMAT parameter being equal to HT_MF.

high-throughput (HT) physical layer (PHY) protocol data unit (PPDU): A

Clause 20 (High Throughput (HT) PHY specification) PPDU with the TXVECTOR FORMAT parameter equal to HT_MF or HT_GF.

modulation and coding scheme 32 (MCS 32) format: A physical layer (PHY) protocol data unit (PPDU) format of the high-throughput (HT) physical layer (PHY) in which signals in two

halves of the occupied channel width contain the same information. This HT PPDU format supports the lowest rate.

modulation and coding scheme (MCS) feedback (MFB) requester: A station (STA) that transmits a physical layer (PHY) protocol data unit (PPDU) containing an HT Control field in which the modulation and coding scheme (MCS) request (MRQ) subfield is equal to 1.

modulation and coding scheme (MCS) feedback (MFB) responder: A station (STA) that transmits a physical layer (PHY) protocol data unit (PPDU) containing an HT Control field with the MFB field containing an MCS index or the value 127 in response to a PPDU containing an HT Control field in which the modulation and coding scheme (MCS) request (MRQ) subfield is equal to 1.

nonaggregate medium access control (MAC) protocol data unit (non-A-MPDU) frame: A frame that is transmitted in a physical layer (PHY) protocol data unit (PPDU) with the

TXVECTOR AGGREGATION parameter either absent or equal to NOT_AGGREGATED.

non-high-throughput (non-HT) duplicate frame: A frame transmitted in a non-HT duplicate physical

layer (PHY) protocol data unit (PPDU).

non-high-throughput (non-HT) duplicate physical layer (PHY) protocol data

unit (PPDU): A PPDU transmitted by a Clause 20 (High Throughput (HT) PHY specification)

PHY with the TXVECTOR FORMAT parameter equal to NON_HT and the CH_BANDWIDTH

parameter equal to NON_HT_CBW40.

non-high-throughput (non-HT) physical layer (PHY) protocol data unit

(PPDU): A Clause 20 (High Throughput (HT) PHY specification) PHY PPDU with the

TXVECTOR FORMAT parameter equal to NON_HT.

non-space-time-block-coding (non-STBC) frame: A frame that is transmitted in a physical layer

(PHY) protocol data unit (PPDU) that has the TXVECTOR STBC parameter equal

to 0, or a frame that is received in a PPDU that has the RXVECTOR STBC parameter equal to 0.

null data packet (NDP) announcement: A physical layer (PHY) protocol data

unit (PPDU) that contains one or more +HTC frames (i.e., frames with an HT (high-throughput) Control

field) that have the NDP Announcement subfield equal to 1.

sounding physical layer (PHY) protocol data unit (PPDU):

space-time block coding (STBC) frame: A frame that is transmitted in a physical layer

(PHY) protocol data unit (PPDU) that has a nonzero value of the TXVECTOR STBC parameter,

or a frame that is received in a PPDU that has a nonzero value of the RXVECTOR STBC parameter.

staggered preamble: A physical layer (PHY) preamble in a sounding PLCP

protocol data unit (PPDU) that is not a null data packet (NDP) and that includes one or more Data Long

Training fields (DLTFs) and one or more Extension Long Training fields (ELTFs).

staggered sounding: The use of a sounding physical layer (PHY) protocol data

unit (PPDU) that is not a null data packet (NDP) and that includes one or more Data Long Training fields

(DLTFs) and one or more Extension Long Training fields (ELTFs).

3.3 Abbreviations and acronyms

REVmc editor: modify REVmc D0.5 Clause 3.3 as follows:

PPDU 		PHY protocol data unit

PSDU 		PHY service data unit

PSF 		PHY Signaling field

4. General description

4.3.10 High-throughput (HT) STA

REVmc editor: modify REVmc D0.5 Clause 4.3.10 as follows:

An HT STA has PHY features consisting of the modulation and coding scheme (MCS) set described in

20.3.5 (Modulation and coding scheme (MCS)) and physical layer (PHY) protocol

data unit (PPDU) formats described in 20.1.4 (PPDU formats). Some PHY features that distinguish an HT

STA from a non-HT STA are referred to as multiple input, multiple output (MIMO) operation; spatial

multiplexing (SM); spatial mapping (including transmit beamforming); space-time block coding (STBC);

low-density parity check (LDPC) encoding; and antenna selection (ASEL). The allowed PPDU formats are

non-HT format, HT-mixed format, and HT-greenfield format (see 20.1.4 (PPDU formats))(#367). The

PPDUs may be transmitted with 20 MHz or 40 MHz bandwidth.

Clause 4.9 Reference Model

REVmc editor: modify REVmc D0.5 Figure 4-14 as follows:

REVmc editor: modify REVmc D0.5 Figure 4-15 as follows:

6. Layer management

REVmc editor: modify REVmc D0.5 Clause 6.1 as follows:

6.1 Overview of management model

Both the MAC sublayer and PHY conceptually include management entities, called MLME and PLME,

respectively. These entities provide the layer management service interfaces through which layer

management functions are invoked.

In order to provide correct MAC operation, an SME is present within each STA. The SME is a layer independent

entity that resides in a separate management plane or resides “off to the side.” Some of the

functions of the SME are specified in this standard. Typically this entity is responsible for such functions as

the gathering of layer-dependent status from the various layer management entities (LMEs), and similarly

setting the value of layer-specific parameters. The SME would typically perform such functions on behalf of

general system management entities and would implement standard management protocols. Figure 4-14 (in

4.9) depicts the relationship among management entities.

The various entities within this model interact in various ways. Certain of these interactions are defined

explicitly within this standard, via a SAP across which defined primitives are exchanged. This definition

includes the GET and SET operations between MLME, PLME and SME as well as other individually

defined service primitives, represented as double arrows within Figure 6-1. Other interactions are not

defined explicitly within this standard, such as the interfaces between the MAC and MLME and between the

PLME andPHY; the specific manner in which these MAC and PHY LMEs are integrated into

the overall MAC sublayer and PHY is not specified within this standard.

REVmc editor: modify REVmc D0.5 Figure 6-1 as follows:

Clause 6.5 PLME SAP interface

REVmc editor: modify REVmc Clause 6.5 as follows:

6.5 PLME SAP interface

6.5.4 PLME-CHARACTERISTICS.confirm

6.5.4.2 Semantics of the service primitive

The primitive provides the following parameters:

PLME-CHARACTERISTICS.confirm(

aSlotTime,

aSIFSTime,

aSignalExtension,

aCCATime,

aPHY-RX-START-Delay,

aRxTxTurnaroundTime,

aTxPHYDelay,

aRxPHYDelay,

aRxTxSwitchTime,

aTxRampOnTime,

aTxRampOffTime,

aAirPropagationTime,

aMACProcessingDelay,

aPreambleLength,

aRIFSTime,

aSymbolLength,

aSTFOneLength,

aSTFTwoLength,

aLTFOneLength,

aLTFTwoLength,

aPHYHeaderLength,

aPHYSigTwoLength,

aPHYServiceLength,

aPHYConvolutionalTailLength,

aMPDUDurationFactor,

aMPDUMaxLength,

aPSDUMaxLength,

aPPDUMaxTime,

aIUSTime,

aDTT2UTTTime,

aCWmin,

aCWmax,

aMaxCSIMatricesReportDelay

aMaxTODError,

aMaxTOAError,

aTxPHYTxStartRFDelay,

aTxPHYTxStartRMS

)

The values assigned to the parameters is as specified in the PLME SAP interface specification contained

within each PHY subclass of this standard. The parameter aMPDUDurationFactor is not used by all PHYs

defined within this standard. The parameters aSignalExtension, aRIFSTime, aSymbolLength,

aSTFOneLength, aSTFTwoLength, aLTFOneLength, aLTFTwoLength, aPHYSigTwoLength,

aPHYServiceLength, aPHYConvolutionalTailLength, aMPDUDurationFactor, aMPDUMaxLength, aPSDUMaxLength, aPPDUMaxTime, aIUSTime, aDTT2UTTTime, and aMaxCSIMatricesReportDelay are not used by all PHYs defined within this standard.

| Name | Type | Description |
| --- | --- | --- |
| … |  |  |
| aRxTxTurnaroundTime | integer | The maximum time (in microseconds) that the PHY requires to change from receiving to transmitting the start of the first symbol. The following equation is used to derive the RxTxTurnaroundTime: aTxPHYDelay + aRxTxSwitchTime + aTxRampOnTime. |
| aTxPHYDelay | integer | The nominal time (in microseconds) that the PHY uses to deliver a symbol from the MAC interface to the air interface. |
| aRxPHYDelay | integer | The nominal time (in microseconds) that the PHY uses to deliver the last bit of a received frame from the end of the last symbol at the air interface to the MAC. |
| aRxTxSwitchTime | integer | The nominal time (in microseconds) that the PHY takes to switch from Receive to Transmit. |
| aTxRampOnTime | integer | The maximum time (in microseconds) that the PHY takes to turn the Transmitter on. |
| aTxRampOffTime | integer | The nominal time (in microseconds) that the PHY takes to turn the Transmit Power Amplifier off. |
| … |  |  |
| aPHYHeaderLength | integer | The current PHY’s header length (in microseconds), excluding aPHYSigTwoLength if present. If the actual value of the length of the modulated header is not an integral number of microseconds, the value is rounded up to the next higher value. |
| aPHYSigTwoLength | integer | Length of the HT SIGNAL field (HT-SIG) (in microseconds). |
| aPHYServiceLength | integer | The length of the PHY SERVICE field (in number of bits). |
| aPHYConvolutionalTailLength | integer | The length of the sequence of convolutional code tail bits (in number of bits). |
| aMPDUDurationFactor | integer | The overhead added by the PHY to the MPDU as it is transmitted through the WM expressed as a scaling factor applied to the number of bits in the MPDU. The value of aMPDUDurationFactor is generated by the following equation: Truncate[((PPDUbits/PSDUbits)–1)  109)]. The total time to transmit a PPDU over the air is generated by the following equation rounded up to the next integer μs: aPreambleLength + aPHYHeaderLength + ( ( (aMPDUDurationFactor  8  PSDUoctets) / 109) + (8  PSDUoctets) ) / data rate where data rate is in Mb/s. The total time (in μs) to the beginning of any octet in a PPDU from the first symbol of the preamble can be calculated using the duration factor in the following equation: Truncate[aPreambleLength + aPHYHeaderLength + ( ( (aMPDUDurationFactor  8  N) / 109) + (8  N) ) / data rate] + 1, where data rate is in Mb/s and where N counts the number of octets in the PPDU prior to the desired octet, but does not count the number of octets in the preamble PHY header. |
| aMPDUMaxLength | integer | The maximum number of octets in an MPDU that can be conveyed by a PHY protocol data unit (PPDU). |
| … |  |  |
| aTxPHYTxStartRFDelay | Integer | The delay (in units of 0.5 ns) between PHY-TXSTART.request being issued and the first frame energy sent by the transmitting port, for the current channel. |
| aTxPHYTxStartRMS | Integer | The RMS time of departure error (in units of 0.5 ns), where the time of departure error equals the difference between TIME_OF_DEPARTURE and the time of departure measured by a reference entity using a clock synchronized to the start time and mean frequency of the local PHY entity’s clock. |

Clause 7 PHY service specification

REVmc editor: modify REVmc D0.5 Clause 7 as follows:

7. PHY service specification

7.1 Scope

The PHY services provided to the IEEE 802.11 WLAN MAC are described in this clause. Different PHYs

are defined as part of this standard. Each PHY can consist of the following protocol functions:

a) A function that defines a method of mapping the IEEE 802.11 MPDUs into a framing format suitable for sending and receiving user data and management information between two or more STAs.

b) A function that defines the characteristics of, and method of transmitting and receiving data through, a WM between two or more STAs.

7.2 PHY functions

The protocol reference model for the IEEE Std 802.11 architecture is shown in Figure 4-14 (Portion of the ISO/IEC basic reference model covered in this standard) (in 4.9 (Reference model)). Most PHY definitions contain two functional entities: the PHY functionand the layer management function.

The PHY service is provided to the MAC entity at the STA through a SAP, called the PHY-SAP, as shown

in Figure 4-14 (Portion of the ISO/IEC basic reference model covered in this standard).

7.3 Detailed PHY service specifications

7.3.1 Scope and field of application

The services provided by the PHY to the IEEE Std 802.11 MAC are specified in this subclause. These services are

described in an abstract way and do not imply any particular implementation or exposed interface.

7.3.2 Overview of the service

The function of the PHY sublayer is to provide a mechanism for transferring MPDUs between

two or more STAs.

7.3.5 PHY-SAP detailed service specification

7.3.5.2 PHY-DATA.request

7.3.5.2.4 Effect of receipt

The receipt of this primitive by the PHY entity causes the PHY transmit state machine to transmit an octet

of data. When the PHY entity receives the octet, it issues a PHY-DATA.confirm primitive to the MAC

sublayer.

7.3.5.3 PHY-DATA.indication

7.3.5.3.3 When generated

The PHY-DATA.indication primitive is generated by a receiving PHY entity to transfer the received octet of

data to the local MAC entity. The time between receipt of the last bit of the last provided octet from the WM and the receipt of this primitive by the MAC entity is aRxPHYDelay.

7.3.5.4 PHY-DATA.confirm

7.3.5.4.3 When generated

This primitive is issued by the PHY to the MAC entity when the transfer of data

from the MAC entity to the PHY has completed. The PHY issues this primitive in response to every PHY-DATA.request primitive issued by the MAC sublayer.

7.3.5.5 PHY-TXSTART.request

7.3.5.5.2 Semantics of the service primitive

The primitive provides the following parameter:

PHY-TXSTART.request(

TXVECTOR

)

The TXVECTOR represents a list of parameters that the MAC sublayer provides to the local PHY entity in

order to transmit a PSDU. This vector contains both PHY and PHY management parameters. The minimum

required PHY parameters are listed in 7.3.4.5 (Vector descriptions).

7.3.5.6 PHY-TXSTART.confirm

7.3.5.6.2 Semantics of the service primitive

The semantics of the primitive are as follows:

PHY-TXSTART.confirm(

TXSTATUS

)

The TXSTATUS represents a list of parameters that the local PHY entity provides to the MAC sublayer

related to the transmission of an MPDU. This vector contains both PHY and PHY operational parameters.

The required PHY parameters are listed in 7.3.4.4 (PHY-SAP service primitives parameters).

7.3.5.6.3 When generated

This primitive is issued by the PHY to the MAC entity once all of the following conditions are met:

— The PHY has received a PHY-TXSTART.request primitive from the MAC entity.

—If dot11MgmtOptionTODActivated is true and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED in the PHY-TXSTART.request(TXVECTOR) primitive is true.

— The PHY is ready to begin accepting outgoing data octets from the MAC.

7.3.5.9 PHY-CCARESET.request

7.3.5.9.4 Effect of receipt

The effect of receipt of this primitive by the PHY entity is to reset the PHY CS/CCA timers to the state

appropriate for the end of a received frame. If IPI-STATE parameter is IPI-ON, the PHY entity collects IPI

values when it is not transmitting or receiving and provides those values to the MAC sublayer using the IPIREPORT

parameter.

7.3.5.12 PHY-RXSTART.indication

7.3.5.12.1 Function

This primitive is an indication by the PHY to the local MAC entity that the PHY has received a valid start of a PPDU, including a valid PHY header.

7.3.5.12.2 Semantics of the service primitive

The primitive provides the following parameter:

PHY-RXSTART.indication(

RXVECTOR

)

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of

a valid PHY header or upon receipt of the last PSDU data bit in the received frame. The required

parameters are listed in 7.3.4.5 (Vector descriptions).

7.3.5.12.3 When generated

This primitive is generated by the local PHY entity to the MAC sublayer when the PHY has successfully

validated the PHY header at the start of a new PPDU.

After generating a PHYRXSTART.indication primitive, the PHY is expected to maintain physical medium

busy status (not generating a PHY-CCA.indication(IDLE) primitive) during the period required by that PHY

to transfer a frame of the indicated LENGTH at the indicated DATARATE. This physical medium busy

condition should be maintained even if a PHY-RXEND.indication(CarrierLost) or a

PHYRXEND.indication(Format-Violation) primitive is generated by the PHY prior to the end of this period.

7.3.5.13 PHY-RXEND.indication

7.3.5.13.2 Semantics of the service primitive

The primitive provides the following parameters:

PHY-RXEND.indication(

RXERROR,

RXVECTOR

)

The RXERROR parameter can convey one or more of the following values: NoError, FormatViolation,

CarrierLost, or UnsupportedRate. A number of error conditions may occur after the PHY’s receive state

machine has detected what appears to be a valid preamble and SFD. The following describes the parameter

returned for each of those error conditions.

— NoError. This value is used to indicate that no error occurred during the receive process in the PHY.

— FormatViolation. This value is used to indicate that the format of the received PPDU was in error.

— CarrierLost. This value is used to indicate that during the reception of the incoming PSDU, the carrier was lost and no further processing of the PSDU can be accomplished.

— UnsupportedRate. This value is used to indicate that during the reception of the incoming PPDU, a nonsupported date rate was detected.

The RXVECTOR represents a list of parameters that the PHY provides the local MAC entity upon receipt of

a valid PHY header or upon receipt of the last PSDU data bit in the received frame. RXVECTOR is an

included parameter only when dot11RadioMeasurementActivated is true. This vector may contain both

MAC and MAC management parameters. The required parameters are listed in 7.3.4.5 (Vector

descriptions).

Clause 8 Frame formats

REVmc editor: modify REVmc D0.5 Clause 8 as follows:

8.2.2 Conventions

Structures defined in the MAC sublayer are described as a sequence of fields in specific order. Each figure

in Clause 8 (Frame formats) depicts the fields/subfields as they appear in the MAC frame and in the order in

which they are passed to the physical layer (PHY), from left to right.

In figures, all bits within fields are numbered, from 0 to k, where the length of the field is k + 1 bits. Bits

within numeric fields that are longer than a single bit are depicted in increasing order of significance, i.e.,

with the lowest numbered bit having the least significance. The octet boundaries within a field can be

obtained by taking the bit numbers of the field modulo 8. Octets within numeric fields that are longer than a

single octet are depicted in increasing order of significance, from lowest numbered bit to highest numbered

bit. The octets in fields longer than a single octet are sent to the PHY in order from the octet containing the

lowest numbered bits to the octet containing the highest numbered bits.

8.3.2 Data frames

8.3.2.1 Data frame format

REVmc editor: modify REVmc D0.5  P470L8 as follows:

— The PHY’s maximum PHY service data unit (PSDU) length

8.4.2.73.8 Time of Departure subelement

The TOD RMS field specifies the RMS time of departure error in units equal to 1/TOD Clock Rate, where

the TOD Clock Rate is specified in the TOD Clock Rate field, where the time of departure error equals the

difference between the TOD Timestamp field and the time of departure measured by a reference entity using

a clock synchronized to the start time and mean frequency of the local PHY entity's clock. TOD RMS field

is determined from aTxPHYTxStartRMS  in units equal to 1/TOD Clock Rate, where the TOD Clock Rate is specified in the TOD Clock Rate field.

Clause 9 MAC sublayer functional description

REVmc editor: modify REVmc D0.5 Clause 9 as follows:

9.3.7 DCF timing relations

The relationships between the IFS specifications are defined as time gaps on the medium. The associated

attributes are provided by the specific PHY. (See Figure 9-14 (DCF timing relationships).)

All medium timings that are referenced from the end of the transmission are referenced from the end of the last

symbol, or signal extension if present, of the PPDU. The beginning of transmission refers to the first symbol of

the preamble of the next PPDU. All MAC timings are referenced from the PHY-TXEND.confirm, PHYTXSTART.

confirm, PHY-RXSTART.indication, and PHY-RXEND.indication primitives.

aSIFSTime and aSlotTime are determined per PHY, aSIFSTime is fixed, and aSlotTime can change

dynamically as aAirPropagationTime changes (see 9.18.6 (Operation with coverage classes)).

aSIFSTime is: 	aRxPHYDelay + aMACProcessingDelay + aRxTxTurnaroundTime.

aSlotTime is: 	aCCATime + aRxTxTurnaroundTime + aAirPropagationTime + aMACProcessingDelay.

The PIFS and DIFS are derived by the Equation (9-2) and Equation (9-3), as illustrated in Figure 9-14 (DCF

timing relationships).

PIFS = aSIFSTime + aSlotTime (9-2)

DIFS = aSIFSTime + 2  aSlotTime (9-3)

The EIFS is derived from the SIFS and the DIFS and the length of time it takes to transmit an ACK frame at the

lowest PHY mandatory rate by Equation (9-4).

EIFS = aSIFSTime + DIFS + ACKTxTime (9-4)

where

ACKTxTime is the time expressed in microseconds required to transmit an ACK frame, including

preamble, PHY header and any additional PHY dependent information, at the lowest PHY

mandatory rate.

REVmc editor: modify REVmc D0.5 Figure 9-14 as follows: replace “aRxRFDelay + aRxPLCPDelay” with “aRxPHYDelay”

9.19.2.3 Obtaining an EDCA TXOP

REVmc editor: modify REVmc D0.5  P980L14 as follows:

2) The end of the first AIFSN[AC] × aSlotTime – aRxTxTurnaroundTime of idle medium after

SIFS (not necessarily medium idle during the SIFS(#156), the start of the SIFS(#156) implied

by the length in the PHY header of the previous frame) when a PHY-RXEND.indication

primitive occurs as specified in 9.3.2.8 (ACK procedure).

9.23.4 L_LENGTH and L_DATARATE parameter values for HT-mixed format PPDUs

REVmc editor: modify REVmc D0.5  P1036L29 as follows: in Eq 9-12 replace aPLCPHeaderLength with aPHYHeaderLength, aPLCPServiceLength with aPHYServiceLength, and aPLCPConvolutionalTailLength with aPHYConvolutionalTailLength

REVmc editor: modify REVmc D0.5  P1036L46 as follows:

(aPreambleLength + aPHYHeaderLength) is the duration (in microseconds) of the non-HT PHY preamble and L-SIG, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm)

REVmc editor: modify REVmc D0.5  P1036L51-55 as follows:

aPHYServiceLength	 is the number of bits in the PHY SERVICE field, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm) (PLME-CHARACTERISTICS.confirm)

aPHYConvolutionalTailLength is the number of bits in the convolutional code tail bit sequence, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm)

9.23.5 L-SIG TXOP protection

REVmc editor: modify REVmc D0.5  P1037L63 as follows:

b) The duration remaining in the current packet after the L-SIG, which is equal to the duration of the

current packet less (aPreambleLength + aPHYHeaderLength)

REVmc editor: modify REVmc D0.5  Figure 9-31 as follows:

REVmc editor: modify REVmc D0.5  P1038L10 as follows: in Eq 9-13 replace aPLCPServiceLength with aPHYServiceLength, and aPLCPConvolutionalTailLength with aPHYConvolutionalTailLength

REVmc editor: modify REVmc D0.5  P1038L27-31 as follows:

aPHYServiceLength	 is the number of bits in the PHY SERVICE field, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm) (PLME-CHARACTERISTICS.confirm)

aPHYConvolutionalTailLength is the number of bits in the convolutional code tail bit sequence, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm)

REVmc editor: modify REVmc D0.5  P1039L39 as follows:

L-SIG Duration = (TInit_PPDU – (aPreambleLength + aPHYHeaderLength)) + SIFS + TRes_PPDU

REVmc editor: modify REVmc D0.5  P1039L47 as follows:

(aPreambleLength + aPHYHeaderLength) is the length in time (in microseconds) of the non-HT PHY header, defined in 6.5.4 (PLME-CHARACTERISTICS.confirm)

REVmc editor: modify REVmc D0.5  P1039L55 as follows:

L-SIG Duration = (TInit_MACDur + TInit_PPDU – (aPreambleLength + aPHYHeaderLength))

REVmc editor: modify REVmc D0.5  P1040L37 as follows:

L-SIG Duration = (TMACDur – SIFS – (aPreambleLength + aPHYHeaderLength))

REVmc editor: modify REVmc D0.5  P1040L59 as follows:

L-SIG duration – (TXTIME – (aPreambleLength + aPHYHeaderLength))

Annex B

(normative)

Protocol Implementation Conformance Statement (PICS) proforma

B.4.6 Direct sequence PHY functions

REVmc editor: modify REVmc D0.5  P1995L7 as follows:

PHY sublayer procedures

REVmc editor: modify REVmc D0.5  P1995L12 as follows:

PHY frame format

REVmc editor: modify REVmc D0.5  P1995L17 as follows:

PHY integrity check generation

REVmc editor: modify REVmc D0.5  P1995L47 as follows:

PHY frame format

REVmc editor: modify REVmc D0.5  P1995L51 as follows:

PHY integrity check generation

REVmc editor: modify REVmc D0.5  P2003L34 as follows:

Hold CCA busy for packet duration of

a correctly received PPDU but carrier

lost during reception of MPDU

REVmc editor: modify REVmc D0.5  P2003L34 as follows:

Hold CCA busy for packet duration of

a correctly received but out of

specification PPDU

REVmc editor: modify REVmc D0.5  P2003L40 as follows:

16.2.2.7 (TXVECTOR TX_ANTENNA)

REVmc editor: modify REVmc D0.5  P2003L47 as follows:

16.2.3.6 (RXVECTOR RX_ANTENNA)

REVmc editor: modify REVmc D0.5  P2004L6 as follows:

16.4.7.4

(Transmit power

level control)

B.4.8 OFDM PHY functions

REVmc editor: modify REVmc D0.5  P2013L53 as follows:

OF2: OFDM PHY Sublayer

REVmc editor: modify REVmc D0.5  P2014L6-49 as follows:

PHY preamble: SYNC

PHY header: SIGNAL

PHY header: LENGTH

PHY header: RATE

PHY header: parity, reserve

PHY header: SIGNAL TAIL

PHY header: SERVICE

PHY protocol data unit (PPDU): TAIL

PPDU: PAD

OFDM PHY data scrambler

and descrambler

REVmc editor: modify REVmc D0.5  P2016L11 as follows:

Hold CCA busy for packet duration of a

correctly received PPDU, but carrier lost

during reception of MPDU

REVmc editor: modify REVmc D0.5  P2016L11 as follows:

PHY data modulation and modulation

rate change

REVmc editor: modify REVmc D0.5  P2016L33 as follows:

PHY header: RATE

(10 MHz channel spacing)

REVmc editor: modify REVmc D0.5  P2016L46 as follows:

PHY header: RATE

(5 MHz channel spacing)

REVmc editor: modify REVmc D0.5  P2018L53 as follows: delete OF3.7

REVmc editor: modify REVmc D0.5  P2019L16 as follows:

OF4: PHY Transmit Specification

REVmc editor: modify REVmc D0.5  P2023L59 as follows:

OF5: PHY Receiver Specifications

REVmc editor: modify REVmc D0.5  P2027L36 as follows:

OF6: Transmit PHY

REVmc editor: modify REVmc D0.5  P2027L48 as follows:

OF7: Receive PHY

REVmc editor: modify REVmc D0.5  P2028L15 as follows:

REVmc editor: modify REVmc D0.5  P2028L16-P2030L27 as follows: delete OF9.1, OF9.2, OF9.3, and OF9.4 and all sub-parameters

B.4.9 High Rate, direct sequence PHY functions

REVmc editor: modify REVmc D0.5  P2032L6 as follows:

Long PHY integrity check generation

REVmc editor: modify REVmc D0.5  P2033L9-12 as follows:

PHY format

PHY integrity check verify

REVmc editor: modify REVmc D0.5  P2033L21-24 as follows:

PHY format

PHY integrity check verify

REVmc editor: modify REVmc D0.5  P2038L44-49 as follows:

Hold CCA busy for packet duration of a

correctly received PPDU, but carrier lost

during reception of MPDU.

Hold CCA busy for packet duration of a

correctly received, but out of spec, PPDU.

REVmc editor: modify REVmc D0.5  P2038L50 as follows:

17.3.5 (Vector descriptions)

REVmc editor: modify REVmc D0.5  P2038L53 as follows:

17.3.5 (Vector descriptions)

REVmc editor: modify REVmc D0.5  P2039L11 as follows:

17.4.7.3

(Transmit power

level control)

B.4.17 Radio Management extensions

REVmc editor: modify REVmc D0.5  P2063L7 as follows:

18.2.3.6

(RXVECTOR

RCPI),

18.3.10.7 (Received

Channel Power

Indicator

Measurement),

REVmc editor: modify REVmc D0.5  P2063L20 as follows:

17.4.8.6 (Received

Channel Power

Indicator

Measurement)

REVmc editor: modify REVmc D0.5  P2063L27 as follows:

19.2

(PHY-specific service parameter list)

B.4.19 High-throughput (HT) features

REVmc editor: modify REVmc D0.5  P2076L41-46 as follows:

PHY frame format

HT-mixed format PPDU format

HT-greenfield PPDU format

REVmc editor: modify REVmc D0.5  P2088L17-24 as follows:

PHY transmit specification

PHY transmit specification for

20 MHz channel

PHY transmit specification for

40 MHz channel

REVmc editor: modify REVmc D0.5  P2088L30-37 as follows:

PHY receive specification

PHY receive specification for

20 MHz channel

PHY receive specification for

40 MHz channel

Annex C

(normative)

ASN.1 encoding of the MAC and PHY MIB

REVmc editor: modify REVmc D0.5  P2445L1 as follows:

This is an 8-bit integer value that identifies the supported PHY type

.

REVmc editor: modify REVmc D0.5  P2445L18 as follows:

The current regulatory domain this instance of the PHY is supporting.

REVmc editor: modify REVmc D0.5  P2449L34 as follows:

The number of power levels supported by the PHY.

REVmc editor: modify REVmc D0.5  P2452L3 as follows:

The time in microseconds for the PHY to change from channel 2 to channel

80.

REVmc editor: modify REVmc D0.5  P2458L34 as follows:

There are different operational requirements dependent on the regulatory

domain. This attribute list describes the regulatory domains the PHY

supports in this implementation.

REVmc editor: modify REVmc D0.5  P2459L21 as follows:

There are different operational requirements dependent on the regulatory

domain. This attribute list describes the regulatory domains the PHY

supports in this implementation.

REVmc editor: modify REVmc D0.5  P2461L2 as follows:

"The Transmit bit rates supported by the PHY, represented by a

count from X'02-X'7f, corresponding to data rates in increments of

500kbit/s from 1 Mb/s to 63.5 Mb/s subject to limitations of each individual

PHY."

REVmc editor: modify REVmc D0.5  P2461L42 as follows:

"The Transmit bit rates supported by the PHY, represented by a

count from X'02-X'7f, corresponding to data rates in increments of

500kbit/s from 1 Mb/s to 63.5 Mb/s subject to limitations of each individual

PHY."

REVmc editor: modify REVmc D0.5  P2461L61 as follows:

"The Transmit bit rates supported by the PHY, represented by a

count from X'02-X'7f, corresponding to data rates in increments of

500kbit/s from 1 Mb/s to 63.5 Mb/s subject to limitations of each individual

PHY."

REVmc editor: modify REVmc D0.5  P2462L35 as follows:

"The Transmit bit rates supported by the PHY, represented by a

count from X'02-X'7f, corresponding to data rates in increments of

500kbit/s from 1 Mb/s to 63.5 Mb/s subject to limitations of each individual

PHY."

REVmc editor: modify REVmc D0.5  P2476L50 as follows:

"The Transmit MCS supported by the PHY, represented by a count

from 1 to 127, subject to limitations of each individual PHY."

REVmc editor: modify REVmc D0.5  P2477L21 as follows:

The Transmit MCS supported by the PHY, represented by a count

from 1 to 127, subject to limitations of each individual PHY.

REVmc editor: modify REVmc D0.5  P2477L39 as follows:

"The receive MCS supported by the PHY, represented by a count

from 1 to 127, subject to limitations of each individual PHY."

REVmc editor: modify REVmc D0.5  P2478L10 as follows:

"The receive MCS supported by the PHY, represented by a count

from 1 to 127, subject to limitations of each individual PHY."

Annex T

(informative)

Location and Time Difference accuracy test

REVmc editor: modify REVmc D0.5   Clause T as follows:

T.2 Time Difference of departure accuracy test

l) The Time of Departure accuracy test is passed if

1) The RMS value of e is less than aTxPHYTxStartRMS, and

2) aTxPHYTxStartRMS is less than TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH, where the units of e, aTxPHYTxStartRMS, and TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH are properly accounted for.

NOTE 1—One possible implementation of a time of departure measurement system is a free-running oscillator clocking

(a) the digital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously counting counter and (c) a hardware finite state machine such that PHY-TXSTART.request causes a transition within the finite state machine that

in turn causes frame transmission at the DACs a fixed number of cycles later; where the time of departure is recorded as

the value of the counter at that transition minus aTxPHYTxStartRFDelay  (using TIME_OF_DEPARTURE_ClockRate), where aTxPHYTxStartRFDelay can vary by channel. In this implementation, the principal source of time of departure error is short term oscillator imperfection (e.g., phase noise) and RF group delay variation across channels uncompensated by aTxPHYTxStartRFDelay.

Annex W

(informative)

Mesh BSS operation

REVmc editor: modify REVmc D0.5  P2976L27 as follows:

The overhead O for the Data frame is comprised of the PHY preamble (144 μs) and the PHY header

(48 μs).

REVmc editor: modify REVmc D0.5  P2976L33 as follows:

If RTS/CTS is used, the data transmission time (including PHY preamble and header) is 8416 μs, the RTS

transmission time (including PHY preamble and header) is 352 μs, the CTS transmission time (including

PHY preamble and header) is 304 μs, the ACK frame transmission time (including PHY preamble and header) is 304 μs and the interframe spacing overhead is 390 μs.

16. DSSS PHY specification for the 2.4 GHz band designated for ISM

applications

REVmc editor: modify REVmc D0.5  P1667L20-33 as follows:

16.1.2 Scope

The PHY services provided to the IEEE Std(#130) 802.11 WLAN MAC by the 2.4 GHz DSSS system are

described in this clause. The DSSS PHY consists of the following protocol functions:

a) A

function that defines a method of mapping the IEEE

Std(#130) 802.11 MPDUs into a framing format suitable for sending and receiving user data and

management information between two or more STAs.

b) A function that defines the characteristics of, and method of transmitting and

receiving data through, a WM between two or more STAs each using the DSSS system.

REVmc editor: modify REVmc D0.5  P1667L34 as follows:

16.1.3 DSSS PHY functions

16.1.3.1 General

The 2.4 GHz DSSS PHY architecture is depicted in the reference model shown in Figure 4-14 (Portion of

the ISO/IEC basic reference model covered in this standard) (in 4.9 (Reference model)). The DSSS PHY

contains two functional entities: the PHY functionand the layer

management function. Each of these functions is described in detail in the following subclauses.

The DSSS PHY service shall be provided to the MAC through the PHY service primitives described in

Clause 7 (PHY service specification).

16.1.3.4 PLME

The PLME performs management of the local PHY functions in conjunction with the MLME.

REVmc editor: add a new subclause after 16.1as follows:

16.2 DSSS PHY specific service parameter list

16.2.1 Introduction

The architecture of the IEEE Std(#130) 802.11 MAC is intended to be PHY independent. Some PHY

implementations require medium management state machines running in the MAC sublayer in order to meet

certain PHY requirements. These PHY-dependent MAC state machines reside in a sublayer defined as the

MLME. In certain PHY implementations, the MLME may need to interact with the PLME as part of the

normal PHY-SAP primitives. These interactions are defined by the PLME parameter list currently defined

in the PHY service primitives as TXVECTOR and RXVECTOR. The list of these parameters, and the

values they may represent, are defined in the specific PHY specifications for each PHY. Subclause 16.2 (DSSS PHY specific service parameter list) addresses the TXVECTOR and RXVECTOR for the DSSS PHY.

16.2.2 TXVECTOR parameters

16.2.2.1 General

The parameters in Table 16-1 (TXVECTOR parameters) are defined as part of the TXVECTOR parameter

list in the PHY-TXSTART.request primitive.

16.2.2.2 TXVECTOR LENGTH

The LENGTH field provided in the TXVECTOR is in octets and is converted to microseconds for inclusion in the PHY LENGTH field.

16.2.2.3 TXVECTOR DATARATE

The DATARATE parameter describes the bit rate at which the PHY shall transmit the PSDU. Its value

takes any of the rates defined in Table 16-1 (TXVECTOR parameters).

16.2.2.4 TXVECTOR SERVICE

The SERVICE parameter consists of 8 bits and is reserved for future use.

16.2.2.5 TXVECTOR TXPWR_LEVEL

The allowed values for the TXPWR_LEVEL parameter are in the range from 1 to 8. This parameter is used

to indicate which of the available TxPowerLevel attributes defined in the MIB shall be used for the current

transmission.

16.2.2.6 TXVECTOR TIME_OF_DEPARTURE_REQUESTED

The allowed values are false or true. A parameter value of true indicates that the MAC sublayer is requesting

that the PLCP entity provides measurement of when the first frame energy is sent by the transmitting port

and reporting within the PHY-TXSTART.confirm(TXSTATUS) primitive. A parameter value of false

indicates that the MAC sublayer is requesting that the PLCP entity not provide time of departure

measurement nor reporting in the PHY-TXSTART.confirm(TXSTATUS) primitive.

16.2.2.7 TXVECTOR TX_ANTENNA

Selects the antenna used by the PHY for transmission (when diversity is disabled), in the range from 1 to 256.  The number of available antennas shall be determined from the MIB table parameters aSuprtRxAntennas and aSuprtTxAntennas.

Table 16-1—TXVECTOR parameters

| Parameter | Value |
| --- | --- |
| LENGTH | 0 to 213 – 1 |
| DATARATE | 1, 2 Mb/s |
| SERVICE | 1, 2 Mb/s |
| TXPWR_LEVEL | Level1, Level2, Level3, Level4 |
| TIME_OF_ DEPARTURE_ REQUESTED | False, true. When true, the MAC entity requests that the PHY PLCP entity measures and reports time of departure parameters corresponding to the time when the first frame energy is sent by the transmitting port; when false, the MAC entity requests that the PHY PLCP entity neither measures nor reports time of departure parameters. |
| TX_ANTENNA | 1 to 256 |

16.2.3 RXVECTOR parameters

16.2.3.1 General

The parameters listed in Table 16-2 (RXVECTOR parameters) are defined as part of the RXVECTOR

parameter list in the PHY-RXSTART.indication primitive.

16.2.3.2 RXVECTOR LENGTH

The MPDU length in octets (calculated from the LENGTH field in microseconds).

18.2.3.3 RXVECTOR RSSI

This parameter is a measure by the PHY of the energy observed at the antenna used to receive the current PPDU. RSSI shall be measured during the reception of the PLCP preamble. RSSI is intended to be used in a relative manner, and it shall be a monotonically increasing function of the received power.

16.2.3.4 RXVECTOR SIGNAL

SIGNAL shall represent the data rate at which the current PPDU was received.

16.2.3.5 SERVICE

The SERVICE field shall be null.

16.2.3.6 RXVECTOR RCPI

The allowed values for the RCPI parameter are in the range from 0 to 255, as defined in 18.3.10.7 (Received

Channel Power Indicator Measurement). This parameter is a measure by the PHY of the received channel

power. RCPI indications of 8 bits are supported. RCPI shall be measured over the entire received frame or

by other equivalent means that meet the specified accuracy.

16.2.3.6 RXVECTOR SQ

SQ provides to the MAC entity the signal quality of the DSSS PHY PN code correlation. The SQ shall be sampled when the DSSS PHY achieves code lock and shall be held until the next code lock acquisition.

The SQ may be used in conjunction with RSSI as part of a CCA scheme.

16.2.3.6 RXVECTOR RX_ANTENNA

RX_ANTENNA reports the antenna used by the PHY for reception of the most recent packet.

Table 16-2—RXVECTOR parameters

| Parameter | Value |
| --- | --- |
| LENGTH | 0 to 213 – 1 |
| RSSI | 0–255 |
| SIGNAL | 1, 2 Mb/s |
| SERVICE | 1, 2 Mb/s |
| RCPI (see NOTE) | 0–255 |
| SQ | 0–255 |
| RX_ANTENNA | 1–256 |
| RX_START_OF_FRAM E_OFFSET | 0 to 232– 1. An estimate of the offset (in 10 ns units) from the point in time at which the start of the preamble corresponding to the incoming frame arrived at the receive antenna port to the point in time at which this primitive is issued to the MAC. |
|  | NOTE—Parameter is present only when dot11RadioMeasurementActivated is true. |

16.2.4 TXSTATUS parameters

16.2.4.1 General

The parameters listed in Table 16-3 (TXSTATUS parameters) are defined as part of the TXSTATUS

parameter list in the PHY-TXSTART.confirm service primitive.

Table 16-3—TXSTATUS parameters

| Parameter | Value |
| --- | --- |
| TIME_OF_DEPARTURE | 0 to 232– 1. The locally measured time when the first frame energy is sent by the transmitting port, in units equal to 1/ TIME_OF_DEPARTURE_ClockRate. This parameter is present only if TIME_OF_DEPARTURE_REQUESTED is true in the corresponding request. |
| TIME_OF_DEPARTURE_ClockRate | 0 to 216– 1. The clock rate, in units of MHz, is used to generate the TIME_OF_DEPARTURE value. This parameter is present only if TIME_OF_DEPARTURE_REQUESTED is true in the corresponding request. |
| TX_START_OF_FRAME_OFFSET | 0 to 232– 1. An estimate of the offset (in 10 ns units) from the point in time at which the start of the preamble corresponding to the frame was transmitted at the transmit antenna port to the point in time at which this primitive is issued to the MAC. |

16.2.4.2 TXSTATUS TIME_OF_DEPARTURE

The allowed values for the TIME_OF_DEPARTURE parameter are integers in the range of 0 to 232– 1. This

parameter is used to indicate when the first frame energy is sent by the transmitting port in units equal to 1/

TIME_OF_DEPARTURE_ClockRate. TIME_OF_DEPARTURE may be included in the transmitted frame

in order for recipients on multiple channels to determine the time differences of air propagation times

between transmitter and recipients and hence to compute the location of the transmitter.

16.2.4.3 TXSTATUS TIME_OF_DEPARTURE_ClockRate

TIME_OF_DEPARTURE_ClockRate indicates the clock rate used for TIME_OF_DEPARTURE.

REVmc editor: modify REVmc D0.5  P1668L14 as follows:

16.2 DSSS PHY sublayer

16.2.1 Overview

Subclause 16.2 (DSSS PLCP sublayer) provides a convergence procedure in which MPDUs are converted to

and from PPDUs. During transmission, the MPDU shall be prepended with a PHY preamble and header to

create the PPDU. At the receiver, the PHY preamble and header are processed to aid in demodulation and

delivery of the MPDU.

16.2.2 PHY frame format

Figure 16-1 (PLCP frame format) shows the format for the PPDU including the DSSS PHY preamble, the

DSSS PHY header, and the MPDU. The PHY preamble contains the following fields: SYNC and SFD.

The PHY header contains the following fields: (Ed)Signaling (SIGNAL), (Ed)Service (SERVICE), length

(LENGTH), and CRC-16 (CRC). Each of these fields is described in detail in 16.2.3 (PLCP field

definitions).

REVmc editor: modify REVmc D0.5  P1768L537as follows: modify Figure 16-1, change PLCP to PHY twice

REVmc editor: modify REVmc D0.5  P1668L51 as follows:

Figure 16-1—PPDU frame format

REVmc editor: modify REVmc D0.5  P1668L53 as follows:

16.2.3 PHY field definitions

16.2.3.1 General

The entire PHY preamble and header shall be transmitted using the 1 Mb/s DBPSK modulation described

in 16.4.7 (PMD transmit specifications). All transmitted bits shall be scrambled using the feedthrough

scrambler described in 16.2.4 (PLCP/DSSS PHY data scrambler and descrambler).

REVmc editor: modify REVmc D0.5  P1669L1-65 as follows:

16.2.3.2 PHY SYNC field

The SYNC field shall consist of 128 bits of scrambled (#273)1s. This field shall be provided so that the

receiver can perform the necessary operations for synchronization.

16.2.3.3 PHY SFD

The SFD shall be provided to indicate the start of PHY-dependent parameters within the PHY preamble.

The SFD shall be a 16-bit field, X'F3A0' (MSB to LSB). The LSB shall be transmitted first in time.

16.2.3.4 PHY (Ed)SIGNAL field

The 8-bit (Ed)SIGNAL field indicates to the PHY the modulation that shall be used for transmission (and

reception) of the MPDU. The data rate shall be equal to the signal field value multiplied by 100 kbit/s. The

DSSS PHY currently supports two mandatory modulation services given by the following 8-bit words,

where the LSB shall be transmitted first in time:

a) X'0A' (MSB to LSB) for 1 Mb/s DBPSK

b) X'14' (MSB to LSB) for 2 Mb/s DQPSK

The DSSS PHY rate change capability is described in 16.2.5 (PLCP data modulation and modulation rate

change). This field shall be protected by the CRC-16 FCS described in 16.2.3.7 (PLCP CRC field).

16.2.3.5 PHY (Ed)SERVICE field

The 8-bit (Ed)SERVICE field shall be reserved for future use. The LSB shall be transmitted first in time.

This field shall be protected by the CRC-16 FCS described in 16.2.3.7 (PLCP CRC field).

16.2.3.6 PHY LENGTH field

The PHY LENGTH field shall be an unsigned 16-bit integer that indicates the number of microseconds (16

to 216–1 as defined by aMPDUMaxLength) required to transmit the MPDU. The transmitted value shall be

determined from the LENGTH parameter in the TXVECTOR issued with the PHY-TXSTART.request

primitive described in 7.3.5.5 (PHY-TXSTART.request). The LENGTH field provided in the TXVECTOR

is in octets and is converted to microseconds for inclusion in the PHY LENGTH field. The LSB shall be

transmitted first in time. This field shall be protected by the CRC-16 FCS described in 16.2.3.7 (PLCP CRC

field).

16.2.3.7 PHY CRC field

The (Ed)SIGNAL, (Ed)SERVICE, and LENGTH fields shall be protected with a CRC-16 FCS. The CRC-

16 FCS shall be the (#273)1s complement of the remainder generated by the modulo 2 division of the

protected PHY fields by the polynomial:

x16 + x12 + x5 + 1

The protected bits shall be processed in transmit order. All FCS calculations shall be made prior to data

scrambling.

As an example, the SIGNAL, SERVICE, and LENGTH fields for a DBPSK signal with a packet length of

192 μs (24 octets) would be given by the following:

0101 0000 0000 0000 0000 0011 0000 0000 (leftmost bit transmitted first in time)

The (#273)1s complement FCS for these protected PHY preamble bits would be the following:

REVmc editor: modify REVmc D0.5  P1671L42 as follows:

16.2.4 DSSS PHY data scrambler and descrambler

REVmc editor: modify REVmc D0.5  P1671L54 as follows:

16.2.5 PHY data modulation and modulation rate change

The PHY preamble shall be transmitted using the 1 Mb/s DBPSK modulation.  The (Ed)SIGNAL field shall

indicate the modulation that shall be used to transmit the MPDU. The transmitter and receiver shall initiate

the modulation indicated by the (Ed)SIGNAL field starting with the first symbol (1 bit for DBPSK or 2 bits

for DQPSK) of the MPDU. The MPDU transmission rate shall be set by the DATA-RATE parameter in the

TXVECTOR issued with the PHY-TXSTART.request primitive described in 16.2.2 (TXVECTOR parameters).

REVmc editor: modify REVmc D0.5  P1672L35-P1674L3 as follows:

16.2.6 Transmit PHY

The transmit PHY is shown in Figure 16-6 (Transmit PLCP).

In order to transmit data, PHY-TXSTART.request primitive shall be enabled so that the PHY entity shall be

in the transmit state. Further, the PHY shall be set to operate at the appropriate channel through STA

management via the PLME. Other transmit parameters such as DATARATE, TX antenna, and TX power

are set via the PHY-SAP with the PHY-TXSTART.request(TXVECTOR) primitive as described in 16.2.2 (TXVECTOR parameters).

Based on the status of CCA indicated by PHY-CCA.indication primitive, the MAC assesses that the channel is

clear. A clear channel shall be indicated by PHY-CCA.indication(IDLE) primitive. If the channel is clear,

transmission of the PPDU shall be initiated by issuing the PHY-TXSTART.request(TXVECTOR) primitive.

The TXVECTOR elements for the PHY-TXSTART.request primitive are the PLCP header parameters

SIGNAL (DATARATE), SERVICE, and LENGTH, and the PHY parameters of TX_ANTENNA,

TXPWR_LEVEL, and TIME_OF_DEPARTURE_REQUESTED. The PHY header parameter LENGTH is

calculated from the TXVECTOR element by multiplying by 8 for 1 Mb/s and by 4 for 2 Mb/s.

The PHY is configured with the TXVECTOR elements TX_ANTENNA, DATARATE, and TXPWR_LEVEL.  The PHY entity shall immediately initiate data scrambling and transmission of the PLCP preamble based on the parameters passed in the PHYTXSTART.request primitive. The time required for transmit power-on ramp described in 16.4.7.8 (Transmit

power-on and power-down ramp) shall be included in the PHY SYNC field. If

dot11MgmtOptionTODImplemented and dot11MgmtOptionTODActivated are set to true and the

TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is true, then the PHY shall issue a

PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE

corresponding to the time when the first frame energy is sent by the transmitting port, and the

TIME_OF_DEPARTURE_ClockRate parameters within the TXSTATUS vector. If

dot11MgmtOptionTimingMsmtActivated is true, then the PHY shall forward the value of

TX_START_OF_FRAME_OFFSET in TXSTATUS vector. Once the PHY preamble transmission is

complete, data shall be exchanged between the MAC and the PHY by a series of PHY-DATA.request(DATA)

primitives issued by the MAC and PHY-DATA.confirm primitives issued by the PHY. The modulation rate

change, if any, shall be initiated with the first data symbol of the MPDU as described in 16.2.5 (PLCP data

modulation and modulation rate change). The PHY proceeds with MPDU transmission through a series of data

octet transfers from the MAC.

. Transmission can be prematurely terminated by the

MAC through the PHY-TXEND.request primitive. PHY-TXSTART shall be disabled by the issuance of the

PHY-TXEND.request primitive. Normal termination occurs after the transmission of the final bit of the last

MPDU octet according to the number supplied in the TXVECTOR LENGTH field. The packet transmission

shall be completed and the PHY entity shall enter the receive state (i.e., PHY-TXSTART shall be disabled). It

is recommended that chipping continue during power-down. Each PHY-TXEND.request primitive is

acknowledged with a PHY-TXEND.confirm primitive from the PHY.

A typical state machine implementation of the transmit PHY is provided in Figure 16-7 (PLCP transmit

state machine).

Figure 16-6—Transmit PHY

Figure 16-7—PHY transmit state machine

REVmc editor: modify REVmc D0.5  P1674L49-P1677L52 as follows:

16.2.7 Receive PHY

The receive PHY is shown in Figure 16-8 (Receive PLCP).

In order to receive data, PHY-TXSTART.request primitive shall be disabled so that the PHY entity is in the

receive state. Further, through STA management via the PLME, the PHY is set to the appropriate channel

and the CCA method is chosen. Other receive parameters such as RSSI, RCPI, signal quality (SQ), and

indicated DATARATE may be accessed via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode,

a PHY-CCA.indication(BUSY) primitive shall be

issued for energy detection (ED) and/or code lock prior to correct reception of the PHY header. The PHY measures the RSSI and SQ and the parameters and are reported to the MAC in the RXVECTOR.

After a PHY-CCA.indication primitive is issued, the PHY entity shall begin searching for the SFD field.

Once the SFD field is detected, CRC-16 processing shall be initiated and the PHY (Ed)SIGNAL,

(Ed)SERVICE and LENGTH fields are received. The CRC-16 FCS shall be processed. If the CRC-16 FCS

check fails, the PHY receiver shall return to the RX IDLE state as depicted in Figure 16-9 (PLCP receive

state machine). Should the status of CCA return to the IDLE state during reception prior to completion of the

full PPDU processing, the PHY receiver shall return to the RX IDLE state.

If the PHY header reception is successful (and the SIGNAL field is completely recognizable and

supported), a PHY-RXSTART.indication(RXVECTOR) primitive shall be issued. If

dot11MgmtOptionTimingMsmtActivated is true, the PHY shall do the following:

— Complete receiving the PHY header and verify the validity of the PHY Header.

— If the PHY header reception is successful (and the SIGNAL field is completely recognizable

and supported), a PHY-RXSTART.indication(RXVECTOR) shall be issued and

RX_START_OF_FRAME_OFFSET parameter within the RXVECTOR shall be forwarded (see

16.2.3 (RXVECTOR parameters)).

NOTE—The RX_START_OF_FRAME_OFFSET value is used as described in 6.3.57 (Timing measurement) in order

to estimate when the start of the preamble for the incoming frame was detected on the medium at the receive antenna

port.

The RXVECTOR associated with this primitive includes the SIGNAL field, the SERVICE field, the MPDU

length in octets (calculated from the LENGTH field in microseconds), the antenna used for receive

(RX_ANTENNA), RSSI, RCPI, and SQ.

The received MPDU bits are assembled into octets and presented to the MAC using a series of PHY-DATA.

indication(DATA) primitive exchanges. The rate change indicated in the (Ed)SIGNAL field shall be

initiated with the first symbol of the MPDU as described in 16.2.5 (PLCP data modulation and modulation

rate change). The PHY proceeds with MPDU reception. After the reception of the final bit of the last MPDU

octet indicated by the PHY preamble LENGTH field, the receiver shall be returned to the RX IDLE state as

shown in Figure 16-9 (PLCP receive state machine). A PHY-RXEND.indication(NoError) primitive shall be

issued. A PHY-CCA.indication(IDLE) primitive shall be issued following a change in PHY carrier sense

(PHYCS) and/or PHY energy detection (PHYED) according to the selected CCA method.

In the event that a change in PHYCS or PHYED would cause the status of CCA to return to the IDLE state

before the complete reception of the MPDU as indicated by the PHY LENGTH field, the error condition

shall be reported to the MAC using a PHY-RXEND.indication(CarrierLost) primitive. The DSSS PHY shall

ensure that the CCA indicates a busy medium for the intended duration of the transmitted packet.

If the PHY header is successful, but the indicated rate in the SIGNAL field is not receivable, a PHY-RXSTART.indication primitive shall not be issued. The PHY shall indicate the error condition by issuing a

PHY-RXEND.indication(UnsupportedRate) primitive. If the PHY header is successful, but the SERVICE

field is out of IEEE Std(#130) 802.11 DSSS specification, a PHY-RXSTART.indication primitive shall not

be issued. The PHY shall indicate the error condition using a PHY-RXEND.indication(FormatViolation)

primitive. Also, in both cases, the DSSS PHY shall ensure that the CCA indicates a busy medium for the

intended duration of the transmitted frame as indicated by the LENGTH field. The intended duration is

indicated by the LENGTH field (length 1 μs).

A typical state machine implementation of the receive PHY is provided in Figure 16-9 (PLCP receive state

machine).

Figure 16-8—Receive PHY

Figure 16-9—PHY receive state machine

REVmc editor: modify REVmc D0.5 Table 16-2 P1679L20-46 as follows

aTxPHYDelay

aRxPHYDelay

aPHYHeaderLength

REVmc editor: delete REVmc D0.5  Clause 16.4-16.4.5

REVmc editor: modify REVmc D0.5  P1692L1 as follows:

16.4.6 PHY operating specifications, general

Subclause 16.4.6 (PMD operating specifications, general) provides general specifications for the DSSS

PHY sublayer. These specifications apply to both the Receive and the Transmit functions and general

operation of a DSSS PHY.

REVmc editor: modify REVmc D0.5  P1694L20 as follows:

16.4.7 PHY transmit specifications

16.4.7.1 Introduction

The transmit functions and parameters associated with the PHY sublayer are described in 16.4.7.2

(Transmit power levels) to 16.4.7.10 (Transmit modulation accuracy).

REVmc editor: modify REVmc D0.5  P1698L35 as follows:

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPHYTxStartRMS and aTxPHYTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined Annex T with the following test parameters:

REVmc editor: modify REVmc D0.5  P1699L1 as follows:

16.4.8 PHY receiver specifications

16.4.8.1 Introduction

The receive functions and parameters associated with the PHY sublayer are described in 16.4.8.2 (Receiver

minimum input level sensitivity) to 16.4.8.5 (CCA).

REVmc editor: modify REVmc D0.5  P1699L44-P1700L27 as follows:

16.4.8.5 CCA

The DSSS PHY shall provide the capability to perform CCA according to at least one of the following three

methods:

— CCA Mode 1: Energy above threshold. CCA shall report a busy medium upon detection of any

energy above the ED threshold.

— CCA Mode 2: CS only. CCA shall report a busy medium only upon detection of a DSSS signal. This

signal may be above or below the ED threshold.

— CCA Mode 3: CS with energy above threshold. CCA shall report a busy medium upon detection of a

DSSS signal with energy above the ED threshold.

A busy channel shall be indicated by a PHY-CCA.indication primitive of class BUSY.

A clear channel shall be indicated by a PHY-CCA.indication primitive of class IDLE.

The dot11CCAModeSupported shall indicate the appropriate operation modes. The PHY shall be configured

through dot11CurrentCCAMode.

The CCA shall be true if there is no energy detect or CS. The CCA parameters are subject to the following

criteria:

a) The ED threshold shall be  –80 dBm for TX power > 100 mW, –76 dBm for 50 mW < TX power 

100 mW, and –70 dBm for TX power  50 mW.

b) With a valid signal (according to the CCA mode of operation) present at the receiver antenna within

5 μs of the start of a MAC slot boundary, the CCA indicator shall report channel busy before the end

of the slot time. This implies that the CCA signal is available as an exposed test point. Refer to

Figure 9-14 (DCF timing relationships) (in 9.3.7 (DCF timing relations)) for a definition of slot time

boundary.

c) In the event that a correct PHY header is received, the DSSS PHY shall hold the CCA signal

inactive (channel busy) for the full duration as indicated by the PHY LENGTH field. Should a loss

of CS occur in the middle of reception, the CCA shall indicate a busy medium for the intended

duration of the transmitted packet

Conformance to DSSS PHY CCA shall be demonstrated by applying a DSSS-compliant signal, above the

appropriate ED threshold (item a), so that all conditions described in item b and item c are demonstrated.

17. High Rate direct sequence spread spectrum (HR/DSSS) PHY specification

REVmc editor: modify REVmc D0.5  P1701L23 as follows:

The basic High Rate PHY uses the same PHY preamble and header as the DSSS PHY, so both PHYs can co-exist in the same BSS and can use the rate switching mechanism as provided.

REVmc editor: modify REVmc D0.5  P1701L37 as follows:

Another optional mode is provided that allows data throughput at the higher rates (2, 5.5, and 11 Mb/s) to be

significantly increased by using a shorter PHY preamble.

REVmc editor: modify REVmc D0.5  P1701L50-P1702L3 as follows:

17.1.2 Scope

This clause specifies the PHY entity for the HR/DSSS extension and explains how this standard

accommodates the High Rate PHY.

The High Rate PHY consists of the following two protocol functions:

a) A

function that defines a method for mapping the MPDUs into a

framing format suitable for sending and receiving user data and management information between

two or more STAs. The PHY exchanges PPDUs that contain

PSDUs. The MAC uses the PHY service, so each MPDU corresponds to a PSDU that is carried in a

PPDU.

b) A function that defines the characteristics of, and method of transmitting and

receiving data through, a WM between two or more STAs, each using the High Rate PHY system.

REVmc editor: modify REVmc D0.5  P1702L4-33 as follows:

17.1.3 High Rate PHY functions

17.1.3.1 General

The 2.4 GHz High Rate PHY architecture is depicted in the ISO/IEC basic reference model shown in

Figure 17-10 (Layer reference model) (in 17.4.1 (Scope and field of application)). The High Rate PHY

contains two functional entities: the PHY functionand the layer

management function. Each of these functions is described in detail in 17.2 (High Rate PLCP sublayer) and 17.3 (High Rate PLME). For the purposes of MAC and MAC management, when Channel Agility is both present and enabled (see 17.3.2 (High Rate PHY MIB) and Annex J), the High Rate PHY

shall be interpreted to be both a High Rate and an FH PHY.

The High Rate PHY service shall be provided to the MAC through the PHY service primitives described in

Clause 7 (PHY service specification).

REVmc editor: modify REVmc D0.5  P1702L53-65 as follows:

17.2 High Rate PHY sublayer

17.2.1 Overview

Subclause 17.2 (High Rate PLCP sublayer) provides a convergence procedure for the 2 Mb/s, 5.5 Mb/s, and

11 Mb/s specification, in which PSDUs are converted to and from PPDUs. During transmission, the PSDU

shall be appended to a PHY preamble and header to create the PPDU. Two different preambles and headers

are defined: the mandatory supported long preamble and header, which interoperates with the current 1 Mb/

s and 2 Mb/s DSSS specification, and an optional short preamble and header. At the receiver, the PHY

preamble and header are processed to aid in demodulation and delivery of the PSDU.

REVmc editor: modify REVmc D0.5  P1703L15-26 as follows:

17.2.2.2 Long PPDU format

Figure 17-1 (Long PPDU format(#273)) shows the format for the interoperable (long) PPDU, including the

High Rate PHY preamble, the High Rate PHY header, and the PSDU. The PHY preamble contains the following fields: SYNC and SFD. The PHY header contains the following fields: signaling (SIGNAL), service (SERVICE), length (LENGTH), and CRC-16. Each of these fields is described in detail in 17.2.3 (PPDU field definitions). The format for the PPDU, including the long High Rate PHY preamble, the long High Rate PHY header, and the PSDU, does not differ from the format for 1 Mb/s and 2 Mb/s. The only

exceptions are:

REVmc editor: modify REVmc D0.5  P1703L38 as follows: modify Figure 17-1, change PLCP to PHY twice

REVmc editor: modify REVmc D0.5  P1703L55-P1705L28 as follows:

17.2.2.3 Short PPDU format

The short PHY preamble and header (HR/DSSS/short) is defined as optional for HR/DSSS. The short

preamble and header may be used to minimize overhead and, thus, maximize the network data throughput.

The format of the PPDU, with HR/DSSS/short, is depicted in Figure 17-2 (Short PPDU format(#273)). For

Clause 19 (Extended Rate PHY (ERP) specification) STAs support of this preamble type is mandatory.

A transmitter using the short PPDU is only interoperable with another receiver that is also capable of

receiving this short PPDU. To interoperate with a receiver that is not capable of receiving a short preamble

and header, the transmitter shall use the long PHY preamble and header. The short PHY preamble uses the 1 Mb/s Barker code spreading with DBPSK modulation. The short PHY header uses the 2 Mb/s Barker

code spreading with DQPSK modulation, and the PSDU is transmitted at 2 Mb/s, 5.5 Mb/s, or 11 Mb/s.

REVmc editor: modify REVmc D0.5  P1704L1 as follows: modify Figure 17-2, change PLCP to PHY twice

REVmc editor: modify REVmc D0.5  P1704L29-38 as follows:

17.2.3 PPDU field definitions

17.2.3.1 General

In the PHY field definition subclauses (17.2.3.2 (Long PLCP SYNC field) to 17.2.3.15 (Short PLCP data

modulation and modulation rate change)), the definitions of the long (Clause 16 (DSSS PHY specification

for the 2.4 GHz band designated for ISM applications)) PHY fields are given first, followed by the

definitions of the short PHY fields. The names for the short PHY fields are preceded by the term short.

REVmc editor: modify REVmc D0.5  P1704L40 as follows:

17.2.3.2 Long PHY SYNC field

REVmc editor: modify REVmc D0.5  P1704L54 as follows:

17.2.3.3 Long PHY SFD

The SFD shall be provided to indicate the start of PHY-dependent parameters within the PHY preamble.

REVmc editor: modify REVmc D0.5  P1705L1 as follows:

17.2.3.4 Long PHY SIGNAL field

REVmc editor: modify REVmc D0.5  P1705L20 as follows:

17.2.3.5 Long PHY SERVICE field

REVmc editor: modify REVmc D0.5  P1705L45-56 as follows:

17.2.3.6 Long PLCP LENGTH field

The PHY LENGTH field shall be an unsigned 16-bit integer that indicates the number of microseconds

required to transmit the PSDU. The transmitted value shall be determined from the LENGTH and

DATARATE parameters in the TXVECTOR issued with the PHY-TXSTART.request primitive described

in 17.3.5 (Vector descriptions).

The LENGTH field provided in the TXVECTOR is in octets and is converted to microseconds for inclusion

in the PHY LENGTH field.

REVmc editor: modify REVmc D0.5  P1707L48-53 as follows:

17.2.3.7 PHY CRC (CRC-16) field

The SIGNAL, SERVICE, and LENGTH fields shall be protected with a CRC-16 FCS. The CRC-16 FCS

shall be the (#273)1s complement of the remainder generated by the modulo 2 division of the protected

PHY fields by the polynomial

REVmc editor: modify REVmc D0.5  P1708L3 as follows:

The (#273)1s complement FCS for these protected PHY preamble bits would be the following:

REVmc editor: modify REVmc D0.5  P1708L4 as follows: modify Figure 17-3, change PLCP to PHY once

REVmc editor: modify REVmc D0.5  P1709L43 as follows:

17.2.3.8 Long PHY data modulation and modulation rate change

The long PHY preamble and header shall be transmitted using the 1 Mb/s DBPSK modulation. The

SIGNAL and SERVICE fields combined shall indicate the modulation that shall be used to transmit the

PSDU. The SIGNAL field indicates the rate, and the SERVICE field indicates the modulation. The

transmitter and receiver shall initiate the modulation and rate indicated by the SIGNAL and SERVICE

fields, starting with the first octet of the PSDU. The PSDU transmission rate shall be set by the DATARATE

parameter in the TXVECTOR, issued with the PHY-TXSTART.request primitive described in 17.3.5 (Vector descriptions).

REVmc editor: modify REVmc D0.5  P1709L55 as follows:

17.2.3.9 Short PHY synchronization (shortSYNC)

REVmc editor: modify REVmc D0.5  P1710L1 as follows:

17.2.3.10 Short PHY SFD field (shortSFD)

The shortSFD shall be a 16-bit field and be the time reverse of the field of the SFD in the long PHY

preamble (17.2.3.3 (Long PLCP SFD)).

REVmc editor: modify REVmc D0.5  P1710L9 as follows:

17.2.3.11 Short PHY SIGNAL field (shortSIGNAL)

REVmc editor: modify REVmc D0.5  P1710L22 as follows:

17.2.3.12 Short PHY SERVICE field (shortSERVICE)

REVmc editor: modify REVmc D0.5  P1710L28 as follows:

17.2.3.13 Short PHY LENGTH field (shortLENGTH)

REVmc editor: modify REVmc D0.5  P1710L40 as follows:

17.2.3.15 Short PHY data modulation and modulation rate change

The short PHY preamble shall be transmitted using the 1 Mb/s DBPSK modulation. The short PHY header shall be transmitted using the 2 Mb/s modulation. The SIGNAL and SERVICE fields combined shall

indicate the modulation that shall be used to transmit the PSDU. The SIGNAL field indicates the rate, and

the SERVICE field indicates the modulation. The transmitter and receiver shall initiate the modulation and

rate indicated by the SIGNAL and SERVICE fields, starting with the first octet of the PSDU. The PSDU

transmission rate shall be set by the DATARATE parameter in the TXVECTOR, issued with the

PHY-TXSTART.request primitive described in 17.3.5 (Vector descriptions).

REVmc editor: modify REVmc D0.5  P1710L52 as follows:

17.2.4 High Rate PHY data scrambler and descrambler

REVmc editor: modify REVmc D0.5  P1710L62 as follows:

The scrambler shall be initialized as specified in 17.2.3.9 (Short PLCP synchronization (shortSYNC)) for

the short PHY and 17.2.3.2 (Long PLCP SYNC field) for the long PHY

REVmc editor: modify REVmc D0.5  P1711L42-P1713L50 as follows:

17.2.5 Transmit PHY

The transmit procedures for a High Rate PHY using the long PHY preamble and header are the same as the

transmit procedures described in 16.2.6 (Transmit PLCP) and 16.2.7 (Receive PLCP) and do not change

apart from the ability to transmit 5.5 Mb/s and 11 Mb/s.

The transmit PHY is shown in Figure 17-7 (Transmit PLCP).

A PHY-TXSTART.request(TXVECTOR) primitive is issued by the MAC to start the transmission

of a PPDU. In addition to parameters DATARATE and LENGTH, other transmit parameters

such as PREAMBLE_TYPE and MODULATION are set via the PHY-SAP with the

PHY-TXSTART.request(TXVECTOR) primitive, as described in 17.3.5 (Vector descriptions). The

SIGNAL, SERVICE, and LENGTH fields of the PHY header are calculated as described in 17.2.3 (PPDU

field definitions).

The PHY is configured with the TXVECTOR elements TX_ANTENNA, DATARATE, and TXPWR_LEVEL.  The PHY entity shall immediately initiate data scrambling and transmission of the PHY preamble based on the parameters passed in the PHY-TXSTART.request primitive. The time required for transmit power-on ramp, described in 17.4.7.7 (Transmit power-on and power-down ramp), shall be included in the PHY SYNC field. If dot11MgmtOptionTODImplemented and dot11MgmtOptionTODActivated are true or if dot11MgmtOptionTimingMsmtActivated is true and the TXVECTOR parameter TIME_OF_DEPARTURE_REQUESTED is true, then the PHY shall issue a PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE

corresponding to the time when the first frame energy is sent by the transmitting port and TIME_OF_DEPARTURE_ClockRate parameter within the TXSTATUS vector. If

dot11MgmtOptionTimingMsmtActivated is true, then the PHY shall forward the value of

TX_START_OF_FRAME_OFFSET in TXSTATUS vector. Once the PHY preamble transmission is

complete, data shall be exchanged between the MAC and the PHY by a series of PHYDATA.

request(DATA) primitives issued by the MAC and PHY-DATA.confirm primitives issued by the

PHY. The modulation and rate change, if any, shall be initiated with the first data symbol of the PSDU, as

described in 17.2.3.8 (Long PLCP data modulation and modulation rate change) and 17.2.3.15 (Short PLCP

data modulation and modulation rate change). The PHY proceeds with PSDU transmission through a series

of data octet transfers from the MAC.

Transmission can be prematurely terminated

by the MAC through the PHY-TXEND.request primitive. PHY-TXSTART shall be disabled by the issuance

of the PHY-TXEND.request primitive. Normal termination occurs after the transmission of the final bit of

the last PSDU octet, calculated from the number supplied in the PHY preamble LENGTH and SERVICE

fields using the equations specified in 17.2.3.6 (Long PLCP LENGTH field). The PPDU transmission shall

be completed and the PHY entity shall enter the receive state (i.e., PHY-TXSTART shall be disabled). It is

recommended that modulation continue during power-down to prevent radiating a continuous wave carrier.

Each PHY-TXEND.request primitive is acknowledged with a PHY-TXEND.confirm primitive from the

PHY.

Figure 17-7—Transmit PHY

REVmc editor: modify REVmc D0.5  P1712L36-P1716L46 as follows:

17.2.6 Receive PHY

The receive procedures for receivers configured to receive the mandatory and optional PHYs, rates, and

modulations are described in this subclause. A receiver that supports this High Rate extension of the

standard is capable of receiving 5.5 Mb/s and 11 Mb/s, in addition to 1 Mb/s and 2 Mb/s. If the PHY

implements the short preamble option, it shall detect both short and long preamble formats and indicate

which type of preamble was received in the RXVECTOR. If the PHY implements the PBCC modulation

option, it shall detect either CCK or PBCC modulations, as indicated in the SIGNAL field, and shall report

the type of modulation used in the RXVECTOR.

The receiver shall implement the CCA procedure as defined in 17.4.8.5 (CCA). Upon receiving a PPDU, the

receiver shall distinguish between a long and short header format by the value of the SFD, as specified in

17.2.2 (PPDU format). The receiver shall demodulate a long PHY header using BPSK at 1 Mb/s. The

receiver shall demodulate a short PHY header using QPSK at 2 Mb/s. The receiver shall use the SIGNAL

and SERVICE fields of the PHY header to determine the data rate and modulation of the PSDU.

The receive PHY is shown in Figure 17-8 (Receive PLCP). In order to receive data, the PHY-TXSTART.

request primitive shall be disabled so that the PHY entity is in the receive state. Further, through

station management via the PLME, the PHY shall be set to the appropriate channel and the CCA method

chosen. Other receive parameters, such as RSSI, RCPI, SQ, and indicated DATARATE, may be accessed

via the PHY-SAP.

Upon receiving the transmitted energy, according to the selected CCA mode,

a PHY-CCA.indication(BUSY) primitive

shall be issued for ED and/or code lock prior to correct reception of the PHY header. The

PHY measures the SQ, RSSI, and RCPI parameters and are reported to the MAC in the RXVECTOR.

After a PHY-CCA.indication primitive is issued, the PHY entity shall begin searching for the SFD field.

Once the SFD field is detected, CRC-16 processing shall be initiated and the PHY SIGNAL, SERVICE,

and LENGTH fields shall be received. The CRC-16 FCS shall be processed. If the CRC-16 FCS check fails,

the PHY receiver shall return to the RX IDLE state, as depicted in Figure 17-9 (PLCP receive state

machine). Should the status of CCA return to the IDLE state during reception prior to completion of the full

PPDU processing, the PHY receiver shall return to the RX IDLE state.

If the PHY header reception is successful (and the SIGNAL field is completely recognizable and

supported), a PHY-RXSTART.indication(RXVECTOR) primitive shall be issued. The RXVECTOR

associated with this primitive includes:

a) The SIGNAL field

b) The SERVICE field

c) The PSDU length in octets (calculated from the LENGTH field in microseconds and the

DATARATE in Mb/s, in accordance with the formula in 17.2.3.6 (Long PLCP LENGTH field))

d) RXPREAMBLE_TYPE (which is an enumerated type taking on values SHORTPREAMBLE or

LONGPREAMBLE)

e) ANT_STATE (the antenna used for receive), RSSI, RCPI, and SQ

If dot11MgmtOptionTimingMsmtActivated is true, the PHY shall do the following:

— Complete receiving the PHY header and verify the validity of the PHY Header.

— If the PHY header reception is successful (and the SIGNAL field is completely recognizable

and supported), a PHY-RXSTART.indication(RXVECTOR) shall be issued and

RX_START_OF_FRAME_OFFSET parameter within the RXVECTOR shall be forwarded (see

17.3.5 (Vector descriptions)).

NOTE—The RX_START_OF_FRAME_OFFSET value is used as described in 6.3.57 (Timing measurement) in order

to estimate when the start of the preamble for the incoming frame was detected on the medium at the receive antenna

port.

The received PSDU bits are assembled into octets and presented to the MAC using a series of PHY-DATA.

indication(DATA) primitive exchanges. The rate and modulation change indicated in the SIGNAL

field shall be initiated with the first symbol of the PSDU, as described in 17.2.5 (Transmit PLCP). The PHY

proceeds with PSDU reception. After reception of the final bit of the last PSDU octet, indicated by the PHY

preamble LENGTH field, the receiver shall be returned to the RX IDLE state shown in Figure 17-9 (PLCP

receive state machine).

A PHY-RXEND.indication(NoError) primitive shall be issued. A PHY-CCA.indication(IDLE) primitive

shall be issued following a change in PHYCS and/or PHYED according to the selected CCA method.

In the event that a change in PHYCS or PHYED would cause the status of CCA to return to the IDLE state

before the complete reception of the PSDU, as indicated by the PHY LENGTH field, the error condition

shall be reported to the MAC using a PHY-RXEND.indication(CarrierLost) primitive. The High Rate PHY

shall ensure that the CCA indicates a busy medium for the intended duration of the transmitted PPDU.

If the PHY header is successful, but the indicated rate or modulation in the SIGNAL and SERVICE fields

is not within the capabilities of the receiver, a PHY-RXSTART.indication primitive shall not be issued. The

PHY shall indicate the error condition using a PHY-RXEND.indication(UnsupportedRate) primitive. If the

PHY header is invalid, a PHY-RXSTART.indication primitive shall not be issued, and the PHY shall

indicate the error condition using a PHY-RXEND.indication(FormatViolation) primitive. Also, in both

cases, the High Rate PHY shall ensure that the CCA indicates a busy medium for the intended duration of

the transmitted PSDU, as indicated by the LENGTH field. The intended duration is indicated by the

LENGTH field (LENGTH  1 s).

A typical state machine implementation of the receive PHY is shown in Figure 17-9 (PLCP receive state

machine).

Figure 17-9—PHY receive state machine

REVmc editor: modify REVmc D0.5 Table 17-5 P1718L25-64 as follows

aTxPHYDelay

aRxPHYDelay

aPHYHeaderLength

REVmc editor: modify REVmc D0.5 P1719L7 as follows

TXTIME = PreambleLength + PHYHeaderTime + Ceiling(((LENGTH+PBCC)  8) / DATARATE)

REVmc editor: modify REVmc D0.5 P1719L25 as follows

The value of PHYHeaderTime

REVmc editor: add the following rows to REVmc D0.5 Table 17-6 P1719L38

| TX_ANTENNA | TXVECTOR | Selects which of the available antennas should be used for transmit, in the range from 1 to 256. The number of available antennas is determined from the MIB table parameters, aSuprtRxAntennas and aSuprtTxAntennas. |
| --- | --- | --- |

REVmc editor: modify REVmc D0.5 P1720L7 as follows

false, true. When true, the MAC entity requests that the

PHY entity measures and reports time of departure

parameters corresponding to the time when the first frame

energy is sent by the transmitting port; when false, the MAC

entity requests that the PHY entity neither measures

nor reports time of departure parameters.

REVmc editor: delete REVmc D0.5  Clause 17.4-17.4.5

REVmc editor: modify REVmc D0.5 P1732L49 as follows

17.4.6 PHY operating specifications, general

17.4.6.1 General

General specifications for the High Rate PHY sublayer are provided in 17.4.6.2 (Operating frequency

range) to 17.4.6.14 (Transmit and receive antenna port impedance). These specifications apply to both the

receive and transmit functions and general operation of a High Rate PHY.

WLANs implemented in accordance with this standard are subject to equipment certification and operation

requirements established by regional and national regulatory administrations. The PHY specification

establishes minimum technical requirements for interoperability, based upon established regulations at the

time this standard was issued. These regulations are subject to revision, or may be superseded. Requirements

that are subject to local geographic regulations are annotated within the PHY specification. Regulatory

requirements that do not affect interoperability are not addressed in this standard. Implementers are referred

REVmc editor: modify REVmc D0.5 P1741L46 as follows

A conformant PHY

meets this switching time specification when the operating channel center frequency has settled to within

REVmc editor: modify REVmc D0.5 P1741L55 as follows

17.4.7 PHY transmit specifications

17.4.7.1 Introduction

The transmit functions and parameters associated with the PHY sublayer are described in 17.4.7.2

(Transmit power levels) to 17.4.7.9 (Transmit modulation accuracy).

REVmc editor: modify REVmc D0.5 P1746L18 as follows

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPHYTxStartRMS and aTxPHYTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters:

REVmc editor: modify REVmc D0.5 P1746L44 as follows

17.4.8 PHY receiver specifications

17.4.8.1 Introduction

The receive functions and parameters associated with the PHY sublayer are described in 17.4.8.2 (Receiver

minimum input level sensitivity) to 17.4.8.5 (CCA).

REVmc editor: modify REVmc D0.5 P1747L24-P1748L11 as follows

17.4.8.5 CCA

The High Rate PHY shall provide the capability to perform CCA according to at least one of the following

three methods:

— CCA Mode 1: Energy above threshold. CCA shall report a busy medium upon detecting any energy

above the ED threshold.

— CCA Mode 4: CS with timer. CCA shall start a timer whose duration is 3.65 ms and report a busy

medium only upon the detection of a High Rate PHY signal. CCA shall report an IDLE medium

after the timer expires and no High Rate PHY signal is detected. The 3.65 ms timeout is the duration

of the longest possible 5.5 Mb/s PSDU.

— CCA Mode 5: A combination of CS and energy above threshold. CCA shall report busy at least

while a High Rate PPDU with energy above the ED threshold is being received at the antenna.

A busy channel shall be indicated by PHY-CCA.indication primitive of class BUSY. A clear channel shall

be indicated by PHY-CCA.indication primitive of class IDLE.

dot11CCAModeSupported shall indicate the appropriate operation modes. The PHY shall be configured

through dot11CurrentCCAMode.

The CCA shall indicate true if there is no energy detect or CS. The CCA parameters are subject to the

following criteria:

a) If a valid High Rate signal is detected during its preamble within the CCA window, the ED threshold

shall be less than or equal to –76 dBm for TX power > 100 mW; –73 dBm for 50 mW < TX power

 100 mW; and –70 dBm for TX power 50 mW.

b) With a valid signal (according to the CCA mode of operation) present at the receiver antenna within

5 s of the start of a MAC slot boundary, the CCA indicator shall report channel busy before the end

of the slot time. This implies that the CCA signal is available as an exposed test point. Refer to

Figure 9-14 (DCF timing relationships) (in 9.3.7 (DCF timing relations)) for a slot time boundary

definition.

c) In the event that a correct PHY header is received, the High Rate PHY shall hold the CCA signal

inactive (channel busy) for the full duration, as indicated by the PHY LENGTH field. Should a loss

of CS occur in the middle of reception, the CCA shall indicate a busy medium for the intended

duration of the transmitted PPDU. Upon reception of a correct PHY header, the timer of CCA

Mode 2 shall be overridden by this requirement.

Conformance to the High Rate PHY CCA shall be demonstrated by applying an equivalent High-Rate-compliant

signal above the appropriate ED threshold (item a) so that all conditions described in item b and

item c are demonstrated.

18. Orthogonal frequency division multiplexing (OFDM) PHY specification

REVmc editor: modify REVmc D0.5  P1749L33-46 as follows:

18.1.2 Scope

Subclause 18.1 (Introduction) describes the PHY services provided to the IEEE Std(#130) 802.11 WLAN

MAC by the OFDM PHY. The OFDM PHY consists of the following protocol functions:

a) A

A function that defines a method of mapping the IEEE Std

802.11 PSDUs into a framing format suitable for sending and receiving user data and management

information between two or more STAs.

b) A function that defines the characteristics and method of transmitting and receiving

data through a WM between two or more STAs, each using the OFDM system.

REVmc editor: modify REVmc D0.5  P1749L48- P1750L13 as follows:

18.1.3.1 General

The OFDM PHY architecture is depicted in the reference model shown in Figure 4-14 (Portion of the ISO/

IEC basic reference model covered in this standard) (in 4.9 (Reference model)). The OFDM PHY contains

two functional entities: the PHY functionand the layer management

function. Each of these functions is described in detail in 18.3 and 18.

4.

The OFDM PHY service is provided to the MAC through the PHY service primitives described in Clause 7

(PHY service specification).

REVmc editor: modify REVmc D0.5  P1750L33-48 as follows:

18.2 OFDM PHY specific service parameter list

18.2.1 Introduction

The architecture of the IEEE Std(#130) 802.11 MAC is intended to be PHY independent. Some PHY

implementations require medium management state machines running in the MAC sublayer in order to meet

certain PHY requirements. These PHY-dependent MAC state machines reside in a sublayer defined as the

MLME. In certain PHY implementations, the MLME may need to interact with the PLME as part of the

normal PHY-SAP primitives. These interactions are defined by the PLME parameter list currently defined

in the PHY service primitives as TXVECTOR and RXVECTOR. The list of these parameters, and the

values they may represent, are defined in the specific PHY specifications for each PHY. Subclause 18.2

(OFDM PHY specific service parameter list) addresses the TXVECTOR and RXVECTOR for the OFDM

PHY.

REVmc editor: modify REVmc D0.5  P1751L24-30 as follows:

False, true. When true, the MAC entity requests that the PHY

entity measures and reports time of departure parameters

corresponding to the time when the first frame energy is sent by

the transmitting port; when false, the MAC entity requests that

the PHY entity neither measures nor reports time of

departure parameters.

REVmc editor: modify REVmc D0.5  P1751L36 as follows:

18.2.2.3 TXVECTOR DATARATE

The DATARATE parameter describes the bit rate at which the PHY shall transmit the PSDU.

REVmc editor: modify REVmc D0.5  P1751L55 as follows:

18.2.2.6 TIME_OF_DEPARTURE_REQUESTED

The allowed values are false or true. A parameter value of true indicates that the MAC sublayer is requesting

that the PHY entity provides measurement of when the first frame energy is sent by the transmitting port

and reporting within the PHY-TXSTART.confirm(TXSTATUS) primitive. A parameter value of false

indicates that the MAC sublayer is requesting that the PHY entity not provide time of departure

measurement nor reporting in the PHY-TXSTART.confirm(TXSTATUS) primitive.

REVmc editor: modify REVmc D0.5  P1752L58 as follows:

18.2.3.2 RXVECTOR LENGTH

The allowed values for the LENGTH parameter are in the range from 1–4095. This parameter is used to

indicate the value contained in the LENGTH field which the PHY has received in the PHY header. The

MAC and PHY use this value to determine the number of octet transfers that will occur between the two

sublayers during the transfer of the received PSDU.

REVmc editor: modify REVmc D0.5  P1752L58 as follows:

18.2.3.3 RXVECTOR RSSI

The allowed values for the RSSI parameter are in the range from 0 to RSSI maximum. This parameter is a

measure by the PHY of the energy observed at the antenna used to receive the current PPDU. RSSI shall be

measured during the reception of the PHY preamble. RSSI is intended to be used in a relative manner, and

it shall be a monotonically increasing function of the received power.

REVmc editor: modify REVmc D0.5  P1754L16 as follows:

18.3 OFDM PHY sublayer

18.3.1 Introduction

Subclause 18.3 (OFDM PHY sublayer) provides a convergence procedure in which PSDUs are converted

to and from PPDUs. During transmission, the PSDU shall be provided with a PHY preamble and header to

create the PPDU. At the receiver, the PHY preamble and header are processed to aid in demodulation and

delivery of the PSDU.

REVmc editor: modify REVmc D0.5  P1754L27 as follows:

18.3.2 PHY frame format

18.3.2.1 General

Figure 18-1 (PPDU frame format) shows the format for the PPDU including the OFDM PHY preamble,

OFDM PHY header, PSDU, tail bits, and pad bits. The PHY header contains the following fields:

LENGTH, RATE, a reserved bit, an even parity bit, and the SERVICE field. In terms of modulation, the

LENGTH, RATE, reserved bit, and parity bit (with 6 zero tail bits appended) constitute a separate single

OFDM symbol, denoted SIGNAL, which is transmitted with the most robust combination of BPSK

modulation and a coding rate of R = 1/2. The SERVICE field of the PHY header and the PSDU (with 6 zero

tail bits and pad bits appended), denoted as DATA, are transmitted at the data rate described in the RATE

field and may constitute multiple OFDM symbols.

REVmc editor: modify REVmc D0.5  P1755L5 as follows: modify Figure 18-1, change PLCP to PHY twice

REVmc editor: modify REVmc D0.5  P1755L30 as follows:

- Produce the PHY Preamble field,

REVmc editor: modify REVmc D0.5  P1755L37-44 as follows:

- Produce the PHY header field from the RATE, LENGTH, and SERVICE fields of the

TXVECTOR by filling the appropriate bit fields. The RATE and LENGTH fields of the PHY

header are encoded by a convolutional code at a rate of R = 1/2, and are subsequently mapped onto a

single BPSK encoded OFDM symbol, denoted as the SIGNAL symbol. In order to facilitate a

reliable and timely detection of the RATE and LENGTH fields, 6 zero tail bits are inserted into the

PHY header.

REVmc editor: modify REVmc D0.5  P1757L11 as follows:

18.3.2.4 Timing related parameters

Table 18-5 (Timing-related parameters) is the list of timing parameters associated with the OFDM PHY.

REVmc editor: modify REVmc D0.5  P1757L38 as follows:

TPREAMBLE: PHY preamble

duration

REVmc editor: modify REVmc D0.5  P1760L26 as follows:

18.3.3 PHY preamble (SYNC)

The PHY Preamble field is used for synchronization.

REVmc editor: modify REVmc D0.5  P1760L50 as follows:

Figure 18-4 (OFDM training structure) shows the OFDM training structure (PHY preamble), where t1 to

t10 denote short training symbols and T1 and T2 denote long training symbols. The PHY preamble is

followed by the SIGNAL field and DATA.

REVmc editor: modify REVmc D0.5  P1762L57 as follows:

18.3.4.3 PHY LENGTH field

The PHY LENGTH field shall be an unsigned 12-bit integer that indicates the number of octets in the

PSDU that the MAC is currently requesting the PHY to transmit.

REVmc editor: modify REVmc D0.5  P1763L43 as follows:

18.3.5.3 PPDU TAIL field

The PPDU TAIL field shall be six bits of 0, which are required to return the convolutional encoder to the

zero state. This procedure improves the error probability of the convolutional decoder, which relies on future

bits when decoding and which may be not be available past the end of the message. The PPDU TAIL field shall be produced by replacing six scrambled zero bits following the message end with six nonscrambled

zero bits.

REVmc editor: modify REVmc D0.5  P1764L14 as follows:

18.3.5.5 PHY DATA scrambler and descrambler

REVmc editor: modify REVmc D0.5  P1771L50 as follows:

The PHY shall provide the capability to perform CCA and report the result to the MAC.

REVmc editor: modify REVmc D0.5  P1771L55 as follows:

18.3.7 PHY data modulation and modulation rate change

The PHY preamble shall be transmitted using an OFDM modulated fixed waveform.

REVmc editor: modify REVmc D0.5  P1772L1 as follows:

18.3.8 PHY operating specifications (general)

18.3.8.1 General

General specifications for the BPSK OFDM, QPSK OFDM, 16-QAM OFDM, and 64-QAM OFDM PHY

sublayers are provided in 18.3.8.2 (Outline description) to 18.3.8.8 (Transmit and receive antenna port

impedance). These specifications apply to both the receive and transmit functions and general operation of

the OFDM PHY.

REVmc editor: modify REVmc D0.5  P1773L18 as follows:

18.3.8.3 Regulatory requirements

WLANs implemented in accordance with this standard are subject to equipment certification and operating

requirements established by regional and national regulatory administrations. The PHY specification

establishes minimum technical requirements for interoperability, based upon established regulations at the

time this standard was issued. These regulations are subject to revision, or may be superseded. Requirements

that are subject to local geographic regulations are annotated within the PHY specification. Regulatory

requirements that do not affect interoperability are not addressed in this standard. Implementers are referred

to the regulatory sources in Annex D for further information. Operation in countries within defined

regulatory domains may be subject to additional or alternative national regulations.

REVmc editor: modify REVmc D0.5  P1774L27 as follows:

REVmc editor: modify REVmc D0.5  P1774L51 as follows:

18.3.9 PHY transmit specifications

18.3.9.1 General

The transmit specifications associated with the PHY sublayer are described in 18.3.9.2 (Transmit power

levels) to 18.3.9.8 (Transmit modulation accuracy test).

REVmc editor: modify REVmc D0.5  P1778L53 as follows:

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPPHTxStartRMS and aTxPHYTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined Annex T with the following test parameters:

REVmc editor: modify REVmc D0.5  Clause 18.3.11 P1782L47-P1785L52 as follows:

18.3.11 Transmit PHY

The transmit PHY is shown in Figure 18-17 (Transmit PLCP).  In order to transmit data, the PHY-TXSTART.request primitive shall be enabled so that the PHY entity shall be in the transmit state. Further,

the PHY shall be set to operate at the appropriate frequency through STA management via the PLME. Other

transmit parameters, such as DATARATE and TX power, are set via the PHY-SAP with the PHYTXSTART.

request(TXVECTOR) primitive, as described in 18.2.2 (TXVECTOR parameters).

Figure 18-17—Transmit PHY

A clear channel shall be indicated by a PHY-CCA.indication(IDLE) primitive. The MAC considers this

indication before issuing the PHY-TXSTART.request primitive. Transmission of the PPDU shall be

initiated after receiving the PHY-TXSTART.request(TXVECTOR) primitive.  The TXVECTOR elements for the PHY-TXSTART.request primitive are the PHY header parameters DATARATE, SERVICE, and LENGTH and the PHY parameters TXPWR_LEVEL and TIME_OF_DEPARTURE_REQUESTED.

The PHY is configured with the TXVECTOR elements DATARATE and TXPWR_LEVEL.

Transmission of the PHY preamble and PHY header, based on the parameters passed in the PHY-TXSTART.request primitive, shall be immediately

initiated. If dot11MgmtOptionTODImplemented and dot11MgmtOptionTODActivated are true or if

dot11MgmtOptionTimingMsmtActivated is true and the TXVECTOR parameter

TIME_OF_DEPARTURE_REQUESTED is true, then the PHY shall issue a

PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE

corresponding to the time when the first frame energy is sent by the transmitting port and the

TIME_OF_DEPARTURE_ClockRate parameter within the TXSTATUS vector. If

dot11MgmtOptionTimingMsmtActivated is true, then the PHY shall forward the value of

TX_START_OF_FRAME_OFFSET in the TXSTATUS vector. Once PHY preamble transmission is

started, the PHY entity shall immediately initiate PHY header encoding then data scrambling and data

encoding, where the data shall be exchanged between the MAC and the PHY through a series of PHYDATA.

request(DATA) primitives issued by the MAC, and PHY-DATA.confirm primitives issued by the

PHY. The modulation rate change, if any, shall be initiated from the SERVICE field data of the PHY

header, as described in 18.3.2 (PLCP frame format).

The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The

PHY header parameter, SERVICE, and PSDU are encoded by the convolutional encoder with the

bit-stealing function described in 18.3.5.6 (Convolutional encoder).

Transmission can

be prematurely terminated by the MAC through the PHY-TXEND.request primitive. PHY-TXSTART shall

be disabled by the issuance of the PHY-TXEND.request primitive. Normal termination occurs after the

transmission of the final bit of the last PSDU octet, according to the number supplied in the OFDM PHY

preamble LENGTH field.

The packet transmission shall be completed and the PHY entity shall enter the receive state (i.e., PHY-TXSTART

shall be disabled). Each PHY-TXEND.request primitive is acknowledged with a PHY-TXEND.

confirm primitive from the PHY. If the coded PSDU (C-PSDU) length is not a multiple of the

OFDM symbol length NCBPS, the PSDU is padded prior to scrambling and coding (see 18.3.5.4 (Pad bits

(PAD))).

The GI shall be inserted in every OFDM symbol as a countermeasure against severe delay

spread.

A typical state machine implementation of the transmit PHY is provided in Figure 18-18 (PLCP transmit

state machine). Requests (.request) and confirmations(.confirm) are issued once with designated states.

Figure 18-18—PHY transmit state machine

REVmc editor: modify REVmc D0.5  Clause 18.3.12 P1786L1-P1788L39 as follows:

18.3.12 Receive PLCP

The receive PHY is shown in Figure 18-19 (Receive PLCP). In order to receive data, the PHY-TXSTART.

request primitive shall be disabled so that the PHY entity is in the receive state. Further, through

STA management (via the PLME) the PHY is set to the appropriate frequency. Other receive parameters,

such as RSSI, RCPI, and indicated DATARATE, may be accessed via the PHY-SAP.

Figure 18-19—Receive PLCP

Upon receiving the transmitted PHY preamble, the PHY measures a significant received signal strength level. This indicates activity to the MAC via PHY_CCA.indication primitive. A PHY_CCA.indication(BUSY) primitive shall be issued for reception of a signal prior to correct reception of the PPDU. The RSSI parameter is reported to the MAC in the RXVECTOR.

After a PHY-CCA.indication primitive is issued, the PHY entity shall begin receiving the training symbols

and searching for the SIGNAL in order to set the length of the data stream, the demodulation type, and the

decoding rate. Once the SIGNAL is detected, without any errors detected by a single parity (even), FEC

decode shall be initiated and the PHY (Ed)SERVICE fields and data shall be received, decoded (a Viterbi

decoder is recommended), and checked by ITU-T CRC-32. If the FCS by the ITU-T CRC-32 check fails, the

PHY receiver shall return to the RX IDLE state, as depicted in Figure 18-19 (Receive PLCP). Should the

status of CCA return to the IDLE state during reception prior to completion of the full PPDU processing, the

PHY receiver shall return to the RX IDLE state.

If the PHY header reception is successful (and the SIGNAL field is completely recognizable and

supported), a PHY-RXSTART.indication(RXVECTOR) primitive shall be issued. If

dot11MgmtOptionTimingMsmtActivated is true, the PHY shall do the following:

— Complete receiving the PHY header and verify the validity of the PHY Header.

— If the PHY header reception is successful (and the SIGNAL field is completely recognizable

and supported), a PHY-RXSTART.indication(RXVECTOR) primitive shall be issued and

RX_START_OF_FRAME_OFFSET parameter within the RXVECTOR shall be forwarded (see

18.2.3 (RXVECTOR parameters)).

NOTE—The RX_START_OF_FRAME_OFFSET value is used as described in 6.3.57 (Timing measurement) in order

to estimate when the start of the preamble for the incoming frame was detected on the medium at the receive antenna

port.

The RXVECTOR associated with this primitive includes the SIGNAL field, the SERVICE field, the PSDU

length in octets, and the RSSI. Also, in this case, the OFDM PHY shall ensure that the CCA indicates a busy

medium for the intended duration of the transmitted frame, as indicated by the LENGTH field.

The received PSDU bits are assembled into octets, decoded, and presented to the MAC using a series of

PHY-DATA.indication(DATA) primitive exchanges. The rate change indicated in the (Ed)SIGNAL field

shall be initiated from the SERVICE field data of the PHY header, as described in 18.3.2 (PLCP frame

format). The PHY shall proceed with PSDU reception. After the reception of the final bit of the last PSDU

octet indicated by the PHY preamble LENGTH field, the receiver shall be returned to the RX IDLE state,

as shown in Figure 18-19 (Receive PLCP). A PHY-RXEND.indication(NoError) primitive shall be issued.

In the event that a change in the RSSI causes the status of the CCA to return to the IDLE state before the

complete reception of the PSDU, as indicated by the PHY LENGTH field, the error condition shall be

reported to the MAC using a PHY-RXEND.indication(CarrierLost) primitive. The OFDM PHY shall ensure

that the CCA indicates a busy medium for the intended duration of the transmitted packet.

If the indicated rate in the SIGNAL field is not receivable, a PHY-RXSTART.indication primitive shall not

be issued. The PHY shall indicate the error condition using a PHY-RXEND.indication(UnsupportedRate)

primitive and hold CCA busy for the calculated duration of the PPDU. If the PHY header is receivable, but

the parity check of the PHY header is not valid, a PHY-RXSTART.indication primitive shall not be issued.

The PHY shall indicate the error condition using a PHY-RXEND.indication(FormatViolation) primitive.

Any data received after the indicated data length are considered pad bits (to fill out an OFDM symbol) and

should be discarded.

A typical state machine implementation of the receive PHY is given in Figure 18-20 (PLCP receive state

machine).

Figure 18-20—PHY receive state machine

REVmc editor: modify REVmc D0.5  Table 18-17 P1786L16 as follows:

aTxPHYDelay

aRxPHYDelay

REVmc editor: delete REVmc D0.5  Clause 18.5

19. Extended Rate PHY (ERP) specification

REVmc editor: modify REVmc D0.5  P1799L48 as follows:

The ERP has the capability to decode all Clause 16 (DSSS PHY specification for

the 2.4 GHz band designated for ISM applications) and Clause 17 (High Rate direct sequence spread

spectrum (HR/DSSS) PHY specification) PHYs and all ERP-OFDM PHYs.

REVmc editor: modify REVmc D0.5  P1800L11 as follows:

- Support of the short PPDU header format capability of 17.2.2.3 (Short PPDU format) is mandatory.

REVmc editor: modify REVmc D0.5  P1801L6-L23 as follows:

19.1.4 Scope

This clause specifies the ERP entity and the deviations from earlier clauses to accommodate it. It is

organized by reference to the relevant earlier clauses to avoid excessive duplication.

The ERP consists of the following protocol functions:

- A function that defines a method for mapping the

MPDUs into a framing format suitable for sending and receiving user data and management

information between two or more STAs using the associated PHY system. The PHY exchanges

PPDUs that contain PSDUs. The MAC uses the PHY service, so each MPDU corresponds to a

PSDU that is carried in a PPDU.

- A function that defines the characteristics and method of transmitting and receiving

data through a WM between two or more STAs; each using the ERP.

REVmc editor: modify REVmc D0.5  P1801L24-L37 as follows:

19.1.5 ERP functions

The architecture of the ERP is depicted in the ISO/IEC basic reference model shown in Figure 17-10 (Layer

reference model) of 17.4.1 (Scope and field of application). The ERP contains two functional entities: the

PHY functionand the layer management function.

The ERP service is provided to the MAC through the PHY service primitives described in Clause 7 (PHY

service specification). Interoperability is addressed by use of the CS mechanism specified in 9.3.2.1 (CS

mechanism) and the protection mechanism in 9.23 (Protection mechanisms). This mechanism allows

NonERP STAs to know of ERP traffic that they cannot demodulate so that they may defer the medium to

that traffic.

REVmc editor: modify REVmc D0.5  P1801L39-L54 as follows:

19.2 PHY-specific service parameter list

The architecture of the IEEE Std(#130) 802.11 MAC is intended to be PHY independent. Some PHY

implementations require PHY-dependent MAC state machines running in the MAC sublayer in order to

meet certain PHY requirements. The PHY-dependent MAC state machine resides in a sublayer defined as

the MLME. In certain PHY implementations, the MLME may need to interact with the PLME as part of the

normal PHY SAP primitives. These interactions are defined by the PLME parameter list currently defined in

the PHY service primitives as TXVECTOR, TXSTATUS, and RXVECTOR. The list of these parameters

and the values they may represent are defined in the specific PHY specifications for each PHY. This

subclause addresses the TXVECTOR, TXSTATUS, and RXVECTOR for the ERP. The service parameters

for TXVECTOR, TXSTATUS, and RXVECTOR shall follow 18.2.2 (TXVECTOR parameters), 18.2.4

(TXSTATUS parameters), and 18.2.3 (RXVECTOR parameters), respectively.

REVmc editor: modify REVmc D0.5  P1802L38-L42 as follows:

False, true. When true, the MAC entity requests that the PHY entity measures

and reports time of departure parameters corresponding to the time when the first

frame energy is sent by the transmitting port; when false, the MAC entity requests that

the PHY entity neither measures nor reports time of departure parameters.

REVmc editor: modify REVmc D0.5  P1803L36 as follows:

The RCPI is a measure of the received channel power and is included when

dot11RadioMeasurementActivated is true. The 8-bit RCPI value is described in

18.2.3.6 (RXVECTOR RCPI) and 17.4.8.6 (Received Channel Power Indicator Measurement).

REVmc editor: modify REVmc D0.5  P1803L46-L55 as follows:

19.3 Extended Rate PHY sublayer

19.3.1 Introduction

Subclause 19.3 (Extended Rate PLCP sublayer) provides a PHY for the ERP. The convergence procedure

specifies how PSDUs are converted to and from PPDUs at the transmitter and receiver. The PPDU is formed

during data transmission by appending the PSDU to the Extended Rate PHY preamble and header. At the

receiver, the PHY preamble and header are processed to aid in the demodulation and delivery of the PSDU.

REVmc editor: modify REVmc D0.5  P1805L1 as follows:

19.3.2.2.2 ERP length field calculation

REVmc editor: modify REVmc D0.5  P1806L13-L21 as follows:

19.3.2.3 Short preamble PPDU format

Figure 17-2 (Short PPDU format(#273)) of 17.2.2.3 (Short PPDU format) shows the basic format for the

short preamble PPDU. For the ERP, support for this preamble is mandatory. The short preamble is

appropriate for use with 2, 5.5, and 11 Mb/s modes. The bits of the Short PHY SERVICE field and RATE

field are the same as for the Long PHY SERVICE field and RATE field and are defined in 19.3.2.2 (Long

preamble PPDU format).

REVmc editor: modify REVmc D0.5  P1806L23-L27 as follows:

19.3.2.4 ERP-OFDM PPDU format

The format, preamble, and headers for the ERP-OFDM PPDU are described in 18.3.2 (PLCP frame

format) to 18.3.5 (DATA field).

REVmc editor: modify REVmc D0.5  P1808L40-L53 as follows:

19.3.3 PHY data modulation and rate change

19.3.3.1 Long and short preamble formats

The long and short PHY preamble and the long PHY header shall be transmitted using the 1 Mb/s DBPSK modulation. The short PHY header shall be transmitted using the 2 Mb/s modulation. The SIGNAL and SERVICE fields combined shall indicate the modulation and rate that shall be used to transmit the PSDU.

The transmitter and receiver shall initiate the modulation and rate indicated by the SIGNAL and SERVICE

fields, starting with the first octet of the PSDU. The PSDU transmission rate shall be set by the DATARATE

parameter in the TXVECTOR, issued with the PHY-TXSTART.request primitive described in 17.3.5 (Vector descriptions).

REVmc editor: modify REVmc D0.5  P1811L37-L42 as follows:

19.3.3.3 ERP-OFDM format

PHY modulation and rate change for the ERP-OFDM frame format follows 18.3.7 (PLCP data modulation

and modulation rate change).

REVmc editor: modify REVmc D0.5  P1812L55-P1813L4 as follows:

19.3.4 PHY transmit procedure

The transmit procedure depends on the data rate and modulation format requested. For data rates of 1, 2, 5.5,

11, 22, and 33 Mb/s, the PHY transmit procedure shall follow 17.2.5 (Transmit PLCP). For the

ERP_OFDM rates of 6, 12, and 24 Mb/s and the rates of 9, 18, 36, 48, and 54 Mb/s, the PHY transmit

procedure shall follow 18.3.11 (Transmit PLCP).

REVmc editor: modify REVmc D0.5  P1813L6 as follows:

19.3.5 CCA

The PHY shall provide the capability to perform a CCA and report the results of the assessment to the

MAC. The CCA mechanism shall detect a “medium busy” condition for all supported preamble and header

types. That is, the CCA mechanism shall detect that the medium is busy for the PPDUs specified in

18.3.3 (PLCP preamble (SYNC)) and 17.2.2 (PPDU format). The CCA mechanism performance

requirements are given in 19.4.7 (CCA performance).

The ERP shall provide the capability to perform CCA according to the following method:

CCA Mode (ED and CS): A combination of CS and energy above threshold. CCA shall have a

mechanism for CS that detects all mandatory Clause 19 (Extended Rate PHY (ERP) specification)

sync symbols. This CCA’s mode’s CS shall include both Barker code sync detection and OFDM

sync symbol detection. CCA shall report busy at least while a PPDU with energy above the ED

threshold is being received at the antenna.

A busy channel shall be indicated by PHY-CCA.

indication primitive of class BUSY. A clear channel shall be indicated by PHY-CCA.indication

primitive of class IDLE.

REVmc editor: modify REVmc D0.5  P1813L31 as follows:

19.3.6 PLCP receive procedure

This subclause describes the procedure used by receivers of the ERP. An ERP receiver shall be capable of

receiving 1, 2, 5.5, and 11 Mb/s PPDUs using either the long or short preamble formats described in

REVmc editor: modify REVmc D0.5  P1813L48 as follows:

In the case where the preamble is an ERP-OFDM preamble, the PHY receive procedure shall follow the procedure described in 18.3.12 (Receive PLCP).

REVmc editor: modify REVmc D0.5  P1814L6 as follows:

19.4 ERP operating specifications (general)

19.4.1 Introduction

Subclauses 19.4.2 (Regulatory requirements) to 19.4.8 (PMD transmit specifications) provide general

specifications for the ERP sublayers. These specifications are based on 18.3.8 (PMD operating

specifications (general)) except where noted.

REVmc editor: modify REVmc D0.5  P1814L64 as follows:

b) In the event that a correct PHY header is received, the ERP shall hold the CCA signal inactive (channel busy) for the full duration, as indicated by the PHY LENGTH field.

REVmc editor: modify REVmc D0.5  P1815L19 as follows:

19.4.8 PHY transmit specifications

19.4.8.1 General

The PHY transmit specifications shall follow 18.3.9 (PMD transmit specifications) with the exception of the

REVmc editor: modify REVmc D0.5  P1815L64 as follows:

Subclause 19.5 (ERP operation specifications) describes the receive specifications for the PHY sublayer.

REVmc editor: modify REVmc D0.5  P1817L47 as follows:

REVmc editor: modify REVmc D0.5  P1832L57 as follows:

19.8.4 ERP-OFDM PHY characteristics

REVmc editor: modify REVmc D0.5  Table 19-8 P1833L1 as follows:

aTxPHYDelay

aRxPHYDelay

aPHYHeaderLength

REVmc editor: delete REVmc D0.5  Clause 19.9

20. High Throughput (HT) PHY specification

REVmc editor: modify REVmc D0.5  P1839L48 as follows:

20.1.2 Scope

The services provided to the MAC by the HT PHY consist of the following protocol functions:

a) A

function

that defines a method of mapping the PSDUs into a framing format (PPDU)

suitable for sending and receiving PSDUs between two or more STAs

.

b) A function that defines the characteristics and method of transmitting and receiving

data through a wireless medium between two or more STAs. Depending on the PPDU format, these

STAs support a mixture of HT PHY and Clause 16 (DSSS PHY specification for the 2.4 GHz band

designated for ISM applications), Clause 18 (Orthogonal frequency division multiplexing (OFDM)

PHY specification), Clause 17 (High Rate direct sequence spread spectrum (HR/DSSS) PHY

specification), or Clause 19 (Extended Rate PHY (ERP) specification) PHYs.

REVmc editor: modify REVmc D0.5  P1840L1 as follows:

20.1.3 HT PHY functions

20.1.3.1 General

The HT PHY contains two functional entities: the PHY

and the layer management function (i.e., the PLME). Both of these functions are described in detail

in 20.3 (HT PLCP sublayer) and 20.4 (HT PLME).

The HT PHY service is provided to the MAC through the PHY service primitives defined in Clause 7 (PHY

service specification).

REVmc editor: modify REVmc D0.5  P1843L36 as follows:

RSSI shall be measured during the reception of the PHY preamble.

REVmc editor: modify REVmc D0.5  P1848L12 as follows:

Enumerated type:

True indicates that the MAC entity requests that the PHY entity

measures and reports time of departure parameters corresponding to the

time when the first frame energy is sent by the transmitting port. False

indicates that the MAC entity requests that the PHY entity neither

measures nor reports time of departure parameters.

REVmc editor: modify REVmc D0.5  P1852L23 as follows:

20.3 HT PHY sublayer

20.3.1 Introduction

This subclause provides a procedure in which PSDUs are converted to and from PPDUs. During transmission, the PSDU is processed (i.e., scrambled and coded) and appended to the PHY preamble to create the PPDU. At the receiver, the PHY preamble is processed to aid in demodulation and delivery of the PSDU.

REVmc editor: modify REVmc D0.5  P1852L55 as follows:

20.3.2 PPDU format

Two formats are defined for the PPDU: HT-mixed format and HT-greenfield format.

REVmc editor: modify REVmc D0.5  P1853L33 as follows:

The elements of the PPDU are summarized in Table 20-5 (Elements of the HT PLCP packet).

Table 20-5—Elements of the HT PPDU

REVmc editor: modify REVmc D0.5  P1854L9 as follows:

PHY preambles in which HT-DLTFs are followed by HT-ELTFs are referred to as staggered preambles.

REVmc editor: modify REVmc D0.5  P1855L62 as follows:

Produce the PHY preamble training fields for each of the

REVmc editor: modify REVmc D0.5  P1857L1 as follows:

relative placement of the PHY preamble training fields vary depending on the frame format being

REVmc editor: modify REVmc D0.5  P1857L7 as follows:

b) Construct the PHY preamble SIGNAL fields from the appropriate fields of the TXVECTOR by

adding tail bits, applying convolutional coding, formatting into one or more OFDM symbols,

applying cyclic shifts, applying spatial processing, calculating an inverse Fourier transform for each

OFDM symbol and transmit chain, and prepending a cyclic prefix or GI to each OFDM symbol in

each transmit chain. The number and placement of the PHY preamble SIGNAL fields depend on

the frame format being used. Refer to 20.3.9.3.5 (L-SIG definition), 20.3.9.4.3 (HT-SIG definition),

and 20.3.9.5.4 (HT-greenfield format HT-SIG).

c) Concatenate the PHY preamble training and SIGNAL fields for each transmit chain one field after

another, in the appropriate order, as described in 20.3.2 (PPDU format) and 20.3.7 (Mathematical

description of signals).

REVmc editor: modify REVmc D0.5  P1859L39 as follows:

s) Append the OFDM symbols associated with each transmit chain one after another, starting after the

final field of the PHY preamble. Refer to 20.3.2 (PPDU format) and 20.3.7 (Mathematical

description of signals) for details.

REVmc editor: modify REVmc D0.5  P1880L23 as follows:

The result is a shorter and more efficient PPDU frame format that includes a STF, LTF(s),

and an HT-SIG.

REVmc editor: modify REVmc D0.5  P1908L58 as follows:

The MIMO channel measurement takes place in every PPDU as a result of transmitting the HT-LTFs as part

of the PHY preamble.

REVmc editor: modify REVmc D0.5  P1911L15 as follows:

20.3.14 Regulatory requirements

Wireless LANs (WLANs) implemented in accordance with this standard are subject to equipment

certification and operating requirements established by regional and national regulatory administrations. The

PHY specification establishes minimum technical requirements for interoperability, based upon established

regulations at the time this standard was issued. These regulations are subject to revision or may be

superseded. Requirements that are subject to local geographic regulations are annotated within the PHY

specification. Regulatory requirements that do not affect interoperability are not addressed in this standard.

Implementers are referred to the regulatory sources in Annex D for further information. Operation in

countries within defined regulatory domains may be subject to additional or alternative national regulations.

REVmc editor: modify REVmc D0.5  P1912L29 as follows:

REVmc editor: modify REVmc D0.5  P1912L44 as follows:

20.3.20 PHY transmit specification

REVmc editor: modify REVmc D0.5  P1918L3 as follows:

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPHYTxStartRMS and aTxPHYTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters:

REVmc editor: modify REVmc D0.5  P1918L39 as follows:

20.3.21 HT PHY receiver specification

REVmc editor: modify REVmc D0.5  P1921L57-P1924L37 as follows:

20.3.22 PHY transmit procedure

There are three options for the transmit PHY procedure. The first two options, for which typical transmit

procedures are shown in Figure 20-22 (PLCP transmit procedure (HT-mixed format PPDU)) and Figure 20-

23 (PLCP transmit procedure (HT-greenfield format PPDU)), are selected if the FORMAT field of the

PHY-TXSTART.request(TXVECTOR) primitive is equal to HT_MF or HT_GF, respectively. These

transmit procedures do not describe the operation of optional features, such as LDPC or STBC. The third

option is to follow the transmit procedure in Clause 18 (Orthogonal frequency division multiplexing

(OFDM) PHY specification) or Clause 19 (Extended Rate PHY (ERP) specification) if the FORMAT field

is equal to NON_HT. Additionally, if the FORMAT field is equal to NON_HT, CH_BANDWIDTH

indicates NON_HT_CBW20, and NON_HT_MODULATION indicates OFDM, follow the transmit

procedure in Clause 18 (Orthogonal frequency division multiplexing (OFDM) PHY specification). If the

FORMAT field is equal to NON_HT, CH_BANDWIDTH indicates NON_HT_CBW20, and

NON_HT_MODULATION indicates other than OFDM, follow the transmit procedure in Clause 19

(Extended Rate PHY (ERP) specification). And furthermore, if the FORMAT field is equal to NON_HT and

CH_BANDWIDTH indicates NON_HT_CBW40, follow the transmit procedure in Clause 18 (Orthogonal

frequency division multiplexing (OFDM) PHY specification), except that the signal in Clause 18

(Orthogonal frequency division multiplexing (OFDM) PHY specification) is generated simultaneously on

each of the upper and lower 20 MHz channels that constitute the 40 MHz channel as defined in 20.3.8

(Transmission in the upper/lower 20 MHz of a 40 MHz channel) and 20.3.11.12 (Non-HT duplicate

transmission). In all these options, in order to transmit data, the PHY-TXSTART.request primitive shall be

enabled so that the PHY entity shall be in the transmit state. Further, the PHY shall be set to operate at the

appropriate frequency through station management via the PLME, as specified in 20.4 (HT PLME). Other

transmit parameters, such as MCS coding types and transmit power, are set via the PHY-SAP with the

PHY-TXSTART.request(TXVECTOR) primitive, as described in 20.2.2 (TXVECTOR and RXVECTOR

parameters).

Figure 20-22—PHY transmit procedure (HT-mixed format PPDU)

Figure 20-23—PHY transmit procedure (HT-greenfield format PPDU)

A clear channel shall be indicated by a PHY-CCA.indication(IDLE) primitive. Note that under some

circumstances, the MAC uses the latest value of the PHY-CCA.indication primitive before issuing the PHY-TXSTART.request primitive. Transmission of the PPDU shall be initiated after receiving the

PHY-TXSTART.request(TXVECTOR) primitive. The TXVECTOR elements for the PHY-TXSTART.

request primitive are specified in Table 20-1 (TXVECTOR and RXVECTOR parameters).

Transmission of the PHY preamble may start if TIME_OF_DEPARTURE_REQUESTED is false, and shall start immediately if

TIME_OF_DEPARTURE_REQUESTED is true, based on the parameters passed in the PHY-TXSTART.

request primitive. If dot11MgmtOptionTODImplemented and dot11MgmtOptionTODActivated

are true or if dot11MgmtOptionTimingMsmtActivated is true and the TXVECTOR parameter

TIME_OF_DEPARTURE_REQUESTED is true, then the PHY shall issue a

PHY_TXSTART.confirm(TXSTATUS) primitive to the MAC, forwarding the TIME_OF_DEPARTURE

corresponding to the time when the first frame energy is sent by the transmitting port and the

TIME_OF_DEPARTURE_ClockRate parameter within the TXSTATUS vector. If

dot11MgmtOptionTimingMsmtActivated is true, then the PHY shall forward the value of

TX_START_OF_FRAME_OFFSET in TXSTATUS vector. The data shall then be exchanged between the

MAC and the PHY through a series of PHY-DATA.request(DATA) primitives issued by the MAC and PHY-DATA.confirm primitives issued by the PHY. Once PHY preamble transmission is started, the PHY entity

shall immediately initiate data scrambling and data encoding. The encoding method shall be based on the

FEC_CODING, CH_BANDWIDTH, and MCS parameter of the TXVECTOR. A modulation rate change, if

any, shall be initiated starting with the SERVICE field data, as described in 20.3.2 (PPDU format).

The PHY proceeds with PSDU transmission through a series of data octet transfers from the MAC. The

SERVICE field and PSDU are encoded by the encoder selected by the FEC_CODING, CH_BANDWIDTH,

and MCS parameters of the TXVECTOR as described in 20.3.3 (Transmitter block diagram).

Transmission can be prematurely terminated by the MAC through the primitive PHYTXEND.

request primitive. PHY-TXSTART shall be disabled by receiving a PHY-TXEND.request

primitive. Normal termination occurs after the transmission of the final bit of the last PSDU octet, according

to the number supplied in the LENGTH field.

The packet transmission shall be completed, and the PHY entity shall enter the receive state (i.e., PHY-TXSTART

shall be disabled). Each PHY-TXEND.request primitive is acknowledged with a PHY-TXEND.

confirm primitive from the PHY. If the length of the coded PSDU is not an integral

multiple of the OFDM symbol length, bits shall be stuffed to make the coded  PSDU length an integral multiple

of the OFDM symbol length.

The GI or short GI shall be inserted in every OFDM symbol as a countermeasure against delay

spread.

In some PPDU formats (as defined in 20.3.2 (PPDU format)), a signal extension is present. When no signal

extension is present, the PHY-TXEND.confirm primitive is generated at the end of last symbol of the PPDU.

When a signal extension is present, the PHY-TXEND.confirm primitive is generated at the end of the signal

extension.

A typical state machine implementation of the transmit PHY is provided in Figure 20-24 (PLCP transmit

state machine). Requests (.request) and confirmations (.confirm) are issued once per state as shown. This

state machine does not describe the operation of optional features, such as LDPC or STBC.

REVmc editor: modify REVmc D0.5  P1925L1 as follows:

Figure 20-24—PHY transmit state machine

REVmc editor: modify REVmc D0.5  P1924L38-P1929L61 as follows:

20.3.23 PHY receive procedure

Typical PHY receive procedures are shown in Figure 20-25 (PLCP receive procedure for HT-mixed format

PLCP format) and Figure 20-26 (PLCP receive procedure for HT-greenfield format PLCP). The receive

procedures correspond to HT-mixed format and HT-greenfield format, respectively. A typical state machine

implementation of the receive PHY is given in Figure 20-27 (PLCP receive state machine). These receive

procedures and state machine do not describe the operation of optional features, such as LDPC or STBC. If

the detected format indicates a non-HT PPDU format, refer to the receive procedure and state machine in

Clause 18 (Orthogonal frequency division multiplexing (OFDM) PHY specification) or Clause 19

(Extended Rate PHY (ERP) specification). Further, through station management (via the PLME), the PHY

is set to the appropriate frequency, as specified in 20.4 (HT PLME). Other receive parameters, such as RSSI

and indicated DATARATE, may be accessed via the PHY-SAP.

Upon receiving the transmitted PHY preamble, the PHY measures a receive signal strength. This indicates activity to the MAC via the PHY-CCA.indication primitive. A PHY-CCA.indication(BUSY, channel-list) primitive shall also be issued as an initial indication of reception of a signal. The channel-list parameter of the PHY-CCA.indication primitive is determined as

follows:

— It is absent when the operating channel width is 20 MHz.

— It is set to {primary} when the operating channel width is 40 MHz and the signal is present only in

the primary channel.

— It is set to {secondary} when the operating channel width is 40 MHz and the signal is present only in

the secondary channel.

— It is set to {primary, secondary} when the operating channel width is 40 MHz and the signal is

present in both the primary and secondary channels.

The RSSI parameter is reported to the MAC in the RXVECTOR.

After the PHY-CCA.indication(BUSY, channel-list) primitive is issued, the PHY entity shall begin receiving

the training symbols and searching for SIGNAL and HT-SIG in order to set the length of the data stream, the

demodulation type, code type, and the decoding rate. If signal loss occurs before validating L-SIG and/or

HT-SIG, the HT PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the received level

drops below the CCA sensitivity level (for a missed preamble) specified in 20.3.21.5 (CCA sensitivity). If

the check of the HT-SIG CRC is not valid, a PHY-RXSTART.indication primitive is not issued. The PHY

shall indicate the error condition using a PHY-RXEND.indication(FormatViolation) primitive. The HT PHY

shall not generate a PHY-CCA.indication(IDLE) primitive until the received level drops below the CCA

sensitivity level (for a missed preamble) specified in 20.3.21.5 (CCA sensitivity).

If the PHY preamble reception is successful and a valid HT-SIG CRC is indicated:

— Upon reception of an HT-mixed format preamble, the HT PHY shall not generate a PHYCCA.

indication(IDLE) primitive for the predicted duration of the transmitted frame, as defined by

TXTIME in 20.4.3 (TXTIME calculation), for all supported and unsupported modes except

Reserved HT-SIG Indication. Reserved HT-SIG Indication is defined in the fourth list item below.

— Upon reception of a GF preamble by an HT STA that does not support GF, the HT PHY shall not

generate a PHY-CCA.indication(IDLE) primitive until either the predicted duration of the packet

from the contents of the HT-SIG field, as defined by TXTIME in 20.4.3 (TXTIME calculation),

except Reserved HT-SIG Indication, elapses or until the received level drops below the receiver

minimum sensitivity level of BPSK, R=1/2 in Table 20-23 (Receiver minimum input level

sensitivity) + 10 dB (–72 dBm for 20 MHz, –69 dBm for 40 MHz). Reserved HT-SIG Indication is

defined in the fourth list item below.

— Upon reception of a GF preamble by an HT STA that supports GF, the HT PHY shall not generate a

PHY-CCA.indication(IDLE) primitive for the predicted duration of the transmitted frame, as

defined by TXTIME in 20.4.3 (TXTIME calculation), for all supported and unsupported modes

except Reserved HT-SIG Indication. Reserved HT-SIG Indication is defined in the fourth list item

below.

— If the HT-SIG indicates a Reserved HT-SIG Indication, the HT PHY shall not generate a PHYCCA.

indication(IDLE) primitive until the received level drops below the CCA sensitivity level

(minimum modulation and coding rate sensitivity + 20 dB) specified in 20.3.21.5 (CCA sensitivity).

Reserved HT-SIG Indication is defined as an HT-SIG with MCS field in the range 77–127 or

Reserved field = 0 or STBC field = 3 and any other HT-SIG field bit combinations that do not

correspond to modes of PHY operation defined in Clause 20 (High Throughput (HT) PHY

specification).

Subsequent to an indication of a valid HT-SIG CRC, a PHY-RXSTART.indication(RXVECTOR) primitive

shall be issued. If dot11MgmtOptionTimingMsmtActivated is true, the PHY shall do the following:

— Complete receiving the PHY header and verify the validity of the PHY Header.

— If the PHY header reception is successful (and the SIGNAL field is completely recognizable

and supported), a PHY-RXSTART.indication(RXVECTOR) shall be issued and

RX_START_OF_FRAME_OFFSET parameter within the RXVECTOR shall be forwarded (see

20.2.2 (TXVECTOR and RXVECTOR parameters)).

NOTE—The RX_START_OF_FRAME_OFFSET value is used as described in 6.3.57 (Timing measurement) in order

to estimate when the start of the preamble for the incoming frame was detected on the medium at the receive antenna

port.

The RXVECTOR associated with this primitive includes the parameters specified in Table 20-1

(TXVECTOR and RXVECTOR parameters). Upon reception of a GF preamble by an HT STA that does not

support GF, the FORMAT field of RXVECTOR is equal to HT_GF, the remaining fields may be empty, and

the PHY shall indicate the error condition using a PHY-RXEND.indication(FormatViolation) primitive. If

the HT-SIG indicates an unsupported mode or Reserved HT-SIG Indication, the PHY shall indicate the error

condition using a PHY-RXEND.indication(UnsupportedRate) primitive.

Following training and SIGNAL fields, the coded PSDU (which comprises the coded PHY

SERVICE field and scrambled and coded PSDU) shall be received. If signal loss occurs during reception

prior to completion of the PSDU reception, the error condition shall be reported to the MAC using a PHYRXEND.

indication(CarrierLost) primitive. After waiting for the intended end of the PSDU, if no signal

extension is present (as defined in 20.3.2 (PPDU format)), the PHY shall generate a PHY-CCA.

indication(IDLE) primitive and return to RX IDLE state. Otherwise, the receiver waits for the duration

of the signal extension before returning to the RX IDLE state.

The received PSDU bits are assembled into octets, decoded, and presented to the MAC using a series of

PHY-DATA.indication(DATA) primitive exchanges. The number of PSDU octets is indicated in the HT

Length field of the HT-SIG. The PHY shall proceed with PSDU reception. After the reception of the final bit

of the last PSDU octet and possible tail and padding bits, the receiver shall be returned to the RX IDLE state

if no signal extension is present (as defined in 20.3.2 (PPDU format)), as shown in Figure 20-27 (PLCP

receive state machine). Otherwise, the receiver waits for the duration of the signal extension before returning

to the RX IDLE state. A PHY-RXEND.indication(NoError) primitive shall be issued on entry to the RX

IDLE state.

While in the Signal Extension state, if the receiver detects a CS/CCA event, it issues an RXEND.indication

primitive (with the RXERROR parameter set to NoError or CarrierLost, depending on whether a carrier lost

event occurred during the reception of the PPDU), leaves the Signal Extension state, and enters the Detect

SIG state. This sequence occurs when signal-extended PPDUs are transmitted while separated by a RIFS.

If the binary convolutional code is used, any data received after the indicated data length are considered pad

bits (to fill out an OFDM symbol) and should be discarded.

Figure 20-25—PHY receive procedure for HT-mixed format PPDU format

Figure 20-26—PHY receive procedure for HT-greenfield format PPDU

Figure 20-27—PHY receive state machine

REVmc editor: modify REVmc D0.5  P1934L31 as follows:

20.4.3 TXTIME calculation

The value of the TXTIME parameter returned by the PLME-TXTIME.confirm primitive or calculated for

the PHY receive procedure shall be calculated for HT-mixed format according to the Equation (20-91) and

Equation (20-92) for short and regular GI, respectively, and for HT-greenfield format according to

Equation (20-93) and Equation (20-94) for short and regular GI, respectively:

REVmc editor: modify REVmc D0.5  Table 20-25  P1935L25 as follows:

aTxPHYDelay

aRxPHYDelay

aPHYHeaderLength

aPHYSigTwoLength

aPHYServiceLength

aPHYConvolutionalTailLength

REVmc editor: delete REVmc D0.5  Clause 20.5