IEEE P802.11
Wireless LANs

| Higher Layer Packet Container Proposal Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-01-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | Allied Telesis R&D Center | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 JAPAN | +81 92 771 7630 | hmorioka@root-hq.com |
| Hiroki Nakano | Trans New Technology, Inc. | Sumitomo Seimei Kyoto Bldg. 8F, 62 Tukiboko-cho, Shimogyo, Kyoto 600-8492 JAPAN | +81-75-213-1200 | cas@trans-nt.com |

Abstract

This document is a proposal normative text for the TGai draft and addresses higher layer setup.

# 3. Definitions, acronyms and abbreviations

## 3.2 Definitions specific to IEEE802.11

# Insert the following new definitions into clause 3 in alphabetical order:

Fast Initial Link Setup (FILS): The service used to establish access point/station (AP/STA) mapping and enable STA invocation of the distribution system service and higher layer packets transport during association in short latency and minimized frame exchanges.

Higher Layer Packet (HLP): A packet used in the layer higher than IEEE802.11.

# 6. Layer management

## 6.3 MLME SAP interface

### 6.3.3 Scan

### 6.3.3.3 MLME-SCAN.confirm

### 6.3.3.3.2 Semantics of the service primitive

Insert the following row to the BSSDescriptionFromFDSet:

| Name | Type | Valid Range | Description |
| --- | --- | --- | --- |
| HLP Max Wait Time | Integer | 1-100 | The maximum time to wait HLP by AP in unit of msec. |

### 6.3.7 Associate

### 6.3.7.2 MLME-ASSOCIATE.request

### 6.3.7.2.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.2.2 as shown:

MLME-ASSOCIATE.request(

PeerSTAAddress,

AssociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capability,

Extended Capability,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

HLP Wait Time,

HLP Container,

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.2.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HLP Wait Time | Integer | 1-100 | The time to wait HLP by AP in unit of msec. |
| HLP Container | As defined in frame format | As defined in 8.4.2.a (HLP Container IE) | This field is used for transporting higher layer packets in FILS association. The parameter is present only if dot11HLPTransportDuringAssocActivated is true. |

### 6.3.7.3 MLME-ASSOCIATE.confirm

### 6.3.7.3.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.3.2 as shown:

MLME-ASSOCIATE.confirm(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

RCPI.request,

RSNI.request,

RCPI.response,

RSNI.response,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOpeartiongClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

HLP Container,

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.3.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HLP Container | As defined in frame format | As defined in 8.4.2.a (HLP Container IE) | This field is used for transporting higher layer packets in FILS association. The parameter is present only if dot11HLPTransportDuringAssocActivated is true. |

### 6.3.7.4 MLME-ASSOCIATE.indicate

### 6.3.7.4.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.4.2 as shown:

MLME-ASSOCIATE.indicate(

PeerSTAAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capability,

Extended Capability,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

HLP Container,

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.4.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HLP Container | As defined in frame format | As defined in 8.4.2.a (HLP Container IE) | This field is used for transporting higher layer packets in FILS association. The parameter is present only if dot11HLPTransportDuringAssocActivated is true. |

### 6.3.7.5 MLME-ASSOCIATE.response

### 6.3.7.5.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.5.2 as shown:

MLME-ASSOCIATE.response(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

EDCAParameterSet,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOpeartiongClasses,

DSERegisteredLocation,

HTCapabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

HLP Container,

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.5.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| HLP Container | As defined in frame format | As defined in 8.4.2.a (HLP Container IE) | This field is used for transporting higher layer packets in FILS association. The parameter is present only if dot11HLPTransportDuringAssocActivated is true. |

# 8. Frame formats

## 8.3 Format of individual frame types

### 8.3.3 Management Frames

### 8.3.3.2 Beacon frame format

Insert the following rows to the contents of Table 8-20:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | HLP Max Wait Time | The HLP Max Wait Time element is present if dot11HLPTransportDuringAssocActivated is true. |

### 8.3.3.5 Association Request frame format

Insert the following rows to the contents of Table 8-22:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | HLP Wait Time | The HLP Wait Time element is present if dot11HLPTransportDuringAssocActivated is true. |
| <ANA> | HLP Container | The HLP Container element is present if dot11HLPTransportDuringAssocActivated is true. |

### 8.3.3.6 Association Response frame format

Insert the following rows to the contents of Table 8-23:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | HLP Container | The HLP Container element is present if dot11HLPTransportDuringAssocActivated is true. |

### 8.3.3.10 Probe Response frame format

Insert the following rows to the contents of Table 8-27:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | HLP Max Wait Time | The HLP Max Wait Time element is present if dot11HLPTransportDuringAssocActivated is true. |

## 8.4 Management frame body components

### 8.4.2 Information elements

### 8.4.2.1 General

Insert the following rows to the contents of Table 8-54:

| Element | Element ID | Length of indicated element (in octets) | Extensible |
| --- | --- | --- | --- |
| HLP Max Wait Time | ANA | 3 |  |
| HLP Wait Time | ANA | 3 |  |
| HLP Container (see 8.4.2.a HLP Container) | ANA | 3 to 257 |  |

# Insert the following new subclauses into clause 8.4.2:

### 8.4.2.a HLP Max Wait Time element

The HLP Max Wait Time element contains the maximum time that the AP allows to wait HLP from the AP receives Association Request. This element is transmitted in Beacon and Probe Response. The element format is shown in Figure 8-b.

|  | Element ID | Length | Max Wait Time |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 8-b – HLP Max Wait Time element format

The Element ID field is equal to the HLP Max Wait Time value in Table 8-54 (Element IDs).

The value of the Length field is 1.

The value of the Max Wait Time field is dot11HLPMaxWaitTime in unit of millisecond.

### 8.4.2.c HLP Wait Time element

The HLP Wait Time element contains the time that the non-AP STA requests to wait HLP from the AP receives Association Request. This element is transmitted in Association Request. The element format is shown in Figure 8-d.

|  | Element ID | Length | Wait Time |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |

Figure 8-d – HLP Wait Time element format

The Element ID field is equal to the HLP Wait Time value in Table 8-54 (Element IDs).

The value of the Length field is 1.

The value of the Max Time field is dot11HLPWaitTime in unit of millisecond.

### 8.4.2.e HLP Container element

The HLP Container element contains higher layer packets which non-AP STA intends to transfer during association.  This element is transmitted in Association Request and Association Response and includes higher layer packets to be transferred.  The element has two types of format. Type 1 format is shown in Figure 8-f (HLP Container element Type 1 format). Type 2 format is shown in Figure 8-g (HLP Container element Type 2 format).

Multiple HLP Container elements may be included in an Association Request or an Association Response.

|  | Element ID | Length | NFrag | Destination MAC Address |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 |  |  |
|  | Source MAC Address |  |  |  | LLC/SNAP | HLP |
| Octets: | 6 |  |  |  | variable | variable |

Figure 8-f –HLP Container element Type 1 format

Type 1 format is used for non-fragmented HLP and the first element of fragmented HLP.

The Element ID field is equal to the HLP Container value in Table 8-54 (Element IDs).

The value of the Length field is 15 plus HLP length.

The value of the NFrag field is number of subsequent fragmentation elements of the HLP.

The value of Destination MAC Address field is the destination MAC address of the HLP.

The value of Source MAC Address field is the source MAC address of the HLP.

The value of LLC/SNAP field is the LLC header and SNAP header (if applicable) of the HLP.

The HLP field contains the HLP.

|  | Element ID | Length | HLP |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure 8-g –HLP Container element Type 2 format

Type 2 format is used for the subsequent elements of fragmented HLP.

The Element ID field is equal to the HLP Container value in Table 8-54 (Element IDs).

The value of the Length field is HLP length.

When an HLP is larger than (242 - LLC/SNAP length) octets, the HLP must be fragmented to multiple HLP Container elements. In case of fragmentation, the first element uses Type 1 format and the subsequent element uses Type 2 format. The elements must be ordered original order in the frame.

If the STA intends to transmit multiple HLPs, the STA constructs multiple HLP Container elements for each HLP.

# 10. MLME

# Insert the following new subclause into clause 10:

## 10.h FILS higher layer packets transport during association

### 10.h.1 General

This subclause describes the procedures that provide higher layer packets transport capability during association in FILS.

### 10.h.2 FILS higher layer packets transport during association procedure

If dot11HLPTransportDuringAssocActivated is true, the AP transmits beacons and probe responses which include HLP Max Wait Time element with the value of dot11HLPMaxWaitTime.

If the non-AP STA receives HLP Max Wait Time element, the non-AP STA selects the value of dot11HLPWaitTime. The value of dot11HLPWaitTime must be less than dot11AssociationResponseTimeOut and less than or equal to dot11HLPMaxWaitTime.

After exchanging FILS authentication frames, the non-AP STA may transmit HLP Wait Time element with the value of dot11HLPWaitTime and higher layer packets in HLP Container elements in Association Request as described in 8.4.2.c and 8.4.2.e.

When the AP receives Association Request including FILS HLP Container elements, the AP reconstruct the HLP(s) as described in 8.4.2.e. The AP must hold the HLP until the authentication completed. After successful authentication, the AP forwards the HLP(s) to the uplink. If the authentication fails, the AP silently discards the HLP(s).

If the AP receives HLP(s) to the STA that transmitted Association Request with FILS HLP Container elements from the uplink before dot11HLPWaitTime, the AP transmits Association Response including the HLP(s) as FILS HLP Container elements.

When the non-AP STA receives Association Response, the non-AP STA reconstruct the HLP(s) and generates MA-UNITDATA.request primitive for each HLP(s).