IEEE P802.11
Wireless LANs

| FILS-Capabilities-Indications & IP Address assignment |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2012-09-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Santosh Abraham George Cherian Jouni Malinen | Qualcomm | 5775 Morehouse Dr, San Diego, CA, USA | +1 858 651 6645 | sabraham@qti.qualcomm.com gcherian@qti.qualcomm.com jouni@qca.qualcomm.com |

Abstract

Normative text for FILS capability indications & IP Address Assignment.  FILS capability indications should be sent in beacons sent by APs that have dot11FILSActivated set to true.  Current text only describes amendments to the standard 802.11 beacon, probe request, probe response, association request and response.

Note:   Similar amendments may be needed in the FILS beacon which will be incorporated in future revisions.

The text builds on edits proposed in 11-12-1164-02-00ai-fils-auth-protocol.docx.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instruction specifies the location of the change and describes what is being changed by using strikethrough (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

This amendment’s baseline is IEEE 802.11-2012

8. Frame formats

  - Format of individual frame types

8.3.3 	Management frames

      - Beacon frame format

(11k)

| Table 8-20 -- Beacon frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | FILS Indication | The FILS Indication is present if dot11FILSActivated (#1005)is(#1217) true(#1535). |

8.3.3.5 Association Request frame format

Modify table 8-22 in section 8.3.3.5 by inserting by inserting an entry preceding vendor specific elements.

| Table 8-22—Association Request frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | Upper Layer Information | Optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |
| Last | Vendor Specific | One or more vendor-specific (#1684)elements are optionally present(#29). These (#1684)elements follow all other (#1684)elements(#1221). |

8.3.3.6 Association Response frame format

Modify table 8-23 in section 8.3.3.6 by inserting an entry preceding vendor specific elements.

| Table 8-23—Association Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | Upper Layer Information | Optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |
| Last | Vendor Specific | One or more vendor-specific (#1684)elements are optionally present(#29). These (#1684)elements follow all other (#1684)elements(#1221). |

8.3.3.7 Reassociation Response frame format

Modify table 8-23 in section 8.3.3.7 by inserting an entry preceding vendor specific elements.

| Table 8-23—Association Response frame body |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA> | Upper Layer Information | Optionally present if dot11FILSActivated (#1005)is(#1217) true(#1535). |
| Last | Vendor Specific | One or more vendor-specific (#1684)elements are optionally present(#29). These (#1684)elements follow all other (#1684)elements(#1221). |

      - Probe Response frame format

| Table 8-27 - Probe Response frame bodyinput.doc |  |  |
| --- | --- | --- |
| Order | Information | Notes |
| <ANA>(11n) | FILS Indication | The FILS Indication is present if dot11FILSActivated (#1005)is(#1217) true(#1535). |

8.4.2 Information elements

8.4.2.1 General(#28)

| Table 8-54 - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| (#1684)Element | Element ID | Length (in octets) | Extensible |
| FILS Indication (see 8.4.2.ai1) | <ANA> | 8 to 257 |  |
| Upper Layer Information (see 8.4.2.ai2) | <ANA> | 8 to 257 |  |

8.4.2.ai1 FILS Indication

The FILS Indication element information related to FILS Capabilities of the AP.

|  | Element ID |  | Length | FILS Information | Domain Information fields (3 Octets per domain) Optionally present (see discussion) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 |  | 1 | 1 | Variable |

Figure ai1 -  FILS Indication

The definitions of fields are as follows:

|  | FILS Security Type |  | IP Address Type |  | Number of Domains | Reserved |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: | B0 B1 |  | B2-B3 |  | B4-B6 | B7 |  |
|  |  |  |  | Figure ai2 - FILS Information |  |  |  |

Table 8-ai1 – FILS Indication Element Field Sttings

| FILS Security type (B0 B1) | IP Address type (B2 B3) | Number of Domains indication (B4-B6) |
| --- | --- | --- |
| ‘00’: EAP-RP with no PFS | Reserved | 000 to 111 |
| ‘01’: EAP-RP with PFS | Reserved | 000 to 111 |
| ‘10’: Non-TTP with PFS | See < Table 8-ai2 > | Reserved |
| ‘11’: Reserved | Reserved | Reserved |

Table 8-ai1 shows the possible field values for the FILS security indication element.

When the FILS Security type is EAP-RP (with or without PFS), information on IP address type is carried in the domain information fields.  With Non-TTP type security, the IP address type information is carried in B2 and B3.

When FILS Security type is EAP-RP (with or without PFS), if B4-B6 is between 0 and 6, it indicates between 1 and 7 domains available.    A 3 octet information field per domain is present in the FILS indications when B4-B6 takes values from 0 to 6.

If B4-B6 indicates a value of 7, it indicates that more than 7 domains are available.  Per domain information is absent in FILS indication Element if B4-B6 indicate a value of 7.   The STA shall use ANQP to obtain domain information if B4-B6 is set to 7.

The domain information field is a 3 octet field formatted as shown in Figure 8-ai3.  The domain information field is only present when when EAP-RP is used, i.e., when FILS security type is set to “00” or “01”.

|  | Hashed Domain name | IP Address Type See <Table 8-ai2> | Reserved |
| --- | --- | --- | --- |
| Bits: | B0-B15 | B16- B17 | B18-B23 |

Figure 8-ai3: Domain Name Field

Table 8-ai2 – IP Address Type Element

| Bit values | IP Address type |
| --- | --- |
| ‘00’ | IPv4 only |
| ‘01’ | IPv6 only |
| ‘10’ | IPv4 & IPv6 |
| ‘11’ | Reserved |

8.4.2. ai2 Upper Layer Element

Design of upper layer Element including the encryption method are TBD.  The Upper layer element wil be able to carry one or more TLV fields  for different fucntions

8.4.2.ai2.ai1  TLVs in Upper Layer Element

| Table 8-54 – Upper Layer Element TLV |  |  |  |
| --- | --- | --- | --- |
| Type(#1684)TLV | TLV ID | Length (in octets) | Extensible |
| IP Address Request/Assignment | <ANA> | 3 to 27 | No |

8.4.2.ai2.ai1.ai1 IP Address Request/Assignment Element

|  | TLV ID | Length | Request/Response Information | IP address | TTL (for IP address) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable | 4 |

Figure 8-ai4: Domain Name Field

IP address assignment element may be present in Association Request and Association Response if  dot11FILSActivated set to true

The format of the Request/Response Information field and values are given below.

|  | Request/Respose |  | Request Type |  | IP Address Type | Reserved |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bits: | B0 |  | B1 |  | B2-B3 | B4-B7 |  |
|  |  |  |  | Figure 8-ai5 - Request/Response Information |  |  |  |

Table 8-ai3 – Request Response Info field settings

| Request /Response (B0) | Request Type | IP Address Type |
| --- | --- | --- |
| 0 Indicates request for IP address | 0 – Request for new IP address 1 – Indicates Request for existing IP address | See < Table 8.ai4> for settings |
| 1 Indicates Assignment | Reserved | See < Table 8.ai4> for settings |

The values for the IP address type setting are given in Table 8-ai4

Table 8-ai4 – IP address type indication

| B2 B3 | IP address type indication |
| --- | --- |
| ‘00’ | IPv4 only |
| ‘01’ | IPv6 only |
| ‘10’ | IPv4 & IPv6 |
| ‘11’ | Reserved |

Note that if the Request/Response (B0) is set to 0 indicating a request and the request type is set to 1 indicating request for existing IP address, then the requested IP address is sent in the IP address field as show in Fig. 8-ai4.

|  | IP V4 Address |
| --- | --- |
| Octets | 4 |

Figure 8-ai6: IP address field with IPv4 address only (B2B3 = 00)

|  | IP V6 Address |
| --- | --- |
| Octets | 16 |

Figure 8-ai6: IP address field with IPv4 address only (B2B3 = 01)

|  | IP V6 Address |
| --- | --- |
| Octets | 16 |

Figure 8-ai6: IP address field with IPv6 address only (B2B3 = 01)

|  | IP V4 Address | IP V6 Address |
| --- | --- | --- |
| Octets | 4 | 16 |

Figure 8-aiy: IP address field with IPv4 and IPv6 address only (B2B3 = 10)

The TTL field is present if an IP address Is present

10. MLME

10.ai1 Management Frame Fast Initial Link Setup procedures

10.ai1.ai1 FILS Indication Element

In Beacon and Probe Response frames, a FILS indication element is included  by an AP with dot11FILSActivated set to true.  FILS indication element indicates properties of the FILS authentication protocol used and also indicates if concurrent IP address assignment is performed by the AP.  The IP address type is also indicated.

For an AP supporting upto 7 network domains, the FILS indication element carries hash of the network domain name. The hash of the domain name is produced by using the first three octets of the hash of the network domain name.

Hashed Domain Name  = Truncate-16( HMAC-SHA256(”FILS”,Domain Name)  )

For each domain, the type of IP address available is also indicated ( Table 8-ai1).

10.ai1.ai2 Association or Reassociatio Request with IP Address Assignment

An STA may include an Upper layer element with a IP address assigment TLV  in an Association or Reassociatio request sent to a an AP with  AP with dot11FILSActivated set to true.    An STA may request for a pre-assigned IP address by including the IP address in the IP address assignment TLV or may request a new IP address.

10.ai1.ai3 Association Response with IP Address Assignment

In response to an association request with an Upper layer element with a IP address assigment TLV an an  AP with dot11FILSActivated set to true may include a IP address assighment element with an assigned IP address.    The IP address included may be the same as the requested IP address if one was included in the Association Request, or may be a new IP address.  If an IP address assignment element is not included in the Association Response, then the STA may request an IP address subsequent to completion of association using any of the avaialable higher layer methods.