IEEE P802.11
Wireless LANs

| LB192 GDC Comment Resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-1-7 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics |  | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission proposes comment resolutions of the following CIDs from TGaf Draft 3.0 Recirculation Letter Ballot 192.

- 2006, 2007, 2030, 2032, 2033, 2064, 2097, 2098, 2109, 2110, 2112, 2113, 2114, 2160, 2161

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGaf Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGaf Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGaf Editor: Editing instructions preceded by “TGaf Editor” are instructions to the TGaf editor to modify existing material in the TGaf draft.  As a result of adopting the changes, the TGaf editor will execute the instructions rather than copy them to the TGaf Draft.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2006 | The opening sentence of this paragraph ("Before STAs are permitted to transmit in some regulatory domains, regulatory provisions for the use of frequencies are under the control of a GDB.") is poorly constructed. I believe the intent is that a STA is not permitted to transmit until it knows the allowed frequencies and operating parameters for its location at the current time. To determine this information, it consults the GDB. Correct? | Replace the first sentence with this text and remove the 2nd sentence. "In some regulatory domains, STAs must consult a GDB to determine permissable operating frequencies and parameters before transmitting. Such STAs may operate..." | Accepted |

Discussion:

It seems that the first sentence is redundant with the second sentence. The proposed change is reasonable.

Propose

Accepted for CID 2006, per discussion and editing instructions in 11-13/0237r0.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2007 | The phrase "STA fails to attain GDC attainment" is not well written. The meaning of GDC attainment is not precise. | Try "If the GDC dependent STA is unable to obtain a GDC Enablement Response from one GDC enabling STA, it may make further attempts with additional GDC enabling STAs....." | Accepted |

Discussion:

The proposed change is describing more precisely the meaning of the GDC attainment.

Propose

Accepted for CID 2007, per discussion and editing instructions in 11-13/0237r0.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2030 | "The Map ID field is set to a number that is equal to the Map ID of the current identifies the current valid WSM" - The 'current' what? | Add the appropriate word after "current" and may need to also add "and". | Revised |

Discussion:

The corresponding sentence has some grammer error and ambiguous point.

Propose

Revised for CID 2030, per discussion and editing instructions in 11-13/0237r0.

“The Map ID field is set to a number that is equal to the Map ID of the recently received WSM current identifies the current valid WSM. The WSM is defined in Table 8-14i (WSM information value fields) in 8.2.6.2.6 (WSM Information Values).”

|  |  | Proposed Change |  |
| --- | --- | --- | --- |

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2098 | There is missing a general statement that applies to the use of 10.43 procedures. | Insert paragraph "When a STA implements support for one or more of the procedures described in this subclause, it shall set dot11TVHTOptionImplemented to true. When dot11TVHTOptionImplemented is true, and a STA is initialized for operation in a band that requires geolocation database control, then dot11GDCActivated shall be true." | Accepted |

Discussion:

The comment is correct. Clause 10.43 is missing the description about MIB variable.

Propose

Accepted for CID 2098, per discussion and editing instructions in 11-13/0237r0.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2064 | 11AF/D2.0 contained clauses 8.4.5.6 and 8.4.5.7 describing the Neighbor Network Information query and response. These useful elements of the RLQP protocol were deleted during LB189 comment resolution without proper justification. The comment spreadsheet does not capture any reason for the deletion. These elements are a crucial part of RLQP to maintain coexistence with other 802.11 neighbors in the crowded TVWS. | Insert D2.0 clauses 8.4.5.6 and 8.4.5.7 describing the Neighbor Network Information query and response back into the current draft. | Rejected |

Discussion:

The main concern of the neighbour network information query and response procedure was that the gain achieved from the database query protocol is not obvious. It is recommended that the commenter submits the additional simulation result.

Propose

Rejected for CID 2064, it is encouraged that the commenter will submit the proposal with the performance evaluation.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2160 | Remove GDC enablement request/response frame as there is no regulatory requirement for the enablement procedure in TVWS. Enabling signal together with CAQ is sfficient. Enablement procedure adds unecessary implmentation complexity to the standard. | per comment | Rejected |
| 2161 | Remove GDC enablement request/response frame because after receiving the request, AP need to send the FCC ID of the slave device to database for verification. There will be not immediate response that can be sent to slave device. RLQP can handle this issue with delayed response. | per comment | Rejected |

Discussion:

STA transmits GDC Enablement Request frame to AP. If AP correctly receives GDC Enablement Request frame, it replies with ACK frame. Then, AP validates the Device Identification Information of the STA before transmitting GDC Enablement Response frame.

AP is not required to transmit GDC Enablement Response frame immediately after receiving GDC Enablement Request frame. So, there is no issue.

Propose

Rejected for CID 2160 and 2161, AP is not required to transmit GDC Enablement Response frame immediately after receiving GDC Enablement Request frame.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2109 | The Table E-12 WSM information value fields, e.g., channel number are not the same as used elsewhere in the standard. This can be changed to say the fields are managed by upper layers. | Replace Length and Value fields of each with "Per EN 301 598 Device Parameters information. | Revised. |

Discussion:

WSM information value fields in Table E-12 shall be aligned with other upper layer standard (such as ETSI).

Proposed change Length and Values fields with “Per EN 301 598 Generic Operational Parameters”.

Propose

Revised for CID 2109, per discussion and editing instructions in 11-13/0237r0.

TGaf Editor: Replace Table E-12 with the following:

| Name | Length | Value | Scope |
| --- | --- | --- | --- |
| Device Class | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM |
| Map ID | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM |
| Channel Number | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM, UK |
| Maximum Power Level | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM, UK |
| Validity | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM, UK |
| Maximum Channel Bandwidth | Per EN 301 598 Generic Operational Parameters | Per EN 301 598 Generic Operational Parameters | WSM, UK |

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2113 | In some regulatory domains the AP is allowed to translate between the database information and what is communicated to/from slave STAs, while in other regulatory domains, the database information is transferred at upper layers, not layer 2. Revise the T:V field descriptions in Annex E to say whether the values can be translated, where they are not specified in upper layers. | Add text to Table E-8 fields indicating when they are layer 2 specific, not from upper layers. |  |
| 2114 | As IETF paws has nearly completed database communications in FCC regulatory domain, someone should remove from layer 2 things that are specified in upper layers, as all STAs opeating in FCC domain will have been homologated to show use of that information. | Spectrum Mask descriptor and WSM field values can be specified in upper layers, and the layer 2 descriptions be containers for them. |  |

Discussion:

In some regulatoru domain, STA can use the upper layer protocol for obtainting a database information (such as EN 301 598 Generic Operational Parameters). It is needed to clarify the relationship between the upper layer meesage and 802.11af MAC management frame.

Propose

Revised for CID 2109, per discussion and editing instructions in 11-13/0237r0.

TGaf Editor: Modify P297 L59 of “E.2.5.1 General” as follows:

“…

The WSM information value fields format is shown in Table E-8 (WSM information value fields). In some regulatory domains, an AP can retrieve the WSM information from the geolocation database through the upper layer protocol (e.g., IETF Protocol to Access WS database). When the AP retrieves the WSM information from the upper layer, it may translate it to WSM information field format shown in Table E-8 which is the link-layer specific information.”

…”

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2033 | "A GDC enabling STA may transmit a GDC enabling signal in-band on an available frequency..." May it transmit it out of band? What does in-band mean here and is it needed? | Delete "in-band" | Rejected. |
| 2097 | 10.43.2 specifies attempting enablement follows only after receiving an In-band Enabling Signal, but some regulatory domains allow multiband enablement. Add text describing how multiband enablement relates to the Inband Enabling Signal. | Commenter will provide a submission if assigned any multiband enablement technical comments, including this one. | Rejected. |
| 2110 | From OFCOM rules, multiband operation is permitted, but current text is silent on whether reception of an enabling signal is not required before TV band transmission if GDC enablement was in another band. | Complete the description of multiband enablement. | Rejected. |

This is an invalid comment.

| CID | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- |
| 2112 | 10.43 GDC procedures is missing text describing how the procedures use data frames to communicate, rather than Public Action frames or RLQP. Add the descriptions of using data frames. | Revise text with Contact Verification signal is a field in a data frame. Revise text with White Space Map is conveyed in a data frame. | ??? |