IEEE P802.11
Wireless LANs

| Text for Reduced Neighbor Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-03-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | Hangyu.cho@lge.com |

Abstract

The submission provides clarified text for the Reduced Neighbor Report as following reason:

- When the AP considers its operating channel too congested to accommodate STAs, AP may redirect the STAs from current operating channel to the other channel via Reduced Neighbor Report element. However, there is no description of this Channel rediretion. Draft 0.4 only describes the Band redirection (i.e., from current operating band to other band) and AP redirection (i.e., recommend the STA to switch to the other AP). Thus, we added the Channel redirection related text in the Draft.

- Draft 0.4 does not describe the STA’s behaviour when the STA receives the Reduced Neighbor Report (i.e., Band redirection, Channel Redirection, AP Redirection) via the Reduced Neighbor Report element of the Beacon frame, FILS Discovery frame and Probe Response frame. Draft 0.4 only describes the AP’s procedure for Redirection. Thus, we add ed the description of the STA side procedure in the draft.

8.4.2.175 Reduced Neighbor Report element

Instructions to Editor: Modify the text as shown (page 25, line 31):

The TBTT Information Field Type subfield is 2 bits in length and defines the structure of the TBTT Information field. Value 0 indicates the presence of the informative Neighbor AP Information that is used to help the STA in AP discovery. Value 1 indicates the presence of the Neighbor AP Information that is used to recommend the STA to switch to the other AP of the current operating channel or to switch to the other band or to switch to other channel on the current operating band. Values 2, and 3 are reserved.

The TBTT Information Count subfield is 4 bits in length and contains the number of TBTT Information fields that are included in the Neighbor AP Information field, minus one. A value of 0 indicates one TBTT Information field is present.

The TBTT Information Length subfield is 1 octet in length and contains the length in octets of each TBTT Information field included in the Neighbor AP Information field. If the TBTT Information Field Type is set to 1 and the TBTT Information Length subfield is set to non-zero, this indicates that AP redirects the STAs to other BSS by indicating the information of the BSS of a neighbor AP and the channel. If the TBTT Information Field Type is set to 1 and the TBTT Information Length subfield is set to 0, this indicates that AP redirects the STA from current operating band to other band by including the other band information or also this indicates that AP redirects the STA from current operating channel to other channel by including the other channel information.

10. MLME

10.25.3 Reduced Neighbor Report

Instructions to Editor: Modify the text and add the new text as shown (page 57, line 59):

If the AP considers its operating channel too congested to accommodate STAs, the AP should include one (or more) Neighbor AP Information element with TBTT Information Field Type indicating value 1 to the TBTT Information Header of the Neighbor AP Information field of the Redueced Neighbor AP Report element in the Beacon or FILS Discovery frame or Probe Response frame to redirect the STA from current operating band to other band or to redirect the STA to other AP of the current operating channel or to redirect the STA to other channel on the current operating band. The decision when AP considers its operating channel the too congested is implementation specific.

Based on the Reduced Neighbor Report of Beacon frame or FILS Discover frame or Probe Response fraem which was transmitted by an AP, STA can switch to the other AP of the current operating channel or switch to the other band from current operating band or switch to other channel on the current operating band.

If the STA detects that the TBTT Information Field Type is set to 1 and the TBTT Information Length subfield is set to 0 in the TBTT Information Header of the Neighbor AP information field of the Reduced Neighbor Report elememt, STA can switch to the other band or switch to other channel on the current operating band with received Operating Class field and Channel Numbef field. That is, STA can monitor the Beacon frame or FILS Discovery Frame or can send the Probe Request frame on the indicated channel and band.

If the STA detects that the TBTT Information Field Type is set to 1 and the TBTT Information Length subfield is set to non-zero in the TBTT Information Header of the Neighbor AP information field of the Reduced Neighbor Report elememt, STA can switch to the AP which is having the BSSID indicated in the TBTT Information field.  That is, STA can send the unicast Probe Request frame including the BSSID wihich is indicated in the TBTT Informaion field for AP discovery.