IEEE P802.11
Wireless LANs

| FILS IP address Setup Proposal Text |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2013-03-18 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Affiliation | Address |  | Phone | email |
| Santosh Abraham George Cherian Jouni Malinen | Qualcomm | 5775 Morehouse Dr, San Diego, CA, USA | +1 858 651 6645 |  | sabraham@qti.qualcomm.com gcherian@qti.qualcomm.com jouni@qca.qualcomm.com |

Abstract

This document is a proposal normative text for the TGai draft IP address setup.

# 6. Layer management

## 6.3 MLME SAP interface

### 6.3.7 Associate

### 6.3.7.2 MLME-ASSOCIATE.request

### 6.3.7.2.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.2.2 as shown:

MLME-ASSOCIATE.request(

PeerSTAAddress,

AssociateFailureTimeout,

CapabilityInformation,

ListenInterval,

Supported Channels,

RSN,

QoSCapability,

Content of FT Authentication elements,

SupportedOperatingClasses,

HT Capability,

Extended Capability,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

FILS IP Address Request data,

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.2.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| FILS IP Address Request | As defined in frame format | As defined in 8.4. ai2.ai3 | FILS IP Address Request is used to request for an IP address. The request may be for a any new IP address or a specified IP address |

### 6.3.7.3 MLME-ASSOCIATE.confirm

### 6.3.7.3.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.3.2 as shown:

MLME-ASSOCIATE.confirm(

ResultCode,

CapabilityInformation,

AssociationID,

SupportedRates,

EDCAParameterSet,

RCPI.request,

RSNI.request,

RCPI.response,

RSNI.response,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOpeartiongClasses,

DSERegisteredLocation,

HT Capabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

FILS IP Address Assignment data,

FILS DNS Server Information data

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.3.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| FILS IP Address Assignment | As defined in frame format | As defined in 8.4. ai2.ai4 | FILS IP Address Assignment is used to send an IP address for the device |
| FILS DNS Information | As defined in frame format | As defined in 8.4.ai2.ai5 | FILS DNS Server Information is used to communicate DNS Server address information |

### 6.3.7.4 MLME-ASSOCIATE.indicate

### 6.3.7.4.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.4.2 as shown:

MLME-ASSOCIATE.indicate(

PeerSTAAddress,

CapabilityInformation,

ListenInterval,

SSID,

SupportedRates,

RSN,

QoSCapability,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOperatingClasses,

DSERegisteredLocation,

HT Capability,

Extended Capability,

20/40 BSS Coexistence,

QoSTrafficCapability,

TIMBroadcastRequest,

EmergencyServices,

FILS IP Address Request data,					VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.4.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| FILS IP Address Request | As defined in frame format | As defined in 8.4. ai2.ai3 | FILS IP Address Request is used to request for an IP address. The request may be for a any new IP address or a specified IP address |

### 6.3.7.5 MLME-ASSOCIATE.response

### 6.3.7.5.2 Semantics of the service primitive

# Change the primitive parameter list in 6.3.7.5.2 as shown:

MLME-ASSOCIATE.response(

PeerSTAAddress,

ResultCode,

CapabilityInformation,

AssociationID,

EDCAParameterSet,

RCPI,

RSNI,

RMEnabledCapabilities,

Content of FT Authentication elements,

SupportedOpeartiongClasses,

DSERegisteredLocation,

HTCapabilities,

Extended Capabilities,

20/40 BSS Coexistence,

TimeoutInterval,

BSSMaxIdlePeriod,

TIMBroadcastResponse,

QoSMapSet,

FILS IP Address Assignment data,

FILS DNS Server Information data

VendorSpecificInfo

)

# Insert the following row before the VendorSpecificInfo row of the untitled table defining the primitive parameters in 6.3.7.5.2:

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| FILS IP Address Assignment | As defined in frame format | As defined in 8.4. ai2.ai4 | FILS IP Address Assignment is used to send an IP address for the device |
| FILS DNS Information | As defined in frame format | As defined in 8.4.ai2.ai5 | FILS DNS Server Information is used to communicate DNS Server address information |

# 8. Frame formats

## 8.3 Format of individual frame types

### 8.3.3 Management Frames

### 8.3.3.5 Association Request frame format

Insert the following rows to the contents of Table 8-22:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Secure Encapsulation | Optionally present if dot11FILSActivated |

### 8.3.3.6 Association Response frame format

Insert the following rows to the contents of Table 8-23:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | Secure Encapsulation | Optionally present if dot11FILSActivated. |

## 8.4 Management frame body components

Insert the following rows to the contents of Table 8-38:

8.4.1 Fields that are not information elements

8.4.1.11 Action field

| Code | Meaning | See Subclause | Robust | Group Addressed Privacy |
| --- | --- | --- | --- | --- |
| <ANA> | FILS | TBD | Yes | No |

### 8.4.2 Information elements

### 8.4.2.1 General

Insert the following rows to the contents of Table 8-54:

| Table 8-54 - Element IDsinput.doc |  |  |  |
| --- | --- | --- | --- |
| (#1684)Element | Element ID | Length (in octets) | Extensible |
| Secure Encapsulation | <ANA> | 8 to 257 |  |

Modify section 8.4.2.186 as follows:

8.4.2.186 FILS Indication

The FILS Indication element information related to FILS Capabilities of the AP.

|  | Element ID | Length | FILS Information | Domain name information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | Variable |

Figure 8-401df -  FILS Indication

The definitions of fields are as follows:

|  | FILS Security Type | IP Address Type | Number of Domains | IP-Address Assignment Method | Reserved |
| --- | --- | --- | --- | --- | --- |
| Bits: | B0 B1 | B2-B4 | B5-B7 | B8-B9 | B10-B15 |

Table 8-183af – FILS Indication Element Field Sttings

| FILS Security type (B0 B1) | IP Address type (B2 –B4) | Number of Domains indication (B5-B7) |
| --- | --- | --- |
| ‘00’: EAP-RP with no PFS | Reserved | 000 to 111 |
| ‘01’: EAP-RP with PFS | Reserved | 000 to 111 |
| ‘10’: Non-TTP with PFS | See < Table 8-ai2 > | Reserved |
| ‘11’: Reserved | Reserved | Reserved |

Table 8-ai1 shows the possible field values for the FILS security indication element.

When the FILS Security type is EAP-RP (with or without PFS), information on IP address type is carried in the domain information fields.  With Non-TTP type security, the IP address type information is carried in B2 and B3.

When FILS Security type is EAP-RP (with or without PFS), if B4-B6 is between 0 and 6, it indicates between 1 and 7 domains available respectively.    A 3 octet information field per domain is present in the FILS indications when B4-B6 takes values from 0 to 6.

If B4-B6 indicates a value of 7, it indicates that more than 7 domains are available.  Per domain information is absent in FILS indication Element if B4-B6 indicate a value of 7.   The STA shall use ANQP to obtain domain information if B4-B6 is set to 7.

The domain information field is a 3 octet field formatted as shown in Figure 8-ai3.  The domain information field is only present when EAP-RP is used, i.e., when FILS security type is set to “00” or “01”.  The domain information consists of a hashed domain name and the IP address type corresponding to the information.  The hashed domain name is computed from the Domain Name that is compliant with the “Preferred Name Syntax” as defined in IETF RFC 1035 (same as the domain name used in clause 8.4.4.15 Domain Name ANQP element).  The exact computation method for the hashed domain name is given in clause 10.ai1.ai1.

Table 8-183ag-ai-a – IP Address Assignment Method

| Bit values | IP Address Assignment Method supported by the AP |
| --- | --- |
| ‘00’ | IP Address assignment during Association is not supported by the AP |
| ‘01’ | STA may use FILS IP Addr Request TLV to request IP address during Association |
| Others | Reserved |

Insert the following new subclauses into clause 8.4.2:

8.4.2.ai1 Secure Encapsulation Element

Design of Secure Encapsulation Element including the encryption method are TBD.  The Secure Encapsulation  element will be able to carry one or more Higher Layer Encapsulation TLV fields  for different functions

### 8.4.ai1 Higher Layer Encapsulation TLVs

Higher Layer Encapsulation TLVS allow the transport of information for  various Higher Layer protocols in Information Elements of Management Frames or in Higher Layer Transport Action Frames.  Table 8-55 shows the currently defined TLVs.  Higher Layer Encapsulation TLVs can be included in the Secure Encapsulation Element..

Table 8-54 –  Higher Layer Encapsulation TLV

| Type(#1684)TLV | TLV ID | Length (in octets) | Extensible |
| --- | --- | --- | --- |
| FILS IP Address Request | <ANA> | 3 to 255 | No |
| FILS IP Address Assignment | <ANA> | 3 to 255 | No |
| FILS DNS Information | <ANA> | 3 to 255 |  |

8.4.2.ai1.ai1 IP Address Request Element

|  | TLV ID | Length | IP Address Request Control | Requested IPv4 address (Optional) | Requested IPv6 address (Optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 16 |

IP address request element may be present in Association Request and Association Response if  dot11FILSActivated set to true.

The format of the Request/Response Information field and values are given below.

|  | IPv4 Request | IPv4 Request type | IPv6 Request | IPv6 Request type | DNS server address Req | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: | 1 | 1 | 1 | 1 | 1 | 3 |

The following table explains the operation of  the IP Address Request Control field

| IPv4 Request | Set to 1 if an IPv4 address is requested |
| --- | --- |
| IPv4 Request type | - Request the IPv4 address that is present in the TLV - Request a new IPv4 address (no IPv4 address present in TLV) |
| IPv6 Request | Set to 1 if an IPv6 address is requested |
| IPv6 Request type | 0-Request the IPv6 address that is present in the TLV 1-Request a new IPv6 address (no IPv6 address present in TLV) |
| DNS Server Address Req | Set to 1 if STA is requesting DNS server(s) addresss(es) |

8.4.2.ai1.ai2 FILS IP Address Assignment TLV

|  | TLV ID | Length | IP Address Response Control |  |  | Assigned IPv4 address (Optional) |  |  | Gateway IPv4 address (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 |  |  | 4 |  |  | 4 |
|  | IPv4 Gateway MAC Address |  |  |  | Subnet Mask |  | Assigned IPv6 address (Optional) |  |  |
| Octets: | 6 |  |  |  | 4 |  | 16 |  |  |
|  | Gateway IPv6 address (Optional) |  |  | IPv6 Gateway MAC Address |  |  |  | IPv6 Prefix Length (Optional) |  |
| Octets: | 16 |  |  | 6 |  |  |  | 1 |  |

Figure -8.4.2.ai2.ai3-1 IP Address Assignment TLV

|  | IPv4 Assigned | Subnet mask included | IPv6 Assigned | Prefix Length included | IP address assignment pending | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: | 1 | 1 | 1 | 1 | 1 | 3 |

Figure -8.4.2.ai1.ai2-2 IP Address Response Control

Table -8.4.2.ai1.ai2-1 IP Address Response Control

| IPv4 Assigned | Set to 1 if an IPv4 address is present. Assigned IPv4 address, Gateway IPv4 address and IPv4 Gateway MAC Address are included if this bit is set to ‘1’ |
| --- | --- |
| Subnet mask included | Set to 1 if IPv4 Assigned is set to ‘1’ and if the subnet mask is included for the IPv4 address |
| IPv6 Assigned | Set to 1 if an IPv6 address is present. Assigned IPv6 address, Gateway IPv6 address and IPv6 Gateway MAC Address are included if this bit is set to ‘1’ |
| Prefix Length included | Set to 1 IPv6 Assigned=1 and if the prefix length is included |
| IP address assignment pending | Set to 1 if the AP is still working on obtaining the IP address for the STA |

8.4.2.ai1.ai3 FILS DNS Information TLV

|  | TLV ID | Length | DNS Info Control | DNS Server IPv4Address (Optional) | DNS Server IPv6Address (Optional) | IPv4 DNS Server MAC Address (Optional) | IPv6 DNS Server MAC Address (Optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 16 | 6 | 6 |

Figure -8.4.2.ai1.ai3-1 DNS Server Information TLV

|  | DNS Server IPv4 address Present | DNS Server IPv6 address Present | IPv4 DNS Server MAC Address Present | IPv6 DNS Server MAC Address Present | Reserved |
| --- | --- | --- | --- | --- | --- |
| Bits: | 1 | 1 | 1 | 1 | 5 |

Figure -8.4.2.ai1.ai3-2 DNS Server Info Control field

DNS server address TLV carries IP address and MAC address information of the DNS Server to which the DNS queries may be sent

| DNS Server IPv4 address Present | Set to 1 if an IPv4 address is present |
| --- | --- |
| DNS Server IPv4 address Present | Set to 1 if an IPv6 address is present |
| IPv4 DNS Server MAC Address Present | Set to 1 if MAC address to which IPv4 based DNS queries may be sent is present |
| IPv6 DNS Server MAC Address Present | Set to 1 if MAC address to which IPv6 based DNS queries may be sent is present |

8.5 Action frame format details

### 8.5.ai1 FILS Action Frames

The FILS Action Frame is used for FILS operation post association

| FILS Action field value |  |
| --- | --- |
| 0 | Sequence of Higher Layer Encapsulation TLVs |
| 1-255 | Reserved |

8.5.ai1.ai1  Higher Layer Transport

With an action value of 0, the FILS action frame carries one or more Higher Layer Encapsulation TLVs.

# 10. MLME

# Insert the following new subclause into clause 10:

10.ai1 Management Frame Fast Initial Link Setup procedures

### 10.ai1.ai1 IP address setup during association procedure

An STA may include a Secure Encapsulation element with IP Config data TLV in an Association or Reassociation request sent to a an AP with  AP with dot11FILSActivated set to true.    An STA may request for a pre-assigned IP address by including the IP address in the IP address assignment TLV or may request a new IP address.

When the AP receives an Association Request including Secure Encapsulation element  or a  FILS Action Frame with FILS IP Address Request TLV, the AP must initiate procedure to assign IP address for the STA using mechanisms that is not specified in this standard.

The AP may send Association Response with Security Encapsulation Element  or FILS Action Frame including the assigned IP address  in a FILS IP Address Assignment TLV.   In addition the AP may also send one or more DNS Information TLVs to provide address information of one or more DNS Servers.  If The IP address included may be the same as the requested IP address if one was included in the Association Request/FILS Action Frame, or may be a new IP address.

If the STA has included IP Address Request TLV in the Association Request frame, and if the AP has set the IP address assignment pending flag in the IP Address Response Control field of the FILS IP Address Assignment TLV to ‘1’ in the Association Response frame, then the AP should send the FILS IP Address Assignment TLV in the FILS Action Frame. If the STA does not receive the FILS Action Frame within a duration chosen by the implementation, then the STA may initiate IP address assignment procedure using mechanisms that are out of scope of this specification.

STA may use FILS Action Frame to re-request an IP Address if the TTL of the IP address is about to expire.