IEEE P802.11
Wireless LANs

| IP Subnet indication Proposal Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Santosh Abraham George Cherian Jouni Malinen | Qualcomm | 5775 Morehouse Dr, San Diego, CA, USA | +1 858 651 6645 | sabraham@qti.qualcomm.com gcherian@qti.qualcomm.com jouni@qca.qualcomm.com |

Abstract

This document is a proposal normative text for indicating a token that is representative of IP subnet. This is used by STA for selecting the AP that could provide IP continuity

Modify section 8.4.2.186 as follows:

8.4.2.186 FILS Indication

The FILS Indication element information related to FILS Capabilities of the AP.

|  | Element ID | Length | FILS Information | Domain name information |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | Variable |

Figure 8-401df -  FILS Indication

The definitions of fields are as follows:

|  | FILS Security Type | IP Address Type | Number of Domains | Subnet-ID token present | Reserved |
| --- | --- | --- | --- | --- | --- |
| Bits: | B0 B1 | B2-B4 | B5-B7 | B8 | B9-B15 |

Table 8-183af – FILS Indication Element Field Sttings

| FILS Security type (B0 B1) | IP Address type (B2 –B4) | Number of Domains indication (B5-B7) |
| --- | --- | --- |
| ‘00’: EAP-RP with no PFS | Reserved | 000 to 111 |
| ‘01’: EAP-RP with PFS | Reserved | 000 to 111 |
| ‘10’: Non-TTP with PFS | See < Table 8-ai2 > | Reserved |
| ‘11’: Reserved | Reserved | Reserved |

Table 8-ai1 shows the possible field values for the FILS security indication element.

When the FILS Security type is EAP-RP (with or without PFS), information on IP address type is carried in the domain information fields.  With Non-TTP type security, the IP address type information is carried in B2 and B3.

When FILS Security type is EAP-RP (with or without PFS), if B4-B6 is between 0 and 6, it indicates between 1 and 7 domains available respectively.    A 3 octet information field per domain is present in the FILS indications when B4-B6 takes values from 0 to 6.

If B4-B6 indicates a value of 7, it indicates that more than 7 domains are available.  Per domain information is absent in FILS indication Element if B4-B6 indicate a value of 7.   The STA shall use ANQP to obtain domain information if B4-B6 is set to 7.

The domain information field is a 3 octet field formatted as shown in Figure 8-ai3.  The domain information field is only present when when EAP-RP is used, i.e., when FILS security type is set to “00” or “01”.  The domain information consists of a hashed domain name and the IP address type corresponding to the information.  The hashed domain name is computed from the Domain Name that is compliant with the “Preferred Name Syntax” as defined in IETF RFC 1035 (same as the domain name used in clause 8.4.4.15 Domain Name ANQP element).  The exact computation method for the hashed domain name is given in clause 10.ai1.ai1.

|  | Hashed Domain Name | IP Address Type See <Table 8-ai2> | Reserved | Subnet ID token |
| --- | --- | --- | --- | --- |
| Bits: | B0-B15 | B16- B18 | B19-B23 | B24-B31 |

Figure 8-401dg: Domain Information Field

The Subnet-ID token is an identifier derived from the subnet using a hash of the subnet or other means that is out of scope of this specification. The Subnet-ID token is used by the STA to select an AP that is connected to the same IP domain as the current AP.

Table 8-183ag – IP Address Type

| Bit values | IP Address type |
| --- | --- |
| ‘000’ | IPv4 only |
| ‘001’ | IPv6 only |
| ‘010’ | IPv4 & IPv6 |
| ‘011-111’ | Reserved |

Table 8-183ag-ai-b – Subnet-ID token present

| Bit values | Subnet-ID token present |
| --- | --- |
| ‘0’ | A subnet-ID token corresponding to the IP subnet to which the domain is connected is not present in the Domain information field |
| ‘1’ | A subnet-ID token corresponding to the IP subnet to which the domain is connected is present in the Domain information field |