IEEE P802.11
Wireless LANs

| 802.11 REVmc – A proposal for re-expressing the 802.11 rate rules |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-01-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission contains a proposal (in outline) of how to re-express the 802.11-2012 rate rules.

# Problem Statement

The subclause 9.7 rate rules in 802.11-2012 are not fit for purpose.  They have grown organically since the days when rate selection was a selection of 1 versus 2 Mbps.  The structure of the rules is chaotic.  Transmit  “rate” is a function of:

- Rate,  or MCS

- Channel bandwidth (5, 10, 20, 40 MHz)

- Short/long GI

- Packet format (DSSS, ERP, OFDM, HT_GF, HT_MF, non-HT duplicate)

The purpose of the rules broadly follows these intents:

- To provide protection / communication of NAV values in response frames

- To make the durations of response frame predictable

- To make the frames receivable by the intended recipient(s)

Along the way we invented and then appear to lose interest in the concept of “modulation class”.

Another problem is that there is no assurance from the existing structure that everything that needs to be specified is actually specified.   Although rate/MCS selection might be adequately covered,   the rules for channel bandwidth are somewhat distributed between this section and the protection section,  and there is no assurance that something didn’t slip through the cracks here.

# What shall we do?

I propose that we can re-structure the rate selection process into an expression driven by a tabular expression based on conditions and rules.    A particular rule applies based on specified conditions.  If the conditions can be made self-documenting and unambiguous, there is an improved likelihood that we will cover all (or all the relevant) conditions.

# Proposal

Replace Clause 9.7 with the following structure.

9.7 Multirate support

9.7.1. Overview

Multirate support defines rules for the selection of:

- PPDU format

- Rate or MCS

- Channel bandwidth

for use in the transmission of a PPDU based on conditions known to the transmitter.

9.7.2. Conditions

Table x-1 defines conditions referenced in Table x-3.

Table x-1 – Multirate condition definitions

| Condition Name | Definition of condition |
| --- | --- |
| BSS | The STA is a member of a BSS |
| Joined | The STA has joined a BSS (either infrastructure or BSS) |
| Associated | The STA is a non-AP STA associated with an AP |
| IBSS | The STA is a member of an IBSS |
| Mesh | The STA is a mesh STA |
| AP | The STA is contained in an AP |
| CapKnown | The capabilities of the intended individual receiver are known |
| ProtRequired | Protection for non-ERP receivers is enabled, the frame is a protection mechanism frame, or the frame initiates an exchange |
| DualBeacon | The Dual Beacon field in the HT Operation element is equal to 1 |
| DualCTS | The DualCTS Protection field in the HT Operation element is equal to 1 |
| BasicMCS | The BSSBasicMCSSet is non-empty |
| Individual | The receiver address is an individual address |
| Group | The receiver address is a group address |
| Control | The transmission is a Control frame |
| Data | The transmission is a Data frame |
| Management | The transmission is a Management frame |
| PSMP | The transmission is a PSMP frame |
| CFPoll | The transmission is a +CF-Poll frame |
| CFAck | The transmission is a +CF-Ack frame |
| CFEnd | The transmission is a CF-End frame |
| BasicBA | The transmission is a Basic BockAckReq frame or Basic BlockAck frame |
| A-MPDU | The transmission is a frame within an A-MPDU that contains Data or Management frames |
| Beacon | The transmission is a Beacon frame |
| TXOPStart | The transmission initiates a TXOP |
| NAVSet | An initial exchange has already established protection and the Duration/ID field in the frame establishing protection covers the entire TXOP |
| NAVSetDup | A non-HT duplicate frame was used to establish protection of the current TXOP |
| DualCTSSet | Dual CTS protection was used to establish protection of the current TXOP |
| LSIGStart | The transmission carries an L-SIG duration value |
| PCO40 | The transmission occurs during the 40 MHz phase of PCO operation |
| MultipleBSSID | The transmission is by an AP that supports multiple BSSID |
| STBC | The transmission is an STBC frame |
| nonHT | The transmission is a non-HT frame |
| HT | The transmission is an HT frame |
| Otherwise | None of the preceding conditions from the same column match for the same conditions of any columns to the left. |

9.7.3 Rules

Table x-2 defines rules to be followed, as selected by Table x-3.

Table x-2 – Multirate rule definitions

| Rule Name | Definition of rule |
| --- | --- |
| CapReceivable | A STA shall not transmit a frame using a rate or MCS that is not supported by the receiver STA or STAs, as reported in any Supported Rates element, Extended Supported Rates element, or Supported MCS field in management frames transmitted by the receiver STA. A STA shall not transmit a frame using a value for the CH_BANDWIDTH parameter of the TXVECTOR that is not supported by the receiver STA |
| RateReceivable | STA shall using a rate supported by the receiver STA, as reported in the Supported Rates element and/or ExtendedSupported Rates element in frames transmitted by that STA. |
| TxCap | A STA shall not initiate transmission of a frame at a data rate higher thanthe greatest rate in the OperationalRateSet or the HTOperationalMCSset, which are parameters of the MLME-JOIN.request primitive. |
| NonHTBSSReceivable | The transmitting STA shall transmit a non-HT PPDU using a rate in the BSSBasicRateSet parameter or a rate from the mandatory rate set of the attached PHY if the BSSBasicRateSet is empty. |
| BSSReceivable | The transmitting STA shall transmit using a rate in the BSSBasicRateSet parameter, or an MCS in the BSSBasicMCSSet parameter, or a rate from the mandatory rate set of the attached PHY if both the BSSBasicRateSet and the BSSBasicMCSSet are empty. |
| HTBSSReceivable | The STA shall select an MCS from the BSSBasicMCSSet parameter |
| HTTXOPInitial | The STA shall select an MCS from the BSSBasicMCSSet parameter when protection is required (as defined in 9.23) and shall select an MCS from the SupportedMCSSet parameter of the intended receiver when protection is not required. |
| MCSReceivable | The STA shall select an MCS from the SupportedMCSSet parameter of the intended receiver. |
| BSSReceivablePreferNonHT | If the BSSBasicRateSet parameter is not empty, the STA shall transmit a non-HT PPDU using one of the rates included in the BSSBasicRateSet parameter. If the BSSBasicRateSet parameter is empty and the BSSBasicMCSSet parameter is not empty, the STA shall transmit an HT PPDU using one of the MCSs included in the BSSBasicMCSSet parameter. Otherwise the STA shall transmit a non-HT PPDU using one of the mandatory PHY rates. |
| BasicSTBCMCS | The STA shall transmit using a mandatory MCS of the attached PHY any of the following are true: — The Dual Beacon field in the HT Operation element is equal to 0, and the DualCTS Protection field in the HT Operation element is equal to 0. — No HT Operation element is present in the mostrecently received Association Response frame that was addressed to this STA. — The BSSBasicMCSSet is empty or does not exist. — The lowest MCS of the BSSBasicMCSSet has NSS value greater than 1 (the mapping of MCS to NSS is PHY dependent, for the HT PHY see 20.6). Otherwise, the STA shall transmit using an MCS that is the lowest MCS index of the BSSBasicMCSSet parameter. |
| ControlHTSelect | a) A control frame shall be carried in an HT PPDU when the control frame meets any of the following conditions: 1) The control frame contains an L-SIG duration value (see 9.23.5), or 2) The control frame is sent using an STBC frame. b) A control response frame shall be carried in an HT PPDU when the control frame is a response to a frame that meets any of the following conditions: 1) The frame eliciting the response included an HT Control field with the TRQ field equal to 1 and the NDP Announcement subfield equal to 0, and this responder set the Implicit Transmit Beamforming Receiving Capable field to 1 in itslast transmitted HT Capabilities element; or 2) The frame eliciting the response was an RTS frame carried in an HT PPDU; or 3) The frame eliciting the response was an STBC frame, and the Dual CTS Protection field was equal to 1 in the last HT Operation element received from its APor transmitted by the STA (see 9.3.2.7). c) A control frame may be carried inan HT PPDU when the control frame meets any of the following conditions: 1) The control frame contains an HT Control field with the MRQ subfield equal to 1, or 2) The control frame contains an HT Control field with the TRQ field equal to 1. NOTE—In these cases, requirements specified in 9.27, 9.28.2, and 9.29 further constrain the choice of non-HT or HT PPDU. d) Otherwise, the control frame shall be carried in a non-HT PPD |
| ControlWidthSelect | <text from 9.7.6.6 here> |
| NoShortGI | The TXVECTOR parameter GI_TYPE, if present, shall not be set to the value SHORT_GI |
| NoLDPC | The TXVECTOR parameter FEC_CODING, if present, shall not be set to the value LDPC_CODING |
| MultipleBSSReceivable | The frame shall be transmitted using any basic rate valid for all of the BSSs supported. If no such rate exists, thenBeacon frames shall be transmitted using any mandatory PHY rate for any PHY type that all BSSs have in common. |
| MeshConstantBasicRate | A mesh STA shall not establish a mesh peering with a mesh STA using a different BSSBasicRateSet (see 13.2.7 and 13.2.8). Mesh STAs should adopt the mandatory PHY rates as the default BSSBasicRateSet to reduce the risk that a candidate peer mesh STA utilizes a different BSSBasicRateSet. If the mesh STA is also an HT STA, it should adopt the MCSs of mandatory MCSs as the default BSSBasicMCSSet. Once the mesh STA establishes a mesh peering with a mesh STA, it shall change neither the BSSBasicRateSet nor the BSSBasicMCSSet parameters. |
| CFAckReceivable | The STA shall transmit using a rate or MCS and TXVECTOR parameter CH_BANDWIDTH chosen from among those supported by both the addressed recipient STA and the STA to which the acknowledgement is intended. |
| DualCTSNonAPCFEnd | The STA shall seect the same value for the TXVECTOR parameter STBC, TXVECTOR parameter MCS (if present), and TXVECTOR parameter RATE as was used for the transmission of the matching control frame at the beginning of the TXOP. The matching control frame is defined as follows: — For the first CF-End transmitted in the TXOP, the matching control frame is the first RTS transmitted in the TXOP. — For the second CF-End transmitted in the TXOP, the matching control frame is the first CTS that follows the first RTS transmitted in the TXOP. — For the third CF-End transmitted in the TXOP, the matching control frame is the second CTS that follows the first RTS transmitted in the TXOP. |
| DualCTSAPCFEnd | The STA shall select the same value for the TXVECTOR parameter STBC, TXVECTOR parameter MCS (if present), and TXVECTOR parameter RATE as was used for the transmission of the matching control frame at the beginning of the TXOP. The matching control frame is defined as follows: — For the first CF-End transmitted in the TXOP, the matching control frame is the first CTS-to-self transmitted in the TXOP. — For the second CF-End transmitted in the TXOP, the matching control frame is the first RTS transmitted in the TXOP. |

9.7.4 Multirate selection table

Table x-3 defines which rules are applicable based on the specified conditions.    Conditions are combined using AND, OR and NOT operators.

A STA shall comply with all applicable rules for which the condition is true.

Table x-3 – Multirate applicable rules

| Condition |  |  |  | Applicable Rules |
| --- | --- | --- | --- | --- |
| Joined |  |  |  | TxCap |
| CapKnown |  |  |  | CapReceivable |
| BSS AND NOT CapKnown |  |  |  | BSSReceivable |
| Control AND NOT A-MPDU | <always> |  |  | ControlHTSelect, ControlWidthSelect |
|  | TXOPStart | <always> |  | NoShortGI, NoLDPC |
|  |  | BasicBA AND nonHT | CapKnown | RateReceivable |
|  |  |  | NOT CapKnown | NonHTBSSReceivable |
|  |  | NOT BasicBA AND nonHT |  | NonHTBSSReceivable |
|  |  | NOT LSIGStart AND HT |  | HTTXOPInitial |
|  |  | LSIGStart |  | MCSReceivable |
|  | CFEnd | NOT DualCTSSet | NOT PCO40 | NonHTBSSReceivable |
|  |  |  | PCO40 | HTBSSReceivable |
|  |  | DualCTSSet | NOT AP | DualCTSNonAPCFEnd |
|  |  |  | AP | DualCTSAPCFEnd |
| other control tbd |  |  |  |  |
| Beacon AND MultipleBSSID |  |  |  | MultipleBSSReceivable |
| Mesh |  |  |  | MeshConstantBasicRate |
| Data OR Management OR (Control and A-MPDU) | Group | (Beacon OR PSMP) AND NOT STBC |  | NonHTBSSReceivable |
|  |  | STBC |  | BasicSTBCMCS |
|  |  | Otherwise |  | BSSReceivablePreferNonHT |
|  | Individual | (CFPoll AND NOT CFAck) AND NOT NAVSET |  | BSSReceivablePreferNonHT |
|  |  | CFAck |  | CFAckReceivable |
|  |  | Otherwise | CapKnown | CapReceivable (already covered above) |
|  |  |  | NOT CapKnown | BSSReceivable |