IEEE P802.11
Wireless LANs

| Text for Probe Timer setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-6 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | hg.cho@lge.com |

Abstract

The submission provides normative text for Setting of ProbeTimer (11-12-0151-r15).

6.2.11 Setting of ProbeTimer

TGai may define a mechanism for a STA to accelerate the scanning of an empty channel with no AP in operation.

In this contribution, the proposed 802.11ai Specification Document text will be presented as modifications to the TGai draft specification 802.11ai/D05. The following format conventions are used:

- The new added text is marked as blue underline text;

- The deleted text is marked as red strikethrough text;

- The unchanged baseline standard text stays in black text in the context of proposed TGai specification text;

- The editorial instruction is marked as italic text highlighted by Yellow;

10.1.4.3.2 Active scanning procedure of the scanning STA

Instructions to Editor:  modify the following text in section 10.1.4.3.2.

e) If PHY-CCA.indication (busy) primitive has not been detected before the Probe Timer reaches MinChannelTime, then go to step f, else while the Probe Timer is less than the MaxChannelTime:

- Process any received pribe responses;

- Process any received beacons, measurement pilots and FILS Discovery frames if dot11FILSActivated is true in the STA; and

- If dot11FILSActivated is true in the STA, ReportingOption is IMMEDIATE, and new AP or new information of the AP is detected, issue MLME-SCAN.confirm primitive with the ResultCode equal to IMMEDIATE_SCAN_RESULT and the BSSDescriptionSet containing information of the detected AP.

- If dot11FILSActivated is true and the ReportingOption is CHANNEL_SPECIFIC, issue at the time when the Probe Timer reaches the MaxChannelTime an MLME-SCAN.confirm primitive, with the ResultCode equal to SUCCESS and the BSSDescriptionSet containing information of all APs that have been discovered from the scanned channel.

Optionally, if the PHY-CCA.indication (busy) primitive has been detected, but no PHY-RxStart.indication primitive has been received, STAs with dot11FastChannelScanRequest equal to true set NAV to 0 and scan the next channel when ProbeTimer reaches MinChannelTime.

f) Set NAV to 0 and scan the next channel.

Instructions to Editor:  Add the following new subsection 8.4.2.188 in the page 48, line 23.

8.4.2.188 Fast Channel Scan Request element

The Fast Channel Scan Request element contains information related to Fast Channel Scan capability of the STA. The Fast Channel Scan Request element is optionally present in the Probe Request frame.

|  | Element ID | Length | Fast Channel Scan Request |
| --- | --- | --- | --- |
| Octets | 1 | 1 | 1 |

Figure 8-ai1 – Fast Channel Scan Request

Fast Channel Scan Request subfield indicates whether the STA with dot11FILSActivated equal to true scans the next channel or not when the PHY-RxStart.indication primitive has not been received befor ProbeTimer reaches MinChannelTime. A value 1 indicates that the STA performs the fast channel scan procedure.

If the PHY-CCA.indication (busy) primitive has been detected, but no PHY-RxStart.indication primitive has been received, STAs with dot11FastChannelScanRequest equal to true set NAV to 0 and scan the next channel when the ProbeTimer reaches MinChannelTime. STAs with dot11FastChannelScanRequest equal to false monitors the channel until the Probe Timer reaches the MaxChannelTime if the PHY-CCA.indication (busy) primitive has been detected, but no PHY-RxStart.indication primitive has been received.