IEEE P802.11
Wireless LANs

| Text for Probe Timer setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | Kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | hg.cho@lge.com |

Abstract

The submission provides normative text for Setting of ProbeTimer (11-12-0151-r15).

6.2.11 Setting of ProbeTimer

TGai may define a mechanism for a STA to accelerate the scanning of an empty channel with no AP in operation.

# Introduction

If other BSSs occupy adjacent channels of a scan channel, the transmission from other BSSs may trigger the PHY-CCA.indication (busy) primitive. If there is no BSS occupying the current scan channel, the scan time overhead is increased to MaxChannelTime - MinChannelTime.

Thus, this contribution proposes to scan the next channel for fast channel scan of the STA if a STA does not receive a valid PLCP header during MinChannelTime.

# Conventions

In this contribution, the proposed 802.11ai Specification Document text will be presented as modifications to the TGai draft specification 802.11ai/D05. The following format conventions are used:

- The new added text is marked as blue underline text;

- The deleted text is marked as red strikethrough text;

- The unchanged baseline standard text stays in black text in the context of proposed TGai specification text;

- The editorial instruction is marked as italic text highlighted by Yellow;

10.1.4.3.2 Active scanning procedure

Instructions to Editor:  modify the following text in section 10.1.4.3.2

e) If PHY-CCA.indication (busy) primitive has not been detected before the ProbeTimer reaches Min-

ChannelTime, then go to step f, else while the Probe Timer is less than the MaxChannelTime:

Optionally,  if PHY-CCA.indication (busy) primitive has been detected, but no PHY-RxStart.indication primitive has been received,  then go to step f; else process all the received probe responses, beacon and FD frames when the ProbeTimer reaches MaxChannelTime.

1) Process any received probe responses;

2) Process any received Beacons, measurement pilots and FILS Discovery frames if

dot11FILSActivated is true in the STA; and

3) dot11FILSActivated is true in the STA, ReportingOption is IMMEDIATE, and new AP or new

information of the AP is detected, issue MLME-SCAN.confirm primitive with the ResultCode

equal to IMMEDIATE_SCAN_RESULT and the BSSDescriptionSet containing information

of the detected AP.

4) If dot11FILSActivated is true and the ReportingOption is CHANNEL_SPECIFIC, issue at the

time when the Probe Timer reaches the MaxChannelTime an MLME-SCAN.confirm primitive,

with the ResultCode equal to SUCCESS and the BSSDescriptionSet containing information of

all APs that have been discovered from the scanned channel.

f) set NAV to 0 and scan the next channel.

Motion-1: Move to authorize the Editor to incorporate the text changes proposed in contribution 11-13-0480-02-00ai- normative-text-for-probe-timer-setting to the draft TGai Specification Document.

Yes: ____________;  No: _________________;  Abstain: _____________

[Result of Motion]