IEEE P802.11
Wireless LANs

| GTK Transfer in FILS Association |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-09 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | Allied Telesis R&D Center | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 JAPAN | +81 92 771 7630 | hmorioka@root-hq.com |

Abstract

This document is a proposed TGai draft text about GTK transfer, as a resolution to a comment submitted for IEEE802.11 Comment Collection 8 for IEEE802.11ai Draft 0.5.

# 8. Frame formats

## 8.3 Format of individual frame types

### 8.3.3 Management Frames

### 8.3.3.5 Association Request frame format

Insert the following rows to the contents of Table 8-22:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | FILS Secure Container | The FILS Secure Container element contains encrypted TLVs. |

### 8.3.3.6 Association Response frame format

Insert the following rows to the contents of Table 8-23:

| Order | Information | Notes |
| --- | --- | --- |
| <ANA> | FILS Secure Container | The FILS Secure Container element contains encrypted TLVs. |

## 8.4 Management frame body components

### 8.4.2 Information elements

# Change subclause 8.4.2.186 as following:

### 8.4.2.186 FILS Secure Container element

# Change subclause 8.4.2.186.1 as following:

### 8.4.2.186.1 FILS Secure Container TLVs

FILS Secure Container TLVs are used to carry out various purpose such as IP address assignment and GTK transfer.

Change the caption of Table 8-183ah and insert the following rows to the contents of Table 8-183ah:

Table 8-183ah –FILS Secure Container TLVs

| Type | TLV ID | Length (octets) | Extensible |
| --- | --- | --- | --- |
| Key RSC | <ANA> | 18 | No |
| GTK Transfer | <ANA> | 3-255 | No |

Insert the following two new clauses after 8.4.2.186.1.4:

### 8.4.2.186.1.a Key RSC TLV

Key RSC TLV contains Key RSC of the GTK. The format is shown in Figure 8-a.

|  | TLV ID | Length | Key RSC |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 16 |

Figure 8-a –Key RSC TLV format

The TLV ID field is equal to the Key RSC value in Table 8-183ah (FILS Secure Container TLVs).

The value of the Length field is 16.

The value of Key RSC is the current Key RSC of the GTK.

### 8.4.2.186.1.b GTK Transfer TLV

GTK Transfer TLV contains a GTK. The format is shown in Figure 8-b.

|  | TLV ID | Length | Key ID | GTK |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |

Figure 8-b –GTK Transfer TLV format

The TLV ID field is equal to the GTK Transfer value in Table 8-183ah (FILS Secure Container TLVs).

The value of the Length field is 1 plus GTK length in octets.

The value of Key ID is the Key ID of the GTK.

# 11. Security

## 11.11 Authentication for Fast Initial Link set-up

### 11.11.2 FILS Authentication

Insert the following new clause in 11.11.2:

### 11.11.2.c GTK Transfer

In FILS Authentication, GTK is transferred in Association Response frame from AP to STA. After successful authentication (11.11.2.2), key derivation (11.11.2.3) and key confirmation (11.11.2.4) by the AP, the AP shall construct Key RSC TLV (8.4.2.186.1.a) and GTK Transfer TLV (8.4.2.186.1.b). The AP shall put these TLVs into FILS Secure Container (8.4.2.186) and shall encrypt it as described in 8.4.2.186 and 11.11.2.6.

STA shall decrypt FILS Secure Container as described in 8.4.2.186 and 11.11.2.7. The STA install GTK and set key RSC.

GTK rekeying shall be performed as described in 11.6.7 (Group Key Handshake).