IEEE P802.11
Wireless LANs

| Draft 4.0 MIB Revision |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics |  | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission proposes MIB revision of Draft 4.0.

Editing Instruction: TGaf Editor revises Annex C as the following:

dot11smt OBJECT IDENTIFIER ::= { ieee802dot11 1 }

…

--  dot11APCTable 		::= { dot11smt 30 }

--  dot11STALCITable 		::= { dot11smt 32 }

dot11RLSSActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by the SME or an external management entity.

Changes take effect as soon as possible in the device.

This attribute, when true, indicates that the RLQP capability is activated. The capability is disabled,	otherwise. ”"

DEFVAL { false }

::= { dot11StationConfigEntry 149 }

dot11WSSTAType OBJECT-TYPE

SYNTAX INTEGER { GDDAP(1), GDDnonAPSTA(2), GDDFixedSTA(3) gddap(1), gddnonapsta(2), gddfixedsta(3), reserved(4) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME when the device is initialized for operation in a band that requires Geolocation Database Control.

This attribute specifies the type of station mode for the purpose of channel availability query requirements.

This value is used to set the desired channel availability query request parameters in using GAS frames carrying channel availability query element of RLQP.

When set to GDDAP gddap, the station indicates that it operates as a personal/portable AP STA after channel availability query.

When set to GDDnonAPSTA gddnonapsta, the station indicates that it operates as a personal/portable non-AP station after channel availability query.

When set to GDDFixedSTA gddfixedsta, the station indicates that it operates as a fixed STA after channel availability query.

When set to any other values, its use is reserved, and remains unspecified."

::= { dot11StationConfigEntry 151}

dot11GeolocationCapabilityActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the has obtained its device location information in geospatial coordinates for its position using its geolocation capability, as required for its device class, otherwise this is set to false. ”"

DEFVAL { false }

::= { dot11StationConfigEntry 152 }

dot11GDBAccessActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the SME.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the STA has obtained permissible frequencies and other operating parameters valid for its location from an authorized geolocation database, otherwise this is set to false."

DEFVAL { FALSE false }

::= { dot11StationConfigEntry 154 }

dot11STALCITable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11STALCIEntry

MAX-ACCESS read-write not-accessible

STATUS current

DESCRIPTION

"This table represents the geolocation of the STA as specified in clause 10.41.2."

::= { dot11smt 33 dot11smt 32 }

dot11STALCIEntry OBJECT-TYPE

SYNTAX Dot11STALCIEntry

MAX-ACCESS read-write not-accessible

STATUS current

DESCRIPTION

"STA's location information in Geospatial coordinates"

INDEX { dot11STALCIDIndex dot11STALCIIndex }

::= { dot11STALCITable 1 }

After the end of the dot11DMGCountersComplianceGroup dot11DBandCountersComplianceGroup, insert the dot11TVWSComplianceGroup as defined below:

dot11TVWSComplianceGroup OBJECT-GROUP

OBJECTS { dot11ChannelScheduleManagementActivated,

dot11ContactVerificationSignalActivated,

dot11NetworkChannelControlActivated,

dot11WhiteSpaceMapActivated, dot11GDDActivated,

dot11GDBAccessActivated

}

STATUS current

DESCRIPTION

“"Attributes that configure the TVWS Group for IEEE 802.11. ”"

::= { dot11Groups 68 dot11Groups 70 }

Within dot11Compliance, after the end of the dot11DMGCountersComplianceGroup dot11DBandCountersComplianceGroup, insert the following:

GROUP dot11SMTbase13

DESCRIPTION

“At least the dot11WirelessManagementImplemented object is required from dot11SMTbase13.”

Within dot11Compliance, change the last entry in dot 11 Compliances 1 as follows:

--	dot11WNMCompliance,

--	dot11DBandComplianceGroup,

--	dot11TVWSComplianceGroup}

Aafter the end of the dot11Compliances 9 14, insert the following:dot11TVWSCompliance MODULE-COMPLIANCE

-- ********************************************************************

-- * Compliance Statements - TVWS

-- ********************************************************************

dot11TVWSCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

“The compliance statement for SNMPv2 entities that implement the IEEE 802.11 MIB for TVWS operation.”

MODULE -- this module

MANDATORY-GROUPS {

dot11TVWSComplianceGroup }

::= { dot11Compliances 15 }