IEEE P802.11
Wireless LANs

| IP address Setup Proposal Text |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Date: 2013-05-14 |  |  |  |  |  |
| Author(s): |  |  |  |  |  |
| Name | Affiliation | Address |  | Phone | email |
| George Cherian | Qualcomm | 5775 Morehouse Dr, San Diego, CA, USA | +1 858 651 6645 |  | gcherian@qti.qualcomm.com |

Abstract

This document addresses comments for section 8.4.2.185.

8.4.2.185 FILS Indication

The FILS Indication element information related to FILS Capabilities of the AP.

|  | Element ID | Length | FILS Information | Domain information[CID 1295] |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | Variable |

Figure 8-401df -  FILS Indication

CID 1216, 1210, 1431

The definitions of FILS Information field is as follows:

|  | FILS Security Type | IP Address Type | Number of Domains | IP-Address Assignment Method | Subnet-ID Token present | Reserved |
| --- | --- | --- | --- | --- | --- | --- |
| Bits: | B0 B1 | B2-B4 | B5-B7 | B8-B9 | B10 | B11 B15 |

Figure 8-401dg —FILS Information field definition [CID 1014, 1184]

CID 1022, 1315

Table 8-183af – FILS Indication Element Field Settings

| FILS Security type (B0 B1) | IP Address type (B2 –B4) | Number of Domains indication (B5-B7) |
| --- | --- | --- |
| ‘00’: EAP-RP with no PFS | Reserved | 000 to 111 |
| ‘01’: EAP-RP with PFS | Reserved | 000 to 111 |
| ‘10’: Non-TTP with PFS | See Table 8-183ag | Reserved |
| ‘11’: Reserved | Reserved | Reserved |

Table 8-183af shows the possible field values for the FILS security indication element.

When the FILS Security type is EAP-RP (with or without PFS), information on IP address type is carried in the domain information fields.  With Non-TTP type security, the IP address type information is carried in B2 to B4.

When FILS Security type is EAP-RP (with or without PFS), if B5-B7 is between 0 and 6, it indicates between 1 and 7 domains available respectively.    A 4 [CID 1215, 1296] octet information field per domain is present in the FILS indications when B4-B6 takes values from 0 to 6.

If B5-B7 indicates a value of 7, it indicates that more than 7 domains are available.  Per domain information is absent in FILS indication Element if B5-B7 indicate a value of 7.   The STA shall use ANQP to obtain domain information if B5-B7 is set to 7.

[CID 1317, 1214]

Table 8.4.2.186-ai1 – IP-Address Assignment Method

| Bit values | IP Address Assignment Method supported by the AP |
| --- | --- |
| ‘00’ | IP Address assignment during Association is not supported by the AP |
| ‘01’ | STA may use FILS HLP wrapped data to request IP address during Association |
| ‘10’ | STA may use FILS IP Address Request TLV to request IP address during Association |
| ‘11’ | STA may use either FILS HLP wrapped data or FILS IP Address Request TLV to request IP address during Association |

The domain information field is a 4 [CID 1215, 1296]octet field formatted as shown in Figure 8-401dg. The domain information field is only present when EAP-RP is used, i.e., when FILS security type is set to “00” or “01”.

[CID 1430: Moved the table 8-401ag to follow the order of description of the fields]

[CID 1022]

|  | Hashed Domain Name | IP Address Type See Table 8-183ag | Reserved | Subnet ID Token |
| --- | --- | --- | --- | --- |
| Bits: | B0-B15 | B16- B18 | B19-B23 | B24-B31 |

Figure 8-401dg: Domain Information Field

[CID 1430] The hashed domain name is computed from the Domain Name that is compliant with the “Preferred Name Syntax” as defined in IETF RFC 1035 (same as the domain name used in clause 8.4.4.15 Domain Name ANQP element).  The exact computation method for the hashed domain name is given in clause 10.43.11.[CID 1429, 1316, 1187]

[CID 1430: Moved the table 8-183ag to follow the order of the fields defined in 8-401dg]

Table 8-183ag – IP Address Type

| Bit values | IP Address type |
| --- | --- |
| ‘000’ | IPv4 only |
| ‘001’ | IPv6 only |
| ‘010’ | IPv4 & IPv6 |
| ‘011-111’ | Reserved |

[CID 1432] The Subnet-ID Token is an identifier derived from the subnet using a hash of the subnet or other means that is out of scope of this specification. The Subnet-ID Token is used by the STA to select an AP that is connected to the same IP domain as the current AP.

Table 8-183ah – Subnet-ID Token present

| Bit values | Subnet-ID Token present |
| --- | --- |
| ‘0’ | A subnet-ID token corresponding to the IP subnet to which the domain is connected is not present in the Domain information field |
| ‘1’ | A subnet-ID token corresponding to the IP subnet to which the domain is connected is present in the Domain information field |