IEEE P802.11
Wireless LANs

| Text proposal for LB 193 TFS related comments resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-05-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Qi Wang | Broadcom Corporation | 190 Mathilda Place, Sunnyvale, CA 94086 | +1-408-922-8798 | Qi.wang@broadcom.com |

Abstract

CID-50, 309. 311, 312, 313, 316, 1152, 1158, 1159, 1160, 1161, 1162, 1163, 1402 and 1448 from LB 193mc are pertinent to the TFS feature as defined in IEEE P802.11-REVmc_D1.4 [1]. This submission contains a proposal to resolve these CIDs.

Introduction

CID-50, 309. 311, 312, 313, 316, 1152, 1158, 1159, 1160, 1161, 1162, 1163, 1402 and 1448 from LB193mc are pertinent to the TFS feature as defined in IEEE P802.11-REVmc_D1.4 [1]. This submission contains a proposal to resolve these CIDs.

The proposed modifications are in reference to the text in IEEE P802.11REVmc/D1.4 and are indicated by the change marks as follows:

8.6.14 WNM Action details

8.6.14.1 WNM Action fields

Several Action frame formats are defined for wireless network management (WNM) purposes. An Action

field, in the octet field immediately after the Category field, differentiates the formats. The Action field

values associated with each frame format are defined in Table 8-299 (WNM Action field values).

Replace the last row of “Table 8-299 – WNM Action field values” from:

| 28-255 | Reserved |
| --- | --- |

to the following:

| 28 | WNM-Notify Response |
| --- | --- |
| 29-255 | Reserved |

8.4.2.79 TFS Request element

The TFS Request element defines information about the traffic filters that are enabled at the AP for the requesting non-AP STA. The format of the TFS Request element is defined in Figure 8-367 (TFS Request element format(#240)).

|  |  |  |  | One or more TFS Request subelements |
| --- | --- | --- | --- | --- |
| Element ID | Length | TFS ID | TFS Action Code | TFS Request Subelements (#1447) |
| Octets: 1 | 1 | 1 | Variable |  |

Figure 8-367—TFS Request element format (#240)

The Element ID and Length fields are defined in 8.4.2.1 (General).(#139)(#1429)

The TFS ID field is assigned by the STA and provides a unique identifier for the Traffic Filter Set specified in the TFS (#1447) Request Subelements field(Ed).

The TFS Action Code field defines the actions taken at the AP when a frame matches a Traffic Filter Set. The functions of the bits in this field are shown in Table 8-175 (TFS Action Code field values).

The TFS Request Subelements field contains one or more TFS Request subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length filed, and a variable-length Data field, as shown in Figure 8-367, 8-368 and 8-xxx1 .  The Subelement ID field values for the defined subelements are shown in Table 8-176.  Each TFS Request subelement specifies one Traffic Filter.  Using multiple TFS Request subelements in a TFS Request element is the equivalent to a logical AND operation on the match conditions of each TFS Request subelement.

In a TFS Response element, the number of the Response sub-elements is the same as the number of the TFS Request subelements in the corresponding TFS Request element, where the TFS Response subelements appear in the same order as the corresponding TFS Request subelements in the corresponding TFS Request frame.

Table 8-175 – TFS Action Code field values

| Bit(s) | Name | Notes |
| --- | --- | --- |
| 0 | Delete After Match | Setting this field to 1for any Traffic Filter Set indicates all Traffic Filter Sets established at the AP for the non-AP STA are deleted when a frame matches any of the Traffic Filter Sets established for the non-AP STA... A value of 0 for this field indicates no deletion of the Traffic Filter Set upon a match. |
| 1 | Notify | Setting this field to 1 indicates the STA is to be sent a TFS Notify frame upon the first frame matching to the Traffic Filter Set or the first frame match after the AP receives a Notify Response frame containing the corresponding TFS ID. Setting this field to 0 indicates the AP does not send TFS Notify frame to the requesting STA. |
| 2-7 | Reserved | All other bits are reserved (#366) |

Table 8-176 – TFS Request subelements (#1429)

| Subelement ID (#1449) | Subelement name |
| --- | --- |
| 1 | TFS subelement |
| 221 | Vendor Specific subelement |
| 0, 2 to 220, 222 to 255 | Reserved |

The format of the TFS subelement is shown in Figure 8-368 (TFS (#1431) subelement format (#240)).

|  |  | One or more TCLAS Elements |  |
| --- | --- | --- | --- |
| Subelement ID | Length | TCLAS Elements | TCLAS Processing Element (optional) |
| Octets: 1 | 1 | variable | 0 or 3 |

Figure 8-368—TFS (#1431) subelement format (#240)

The Subelement ID field uniquely identifies this subelement to be the TFS subelement.

The Length field is defined in 8.4.3 (Information Subelements).(#1429)

The TCLAS Elements field contains one or more TCLAS elements, each  specifying  a  set of filtering parameters as defined in 8.4.2.30 (TCLAS element). The number of TCLAS elements is limited and the total size of the TFS Request element is less than 255 octets.

The TCLAS Processing Element field is optionally present and defines how multiple TCLAS elements are processed as defined in 8.4.2.32 (TCLAS Processing element).

| Subelement ID | Length | Vender Specific |
| --- | --- | --- |
| Octets: 1 | 1 | Variable, |

Figure 8-xxx1—TFS Request Vendor Specific Subelement format

The Subelement ID field uniquely identifies this subelement to be the TFS Request subelement.

The Length field is defined in 8.4.3 (Information Subelements).

The Vendor Specific field contains the vendor specific information.

The TFS Request element is included in TFS Request frames, as described in 8.6.14.15 (TFS Request frame format), and WNM-Sleep Mode Request frames, as described in 8.6.14.18 (WNM-Sleep Mode Request frame format). The use of the TFS Request element and frames is described in 10.24.12 (TFS procedures).

8.4.2.80 TFS Response element

The TFS Response element defines information about the status of the requested filtering parameters. The format of the TFS Response element is defined in Figure 8-369 (TFS Response element format(#240)).

| Octets |  |  |
| --- | --- | --- |

|  |  |  | One or more TFS Response subelements |
| --- | --- | --- | --- |
| Element ID | Length | TFS ID | TFS Response Subelements |
| Octets: 1 | 1 | 1 | Variable, |

Figure 8-369 – TFS Response element format

The Element ID and Length fields are defined in 8.4.2.1 (General).(#139)(#1429)

The TFS ID field indicates the unique ID for the TFS Traffic Filter Set.

The  TFS Response Subelements field contains one or more subelements, , each consisting of a 1-octet Subelement ID field, a 1-octet Length filed, and a variable-length Data field, as shown in Figure 8-369, 8-370 and 8-xxx2. The Subelement ID field values for the defined subelements are shown in Table 8-177.

In a TFS Response element, the number of the Response sub-elements is the same as the number of the TFS Request subelements in the corresponding TFS Request element, where the TFS Response subelements appear in the same order as the corresponding TFS Request subelements in the corresponding TFS Request frame.

Table 8-177 –  TFS Response subelements (#1429)

| Subelement ID (#1449) | Subelement name |
| --- | --- |
| 1 | TFS Status subelement |
| 221 | Vendor Specific subelement |
| 0, 2 to 220, 222 to 255 | Reserved |

The TFS  Response Subelement field contains the information as defined in Figure 8-370 (TFS Status (#1431) subelement format).

| Octets |  |  |  |
| --- | --- | --- | --- |

| Subelement ID | Length | TFS Response Status | TFS Subelements |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | Variable, optionally present |

Figure 8-370—TFS Status Subelement format

When the value of  the Subelement ID field is 1, the TFS Response Subelement is a TFS Status subelement.

The Length field is defined in 8.4.3 (Information Subelements).(#1429)

The TFS Response Status field indicates the status returned by the AP responding to the STA’s requested Traffic Filter, as indicated in Table 8-173 (FMS Element Status and TFS Response Status definition).

If present, the TFS Subelements field contains the alternative filtering parameters preferred by the AP.

| Subelement ID | Length | Vender Specific |
| --- | --- | --- |
| Octets: 1 | 1 | Variable, |

Figure 8-xxx2—TFS Response Vendor Specific Subelement format

The Subelement ID field uniquely identifies this subelement to be the TFS Response subelement.

The Length field is defined in 8.4.3 (Information Subelements).

The Vendor Specific field contains the vendor specific information.

The TFS Response element is included in TFS Response frames, as described in 8.6.14.16 (TFS Response frame format), and WNM-Sleep Mode Response frames, as described in 8.6.14.19 (WNM-Sleep Mode Response frame format). The use of the TFS Response element and frames is described in 10.24.12 (TFS procedures).

8.5.14.15 TFS Request frame format

The TFS Request frame is sent by a non-AP STA to the AP to request the specified traffic filtering. The

format of TFS Request frame is defined in Figure 8-484.

|  |  |  | One or more TFS Request elements |
| --- | --- | --- | --- |
| Category | Action | Dialog Token | TFS Request Elements |
| Octets: 1 | 1 | 1 | Variable |

Figure 8-601—TFS Request frame format (#240)

The Category field is the value indicating the WNM category, as specified in Table 8-43 (Category values)  in 8.4.1.11 (Action field).

The Action field is the value indicating TFS Request frame, as specified in Table 8-299 (WNM Action field value) in 8.6.14.1 (WNM Action fields).

The Dialog Token field is a value chosen by the STA sending the TFS Request frame to identify the request/response transaction.

The TFS Request Elements field contains one or more TFS Request elements to specify  one or more Traffic Filter Sets that are requested by the non-AP STA, as defined in 8.4.2.79 (TFS Request element), or zero TFS Request elements to cancel all of the existing Traffic Filter Sets (see 10.24.12.2 (TFS non-AP STA operation)) (#313).

8.5.14.16 TFS Response frame format

The TFS Response frame is sent by an AP in response to a TFS Request frame. The format of the TFS Response frame is defined in Figure 8-602 (TFS Response frame format (#240)).

|  |  |  | One or more TFS TFS Response elements |
| --- | --- | --- | --- |
| Category | Action | Dialog Token | TFS Response Elements |
| Octets: 1 | 1 | 1 | Variable |

Figure 8-602—TFS Response frame format (#240)

The Category field is the value indicating the WNM category, as specified in Table 8-43 (Category value) in 8.4.1.11 (Action field).

The Action field is the value indicating TFS Response frame, as specified in Table 8-299 (WNM Action field value) in 8.6.14.1 (WNM Action field).

The Dialog Token field is the value in the corresponding TFS Request frame.

The TFS Response Elements field contains one or more TFS Response elements to indicate the status of the filtering parameters that the AP is requested to support, as defined in 8.4.2.80 (TFS Response element). The number of the TFS Response elements in a TFS Response frame is the same as the number of the TFS Request elements in the corresponding TFS Request frame, where the TFS Response elements appear in the same order as the corresponding TFS Request elements in the corresponding TFS Request frame.

8.5.14.17 TFS Notify frame format

The TFS Notify frame is sent by an AP to a STA when a frame matching a traffic filter is encountered. The format of the TFS Notify frame is defined in Figure 8-603 (TFS Notify frame format (#240)).

|  |  |  | One or more TFS IDs |
| --- | --- | --- | --- |
| Category | Action | Number of TFS IDs | TFS ID List |
| Octets: 1 | 1 | 1 | Variable |

Figure 8-603—TFS Notify frame format (#240)

The Category field is the value indicating the WNM category, as specified in Table 8-43 (Category Value) in 8.4.1.11 (Action field).

The Action field is the value indicating TFS Notify frame, as specified in Table 8-299 (WNM Action field values) in 8.6.14.1 (WNM Action field).

The Number of TFS IDs field indicates the number of 1-octet TFS IDs present in the TFS ID List field.

The TFS ID field indicates the traffic filter set containing the matched TCLAS element.

8.5.14.xx TFS Notify Response frame format

A TFS Notify Response frame is transmitted by a non-AP STA to an AP to request the AP to resume generation of the TFS Notify frame when a frame matches any Traffic Filter Set identified by the TFS ID(s) included in the TFS Notify Response frame.

| Category | Action | Number of TFS IDs | TFS ID List |
| --- | --- | --- | --- |
| Octets: 1 | 1 | 1 | Variable |

Figure 8-xxx3—TFS Notify Response frame body format

The Category field is the value indicating the WNM category, as specified in Table 8-43 (Category Value) in 8.4.1.11 (Action field).

The Action field is the value indicating TFS Notify Response frame, as specified in Table 8-299 (WNM Action field values) in 8.6.14.1 (WNM Action field).

The Number of TFS IDs field indicates the number of 1-octet TFS IDs present in the TFS ID List field.

The TFS ID List field indicates the identifiers of the Traffic Filter Sets.

10.24.12 TFS procedures

10.24.12.1 TFS capability

Implementation of the TFS capability is optional for a WNM STA. A STA that implements TFS has dot11(#1676)TFSImplemented set to true. When dot11(#1676)TFSImplemented is true, dot11WirelessManagementImplemented shall be true. A STA that has a value of true for dot11(#1676)TFSActivated is defined as a STA that supports TFS. A STA for which dot11(#1676)TFSActivated is true shall set the TFS field of the Extended Capabilities element to 1.

A STA with a value of true for dot11(#1676)TFSActivated may send a TFS Request, TFS Response, TFS Notify frame, or TFS Notify Response frame  to a STA within the same infrastructure BSS whose last received Extended Capabilities element contained a value of 1 for the TFS bit in the Capabilities field. The Traffic Filtering service is not supported in an IBSS.

The traffic filtering parameters are established using a TFS Request frame transmitted from a non-AP STA to an AP STA, and a TFS Response frame transmitted from the AP STA to the non-AP STA.

A TFS Request frame contains one or more TFS Request elements.  Using multiple TFS Request elements in a TFS Request frame is the equivalent to a logical OR operation on the match conditions of each TFS Request element.

A TFS Request element contains the filtering parameters for one Traffic Filter Set.  One TFS Request element contains one or more TFS Request subelements.  A TFS Request subelement might be a TFS subelement, in which case it contains the filtering parameters for one Traffic Filter.  Using multiple TFS subelements in a TFS Request element is the equivalent to a logical AND operation on the match conditions of each TFS subelement.

A TFS subelement contains one or more TCLAS elements and an optional TCLAS Processing element that defines the relationship among multiple TCLAS elements within the TFS subelement. The TCLAS Processing element shall be present if more than one TCLAS element is included in the TFS subelement.

A TFS Response frame contains the status of the filter parameters requested in the corresponding TFS Request frame.

An AP may propose alternative filtering parameters by returning a TFS subelement in the corresponding TFS Response subelement. (See 10.24.12.2 and 10.24.12.3)

When an AP transmits a TFS Response frame to a STA in which the Status for all Traffic Filter requests is “Accept”, it shall enable TFS for the STA.  The previously established filter parameters, if any, remain in effect when the AP transmits a TFS Response frame with any other Status.

A frame match occurs when a frame matches the filtering parameters for a TFS Traffic Set.

NOTE-- The frame match can occur when the matched frames are either individually addressed or group addressed.

When a traffic filter for group addressed frames is enabled at the AP, the group addressed frames are still delivered, without regard to the frames matching the traffic filter, since other associated STAs may also receive these frames. Because a STA using TFS can be in power save mode for an extended period of time, group addressed frames that match the traffic filter might be delivered before the STA is aware that the traffic filter has been matched. It is likely (but not guaranteed) that the STA does not receive those group addressed frames matching the traffic filter at the scheduled group addressed delivery time. To prevent this from happening, the STA can request a notification frame be sent when requesting the establishment of the traffic filter. If negotiated with the AP, a frame match is indicated to the non-AP STA via a notification frame.

10.24.12.2 TFS non-AP STA operation

To use the TFS, the non-AP STA’s SME that supports TFS shall issue an MLME-TFS.request primitive to send a TFS Request frame. The MLME-TFS.request primitive shall include a valid TFSRequest parameter as defined in the TFS Request elements.

The receipt of an MLME-TSF.confirm primitive with a valid TFSResponse parameter indicates to the STA’s SME that the AP has processed the corresponding TFS request. The content of the TFSResponse parameter provides the status of each of the TFS Request(#313) elements processed by the AP. A TFSResponse parameter optionally contains modified filtering parameters for the corresponding TFS request.

A TFS Response frame in which the Status for all Traffic Filter requests is “Accept” indicates to the non-AP STA that the filtering operation at the AP has started.  Otherwise, the STA might use the information in the TFS Response frame to form a new TFS request.

In order to request the AP to delete a Traffic Filter Set (identified by a unique TFS ID) when a frame matches to the Traffic Filter Set, a non-AP STA shall set the Delete After Match subfield of the TFS Action Code field of the TFS Request element in the TFS Request frame to 1.

In order to request a TFS Notify frame to be sent by the AP upon a frame match, a non-AP STA shall set the Notify subfield of the TFS Action Code field of the TFS Request element in the TFS Request frame to 1.

After receiving a TFS Notify frame, in order to request the AP to resume generation of the TFS Notify frame upon a future frame match, a non-AP STA shall transmit to the AP a TFS Notify Response frame containing one or more TFS IDs.

The non-AP STA may indicate that it is no longer using a particular TFS Request(#313) element by transmitting a TFS Request frame without that TFS Request(#313) element. The AP shall send a TFS Response frame with the corresponding Status field value set to Accept, upon receipt of the TFS Request frame.

The non-AP STA may choose to terminate use of the TFS service by sending a TFS Request frame with no

TFS Request(#313) elements in the request thereby canceling all traffic filters at the AP.

10.24.12.3 TFS AP operation

When an AP’s SME receives an MLME-TFS.indication primitive with a valid TFSRequest parameter, it shall issue an MLME-TFS.response primitive with a TFSResponse parameter indicating the status of the associated request. When the AP accepts any filtering parameter for a requesting STA, the AP shall establish a traffic filter that matches individually addressed (#99)EAPOL-Key frames addressed to the requesting STA, with bits 0 and 1 of the TFS Action Code field set to 0.

When an AP’s SME receives an MLME-TFS.indication primitive with a valid TFSRequest parameter having a requested TCLAS-based classifier which it is unable to provide, the SME shall issue an MLME-TFS.response primitive indicating the status of the corresponding request and may include a TFSResponse parameter having a suggested modified TCLAS-based classifier.

When TFS is enabled for an associated STA, the AP shall discard all individually addressed frames destined for the non-AP STA until a frame is found that matches one or more Traffic Filter Sets established by the STA. When a frame is found that matches one or more of the Traffic Filter Sets  established by the STA (a matching frame), the AP shall perform the following actions.

If a non-AP STA sets both  the Delete After Match subfield  and Notify subfield of the TFS Action Code field of the TFS Request element in the TFS Request frame to 1,  the AP shall generate a single individually addressed Notify frame for the frames matching to the Traffic Filter set identify by the corresponding TFS ID.  Subsequently, the AP shall delete all Traffic Filter Sets established at the AP for the non-AP STA.

If a non-AP STA sets the Delete After Match subfield of the TFS Action Code field of the TFS Request element in the TFS Request frame to 0, and sets the Notify subfield of the TFS Action Code field of the TFS Request element in the TFS Request frame to 1, the AP shall generate a single individually address TFS Notify frame for all frames matching to the Traffic Filter Set identified by the corresponding TFS ID.  All Traffic Filter Sets established at the AP for the non-AP STA remain to be effective upon a match.   If the AP receives a TFS Notify Response frame from the corresponding non-AP STA, the AP resumes generation of a single individually addressed TFS Notify frame for the next frame matching to the Traffic Filter Sets identified by the TFS IDs included in the TFS Notify frame.  Otherwise, the AP shall not send additional TFS Notify frames upon frame matches.

A TFS Notify frame is delivered per the rules for the transmission for individually addressed frames.

NOTE—Due to the operation of group addressed frame delivery, a group addressed frame that matches a traffic filter might result in the STA receiving indication of the group addressed frame either before or after the group addressed frame is transmitted by the AP, if the TFS Notify frame is queued in the STA’s power save queue. This might result in the STA receiving the group addressed frame in some cases and not receiving it in other cases.

Upon receiving an MLME-TFS.indication primitive, the AP’s SME shall disregard any previous MLMETFS. indication primitive received from the same STA.

The AP shall terminate any TFS operation for a STA when no traffic filters remain for a STA or if the AP’s SME receives an MLME-TFS.indication primitive with a null TFSRequest.

References:

1. IEEE P802.11REVmc/D1.4, Draft standard for information technology – telecommunications and information exchange between systems – local and metropolitan area networks – specific requirements – Part 11: Wireless LAN medium access control (MAC) and physical layer (PHY) specifications