IEEE P802.11
Wireless LANs

| 802.11 REVmc Some More LB193 Resolutions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-06-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission proposes resolutions to the following comments received from the initial working group ballot (LB193):

R0 contains a start on unassigned GEN comments, less those where a separate submission is necessary.

R1 is the completion of the work started in R0.

Resolutions are proposed to the following comments:

(those with a discuss/tbd status are highlighted)

- 1603, 1589, 1465, 1428, 1427, 1089, 1649, 1471, 1621, 1647, 1626, 1633, 1637, 1011, 1673, 1121, 1122, 1179, 1123, 1595, 1193, 1182, 1013, 1192, 1563, 1412, 1014, 1228, 1018, 1019, 1239, 1240, 1244, 1248, 1605, 1513, 1512, 1404, 1024, 1267, 1642, 1659, 1643, 1702, 1445, 1539, 1599, 1004, 1648, 1110

R2 – reviewed at 2013-06-07 telecon.

# Comment Resolutions

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- |
| 1603 |  |  | What does "PHYCCA. indication primitive of class BUSY" mean | Change to refer to argument BUSY | GEN |

Context: 1626.15:

A busy channel shall be indicated by a PHY-CCA.indication primitive of class BUSY. A clear channel shall be indicated by a PHY-CCA.indication primitive of class IDLE.

This doesn’t match the terminology of  433.12,  which uses a STATE parameter.

There are various ways to resolve this.  Proposed is one such:

Proposed Resolution:

Revised.

At the cited location replace “PHY-CCA.indication primitive of class BUSY” with “PHY-CCA.indication(BUSY)”

and on the following line replace

“PHY-CCA.indication primitive of class IDLE” with “PHY-CCA.indication(IDLE)”

Make matching change at 1657.13 and 1708.47.

| 1589 |  |  | Kill aTxRampOffTime (not actually used anywhere) | As it says | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree with the sentiment.   There are 7 references

aRxPHYDelay, aRxTxSwitchTime, aTxRampOnTime, aTxRampOffTime, aAirPropagationTime, aMACProcessingDelay, aPr that the PHY takes to turn the Transmitter on. aTxRampOffTime integer The nominal time (in microseconds) that t ion dependent, see 9.3.7 (DCF timing relations). aTxRampOffTime Implementation dependent, see 9.3.7 (DCF timing r ion dependent, see 9.3.7 (DCF timing relations). aTxRampOffTime Implementation dependent, see 9.3.7 (DCF timing r ion dependent, see 9.3.7 (DCF timing relations). aTxRampOffTime Implementation dependent, see 9.3.7 (DCF timing r ion dependent, see 9.3.7 (DCF timing relations). aTxRampOffTime Implementation dependent, see 9.3.7 (DCF timing r ion dependent, see 9.3.7 (DCF timing relations). aTxRampOffTime Implementation dependent, see 9.3.7 (DCF timing r

The first two define it in the PHY characteristics primitive.  The Latter exist in PHYs describing the value as implementation dependent.  There is no normative behaviour that references this variable.

Proposed resolution:

Revised.   At 416.18,  delete the aTxRampOffTime parameter and any references to it in this subclause.

at 1618.08, 1644.44, 1701.27, 1714.34, 1809.53 delete the table row that includes this attribute.

| 1465 |  |  | Has anyone ever used/does anyone still use StrictlyOrdered? | Deprecate StrictlyOrdered | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The strictly ordered service class was added as a sop to a single sponsor ballot commenter during the balloting of the first 802.11 standard.  Nobody ever implemented it.

The Order field was re-purposed by 802.11n to carry an indication of the HT control field.

Agree with the comment.  REVmc used language similar to that proposed below to mark features as obsolete.

Proposed changes:  at 102.08

5.1.3 MSDU ordering The services provided by the MAC sublayer permit, and may in certaincases require, the reordering of MSDUs. In a non-QoS STA, the MAC does not intentionally reorder MSDUs except as may be necessary to improve the likelihood of successful delivery based on the current operational (“power management,” FMS, DMS) mode of the designated recipient STA(s). The sole effect of this reordering (if any), for the set of MSDUs received at the MAC service interface of any single STA, is a change in the delivery order of group addressed MSDUs, relative to individually addressed MSDUs, originating from a single source STA address. <Note insertion of new para> If a higher layer protocol using the data service cannot tolerate this possible reordering, the optional StrictlyOrdered service class might be used. MSDUstransferred between any pair of STAs using the StrictlyOrdered service class are not subject to the relative reordering that is possible when the ReorderableGroupAddressed service class is used. However, the desire to receive MSDUs sent using the StrictlyOrdered service class at a STA precludes simultaneous use of the MAC power management facilities at that STA. Note that the use of the StrictlyOrdered service class is obsolete and the StrictlyOrdered service class might be removed in a future revision of the standard.

At 1835.50:

| PC8 | MAC data service | 9.2.8 (MAC data service), 9.8 (MSDU transmission restrictions), Annex J | M | Yes  No  |
| --- | --- | --- | --- | --- |
| PC8.1 | ReorderableGroupAddressed service class | 9.8 (MSDU transmission restrictions) | M | Yes  No  |
| PC8.2 | StrictlyOrdered service class Note that the use of the StrictlyOrdered service class is obsolete and the StrictlyOrdered service class might be removed in a future revision of the standard. | 9.8 (MSDU transmission restrictions) | O | Yes  No  |

Proposed Resolution:

Revised.  Make changes as shown in <this-document> under CID1465.  These mark the StrictlyOrdered service class as obsolete.

| 1428 |  |  | Some comments made in the technical comment collection on 802.11-2012 were rejected but should not have been | Revisit those comments | GEN |
| --- | --- | --- | --- | --- | --- |
| 1427 |  |  | Some comments made in the technical comment collection on 802.11-2012 were addressed incorrectly or incompletely | Address all the comments correctly and completely | GEN |

Discussion:

The commenter may believe that these comments achieve some useful end.  That is not the case.

A valid comment needs to indicate a specific problem and a specific solution.  These do neither.   They are invalid comments.  Note that this does not prevent the commenter from attempting to seek approval of changes on any topic, including previous rejected comment resolutions from 802.11-2012.

Proposed resolution (to both):

Rejected.  The comment does not indicate a specific issue to resolve or a specific change to be made.

| 1089 |  |  | There is an opportunity to better exploit indoor-only spectrum from a mobile AP. Given that there are APs or non-AP STAs in the vicinity that know whether they are indoor or outdoor, an AP might be able to use indoor spectrum if it can determine that is is "close" to a "known indoor" device. (Disclaimer - This is not an assertion of applicability for any particular regulation, merely an assertion that having this information might be of value). | Provide an element (or extend an existing element) that indicates indoor/outdoor location, if such is known at the STA. The information would be present in beacons and probe responses when known. | GEN |
| --- | --- | --- | --- | --- | --- |

Disclosure:   this comment is the author’s.

Discussion:

dot11Country string mentions indoor/outdoor environments as follows:

dot11CountryString OBJECT-TYPE SYNTAX OCTET STRING (SIZE(3)) MAX-ACCESS read-only STATUS current DESCRIPTION "This is a control variable. It is written by the SME. Changes take effect for the next MLME-START.request primitive. This attribute identifies the country or noncountry entity in which the station is operating. If it is a country, the first two octets of this string is the two character country code as described in document ISO/IEC 3166-1. The third octet is one of the following: 1. an ASCII space character, if the regulations under which the station is operating encompass all environments for the current frequency band in the country, 2. an ASCII 'O' character, if the regulations under which the station is operating are for an Outdoor environment only, or 3. an ASCII 'I' character, if the regulations under which the station is operating are for an Indoor environment only.

I was not aware of this at the time of the comment.

Status TBD:

Proposed Resolution:

TBD – I’m researching whether the existing definition meets my needs.

| 1649 |  |  | What does "monotonically increasing" mean? How does it differ from "increasing"? | Clarify | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.   The commenter does not indicate a specific problem to solve or a specific change to make.

In reply to the commenter, this term is well known.

NIST defines it thus: “A function from a partially ordered domain to a partially ordered range such that x ≤ y implies f(x) ≤ f(y).”

| 1471 |  |  | For a mesh channel switch, 10.9.8.4.3 mandates a probe [sic] delay. Why not for vanilla BSS switches too? | Extend 6.3.3.2.2, 6.3.4.2.2, 6.3.11.2.2 and 10.2.2.2 to say the ProbeDelay is also used when switching to a different channel | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

Making this change for non-MBSS would make existing devices non-compliant.

The benefit of the additional protection is minimal because channel switches are infrequent affairs.

| 1621 |  |  | Kill 11e and the non-HT BA stuff (i.e. just left with HT-delayed and HT-immediate) | As it says | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

The commenter does not indicate a specific problem to be solved or a specific change to make.

| 1647 |  |  | If the aAirPropagationTime is large and the frame is short, then a STA may do CCA before a frame has started to arrive at its receiver | Introduce a minimum frame duration, as in 802.3? | GEN |
| --- | --- | --- | --- | --- | --- |
| 1626 |  |  | Does 802.11 need a minimum frame size (like 802.3) to ensure short frames do not get missed by the ED mechanism? | Consider the earliest/latest CCA detect times | GEN |

Proposed Resolution: (to both)

Rejected.

The STA does not perform CCA or ED at a specific time within the slot.  It is performed continuously during the slot, except for the aRxTxTurnaround time when it transmits in the following slot.   So it doesn’t matter whether a frame is shorter than the slot duration or not because it will still be detected by STAs in the BSS in the slot in which it was transmitted,  provided that aAirPropagationTime is set to a large enough value.

| 1633 |  |  | 13.1 says mesh STAs necessarily have SM enabled, but wording like "MBSS in which the Spectrum Management bit is equal to 1" implies this is not the case | Mandatory for mesh STAs or not? | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree this is an inconsistency.   Should an MBSS require spectrum management?   That seems excessive, for example in an MBSS running across 802.11g.

I have asked Kaz to comment.  He agrees that the requirement to operate spectrum management in 2.4 GHz seems excessive,  but doesn’t have cycles at the moment to provide a detailed resolution.

I propose to make spectrum management optional.  I have reviewed 10.10.3.4 (1171.31), and it appears to me that this correctly supports having dot11SpectrumManagementRequired optional.  There appears to be no assumption that this is mandatory for mesh in the PICS.

However,  the language here: (1167.01)

A mesh shall inform each of the peer mesh STAs that the mesh STA is moving to a new channel while maintaining mesh peerings by advertising the switch using Channel Switch Announcement elements together with Mesh Channel Switch Parameters element in Beacon frames, Probe Response frames, and Channel Switch Announcement frames until the intended channel switch time. The channel switch should be scheduled so that all mesh STAs in the MBSS, including mesh STAs in power save mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch.

Is problematical.  These frames are part of the Spectrum Management feature.

It certainly appears from these that use of these frames is required, even in bands where this feature makes no sense.

Status:  Discuss.

At this point, I don’t have a firm proposal.   I’m happy to hand this off to somebody more expert. (Brian Hart?)

Possible Proposed changes:

Change 1507.04 as follows:

13.1 Mesh STA dependencies When dot11MeshActivated is true, the STA is a mesh STA. When dot11MeshActivated is true, followingMIB attributes shall be set to true. — dot11QosOptionImplemented — dot11ExtendedChannelSwitchActivated When dot11MeshActivated is true, followingMIB attributes shall be set to false. — dot11OCBActivated — dot11FastBSSTransitionActivated

| 1637 |  |  | Might CCA-ED be required on any channel which HT might be used on? | If so, add CCA-ED to clause 20, modelled on clause 18 | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

It is clear that CCA-ED is defined only for 20MHz (and narrower) channel spacing.

However, is there anything to stop an HT AP operating a 20MHz BSS in operating classes 13-15?   If so, it is required to do CCA-ED.   Clause 20.3.20.5.1 references 18.3.10.6 for non-HT PPDUs.

It is questionable whether an energy detect (i.e. a burst of noise) comprises any kind of PPDU.  A burst of noise certainly does not comprise an HT PPDU.

So, it arguable that we are already covered, but it could be made explicit.

Proposed Resolution:

Revised.

Insert a new subclause “20.3.20.5.0a CCA-Energy Detect (CCA-ED)

For improved spectrum sharing, CCA-ED is required in some bands. The behavior class indicating CCA-ED is given in Table D-2 (Behavior limits sets). The operating classes requiring the corresponding CCA-ED behavior class are given in E.1 (Country information and operating classes). An HT STA that is operating within an operating class that requires CCA-ED shall operate with CCA-ED as defined in 18.3.10.6. “

| 1011 | 2.30 | 1.3 | Do we want to add a list item for .11ae? | Add an entry | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.   At 2.30 add:

“—Defines medium access control mechanisms to support the prioritization of management frames.

| 1673 | 2.30 | 1.3 | The last item in the list is redundant with the 8th item | Delete the last item in the list. Delete the Editor's Note that appears in the redline version of 1.0 (but is not present in the non-redline version). | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.  The 8th list item talks about support for QoS generally.  The last list item describes specific support for streaming audio video without degrading data and voice performance.

| 1121 | 6.36 | 3.1 | Term "ad hoc network" is also used as vernacular of mesh network. | Modify the definition as "Often used as a venacular term for an independent basic service set (IBSS) and mesh basic service set (MBSS)". | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

This term wasn’t used in 802.11s.  However, one of the mesh routing algorithms refers to “Ad Hoc On-Demand Distance Vector (AODV) protocol (IETF RFC 3561 [B35])”,  so the commenter does have a point.

Annex Q states:  “Ad hoc mobile STAs operate in IBSS mode”

To change “ad-hoc” to include mesh in the context of 802.11 is probably going to create more confusion than light.  I propose therefore to delete the term, and substitute IBSS where necessary.

Changes:

Delete the definition at 106.36.

ad hoc network:Often used as a venacular term for anindependent basic service set (IBSS).

Change 49.28 as follows:

4.3.2 The independent BSS (IBSS) The IBSS is the most basic type of IEEE Std 802.11 LAN. A minimum IEEE Std 802.11 LAN may consist of only two STAs. Since the BSSs shown in Figure 4-1 (BSSs) are simple and lack other components (contrast this with Figure 4-2 (DSs and APs)), the two can be taken to be representative of two IBSSs. This mode of operation is possible when IEEE Std 802.11 STAs are able to communicate directly. Because this type of IEEE Std 802.11 LAN is often formed without preplanning, for only as long as the LAN is needed.

Change 2561.23 as follows:

Q.2 Terminology An enhanced description of these access entities begins with clarification of several terms. This standard defines an entity called a STA. STAs can operate in different modes. The possible operational modes of a STA are a) Infrastructure mobile STAs b) Independent mobile STAs c) Access control mode STAs d) Mesh STAs The mobile STAs are the STA entities that are ordinarily moving around, but may also be in a fixed location. The mobile adjective prefix often helps in visualizing the type of STA under discussion. Infrastructure mobile STAs operate in infrastructureBSS mode, i.e., they are the users of an AP. Devices that incorporate an infrastructure mobile STA are referred to in this annex by the term mobile unit(MU). An MU device may consist of just a mobile STA implementation, but also likely includes an SME and a client. The exact configuration of the MU is not relevant to the descriptions in this annex. Independent mobile STAs operate in IBSS mode. Independent mobile STAs form autonomous networks that do not require an AP.

Note, remaining instances of “ad hoc” form part of the name of AODV, and cannot be removed.

Proposed Resolution:

Revised.

Make changes as shown in <this-document> under CID 1121.  These remove the definition of the term “ad hoc” and remove its use in the context of IBSS.

| 1122 | 12.17 | 3.1 | The word "FMS Token" is used without definition. | flexible multicast service (FMS) Token: A unique identifier for the FMS Stream Set that is the set of FMS subelements specified in the FMS Request procedure. Its value is assigned by the AP. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

“FMS Token” is the name of a field.   Except in defining the field and at the cited location,  the only reference is to “FMS Token” <some-type-of-thing>.

Rather than introduce a definition for a little used concept,  we can change references to the “FMS Token” used by itself,  with a reference to the field.  There is no need to create definitions for field names in 3.1.

The FMS definitions are in 3.1 – the generic definitions.  However most if not all of these talk about 802.11-specific constructs.  And naming a specific field is the icing on the cake that breaks the camel’s back.  So at least this,  and probably all FMS definitions should be specific to 802.11.

In the definition of the FMS Token field,  we have:

“The FMS Token field contains a unique identifier for the FMS Stream Set that is the set of FMS subelements specified in the request” and “The FMS Token is fixed for the lifetime of the FMS Stream Set.”

The first of these creates no problems.   The last one creates a problem,  because it implies the existence of FMS Token as a “thing” outside the context of an FMS Action frame.  I claim that the first statement is definitive and sufficient, and propose to delete the second.

Changes:

At 12.17:

flexible multicast service (FMS) stream set: A collection of FMS streams identified by the value of the FMS Token field, used during the FMS Request procedure.

Move the four FMS definitions at 12.05-12.19 to subclause 3.2.

At 731.17:

The FMS Token field contains a unique identifier for the FMS Stream Set that is the set of FMS subelements specified in the request. If this is a new request, then the FMS Token field is set to 0. Otherwise, the FMS Token field is set to the value assigned by the AP in the FMS Response element.

Proposed Resolution:

Revised.  Make changes in <this-document> under CID 1122.   These changes change references to an “FMS Token” that is not a field or type of subelement so that they refer to “FMS Token field”,  thereby eliminating the need for an additional definition.

| 1179 | 14.58 | 3.1 | The inclusion of "Syn: frame." in this text is a serious mistake. If the claim of synonymy were accurate, then, per the 2012 IEEE Style Manual page 9, "frame" would need to be replaced with "MPDU" throughout the draft (also other terms, such as "QMF" and "sounding frame", that use "frame" would need to be replaced --for instance, if an NDP sounding frame really is an MPDU, then what is the NDP sounding frame but a PPDU that is inside an MPDU?). Also: the concept of RCPI would be nonsensical if it were applied only to MPDUs. And, after all these changes are made, then "Syn: frame." would still need to be deleted because of the Style Manual discouragement of using two terms that mean the same thing. In truth, however, "frame" actually is a more general term than "MPDU", and the two concepts should not be confused. | Delete "Syn: frame.". (And please discourage contributors from replacing "frame" with "packet" in PHY clauses -- "packet" is a far more confused term than "frame" -- for instance, NDPs are PPDUs, but is the "packet" in "NDP" the same concept as "packet" in "IPN", "PER", "Ethernet packet" and interworking with external network's "layer 3 end to end packet marking practice"? "Packet" is best kept with layer 3 and above types of frames -- up there with messages, transactions, UDP and similar nebulous oddities.) | GEN |
| --- | --- | --- | --- | --- | --- |
| 1123 | 14.59 | 3.1 | The word "frame" is used not only "MAC frame" but also "PHY frame"/"PPDU frame" in this document. | Change "Syn: frame" to "Syn: MAC frame". | GEN |

| CID | Page | Clause | Comment | Proposed Change | Previous proposed resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 1595 |  |  | The PHYs sometimes use the term "frame", apparently to refer to the PPDU. Unfortunately, "frame" is defined to refer to an MPDU | Replace errant "frame"s in the PHY sections with "PPDU"s | REVISED (EDITOR: 2013-03-11 14:52:24Z) - Replace all "frame" in the PHY clause with "PPDU" where it relates to the on-the-air PHY packet/frame/structure. Also check affected definitions (RCPI ...). Also change "PHY frame" to "PPDU" Also change "PPDU frame" to "PPDU" At 1668.09, change "frame" to "symbol" | EDITOR |

Discussion:

We have related comment, CID 1595, which was originally treated by removing the terminology “PHY frame”.   This (in-)famously found no consensus in the group.

And worse than having to undo a bunch of speculative edits, I’m even starting to see things the same way as the commenter.  However, before we all go “frame crazy”, I’d suggest we limit 802.11 uses to two contexts:  MAC frame and PHY frame, and include these in the definition of frame.

We then change the definitions of MPDU, PPDU and add a definition of frame to make this relationship clear.  I will unmake the previous speculative edits for CID 1595, and we can resolve it here.

Also note that we have comments 1128 and 1129 which replace the “PHY frame” terminology with “PPDU” in the specific context of the subclause heading that defines the PPDU format.  This is for consistency with Clauses 17, 19 and 20.   The same edits were applied when rolling in 802.11ad for consistency.

Also for discussion (no changes proposed as yet), what do we interpret “frame” in the following context to mean: 1668.06?

The parameters F and NST are described in Error: Reference source not found. The resulting waveform is periodic with a period of TFFT = 1/F. Shifting the time by TGUARD creates the “circular prefix” used in OFDM to avoid ISI from the previous frame.

Also for the comment on use of “packet” – see CID 1193 below.

Changes:

In 3.1 add:

frame: A unit of data exchanged between peer protocol entities.

MAC frame:  The unit of data exchanged between MAC entities.  Syn: MPDU.

NOTE—References to a “frame” from within the clauses describing the MAC are implicitly references to a MAC frame, unless otherwise qualified.

PHY frame: The unit of data exchanged between PHY entities. Syn: PPDU.

NOTE—References to a “frame” from within the clauses describing the PHYs are implicitly references to a PHY frame, unless otherwise qualified.

Change the definition of MPDU thus:

medium access control (MAC) protocol data unit (MPDU):The unit of data exchanged between two peer MAC entities using the services of the physical layer (PHY). Syn: MAC frame.

Globally change any “PPDU frame” to “PHY frame”

Proposed Resolution (to all 3 comments):

Revised.  Make changes in <this-document> under CID 1179.   These introduce definitions of frame, MAC frame and PHY frame, and modify the definition of MPDU so that it is clear that the term “frame” is dependent on context.

| 1193 | 32.29 | 3.2 | "packet" is used in "null data packet", "packet number", "IGTK packet number", "LDPC packet", ... but what is a packet? Is it a general name of structure similar to a frame, a specific type of frame, or what? | Define "packet". If no single definition covers all of the uses in this standard, create a definition that fits "null data packet" and change the other instances to "frame", "PPDU", etc. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

There are 226 occurences of this term in D1.0 distributed as shown here:

The big cluster is in the PHY.

Detail:

.......................... 1066 9.31 Null data packet (NDP) sounding................................... ............................... 1790 20.3.19.5 Packet alignment........................................ ........................ 2468 L.1.8 The entire packet for the BCC example ............................. 40 Table 20-9—Cyclic shift for non-HT portion of packet.................................................. Table 20-10—Cyclic shift values of HT portion of packet ................................................. .............................. 1789 Figure 20-21—Packet alignment example (HT-greenfield format packet wi 21—Packet alignment example (HT-greenfield format packet with short GI)......................... 1790 Fig oded in the transmitted beacon frames. null data packet (NDP): A physical layer (PHY) protocol data unit e RXVECTOR STBC parameter equal to 0. null data packet (NDP) announcement: A physical layer (PHY) protoc rotocol data unit (PPDU) that is not a null data packet (NDP) and that includes one or more Data Long Tra rotocol data unit (PPDU) that is not a null data packet (NDP) and that includes one or more Data Long Tra , January 2013 GNonce group nonce GPRS general packet radio service GPS Global Positioning System GQM tor nonce IPI idle power indicator IPN IGTK packet number IQMF individually addressed quality-of- ver NAV network allocation vector NDP null data packet NonERP nonextended rate PHY NTP Network Time Pr oexistence operation PDU protocol data unit PER packet error ratio PERR path error PHB per-hop behavio MKSA pairwise master key security association PN packet number PN pseudonoise (code sequence) PNonce pe ernal network may have its own layer-3 end-to-end packet marking practice (e.g., differentiated services ice provides STAs a mapping of network-layer QoS packet marking to over-the-air QoS frame marking (i.e., on is negotiated, transport the IGTK and the IGTK packet number (IPN) from the Authenticator to the Suppl frame protection is negotiated, the IGTK and IGTK packet number are sent from the Authenticator to the Su ng frames following the current one. If null data packet (NDP) sounding frame is used, then the value in cording to the rules described in 9.31 (Null data packet (NDP) sounding)). It is set to 1 to indicate tha onds to the end of the reception of the sounding packet that was used to generate feedback information c the following parameters contained in an Ethernet packet header: Source Address, Destination Address, and t octet of the transmit sequence counter (TSC) or packet number (PN) is in the first octet of the RSC fie eHT-LTFs corresponding to the data portion of the packet is limited by the Rx MCS Bitmask subfield of the stored in the first 6 octets; for CCMP it is the Packet Number (PN), and is stored in the first 6 octets; er and b) The duration remaining in the current packet after the L-SIG, which is equal to the duration o , which is equal to the duration of the current packet less (aPreambleLength + aPHYHeaderLength) A dur HT-LTFs corresponding to the data portion of the packet is limited by the Rx MCS Bitmask subfield of the edback on the receipt of NDP (see 9.31 (Null data packet (NDP) sounding)) if this STA declares support fo a sounding PPDU is described in 9.31 (Null data packet (NDP) sounding). NOTE—A STA that acts as a beam - TXSTART.request primitive corresponding to each packet that is used for sounding. A beamformer shall s ter value equal to SOUNDING indicates a sounding packet. A non-NDP request for feedback is a sounding PPD in a SIFS after the end of the received sounding packet and the beamformee is subsequently required to tr onse, the beamformer shall not transmit the next packet to the beamformee until PIFS after transmitting t onse, the beamformer shall not transmit the next packet to the beamformee until PIFS after the sounding r onse, the beamformer shall not transmit the next packet to the beamformee until PIFS after transmitting t and Figure 9-43 (Receive ASEL). 9.31 Null data packet (NDP) sounding 9.31.1 NDP rules Sounding may be uring association processes an NDP as a sounding packet if the destination of the sounding packet is dete ounding packet if the destination of the sounding packet is determined to match itself as described in 9. DP destination) and if the source of the sounding packet can be ascertained as described in 9.31.4 (Deter packets would be treated as a type of management packet by the higher layers. The time stamp in the Sync by the higher layers. The time stamp in the Sync packet would contain the higher layer clock value at the r clock value at the time when the previous Sync packet was transmitted. The sequence number parameter ha hich the time stamp is provided. The reason the packet would contain the time stamp for the previous Syn ould contain the time stamp for the previous Sync packet (rather than the current packet) is that hardwar he previous Sync packet (rather than the current packet) is that hardware and layering constraints would e a time stamp for the exact instant the current packet is transmitted within that packet. However, a MLM ant the current packet is transmitted within that packet. However, a MLME-HL-SYNC.indication primitive al e transmitting STA to know exactly when each Sync packet is transmitted. A receiving STA might note the t receiving STA might note the time when each Sync packet is received as well as the sequence number for th would save this receive time indication for each packet along with its sequence number and compare the i re the indication of the previously received Sync packet to the time stamp in the most currently received to the time stamp in the most currently received packet. This comparison allows the STA to compute an off of the sequence number verifies that the correct packet is being used for time stamp comparison. It is po used for time stamp comparison. It is possible a packet is lost; in this case, the received time stamp f this case, the received time stamp for the lost packet should be discarded. The last symbol of the Syn would also similarly note the time when each Sync packet is received from the AP. The sequence number wou e IPv4 address being resolved in the ARP request packet is used by a non-AP STA currently associated to t as the Sender’s MAC Address in the ARP Response packet. When an IPv6 address is being resolved, the Pr d by a given temporal key, and CCMP uses a 48-bit packet number (PN) for this purpose. Reuse of a PN with ransmission) and 11.4.4.6 (BIP reception) for per packet BIP processing. NOTE—When the IPN space is exha rType 88-8E. Only IEEE Std 802.1X frame types EAP-Packet and EAPOL-Start are valid for preauthentication. 1.0, January 2013 Protocol Version – 1 octet Packet Type – 1 octet Packet Body Length – 2 octets ocol Version – 1 octet Packet Type – 1 octet Packet Body Length – 2 octets Descript DescriptDescrip GTK KDE format). The IPN corresponds to the last packet number used by the broadcast/multicast transmitte ocedures). In order to recover from over-the-DS packet losses, the FTO may retransmit the FT Confirm fra ength fields, along with the AP Address. The FT Packet Type field shall be set to 0 for remote request a /Response Payload format Size Information 1 FT Packet Type 2 FT Action Length 6 AP Address Variable used by the PHY for reception of the most recent packet. Table 16-2—RXVECTOR parameters Parameter Val VICE, and LENGTH fields for a DBPSK signal with a packet length of 192 µs (24 octets) would be given by t number supplied in the TXVECTOR LENGTH field. The packet transmission shall be completed and the PHY enti dium for the intended duration of the transmitted packet. If the PHY header is successful, but the indic ium for the intended duration of the transmitted packet. Conformance to DSSS PHY CCA shall be demonstra for CCK) shows an example calculation for several packet lengths of CCK at 11 Mb/s. Table 17-2—Example s are required for decoding the DATA part of the packet. In addition, the CCA mechanism is augmented by p m is augmented by predicting the duration of the packet from the contents of the RATE and LENGTH fields, ting bit string constitutes the DATA part of the packet. Refer to 18.3.5.4 (Pad bits (PAD)) for details. n and the coding rate as used in the rest of the packet. The encoding of the SIGNAL single OFDM symbol s se operations is also shown in L.1.8 (The entire packet for the BCC example). 18.3.6 CCA The PHY shall ine frequency offsets shall be estimated. d) The packet shall be derotated according to estimated frequen h) Compute the RMS average of all errors in a packet. It is given by Nf . i =1 ErrorRMS = ---- --- --(18-28) Nf where LP is the length of the packet; Nf is the number of frames for the measurement; 18.3.10.2 Receiver minimum input sensitivity The packet error ratio (PER) shall be 10% or less when the P ied in the OFDM PHY preamble LENGTH field. The packet transmission shall be completed and the PHY entit dium for the intended duration of the transmitted packet. If the indicated rate in the SIGNAL field is n 18.3.5 (DATA field). For ERP-OFDM modes, an ERP packet is followed by a period of no transmission with a te PHY (ERP) specification) STAs. The rest of the packet cannot be decoded by Clause 18 (Orthogonal frequ support the reception of an HT-greenfield format packet shall be able to detect that an HT-greenfield for ll be able to detect that an HT-greenfield format packet is an HT transmission (as opposed to a non-HT tr IG_VECTOR. The TXVECTOR supplies the PHY with per-packet transmit parameters. Status of the transmission RXVECTOR, the PHY informs the MAC of the received packet parameters. Using the Copyright © 2013 IEEE. Al nd coding scheme used in the transmission of the packet. The value used in each MCS is the index defined HFORMAT is HT_MF or HT_GF Indicates whether the packet is transmitted using 40 MHz or 20 MHz channel wi FORMAT is HT_MF or HT_GF Indicates whether this packet is a sounding packet. Enumerated type: SOUNDING T_GF Indicates whether this packet is a sounding packet. Enumerated type: SOUNDING indicates this is a ated type: SOUNDING indicates this is a sounding packet. NOT_SOUNDING indicates this is not a sounding p t. NOT_SOUNDING indicates this is not a sounding packet. Y Y Otherwise Not present N N AGGREGATIONFORM PDU. Enumerated type: AGGREGATED indicates this packet has A-MPDU aggregation. NOT_AGGREGATED indicates -MPDU aggregation. NOT_AGGREGATED indicates this packet does not have A-MPDU aggregation. Y Y Otherwis uard interval is used in the transmission of the packet. Enumerated type: LONG_GI indicates short GI is e: LONG_GI indicates short GI is not used in the packet. SHORT_GI indicates short GI is used in the pack cket. SHORT_GI indicates short GI is used in the packet. Y Y Otherwise Not present N N Copyright © 20 th transmits an HT-mixed or HT-greenfield format packet of 20 MHz bandwidth with one to four spatial stre TA transmits an HT-mixed or HT- greenfield format packet of 20 MHz bandwidth with one to four spatial stre TA transmits an HT-mixed or HT- greenfield format packet of 20 MHz bandwidth with one to four spatial stre l to transmit an HT-mixed or HT-greenfield format packet of 40 MHz bandwidth with one to four spatial str operating channel width transmits a non-HT format packet according to Clause 18 (Orthogonal frequency div Hz non-HT upper format—The STA transmits a non-HT packet of type ERP-DSSS, ERP-CCK, ERP-OFDM, or OFDM in Hz non-HT lower format—The STA transmits a non-HT packet of type ERP-DSSS, ERP-CCK, ERP-OFDM, or OFDM in transmission)) that duplicates the 20 MHz non-HT packet in two 20 MHz halves of a 40 MHz channel. L-STF n Table 20-9 (Cyclic shift for non-HT portion of packet) (a possible implementation is shown in Figure 20 able 20-10 (Cyclic shift values of HT portion of packet) (a possible implementation is shown in Figure 20 lting bit string constitutes the DATA part of the packet. f) Initiate the scrambler with a pseudorandom all the information required to interpret the HT packet format. In the case of multiple transmit chains, ields and the L-SIG as part of an HT-mixed format packet is described in 20.3.9.3.2 (Cyclic shift definit yclic shift is applied to each OFDM symbol in the packet separately. Table 20-9 (Cyclic shift for non- HT . Table 20-9 (Cyclic shift for non- HT portion of packet) specifies the values for the cyclic shifts that are applied in the L-STF (in an HT-mixed format packet), the L-LTF, and L-SIG. It also applies to the HT also applies to the HT-SIG in an HT-mixed format packet. Table 20-9—Cyclic shift for non-HT portion of . Table 20-9—Cyclic shift for non-HT portion of packet values for non-HT portion of packet TCS iTX N portion of packet values for non-HT portion of packet TCS iTX Number of transmit chains Cyclic shif m Table 20-9 (Cyclic shift for non-HT portion of packet) .k is defined in Equation (20-5) and Equation ( n Table 20-9 (Cyclic shift for non-HT portion of packet) .k is defined in Equation (20-5) and Equation ( y Table 20-9 (Cyclic shift for non-HT portion of packet) for HT-mixed format PPDUs Mr NOTE— exists for Table 20-10 (Cyclic shift values of HT portion of packet). Copyright © 2013 IEEE. All rights reserved. Table 20-10—Cyclic shift values of HT portion of packet values for HT portion of packet TCS iSTS Numb f HT portion of packet values for HT portion of packet TCS iSTS Number of space-time streams Cyclic to carry information required to interpret the HT packet formats. The fields of the HT-SIG are described indicate that the PPDU in the data portion of the packet contains an AMPDU; otherwise, set to 0. STBC 2 s only (see 9.31.2 (Transmission of an NDP)). The packet ends after the last HT-LTF or the HT-SIG. The s for the HT-SIG in an HT ,== , , mixed format packet in a 20 MHz transmission shall be as shown in Equ y Table 20-9 (Cyclic shift for non-HT portion of packet) for HT-mixed format PPDUs. NOTE—This definitio Table 20-10 (Cyclic shift values of HT portion of packet) Qk is defined in 20.3.11.11.2 (Spatial mapping) cludes HT-ELTFs or decode that frame. (When an HT packet includes one or more HT-ELTFs, it is optional fo Table 20-10 (Cyclic shift values of HT portion of packet). The generation of HT-DLTFs is shown in Figure Table 20-10 (Cyclic shift values of HT portion of packet) iESS TCS cyclic shift values are given in Tab Table 20-10 (Cyclic shift values of HT portion of packet) with = iESS iSTS Qk is defined in 20.3.11.11. Table 20-10 (Cyclic shift values of HT portion of packet). 20.3.9.5.3 HT-GF-STF definition The HT-GF-ST ere iSTS (Cyclic shift values of HT portion of packet) Qk is defined in 20.3.11.11.2 (Spatial mappin ble 20-10 (Cyclic shift values of HT portion of packet) Qk is defined in 20.3.11.11.2 (Spatial mapping able 20-10 (Cyclic shift values of HT portion of packet) Qk is defined in 20.3.11.11.2 (Spatial mapping) able 20-10 (Cyclic shift values of HT portion of packet) Qk is defined in 20.3.11.11.2 (Spatial mapping) n Table 20-9 (Cyclic shift for non-HT portion of packet) to the transmission in each chain. 20.3.11 Dat r of OFDM symbols in which the Data field of the packet may fit. Npld = length × 8 + 16 (20-35) Npld to the total number of space-time streams for the packet. Table 20-19—Pilot values for 20 MHz transmiss s, NSTS <NTX — As part of (an optional) sounding packet — As part of (an optional) calibration procedure f (an optional) calibration procedure — When the packet is transmitted using one of the (optional) beamfo s feedback from the STA to which the beamformed packet is addressed. When there are fewer space-time s e applied to subcarrier k during all parts of the packet in HT-greenfield format and all parts of the pac ket in HT-greenfield format and all parts of the packet following and including the HT-STF field in an HT including the HT-STF field in an HT-mixed format packet. This operation is transparent to the receiver. able 20-10 (Cyclic shift values of HT portion of packet) and the frequency-dependence in the matrix Qk , Table 20-10 (Cyclic shift values of HT portion of packet) shall be applied at the input of the spatial ma SYM TGI TCS where z is 3 in an HT-mixed format packet and 2 in an HT-greenfield format packet pn is de ed format packet and 2 in an HT-greenfield format packet pn is defined in 18.3.5.10 (OFDM modulation) . )) F tnTSYM where z is 3 in an HT-mixed format packet and 2 in an HT-greenfield format packet pn is de ed format packet and 2 in an HT-greenfield format packet pn is defined in 18.3.5.10 (OFDM modulation) C ort GI Short GI is used in the data field of the packet when the Short GI field in the HT-SIG is equal to Table 20-9 (Cyclic shift for non-HT portion of packet) Tone NNon-HT Duplicate is defined in Table 20 ming, in order for STA A to transmit a beamformed packet to STA B, STA B measures the channel matrices an ing matrix that was used to transmit the sounding packet that elicited theVk feedback. The effective chann Table 20-10 (Cyclic shift values of HT portion of packet) from the measured channel matrix. The matrice Table 20-10 (Cyclic shift values of HT portion of packet) from the measured channel before computing a se Table 20-10 (Cyclic shift values of HT portion of packet) from the measured channel before computing a set referred to as MIMO channel sounding. A sounding packet may be used to sound available channel dimension n Table 20-9 (Cyclic shift for non-HT portion of packet) PCAL is one of the following unitary matrices: d from the same reference oscillator. 20.3.19.5 Packet alignment If no signal extension is required (se ollowing the reception of the last symbol of the packet. If a signal extension is required, the receiver ollowing the reception of the last symbol of the packet. This situation is illustrated for an HT-greenfie uation is illustrated for an HT-greenfield format packet using short GI in Figure 20-21 (Packet alignment ld format packet using short GI in Figure 20-21 (Packet alignment example (HT-greenfield format packet wi 1 (Packet alignment example (HT-greenfield format packet with short GI)). If no signal extension is requ the trailing boundary of the last symbol of the packet on the air. If a signal extension is required, th g the trailing boundary of the last symbol of the packet on the air. This situation is illustrated in Fig . This situation is illustrated in Figure 20-21 (Packet alignment example (HT-greenfield format packet wi 1 (Packet alignment example (HT-greenfield format packet with short GI)). Signal Extension HT-GF-STF HT- in some PPDU formats. See 20.3.2. Figure 20-21—Packet alignment example (HT-greenfield format packet wi 21—Packet alignment example (HT-greenfield format packet with short GI) Copyright © 2013 IEEE. All right 20.3.20.1 Receiver minimum input sensitivity The packet error ratio (PER) shall be less than 10% for a PS ce (RIFS) The receiver shall be able to decode a packet that was transmitted by a STA with a RIFS separat y a STA with a RIFS separation from the previous packet. 20.3.21 PHY transmit procedure There are thre o the number supplied in the LENGTH field. The packet transmission shall be completed, and the PHY enti mitive until either the predicted duration of the packet from the contents of the HT-SIG field, as define continued) DS7.4 DS7.5 Item Hold CCA busy for packet duration of a correctly received PPDU but carrie lost during reception of MPDU Hold CCA busy for packet duration of a correctly received but out of spe .5.10 (OFDM modulation ) M Yes . No . OF2.18 Packet duration calculation M Yes . No . OF2.19 CCA OF M Yes . No . N/A . OF2.19.3.2 Hold CCA busy for packet duration of a correctly received PPDU, but carri cations OF5.1 Minimum input level sensitivity at packet error ratio (PER) = 10% with 1000 octet frames s .No .. OF5.6 Maximum input level sensitivity at packet error ratio (PER) = 10% with 1000 octet frames N/A . OF5.11 Maximum input level sensitivity at packet error ratio (PER) = 10% with 1000 octet frames ergy detect with High Rate CS Hold CCA busy for packet duration of a correctly received PPDU, but carri ost during reception of MPDU. Hold CCA busy for packet duration of a correctly received, but out of spe ASEL)) CF16:O Yes . No . N/A . *HTM15 Null data packet (NDP) 9.31 (Null data packet (NDP) sounding) C . *HTM15 Null data packet (NDP) 9.31 (Null data packet (NDP) sounding) CF16:O Yes . No . N/A . HTM16 as soon as practical in the implementation. A packet count (in 1000s of packets) after which the RSNA P802.11-REVmc/D1.0, January 2013 refreshed. The packet counter starts at the moment the GTK was set usin k frame and Block Ack Request, either in the same packet, or in separate packets. *) burst-ba-bar = (ppd tiator measures the channel based on the sounding packet and updates its beamforming feedback matrices ba atrices based on its observations of the sounding packet. No channel measurements are sent over the air.* j –1.0 +0 j –1.0 +0 j 1.0 +0 j L.1.8 The entire packet for the BCC example The packet in its entirety i L.1.8 The entire packet for the BCC example The packet in its entirety is shown in the tables in this su significant octet of the timestamp when Ethernet packet is received Concatenate this octet onto result E-TXTIME primitive that returns the duration of a packet based on its payload size and the PHY data rate w) to meet a particular probability of dropping a packet because it exceeds its delay bound. Note that fo the frame to be p. The probability of any given packet being dropped in such a channel after Np retries e by the scheduler to so that application dropped packet rates are bounded. For example, this parameter c can be chosen to ensure that when there is a 10% packet error ratio (PER) for 1000-octet packets, that t 0%, with the errors happening independently from packet to packet. To accomplish this, the number of pack he errors happening independently from packet to packet. To accomplish this, the number of packets transm e required to keep the probability of dropping a packet to less than 10–8 to send only 100 packets. For t e probability that nretries are required for any packet is given by 0.1n). In fact, assuming a finite d nterval, then the delay incurred in waiting for a packet to be transmitted can be inferred from examining re required to keep the probability of a dropped packet below a certain amount. The number of retries (an rotocol 0x80F3 80 F3 Novell NetWare Internetwork Packet exchange (IPX) 0x8137 81 37 P.3 Example In ord -65AA- AA-03-00-00-00-08-00c aThis format of IP packet over IEEE Std 802.3 is denigrated, and the change -00-08-06 81-00 87-65-08-06 aThis format of IP packet does not survive the trip across the non-IEEE-802 d fine frequency offsets are estimated. e) The packet is derotated according to estimated frequency off gital-to-analog converter(s) used to transmit the packet, (b) a 32-bit continuously counting counter and ( 34). See TR 21.905 [B2] for definition of general packet radio service (GPRS) roaming exchange. Table V-1 E-TXTIME primitive that returns the duration of a packet based on its payload size and the PHY data rate um time). This value includes the duration of the packet plus SIFS and ACK times. The medium time, theref e, for a single stream, between each transmitted packet, there is a time period due to SIFS, AIFSN, and c r more streams, there is also the time when each packet is delayed while another packet is being transmit he time when each packet is delayed while another packet is being transmitted. Hence, in order to calcula

Most of these “packet”s could be replaced with PPDU, although “packet error ratio” is a rather curious term because it is detecting the error at the MAC layer (FCS failure).

Status:  for discussion.  Should we try and fix this?

Deferred

| 1182 | 24.10 | 3.1 | The terms "TDLS peer PSM Request frame" and "TDLS peer PSM Response frame" don't appear to be defined anywhere. Yes, these probably are data frames that include appropriate values in Action fields, but that state is not specifically defined anywhere. Note that other frames that are types of action frames are still specifically defined to be frames that have Action fields -- such as 8.5.12.4 (PSMP frame format). | Add definitions of "TDLS Peer PSM Request frame" and "TDLS Peer PSM Response frame" to 6.3.49. | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.  8.5.13.1 states: “References to one of the TDLS Action field values as a frame, e.g., “TDLS Setup Request frame,” denote a Data frame carrying a TDLS Action Field and any vendor-specific elements tunneled as described in 10.23.1(General).”

“TDLS Peer PSM Request” and “TDLS Peer PSM Response” appear in the list of TDLS Action field values.

This suffices to explain the meaning of “TDLS Peer PSM Request frame” and “TDLS Peer PSM Response frame”.

| 1013 | 27.55 | 3.2 | "A Data MPDU which carries all or part of an 802.1X EAPOL PDU of the mentioned type" Mentioned where? Perhaps in the personals column of the Times. Or perhaps in dispatches. | Insert reference to where it's mentioned, or reword so that it makes sense. | GEN |
| --- | --- | --- | --- | --- | --- |

Change: 27.55

EAPOL-Key frame: A Data MPDU that carries all or part of an 802.1X EAPOL PDU of type EAPOL-Key. (#1674)

Proposed resolution:

Revised.   In cited definition change “of the mentioned type” to “of type EAPOL-Key”, and change “which” to “that”.

| 1674 | 27.55 | 3.2 | EAPOL-Key and EAPOL-Start are not the same. See 802.1X-2010 clause 11.3.2 | Delete Syn: EAPOL-Start frame. |
| --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted

| CID | Page | Clause | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 1675 | 27.62 | 3.2 | EAPOL-Key and EAPOL-Start are not the same. See 802.1X-2010 clause 11.3.2 | Add an appropriate definiteion for EAPOL-Start frame |  | GEN |

Proposed Resolution:

Rejected.

This term is adequately defined in 802.1X-2010.

| 1192 | 31.47 | 3.2 | "Mesh Data frame" names a type of frame, and therefore needs to be defined in clause 8. It is bad policy to split the definitions of frames between two different clauses. | Move the definiiton "Mesh Data frame" to 8.3.2. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The cited text is: 30.47:

Mesh Data frame:An individually addressed Data frame with both the From DS and To DS bits set to 1 and that is transmitted from a mesh station (STA) to a peer mesh STA, or a group addressed Data frame that has From DS set to 1 and To DS set to 0 that is transmitted by a mesh STA.

At 438.14 (8.2.2 (conventions)), we have:

A QoS Data frame that is transmitted by a mesh STA is referred to as a Mesh Data frame.

AT 441.25 (8.2.4.1.4 (To/From DS)…) we have:

Clause 8 is generally about describing structure.   The structure of a Mesh Data frame has already been described through 8.2.4.7.3 (which describes a more general case).  The statement in “conventions” creates a more general definition than in 3.2, i.e., it is conflicting.

Propose we move any normative definitions more clearly in Table 8-2, and reference 8.2.4.1.4 from any other occurences.  Then make the definition in 3.2 more general and reference 8.2.4.1.4.

Proposed changes:

Change 30.47 as follows:

Mesh Data frame:A Data frame that is transmitted by a mesh STA. See 8.2.4.1.4.

Change 437.14 as follows:

A QoS Data frame that is transmitted by a mesh STA is referred to as a Mesh Data frame. NOTE—8.2.4.1.4 constrains the setting of the FromDS/ToDS fields in Mesh Data frames.

Change 441.18 as follows:

- To DS and From DS fields The meaning of the combinations of values for the To DS and From DS fields in (#100)Data frames(Ed) are shown in To/From DS combinations in . - To/From DS combinations in (#100)Data frames. To DS and From DS values; Meaning. To DS = 0 From DS = 0; A (#100)Data frame direct from one STA to another STA within the same IBSS, a (#100)Data frame direct from one non-AP STA to another non-AP STA within the same BSS, or a (#100)Data frame outside the context of a BSS.(11ae). To DS = 1 From DS = 0; A (#100)Data frame destined for the DS or being sent by a STA associated with an AP to the Port Access Entity in that AP.. To DS = 0 From DS = 1; A (#100)Data frame exiting the DS or being sent by the Port Access Entity in an AP, or a group addressed Mesh Data frame with Mesh Control field present using the three-address MAC header format. This is the only valid combination for group addressed Data frames transmitted by a mesh STA.. To DS = 1 From DS = 1; A (#100)Data frame using the four-address MAC header format. This standard defines procedures for using this combination of field values only in a mesh BSS. This is the only valid combination for individually addressed Data frames transmitted by a mesh STA..

Proposed resolution:

Revised.  Make changes in <this-document> under CID 1192.  These changes clarify in the specification of the FromDS/ToDS fields which settings must be used by a mesh STA and simplify the definition of Mesh Data frame, now referencing the FromDS/ToDS section.

| 1563 | 113.32 | 6.3.2.2.2 | What is the point of WakeUp in MLME-POWERMGT.request? It's not referred to in clause 10 (all the references are to the Wakeup Schedule, which is something different) | Delete this parameter | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The commenter is correct.  The necessary changes are obvious.

Proposed Resolution:

Accepted.

| 1412 | 185.33 | 6.3.26.2.2 | Why do ADDTS, DELTS, ADDBA and DELBA need any special "timer" treatment? | Remove the timeout from the service primtives (6.3.26.2.2's parameters and 6.3.26.3.2's ResultCode and associated text, etc.). Note that for ADDBA, this is the ADDBAFailureTimeout, not the BlockAckTimeout which is needed. Remove the timeout from the figures and text in 10.4 and 10.5. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

There is no default value for the timeouts, so they are of practical use.

Agree that there is no reason these primitives should be treated any differently to any other management exchange resulting in the exchange of action frames.

ADDTS has a timeout parameter.  No normative behaviour is specified.

Note that normative behaviour is specific for the timing of the ADDTS.response (within dot11ADDTSResponseTimeout).  This is a rather curious thing to do.  We have a control variable somehow modifying the SME’s guaranteed timing.

DELTS does not have a timeout parameter.  Neither does DELBA.

ADDBA links this parameter to a MIB variable, so, in theory at least, this does have a potential practical benefit, because an external management entity can change the STA’s expectations about response times and avoid unnecessary timeouts.

We have two different timeout models:  ADDTS.request “stupid” and ADDBA.request “possibly useful”.

What do we do:

- Leave it as it

- Remove them because the MIB is only a theoretical construct,  so the timeout provides no practical benefit

- Change ADDTS to match ADDBA.

Straw poll?

Status:  waiting for discussion before proposing a resolution

| 1228 | 204.35 | 6.3.29.3.2 | Where is the "Block Ack Action primitive" defined? There is a Block Ack Action frame, but what is the primitive and in what SAP is it located? | Define the Block Ack Action primitive somewhere, or delete all references to it. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The language used at this point (MLME-ADDBA.confirm) is:

Specifies a number unique to the Block Ack Action primitives and frames used in defining the Block Ack. This value matches the DialogToken parameter specified in MLME-ADDBA.request primitive.

There are a fair number of similar descriptions (“primitives and frames” occurs 7 times).

Probably the most prevalent form used is that of 214.28 (MLME-NEIGHBORRESPRESP.request):

The Dialog Token to identify the neighbor report transaction. Set to the value received in the corresponding MLME-NEIGHBORREPREQ.indication primitive or to 0 for an autonomous report

I believe usage this is preferable.   The “and frames” of the cited text is problematical – the MLME interface should only care about primitives and their parameters.  The “in defining the Block Ack” begs a whole host of questions.

Note the wording of the ADDTS.response parameter is already OK, because it doesn’t use this language.

I propose to reword the similar parameter descriptions to more closely follow “transaction” language.

Proposed Resolution:

Revised.

Replace first sentence of description of ADDBA .request, .confirm, .indication and .response primitives DialogToken with: “Identifies the ADDBA transaction.”

Replace first sentence of description of ADDTS .request, .confirm and .indication primitives DialogToken with: “Identifies the ADDTS transaction.”

| 1018 | 329.03 | 6.3.68.6.1 | All requestion/indication and response/confirm pairs that are transported using action frames should include Vendor Specific in their primitive parameters, asthe action frames include Vendor Specific elements by default. The GATS-TERM primitives do not contain these elements. | Review all MLME primitives that either generate or are generated by an action frame and insert any missing vendor specific parameters. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

Another “permission to do more work”.  However we have done this work in the past, so might as well do it again.

Note that MLME-FINETIMINGMSMT.confirm wrongly has a VendorSpecific parameter.  This is covered by a separate comment (CID 1015).

For discussion: are the MLME-RESOURCE-REQUEST primitives already covered.  i.e., does the “Content of FT Authentication elements” include vendor specific elements. If so, nothing to do.

I’m making this assumption – i.e., least changes.

Same discussion for MLME-TDLSDISCOVERY “TDLSDiscoveryRequest: Specifies the proposed service parameters for the TDLS Discovery Request frame.”, TDLSSETUPREQUEST.*, TLDSSETUPRESPONSE.*, TDLSSETUPCONFIRM.*, TDLSTEARDOWN.*, TDLSPTI.*, TDLSCHANNELSWITCH.*, TDLSPEERPSM.*

For discussion.  Where does the vendor specific content in MLME-DLSPOTENTIALPEERSTA.confirm come from?  I’m assuming it is an error,  because there is no matching .response that can provide it.

Proposed Resolution:

Revised.

Remove the Vendor Specific parameter from:

MLME-MMEASURE.*

MLME-TDLSPOTENTIALPEERSTA.confirm

Add a Vendor Specific parameter (using a nearby example to maintain local style) to:

MLME-ADDTSRESERVE.confirm

MLME-SCHEDULE.request / .indication

MLME-SAQuery.*

MLME-EVLREQUEST.*

MLME-EVLREPORT.*

MLME-DIAGREQUEST.*

MLME-DIAGREPORT.*

MLME-LOCATIONCFG.*

MLME-LOCATIONTRACKNOTIF.*

MLME-BTMQUERY.*

MLME-BTM.*

MLME-FMS.*

MLME-CLINTERFERENCEREQUEST.*

MLME-CLINTERFERENCEREPORT.*

MLME-TIMBROADCAST.*

MLME-QOSTRAFFICCAPUPDATE.*

MLME-GATS.*

MLME-GATS-TERM.*

MLME-TIMINGMSMTRQ.*

MLME-WNMNOTIFICATIONREQUEST.*

MLME-WNMNOTIFICATIONRESPONSE.*

MLME-GAS.*

MLME-QoSMap.*

MLME-MESHPEERINGMANAGEMENT.*

MLME-MESHPOWERMGT.*

MLME-MCCATEARDOWN.*

MLME-QMFPOLICY.*

MLME-QMFPOLICYCHANGE.*

MLME-SCS.*

MLME-SCS-TERM.*

MLME-QLOAD.*

MLME-TXOPADVERTISEMENT.*

MLME-GROUP-MEMBERSHIP.*

MLME-APPEERKEY.*

Add an MLME-DLS.response primitive, with the following parameters:  (PeerMACAddress, from .indication; ResultCode, CapabilityInformation “Specifies the capabilities of the local MAC entity” to replace description, DLSTimeoutValue, Supported Rates, HT Capabilities, VendorSpecificInfo from .confirm).

Status: Defer

| 1019 | 379.35 | 6.3.86.3.2 | Note that according to WG11 style, confirms are not issued in the case of locally-generated errors such as invalid parameters and timeout. The SCS.confirm doesn't conform. | Remove any locally generated errors. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

We did this once before during REVmb, setting the style cited above.  I have reviewed the new material with the following findings.

Findings:

SCS.confirm.   Frame doesn’t carry a Status Code field, so the parameter is spurious.

MLME-QMFPOLICYCHANGE.confirm. Local timeout.

MLME-QLOAD.confirm. Local generation of UNSPECIFIED_FAILURE.

MLME-TXOPADVERTISEMENT.confirm. Local generation of UNSPECIFIED_FAILURE.

MLME-GROUP-MEMBERSHIP.confirm. Local generation of UNSPECIFIED_FAILURE.

MLME-MCCASETUP.confirm. Local timeout.

Proposed Resolution:

Revised.

Remove the ResultCode parameter from the following primitives:

MLME-SCS.confirm

MLME-QLOAD.confirm

MLME-GROUP-MEMBERSHIP.confirm

Remove the *TIMEOUT enumeration value for ResultCode from:

MLME-QMFPOLICYCHANGE.confirm

MLME-MCCASETUP.confirm

Remove the UNSPECIFIED_FAILURE enumeration for ResultCode from:

MLME-TXOPADVERTISEMENT.confirm

| 1239 | 395.46 | 6.4.1 | "correlates", "converges" information: what processes are these? Also, it is unlikely that the MSGCF transmits events to upper layer protocols. And what other information than 802.11 information? | Replace "correlates information exchanged between the MAC management entiteis regarding the state of an IEEE 802.11 Std interface and converges this information into events and status for consumption by higher layer protocols." with "merges informaton about a MAC interface into event and state reports that are understood by higher layer protocols.". | GEN |
| --- | --- | --- | --- | --- | --- |

| 1240 | 395.57 | 6.4.1 | "operates at the level of an IEEE Std 802.11 ESS": what "level" is that? Where are the levels defined? Fortunately this is not a very useful claim, and so can simply be deleted. | Delete "operates at the level of an IEEE Std 802.11 ESS, and ". | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The MSGCF is an adaptor layer that provides information on internal MLME/SME state and provides control of link establishment with an ESS in a form that is understood by 802.21 (MIH).

The cited text is:

6.4.1 Overview of the convergence function The MSGCF and its interaction with other management entities is defined in 6.4 (MAC state generic convergence function (MSGCF)). The MSGCF correlates information exchanged between the MAC management entities regarding the state of an IEEE Std 802.11 interface and converges this information into events and status for consumption by higher layer protocols. Non-AP STAs when dot11MSGCFActivated is set to true shall support the MSGCF procedures in this clause; APs do not support the MSGCF.

I don’t see how: “The MSGCF correlates information exchanged between the MAC management entities” reflects the purpose of MSGCF.

I suppose the verb “converge” is almost, on a good day, kind-of OK-ish.   But I don’t want to damn it with faint praise.  Clearly it relates to “convergence” which is part of its name.   Better to avoid the question and use a verb that kind-of almost makes better sense.

Proposed change:

The MSGCF provides an abstraction of a link between a non-AP STA and an ESS (an ESS-link) to its higher layer entity. The MSGCF provides control of an ESS-link and generates events based on the state of an ESS-link. A non-AP STA that transitions between two APs in the same ESS can operate transparently to the LLC sublayer, and does not change state in the state machine defined within this clause. This clause defines interactions between the MSGCF and MLME and PLME through the MLME_SAP and PLME_SAP respectively, as well as with the SME via the MSGCF-SME_SAP. The detailed manner in which the SAPs are implemented is not specified within this standard.

Proposed resolution: (to both)

Revised.

Change text as shown in <this-document> under CIDs 1239 and 1240.  These changes clarify the language in the cited locations.

| 1244 | 399.09 | 6.4.7.1.2 | "IEEE Std 802.11" is redundant here. | Delete "IEEE Std 802.11 " here; on line 46 replace "an IEEE Std 802.11 network" with "a WLAN" and on line 61 delete "IEEE Std 802.11 ". | GEN |
| --- | --- | --- | --- | --- | --- |
| 1248 | 413.15 | 6.4.7.2.2 | "IEEE Std 802.11" is redundant here. | Delete "IEEE Std 802.11 " and on line 55 replace "IEEE 802.11 networks" with "WLANs"; on page 414 lines 25 and 47 delete "IEEE Std 802.11". | GEN |

Discussion:

We already decided to grandfather this term.   See CID 1212 et al.  We copy the resolution from that CID here.

Proposed resolution:

Rejected.

The cited text is not incorrect.  There are a substantial number of occurences of "IEEE Std 802.11" in the document and the TG has determined not to remove them.

| 1605 | 415.00 | 6.5 | PLME-DSSSTESTMODE.request, PLME-DSSSTESTOUTPUT.request -- seriously? | Kill them | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.

The commenter does not specify an issue to be resolved or a specific change to be made.

| 1513 | 416.00 | 6.5.4.2 | What is the "point in time specified by the PHY" in the context of aPHY-RX-START-Delay? | Clarify (might be the start of the PHY header?) | GEN |
| --- | --- | --- | --- | --- | --- |

| 1512 |  |  | The various PHY's aPHY-RX-START-Delay don't make obvious sense, apart for DSSS and HR/DSSS | Clarify where the numbers came from |  |
| --- | --- | --- | --- | --- | --- |

Status:  asking PHY guys to comment.

| 1404 | 425.57 | 7.3.4.1 | "All of the service primitives described here are considered mandatory, unless otherwise specified." Really?? Mandatory service primitives?!! | Delete this sentence. Therefore, delete this subclause? Note that similar language that was in many of the PHY clauses has already been removed, along with the PMD changes. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

Agree that the concept of a mandatory service primitive is meaningless, because being an abstact interface, it is not testable.

Proposed resolution:

Revised.  At 425.55, delete subclause 7.3.4.1.

| 1024 | 429.52 | 7.3.5.5.2 | "both PHY and PHY management" is questionable given that the PHY entity arguably includes the PLME. Ditto at 430.24. | Perhaps replace with something like "PHY data transmission and management" | GEN |
| --- | --- | --- | --- | --- | --- |

Context and commenter’s proposed change: (429.50):

The TXVECTOR represents a list of parameters thatthe MAC sublayer provides to the local PHY entity in order to transmit a PSDU. This vector contains PHY data transmission and management parameters. The minimum required PHY parameters are listed in 7.3.4.5 (Vector descriptions).

And 430.23:

The TXSTATUS represents a list ofparameters that the local PHY entity provides to the MAC sublayer related to the transmission of an MPDU. This vector contains both PHY and PHY operational parameters. The required PHY parameters are listed in 7.3.4.4 (PHY-SAP service primitives parameters)

Note that, unlike TXVECTOR, TXSTATUS is almost unspecified.  The only one that appears to be specified is TX_START_OF_FRAME_OFFSET. Seeing as there is only one parameter, describing it as “PHY and PHY operational” is meaningless.   The cited sentence adds no value.

Proposed Resolution:

Revised.

At 429.51, delete  the sentence “This vector contains … parameters.”

At 430.23, delete the sentence “This vector contains both PHY and PHY operational parameters.”

| 1267 | 586.39 | 8.4.2.21.2 | "It is mandatory" is a de facto "shall" statement, which does not belong in a definition clause. | Delete the sentence "It is mandatory for a STA to support the generation of this report.". If this requirement is needed, then locate the appropriate "shall" statement in a non-definition, normative clause. | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The normative requirement is at 1162.38: D1.0

10.9.7 Requesting and reporting of measurements The response to a basic request is a basic report. It is mandatory for a STA in an infrastructure BSS to generate a basic report in response to a basic request if the request is received from the AP with which it is associated, except as specified in this subclause.

The “is mandatory” language was adjusted in response to CID 1317,  so it now reads:

1452.23: (D1.5 pre-final version)

The response to a basic request is a basic report. A STA in an infrastructure BSS or PBSS(11ad)(Ed)shall generate(#1317)a basic report in response to a basic request if the request is received from the PCP/ AP(11ad)with which it is associated, except as specified in this subclause.

So, the cited statement is redundant, and we can fully address the commenter’s concern by deleting it.

Proposed Resolution:

Accepted.

In reply to the commenter, the related normative statement exists in 10.9.7, and the language in D1.0 is changed from “is mandatory” to “shall” in response to CID 1317.

| 1642 | 1345.09 | 11.4.2.3.3 | What does the $circledplus in 11.4.2.3.3 mean? | Probably XOR, but where is this stated? 11.4.2.5.3 doesn't count, as it's in the future, as is 11.4.2.5.2 | GEN |
| --- | --- | --- | --- | --- | --- |

Context: 1344.54:

Figure 11-11 (Michael block function) defines the Michael block function b. It is a Feistel-type construction with alternating additions and XOR operations. It uses <<< to denote the rotate-left operator on 32-bit values, >>> for the rotate-right operator, and XSWAP for a function that swaps the position of the 2 least significant octets.

And 1345:

Discussion:

That this is an XOR is unambiguous from 1350.62, in a later subclause:

The XOR (⊕) operation, the bit-wise-and (&) operation, and the addition (+) operation are used in the Phase 1 specification.A loop counter, i, and an array index temporary variable, j, are also employed.

Agree that this term hasn’t been defined where it is used.  We can either define it globally or locally.

The least change is local:

Change: 1344.54:

Figure 11-11 (Michael block function) defines the Michael block function b. It is a Feistel-type construction with alternating additions and XOR operations. It uses ⊕ to denote XOR, <<< to denote the rotate-left operator on 32-bit values, >>> for the rotate-right operator, and XSWAP for a function that swaps the position of the 2 least significant octets.

Proposed Resolution:

Revised.

At 1344.55, change “It uses <<< to denote…” to “It uses <circleplus> to denote XOR, <<< to denote…”

where <circleplus> is the circle-plus symbol appearing in Figure 11-11.

| 1659 | 1702.00 | 19 | "may be used when the BSS consists of only ERP STAs." should not be in Clause 19. If it is, it should at least say "that support this option" as in other places | Fix in 19.1.3 and 19.4.5 | GEN |
| --- | --- | --- | --- | --- | --- |

Context: 1703.24:

The changes to other parts of this standard required to implement the ERP are summarized as follows: … b) ERP-OFDM 1) The PHY uses the capabilities of Clause 18 (Orthogonal frequency division multiplexing (OFDM) PHY specification) with the following exceptions: i) The frequency plan is in accordance with 17.3.6.2 (Operating frequency range) and 17.3.6.3 (Channel Numbering of operating channels) instead of 18.3.8.4 (Operating channel frequencies). ii) CCA has a mechanism that detects all mandatory Clause 19 (Extended Rate PHY (ERP) specification) sync symbols. iii) The frequency accuracy (see 18.3.9.5 (Transmit center frequency tolerance) and 18.3.9.6 (Symbol clock frequency tolerance)) is ±25 PPM. iv) The maximum input signal level (see 18.3.10.5 (Receiver maximum input level)) is –20 dBm. v) The value of the slot time is found in Table 19-6 (ERP characteristics). The optional short slot time may be used when the BSS consists of only ERP STAs. vi) SIFS is 10 µs in accordance with 17.3.3 (DS PHY characteristics). See 19.3.2.4 (ERP-OFDM PPDU format) for more detail.

And 1709.40:

19.4.5 Slot time The value of the slot time is found in Table 19-6 (ERPcharacteristics). The optional short slot time may be used when the BSS consists of only ERP STAs capableof supporting this option. The optional short slot time shall not be used if the network has one or more NonERP STAs associated. For IBSS, the Short Slot Time subfield shall be set to 0, corresponding to the long slot time

Discussion:

This “may” occurs in a “summary of changes”, which is a very odd place to find it.  Further, selection of slot time is a MAC function, described normatively in 9.3.2.12 and 10.1.3.2.  The effect of these MAC specifications (although it’s rather distributed) is to require all STAs in a BSS to use long slot time if a STA that is not capable of short slot time joins the BSS.

The specification in 19.4.5 is even more egregiously non-PHY, as it talks about capabilities, association and the setting of specific MAC frame fields.  The desired behaviour in the IBSS (and MBSS) case is established in 8.4.1.4, 502.56: “For IBSS and MBSS, the Short Slot Time subfield is set to 0.”

(Shame on the mesh guys not spotting this and updating 19.4.5) :0).

The simplest thing to do is to delete the cited sentence and subclause 19.4.5.

Proposed resolution:

Revised.

Delete the cited sentence.  Delete 19.4.5.

The behaviour cited in 19.1.3 and 19.4.5 is already described normatively in the MAC (in 9.3.2.12, 10.1.3.2 and 8.4.1.4).

| 1643 | 1749.28 | 20.3.9.4.4 | What does the $circledplus in (20-18) mean? | Probably XOR, but where is this stated? | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.

At 1749.46,  add to the end of the “variable list”:  “<circle-plus> denotes XOR”

where <circle-plus> is the circle-plus symbol used in equation 20-18.

| 1702 | 1896.26 | B.4.9 | ERP8 should not have "shall" and should say "Set b2 to 1..." | Set b2 to 1 in all long and short preamble PPDU SERVICE fields | GEN |
| --- | --- | --- | --- | --- | --- |

Context: 1896:

Discussion:

Agree it’s weird to have a “shall” in the body of the PICS.   The PICS should reflect normative requirements created elsewhere,  not claim to create any of its own.

Proposed Resolution:

Accepted.

| 1445 | 1970.00 | C | If a MIB variable has type MacAddress, how is it represented? It appears to be represented as a big hex number, but then which is the octet which contains the U/L and G/I bits? The least significant one (i.e. the one at the end when the hex number is written out) or the most significant one? | Spell this out somewhere | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

MacAddress is defined by SNMPv2-TC.  See IETF RFC 2579.

| 1539 | 1970.00 | C | 2E32 is being used to express 2**32. However, 2E32 is actually 2*10**32 | Change the 2E32s to 2**32s | GEN |
| --- | --- | --- | --- | --- | --- |

Propose Resolution:

Revised.

Globally change 2E32 to 2**32.

| 1599 | 1970.00 | C | How does defval work for MAC addresses? Where is the I/G bit, when given as a hex number? | Clarify | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

The commenter is presumably referring to 2257.30, although the commenter does not state this.

(<sarcasm> It would be awefully nice if the commenter could spare some of his valuable time to note the location of a comment, rather than forcing a volunteer to spend some of his obviourly much less valuable time to go search for it. </sarcasm>)

Proposed Resolution:

Rejected.  The commenter does not indicate a problem to resolve or a specific change to make.

| 1004 | 1974.49 | C.3 | There is a mib variable for moredataackoptionimplemented, but there is no behavioral description, and according to 8.3.1.1, the moredata bit is always set to 0 in all control frames. | provide a behavioral description for the use of the mib variable moredataackoptionimplemented and clarify the meaning of the MOREDATA bit within control frames | GEN |
| --- | --- | --- | --- | --- | --- |

Discussion:

This MIB variable relates to the “More Data Ack subfield” of the “QoS Capability element”.

Subclause 8.2.4.1.8 states: (443)

8.2.4.1.8 More Data field … For a STA in which the More Data Ack subfield of its QoS Capability element is 1 and that has APSD enabled, an AP optionally sets the More Data field to 1 in ACK frames to this STA to indicate that the AP has a pending transmission for the STA. … The More Data field is set to 0 in all other individually addressed frames.

8.4.1.17 states: (516)

Non-AP STAs set the More Data Ack subfield to 1 to indicate that they can process ACK frames with the More Data bit in the Frame Control field equal to 1 and remain in the Awake state. Non-AP STAs set the More Data Ack subfield to 0 otherwise. For APs, the More Data Ack subfield is reserved.

And the definition of the dot11MoreDataAckOptionImplemented MIB variable includes: (1985.22)

"This is a capability variable. Its value is determined by device capabilities. This attribute, when true, indicates that the station implementation is capable of interpreting the More Data bit in the ACK frames. The capability is disabled, otherwise."

Although there is no explicit statement that the capability subfield is set based on the MIB variable (such as defined elsewhere for other capabilities),  they are both clearly dependent on the ability of the STA to support this feature (“they can process ACK frames”,  “is capable of interpreting  … in ACK frames”).

I believe this connection is unambiguous.  I also believe that 8.2.4.1.8 is unambiguous about the setting of the More Data field in control frames.   Ack frames are specifically called out, and other control frames are covered by the “all other individually addressed frames”.

However, the commenter does have a point about 8.3.1.1: (463)

This shows the “More Data” field as containing a zero, which it clearly doesn’t in one case.   We can safely remove it,  because 8.2.4.1.8 adequately describes its setting.

Proposed Resolution:

Revised.

At 463.39, remove “(0)” from the “More Data” field.

The relationship between the cited MIB variable and the “More Data Ack subfield” of the “QoS Capability element” is clear, and needs no further clarification.

The operation of the More Data field is specified in 8.2.4.1.8, and this specification includes Ack and other individually addressed frames.  However Figure 8-32 conflicts with the description in 8.2.4.1.8.   This conflict is resolved by the edit above.

| 1648 | 2448.00 | L | "including line breaks" -- what's a "line break"? CR? LF? CRLF? EBCDIC NL? | Clarify | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

The commenter does not indicate an issue to resolve or a specific change to make.

The interpretation of “line break” is determined by looking at the octet listing of the message.  In this case, it is represented by the value 0x0A.

| 1110 | 2614.23 | X.2.4 | The OBSS solution seems to ignore the possibility of an AP supporting multiple BSS. I realize that the OBSS solution is primarily aimed at domestic APs and that multiple BSSs are more common in the enterprise than the domestic market, but there are many examples of companies providing multi-BSS APs. For example to provide a BSS for the private home network and a BSS to provide internet access to other subscribers of the service provider. | Change the calculation of Allocated Traffic Self to be the allocated traffic of all BSSs being serviced by the AP. | GEN |
| --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Rejected.

An AP supports a single BSS.   The multiple BSS mechanism allows a device that contains multiple APs to optimize its beaconing overhead.  Architecturally each of those APs otherwise operates independently.

The commenter has not provided any analysis of the effect on a network of overlapping BSSs of making this change. Such effects would need to be considered before making any change.

# GEN comments not resolved here

There follows a listing of (15) unassigned GEN comments that are not covered by the above proposed resolutions.  I added a reason why I did not attempt to provide a resolution.

| CID | Page | Clause | Comment | Proposed Change | Reason |
| --- | --- | --- | --- | --- | --- |
| 1696 |  |  | It is extremely unlikely for DSSS to be successful communicating large frames at 1 or 2 Mbps in today's 2.4 GHz band. The clause 17, 18 and 19 aMPDUMaxLength is 4095 octets. The maximum lengths in Clause 16 DSSS should be announced to be lowered now and the previous 8191 octet limit is obsolete, and may be removed in a later revision of the standard. Generally, the DSSS PHY maximum frame length should be reduced to 2304 octets from 8191 octets. On a sidenote, EN 300 328 2.4 GHz operation in Europe is being revised, and the maximum time on the air is being considered in calculating the Time X TxPower limit. A 2304 octet limit would give a higher TxPower than an 8191 octet limit would. | Change Tables 16-1 and 16-2 LENGTH to be 0 to 2304. In 16.4.3 DS PHY characteristics table, change aMPDUMaxLength to be 2304. | PHY expertise needed |
| 1651 |  |  | Should be some restriction on devices using LDPC to initiate TXOPs, as otherwise NAV at third parties not supporting LDPC will not be set. Maybe ditto STBC? | As it says | Coex expertise needed |
| 1646 |  |  | Can't the aAirPropagationTime be set in a Coverage Class field of a Country element in an IBSS or MBSS? More generally, some things are only specified in terms of when you're associated to an AP, but they should be more general | As it says | Coex expertise needed. No clear problem specified. |
| 1620 |  |  | Don't all the PHYs need the same fixes as we did in 18.3.10.6? | As it says | No clear problem stated. Needs a lot of work to determine what the fixes were and which are relevant elsewhere. |
| 1591 |  |  | Fine timing measurement stuff was based on the original timing measurement stuff, extended and adapted for the purpose, but some issues with the original stuff were caught and fixed at the same time. These should be fixed in the original stuff too | Fix, based on the document which showed how the fine stuff was derived from the original stuff | Needs a separate submission. Problem statement not clear (“some issues … were caught”) |
| 1561 |  |  | The "PTKSA/GTKSA/STKSA replay counters" stuff is rather opaque, and doesn't seem to operate the way, um, similar mechanisms work | If it means that the transmitter has to stop using new priorities when it's ever transmitted at least one frame at different priorities at the maximum number of replay counters per SA, then say so more clearly | Problem statement not clear. Security expertise. |
| 1558 |  |  | Introduce the term PPDU Transmission Options | As it says | A lot of work. Might consider this (or something not quite so comprehensive) later during 9.7 re-organization. |
| 1546 |  |  | Implicit multiplication symbols are a bad idea if in other similar places explicit symbols are used | Add a multiplication symbol (?) at 1675.40 and 1766.19 | Needs PHY expertise. |
| 1478 |  |  | Slurp out all words starting with "dot11" in 802.11-2011 in all Clauses/Annexes other than Annex C. Slurp out all words following "SYNTAX" in Annex C. Compare | If anything is in one list but not the other, address the discrepancy (in some cases there may be good reasons for the discrepancy, and the simple recipe given in the comment will certainly produce false alerts) | It’s a lot of work. |
| 1444 |  |  | Colons in MAC addresses and suchlike imply bit-reversed notation, which is definitely Not The Done Thing anymore | Change the colons to hyphens in e.g. 10.23.2.5, 10.24.3.2.10, M.10 (note the OO-UU-II:suite_type notation is probably acceptable) | Not adequately plugged in to the “Done Thing”, to which my wife and daughters would attest. |

| 1085 |  |  | New York Times: ""BERLIN -- Google, under pressure from privacy regulators in the Netherlands, said Tuesday that it had agreed to give people around the world the option of keeping the names and locations of their home or business Wi-Fi routers out of a company database. ... Under the agreement, which was announced by Google and the Dutch Data Protection Authority, owners of Wi-Fi routers can add "_nomap" to the end of a router's name to tell Google that they do not want its information included." This chews up 6 octets and has other disadvantages. Ultimately 802.11 should control the semantics for frames/elements/fields defined by 802.11. | Define a means for APs (and STAs) to opt-out of their data being used in an identifiable way. Actually this is a complicated topic - see another comment by the same commenter | We (802.11) have considered and rejected this issue before at least twice, and I’m not minded to do any work on it. |
| --- | --- | --- | --- | --- | --- |
| 1524 | 28.00 | 3.2 | The ERP-CCK, ERP-DSSS, ERP-DSSS/CCK definitions are imprecise | Change to specific subclause refs | I think we covered this in the May session, but can’t find which comment relates. |