IEEE P802.11
Wireless LANs

| 802.11 MIB addition in support of 3GPP manageability |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-01-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian STEPHENS | Intel Corporation |  |  | Adrian.p.stephens@intel.com |
| Joey CHOU | Intel Corporation |  |  | Joey.chou@intel.com |
| CHEN Yanming | China Mobile |  |  | chenyanming@cmdi.chinamobile.com |

Abstract

This submission proposes two additional MIB variables intended to provide additional support for higher layer management entities.

R1 – updated to incorporate additional MIB objects to meet the objectives of the China Mobile presentation 11-13/0692r1.

R2 – Reviewed 2013-09-17 in TGmc.   Minor bugfixes.

R3 – Some changes identified related to bugs in R2.

# R3 changes

After R2 was incorporated into REVmc D2,  some errors were observed after the close of LB199.

This revision fixes those errors.   The changes are shown with this markup.

# Introduction

Offloading from cellular technology to 802.11 is an important goal for cellular operators.  Selection of an AP for offloading (i.e., provisioning a mobile terminal to a specific 802.11 AP) is a management process well above the scope of 802.11.  However, making a good selection requires access to metrics that describe the load/performance at the AP, and these metrics are in scope of 802.11.

We believe that the existing 802.11 BSS Load metrics, new transmitted/received octet counters and new (re-)association success/fail counters are appropriate to the selection process.   The BSS Load metrics are already well defined.   Of the three measurements in BSS Load element, only dot11AssociatedStationCount is currently defined in the MIB. Therefore, we propose to extend the MIB with two new objects to convey the currently unreported fields of the BSS load information to a higher layer management entitity.

The transmitted/received octet counters are new metrics that mirror metrics used in cellular technology today for this purpose.  Note that they are measured at the “top of the MAC” – i.e. in terms of MSDUs, and are, therefore, based on successful transmission or reception.  There already exist octet counters in the 802.11 MIB, but they are specific to A-MPDU and A-MSDU and are therefore unsuitable for a general “top of the MAC” metric.

The (re-)association attempt and fail counters are new metrics that allow the probability of successful (re-) association to be determined.

# The proposed changes

The proposed changes:

- provide access to fields defined in the BSS Load element

- add counters for transmitted and received octets

- add counters for (re-)association attempts and failures

# The edits

Make changes to dot11mac as follows:

dot11mac OBJECT IDENTIFIER ::= { ieee802dot11 2 }

-- MAC GROUPS

-- dot11OperationTable ::= { dot11mac 1 }

-- dot11CountersTable ::= { dot11mac 2 }

-- dot11GroupAddressesTable ::= { dot11mac 3 }

-- dot11EDCATable ::= { dot11mac 4 }

-- dot11QAPEDCATable ::= { dot11mac 5 }

-- dot11QosCountersTable ::= { dot11mac 6 }

-- dot11ResourceInfoTable ::= { dot11mac 7 }

-- dot11DMGOperationTable ::= { dot11mac 8 }

-- dot11DMGCountersTable ::= { dot11mac 9 }

--dot11BSSStatisticsTable ::= { dot11mac <ANA> }

Insert the following two new counters in sequence, assigning the next available numbers:

dot11TransmittedMSDUOctetsCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when an MSDU is successfully transmitted.

This counter is incremented by the number of octets in the MSDU when an MSDU is successfully transmitted."

::= { dot11CountersEntry <next number> }

dot11ReceivedMSDUOctetsCount OBJECT-TYPE

SYNTAX Counter64

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MAC when an MSDU is correctly received that is addressed to the STA

(either individually or globally).

This counter is incremented by the number of octets in the MSDU when an MSDU is correctly received. "

::= { dot11CountersEntry <next number> }

Insert the following table in sequence:

-- *******************************************************************

-- * dot11BSSStatisticsTable TABLE

-- *******************************************************************

dot11BSSStatisticsTable OBJECT-TYPE

SYNTAX SEQUENCE OF Dot11BSSStatisticsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This is a Table management object.

The dot11BSSStatisticsTable contains objects related to BSS Statistics. It is present only in an AP."

::= { dot11mac <ANA> }

dot11BSSStatisticsEntry OBJECT-TYPE

SYNTAX Dot11BSSStatisticsEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"This is an entry in the dot11BSSStatisticsTable Table.

ifIndex - Each IEEE 802.11 interface is represented by an

ifEntry. Interface tables in this MIB module are indexed

by ifIndex."

INDEX { ifIndex }

::= { dot11BSSStatisticsTable 1 }

Dot11BSSStatisticsEntry ::=

SEQUENCE {

dot11BSSLoadChannelUtilization 		Unsigned32,

dot11BSSLoadAvailableAdmissionCapacity	Unsigned32,

dot11BSSLoadAssocAttemptCount		Unsigned32,

dot11BSSLoadAssocFailCount			Unsigned32,

dot11BSSLoadReassocAttemptCount		Unsigned32,

dot11BSSLoadReassocFailCount		Unsigned32

}

dot11BSSLoadChannelUtilization OBJECT-TYPE

SYNTAX Unsigned32 (0..255)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object indicates the percentage of time, linearly scaled with

255 representing 100%, that the AP sensed the medium was busy, as indicated by either

the physical or virtual carrier sense (CS) mechanism. "

REFERENCE "IEEE Std 802.11-<year>, 8.4.2.27"

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 1 }

dot11BSSLoadAvailableAdmissionCapacity OBJECT-TYPE

SYNTAX Unsigned32 (0..65535)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object indicates the remaining amount of medium time available via explicit

admission control, in units of 32 μs/s."

REFERENCE "IEEE Std 802.11-<year>, 8.4.2.27"

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 2 }

dot11BSSLoadAssocAttemptCount OBJECT-TYPE

SYNTAX Unsigned32 (0.. 4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of association procedures started at the AP.  It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request)."

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 3 }

dot11BSSLoadAssocFailCount OBJECT-TYPE

SYNTAX Unsigned32 (0.. 4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of unsuccessful association procedures at the AP (i.e., it is the number of association procedures started less those that completed successfully).  It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request).

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 4 }

dot11BSSLoadReassocAttemptCount OBJECT-TYPE

SYNTAX Unsigned32 (0.. 4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of reassociation procedures started at the AP.  It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request)."

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 5 }

dot11BSSLoadReassocFailCount OBJECT-TYPE

SYNTAX Unsigned32 (0.. 4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of unsuccessful reassociation procedures at the AP (i.e., it is the number of reassociation procedures started less those that completed successfully).  It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request).

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 6 }

Deprecate dot11CountersGroup3 as follows:

dot11CountersGroup3 OBJECT-GROUP

OBJECTS {

<object list omitted here>

}

STATUS deprecated

DESCRIPTION

"Superseded by dot11CountersGroup4.

Attributes from the dot11CountersGroup that are not described in the

dot11MACStatistics group. These objects are mandatory."

::= { dot11Groups 46 }

Insert a new group, dot11CountersGroup4, in sequence, copying the oject list from dot11CountersGroup3 and inserting two new objects:

dot11CountersGroup4 OBJECT-GROUP

OBJECTS {

<object list from dot11CountersGroup3 inserted here>,

dot11TransmittedMSDUOctetsCount,

dot11ReceivedMSDUOctetsCount

}

STATUS current

DESCRIPTION

" Attributes from the dot11CountersGroup that are not described in the

dot11MACStatistics group. These objects are mandatory."

::= { dot11Groups <ANA> }

Insert the following new object group:

dot11BSSStatisticsGroup OBJECT-GROUP

OBJECTS {

dot11AssociatedStationCount,

dot11BSSLoadChannelUtilization,

dot11BSSLoadAvailableAdmissionCapacity,

dot11BSSLoadAssocAttemptCount,

dot11BSSLoadAssocFailCount,

dot11BSSLoadReassocAttemptCount,

dot11BSSLoadReassocFailCount

}

STATUS current

DESCRIPTION

"The BSS Statistics group defines those objects that provide access to local measurements by an

AP of its BSS."

::= { dot11Groups <ANA> }

Change the dot11Compliance module compliance as shown:

-- **********************************************************************

-- * Compliance Statements

-- **********************************************************************

dot11Compliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the IEEE

Std 802.11 MIB."

MODULE -- this module

MANDATORY-GROUPS {

dot11SMTbase12,

dot11MACbase3,

dot11CountersGroup34,

dot11SmtAuthenticationAlgorithms,

dot11ResourceTypeID,

dot11PhyOperationComplianceGroup2 }

GROUP dot11PhyDSSSComplianceGroup

DESCRIPTION

"Implementation of this group is required when object dot11PHYType is

dsss.

This group is mutually exclusive to the following groups:

dot11PhyOFDMComplianceGroup3

dot11PhyHRDSSSComplianceGroup

dot11PhyERPComplianceGroup

dot11PhyHTComplianceGroup

dot11DMGComplianceGroup"

. . .

GROUP dot11BSSStatisticsGroup

DESCRIPTION

“Implementation of this group is optional by an AP”

-- OPTIONAL-GROUPS {

-- dot11SMTprivacy,

. . .

-- dot11WNMCompliance,

-- dot11BSSStatisticsGroup }