IEEE P802.11
Wireless LANs

| Comment resolutions on CID 1003 in Draft P802.11ai_D0.5 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jeongki Kim | LG Electronics |  |  | jeongki.kim@lge.com |
| Giwon Park | LG Electronics |  |  |  |
| HanGyu Cho | LG Electronics |  |  |  |

Abstract

This document contains comment resolutions on CIDs - 1003 for Draft P802.11ai_D0.5

Discussion and resolution on CID 1003

| CID | Page | Line | Clause | Comment | Proposed Change |
| --- | --- | --- | --- | --- | --- |
| 1003 | 66 | 12 | 10.1.4.3.9 | "The AP with dot11FILSActivated equals to true may communicate the association between the AP CCC value and the AP Configuration Information Set by including an AP Configuration Change Count element (as defined in Section 8.4.2.185)". The AP may communicates with whom? | Clarify |

Discussion:

The AP communicates the association between the AP CCC value and the AP Configuration Information Set with the STA with dot11FILSActivated equal to true. For more clarification, the related text is added into the related sentence as proposed remedy. And, wrong reference number is correct from 8.4.2.185 to 8.4.2.184.

Proposed Resolution:

CID 1003: REVISED

Remedy : Modify the first paragraph of  Clause 10.1.4.3.9 as follows:

10.1.4.3.9 FILS active scanning procedure to preferred AP

A non-AP STA with dot11FILSActivated equals to true may retain one or multiple AP Configuration Information Sets, one for each preferred APs which the STA previously obtained. An AP Configuration Information Set is all information fields and information elements of the Beacon frame or the Probe Response frame that include AP CCC element, excluding the following dynamic information fields and elements.

Remedy : Modify the related texts in Clause 10.1.4.3.9 , on page 66, in line 12 as follows:

The AP with dot11FILSActivated equals to true may communicate with the STA with dot11FILSActivated equal to true the association between the AP CCC value and the AP Configuration Information Set by including an AP Configuration Change Count element (as defined in Section 8.4.2.1845) and the complete set of information fields and elements within the AP Configuration Information Set in a Beacon frame, a Probe Response frame that is sent with a broadcast receiver address (RA) or that is sent in response to a Probe Request frame that doesn't contain an AP Configuration Change Count element.