IEEE P802.11
Wireless LANs

| CC08 Proposed Resolutions for CID1082, CID1433 and CID1434 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | Allied Telesis R&D Center | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 JAPAN | +81 92 771 7630 | hmorioka@root-hq.com |

Abstract

This document is a proposed resolution normative text for assigned CIDs of CC08.

Resolved CIDs are 1082, 1433 and 1434.

# 3. Definitions, acronyms, and abbreviations

# 3.1 Definitions

Insert the following new definitions.

fast initial link setup (FILS): Collection of the functions of FILS authentication, FILS discovery and FILS secure container for higher layer information to minimize link setup time. [CID1082]

# 8. Frame formats

## 8.4 Management frame body components

### 8.4.2 Information elements

# Change clause 8.4.2.186 as following:

### 8.4.2.186 Higher Layer Encapsulation FILS Secure Container[CID1086] element

### 8.4.2.186.1 Higher Layer Encapsulation TLVs [CID1108]

FILS Secure Container element includes one or multiple FILS Secure Container TLV(s). FILS Secure Container TLVs are shown in Table 8-183ah. The format of the FILS Secure Container element is shown in Figure 8-c. [CID1433]

Higher Layer Encapsulation TLVs are used to carry out various Higher Layer functions such as IP address assignment in the Higher Layer Encapsulation Information Element.

FILS Secure Container TLVs are used to carry out various purpose such as IP address assignment and GTK transfer.[CID1086]

Table 8-183ah – Higher Layer Encapsulation FILS Secure Container[CID1086] TLV

| Type | TLV ID | Length (octets) | Extensible |
| --- | --- | --- | --- |
| FILS HLP Wrapped data | 1 | TBD variable but limited by MPDU[CID1297] | No |
| FILS IP Address Request | 2 | 3 4 to 255 | No |
| FILS IP Address Assignment | 3 | 3 4 to 255 | No |
| FILS DNS Information | 4 | 3 4 to 255 | No |
| Key RSC | <ANA> | 19 | No |
| GTK Transfer | <ANA> | 4 to 255 | No |

|  | Element ID | Length | FILS Secure Container TLV(s) |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure 8-c – FILS Secure Container element format [CID1433]

### 8.4.2.186.1.1[CID1108, CID1188] FILS HLP wrapped data TLV

The FILS wrapped data element contains higher layer packets transported during association. One or more FILS HLP wrapped data TLVs may be included in an Association Request, an Reassociation Request or an Association Response frames if dot11FILSActivated set to true. One or more FILS HLP wrapped data TLVs may be included in an Association Request, a Reassociation Request, an Association Response, or a Reassociation Response frames if dot11FILSActivated is true. [CID1318]

|  | TLV ID | Length | Flags | Destination MAC Address |  |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 6 |  |
|  | Source MAC Address |  |  | LLC/SNAP | HLP |
| Octets: | 6 |  |  | variable | variable |

|  | TLV ID | Length | Destination MAC Address |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 |  |
|  | Source MAC Address |  | LLC/SNAP | HLP |
| Octets: | 6 |  | variable | variable |

[CID1435]

Figure 8.401dh – FILS HLP Wrapped TLV format

The Element ID field is equal to the FILS HLP Wrapped data value in Table 8-4.ai2-1183ah[CID1015, CID1191, CID1319].

The value of the Length field is 12 plus the length of data after Source MAC Address including LLC/SNAP and HLP

The value of Destination MAC Address field is the destination MAC address of the HLP.

The value of Source MAC Address field is the source MAC address of the HLP.

The value of LLC/SNAP field is the LLC header and SNAP header (if applicable) of the HLP.  LLC is defined in IEEE802.2. If the LLC field is equal to 0xaa 0xaa, 5 octets SNAP header is added. Refer IEEE802.2 and clause 4.2.5 in IEEE802.11-2012. [CID1434]

The HLP field contains the HLP.