IEEE P802.11
Wireless LANs

| TGah D0.1 Comment Resolutions on MIMO Control Field |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Hongyuan Zhang | Marvell | 5488 Marvell Ln, Santa Clara, CA 95054 | 408-222-1837 | hongyuan@marvell.com |

Abstract: D0.1 Comment Resolutions for Clauses 8.4.1.47 MIMO Control Field

| 359 | Matthew Fischer | 8.4.1.47 | 55.44 | The subclause should be renamed to indicate tha the S1G MIMO Control field is also defined in this subclause - readers might spend weeks looking for that field | change the subclause name as suggested - there might be some other subclauses that have a similar problem | Revise, It is expected that 11ac will be finalized soon, therefore it is ok to cite 11ac clauses and indicate the differences, as in 11af amendment. Make this as a subclause of 8.4.1.47, same applies for the next subclause on VHT compressed beamforming report field |
| --- | --- | --- | --- | --- | --- | --- |
| 106 | Anna Pantelidou | 8.4.1.47 | 55.65 | It is not clear which are the values of phi and psi that are set to 0 and 1 in the VHT MIMO Control Field | Clarify | Reject, The commentor does not indicate how to clarify, the current text is the same as 11ac D5.0 Table 8-53c Codebook info field. |
| 830 | Simone Merlin | 8.4.1.47 | 55.44 | Use of the MIMO Control field need be clarid for S1G | Clarify the use of Nss. | Reject, The commenter does not indicate how to clarify, there is no Nss field in MIMO control field. If the commenter meant for Nc field, it was already well explained in the draft. |

TGah editor: please make the following changes on page 55~56:

8.4.1.47 VHT MIMO Control Field

Insert the following paragraph subclause at the end of sub-clause 8.4.1.47:

8.4.1.47.1 VHT MIMO Control Field used in S1G Band

For S1G band, the same VHT MIMO control field is applied in the sounding feedback frame, with “VHT”

replaced by “S1G”, and with the following exceptions.

…

…

8.4.1.48 VHT Compressed Beamforming Report field

Insert the following paragraphs subclause at the end of sub-clause 8.4.1.48:

8.4.1.48.1 VHT Compressed Beamforming Report field used in S1G Band

For S1G band, the same VHT Compressed Beamforming Report field is applied in the sounding feedback

frame, with “VHT” replaced by “S1G”, and with the following exceptions: