IEEE P802.11
Wireless LANs

| CC9 clause 10.1.4.3.4 and 8.3.4a.2.1 comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics |  | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission proposes comment resolutions of the following CIDs from TGah Draft 0.1 Comment Collection 9.

- 763, 921, 902, 105, 901

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGah Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGah Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGah Editor: Editing instructions preceded by “TGah Editor” are instructions to the TGah editor to modify existing material in the TGah draft.  As a result of adopting the changes, the TGah editor will execute the instructions rather than copy them to the TGah Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 763 | 168.00 | 10.1.4.3.4 | Since the quick probing has to be a topic over several TGs, make sure single solution for identical problem. | Make it sure that NDP probing has to be unique solution for energy minimization as well as quick scan. | Rejected- The purpose of NDP probing is to reduce a scanning power consumption by minimizing a MAC overhead of a Probe Request management frame. The NDP probing is an unique solution of TGah. |
| 921 | 168.00 | 10.1.4.3.4 | No action is defined for the case that a STA receives other than (short) Probe Response frame before the Probe Timer reaches MinChannel Time. | Modify the sentence from "If PHY-CCA.indication (busy) primitive has not been detected before the ProbeTimer reaches MinChannelTime," to "If a STA doesnot receive (short) Probe Response frame before the ProbeTimer reaches MinChannelTime, ...". | Rejected- After transmitting a NDP Probe Request frame, a STA can receives other frame (including a Beacon) transmitted from an AP. In that case, STA shall not move the scan channel. |

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 902 | 53.00 | 8.3.4a.2.1 | As frame type for BACK is 6, it should be 7. | Modify the sentece from "NDP MAC Frame Type field is set to 6." to "NDP MAC Frame Type field is set to 7.". | Accepted – Modify the sentece from "NDP MAC Frame Type field is set to 6." to "NDP MAC Frame Type field is set to 7." in Table 8-33q and Table 8-33r. |
| 105 | 53.00 | 8.3.4a.2.1 | The valie of the Frame Type of an NDP Probe Request is equal to 6 and is the same as the value given to the Frame Type of of an NDP Paging frame. | Correct this | Revised – Modify the sentece from "NDP MAC Frame Type field is set to 6." to "NDP MAC Frame Type field is set to 7." in Table 8-33q and Table 8-33r. |
| 901 | 53.00 | 8.3.4a.2.1 | As frame type for BACK is 6, it should be 7. | Modify the sentece from "NDP MAC Frame Type field is set to 6." to "NDP MAC Frame Type field is set to 7.". | Accepted – Modify the sentece from "NDP MAC Frame Type field is set to 6." to "NDP MAC Frame Type field is set to 7." in Table 8-33q and Table 8-33r. |