IEEE P802.11
Wireless LANs

| CC9 cluase 9.19.4a.5 comment resolution |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Yongho Seok | LG Electronics |  | +82-31-450-1947 | yongho.seok@lge.com |

Abstract

This submission proposes comment resolutions of the following CIDs from TGah Draft 0.1 Comment Collection 9.

- 599, 310, 311, 367

Interpretation of a Motion to Adopt

A motion to approve this submission means that the editing instructions and any changed or added material are actioned in the TGah Draft.  This introduction is not part of the adopted material.

Editing instructions formatted like this are intended to be copied into the TGah Draft (i.e. they are instructions to the 802.11 editor on how to merge the text with the baseline documents).

TGah Editor: Editing instructions preceded by “TGah Editor” are instructions to the TGah editor to modify existing material in the TGah draft.  As a result of adopting the changes, the TGah editor will execute the instructions rather than copy them to the TGah Draft.

| CID | Page | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 599 | 133.00 | 9.19.4a.5 | EDCA backoff procedure in RAW is only applicable to QoS devices. | Modify the title of sub-clause 9.19.4a.5 (Page 133, line 15 - 16) as the following:9.19.4a.5 EDCA backoff procedure in RAW for QoS STAs | Rejected – Only QoS STA follows the EDCA procedure. EDCA already is implying that the proposed protocol is applied only to QoS STA. |
| 310 | 133.00 | 9.19.4a.5 | What is backoff function state? |  | Reject – Clause 9.20.2.3 is saying the backoff function, “Each channel access timer shall maintain a backoff function (timer), which has a value measured in backoff slots.” |
| 311 | 133.00 | 9.19.4a.5 | What happens if it is not RAW anymore after its slot when Cross Slot Boundary is allowed? |  | Rejected – If it is a last slot of a RAW, the stored First Backoff State is restored and the backoff function resumes. |
| 367 | 133.00 | 9.19.4a.5 | Applies to all cases - both when participating in a RAW and when NOT participating. Specificity is missing. | Change "When a STA is allowed to contend in the RAW, the STA suspends its backoff at the start of a RAW" to "Each STA suspends its backoff at the start of each RAW and stores the backoff state as First Backoff State" also change "And, at the end of RAW, the previously stored backoff function state is restored and the backoff function resumes" to "At the end of the RAW, the stored First Backoff State is restored and the backoff function resumes" | Accepted – TGah editor to make the changes proposed by the commenter. |