IEEE P802.11
Wireless LANs

| Proposed Resolution of 802.11 WG Ballot 193, Comment 1050 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-07-xx |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission proposes a resolution to LB193 comment 1050, relating to improving the duplicate detection cache description in REVmc D1.0.

| CID | Page | Clause | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 1050 | 936.10 | 9.3.2.10 | This subclause at excels in long-winded paragraphs (see 937.40 for an exemplar of this black art). | Subclause should be restructured to show a table of all caches, identifying each as belonging to a type of cache,and the conditions under which they are required, with some common text per cache type describing the operation of a generic cache. Should also describe a table of sequence numbers maintained at the transmitter, which could be a separate subclause. |  | MAC |

Discussion:

Except where noted,  the replacement text is a restructuring of the existing material in 9.3.2.10.

Proposed Resolution:

Revised. Replace 9.3.2.10 with the replacement text in <this-document>.

# Proposed Replacement Text.

      - Duplicate detection and recovery

9.3.2.10.1 General

Because MAC-level acknowledgments and retransmissions are incorporated into the protocol, there is the possibility that a frame might be received more than once. The procedures defined in this subclause attempt to filter out these duplicates. Additional duplicate filtering is performed during Receive Buffer Operation for frames that are part of a Block Ack agreement as described in Error: Reference source not found and Error: Reference source not found.

Duplicate frame filtering is facilitated through the inclusion of a Sequence Control field (consisting of a sequence number and fragment number) within Data, (#100)Management, and Extension(11ad)(Ed) frames, a(11ae) TID subfield in the QoS Control field within QoS (#100)Data frames, and an ACI subfield in the Sequence Number field within QMFs.(11ae)

9.3.2.10.2 Transmitter Requirements

A STA maintains one or more sequence number spaces that are used when transmitting a frame to determine the sequence number for the frame. When multiple sequence number spaces are supported, the appropriate sequence number space is determined by information from the MAC control fields of the frame to be transmitted. Except as noted below, each sequence number space is represented by a modulo-4096 counter, starting at 0 and incrementing by 1, for each MSDU or MMPDU transmitted using that sequence number space.

NOTE 1—Group addressed retransmissions of BUs use the same sequence number as the initial group addressed transmission of the BU. Unicast retransmissions of a group addressed BU delivered via DMS use the same sequence number as the initial unicast transmission of the BU. When a BU is delivered both using group addressing and unicast (e.g., when DMS is active but there are other associated STAs not using DMS), the sequence number might differ between the group addressed and unicast transmissions of the same BU.(11aa)

MPDUs that are part of the same MSDU or A-MSDU shall have the same sequence number. Different MSDUs or A-MSDUs have (with a high probability) a different sequence number.

A transmitting STA shall support the applicable sequence number spaces defined in Tables 9-x.  Applicability is defined by the “applies to” column.  The “Status” column indicates what support is required if “Applies to” matches the transmission.  The “Multiplicity” column indicates whether the sequence number space contains a single counter, or multiple counters and in the latter case identifies any indexes. The “Transmitter requirements” column identifies requirements for the operation of this sequence number space.  The referenced requirements are defined at the end of the table.

Table 9-x Transmitter Requirements for Sequence Numbering

| Sequence number space | Applies to | Status | Multiplicity | Transmitter requirements |
| --- | --- | --- | --- | --- |
| Non-QoS | A non-QoS STA or QoS STA operating as a non-QoS STA because it is in a non-QoS BSS or non-QoS IBSS transmitting a Management or Data frame (QoS subfield of the Subtype field is equal to 0) | Mandatory | Single Instance |  |
| Non-QoS Management | Non-QoS STA or QoS STA operating as a non-QoS STA because it is in a non-QoS BSS or non-QoS IBSS transmitting a Management frame | Unclear | Single Instance | TR1 TR2 |
| Individually addressed QoS Data | A STA operating as a QoS STA |  | Indexed by <Address 1, TID> |  |
| QoS Non-QMF Other | A QoS STA in which dot11QMFActivated false or not present transmitting a Management frame, Extension frame (when this field is present) or group addressed QoS Data frame | Mandatory | Single Instance | TR1 TR2 |
| Time Priority Management | A QoS STA transmitting a Time Priority Management frame | Optional | Indexed by <Address 1 TID> |  |
| Non-QMF frames from QMF STA | A QMF STA transmitting: - Management and Extension(11ad) frames that are not QMFs - Group addressed QoS (#100)Data frames - All non-QoS (#100)Data frames | Mandatory | Single Instance |  |
| QMF | A QMF STA transmitting a QMF |  | Indexed by <Address 1, AC> |  |
| QoS Null | A STA transmitting a QoS Null frame |  | None | TR4 |
| TR1: A transmitting STA should cache the last used sequence number per RA for frames that are assigned sequence numbers from this sequence number space. TR2: The STA should check that the successively assigned sequence numbers for frames transmitted to a single RA do not have the same value as is found in the cache for that RA. If the check fails the STA should increment the counter by 2, rather than 1. TR3: The STA shall assign the sequence number from one modulo-1024 counter per <Address 1, AC> tuple starting at 0 and incrementing by 1 for each MMPDU carried in one or more QMFs with Address 1 and ACI fields matching the <Address 1, AC> tuple values corresponding to that counter. TR4: Sequence numbers for transmitted QoS (+)Null frames may be set to any value. |  |  |  |  |

9.3.2.10.3 Receiver Requirements

A STA maintains one or more duplicate detection caches, as determined by Table 9-y. When a Data, Management or Extension frame is received, a record of that frame is inserted in an appropriate cache.  That record is identified by a sequence number and possibly other information from the MAC control fields of the frame.  When a Data, Management or Extension frame is received in which the Retry subfield of the Frame Control field is equal to 1, the appropriate cache is searched for a matching frame.  If the search is successful, the frame is considered to be a duplicate.  Duplicate frames are discarded.

NOTE 2—The receiver STA performs the (#1198)Ack and (for an AP) PS procedures on all successfully received frames requiring acknowledgment, even if the frame is discarded due to duplicate filtering.

There is a small possibility that a frame may be improperly rejected due to such a match; however, this occurrence is rare and simply results in a lost frame (similar to an FCS error in other LAN protocols).

A receiving STA shall implement the applicable receiver requirements defined in Table 9-y.  Applicability is defined by the “applies to” column.  The “Status” column indicates what support is require if “Applies to” matches the received frame. The “Multiplicity / Cache size” column indicates the indexes that identify a cache entry and the number of entries that shall be supported. The “Receiver requirements” column identifies requirements for the operation of this cache.  The referenced requirements are defined at the end of the table. The rquirements relate to caching information that identifies a cache entry and discarding duplicate MPDUs.

Table 9-y Receiver Requirements for Duplicate Detection

| Cache Name | Applies to | Status | Multiplicity / Cache size | Receiver requirements |
| --- | --- | --- | --- | --- |
| Not QoS Data | A STA receiving frames that are not QoS Data, excluding Management frames when the STA supports the Non-time priority Managment cache. | Mandatory | Indexed by: <Address 2, sequence number, fragment number>. At least the most recent cache entry per <Address 2>. | RR0 RR1 |
| QoS Data | A QoS STA receiving a QoS Data frame | Mandatory | Indexed by: <Address 2, TID, sequence number, fragment number>. At least the most recent cache entry per <Address 2, TID> pair in this cache. | RR0.7 RR1 |
| Non-time priority Management | A STA receiving a non-time priority Management frame | Optional | Indexed by: <Address 2, sequence number, fragment number> At least the most recent cache entry per <Address 2> value. | RR0 RR1 |
| Time priority Management | A STA receiving a time priority Management frame | Supported if Non-time priority Management is supported | Indexed by: <Address 2, sequence number, fragment number>. At least the most recent cache entry per <Address 2> value. | RR1 |
| QMFs | A QMF STA receiving a QMF | Mandatory | Indexed by: <Address 2, AC, sequence number, fragment number> The most recent cache entry per <Address 2, AC, sequence-number, fragment-number>. | RR0.5 RR1 |
| Non-mesh GCR | A non-mesh STA with dot11RobustAVStreamingImplemented true receiving a group addressed frame subject to a GCR agreement. | Mandatory | Indexed by: <DA, sequence number> One cache entry per <DA, sequence-number> tuple for each group address subject to a GCR agreement. | RR1 |
| Mesh GCR | A mesh STA with dot11RobustAVStreamingImplemented true receiving a group addressed frame subject to a GCR agreement. | Mandatory | Indexed by: <DA, Address 2, sequence number>. One cache entry per <DA, Address 2, sequence-number> tuple for each group address subject to a GCR agreement. | RR1 |
| RR0: A receiving STA with dot11QMFActivated false or not present(11ae) and with dot11RobustAVStreamingImplemented false or not present(11aa) should omit tuples obtained group addressed and ATIM frames from all caches. RR0.5: A receiving QMF STA with dot11RobustAVStreamingImplemented false or not present(11aa) shall omit from the caches all tuples obtained from group addressed (#100)Data frames and tuples obtained from ATIM frames.(11ae) RR0.7: For the purposes of duplicate detection, QoS (+)Null frames shall be ignored. RR1: The STA shall discard the frame if the Retry bit in the Frame Control field is 1 and it matches an entry in the cache. |  |  |  |  |