IEEE P802.11
Wireless LANs

| Proposed Resolution of 802.11 WG Ballot 193, Comment 1050 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-04 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This submission proposes a resolution to LB193 comment 1050, relating to improving the duplicate detection cache description in REVmc D1.0.

R1 – Reviewed at 2013-09 session, then updated 2013-10-09 based on discussion with Mark Hamilton.

R2 – for discussion with Matt and MarkH. 2013-10-11 changes.

R3 – Updated during 2013-10-11 TGmc telecon.  Pending issue is how to handle time priority management frames that might also be QMF frames.

R4 – Updated to ensure that time priority management and QMF frames are mutually exclusive.  Restored “should” semantics rather than optional.

| CID | Page | Clause | Comment | Proposed Change | Resolution | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- | --- |
| 1050 | 936.10 | 9.3.2.10 | This subclause at excels in long-winded paragraphs (see 937.40 for an exemplar of this black art). | Subclause should be restructured to show a table of all caches, identifying each as belonging to a type of cache,and the conditions under which they are required, with some common text per cache type describing the operation of a generic cache. Should also describe a table of sequence numbers maintained at the transmitter, which could be a separate subclause. |  | MAC |

Discussion:

Except where noted,  the replacement text is a restructuring of the existing material in 9.3.2.10.

Proposed Resolution:

Revised. Replace 9.3.2.10 with the replacement text in <this-document>.

# Proposed Replacement Text.

      - Duplicate detection and recovery

9.3.2.10.1 General

Because MAC-level acknowledgments and retransmissions are incorporated into the protocol, there is the possibility that a frame might be received more than once. The procedures defined in this subclause attempt to filter out these duplicates. Additional duplicate filtering is performed during Receive Buffer Operation for frames that are part of a Block Ack agreement as described in Error: Reference source not found and Error: Reference source not found.

Duplicate frame filtering is facilitated through the inclusion of a Sequence Control field (consisting of a sequence number and fragment number) within Data, (#100)Management, and Extension(11ad)(Ed) frames, a(11ae) TID subfield in the QoS Control field within QoS (#100)Data frames, and an ACI subfield in the Sequence Number field within QMFs.(11ae)

9.3.2.10.2 Transmitter Requirements

A STA maintains one or more sequence number spaces that are used when transmitting a frame to determine the sequence number for the frame. When multiple sequence number spaces are supported, the appropriate sequence number space is determined by information from the MAC control fields of the frame to be transmitted. Except as noted below, each sequence number space is represented by a modulo-4096 counter, starting at 0 and incrementing by 1, for each MSDU or MMPDU transmitted using that sequence number space.

NOTE 1—Group addressed retransmissions of BUs use the same sequence number as the initial group addressed transmission of the BU. Unicast retransmissions of a group addressed BU delivered via DMS use the same sequence number as the initial unicast transmission of the BU. When a BU is delivered both using group addressing and unicast (e.g., when DMS is active but there are other associated STAs not using DMS), the sequence number might differ between the group addressed and unicast transmissions of the same BU.(11aa)

MPDUs that are part of the same MSDU or A-MSDU shall have the same sequence number. Different MSDUs or A-MSDUs have (with a high probability) a different sequence number.

A transmitting STA shall support the applicable sequence number spaces defined in Table 9-x.  Applicability is defined by the “applies to” column.  The “Status” column indicates the level of support that is required if “Applies to” matches the transmission.  The “Multiplicity” column indicates whether the sequence number space contains a single counter, or multiple counters and in the latter case identifies any indexes. The “Transmitter requirements” column identifies requirements for the operation of this sequence number space.  The referenced requirements are defined at the end of the table.

Table 9-x Transmitter Requirements for Sequence Numbering

| Sequence number space identifier | Sequence number space | Applies to | Status | Multiplicity | Transmitter requirements |
| --- | --- | --- | --- | --- | --- |
| SNS1 | Non-QoS | A non-QoS STA or QoS STA operating as a non-QoS STA because it is in a non-QoS BSS or non-QoS IBSS transmitting a Management or Data frame (QoS subfield of the Subtype field is equal to 0) | Mandatory | Single Instance | TR1 |
| SNS2 | Individually addressed QoS Data | A STA operating as a QoS STA transmitting a QoS Data frame, excluding SNS7 | Mandatory | Indexed by <Address 1, TID> |  |
| SNS3 | QoS Non-QMF Other | A QoS STA in which dot11QMFActivated is false or not present transmitting a Management frame, Extension frame (when this field is present) or group addressed QoS Data frame, excluding SNS7 and excluding SNS4 if SNS4 is supported | Mandatory | Single Instance | TR1 |
| SNS4 | Time Priority Management | A QoS STA transmitting a Time Priority Management frame | Optional | Indexed by <Address 1 TID> |  |
| SNS5 | Non-QMF frames from QMF STA | A QMF STA transmitting: - Management and Extension(11ad) frames that are not QMFs - Group addressed QoS (#100)Data frames, excluding SNS7 - All non-QoS (#100)Data frames Excluding SNS4 if SNS4 is supported | Mandatory | Single Instance | TR1 |
| SNS6 | QMF | A QMF STA transmitting a QMF | Mandatory | Indexed by <Address 1, AC> | TR2 |
| SNS7 | QoS Null | A STA transmitting a QoS Null frame | Mandatory | None | TR3 |
|  | TR1: A transmitting STA should cache the last used sequence number per RA for frames that are assigned sequence numbers from this sequence number space. The STA should check that the successively assigned sequence numbers for frames transmitted to a single RA do not have the same value as is found in the cache for that RA. If the check fails the STA should increment the counter by 2, rather than 1. TR2: The STA shall assign the sequence number from one modulo-1024 counter per <Address 1, AC> tuple starting at 0 and incrementing by 1 for each MMPDU carried in one or more QMFs with Address 1 and ACI fields matching the <Address 1, AC> tuple values corresponding to that counter. TR3: Sequence numbers for transmitted QoS (+)Null frames may be set to any value. |  |  |  |  |

9.3.2.10.3 Receiver Requirements

A STA maintains one or more duplicate detection caches, as determined by Table 9-y. When a Data, Management or Extension frame is received, a record of that frame is inserted in an appropriate cache.  That record is identified by a sequence number and possibly other information from the MAC control fields of the frame.  When a Data, Management or Extension frame is received in which the Retry subfield of the Frame Control field is equal to 1, the appropriate cache is searched for a matching frame.  If the search is successful, the frame is considered to be a duplicate.  Duplicate frames are discarded.

NOTE 1—The receiver STA performs the (#1198)Ack and (for an AP) PS procedures on all successfully received frames requiring acknowledgment, even if the frame is discarded due to duplicate filtering.

There is a small possibility that a frame may be improperly rejected due to such a match; however, this occurrence is rare and simply results in a lost frame (similar to an FCS error in other LAN protocols).

A receiving STA shall implement the applicable receiver requirements defined in Table 9-y with “Status” indicated as “Mandatory”.  A receiving STA should implement the applicable receiver requirements defined in Table 9-y with “Status” indicated as “Recommended”.  A receiving STA may implement the applicable receiver requirements defined in Table 9-y with “Status” indicated as “Optional”. Applicability is defined by the “applies to” column.  The “Status” column indicates the level of support that is required if “Applies to” matches the received frame. The “Multiplicity / Cache size” column indicates the indexes that identify a cache entry and the number of entries that shall be supported. The “Receiver requirements” column identifies requirements for the operation of this cache.  The referenced requirements are defined at the end of the table. The requirements relate to caching information that identifies a cache entry and discarding duplicate MPDUs.

Table 9-y Receiver Requirements for Duplicate Detection

| Receiver Cache Identifier | Cache Name | Applies to | Status | Multiplicity / Cache size | Receiver requirements |
| --- | --- | --- | --- | --- | --- |
| RC1 | Not QoS Data | A STA receiving frames that are not QoS Data, excluding RC4 if RC4 is supported, and excluding RC5 if RC5 is supported, and excluding RC6 if RC6 is supported | Mandatory | Indexed by: <Address 2, sequence number, fragment number>. At least the most recent cache entry per <Address 2>. | RR0 RR1 |
| RC2 | QoS Data | A QoS STA receiving an individually addressed QoS Data frame, excluding RC3 | Mandatory | Indexed by: <Address 2, TID, sequence number, fragment number>. At least the most recent cache entry per <Address 2, TID> pair in this cache. | RR1 |
| RC3 | QoS Null | A QoS STA receiving a QoS Null frame | Mandatory | None | RR0.7 |
| RC4 | Non-time priority Management | A STA receiving an individually addressed non-time priority Management frame, excluding RC6 if RC6 is supported | Recommended | Indexed by: <Address 2, sequence number, fragment number> At least the most recent cache entry per <Address 2> value. | RR0 RR1 |
| RC5 | Time priority Management | A STA receiving an individually addressed time priority Management frame | Supported if RC4 is supported; otherwise not supported | Indexed by: <Address 2, sequence number, fragment number>. At least the most recent cache entry per <Address 2> value. | RR1 |
| RC6 | QMFs | A QMF STA receiving an individually addressed QMF | Mandatory | Indexed by: <Address 2, AC, sequence number, fragment number> The most recent cache entry per <Address 2, AC, sequence-number, fragment-number>. | RR0.5 RR1 |
| RC7 | Non-mesh GCR | A non-mesh STA with dot11RobustAVStreamingImplemented true receiving a group addressed frame subject to a GCR agreement. | Mandatory | Indexed by: <DA, sequence number> One cache entry per <DA, sequence-number> tuple for each group address subject to a GCR agreement. | RR1 |
| RC8 | Mesh GCR | A mesh STA with dot11RobustAVStreamingImplemented true receiving a group addressed frame subject to a GCR agreement. | Mandatory | Indexed by: <DA, Address 2, sequence number>. One cache entry per <DA, Address 2, sequence-number> tuple for each group address subject to a GCR agreement. | RR1 |
| RR0: A receiving STA with dot11QMFActivated false or not present(11ae) and with dot11RobustAVStreamingImplemented false or not present(11aa) should omit tuples obtained from group addressed and ATIM frames from this cache. RR0.5: A receiving QMF STA with dot11RobustAVStreamingImplemented false or not present(11aa) shall omit from the cache all tuples obtained from group addressed (#100)Data frames and tuples obtained from ATIM frames.(11ae) RR0.7: For the purposes of duplicate detection, QoS (+)Null frames shall be ignored. RR1: The STA shall discard the frame if the Retry bit in the Frame Control field is 1 and it matches an entry in the cache. |  |  |  |  |  |

10.26 Quality-of-service management frame (QMF)

10.26.1 General

10.26.1.1 Overview

A QMF STA shall set dot11QMFActivated and dot11QosOptionImplemented to true. The QMF STA shall assign an access category to each Management frame according to the access category assignments indicated in the QMF policy that has been configured using the configuration procedures described in 10.26.2 (QMF policy advertisement and configuration procedures).

A QMF STA shall set the QMFActivated subfield in the Extended Capabilities element to 1.

A Management frame shall be transmitted as an IQMF when all five of the following conditions are met:

— The RA of the Management frame corresponds to an individual MAC address.

— The frame is transmitted by a QMF STA.

— The transmitting STA has previously received an Extended Capabilities element from the STA corresponding to the RA of the frame being transmitted.

— The most recently received such Extended Capabilities element indicated that the STA is a QoS STA

and has the value of 1 in the QMFActivated subfield.

— The frame is not a time priority management frame.

A QMF AP shall transmit a Management frame as a GQMF when all three of the following conditions are

met:

— The RA of the Management frame corresponds to a group MAC address.

— The transmitting STA has received an Extended Capabilities element that has the QMFActivated

subfield equal to 1 from every member of the BSS corresponding to the BSSID of the Management frame.

— The frame is not a time priority management frame.

A non-AP QMF STA shall transmit a Management frame as a GQMF when all three of the following

conditions are met:

— The RA of the Management frame corresponds to a group MAC address.

— The transmitting STA has received an Extended Capabilities element that has the QMFActivated

subfield equal to 1 from its associated AP.

— The frame is not a time priority management frame.

A Management frame shall not be transmitted as an IQMF or GQMF, except as defined above.

## Original text (9.3.2.10, D2.0)

(Tracked changes show changes necessary to recover from editing error in D2.0.  However,  this does not appear to affect the proposal above.)

- Duplicate detection and recovery

Because MAC-level acknowledgments and retransmissions are incorporated into the protocol, there is the possibility that a frame may be received more than once. The procedures defined in this subclause attempt to filter out these duplicates. Additional duplicate filtering is performed during Receive Buffer Operation for frames that are part of a Block Ack agreement as described in Error: Reference source not found and Error: Reference source not found.

Duplicate frame filtering is facilitated through the inclusion of a Sequence Control field (consisting of a sequence number and fragment number) within data, (#100)Management, and Extension(11ad)(Ed) frames, a(11ae) TID subfield in the QoS Control field within QoS (#100)Data frames, and an ACI subfield in the Sequence Number field within QMFs.(11ae) MPDUs that are part of the same MSDU or A-MSDU shall have the same sequence number, and different MSDUs or A-MSDUs have (with a high probability) a different sequence number.

Sequence numbers for transmitted QoS (+)Null frames may be set to any value. For the purposes of duplicate detection, QoS (+)Null frames shall be ignored.(#158)

A non-QoS STA shall assign sequence numbers to (#100)Management frames and (#100)Data frames (QoS subfield of the Subtype field is equal to 0) from a single modulo-4096 counter, starting at 0 and incrementing by 1, for each MSDU or MMPDU. A QoS STA operating as a non-QoS STA because it is in a non-QoS BSS or non-QoS IBSS shall assign sequence numbers to (#100)Management frames and (#100)Data frames (QoS subfield of the Subtype field is equal to 0) from a single modulo-4096 counter, starting at 0 and incrementing by 1, for each MSDU or MMPDU. A transmitting STA should cache the last used sequence number per RA for frames that are assigned sequence numbers from this counter. The STA should check that the successively assigned sequence numbers for frames transmitted to a single RA do not have the same value as is found in the cache for that RA. If the check fails the STA should increment the counter by 2, rather than 1.(#1285)

A STA operating as a QoS STA shall maintain one modulo-4096 counter, per <Address 1, TID> tuple,(11ae) for individually addressed QoS Data frames. Sequence numbers for these frames are assigned using the counter identified by the Address 1 field and the TID subfield of the QoS Control field of the frame, and that counter is incremented by 1 for each MSDU or A-MSDU corresponding to that <Address 1, TID> tuple. Sequence numbers for (#100)Management frames, Extension frames (when this field is present),(11ad) QoS (#100)Data frames with a group address in the Address 1 field, and all non-QoS (#100)Data frames transmitted by QoS STAs with dot11QMFActivated false or not present(11ae) shall be assigned using an additional single modulo-4096 counter, starting at 0 and incrementing by 1 for each such MSDU, A-MSDU, or MMPDU, except that a QoS STA may use values from additional modulo-4096 counters per <Address 1, TID> for sequence numbers assigned to time priority (#100)Management frames. A transmitting STA should cache the last used sequence number per RA for frames that are assigned sequence numbers from the additional single modulo-4096 counter for management frames(#215).. The STA should check that the successively assigned sequence numbers for frames transmitted to a single RA do not have the same value as is found in the cache for that RA. If the check fails the STA should increment the counter by 2, rather than 1.(#1285)

When transmitted by a QMF STA, the STA shall assign sequence numbers for the frames listed below using a modulo-4096 counter, starting at 0 and incrementing by 1 for each such MSDU or MMPDU:(11ae)

- Group addressed QoS (#100)Data frames

- All non-QoS (#100)Data frames

and shall assign the sequence number for QMFs from one modulo-1024 counter per <Address 1, AC> tuple starting at 0 and incrementing by 1 for each MMPDU carried in one or more QMFs with Address 1 and ACI fields matching the <Address 1, AC> tuple values corresponding to that counter.(11ae)(#158)

A receiving STA shall keep a cache of recently received <Address 2, sequence-number, fragment-number> tuples from frames that are not QoS Data frames. The receiving STA shall keep at least the most recent cache entry per <Address 2> value in this cache. A(11ae) receiving QoS STA shall also keep a cache of recently received <Address 2, TID, sequence-number, fragment-number> tuples from QoS Data frames for(11aa) all STAs from which it has received QoS (#100)Data frames. The receiving QoS STA shall keep at least the most recent cache entry per <Address 2, TID> pair in this cache. The receiving STA should maintain two additional caches, one containing entries of recently received <Address 2, sequence-number, fragment-number> tuples from received (#100)Management frames that are not time priority management frames and the other containing entries of recently received <Address 2, sequence-number, fragment-number> tuples from received time priority management frames. The receiving STA should not include the entries in these two additional caches in any other caches. In each of these two caches, the receiving STA should keep at least the most recent cache entry per <Address 2> value. A receiving STA with dot11QMFActivated false or not present(11ae) and with dot11RobustAVStreamingImplemented false or not present(11aa) should omit tuples obtained from group addressed and ATIM frames from all caches.

- In the previous sentence, it is not clear whether the conditions added by .11ae and .11aa also apply to ATIM frames. Ditto in the para below.

A receiving QMF STA shall also keep a cache of recently received <Address 2, AC, sequence-number, fragment-number> tuples from QMFs for all STAs from which the QoS STA has received QMFs. A receiving QMF STA is required to keep only the most recent cache entry per <Address 2, AC, sequence-number, fragment-number> for QMFs. A receiving QMF STA with dot11RobustAVStreamingImplemented false or not present(11aa) shall omit from the caches all tuples obtained from group addressed (#100)Data frames and tuples obtained from ATIM frames.(11ae)

- This subclause excels in long-winded paragraphs. Subclause should be restructured to show a table of all caches, identifying each as belonging to a type of cache, and the conditions under which they are required, with some common text per cache type.

A receiving STA shall reject as a duplicate frame any frame that is not a QoS Data frame in which the Retry bit in the Frame Control field is 1 and that matches an <Address 2, sequence-number, fragment-number> tuple of an entry in the cache that contains tuples of that format, unless the frame is a (#100)Management frame and the STA is maintaining separate caches for <Address 2, sequence-number, fragment-number> tuples from received (#100)Management frames. A receiving QoS STA shall also reject as a duplicate frame any QoS Data frame in which the Retry bit in the Frame Control field is 1 and that matches an <Address 2, TID, sequence-number, fragment-number> tuple of an entry in the cache that contains tuples of that format. A STA that is maintaining separate caches for <Address 2, sequence-number, fragment-number> tuples from received (#100)Management frames shall reject as a duplicate frame any (#100)Management frame that is not a time priority management frame in which the Retry bit in the Frame Control field is 1 and that matches an <Address 2, sequence-number, fragment-number> tuple of an entry in the management cache that contains tuples from frames that are not time priority management frames. A STA that is maintaining separate caches for <Address 2, sequence-number, fragment-number> tuples from received (#100)Management frames shall reject as a duplicate frame any time priority management frame in which the Retry bit in the Frame Control field is 1 and that matches an <Address 2, sequence-number, fragment-number> tuple of an entry in the cache that contains tuples from time priority management frames. A receiving QMF(#1051) STA shall also reject as a duplicate frame any QMF in which the Retry bit in the Frame Control field is 1 and that matches an <Address 2, AC, sequence-number, fragment number> tuple of an entry in the cache of tuples obtained from QMFs.(11ae)

A receiving non-mesh STA with dot11RobustAVStreamingImplemented true shall keep a cache entry per <DA, sequence-number> tuple for each group address subject to a GCR agreement. A receiving mesh STA with dot11MeshGCRImplemented true shall keep a cache entry per <DA, Address 2, sequence-number> tuple for each group address subject to a GCR agreement. (11aa)

There is a small possibility that a frame may be improperly rejected due to such a match; however, this occurrence would be rare and simply results in a lost frame (similar to an FCS error in other LAN protocols).

NOTE 1—The receiver STA performs the (#1198)Ack and (for an AP) PS procedures on all successfully received frames requiring acknowledgment, even if the frame is discarded due to duplicate filtering.

NOTE 2—Group addressed retransmissions of BUs use the same sequence number as the initial group addressed transmission of the BU. Unicast retransmissions of a group addressed BU delivered via DMS use the same sequence number as the initial unicast transmission of the BU. When a BU is delivered both using group addressing and unicast (e.g., when DMS is active but there are other associated STAs not using DMS), the sequence number might differ between the group addressed and unicast transmissions of the same BU.(11aa)