IEEE P802.11
Wireless LANs

| Clarification of the Link Setup Bursty |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-08 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Giwon Park | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-1879 | giwon.park@lge.ccom |
| Kiseon Ryu | LG Electronics | 10225 Willow Creek Rd, San Diego, CA, 92131, USA | +1 (858)-635-5209 | kiseon.ryu@lge.com |
| HanGyu Cho | LG Electronics | LG R&D Complex 533, Hogye-1dong, Dongan-Gu, Anyang, Kyungki, 431-749, Korea | +82-31-450-7902 | hg.cho@lge.com |
| JinSam Kwak | KAIST | 291 Daehak-ro, Yuseong-gu, Daejeon 305-701, Korea | +82-42-350-7626 | samji@wcslab.kaist.ac.kr |

Abstract

This submission proposes text changes in subclause 8.4.2.187-Link Setup Bursty parameter, as proposed resolutions to some comments of the TGai Working Group Technical Letter Ballot 198 on 802.11ai Draft 1.0.

# Introduction

As a response to IEEE 802.11 Working Group Technical Letter Ballot 198 for 802.11ai Draft 1.0, about several comments have been submitted on Subclause 8.4.2.187-Link Setup Bursty parameter. This contribution proposes resolutions to some of the assigned comments on 8.4.2.187- Link Setup Bursty parameter.

# Conventions

In this contribution, the proposed 802.11ai Specification Document text will be presented as modifications to the TGai draft specification 802.11ai/D1.0. The following format conventions are used:

- The new added text is marked as blue underline text;

- The deleted text is marked as red strikethrough text;

- The unchanged baseline standard text stays in black text in the context of proposed TGai specification text;

- The editorial instruction is marked as italic text highlighted by Yellow;

8.4.2.187 Differentiated Initial Link Setup element

Instructions to Editor:  modify the following text in section 8.4.2.187 (page 55, line 17)

The ILSC Type subfield is 1 octet in length and it is used to indicate the presence of the optional subfields in the ILSC Information field, and the LinkSetup Bursty, as defined in Figure 8-401ds (ILSC Type subfield format). A bit value of 1 in the User Priority, MAC Address Filter and Vendor Specific Category subfields indicates that the corresponding ILSC subfield is present At least one of the bits in ILSC Type subfield is set to 1 when Differentiated Initial Link Setup element is present.

|  | ILS User Priority | MAC address Filter | Vendor Specific Category | Link Setup Bursty | Reserved |
| --- | --- | --- | --- | --- | --- |
| Bits: | 1 | 1 | 1 | 1 | 15 |

Figure 8-401ds – ILSC Type subfield format

8.4.2.187 Differentiated Initial Link Setup element

Instructions to Editor:  Delete the Figure 8-183dv (Page 56, Line 52)

|  | Synchronization Detected | Reserved |
| --- | --- | --- |
| Bits: | 1 | 7 |

Figure 8-183dv-ILS Synchronization subfield

Motion-1: Move to authorize the Editor to incorporate the text changes proposed in contribution 11-13-1019-02-00ai or latest version to the draft TGai Specification Document.

Yes: ____________;  No: _________________;  Abstain: _____________

[Result of Motion]