IEEE P802.11
Wireless LANs

| TGah Coexistence Assurance Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-09-16 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Eldad Perahia | Intel Corporation | 2111 NE 25th Ave Hillsboro, OR 97124 | 503-712-8081 | eldad.perahia@intel.com |
| Yongho Seok | LG Electronics |  |  | yongho.seok@lge.com |

Abstract

This serves as the coexistence assurance document for TGah in meeting the requirement of the 5 criteria.

# Introduction

This document addresses coexistence of IEEE 802.11ah per the PAR and 5C’s [2].  The relevent sections of each are outlined below:

- PAR scope:

  - Provides mechanisms that enable coexistence with other systems in the bands including IEEE 802.15.4 and IEEE P802.15.4g.

- 5C’s:

  - Response to 17.5.4.1: “The working group will create a CA document and specifically reference IEEE P802.15.4g as part of the WG balloting process.”

# Channelization

STAs compliant with the physical layer defined in clause 24 of 802.11ah [1] operate in the channels defined in the additional rows introduced into the tables in Annex E of [1] (see below).

| Table E-1—Operating classes in the United States |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating class | Global operating class (see E-4) |  | Channel starting frequency (GHz) |  | Channel spacing (MHz) |  | Channel set |  | Channel center frequency index |  | Behavior limits set |  |  |  |
| <ANA> | <ANA> |  | 0.902 |  | 1 |  | - |  | 1,3,5,7,9,11,13,15,17,19,21,23,25,27,29,31,33,35,37,39,41,43,45,47,49,51 |  | TBD |  |  |  |
| <ANA+1> | <ANA+1> |  | 0.902 |  | 2 |  | - |  | 2,6,10,14,18,22,26,30,34,38,42,46,50 |  | TBD |  |  |  |
| <ANA+2> | <ANA+2> |  | 0.902 |  | 4 |  | - |  | 8,16,24,32,40,48 |  | TBD |  |  |  |
| <ANA+3> | <ANA+3> |  | 0.902 |  | 8 |  | - |  | 12,28,44 |  | TBD |  |  |  |
| <ANA+4> | <ANA+4> |  | 0.902 |  | 16 |  | - |  | 20 |  | TBD |  |  |  |
| Table E-2—Operating classes in Europe |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating class |  | Global operating class (see E-4) |  | Channel starting frequency (GHz) |  | Channel spacing (MHz) |  | Channel set |  | Channel center frequency index |  | Behavior limits set |  |  |
| <ANA> |  | <ANA> |  | 0.863 |  | 1 |  | - |  | 1,3,5,7,9 |  | TBD |  |  |
| <ANA+1> |  | <ANA+1> |  | 0.863 |  | 2 |  | - |  | 2,6 |  | TBD |  |  |
| Table E-3—Operating classes in Japan |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating class |  | Global operating class (see E-4) |  | Channel starting frequency (GHz) |  | Channel spacing (MHz) |  | Channel set |  | Channel center frequency index |  | Behavior limits set |  |  |
| <ANA> |  | <ANA> |  | 0.9165 |  | 1 |  | - |  | 1,3,5,7,9,11,13,15,17,19,21 |  |  |  |  |
| Table E-4—Global operating classes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Operating class |  | Nonglobal operating class(es) |  | Channel starting frequency (GHz) |  | Channel spacing (MHz) |  | Channel set |  | Channel center frequency index |  | Behavior limits set |  |  |
| <ANA> |  | <ANA> |  | 0.755 |  | 1 |  | - |  | 1,3,5,7,9,11,13,15,17,19,21,23,25,27,29,31,33,35,37,39,41,43,45,47,49,51,53,55,57,59,61,63 |  | TBD (for China) |  |  |
| <ANA+1> |  | <ANA+1> |  | 0.755 |  | 2 |  | - |  | 50,54,58,62 |  | TBD (for China) |  |  |
| <ANA+2> |  | <ANA+2> |  | 0.755 |  | 4 |  | - |  | 52,60 |  | TBD (for China) |  |  |
| <ANA+3> |  | <ANA+3> |  | 0.755 |  | 8 |  | - |  | 56 |  | TBD (for China) |  |  |
| <ANA+4> |  | <ANA+4> |  | 0.9175 |  | 1 |  | - |  | 1,3,5,7,9,11 |  | TBD (for Korea) |  |  |
| <ANA+5> |  | <ANA+5> |  | 0.9175 |  | 2 |  | - |  | 2,6,10 |  | TBD (for Korea) |  |  |
| <ANA+6> |  | <ANA+6> |  | 0.9175 |  | 4 |  | - |  | 8 |  | TBD (for Korea) |  |  |
| <ANA+7> |  | <ANA+7> |  | 0.866 |  | 1 |  | - |  | 1,3,5 |  | TBD (for Singapore) |  |  |
| <ANA+8> |  | <ANA+8> |  | 0.920 |  | 1 |  | - |  | 1,3,5,7,9 |  | TBD (for Singapore) |  |  |
| <ANA+9> |  | <ANA+9> |  | 0.866 |  | 2 |  | - |  | 4 |  | TBD (for Singapore) |  |  |
| <ANA+10> |  | <ANA+10> |  | 0.920 |  | 2 |  | - |  | 2,6 |  | TBD (for Singapore) |  |  |
| <ANA+11> |  | <ANA+11> |  | 0.920 |  | 4 |  | - |  | 4 |  | TBD (for Singapore) |  |  |

# Clear Channel Assessment

As in 11n/ac, the main mechanism for coexistence with non-802.11 devices is clear channel assessment (CCA).  According to 11ah 24.3.19.5 [1], a PHY must set its CCA indication to busy as follows

- “Any signal within the primary 1 MHz channel at or above -75 dBm.”

- “Any signal within the primary 2 MHz channel at or above -72 dBm.”

- “Any signal within the secondary 2 MHz channel at or above -72 dBm.”

- “Any signal within the secondary 4 MHz channel at or above -69 dBm.”

- “Any signal within the secondary 8 MHz channel at or above -66 dBm.”

Since the power spectral density is the same for each case, the CCA performance will be comparable for the various bandwidths.

# Definitions

- primary 1 MHz channel: A 1 MHz channel used by sub 1 GHz (S1G) stations (STAs) for the purpose of creating a 1 MHz channel.

- primary 2 MHz channel: A 2 MHz channel associated with a primary 1MHz channel used by S1G STAs for the purpose of creating a 2 MHz channel.

- secondary 2 MHz channel: In a 4 MHz S1G BSS, the 2 MHz subchannel adjacent to the primary 2 MHz channel that together form the 4 MHz channel of the 4 MHz VHT BSS. In a 8 MHz/16 MHz S1G BSS, the 2 MHz subchannel adjacent to the primary 2 MHz channel that together form the primary 4 MHz channel of the 8 MHz/16 MHz VHT BSS.

- secondary 4 MHz channel: In a 8 MHz S1G BSS, the 4 MHz subchannel adjacent to the primary 4 MHz channel that together form the 8 MHz channel of the 8 MHz VHT BSS. In a 16 VHT BSS, the 4 MHz subchannel adjacent to the primary 4 MHz channel that together form the primary 8 MHz channel.

- secondary 8 MHz channel: In a 16 MHz S1G BSS, the 8 MHz subchannel adjacent to the primary 8 MHz channel that together forms the 16 MHz channel of the 16 MHz S1G BSS.

# References

[1] Draft P802.11ah D1.0

[2] 11-10-0001-13-0wng-900mhz-par-and-5c