IEEE P802.11
Wireless LANs

| TGah Coexistence Assurance Document |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2015-03-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Eldad Perahia | Intel Corporation | 2111 NE 25th Ave Hillsboro, OR 97124 | 503-712-8081 | eldad.perahia@intel.com |
| Yongho Seok | LG Electronics |  |  | yongho.seok@lge.com |

Abstract

This serves as the coexistence assurance document for TGah in meeting the requirement of the 5 criteria.

Revisions

Rev 2 – document version when downloaded from IEEE mentor website on April/01/2014

Rev 3 – Included TGah editor changes as per instructions in document 14/0348r1

Rev 4 – Included TGah editor changes as per instructions in document 14/0394r2

# Introduction

This document addresses coexistence of IEEE 802.11ah per the PAR and 5C’s [2].  The relevent sections of each are outlined below:

- PAR scope:

  - Provides mechanisms that enable coexistence with other systems in the bands including802.15.4-2011 and 802.15.4g sub-GHz PHYs.

- 5C’s:

  - Response to 17.5.4.1: “The working group will create a CA document and specifically reference 802.15.4-2011 and 802.15.4g sub-GHz PHYs as part of the WG balloting process.”

# Channelization

STAs compliant with the physical layer defined in clause 24 of 802.11ah [1] operate in the channels defined in the additional rows introduced into the tables in Annex E of [1] (see below).

|  | Table E-4a—S1G Operating classesinput.doc |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| S1G Operating class | Global operating Class (see Table E-4) | Channel starting frequency (GHz) | Channel spacing (MHz) | Channel center frequency index | CCA Level Classification | Behavior limits set |
| 1 (U.S.) | 68 | 0.902 | 1 | 1,3,37,39,41,43,45,47, 49,51 | Type 1 (902-904 MHz, 920-928 MHz) | Reserved |
|  |  |  |  | 5,7,9,11,13, 15,17,19,21,23, 25,27,29,31,33, 35 | Type 2 (904-920 MHz) |  |
| 2 (U.S.) | 69 | 0.902 | 2 | 2,38,42, 46,50 | Type 1 (902-904 MHz, 920-928 MHz) | Reserved |
|  |  |  |  | 6,10,14,18,22,2 6,30,34 | Type 2 (904-920 MHz) |  |
| 3 (U.S.) | 70 | 0.902 | 4 | 40,48 | Type 1 (920-928 MHz) | Reserved |
|  |  |  |  | 8,16,24,32 | Type 2 (904-920 MHz) |  |
| 4 (U.S.) | 71 | 0.902 | 8 | 44 | Type 1 (920-928 MHz) | Reserved |
|  |  |  |  | 12, 28 | Type 2 (904-920 MHz) |  |
| 5 (U.S.) | 72 | 0.902 | 16 | 20 | Type 2 (904-920 MHz) | Reserved |
| 6 (Europe) | 66 | 0.863 | 1 | 1,3,5,7,9 | Type 1 (863-868 MHz) | Reserved |
| 7 (Europe) | 67 | 0.863 | 2 | 2,6 | Type 1 (863-868 MHz) | Reserved |
| 8 (Japan) | 73 | 0.9165 | 1 | 1,3,5,7,9,11 ,13,15,17,1 9,21 | Type 1 (916.5-927.5 MHz) | Reserved |
| 9 (China) | 61 | 0.755 | 1 | 1, 3, 5, 7, 9, 11, 13, 15, 17, 19, 21, 23, 25, 27, 29, 31, 33, 35, 37, 39, 41, 43, 45, 47 | Type 1 (755-779 MHz) | Reserved |
| 10 (China) | 62 | 0.779 | 1 | 1, 3, 5, 7, 9, 11, 13, 15 | Type 2 (779-787 MHz) | Reserved |
| 11 (China) | 63 | 0.779 | 2 | 2, 6, 10, 14 | Type 2 (779-787 MHz) | Reserved |
| 12 (China) | 64 | 0.779 | 4 | 4, 12 | Type 2 (779-787 MHz) | Reserved |
| 13 (China) | 65 | 0.779 | 8 | 8 | Type 2 (779-787 MHz) | Reserved |
| 14 (Korea) | 74 | 0.9175 | 1 | 1,3,5,7,9,11 | Type 1 (917.5-923.5 MHz) | Reserved |
| 15 (Korea) | 75 | 0.9175 | 2 | 2,6,10 | Type 1 (917.5-923.5 MHz) | Reserved |
| 16 (Korea) | 76 | 0.9175 | 4 | 8 | Type 1 (917.5-923.5 MHz) | Reserved |
| 17 (Singapore) | 66 | 0.863 | 1 | 7, 9, 11 | Type 1 (866-869 MHz) | Reserved |
| 18 (Singapore) | 68 | 0.902 | 1 | 37, 39, 41, 43, 45 | Type 1 (920-925 MHz) | Reserved |
| 19 (Singapore) | 67 | 0.863 | 2 | 10 | Type 1 (866-869 MHz) | Reserved |
| 20 (Singapore) | 69 | 0.902 | 2 | 38, 42 | Type 1 (920-925 MHz) | Reserved |
| 21 (Singapore) | 70 | 0.902 | 4 | 40 | Type 1 (920-925 MHz) | Reserved |
| 22 (Australia) | 68 | 0.902 | 1 | 27, 29, 31, 33, 35 | Type 1 (915-920 MHz) | Reserved |
|  |  |  |  | 37, 39, 41, 43, 45, 47, 49, 51 | Type 2 (920-928 MHz) |  |
| 23 (Australia) | 69 | 0.902 | 2 | 28, 32 | Type 1 (915-920 MHz) | Reserved |
|  |  |  |  | 38, 42, 46, 50 | Type 2 (920-928 MHz) |  |
| 24 (Australia) | 70 | 0.902 | 4 | 30 | Type 1 (915-920 MHz) | Reserved |
|  |  |  |  | 40, 48 | Type 2 (920-928 MHz) |  |
| 25 (Australia) | 71 | 0.902 | 8 | 44 | Type 2 (920-928 MHz) | Reserved |
| 26 (New Zealand) | 68 | 0.902 | 1 | 27, 29, 31, 33, 35, 37, 39, 41, 43 | Type 1 (915-924 MHz) | Reserved |
|  |  |  |  | 45, 47, 49, 51 | Type 2 (924-928 MHz) |  |
| 27 (New Zealand) | 69 | 0.902 | 2 | 28, 32, 36, 40 | Type 1 (915-924 MHz) | Reserved |
|  |  |  |  | 46, 50 | Type 2 (924-928 MHz) |  |
| 28 (New Zealand) | 70 | 0.902 | 4 | 30, 38 | Type 1 (915-924 MHz) | Reserved |
|  |  |  |  | 48 | Type 2 (924-928 MHz) |  |
| 29 (New Zealand) | 71 | 0.902 | 8 | 34 | Type 1 (915-924 MHz) | Reserved |

# Clear Channel Assessment

As in 11n/ac, the main mechanism for coexistence with non-802.11 devices is clear channel assessment (CCA).  According to 11ah 24.3.19.5 [1], a PHY must set its CCA indication to busy as follows

- “Any signal within the primary 1 MHz channel at or above -75 dBm.”

- “Any signal within the primary 2 MHz channel at or above -72 dBm.”

- “Any signal within the secondary 2 MHz channel at or above -72 dBm.”

- “Any signal within the secondary 4 MHz channel at or above -69 dBm.”

- “Any signal within the secondary 8 MHz channel at or above -66 dBm.”

Since the power spectral density is the same for each case, the CCA performance will be comparable for the various bandwidths.

# Definitions

- primary 1 MHz channel: A 1 MHz channel used by sub 1 GHz (S1G) stations (STAs) for the purpose of creating a 1 MHz channel.

- primary 2 MHz channel: A 2 MHz channel associated with a primary 1MHz channel used by S1G STAs for the purpose of creating a 2 MHz channel.

- secondary 2 MHz channel: In a 4 MHz S1G BSS, the 2 MHz subchannel adjacent to the primary 2 MHz channel that together form the 4 MHz channel of the 4 MHz VHT BSS. In a 8 MHz/16 MHz S1G BSS, the 2 MHz subchannel adjacent to the primary 2 MHz channel that together form the primary 4 MHz channel of the 8 MHz/16 MHz VHT BSS.

- secondary 4 MHz channel: In a 8 MHz S1G BSS, the 4 MHz subchannel adjacent to the primary 4 MHz channel that together form the 8 MHz channel of the 8 MHz VHT BSS. In a 16 VHT BSS, the 4 MHz subchannel adjacent to the primary 4 MHz channel that together form the primary 8 MHz channel.

- secondary 8 MHz channel: In a 16 MHz S1G BSS, the 8 MHz subchannel adjacent to the primary 8 MHz channel that together forms the 16 MHz channel of the 16 MHz S1G BSS.

# References

[1] Draft P802.11ah D4.0

[2] 11-10-0001-13-0wng-900mhz-par-and-5c