IEEE P802.11
Wireless LANs

| LB198 Resolution for Clause 8.4.2.186 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-07 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | Allied Telesis R&D Center | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 JAPAN | +81 92 771 7630 | hmorioka@root-hq.com |

Abstract

This document is a proposed resolution normative text of Clause 8.4.2.186 for LB198.

# [CIDfragment]: CID2218, 2555, 3064, 2017, 2159, 2217, 2552, 2553, 2757, 2828

[CIDid]: CID2830, 3048, 3049

# 8. Frame formats

## 8.3 Format of individual frame types

### 8.3.3 Management frames

### 8.3.3.6 Association Response frame format

Remove “FILS KDE Container” from Table 8-23.

### 8.3.3.8 Reassociation Response frame format

Remove “FILS KDE Container” from Table 8-25.

## 8.4 Management frame body components

### 8.4.2 Information elements

### 8.4.2.1 General

Remove “FILS KDE Container” from Table 8-54.

# Remove clause 8.4.2.181:

### 8.4.2.181 KDE container element [CID2219]

# Change clause 8.4.2.186 as following:

### 8.4.2.186 FILS Secure Container element

FILS Secure Container element includes one or more FILS Secure Container TLV(s). A Type Length Value (TLV) encoding consists of three fields: Type, Length, and Value as shown in Figure 8-401dv (FILS Container TLV format).  The first field, Type, specifies the type of the data carried by the value field, and it is unique within the FILS Secure Container element. The second field, Length, specifies the actual length of the value field in octets. The third field, Value, contains the data representing the value for the type field. [CID2016, 2827]

|  | Type ID | Length | Value |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | variable |

Figure 8-401dv—FILS Secure Container TLV format

FILS Secure Container TLVs are shown in Table 8-183ai (FILS Secure Container TLV).. The format of the FILS Secure Container element is shown in Figure 8-401dg (FILS Secure Container element format).

Table 8-183ai – FILS Secure Container TLV

| Type | Type ID | Length (octets) | Extensible |
| --- | --- | --- | --- |
| FILS HLP Wrapped data | 1 | variable but limited by MPDU | No |
| FILS IP Address Request | 2 | 4 to 255 | No |
| FILS IP Address Assignment | 3 | 4 to 255 | No |
| FILS DNS Information | 4 | 4 to 255 | No |
| Key RSC | <ANA> | 19 | No |
| GTK Transfer KDE Container [CID2219] | <ANA> | 4 to 255 | No |

|  | Element ID | Length | FILS Secure Container TLV(s) |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |

Figure 8-401dg – FILS Secure Container element format

FILS Secure Container TLVs are used to carry out various purposes [editorial] such as IP address assignment and GTK transfer.

A FILS Secure Container TLV may not fit into a single element, and therefore the FILS Secure Container element may require fragmentation using Fragment elements (see 8.4.2.189 (Fragment element)). [CIDfragment]

### 8.4.2.186.1 FILS HLP wrapped data TLV

The FILS wrapped data elementTLV [CID2829] contains higher layer packets transported during association. One or more FILS HLP wrapped data TLVs may be included in an Association Request, a Reassociation Request, [editorial] an Association Response, or a Reassociation Response frames if dot11FILSActivated is true.

The ElementType [CIDid] ID field is equal to the FILS HLP Wrapped data value in Table 8-183ai (FILS Secure Container TLV).

The value of the Length field is 12 plus the length of data after Source MAC Address including LLC/SNAP and HLP HLP MSDU field. [CID2157]

The value of Destination MAC Address field is the destination MAC address of the HLP frame [CID2831, 3050]. [editorial]

|  | TLV ID | Length | Destination MAC Address |  |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 |  |
|  | Source MAC Address |  | LLC/SNAP | HLP |
| Octets: | 6 |  | variable | variable |

|  | Type ID | Length | Destination MAC Address | Source MAC Address | HLP MSDU |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 6 | 6 | variable |

[CIDid, 2157]

Figure 8.401dh – FILS HLP Wrapped TLV format

The value of Source MAC Address field is the source MAC address of the HLP frame [CID2831, 3050] It is same as the source address of the STA generating the HLP frame. [CID3209]

The value of LLC/SNAP field is the LLC header and SNAP header (if applicable) of the HLP.  If the LLC field is equal to 0xaa 0xaa, a 5-octet SNAP header is added, see 4.2.5. [CID2157]

The HLP MSDU field contains the MSDU of the HLP (Higher Layer Protocol) frame [CID2831, 3050, 2018, 2157].

### 8.4.2.186.2 FILS IP aAddress rRequest TLV [CID2316]

FILS IP aAddress rRequest TLV is used by STA to request IP address using FILS IP Address assignment method. FILS IP aAddress rRequest TLV may be sent in an Association Request, a Reassociation Request or a FILS Secure Container Action frame if dot11FILSActivated is true.

|  | TLV ID | Length | IP Address Request Control | Requested IPv4 address (optional) | Requested IPv6 address (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 4 | 16 |

|  | Type ID | Length | IP Address Request Control | Requested IPv4 Address (optional) | Requested IPv6 Address (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 1 | 4 | 16 |

[CIDid, 2216, 2316]

Figure 8.401di – FILS IP aAddress rRequest TLV format [CID2316]

The TLVType [CIDid] ID field is setequal to the FILS IP Address AssignmentRequest [CID2507] value in Table 8-183ai (FILS Secure Container TLV) [CID3052, 3053].

The value of the Length field is 1 plus the number of octets of all the optional fields presented after the IP Address Request Control field. [CID2832, 2160, 3051, 3297]

The value of the IP Address Request Control is as follows. [CID2316]

|  | B0 | B1 | B2 | B3 | B4 | B5 B7 |
| --- | --- | --- | --- | --- | --- | --- |
|  | IPv4 rRequest | IPv4 rRequest tType | IPv6 rRequest | IPv6 rRequest) [CID2189] tType | DNS sServer aAddress rRequest | Reserved |
| Bits: | 1 | 1 | 1 | 1 | 1 | 3 |

Figure 8.401dj – FILS IP aAddress rRequest cControl field format [CID2316]

A STA sets the IPv4 rRequest subfield to 1 if the STA is requesting an IPv4 address and sets it to 0 otherwise. [CID2316]

A STA sets the IPv4 rRequest tType subfield to 1 if the STA requests a new IPv4 address and sets it to 0 if the STA requests the IPv4 address that is present in the TLV. [CID2316]

A STA sets the IPv6 rRequest subfield to 1 if the STA is requesting an IPv6 address and sets it to 0 otherwise. [CID2316]

A STA sets the IPv6 rRequest tType subfield to 1 if the STA requests a new IPv6 address and sets it to 0 if the STA requests the IPv6 address that is present in the TLV. [CID2316]

A STA sets the DNS sServer aAddress rRequest subfield to 1 if the STA is requesting DNS server(s) address(es). [CID2316]

The value of the Requested IPv4 address is the IPv4 address requested by the STA if the IPv4 Request type bit of the IP Address Request control field is '0'0 [CID2316]

The value of the Requested IPv6 address is the IPv6 address requested by the STA if the IPv6 Request type bit of the IP Address Request control field is '0'0 [CID2316]

### 8.4.2.186.3 FILS IP Address Assignment TLV

FILS IP Address Assignment TLV is used by AP to include IP address using FILS IP Address assignment method Configuration (10.44.4.2 IP address assignement using FILS IP Address Configuration) [CID3211, 2317]. FILS IP Address Assignment TLV may be sent in an Association Response, a Reassociation Response, or a FILS Secure Container Action frame if dot11FILSActivated is true.

|  | TLVType [CIDid] ID | Length [CID2755] | IP Address Response Control | Assigned IPv4 aAddress (optional) | Gateway IPv4 aAddress (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets | 1 | 2 | 1 | 4 | 4 |

|  | IPv4 Gateway MAC aAddress (optional) | Subnet mMask (optional) | Assigned IPv6 aAddress (optional) |
| --- | --- | --- | --- |
| Octets | 6 | 4 | 16 |

|  | IPv6 Gateway aAddress (optional) | IPv6 Gateway MAC aAddress (optional) | IPv6 prefix length (optional) |
| --- | --- | --- | --- |
| Octets | 16 | 6 | 1 |

|  | TTL IPv4 (optional) | TTL IPv6 (optional) |
| --- | --- | --- |
| Octets | 2 | 2 |

Figure 8.401dk – IP Address Assignment TLV format [CID2316]

The TLVType [CIDid] ID field is setequal to the FILS IP Address Assignment value in Table 8-183ai (FILS Secure Container TLV) [CID3052, 3053].

The value of the Length field is 1 plus the number of bytes of all the optional fields presented after the IP Address Response Control field. [CID2833]

The value of the IP Address Response control field is defined in Table 8-183<ANA>

(Table 8-183<ANA> IP Address Response Control)

An AP sets the IP address assignment pending subfield to 1 if it is still working on obtaining the IP address for the STA. [CID2316]

If the value of the IPv4 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the Assigned IPv4 address field is included and its value is set to the assigned IPv4 address. [CID2316]

If the IPv4 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the Gateway IPv4 address field is included and its value is set to the IP address of the IPv4 Gateway. [CID2316]

If the IPv4 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the Gateway MAC Address field is included and its value is set to the MAC address of the IPv4 Gateway. [CID2316]

If the Subnet mask included bit is included in the IP Address Response Control field, and is set to '1'1, then the Subnet Mask field is included and its value is set to the subnet mask of the IPv4 subnet. [CID2316]

If the value of the IPv6 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the Assigned IPv6 address field is included and its value is set to the assigned IPv6 address. [CID2316]

If the value of the IPv6 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the Gateway IPv6 address field is included and its value is set to the IP address of the IPv6 Gateway. [CID2316]

If the value of the IPv6 Assigned bit is included in the IP Address Response Control field, and is set to '1'1, then the IPv6 Gateway MAC Address field is included and its value is set to the MAC address of the IPv6 Gateway. [CID2316]

If the value of the Prefix Length included bit is included in the IP Address Response Control field, and is set to '1'1, then the IPv6 Prefix Length field is included and its value is set to the prefix length of the IPv6 network. [CID2316]

If the value of the TTL-IPv4 included bit is included in the IP Address Response Control field and is set to '1'1, then the TTL IPv4 field is included and its value is set to the IPv4 Time to Live in the unit of seconds. [CID2316]

If the value of the TTL-IPv6 included bit is included in the IP Address Response Control field and is set to '1'1, then the TTL IPv6 field is included and its value is set to the IPv6 Time to Live in the unit of seconds. [CID2316]

### 8.4.2.186.4 FILS DNS Information TLV

FILS DNS Information TLV is used by AP to send DNS information in the FILS IP Address assignment method Configuration (10.44.4.2 IP address assignement using FILS IP Address Configuration) [CID2321]. FILS DNS Information TLV may be sent in an Association Response, a Reassociation Response or a FILS Secure Container Action frame if dot11FILSActivated set to true. FILS DNS Information TLV carries IP address and MAC address information of the DNS Server to which the DNS queries may be sent. [editorial]

|  | TLV Type [CIDid] ID | Length | DNS Info Control | DNS server IPv4 address (optional ) | DNS server IPv6 address (optional) | IPv4 DNS server MAC address (optional) | IPv6 DNS server MAC address (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 12 [CID2216] | 1 | 4 | 16 | 6 | 6 |

Figure 8.401dm – DNS sServer iInformation TLV format [CID2316]

The TLVType [CIDid] ID field is the FILS DNS Information value in Table 8-183ai (FILS Secure Container TLV) [CID3052, 3053].

The value of the Length field is 1 plus the number of bytes of all the optional fields presented after the DNS Info Control field. [CID2835]

The value of the DNS Info Control is as follows: [CID2316]

|  | B0 | B1 | B2 | B3 | B4 B7 |
| --- | --- | --- | --- | --- | --- |
|  | DNS server IPv4 address present | DNS server IPv6 address present | IPv4 DNS server MAC address present | IPv6 DNS server MAC address present | reserved |
| Bits: | 1 | 1 | 1 | 1 | 4 |

Figure 8.401dn – DNS Info cControl field format [CID2316]

An AP sets the DNS Server IPv4 address Present subfield to 1 if the IPv4 DNS server IPv4 address is present in the TLV and sets it to 0 otherwise. [CID2316]

An AP sets the DNS Server IPv6 address Present subfield to 1 if the IPv6 DNS server IPv6 address is present in the TLV and sets it to 0 otherwise. [CID2316]

An AP sets the IPv4 DNS Server MAC Address Present subfield to 1 if the MAC address to which IPv4 based DNS queries may be sent is present and sets it to 0 otherwise. [CID2316]

An AP sets the IPv6 DNS Server MAC Address Present subfield to 1 if the MAC address to which IPv6 based DNS queries may be sent is present and sets it to 0 otherwise. [CID2316]

The value of the DNS Server IPv4Address is the IPv4 address of the DNS server if the DNS Server IPv4address Present bit of the DNS Info Control is '1'1. [CID2316]

The value of the DNS Server IPv6Address is the IPv6 address of the DNS server if the DNS Server IPv6 address Present bit of the DNS Info Control is '1'1. [CID2316]

The value of the IPv4 DNS Server MAC Address is the MAC address of the IPv4 DNS server if the IPv4 DNS Server MAC Address Present bit of the DNS Info Control is '1'1. [CID2316]

The value of the IPv6 DNS Server MAC Address is the MAC address of the IPv6 DNS server if the IPv6 DNS Server MAC Address Present bit of the DNS Info Control is '1'1. [CID2316]

### 8.4.2.186.5 Key RSC TLV

Key RSC TLV contains Key RSC of the GTK. The format is shown in Figure 8-401do (Key RSC TLV format).

|  | TLVType [CIDid] ID | Length | Key RSC |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | 168 [CID2545] |

Figure 8-401do –Key RSC TLV format

The TLVType [CIDid] ID field is equal to the Key RSC value in Table 8-183ai (FILS Secure Container TLV).

The value of the Length field is 168 [CID2545].

The value of Key RSC is the current Key RSC of the GTK.

### 8.4.2.186.6 GTK Transfer TLV [CID2219]

GTK Transfer TLV contains a GTK. The format is shown in Figure 8-401dp (GTK Transfer TLV format).

The TLV ID field is equal to the GTK Transfer value in Table 8-183ai (FILS Secure Container TLV).

The value of the Length field is 1 plus GTK length in octets.

The value of Key ID is the Key ID of the GTK.

|  | TLV ID | Length | Key ID | GTK |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 2 | 1 | variable |

Figure 8-401dp –GTK Transfer TLV format

### 8.4.2.186.6 KDE Container TLV

The KDE Container TLV is used to communicate one or more KDEs in a FILS authentication exchange. The KDE Container TLV may contain one or more KDEs. The format of the KDE Container TLV is shown in Figure 8-401dp (KDE Container TLV format).

|  | Type ID | Length | KDE(s) |
| --- | --- | --- | --- |
| Octets: | 1 | 2 | variable |

Figure 8-401dp— KDE container TLV format

The Type ID field is equal to the KDE Container value in Table 8-183ai (FILS Secure Container TLV).

The Length field specifies the total number of octets of all the KDEs.

The encoding of the KDE field is defined in Table 11-6 (KDE) of 11.6.2 (EAPOL-Key frames).

### 8.4.2.189 Fragment IE

Insert a following row to the Table 8-183ak. [CIDfragment]

| FILS Secure Container element | <ANA> |
| --- | --- |