IEEE P802.11
Wireless LANs

| LB198 Resolution for Clause 10.44.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-13 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Hitoshi Morioka | Allied Telesis R&D Center | 2-14-38 Tenjin, Chuo-ku, Fukuoka 810-0001 JAPAN | +81 92 771 7630 | hmorioka@root-hq.com |

Abstract

This document is a proposed resolution normative text of Clause 10.44.4 for LB198.

Resolved CIDs are 2152, 2177, 2157, 2089, 2866, 2165, 2166, 2167 and 2378.

# 10. MLME

## 10.44 Fast Initial Link Setup (FILS) procedures

# Replace clause 10.44.4 and 10.44.4.1 as following:

### 10.44.4 Higher layer setup during association/reassociation procedure

Higher layer setup, such as IP layer setup, may be performed during FILS association/reassociation procedure. Two mechanisms are provided for higher layer setup: (a) encapsulation of higher layer protocol frame(s), such as DHCP, (b) FILS IP address configuration. The choice is determined by the STA based on what higher layer setup mechanisms are supported by the AP.

Higher layer setup information in Association Request, Association Response, Reassociation Request and Reassociation Response frame shall be protected by the AEAD scheme (11.11.2.5).

(Action frame protection should be described.)[CID2173, 2863, 2941]

### 10.44.4.1 Higher layer setup using higher layer frame encapsulation

The FILS HLP wrapped data TLV(s) in the FILS Secure Container element (8.4.2.186.1) is used for encapsulating higher layer protocol (HLP) frame(s).

If a non-AP STA uses the higher layer frame encapsulation, the non-AP STA constructs the FILS HLP wrapped data TLV(s). When the non-AP STA transmits multiple HLP frames in Association/Reassociation frame, the non-AP STA shall construct multiple FILS HLP wrapped data TLVs for each HLP frames. Then the non-AP STA transmits Association/Reassociation Request including FILS HLP wrapped data TLV in FILS Secure Container element. The FILS Secure Container element may be fragmented by Fragment element (8.4.2.189) if required. The encapsulation procedure is following.

- The non-AP STA prepares HLP MSDU(s) to transmit.

- The non-AP STA fills FILS HLP wrapped data TLV(s) by the destination MAC address, the source MAC address and the HLP MSDU for each HLP MSDUs.

- The non-AP STA encapsulates the FILS HLP wrapped data TLV(s) into the FILS Secure Container element (8.4.2.186) and Fragment element(s) if required.

When the AP receives Association/Reassociation Request frame including FILS HLP Wrapped data TLV(s), the AP decapsulates the HLP frame(s). The AP shall not transfer the HLP frame(s) until the key confirmation (refer 11.11.2.4) by the AP is completed. After successful key confirmation, the AP forwards the HLP frame(s) to the network according to the destination MAC address of the HLP frame(s). If the key confirmation fails, the AP silently discards the HLP frame(s). The frame decapsulation procedure is following.

- The AP extracts the destination MAC address, the source MAC address and the MSDU from the FILS HLP wrapped data TLV.

- The AP constructs the frame in appropriate format to deliver the MSDU to the network by using the extracted the destination MAC address, the source MAC address and the MSDU.

If the AP receives HLP frames with the STA’s MAC address, multicast address or broadcast address as the destination address from the network before transmitting Association/Reassociation Response, the AP transmits Association/Reassociation Response frame including the HLP frame(s) in the FILS HLP Wrapped data TLV of the FILS Secure Container element. The encapsulation procedure is described previously. If the AP does not receive HLP frames from the network targeted to the STA before transmitting Association/Reassociation Response, the AP transmits Association/Reassociation frame without FILS HLP Wrapped data TLV. The status code of Association/Reassociation Response is not affected whether or not the HLP frame is included in the Association/Reassociation Response frame.

When the non-AP STA receives Association/Reassociation Response with HLP Wrapped data TLV, the non-AP STA decapsulates the HLP(s) and generates MA-UNITDATA.indication primitive for each HLP MSDU(s).