IEEE P802.11
Wireless LANs

| TGaq – Service Transaction Protocol |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2013-11-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Michael Montemurro | BlackBerry Ltd | 4701 Tahoe Blvd Mississauga, ON, CANADA L4W 0B4 | +1-289-261-4183 | mmontemurro@blackberry.com |
| Stephen McCann | BlackBerry Ltd | 200 Bath Road, Slough, Berkshire, SL1 3XE, UK | +44 1753 667099 | smccann@blackberry.com |
| Paul A. Lambert | Marvell Semiconductor, Inc. | 5488 Marvell Lane, Santa Clara, CA, 95054, USA | +1 408 222 8341 | Paul at Marvell dot com |

Abstract

This document describes the basic features of the Service Transaction Protocol

- Definitions, acronyms and abbreviations

  - Definitions

Insert new definitions retaining alphabetic order as follows:

Pre-Association Discovery (PAD): Discovery of service information for a pre-associated STA

Service Identifier (SID): Short identifier (48 bits) of a unique service identifier (USID)

Service Transaction Protocol (STP): The protocol for service transactions transported by generic advertisement service (GAS) Public Action frames.

Unique Service Identifier (USID): A type of UUID which globally uniquely identifies a service.

[13/0478r2]4.5.9 Interworking with external networks

Change text as follows:

The interworking service allows non-AP STAs to access services provided by an external network according to the subscription or other characteristics of that external network. An IEEE 802.11 non-AP STA may have a subscription relationship with an external network, e.g., with an SSPN.

An overview of the interworking functions addressed in this standard is provided below:

— Network discovery and selection

— Discovery of suitable networks through the advertisement of access network type, roaming

consortium and venue information, via management frames

— Selection of a suitable IEEE 802.11 infrastructure using advertisement services (e.g., Access Network Query Protocol (ANQP) or an IEEE 802.21 Information Server) in the BSS or in an external network reachable via the BSS.

— Selection of an SSPN or external network with its corresponding IEEE 802.11 infrastructure

— Pre-Association Discovery

— Discovery of services offered by an infrastructure network in prior to association.

— Emergency services

— Emergency Call and Network Alert support at the link level

— QoS Map distribution

— SSPN interface service between the AP and the SSPN

8.4.2.29 Extended Capabilities element

Insert the following row (ignoring the header row) in Table 8-103 after UTF-8 SSID (or more recent one)

|  |  | Table 8-103 – Capabilities field |  |
| --- | --- | --- | --- |
| Bit | Information |  | Notes |
| 49 | PAD |  | When dot11PADActivated is true, the PAD field is set to 1 to indicate the STA supports the PAD service as described in 10.24.3.4. When dot11PADActivated is false, the PAD field is set to 0 to indicate the STA does not support this capability. |

8.4.2.95 Advertisement Protocol element

Insert the following row (ignoring the header row) in Table 8-175 after Registered location query protocol (RLQP):

| Table 8-175 - Advertisement protocol ID definitions |  |
| --- | --- |
| Name | Value |
| Service Transaction Protocol | 5 |

Insert dashed list text after Registered location query protocol (RLQP) as follows:

—The Service Transaction Protocol (STP) supports service information retrieval. STP is a protocol used by a requesting STA to query another STA (i.e., the receiving STA can respond to queries with and without proxying the query to a server in an external network). See 10.24 (WLAN interworking with external networks procedures) for information on STP procedures.

Insert the following new subclause 8.4.6.

8.4.6 Service Transaction Protocol (STP) elements

8.4.6.1 General

STP-elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet Length field, and a variable-length element-specific Information field. Each element is assigned a unique Info ID as defined in this standard. The STP-element format is shown in Figure 8-431aq (STP-element format).

|  | Info ID | Length | Information |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 8-431aq – STP-element format

Each STP-element in 8.4.6 is assigned a unique 2-octet Info ID. The set of valid Info IDs are defined in Table 8-190aq (STP-element definitions). The 2-octet Info ID field is encoded following the conventions given in 8.2.2 (Conventions).

The Length field is a 2-octet field that indicates the number of octets in the Information field and is encoded following the conventions given in 8.2.2 (Conventions).

The STP-elements are shown in Table 8-190aq (STP-element definitions). Info ID 56 797 indicates Vendor Specific information. When the Info ID is equal to 56 797, the format of the subfield follows the format of the vendor specific element in 8.4.2.28 (Vendor Specific element).

| Table 8-190aq – STP-element definitions |  |  |
| --- | --- | --- |
| STP-element name | Info ID | STP-element (subclause) |
| Reserved | 0 - 275 [13/0725r1 | [previous edits by CID #1102, 13/0573r3 deleted] n/an n/a[13/0725r1 |
| Encapsulation | 276 | 8.4.6.2 |
| Reserved | 280 - 56796 | n/a |
| Vendor Specific | 56797 | 8.4.2.28 (Vendor Specific Element) |
| Reserved | 56798 - 65535 | n/a |

8.4.6.2 Encapsulation STP-element

The Encapsulation element is used to exchange upper layer protocol (ULP) frames between STAs using the GAS protocol. The Encapsulation STP-element is used as a transaction request included in a GAS Query Request or returned as a transaction response in the GAS Query Response.

|  | Info ID | Length | SID Duple #1 (optional) | … | SID Duple #n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 2 | 2 | variable | … | variable |

Figure 8-431aq – Encapsulation STP-element format

The Info ID and Length fields are defined in 8.4.6.1 (General).

The format of the SID Duple is shown in Figure 8-431aq1.

|  | Length | SID | Service Attribute (optional) |
| --- | --- | --- | --- |
| Octets: | 1 | 6 | variable |

Figure 8-431aq1 – SID Duple field

— The length field is a 1 octet field whose value is equal to 6 plus the number of Octets in the Service Attribute field.

— The Service Identifier (SID) field is a 6 octet version of a Unique Service Identifier (USID) for the service which is within the STP query or response. See Annex AQ1 for further information.

— The Service Attribute field is a variable length string used to provide extra service information in addition to that of the SID, e.g. a service name string. See Annex AQ1 for further information.

10. MLME

10.24 WLAN interworking with external networks procedures

10.24.3 Interworking procedures: generic advertisement service (GAS)

Insert new subclause 10.24.3.4 after the end of 10.24.3.3 as follows:

10.24.3.4 Service Transaction Protocol (STP) procedures

When dot11PADActivated is true, a STA may use STP to request information as defined in Table 8-175 (Advertisement protocol ID definitions) from a peer STA that has transmitted an Advertisement Protocol element indicating support for STP (see 8.4.2.95 (Advertisement Protocol element)) in a Beacon or Probe Response frame. The receiving STA might respond to queries with or without proxying the query to a server in an external network. See Annex AQ2 for more information on use of a Proxy Entity

If information is not configured for a particular STP-element, then a query for that element returns that element with no optional fields.

A STA may use STP to exchange service information as defined in Table 8-190 from a peer STA. STP uses the Encapsulation STP-element to transport an Service Identifier between STAs.

A STA that encounters an unknown or reserved STP Info ID value in a GAS frame (see 8-210) received without error shall ignore that STP Info ID and shall parse any remaining STP Info IDs.

A STA that encounters an unknown vendor-specific STP-element field or subfield in a GAS frame (see 8-210) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

Annex C

(normative)

ASN.1 encoding of the MAC and PHY MIB

C.3 MIB Detail

Insert new MIB values as follows:

dot11PADImplemented OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable. Its value is determined by device capabilities.

This attribute when true, indicates the STA is capable of pre-association discovery (PAD)

with external networks. A STA setting this to true implements PAD. When this is false, the STA

does not implement PAD.

DEFVAL {false}

::= { dot11StationConfigEntry <tbd> }

dot11PADActivated OBJECT-TYPE

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME. Changes take

effect as soon as practical in the implementation.

This attribute when true, indicates the capability of the STA to operate PAD

with external networks is enabled. The capability is disabled otherwise."

DEFVAL {false}

::= { dot11StationConfigEntry <tbd> }

Annex AQ1

(normative)

AQ1.1 Unique Service Identifier (USID)

<see https://mentor.ieee.org/802.11/dcn/13/11-13-0893-00-00aq-service-discovery-proposal.pptx>

- 128 bits long (16 octets) is large enough to be statistically unique (~3E+38)

- It is a type of “UUID”, a well defined construct in other standards.

AQ1.2. Service Identifier (SID)

Provides a convenient short identifier (e.g. 6 octets)

May not always be unique, there may be collisions. Collisions, however, can be very rare for well selected sizes and collision impact can be mitigated

Multiple SIDs can be created from the same  Unique Service Identifier by taking different ranges for the truncation (e.g. First 6 octets, next 6 octets ...)

Annex AQ2

(normative)

AQ2.1 Proxy Entity

<placeholder> for description of a service directory in the STA. This is required to pass information up to higher layer applications (e.g. a service ID hash) and for the STP to work.

It is assumed that there is proxy function in the network that maintains a list of services. The upper layer protocols are not exposed to un-associated STAs. The proxy is used to encapsulate the USIDs in a hash and exchange that information to the STA.

Therefore STP is opaque to the service definition and is handled by the proxy and the end STA itself.

Avoids overlap with ANQP and RLQP Info IDs, enabling a simpler parsing mechanism for received frames.