IEEE P802.11
Wireless LANs

| Proposed Resolution to CID 2263 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Michael Montemurro | BlackBerry | 4701 Tahoe Blvd., Mississauga, ON. Canada. L4W 0B4 | +1-905-629-4746 | mmontemurro@blackberry.com |

### Overview

Proposed comment resolution to CID 2263

| CID | Commenter | LB | Draft | Clause Number(C) | Page(C) | Line(C) | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2263 | David Hunter | 199 | 2 | 4.3.9.2 | 62 | 58 | Provide normative text (in a normative clause) that supports all of the informative claims made here about beacon requests/reports. If such normative text is not provided, then just delete all of the sentences in 4.3.9.2 except for the first. | Revised. Incorporate changes given in document 13/1455r1 |

- There looks to be way too much information in the cited text for Clause 4.

- Normative text for Beacon Report request/response is given in 10.11.9.1

… now back to the clause 4 text. Here is the proposed resolution:

Make the changes indicated below:

4.3.9.2 Beacon

The Beacon request/report pair enables a STA to request from another STA a list of APs whose beacons it

can receive on a specified channel or channels.  The Beacon report request/response provides a means for a requesting STA to obtain received beacon, probe response, and measurement pilot information from a responding STA. This measurement may be done by active mode (like active

scan), passive mode (like passive scan), or beacon table modes. If the measurement request is accepted and

is in passive mode, a duration timer is set. Then the measuring STA monitors the requested channel;

measures beacon, probe response, and measurement pilot power levels (received channel power indicator

(RCPI)); and logs all beacons, probe responses, and measurement pilots received within the measurement

duration. If the measurement request is in active mode, the measuring STA sends a probe request on the

requested channel at the beginning of the measurement duration; then monitors the requested channel;

measures beacon, probe response, and measurement pilot power levels (RCPI); and logs all beacons, probe

responses, and measurement pilots received within the measurement duration. If the request is beacon table

mode, then the measuring STA returns a Beacon (#1294) report containing the current contents of any stored

beacon information for any supported channel with the requested service set identifier (SSID) and basic

service set identifier (BSSID) without performing additional measurements.