IEEE P802.11
Wireless LANs

| Some 11ak EtherType Frame Encoding Text |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-02-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Donald Eastlake | Huawei Technologies | 155 Beaver Street, Milford, MA 01757 USA | +1-508-333-2270 | d3e3e3@gmail.com |

Abstract

This document provides some tentative text concerning recognizing General Link (GLK) STAs, basic properties of GLK STAs, their use of EtherType frame encoding (EPD, etc., for a P802.11ak draft. It uses Draft P802.11REVmc_D2.4 as its base document.

Editor’s notes

The editor’s notes do not form a part of this standard. They will be removed before publication. Please do not comment on editor’s notes in any ballot on the draft, as these comments would have no effect on the published standard.

Editor’s Note: Editor’s Notes in the body of the standard appear like this. They will be removed before publication. They indicate some item of work or comment that will be addressed prior to publication.

This text is based on 802.11REV-mc D2.3 and will need to be revised in light of 802.11 amendments not incorporated in that draft and adopted after that draft but before P802.11ak.

Table of Contents

1	Overview	6

2	Normative references	6

3	Definitions, acronyms, and abbreviations	7

3.1	Definitions	7

3.2	Definitions specific to IEEE 802.11	7

3.3	Abbreviations and acronyms	7

4	General Description	7

4.1	General description of the architecture	7

4.2	How wireless local area networks (WLANs) are different	7

4.3	Components of the IEEE Std 802.11 architecture	8

4.3.12 STA transmission of Data frames outside the context of a BSS	8

4.3.20 General Link (GLK)	8

4.4	Logical service interfaces	9

4.5	Overview of the services	9

4.6	Multiple logical address spaces	9

4.7	Differences among ESS, PBSS, and IBSS LANs	10

4.8	Differences between ESS and MBSS LANs	10

4.9	Reference model	10

4.10	IEEE Std 802.11 and IEEE Std 802.1X-2010	10

4.11	Generic advertisement service (GAS)	10

5	MAC service definition	10

5.1	Overview of MAC services	10

5.1.1	Data service	10

5.1.2	Security services	10

5.1.3	MSDU ordering	10

5.1.4	MSDU format	10

5.1.5	MAC data service architecture	11

5.2	MAC data service specification	11

6	Layer management	12

7	PHY service specification	12

8	Frame formats	12

8.1	General requirements	12

8.2	MAC frame formats	12

8.3	Format of individual frame types	12

8.3.1	Control frames	12

8.3.2	Data frames	12

8.3.2.1	Format of Management frames	12

8.3.2.2	Beacon frame format	12

8.3.2.3	ATIM frame format	13

8.3.2.4	Disassociation frame format	13

8.3.2.5	Association frame format	13

8.3.2.6	Reassociation Request frame format	13

8.3.2.7	Probe Request frame format	13

8.3.2.8	Probe Response frame format	13

8.3.2.9	Authentication frame format	14

8.3.2.10	Deauthentication frame format	14

8.3.2.11	Action frame format	14

8.3.2.12	Action No Ack frame format	14

8.3.2.13	Timing Advertisement frame format	14

8.3.3	Management frames	12

8.3.4	Extension frames	14

8.4	Management and Extension frame body components	15

8.4.1	Fields that are not elements	15

8.4.2	Elements	15

8.4.2.1	General	15

8.4.2.2	SSID element	15

8.4.2.30 TCLAS Element	15

8.4.2.157 GLK Capabilities element	16

8.4.2.158 GLK SSIDs element	16

8.4.3	Information Subelements	17

8.4.4	Access network query protocol (ANQP) elements	17

8.5	Fields used in Management and Extension frame bodies and Control frames	17

8.6	Action frame format details	17

8.7	Aggregate MPDU (A-MPDU)	17

9	MAC sublayer functional description	18

9.1	Introduction	18

9.2	MAC architecture	18

9.2.1	General	18

9.2.2	DCF	18

9.2.3	PCF	18

9.2.4	Hybrid coordination function (HCF)	18

9.2.4.2 HCF contention based channel access (EDCA)	18

9.2.5	Mesh coordination function (MCF)	19

9.2.6	Combined use of DCF, PCF, and HCF	19

9.2.7	MAC data service	19

9.3	DCF	20

9.4	PCF	20

9.5	Fragmentation	20

9.6	Defragmentation	20

9.7	Multirate support	20

9.8	MSDU transmission restrictions	20

9.9	HT Control field operation	20

9.10	Control Wrapper operation	20

9.11	A-MSDU operation	20

9.12	A-MPDU operation	20

9.13	PPDU duration constraint	20

9.14	DMG A-PPDU operation	20

9.15	LDPC operation	20

9.16	STBC operation	20

9.17	Short GI operation	20

9.18	Greenfield operation	20

9.19	Operation across regulatory domains	20

9.20	HCF	20

9.21	Mesh coordination function (MCF)	20

9.22	Block acknowledgement (block ack)	20

9.23	No Acknowledgement (No Ack)	21

9.24	Protection mechanisms	21

9.25	MAC frame processing	21

9.26	Reverse direction protocol	21

9.27	PSMP Operation	21

9.28	Sounding PPDUs	21

9.29	Link adaptation	21

9.30	Transmit beamforming	21

9.31	Antenna selection (ASEL)	21

9.32	Null data packet (NDP) sounding	21

9.33	Mesh forwarding framework	21

9.34	DMG channel access	21

9.35	DMG AP or PCP clustering	21

9.36	DMG beamforming	21

9.37	DMG block ack with flow control	21

9.38	DMG link adaptation	21

9.39	DMG dynamic tone pairing (DTP)	21

9.40	DMG relay operation	21

10	MLME	21

11	Security	21

12	Fast BSS transition	22

13	MLME Mesh procedures	22

13.1	Mesh STA dependencies	22

13.2	Mesh discovery	22

13.3	Mesh peering management (MPM)	22

13.4	Mesh peering management finite state machine (MPM FSM)	22

13.5	Authenticated mesh peering exchange (AMPE)	22

13.6	Mesh group key handshake	22

13.7	Mesh security	22

13.8	Mesh path selection and metric framework	22

13.9	Airtime link metric	22

13.10	Hybrid wireless mesh protocol (HWMP)	22

13.11	Interworking with the DS	22

13.11.1	Overview of interworking between a mesh BSS and a DS	22

13.11.2	Gate announcement (GANN)	23

13.11.3	Data forwarding at proxy mesh gates	23

13.11.4	Proxy information and proxy update	23

13.11.5	Mesh STA collocation	23

13.12	Intra-mesh congestion control	23

13.13	Synchronization and beaconing in MBSSs	23

13.14	Power save in mesh BSS	23

14	Frequency-Hopping spread spectrum (FHSS) PHY specification for the 2.4 GHz industrial, scientific, and medical (ISM) band	23

15	Infrared (IR) PHY specification	23

16	DSSS PHY specification for the 2.4 GHz band designated for ISM applications	23

17	High rate direct sequence spread spectrum (HR/DSSS) PHY specification	23

18	Orthogonal frequency division multiplexing (OFDM) PHY specification	23

19	Extended Rat PHY (ERP) specification	23

20	High Throughput (HT) PHY specification	24

21	Directional multi-gigabit (DMG) PHY specification	24

Annex A, Bibliography	24

Annex B, Protocol Implementation Conformance Statement (PICS)	24

Annex C, ASN.1 encoding of the MAC and PHY MIB	24

…	24

Annex P, Integration Function	24

P.1 Introduction	24

P.2 Ethernet V2.0/IEEE Std 802.3 LAN integration function	24

P.3 Example	24

P.4 Integration service versus bridging	25

…	25

Annex V, Interworking with external networks	25

V.1 General	25

V.2 Network discovery and selection	25

V.3 QoS mapping guidelines for interworking with external networks	25

V.3.3 Example of QoS mapping from different networks	25

V.4 Interworking and SSPN interface support	25

V.5 Interworking with external networks and emergency call support	25

V.6 Peer information	25

…	26

NOTE — The editing instructions contained in this amendment define how to merge the material contained therein into the existing base standard and its amendments to form the comprehensive standard.

The editing instructions are shown in bold italic. Four editing instructions are used: change, delete, insert, and replace. Change is used to make corrections in existing text or tables. The editing instructions specify the location of the change and describe what is being changed by using strike through (to remove old material) and underscore (to add new material). Delete removes existing material. Insert adds new material without disturbing the existing material. Insertions may require renumbering. If so, renumbering instructions are given in the editing instruction. Replace is used to make changes in figures or equations by removing the existing figure or equation and replacing it with a new one. Editorial notes will not be carried over into future editions because the changes will be incorporated into the base standard.

# Introduction

This section will not be included when P802.11ak is rolled into the base standard.)

IEEE Std 802.11 was originally designed with the assumption that non-AP non-mesh STAs would be leaf nodes of the network. This amendment extends the 802.11 standard so that communication between STAs can be used as a transit link inside a general network conformant to IEEE Std 802.1Q.

# Overview

## Scope

## Purpose

## Supplementary Information on Purpose

Add the following at the end of Clause 1.3:\

- Define the mechanisms for using IEEE 802.11 media as transit links in a bridged LAN.

## Word Usage

## Mathematical Usage

# Normative references

Insert the following references (maintaining alphabetic order):

IEEE Std 802.1AC-tbd, “Media Access Control (MAC) Service Definition”

IEEE Std 802.1Qbz™-tbd, “Virtual Bridged Local Area Networks — Amendment: Enhancements to Bridging of 802.11 Media”

# Definitions, acronyms, and abbreviations

## Definitions

Insert the following definitions (maintaining alphabetical order):

EtherType Protocol Discrimination (EPD): A frame format that uses an EtherType to identify the protocol of the following information.

LLC Protocol Discrimination (LPD): A frame format that uses a destination LSAP, a source LSAP, and a Control octet (LLC) to identify the protocol of the following information.

## Definitions specific to IEEE 802.11

Insert the following definition (maintaining alphabetical order):

General link (GLK): Communication between two stations (STAs) over the wireless medium that can be used as a link in the middle of an IEEE Std. 802.1Q conformant network.

## Abbreviations and acronyms

Insert the following acronyms (maintaining alphabetical order):

EPD			EtherType Protocol Discrimination

GLK			General Link

LPD			LLC Protocol Discrimination

VID			VLAN ID

# General Description

## General description of the architecture

## How wireless local area networks (WLANs) are different

### Introduction

### Wireless station (STA)

Change the first paragraph as follows:

In the design of wired LANs it is implicitly assumed that an address is equivalent to a physical location. In wireless networks, this is not always the case. In IEEE Std 802.11, the addressable unit is a station (STA). The term implies no more than the origin or/and destination of a message. Physical and operational characteristics are defined by modifiers that are placed in front of them STA. For example, in the case of location and mobility, the addressable units are the fixed STA, the portable STA, and the mobile STA. The STA is a message destination, but not (in general) a fixed location.

### Media impact on design and performance

### The impact of handling mobile STAs

### Interaction with other IEEE 802® layers

### Interaction with non-IEEE-802 protocols

## Components of the IEEE Std 802.11 architecture

## 4.3.5 Distribution system (DS) concepts

### 4.3.5.1 Overview

Add the following:

As a logical entity, the DS can be composed of any technology or mix of technologies providing the required services. Example technologies are IEEE 802.1Q bridges or IETF IP routers. Even for an AP without external network connectivity, a logical DS exists and frames that enter the AP form the WM and are then retransmitted by the AP on the WM logically transit the DS function.

### 4.3.13 STA transmission of Data frames outside the context of a BSS

Note: I am told that users of this service are anxious to save every bit they can. Thus it seems likely they will want to use 802.11ak (EtherType) formatted data frames.

Insert a new sub-Clause at the end of Clause 4.3 as follows:

### 4.3.21 General Link (GLK)

This is probably the right section for a high-level architecture GLK figure…

GLK STAs are extended non-GLK STAs such that a link between GLK STAs is suitable, insofar as the capabilities of 802.11 wireless permit, to be used as a transit link in the interior of an IEEE Std 802.1Q network. All non-GLK STAs use LPD and interpret Priority Code Points according to IEEE Std 802.1D while all GLK STAs use EPD and interpret Priority Code Points according to IEEE Std 802.1Q.

Every STA is either a GLK STA or a non-GLK STA. A GLK STA is also a QoS STA and an HT STA. GLK STAs advertise themselves as such and provide further information on their capabilities through the use of the GLK Capabilities and GLK SSIDs elements in Beacons, Probes, Probe Responses, and DMG Beacons.

A GLK AP uses the wildcard SSID to assures that non-GLK STAs will not try to associate with it. A GLK AP advertises its actual SSID in the GLK SSIDs element. Should a non-GLK STA attempt to associate with a GLK AP, the GLK AP will refuse the association.

A GLK STA shall not attempt to form an infrastructure, IBSS, or PBSS association or mesh peering with any non-GLK STA.

Note: The above provisions should work well to stop non-GLK STAs from associating with GLK APs but I don’t think it will stop a non-GLK Mesh STA from trying to peer with a GLK Mesh STA. We need a bit more.

## Logical service interfaces

### General

Change text as follows:

A DS may be created from many different technologies or combinations of technologies including current IEEE 802.1Q virtual bridging wired LANs or IETF IP routing. IEEE Std 802.11 does not constrain the DS to be either data link or network layer based. Nor does IEEE Std 802.11 constrain a DS to be either centralized or distributed in nature.

### SS

### PBSS control point service (PCPS)

### DSS

## Overview of the services

May need something in 4.5.3.3 and 4.5.3.5 (and/or 4.5.4.2 and 4.5.4.3) about association/disassociation causing bridge ports to go up/down.

May need something in 4.5.9 (Interworking with external networks).

## Multiple logical address spaces

May also need mention of VLANs…

Change text as follows:

The IEEE Std 802.11 choice of address space implies that for many instantiations of the IEEE Std 802.11 architecture, the wired LAN MAC address space and the IEEE Std 802.11 MAC address space might be the same. In those situations where a DS that uses MAC level IEEE 802 addressing is appropriate, for example where the DS is an IEEE 802.1Q bridged LAN, all three of the logical address spaces used within a system could be identical. While this is a common case, it is not the only combination allowed by the architecture. The IEEE Std 802.11 architecture allows for all three logical address spaces to be distinct.

## Differences among ESS, PBSS, and IBSS LANs

May need to include GLK case in Figure 4-14, IEEE Std 802.11 architecture (again).

## Differences between ESS and MBSS LANs

## Reference model

May need to include GLK case in Figure 4-19, ESS link illustration.

## IEEE Std 802.11 and IEEE Std 802.1X-2010

## Generic advertisement service (GAS)

# MAC service definition

## Overview of MAC services

### Data service

Note: In 802.11, the priority for an MSPU sent by a QoS STA is encoded into the TID (TID 0 to 7) or into a TSPEC referenced by the TID (TID 8 to 15). If a frame is tagged, should the TID default to the priority in the tag with some facility for mapping or the like?

Change the first sentence of Clause 5.1.1.2 as follows:

The QoS facility supports eight priority values, referred to as UPs. The values a UP may take are the integer values from 0 to 7 and are identical to the IEEE Std 802.1D priority tags values for non-GLK STAs and to the IEEE Std 802.1Q priority values for GLK STAs.

### Security services

### MSDU ordering

### MSDU format

Change Clause 5.1.4 as follows:

This standard is part of the IEEE 802 family of LAN standards, and as such All all MSDUs sent by non-GLK STAs use LPD are LLC PDUs as defined in IEEE Std 802.1QbzISO/IEC 8802-2: 1998. In order to achieve interoperability between non-GLK STAs and networks using EPD, implementers are recommended to apply the procedures described in ISO/IEC Technical Report 11802-5:1997(E) (previously known as IEEE Std 802.1H-1997 [B21]), along with a selective translation table (STT) that handles a few specific network protocols, with specific attention to the operations required when passing MSDUs to or from LANs or operating system components that use EPD the Ethernet frame format. Note that such translations might be required in a STA. All GLK STA MSDUs use EPD as specified in IEEE Std 802.1Qbz.

### MAC data service architecture

Change text as follows:

During reception, a received Data frame goes through processes of possible A-MPDU deaggregation, MPDU header and cyclic redundancy code (CRC) validation, duplicate removal, possible reordering if the block ack mechanism is used, decryption, defragmentation, integrity checking, and replay detection. After replay detection (or defragmentation if security is not used), possible A-MSDU deaggregation, and possible MSDU rate limiting, one or more MSDUs are, delivered to the MAC_SAP or to the DS. When transparent FST is used, MSDUs originating from different PHY-SAPs go through an additional transparent FST entity that contains a multiplexing process before forwarding the MSDU to the MSDU rate limiting process. The IEEE 802.1X Controlled/Uncontrolled Ports discards any received MSDU if the Controlled Port is not enabled. The Uncontrolled Port admits the frame for use if it is and if the MSDU does not represent an IEEE Std 802.1X frame and optionally for other protocols that use the Uncontrolled Port. Frame order enforcement provided by the enhanced data cryptographic encapsulation mechanisms occurs after decryption, but prior to MSDU defragmentation; therefore, defragmentation fails if MPDUs arrive out of order

## MAC data service specification

Note: Should Drop Eligibility be added to the service primitive interfaces?

To cover the GLK case, we need to model as an AP port per associated STA or primitives below need to be enhanced with a list of receiving STAs or new parallel primitives specified.

### General

### MA-UNITDATA.request

Needs

### MA-UNITDATA.indication

### MA-UNITDATA-STATUS.indication

# Layer management

Need appropriate additional parameter and corresponding changes in 6.3.3, Scan (both request and confirm) and 6.3.11, MLME-START.request. Might also need something in 6.3.4 MLME-JOIN, (request and confirm), 6.3.7 MLME-ASSOCIATE, and MLME-REASSOCIATE.

# PHY service specification

# Frame formats

## General requirements

## MAC frame formats

## Format of individual frame types

### Control frames

### Data frames

### Management frames

#### Format of Management frames

#### Beacon frame format

Change the indicated rows in Table 8-24 – Beacon frame body and add two new rows:

| 4 | Service Set Identifier (SSID) | If dot11MeshActivated or dot11GeneralLink is true, the SSID element is the wildcard value as described in 8.4.2.2 (SSID element). |
| --- | --- | --- |
| 28 | Multiple BSSID | One or more Multiple BSSID elements are present if dot11RMMeasurementPilotActivated is a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 10.11.14 (Multiple BSSID Set)) with two or more members, or if dot11MultiBSSIDActivated is true, or if dot11InterworkingServiceActivated is true and the AP is a member of a Multiple BSSID Set with two or more members and at least one dot11GASAdvertisementID MIB attribute exists. However, if dot11GeneralLink is true, the preceding references to the occurrence of the Multiple BSSID element refer to the Multiple BSSID subelement of the GLK SSIDs element. |
| 57 | GLK Capabilities | The GLK Capabilities element is present if dot11GeneralLink is true. |
| 58 | GLK SSIDs | The GLK SSIDs element is present when dot11GeneralLink is true if, assuming the STA was non-GLK, the Multiple BSSID element or a non-wildcard SSID element would be present. In those cases a Multiple BSSID subelement and/or SSID subelement are present in the GLK SSIDs element. |

#### ATIM frame format

#### Disassociation frame format

#### Association frame format

#### Association Response frame format

#### Reassociation Request frame format

#### Reassociation Response frame format

#### Probe Request frame format

Change the indicated rows in Table 8-30 – Probe Request frame body and add one new row as indicated:

| 1 | Service Set Identifier (SSID) | If dot11MeshActivated or dot11GeneralLink is true, the SSID element is the wildcard value as described in 8.4.2.2 (SSID element). |
| --- | --- | --- |
| 10 | SSID List | The SSID List element is optionally present if dot11SSIDListActivated is true and dot11GeneralLink is false. |
| 58 | GLK SSIDs | The GLK SSIDs element is optionally present if both dot11SSIDListActivated and dot11GeneralLink are true. In that case the GLK SSIDs element contains an SSID List subelement. |

#### Probe Response frame format

Change the indicated rows in Table 8-24 – Probe Response frame body and add two new rows:

| 4 | Service Set Identifier (SSID) | If dot11MeshActivated or dot11GeneralLink is true, the SSID element is the wildcard value as described in 8.4.2.2 (SSID element). |
| --- | --- | --- |
| 21 | Multiple BSSID | One or more Multiple BSSID elements are present if dot11RMMeasurementPilotActivated is a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 10.11.14 (Multiple BSSID Set)) with two or more members, or if dot11MultiBSSIDActivated is true, or if dot11InterworkingServiceActivated is true and the AP is a member of a Multiple BSSID Set with two or more members and at least one dot11GASAdvertisementID MIB attribute exists. However, if dot11GeneralLink is true, the preceding references to the occurrence of the Multiple BSSID element refer to the Multiple BSSID subelement of the GLK SSIDs element. |
| 57 | GLK Capabilities | The GLK Capabilities element is present if dot11GeneralLink is true. |
| 58 | GLK SSIDs | The GLK SSIDs element is present when dot11GeneralLink is true if, assuming the STA was non-GLK, the Multiple BSSID element or a non-wildcard SSID element would be present. In those cases a Multiple BSSID subelement and/or SSID subelement are present in the GLK SSIDs element. |

#### Authentication frame format

#### Deauthentication frame format

#### Action frame format

#### Action No Ack frame format

#### Timing Advertisement frame format

### Extension frames

#### DMG Beacon

Change the indicated row in Table 8-38 – DMG Beacon frame body and add two new rows:

| 10 | Multiple BSSID | One or more Multiple BSSID elements are present if dot11RMMeasurementPilotActivated is a value between 2 and 7 and the AP is a member of a Multiple BSSID Set (see 10.11.14 (Multiple BSSID Set)) with two or more members, or if dot11MultiBSSIDActivated is true, or if dot11InterworkingServiceActivated is true and the AP is a member of a Multiple BSSID Set with two or more members and at least one dot11GASAdvertisementID MIB attribute exists. However, if dot11GeneralLink is true, the preceding references to the occurrence of the BSSID element refer to the BSSID subelement of the GLK SSIDs element. |
| --- | --- | --- |
| 15 | GLK Capabilities | The GLK Capabilities element is present if dot11GeneralLink is true. |
| 16 | GLK SSIDs | The GLK SSIDs element is present when dot11GeneralLink is true if, assuming the STA was non-GLK, the Multiple BSSID element or a non-wildcard SSID element would be present. In those cases a Multiple BSSID subelement and/or SSID subelement are present in the GLK SSIDs element. |

## Management and Extension frame body components

### Fields that are not elements

### Elements

#### General

Add the following to Table 8-62 – Element IDs maintaining order by Element ID number:

| GLK Capabilities (see 8.4.2.166 (GLK Capabilities element)) | <ANA> | Yes |
| --- | --- | --- |
| GLK SSIDs (see 8.4.2.167 (GLK SSIDs element)) | <ANA> | Subelements |

#### SSID element

Change as follows:

The wildcard SSID is also used in Beacon and Probe Response frames transmitted by mesh STAs and GLK STAs.

#### 8.4.2.30 TCLAS Element

Note: PCP used below and in 802.1 means Priority Code Point but in 802.11 it means PBSS Control Point. I have expanded or dropped PCP to avoid this conflict.

Change text in Clause 8.4.2.30 as follows:

For Classifier Type 5, the classifier parameters are the following parameters in an IEEE Std 802.1D/Q-2003 [B22] tag header: Priority Code Point (PCP; equivalent to IEEE Std 802.1D/Q-2004 [B20] User Priority), Canonical Format Indicator (CFI) Drop Eligibility Indicator (DEI), and VLAN ID (VID).

Change Figure 8-238 as follows:

Classifier Type (5)

Octets:                    1                            1                                 1                             1                      1

Figure 8-238—Frame Classifier field of Classifier Type 5

Classifier Mask

802.1Q PCP Priority

Code Point

802.1Q

CFI DEI

802.1Q VID

Change text in Clause 8.4.2.30 as follows:

The PCP Priority Code Point subfield contains the value in the 4 LSBs; the 4 MSBs are reserved.

The CFI DEI subfield contains the value in the LSB; the 7 MSBs are reserved.

Add the following new Clause:

#### 8.4.2.166 GLK Capabilities element

The presence of the GLK Capabilities element, as shown in Figure 8-547a, in a Beacon, Probe, Probe Response, Associate, Association Response, Re-Associate, Reassociation Response, Mesh Peering Open, or Mesh Peering Confirm indicates that the transmitting STA is a GLK STA (and therefore uses the EPD MSDU format) and indicates by non-zero bits in the flag octet whether or not that STA is an AP and what additional GLK capabilities it may have if any. If the STA is a GLK AP or is sending a Probe message for APs, the SSID or SSIDs are provided in the GLK SSIDs element defined in 8.4.2.167.

Element ID           Length          GLK Capability Flags

Octets:                1                        1                              1

Figure 8-547a – GLK Capabilities element format

The Element ID and Length fields are defined in 8.4.2.1.

The Length field for this element is 1.

The flag bits are as show in Figure 8-547b.

GLK AP                Reserved

Bits:                    1                             7

Figure 8-547b – GLK Capability Flags field format

B0                 B1                       B7

The GLK AP bit indicates that the sending STA is an AP. If the GLK AP bit is zero, then the sending STA is a non-AP GLK STA.

#### 8.4.2.167	 GLK SSIDs element

The SSID or SSIDs of a GLK AP or for which a GLK STA Probe message is probing are indicated through subelements of the GLK SSIDs element. Enclosure of this SSID information in the GLK SSIDs element is intended to protect it from detection by non-GLK STAs.

Element ID          Length          Subelements

Octets:               1                         1                     0-255

Figure 8-547c – GLK SSIDs element format

The Element ID and Length fields are defined in 8.4.2.1.

The Length field for this element is variable depending on the subelements present.

Subelement allowed are shown in Table 8-222a. The subelement IDs and format are the same as the elements of the same name.

Table 8-240a – Optional subelement IDs for GLK SSIDs

| Subelement ID | Name | Specification |
| --- | --- | --- |
| 0 | SSID | 8.4.2.2 |
| 1-70 | Reserved |  |
| 71 | Multiple BSSID | 8.4.2.45 |
| 72-83 | Reserved |  |
| 84 | SSID List | 8.4.2.72 |
| 85-220 | Reserved |  |
| 221 | Vendor Specific | 8.4.2.25 |
| 222-255 | Reserved |  |

### Information Subelements

### Access network query protocol (ANQP) elements

## Fields used in Management and Extension frame bodies and Control frames

## Action frame format details

## Aggregate MPDU (A-MPDU)

# MAC sublayer functional description

## Introduction

## MAC architecture

### General

### DCF

### PCF

### Hybrid coordination function (HCF)

#### 9.2.4.2 HCF contention based channel access (EDCA)

Table 9-1 is changed below by adding a column on the left and adding a second section to the table for the GLK case.

Replace Table 9-1 with the following:

| Type | Priority | UP | 802.1 | AC | Transmit queue | Transmit queue | Designation (informative) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Non-GLK (802.1D UP) | Lowest Highest | 1 | BK | AC_BK | BK | BK | Background |
|  |  | 2 | — | AC_BK | BK | BK | Background |
|  |  | 0 | BE | AC_BE | BE | BE | Best Effort |
|  |  | 3 | EE | AC_BE | BE | BE | Best Effort |
|  |  | 4 | CL | AC_VI | VI | A_VI | Video (alternate) |
|  |  | 5 | VI | AC_VI | VI | VI | Video |
|  |  | 6 | VO | AC_VO | VO | VO | Voice |
|  |  | 7 | NC | AC_VO | VO | A_VO | Voice (alternate) |
| GLK (802.1Q UP) | Lowest Highest | 1 | BK | AC_BK | BK | BK | Background |
|  |  | 0 | BE | AC_BE | BE | BE | Best Effort |
|  |  | 2 | EE | AC_BE | BE | BE | Best Effort |
|  |  | 3 | CA | AC_VI | VI | A_VI | Video (alternate) |
|  |  | 4 | VI | AC_VI | VI | VI | Video |
|  |  | 5 | VO | AC_VO | VO | VO | Voice |
|  |  | 6 | IC | AC_VO | VO | VO | Voice |
|  |  | 7 | NC | AC_VO | VO | A_VO | Voice (alternate) |

### Mesh coordination function (MCF)

### Combined use of DCF, PCF, and HCF

### Fragmentation/defragmentation overview

### MAC data service

## DCF

## PCF

## Fragmentation

## Defragmentation

## Multirate support

## MSDU transmission restrictions

## HT Control field operation

## Control Wrapper operation

## A-MSDU operation

## A-MPDU operation

## PPDU duration constraint

## DMG A-PPDU operation

## LDPC operation

## STBC operation

## Short GI operation

## Greenfield operation

## Group ID and partial AID in VHT PPDUs

## Operation across regulatory domains

## HCF

## Mesh coordination function (MCF)

## Block acknowledgement (block ack)

## No Acknowledgement (No Ack)

## Protection mechanisms

## MAC frame processing

## Reverse direction protocol

## PSMP Operation

## Sounding PPDUs

## Link adaptation

## Transmit beamforming

## Antenna selection (ASEL)

## Null data packet (NDP) sounding

## Mesh forwarding framework

## DMG channel access

## DMG AP or PCP clustering

## DMG beamforming

## DMG block ack with flow control

## DMG link adaptation

## DMG dynamic tone pairing (DTP)

## DMG relay operation

# MLME

# Security

# Fast BSS transition

# MLME Mesh procedures

## Mesh STA dependencies

## Mesh discovery

## Mesh peering management (MPM)

## Mesh peering management finite state machine (MPM FSM)

## Authenticated mesh peering exchange (AMPE)

## Mesh group key handshake

## Mesh security

## Mesh path selection and metric framework

## Airtime link metric

## Hybrid wireless mesh protocol (HWMP)

## Interworking with the DS

### Overview of interworking between a mesh BSS and a DS

Change first paragraph as follows:

A mesh STA that has access to a DS is called a mesh gate. Mesh STAs in an MBSS access the DS via the mesh gate. An MBSS functions like an IEEE 802 LAN segment that is compatible with IEEE Std 802.1D if the MBSS is composed of non-GLK mesh STAs and compatible with IEEE Std 802.1Q if the MBSS is composed of GLK mesh STAs. The MBSS appears as a single access domain.

### Gate announcement (GANN)

### Data forwarding at proxy mesh gates

### Proxy information and proxy update

### Mesh STA collocation

## Intra-mesh congestion control

## Synchronization and beaconing in MBSSs

## Power save in mesh BSS

# Frequency-Hopping spread spectrum (FHSS) PHY specification for the 2.4 GHz industrial, scientific, and medical (ISM) band

# Infrared (IR) PHY specification

# DSSS PHY specification for the 2.4 GHz band designated for ISM applications

# High rate direct sequence spread spectrum (HR/DSSS) PHY specification

# Orthogonal frequency division multiplexing (OFDM) PHY specification

# Extended Rat PHY (ERP) specification

# High Throughput (HT) PHY specification

# Directional multi-gigabit (DMG) PHY specification

# Very High Throughput IVHT) PHY

# Annex A, Bibliography

# Annex B, Protocol Implementation Conformance Statement (PICS)

Need to do something about the PICS.

# Annex C, ASN.1 encoding of the MAC and PHY MIB

Need to do something about the MIB.

# …

# Annex P, Integration Function

Note: More extensive changes in Annex P may be required.

## P.1 Introduction

Replace the contents of P.1 with the following:

The purpose of this annex is to guide the implementer of a non-GLK WLAN system that includes a portal that integrates the WLAN systems with a wired LAN. This annex does not apply to GLK WLAN systems.

## P.2 Ethernet V2.0/IEEE Std 802.3 LAN integration function

## P.3 Example

Change the second paragraph as follows:

In the tables below the rows that have a 81-00 Type/Length field value represent bridging between an Ethernet/IEEE Std 802.3 LAN and an IEEE Std 802.11 LAN. Both LANs are carrying VLAN-tagged MSDUs (User Priority=4, CFI-DEI=0, VLAN ID=1893).

## P.4 Integration service versus bridging

# …

# Annex V, Interworking with external networks

## V.1 General

## V.2 Network discovery and selection

## V.3 QoS mapping guidelines for interworking with external networks

### V.3.3 Example of QoS mapping from different networks

Change the first sentence of Clause V.3.3 as follows:

IEEE Std 802.1D/Q UPs map to EDCA ACs, as described in Table 9-1 (UP-to-AC mappings).

Table V-1 is changed below by adding a new column on the right and changing the headings of the two rightmost columns.

Change Table V-1 to the following:

| 3GPP QoS Information |  | DiffServ PHB | DSCP | QoS Requirement on GPRS Roaming Exchange |  |  |  | EDCA Access Category | UP (non-GLK 802.1D) | UP (GLK 802.1Q) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Traffic Class | THP |  |  | Max Delay | Max Jitter | MSDU Loss | MSDU Error Rate |  |  |  |
| Conversational | N/A | EF | 101110 | 20 ms | 5 ms | 0.5% | 1.0E-05 | AC_VO | 7, 6 | 7, 6, 5 |
| Streaming | N/A | AF41 | 100010 | 40 ms | 5 ms | 0.5% | 1.0E-05 | AC_VI | 5, 4 | 4, 3 |
| Interactive | 1 | AF31 | 011010 | 250 ms | N/A | 0.1% | 1.0E-07 | AC_BE | 3 | 2 |
|  | 2 | AF21 | 010010 | 300 ms | N/A | 0.1% | 1.0E-07 | AC_BE | 3 | 2 |
|  | 3 | AF11 | 001010 | 350 ms | N/A | 0.1% | 1.0E-07 | AC_BE | 0 | 0 |
| Background | N/A | BE | 000000 | 400 ms | N/A | 0.1% | 1.0E-07 | AC_BK | 2, 1 | 1 |

## V.4 Interworking and SSPN interface support

## V.5 Interworking with external networks and emergency call support

## V.6 Peer information

# …