IEEE P802.11
Wireless LANs

| TGah D1.0 Comment Resolutions for Clauses 24.1&24.3 – Part I |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-01-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | Email |
| Mingguang Xu | Marvell | 5488 Marvell Ln, Santa Clara, CA 95054 | 408-222-7766 | mxu@marvell.com |
| Hongyuan Zhang | Marvell |  |  | Hongyuan@marvell.com |
| Liwen Chu | Marvell |  |  |  |

Abstract: This document contains proposed resolutions for comments in Clauses 24.1, 24.3.1, 24.3.2, 24.3.3, 24.3.9, 24.3.10, 24.3.14, 24.3.16, and 24.3.17 from TGah D1.0 with the CIDs below. (Note that resolutions for CIDs #2562, #2689, #2690, and #2543 in Clauses 24.3.14, 24.3.16, and 24.3.17 are deferred.)

| Clause 24.1 | Clauses 24.3.1, 24.3.2, 24.3.3 | Clauses 24.3.9, 24.3.10 | Clauses 24.3.14, 24.3.16, 24.3.17 |
| --- | --- | --- | --- |
| - 1299 | - 1584 | - 2002 | - 1781 |
| - 1300 | - 1587 | - 1740 | - 2004 |
| - 2622 | - 1586 | - 1741 | - 2691 |
| - 1301 | - 1310 | - 1742 | - 2003 |
| - 1302 | - 1588 | - 1743 | - 1330 |
| - 2672 | - 1590 | - 1744 |  |
| - 1303 | - 1591 | - 1745 |  |
| - 2623 | - 1589 | - 1746 |  |
| - 2411 | - 1593 | - 1747 |  |
| - 2285 | - 2070 | - 1748 |  |
| - 1304 | - 1592 | - 1749 |  |
| - 1305 | - 1311 | - 1750 |  |
| - 1306 | - 1594 | - 1751 |  |
| - 2280 | - 2072 | - 1752 |  |
| - 1307 | - 1312 | - 1753 |  |
|  | - 1998 | - 1754 |  |
|  | - 1999 | - 1325 |  |
|  | - 2000 | - 1755 |  |
|  |  | - 1756 |  |
|  |  | - 1757 |  |
|  |  | - 1326 |  |
|  |  | - 2688 |  |
|  |  | - 2794 |  |
|  |  | - 1758 |  |
|  |  | - 1327 |  |

CIDs for Clause 24.1

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1299 | Adrian Stephens | 24.1.1 | 237.14 | "which in turn are based" – grammar | ".. is based" | Accept. |
| 1300 | Adrian Stephens | 24.1.1 | 237.15 | " The S1G PHY defines OFDM PHY" --grammar | " The S1G PHY defines an OFDM PHY" | Accept. |
| 2622 | Osama Aboulmagd | 24.1.1 | 237.18 | Need to indicate the MU transmissions are in the downlink direction only | Add downlink MU transmissions | Accept. |
| 1301 | Adrian Stephens | 24.1.1 | 237.22 | "NOTE--An S1G SU PPDU includes individually addressed and group addressed transmissions." Addressing is largly a function of the MAC. The PHY does not know the difference between these. | Remove this note. | Reject, current text is consistent with 11ac. 11ac Clause 22.1.1 has the same sentence, and here 11ah SU packet has the same address properties as that in 11ac. |
| 1302 | Adrian Stephens | 24.1.1 | 237.49 | "Greater than or equal to 2MHz long format PPDU" -- in what sense can PPDUs be compared | "long format PPDU of channel width <greater than or equal symbol> 2 MHz." Ditto at 237.64 | Revised, define new abbreviations for “greater than or equal to 2MHz long format”, “greater than or equal to 2MHz short format”, and “1 MHz format”. |
| 2672 | Ron Porat | 24.1.1 | 237.49 | propose to rename "greater than or equal long or short preamble formats" to something shorter and clearer as this also confuses with the greater than or equal to 2MHz PPDU |  | Revised, refer to CID 1302. |
| 1303 | Adrian Stephens | 24.1.1 | 237.51 | " Detect and decode SIG-A field of the greater than or equal to 2MHz long preamble." - in what sence can preamble be compared? | " Detect and decode SIG-A field of a long preamble with channel width greater than or equal to 2MHz." Ditto at 238.01 | Revised, refer to CID 1302. |
| 2623 | Osama Aboulmagd | 24.1.1 | 237.60 | same text is repeated two times | delete the repeated text. | Reject, the cited text refers to “AP” STA” and the previous paragragh refers to “Non-AP STA”. |
| 2411 | Mark Rison | 24.1.2 | 238.37 | I hope this document describes the S1G PHY, not the VHT PHY | Change "VHT PHY" to "S1G PHY" | Accept. |
| 2285 | Lisa Ward | 24.1.2 | 238.37 | It looks like a copy/paste error as it says under scope "The services provided to the MAC by the VHT PHY consist of the following protocol functions" but it should likely be the "..S1G PHY ..." | Change from: "The services provided to the MAC by the VHT PHY consist of the following protocol functions:" to "The services provided to the MAC by the S1G PHY consist of the following protocol functions:" | Accept, refer to CID 2411. |
| 1304 | Adrian Stephens | 24.1.2 | 238.44 | "These STAs support S1G PHY." - eh? Seeing as we are in the PHY subclause, this is not a blinding revelation. | Remove cited text. | Reject, current text is consistent with 11ac. Clause 22.1.2 in 11ac has similar sentences. |
| 1305 | Adrian Stephens | 24.1.3.3 | 239.7 | "discretion of the S1G-PHY-compliant developer." In what sense can a developer be compliant to a PHY? | delete "S1G-PHY-compliant". | Reject, current text is consistent with 11ac. Clause 22.1.3.3 in 11ac has a similar sentence. |
| 1306 | Adrian Stephens | 24.1.4 | 239.34 | "Greater than or equal to 2MHz short format" - in what sense can formats be compared? | "Short format with channel bandwidth greater than or equal to 2 MHz" Make similar change at line 40. If this term is in general use (which it might be), make this change globally, or define an abbreviation or shorter term (e.g. S1G_SHORT, S1G_LONG and S1G_1M) to avoid creating awkward grammar in sentences that reference it. | Revised, refer to CID 1302. |
| 2280 | Li Chia Choo | 24.1.4 | 239.34 | 2 MHz short and long formats are not defined | Relate the short and long format definition to the short and long preambles on lines 27-29, p. 34 or provide a reference to 24.3.2. | Revised, refer to CID 1302. |
| 1307 | Adrian Stephens | 24.1.4 | 239.55 | "An 1MHz format PPDU or a greater than or equal to 2MHz short format PPDU is only used as an S1G SU PPDU." passive voice considered dangerous. who does the using? How can a PPDU be used as a PPDU, seeing as it is already one? | Reword to something that makes more sense. | Revised, see text re-wording below. |

TGah editor: please make the following changes in Clause 24.1:

- On Page 237.14 (CID #1299):

…The S1G PHY is based on the VHT PHY defined in Clause 22, which in turn are is based on HT PHY defined in Clause 20 and the OFDM PHY defined in Clause 18.…

- On Page 237.15 (CID #1300):

…The S1G PHY defines an OFDM PHY…

- On Page 237.18 (CID #2622):

… The S1G PHY defines OFDM PHY in the sub-1GHz bands using narrower bandwidths than those in 2.4 GHz and 5 GHz bands, with maximum number of spacetime streams supported being four, and with support for downlink multi-user (MU) transmissions. An A downlink MU transmission supports up to four users with up to three space-time streams per user with the total number of space-time streams not exceeding four.…

- In Clause 3.3, on Page 1 (CID #1302): insert the following acronym definitions:

| S1G_1M | 1 MHz format |
| --- | --- |
| S1G_SHORT | greater than or equal to 2MHz short format |
| S1G_LONG | greater than or equal to 2 MHz long format |

- On Page 239.30 in Cluase 24.1.4 (CID #1302):

…

—1MHz format (S1G_1M) is used for 1MHz PPDU or 1MHz Duplicated PPDU, if (FORMAT=S1G and

CH_BANDWIDTH=CBW1) or FORMAT=S1G_DUP_1M. Support for the 1MHz format is

mandatory.

— Greater than or equal to 2MHz short format (S1G_SHORT) is used for 2MHz, 4MHz, 8MHz or 16MHz PPDU with short preamble format, if ((FORMAT=S1G and (CH_BANDWIDTH=CBW2 or CBW4 or CBW8 or CBW16)) or FORMAT=S1G_DUP_2M) and PREAMBLE_TYPE=S1G_SHORT_PREAMBLE. It is similar to the HT-greenfield format as in clause 20, which does not contain an omni-portion in the preamble. Support for the greater than or equal to 2MHz short format is mandatory.

— Greater than or equal to 2MHz long format (S1G_LONG) is used for 2MHz, 4MHz, 8MHz or 16MHz PPDU with long preamble format, …

- (CID #1302) Throughout the whole draft spec, replace all remaining occurrences of

“1MHz format” with “S1G_1M”,

“1MHz PPDU” with “S1G_1M PPDU”,

“1MHz preamble” with “S1G_1M preamble”,

“greater than or equal to 2MHz short format” with “S1G_SHORT”,

“greater than or equal to 2MHz short” with “S1G_SHORT”,

“greater than or equal to 2MHz long format” with “S1G_LONG”, and

“greater than or equal to 2MHz long” with “S1G_LONG”.

- On Page 238.37 (CID #2411): Replace “VHT” with “S1G”.

- On Page 239.55 (CID #1307):

…An S1G MU PPDU carries one or more PSDUs to one or more STAs. An 1MHz format PPDU or a greater than or equal to 2MHz short format PPDU is only used as an S1G SU PPDU, and shall only be an S1G_LONG PPDU.

…

CIDs for Clauses 24.3.1, 24.3.2, and 24.3.3

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1584 | Brian Hart | 24.3.1 | 256.41 | "(i.e.scrambled and coded)" is a gross oversimplication of what happens | Change to append with "etc" | Accept. |
| 1587 | Brian Hart | 24.3.2 | 256.48 | Need a better term than "greater than or equal to 2MHz short format" , etc | e.g. S1G 1MHz format, S1G short wide format and S1G long wide format. Or S1G_1F, S1G_SWF, S1G_LWF | Revised, refer to CID 1302. |
| 1586 | Brian Hart | 24.3.2 | 256.52 | Font size change from L49 to L52 | Use consistent font sizes. Also P275L30 | Accept. |
| 1310 | Adrian Stephens | 24.3.2 | 257.02 | "QBPSK,QBPSK" -- this modulation format was so good, they named it twice | "QBPSK" | Revised, see updated Figure 24-20 below. The modulation format is removed and “DGI” is replaced with “GI2”. |
| 1588 | Brian Hart | 24.3.2 | 257.02 | Fig 24-20 contains too much/too little detail re ":QBPSK QBPSK" | Either show SIG exploded into two subfields (SIG1/SIG2) and associate QBPSK with the appropriate subfield [preferred], or move "QBPSK QBPSK" elsewhere [not preferred] | Revised, refer to CID 1310. |
| 1590 | Brian Hart | 24.3.2 | 257.17 | Space between number and units is the practice in 802.11-2012 - e.g. "2 MHz" not "2MHz" | Add space - many many places | Accept. Should do this globally. |
| 1591 | Brian Hart | 24.3.2 | 257.20 | Fig 24-21 defines D-STF, D-LTF and SIG-B as inside the "Data Portion". This is confusingly similar to "Data field" elsewhere in 802.11 | Rename to "Post-Omni Portion" or Beamformable Portion" or "S1G portion" | Reject, the current definition is clear enough. “data portion” and “data field” are two concepts. |
| 1589 | Brian Hart | 24.3.2 | 257.23 | Fig 24-21 contains too much/too little detail re ":QBPSK BPSK" | Either show SIG exploded into two subfields (SIG-A1/SIG-A2) and associate QBPSK and BPSK with the appropriate subfield [preferred], or move "QBPSK BPSK" elsewhere [not preferred] | Revised, see updated Figure 24-21 below. The modulation format is removed and “DGI” is replaced with “GI2”. |
| 1593 | Brian Hart | 24.3.2 | 257.40 | Fig 24-22 contains too much/too little detail re "all BPSK" | r1/2, (MCS10) is not relevant here since thisi s about format detection - nor balanced with figure 24-20/21. Very unsure about rep2 too. Simplify to "BPSK, 6 symbols" | Revised, see updated Figure 24-22 below. The modulation format is removed and “DGI” is replaced with “GI2”. |
| 2070 | Jens Tingleff | 24.3.2 | 257.60 | Table 24-3 . Need entries for DGI, GI, LTS | Add definitions | Accept. |
| 1592 | Brian Hart | 24.3.2 | 257.62 | Table says "LTF" but diagrams have "LTF1". LTF2, LTFn | Align terms - e.g. "LTFx" ... xth long training field | Reject, current text is consistent with 11ac. The convention from 11ac (e.g., VHTLTF) is followed. |
| 1311 | Adrian Stephens | 24.3.2 | 258.12 | "The Data field carries the PSDU(s)" - not directly it doesn't. | "The Data field carries the PSDU(s) after scrambling and coding" | Reject, current text is consist with 11ac. 11ac has the same sentence. |
| 1594 | Brian Hart | 24.3.2 | 258.18 | "In an S1G NDP, the Data field is not present." - can an NDP be any of the three formats? | Be explicit or provide a cross-reference or both | Revised, add the reference Clause 24.3.11, where the S1G preamble format for NDPs is described. |
| 2072 | Jens Tingleff | 24.3.2 | 258.19 | Why more than one user in "short format" when table 24-1 (MU_SU entry), clause 24.1.4 and clause 24.3.8.2.1 only allows more than one user (and therefore more than one STS) for "long format?" | Tidy up (or explain) | Reject, the refered paragraph talks about number of LTF symbols instead of MU_SU. |
| 1312 | Adrian Stephens | 24.3.3 | 258.50 | "Replicate over multiple 1 MHz or 2MHz" – grammar | "Replicate over multiple 1 MHz or 2MHz widths" | Accept. |
| 1998 | Hongyuan Zhang | 24.3.3 | 259.04 | "S1G 1MHz non-repetition MCS"--there is no such a definition | replaced by 1MHz MCS0~9 | Accept. |
| 1999 | Hongyuan Zhang | 24.3.3 | 259.08 | It is better to use Nes=1 for all MCSs in 11ah, therefore when quoting the 11ac block diagrams, need to add the exception: with Nes=1. | as comment | Revised, see text revisions below. |
| 2000 | Hongyuan Zhang | 24.3.3 | 259.16 | 11ac draft 6.0 clause 22.3.3 added figures for VHT-SIGB in 160MHz and 80+80MHz, when quoting the 11ac block diagrams, need to adjust the 11ac Figure number, and also use separate Figures for SIGB in 2/4/8 MHz, and in 16MHz. | as comment | Revised, see text revisions below. |

TGah editor: please make the following changes in Clauses 24.3.1, 24.3.2, and 24.3.3:

- On Page 256.41 (CID #1584):

… During transmission, a PSDU (in the SU case) or one or more PSDUs (in the MU case) are processed (i.e., scrambled and coded, etc.) and appended to the PHY preamble to create the PPDU.  …

- On Page 256.52 (CID #1586): Adjust the font size in between Lines 49-52 to be consistent with the other part.

- On Page 257 (CID #1310): Replace Figure 24-20 with the following one:

- On Page 257 (CID #1589): Replace Figure 24-21 with the following one:

- On Page 257 (CID #1593): Replace Figure 24-22 with the following one:

- On Page 257.60 (CID #2070): Contatenate the following three rows to Table 24-3.

| GI | Guard interval |
| --- | --- |
| GI2 | Double guard interval |
| LTS | Long training symbol |

- (CID #1590) Throughout the whole spec, replace all occurrences of “1MHz” with “1 MHz”, “2MHz” with “2 MHz”, “4MHz” with “4 MHz”, “8MHz” with “8 MHz”, and “16MHz” with “16 MHz”.

- On Page 258.18 (CID #1594):

… The SIG-A, D-STF, D-LTF, and SIG-B fields exist only in S1G greater than or equal to 2MHz long format. In an S1G NDP, the Data field is not present (See Clause 24.3.11 for the S1G preamble format for NDPs).

…

- On Page 258.50 (CID #1312):

k) Replicate over multiple 1 MHz or 2MHz widths

- On Page 259.04 (CID #1998): Replace “non-repetition MCSs” with “MCS0~9”.

- On Page 259.19 (CID #1999 and CID #2000):

…

Figure 22-6 and Figure 22-7, with “20MHz” replaced by “2MHz”, show the transmit process for generating

the SIG-B field of ana 2 MHz, 4MHz, and 8 MHz long format S1G PPDU in greater than or equal to 2MHz long format, for S1G SU PPDU and S1G MU PPDU, respectively.  Figure 22-8, with “160 MHz” replaced by “16 MHz”, shows the transmit process for generating the SIG-B field of a 16 MHz long format S1G SU PPDU.

Figure 22-810, when there is only one BCC encoder (NES=1), shows the transmitter blocks used to generate the BCC encoded Data field of a 2MHz, 4MHz and 8MHz SU PPDU in short or long format, and of a 1MHz PPDU except MCS10. A subset of these transmitter blocks consisting of the constellation mapper and CSD blocks, as well as the blocks to the right of, and including, the spatial mapping block, are also used to generate the STF and LTF fields in 1MHz format and greater than or equal to 2MHz short format, or the D-STF and D-LTF fields in greater than or

equal to 2MHz long format. This is illustrated in Figure 24-25 (Generation of LTF symbols) in subclause

24.3.8.2.1.3 (LTF definition) for the mentioned LTF fields. A similar set of transmit blocks is used to

generate the mentioned STF fields.

Figure 22-911 shows the transmitter blocks used to generate the LDPC encoded Data field of a 2MHz, 4MHz

and 8MHz SU PPDU in short or long format, and of a 1MHz PPDU except MCS10.

Figure 22-1012, when there is only one BCC encoder (NES=1), shows the transmit process for generating the Data field of a 2MHz, 4MHz and 8MHz MU PPDU in long format with BCC and/or LDPC encoding.

Figure 22-1113, when there is only one BCC encoder (NES=1),  and Figure 22-1214 show the transmit process for generating the Data field of a 16 MHz SU PPDU in short or long format, with BCC and LDPC encoding, respectively.

…

- On Page 275.30 (CID #1586): Adjust the font size of the two formulas in between Lines 27-42 to be consistent with other part.

CIDs for Clauses 24.3.9 and 24.3.10

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 2002 | Hongyuan Zhang | 24.3.9.1 | 312.30 | "Length/Duration field"--it is now named as Length Field | change to "Length field" | Accept. |
| 1740 | David Hunter | 24.3.9.4.3.1 | 313.38 | "For BCC encoder" needs an article. | Replace "For BCC encoder" with "For a BCC endoder". | Accept. |
| 1741 | David Hunter | 24.3.9.4.3.1 | 313.40 | "In the case of" here is a long way of saying "For" | Replace "In the case of" with "For". | Accept. |
| 1742 | David Hunter | 24.3.9.4.4.1 | 314.16 | "In the case of" here is a long way of saying "For" | Replace "In the case of" with "For" both here and on line 20 and delete the comma after "encoding" on both lines. | Revised, see text revisions below. The commas are not deleted according to 11ac conventions. |
| 1743 | David Hunter | 24.3.9.10 | 316.40 | "In the case of" here is a long way of saying "For" | Replace "In the case of" with "For" and delete the comma after "pilots". | Revised, see text revisions below. The comma is not deleted according to 11ac conventions. |
| 1744 | David Hunter | 24.3.9.10 | 316.42 | Per the IEEE Style Manual the introduction to items in a list is followed by a colon. | Replace "(24-45)." with "(24-45):" | Accept. |
| 1745 | David Hunter | 24.3.9.10 | 316.51 | "where" after an equation does not introduce a single item that needs indentation. | Place "where" on the following line (unindented). | Accept. |
| 1746 | David Hunter | 24.3.9.10 | 316.55 | "In the case of" here is a long way of saying "when" | Replace "in the case of fixed pilots," with "when the pilots are fixed," | Reject, current text is consist with 11ac. |
| 1747 | David Hunter | 24.3.9.10 | 316.57 | The title of the subclause is "Pilot subcarriers" | Replace "Subcarriers" with "subcarriers". | Accept. |
| 1748 | David Hunter | 24.3.9.10 | 316.59 | "In the case of" here is a long way of saying "when" | Replace "in the case of traveling pilots," with "When the pilots are traveling, then" | Reject, current text is consist with 11ac. |
| 1749 | David Hunter | 24.3.9.10 | 316.59 | Per the IEEE Style Manual the introduction to items in a list is followed by a colon. | Replace "as" with "as:" | Revised, see text revisions below. |
| 1750 | David Hunter | 24.3.9.10 | 317.01 | Since this "where" introduces another list, it needs to be followed by a colon. | Replace "where" with "where:". | Reject, current text is consist with 11ac. |
| 1751 | David Hunter | 24.3.9.10 | 317.09 | If an item in a list is a statement, it is followed by a period. | Replace "PPDU);" with "PPDU)." | Accept. |
| 1752 | David Hunter | 24.3.9.10 | 317.16 | Per the IEEE Style Manual the introduction to items in a list is followed by a colon. | Replace "specifically," with "specifically:". | Reject, current text is consist with 11ac. |
| 1753 | David Hunter | 24.3.9.10 | 317.36 | Since this sentence does not introduce a list, it needs to be followed by a period. | Replace "paragraphs:" with "paragraphs.". | Accept. |
| 1754 | David Hunter | 24.3.9.10 | 317.46 | Per the IEEE Style Manual the introduction to items in a list is followed by a colon. | Replace "index" with "index:". | Accept. |
| 1325 | Adrian Stephens | 24.3.9.10 | 320.57 | "there are totally 16 pilot subcarriers" - grammar | "there are a total of 16 pilot subcarriers" | Accept. |
| 1755 | David Hunter | 24.3.9.20 | 320.57 | "totally" is a bit too much like "totally, man" | Replace "totally" with "a total of". | Accept, refer to CID 1325. Note that the Section of this CID should be 24.3.9.10. |
| 1756 | David Hunter | 24.3.9.10 | 321.03 | "In the case of" here is a long way of saying "for" | Replace "In the case of single" with "for a single" and on line 5 replace "in the case of" with "for". | Accept. |
| 1757 | David Hunter | 24.3.9.11.1 | 321.07 | "In the case of" here is a long way of saying "for" | Replace "In the case of" with "for" and on line 8 replace "in the case of" with "for the". | Reject, not resolvable, cannot find the cited text. |
| 1326 | Adrian Stephens | 24.3.9.11.1 | 322.24 | Fonts in equations on this page are too big | Adjust to the correct size. | Accept. |
| 2688 | ron porat | 24.3.9.12.2 | 325.39 | correct typo - 1MHz should be 2MHz | replace 1MHz with 2MHz | Accept. |
| 2794 | Stefan Aust | 24.3.10.1 | 325.60 | Beamforming is only for 2,4,8, and 16MHz. The reason remains unclear why beamforming is not defined for 1MHz. | Reason should be added why 1MHz is not included. | Reject, the spec is not responsible for explanations. |
| 1758 | David Hunter | 24.3.10.2 | 326.11 | "in the case of" here is a long way of saying "when" | Replace "in the case of" with "when" | Reject, current text is consist with 11ac. |
| 1327 | Adrian Stephens | 24.3.10.2 | 326.38 | "When used for sounding, an NDP shall indicate multiple space-time streams in SIG field and use more than one LTF fields; when used for NDP MAC frames, an NDP shall use single space-time stream with one LTF field" How can an NDP be responsible "NDP shall" for anything. Passive voice considered dangerious. Is this a requirement on MAC behaviour, in selecting values of TXVECTOR parameters, or on PHY behaviour? | If this is a requirement on the MAC (which I suspect to be the case), move this statement into the MAC and relate to TXVECTOR parameters. Otherwise reword to avoid passive voice and identify the subject responsible for the shall statements. | Revised, see text re-wording below. |

TGah editor: please make the following changes in Clauses 24.3.9 and 24.3.10:

- On Page 312.30 (CID #2002): Replace “Length/Duration field” with “Length field”.

- On Page 313.38 (CID #1740): Replace "For BCC encoder" with "For a BCC endoder".

- On Page 313.40 (CID #1741): Replace "In the case of" with "For".

- On Page 314.16 (CID #1742): Replace "In the case of" with "For" both here and on 314.20.

- On Page 316.40 (CID #1743): Replace "In the case of" with "For".

- On Page 316.42 (CID #1744): Replace "(24-45)." with "(24-45):"

- On Page 316.51 (CID #1745): Place "where" on the following line (unindented).

- On Page 316.57 (CID #1747): Replace "Subcarriers" with "subcarriers".

- On Page 316.59 (CID #1749): Replace "as" with "as follows:".

- On Page 317.09 (CID #1751): Replace "PPDU);" with "PPDU)."

- On Page 317.36 (CID #1753): Replace "paragraphs:" with "paragraphs.".

- On Page 317.46 (CID #1754): Replace "index" with "index:".

- On Page 320.57 (CID #1325): Replace “totally” with “a total of”.

- On Page 321.03 (CID #1756): Replace "In the case of single" with "for a single".

- On Page 321.05 (CID #1756): Replace "in the case of" with "for".

- On Page 322.24 (CID #1326): Adjust the font sizes in the four equations on Page 322 to be consistent with the other part.

- On Page 325.39 (CID #2688): Replace “1MHz” with “2 MHz”.

- On Page 326.38 (CID #1327):

…

An NDP may be used for sounding or for NDP MAC frames. When used for In the case of NDP sounding, the SIG field in an NDP shall indicate multiple space-time streams in SIG field and use more than one LTF fields; when used for NDP MAC frames, an NDP shall use only contain single space-time stream with one LTF field.

…

- On Page 180.13 (CID #1327):

…

9.31.7 Transmission of an S1G NDP Sounding Frame

An S1G NDP Sounding frame shall use the 2 MHz short format as described in 24.1.4 (PPDU formats). An S1G STA shall transmit an S1G NDP Sounding frame using the following TXVECTOR parameters:

— APEP_LENGTH set to 0

— NUM_USERS set to 1

— CH_BANDWIDTH set to the same value as the TXVECTOR parameter CH_BANDWIDTH in the preceding S1G NDP Announcement frame

— NUM_STS indicates two or more space-time streams

— PARTIAL_AID are set as described in 9.17b (Group ID, partial AID, UPLINK and Color in S1G PPDUs)

— NDP_INDICATIONFRAME set to 0

— RESPONSE_INDICATION set to Long_Response

…

- On Page 212 (CID #1327), insert a new subclause at the end of Clause 9:

9.53 Transmission of an S1G NDP MAC Frame

An S1G STA shall transmit an S1G NDP MAC frame using the following TXVECTOR parameters:

— NDP_FRAME set to 1

— NUM_STS indicates one space-time stream

— APEP_LENGTH set to 0

— NUM_USERS set to 1

CIDs for Clauses 24.3.14, 24.3.16, and 24.3.17

| CID | Commenter | Section | Page | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- | --- |
| 1781 | Eugene Baik | 24.3.17.4.3 | 336.23 | The allowed relative constellation error for BPSK rate 1/2 rep 2 (MCS10) is currently TBD | Fill in a value in the entry in the table | Revised, comment has been resolved in 13/1367r1. |
| 2004 | Hongyuan Zhang | 24.3.17.4.3 | 336.23 | TBD needs to be filled up | will bring proposal | Revised, refer to CID 1781. |
| 2691 | ron porat | 24.3.17.4.3 | 336.23 | TBD needs value |  | Revised, refer to CID 1781. |
| 2003 | Hongyuan Zhang | 24.3.17.4.3 | 336.23 | "1/2 rep2"--there is no such a definition | change to "1/4 (1/2 with 2x repetition)" | Revised, see text revisions below. |
| 1330 | Adrian Stephens | 24.3.17.4.4 | 337.46 | "n.a." is "N/A" elsewhere | either delete "n.a." or add table note describing it's meaning. | Revised, replace “n.a.” with “N/A”. |

TGah editor: please make the following changes in Clauses 24.3.14, 24.3.16, and 24.3.17:

- On Page 336.23 (CID #2003): Replace “1/2 rep2” with “1/2 with 2x repetition”.

- On Page 338.56 (CID #2003): Replace “1/4 (1/2 with 2x Repetition)” with “1/2 with 2x repetition”.

- On Page 358.59 (CID #2003): Replace “1/4” with “1/2 with 2x repetition”.

- (CID #1330) Throughout the whole spec, replace all occurrences of “n.a.” with “N/A”.