IEEE P802.11
Wireless LANs

| 802.11 Some resolutions proposed to outstanding LB199 comments by the author |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-02-28 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |

Abstract

This document contains some proposed resolutions to comments from LB199.

R0 comments included:

- GEN (14): 2006, 2120, 2003, 2011, 2178, 2196, 2010, 2185, 2118, 2123, 2182, 2177, 2183, 2160

- MAC (35): 2074, 2075, 2087, 2090, 2091, 2070, 2071, 2092, 2096, 2080, 2105, 2083, 2126, 2148, 2094, 2049, 2051, 2058, 2063, 2079, 2086, 2089, 2153, 2093, 2100, 2101, 2102, 2162, 2109, 2129, 2138, 2158, 2160, 2167, 2157

- Reject (1):  2039 (Editor)

R1: added 2078.    + reviewed and updated during TGmc telecon 2014-02-28

# Easier Comments

| CID | Page | Clause | Resn Status | Comment | Proposed Change | Owning Ad-hoc | Difficulty 1=low |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2006 | 60.58 | 4.3.8 |  | "These transmissions may also be subject to certain channel access restrictions in the form of admissioncontrol. A DMG STA uses EDCA only within a contention-based access period (CBAP). Details of thismechanism are provided in9.20.2 (HCF contention-based channel access (EDCA)"What does "this mechanism" mean? Before the .11ad insert, it was clearly admission control. Now it is apparently EDCA. | Replace "this mechanism" with "admission control". | GEN | 1 |

Context: 60.56

These transmissions may also be subject to certain channel access restrictions in the form of admission control. A DMG STA uses EDCA only within a contention-based access period (CBAP). Details of this mechanism are provided in 9.20.2(HCF contention-based channel access (EDCA)) and, for DMG STAs, additional details are provided in 9.34.4 (DTI transmission rules), 9.34.5 (Contention-based access period (CBAP) transmission rules), and 9.34.6.3 (Contention-based access period (CBAP) allocation).

Discussion:   “This mechanism” is ambiguous because three mechanisms have been cited:  admission control, EDCA and CBAP.

Proposed Resolution:

Revised.   Replace “this mechanism” with “EDCA” at cited location.

| 2120 | 185.33 | 6.3.12.2.3 |  | "This primitive is generated by the SME to terminate an infrastructure BSS (with the MAC entity within anAP) or a PBSS (with the PCP entity within the MAC). " -- very curious. Following the AP example it should be "MAC entity within the PCP". | Swap "PCP" and "MAC" in the second parenthetical phrase. | GEN | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 185.31 & proposed change

6.3.12.2.3 When generated This primitive is generated by the SME to terminate an infrastructure BSS (with the MAC entity within an AP) or a PBSS (with the MAC entity within the PCP). The MLME-STOP.request primitive shall be generated only after successful use ofan MLME-START.confirm primitive.

Discussion:

A PCP is a type of STA,  or it might contain a STA (like and AP).  But regardless of which of these is the case (and we can argue about that in TGmc till we are blue in the collective face),  certainly the MAC is contained within a STA and is therefore contained within the PCP.

Proposed resolution:

Accepted.

| 2003 | 363.41 | 6.3.68.6.2 |  | "stream that is cancelled by the AP" | Should probably say by the AP or mesh STA.Ditto at 364.28. | GEN | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The question is whether GATs can be used by mesh STAs or not.

This is answered at 1559: “GCR service may be provided by the AP to associated STAs in an

infrastructure BSS or by a mesh STA to its peermesh STAs in a mesh BSS.”

And at 1556: “The group addressed transmission service provides delivery of group addressed frames and comprises the two services, DMS and GCR”

Therefore mesh STAs support GATs,  and the primitive descriptions should reflect this.

Note, figure 6-26 is internally inconsistent.   It shows “AP or mesh STA” on the right, but appears to be specific to DMS, which is not supported by mesh.   The  GCR setup exchanges do not mirror the DMS ones,  so we cannot make the figure “generic” to describe both.

Resolution:

Revised.

In figure 6-26:

Change “AP or mesh STA” to “AP”.

Throughout 6.3.68.2-6.3.68.7, replace “AP” by “AP or peer mesh STA” and to replace “non-AP STA” with “non-AP STA or peer mesh STA”.  (excluding “peer” where not appropriate in context).

At 360.22 change “requested DMS stream” to “requested DMS or GCR stream”.

| 2011 | 489.48 | 6.5.9.5.5 |  | "for reference .. shall be" makes no sense" | Reword so this is either an example or a normative requirement, but not both. | GEN | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 489.48

The PN generator is seeded with a 1 in the first stage and a 0 in all other stages at the beginning of the first packet transmitted after a PLME-DMGTESTOUTPUT.request primitive. This means, for reference, that the first 64 payload octets (prior to scrambling) shall be 0x000: 00 00 42 00 08 20 20 84 90 00 00 4A 00 28 24 A0 0x010: 84 92 42 08 43 00 88 20 20 C6 90 08 20 6A 84 B8 0x020: 24 A0 CE 92 6A 2C E3 84 1A 62 28 85 90 80 00 4A 0x030: 42 28 2C 80 A4 16 D2 08 43 4A 88 08 04 66 14 9A

Proposed resolution:

Revised. Change “This means, for reference, that the first 64 payload octets (prior to scrambling) shall be” to “NOTE--The first 64 payload octets (prior to scrambling) are” and reformat lines 53-57 using the smaller NOTE font.

| 2074 | 1228.36 | 9.26.5 |  | "The RD responder can set the RDG/More PPDU subfield to 1 in response to a frame sent by the RD initiator that has the RDG/More PPDU subfield equal to 0.(11ad)"What normative specification supports this behavior? It is not an RD exchange unless the RD initiator transmits an RDG/More PPDU subfield set to 1.This note essentially says that any "response after sifs" can be turned into a response burst, even if the TXOP owner does not support RD. This is clearly wrong and probably breaks a whole lot of existing basic protocol (e.g. RTS/CTS/Data/Ack).Also, once a RD responder has transmitted RDG/More PPDU=0, it has given up "control of the medium". It can thereafter only respond to the TXOP holder. | Remove cited note, or replace it with the following:"NOTE--A STA that is not a TXOP holder does not transmit a frame with RDG/More PPDU set to 1, in response to a frame with RDG/More PPDU equal to 0. Once an RD responder transmits a frame with RDG/More PPDU set to 0 it ceases to be an RD responder and thereafter has no ability to use an earlier RD grant." | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1228.35

NOTE 4—The RD responder can set the RDG/More PPDU subfield to 1 in response to a frame sent by the RD initiator that has the RDG/More PPDU subfield equal to 0.

Discussion:

The purpose of a note is to describe existing behaviour.  But the setting of this bit is described only in the context of an RD initiator or RD responder.  When a non-TXOP holder receives a frame in which the RDG/More PPDU bit is 0,  it has not been given a grant and is therefore not an RD responder.

Proposed resolution:

Revised.  Remove cited NOTE.

| 2075 | 1269.24 | 9.34.1 |  | This equation needs to be set into less of a "paragraph equation" for readability. ceiling function is undefined, but ceil() funtion defined in 1.5 does not take two parameters. | Define short variables for things needing explanation or units, and add them to the variable list.Rename the function ceiling to avoid confusion with ceil() and define it either in the variable list or in 1.5.Ditto at 1227.49 | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1269.24

Context: 2.57 (Mathematical Usage)

Ceil (x), also written … the smallest integer larger than or equal to x. For example, Ceil (2.3) is 3 and Ceil (–2.3) is –2.

Discussion:

Your editor knows how to do all this except resolve the different semantics of “ceiling” vs “ceil”.

We can resolve this by defining semantics for the two-parameter version as shown below.

Note that a definition for this ceiling is given at 1278.10:

The function ceiling(x, y) shall return the value of x rounded towards positive infinity, with rounding to the nearest multiple of y.

This is equivalent to the definition introduced below,  but the wording of the proposed addition below it chosen to be similar to that of the existing Ceil().

Proposed resolution:

Revised.  At the cited location rework the equation into standard form by defining short variables for items needing explanation or units.   Replace “ceiling” with “Ceil”.

At 2.59,  at the end of the para add:

“The two parameter form, Ceil(x,y), is the smallest multiple of y larger than or equal to x.”

Replace “ceiling(“ by “Ceil(“ at 1277.50, 1283.53, 1978.36, 3114.20.

Delete definition of “ceiling” at 1278.10.

Insert “The one parameter form,” before “Ceil(x)” at 2.57.

| 2087 | 1279.36 | 9.34.6.6.2 |  | (From Editor Panel Review of D1.1) "A DMG STA that transmitted an RTS that established a DMG Protected Period shall transmit data framesduring the DMG Protected Period using the same antennaconfiguration as was used for the transmission ofthe RTS." -- Strict interpretation requires always the transmission of more than one data frame. | A DMG STA that transmitted an RTS frame that established a DMG Protected Period and that transmits a Data frameduring the DMG Protected Period shall use for the Data frame the same antenna configuration as was used for the transmission ofthe RTS frame. | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1279.36

A DMG STA that transmitted an RTS that established a DMG Protected Period shall transmit data frames during the DMG Protected Period using the same antenna configuration as was used for the transmission of the RTS.

Discussion:

The objection is reasonable.  The proposed replacement is slightly awkward.

Proposed resolution:

Revised.  Replace cited sentence with:

“A DMG STA that transmitted an RTS that established a DMG Protected Period shall use the same antenna configuration as was used for the transmission of the RTS during transmission of Data frame(s)

during the DMG Protected Period.”

| 2090 | 1283.51 | 9.34.7.2 |  | The equations in 9.34.7.2 need to be reset in the IEEE-SA style | Replace long "self documenting" variable names with short names, and document the meaning in a variable list.Rename the function ceiling to avoid confusion with ceil() and define it either in the variable list or in 1.5.Ditto at 1277.50(See my similar comment at 1269.24) | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1283.51

Context: 1277.50

Proposed Resolution:

Revised.

At the cited locations rework the equation into standard form by defining short variables for items needing explanation or units.

Replace “ceiling” with “Ceil”.

(Note to editor, CID 2075 makes non-conflicting changes to cited locations)

| 2091 | 1287.38 | 9.34.9 |  | "The PCP/AP shall not transmit an SPR frame if it wants to extend an SP in which it is the source. " - a normative statement based on a STA's wants. | Reword so that the normative statement is based on observable conditions. | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1287.38

The PCP/AP shall not transmit an SPR frame if it wants to extend an SP in which it is the source. A PCP/AP that extends an SP for which it is the source DMG STA shall transmit a Grant frame to the destination DMG STA of the SP to indicate the extension of the SP. The Duration field in the transmitted Grant frame shall be set to the remaining time in the SP plus any additional channel time allocated by the PCP/AP following the end of the SP. The Allocation Duration field of the Grant frame shall be set to the additional channel time allocated by the PCP/AP following the end of the SP.

Note also heavy over-use of “DMG STA”.   This is the subject of another comment.

Proposed resolution:

Revised. Replace first two sentences of cited para with:

“A PCP/AP may extend an SP for which it is the source DMG STA by transmitting a Grant frame to the destination DMG STA of the SP. The Grant frame indicates extension of the SP.”

| 2070 | 1290.35 | 9.35.1 |  | (From Editor Panel Review of D1.1) "A PCP/AP that does not transmit the Clustering Control field is clustering disabled." -- this terminology is not used elsewhere. | Delete cited sentence. | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This is the only occurrence of “clustering disabled”.

Proposed resolution:

Accepted.

| 2071 | 1290.38 | 9.35.1 |  | (From Editor Panel Review of D1.1) "Decentralized clustering enabled PCPs/APs operating on the same channel may form a decentralized PCP/AP cluster. A PCP/AP cluster includes one S-PCP/S-AP and zero or more member PCPs/APs. The ClusterID of the decentralized PCP/AP cluster shall be set to the MAC address of the S-PCP/S-AP." -- this is describing the special case before the general case | Swap first two sentences. | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 2092 | 1292.15 | 9.35.2.1 |  | (From Editor Panel Review of D1.1) The "ClusterTimeInterv" arrow is not defined anywhere. It is a hang-on from an early version of the .11ad draft. | Delete cited arrow. | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1292.01

Discussion:

This is the only occurrence of this term.

Proposed Resolution:

Accepted.

| 2096 | 1300.23 | 9.35.5 |  | (From Editor Panel Review of D1.1) "If a non-PCP/non-AP STA becomes a member PCP/AP of the clustering enabled by its current PCP/AP, the non-PCP/non-AP STA can synchronize scheduled CBAP allocations, if any, between the BSS in which it performs the role of PCP/AP and the BSS of its current PCP/AP." -- this is unclear in its meaning since 'clustering enabled' is a property of a PCP/AP and not something enabled by another PCP/AP. | Replace "of the clustering enabled by it current PCP/AP" with "of the cluster formed by its current PCP/AP". | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context and change proposed by comment:

If a non-PCP/non-AP STA becomes a member PCP/AP of the cluster formed by its current PCP/AP, the non-PCP/non-AP STA can synchronize scheduled CBAP allocations, if any, between the BSS in which it performs the role of PCP/AP and the BSS of its current PCP/AP. The non-PCP/non-AP STA can disallow STAs in the BSS in which it plays the role of PCP/AP from transmitting during the Beacon SPs of the cluster it is a part of, and this can be done by allocating anSP time-overlapping with each Beacon SP such that each allocated SP has both the source AID and destination AID fields within the Extended Schedule element set to the AID of the non-PCP/non-AP STA.

Proposed resolution:

Accepted

| CID | Page | Clause | Comment | Proposed Change | Owning Ad-hoc |
| --- | --- | --- | --- | --- | --- |
| 2078 | 1311.47 | 9.36.3.1 | (From Editor Panel Review of D1.1) The sentence starting with "A beam refinement transaction is" refines the definition of "beam refinement transaction" introduced at line 26. | Consider merging this para with the earlier para. | MAC |

Context: 1311.43:

The BC subphase is composed of either an R-BC subphase or an I-BC subphase or both, which are composed of transmission of BRP-RX packets to select a beam, followed by feedback. A beam refinement transaction is a set of BRP frames consisting of beam refinement requests and responses. A beam refinement request can be either a transmit beam refinement request or a receive beam refinement request or both.

Proposed Resolution:

Rejected.   Beam combining (BC) and the beam refinement transaction occupy different subphases of the BRP,  so it is not appropriate to merge this para.

| 2080 | 1314.05 | 9.36.3.2 |  | "or by setting both" -- is vague and unnecessary, especially with the sentence which follows. | Delete cited text | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1314.01

A DMG STA indicates that beam refinement transactions (9.36.6.4.2 (Beam refinement transaction)) occur by setting the L-RX field to a value greater than 0 to indicate the need for receive beam refinement or by setting the value of the TX-TRN-REQ field to 1 to indicate the need for transmit beam refinement or by setting both. The beam refinement transactions shall occur if at least one of these conditions is met.

Discussion:

It is redundant. Could delete highlighted text and change “or” above to “and/or”.  Can’t really delete following sentence because it contains a “shall”.   Overall no real benefit in terms of reduction of duplication from making the proposed change.

Proposed Resolution:

Rejected.   Removal of cited sentence might be read that these settings were alternatives, thus creating conflict with the following  sentence.

| 2105 | 1319.16 | 9.36.5.2 |  | "[0, dot11RSSBackoff), i.e., 0 inclusive through dot11RSSBackoff exclusive." compare with 1318.65: " [0, A-BFT Length), i.e., 0 to A-BFT Length- 1".How many different tortuous new ways can we find to express a range (a to b)? | Replace " 0 inclusive through dot11RSSBackoff exclusive" with "0 to dot11RSSBackoff-1"Make matching change at 1357.32 | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The only thing we need to understand here is that dot11RSSBackoff is an integer,  and therefore “dot11RSSBackoff exclusive” means “dot11RSSBackoff – 1”.

Proposed resolution:

Accepted.

| 2083 | 1319.56 | 9.36.5.2 |  | (From Editor Panel Review of D1.1) "Due to the multiple access nature of RSS in the A-BFT, the PCP/AP might not receive the best sector for communication with the STA. The PCP/AP may schedule an SP to perform BF again with the STA to find the best sector for communication with the STA."It is unclear what the normative effect of this statement is. | Turn into a "NOTE--" and change "may" to "might" | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 2126 | 1363.30 | 10.1.4.2.2 |  | "If at any time during the scan the DMG STA detects a non-DMG Beacon frame,..." -- "non-DMG Beacon frame" is ambiguous. Should be "frame that is not a DMG Beacon frame" | Replace cited text with: "If at any time during the scan the DMG STA detects a frame that is not a DMG Beacon frame," | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted.

| 2148 | 1423.44 | 10.4.1 |  | "When transmitted between DMG STAs" - the act of transmitting is not something that can be shared (unless this is a veiled reference to relay operation). | "between" -> "by" . Ditto line 47 (but not 45). | MAC | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1423.44

When transmitted between DMG STAs of a PBSS or DMG STAs of an infrastructure BSS, the TSPEC defined in 8.4.2.29 (TSPEC element) is used to create or modify a TS between members. When transmitted between DMG STAs of a PBSS or between DMG STAs of an infrastructure BSS, the TSPEC is referred to as a PTP TSPEC.

Discussion:

A TSPEC is always transmitted to an AP or PCP, or order to obtain service periods.  This is true for TSPECs where both source and destination are non-AP.  I think this condition is what is meant by “PTP TSPEC” at the cited location.

Proposed resolution:

Revised.

Replace cited para with:

“To communicate between STAs in a PBSS or between non-AP DMG STAs in an infrastructure BSS, a TSPEC (as defined in 8.4.2.29 (TSPEC element)) is used to create or modify a TS between those STAs.  This TSPEC is referred to as a PTP TSPEC.”

| 2178 | 2188.27 | 21.4.3.3.5 |  | definition for floor() is in 1.5, no need to repeat here | Remove definition of floor. | GEN | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Accepted

| 2196 | 2208.43 | 21.6.3.2.4 |  | The circle-plus operator is undefined | Add "where represents exclusive or" | GEN | 1 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.  Add “<circle-plus symbol> represents exclusive or” to the existing “where list” at cited location.

# More difficult comments

| 2010 | 180.10 | 6.3.11.2.2 |  | There is redundancy among some of the parameters of the START.request. Specifically the HT Operation parameters contains the HT BSS Basic rate, which is also a parameter. | Review the parameter list and remove any parameters that are embedded in some other parameter. | GEN | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

The START.request parameters are:

| Parameter | Analysis |
| --- | --- |
| SSID |  |
| BSSType |  |
| BeaconPeriod |  |
| DTIMPeriod |  |
| CF parameter set |  |
| PHY parameter set |  |
| IBSS parameter set |  |
| ProbeDelay |  |
| CapabilityInformation |  |
| BSSBasicRateSet | The set of rates to be supported by all STAs |
| OperationalRateSet | The STAs own supported rates |
| Country |  |
| IBSS DFS Recovery Interval |  |
| EDCAParameterSet |  |
| DSERegisteredLocation |  |
| HT Capabilities | Includes STA’s supported MCSs |
| HT Operation | Includes BSS’s basic MCSs |
| BSSMembershipSelectorSet | Adds to BSS Basic Rate Set |
| BSSBasicMCSSet | Redundant – already specified in HT Operation element |
| HTOperationalMCSSet | Possibly Redundant – see discussion. |
| Extended Capabilities |  |
| 20/40 BSS Coexistence |  |
| Overlapping BSS Scan Parameters |  |
| MultipleBSSID |  |
| InterworkingInfo |  |
| (#1713)AdvertisementProtocolInfo |  |
| RoamingConsortiumInfo |  |
| Mesh ID |  |
| Mesh Configuration |  |
| QMFPolicy |  |
| DMG Capabilities | Includes a supported MCS set. |
| Multi-band |  |
| MMS |  |
| DMG Operation | DMG doesn’t have the concept of a basic rate/mcs. It has the control MCS, plus mandatory MCSs. Control frames are sent using the control MCS. |
| Clustering Control |  |
| CBAP Only |  |
| PCP Association Ready |  |
| VendorSpecificInfo |  |

Discussion:

Redundancy exists around the BSSBasicMCSSet, which is present in the HT Operation element.

Redundancy possibly exists around the HTOperationalMCSSet, which duplicates information (supported MCSs) present in the HT Capabilities element.   In this case the picture is more complex, because we have differing models emerging.

- The OperationalRateSet parameter represents both a restriction on the devices transmit behaviour and the only thing known by other STAs about its receive capabilities.  The value of this Set is exposed by a read-only MIB variable.

- The HTOperationalMCSSet represents a restriction on a device’s transmit behaviour, independent of its capabilities.  The value of this parameter is related to (presumably set by) a related read-write MIB variable.   But the following are difficult:

  - “The STA’(#1485)s HTOperationalMCSSet shall include all of the MCSs in the BSSBasicMCSSet.” is a requirement on the external entity programming this MIB variable.

  - D2.0 182.16 “The STA shall be able to receive at each of the data rates listed in the set.” indicates that the .11n authors thought this was a receive capability, which it is not.

  - The BSS Description includes the HTOperationalMCSSet,  but this set is not transmitted in a Beacon frame,  so there is no way this can be known.

- .11ad doesn’t have this parameter.   It has mandatory MCSs and it has supported MCSs.

- VHT has yet another model.   Its OperationalVHTMCS_NSSSet provides a transmitter restriction, but is present only in the JOIN.request.  There is no related MIB variable.

I propose to rationalize this by removing the .11n and .11ac Operational MCS sets and related normative specification throughout the standard.

Todo: Delete BSSBasicMCSSet from the MLME-START.request and other redundant uses in Clause 6.

Changes: (Related to D2.4, so that .11ac changes are included)

Delete HTOperationalMCSSet table row at 140.25 (BSS Description), 147.12 (JOIN), 191.19 (START)

Delete HTOperationalMCSSet parameter at 146.30 (JOIN.request), 189.11 (START.request)

Delete OperationalVHTMCS_NSSSet parameter at 146.39 (JOIN)

Delete OperationalVHTMCS_NSSSet table row at 147.51.

Change 1219.30:

A STA that transmits a frame at a rate not specified by an MCS or a <VHT-MCS, NSS> tuple shall not transmit(#2142)the frame at a data rate higher than the greatest rate in the OperationalRateSet which is a parameter of the MLME-JOIN.request primitive.

Change 1614.56:

Change 2700.07: (dot11OperationalRateSet)

DESCRIPTION "This is a status variable. It is written by the SME when joining or establishing a BSS. This attribute specifies the set of non-HT data rates at which the station may transmit data Each octet contains a value representing a rate. Each rate is within the range from 2 to 127, corresponding to data rates in increments of 500 kbit/s from 1 Mb/s to 63.5 Mb/s, and is supported (as indicated in the supported rates table) for receiving data. This value is reported in transmitted Beacon, Probe Request, Probe Response, Association Request, Association Response, Reassociation Request, and Reassociation Response frames, and is used to determine whether a BSS with which the station desires to synchronize is suitable. It is also used when starting a BSS, as specified in 6.3.4 (Synchronization)."

Change 2766.48:

dot11HTOperationalMCSSet OBJECT-TYPE SYNTAX OCTET STRING (SIZE(1..127)) MAX-ACCESS read-write STATUS deprecated DESCRIPTION " Deprecated as no longer used by 802.11-<year> This is a control variable. It is written by an external management entity. Changes take effect for the next MLME-START.request primitive or MLMEJOIN.request primitive.

Create a new group dot11HTMACAdditions2 that is a copy of dot11HTMACAdditions, less the dot11HTOperationalMCSSet.  Deprecate dot11HTMACAdditions.  Change dot11HTMACAdditions to dot11HTMACAdditions2 in dot11Compliances.

| 2094 | 1294.21 | 9.35.2.2 |  | "may ignore DMG Beacon frames" -- what is the normative effect of "ignoring" something? Is this actually an exception to behaviour described elsewhere? | Add the exception to the material it is an exception to. | MAC | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There is a lot of behaviour related to the reception of a DMG Beacon.  It is unclear which of this behaviour is intended to be “ignored”.  Is it as thought the beacon were not received?   Or do we only ignore the beacon for the purpose of cluster management.

Brian originally proposed the following change:

A PCP/AP that receives at least one DMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 sent by an S-AP on every channel supported by the PCP/AP in the Operating Class within the most recent aMinChannelTime may ignore received ECPAC Policy Enforced subfields in DMG Beacon frames that have the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 sent by an from S-APs for 300×aMinChannelTime.

That doesn’t resolve the issue about “ignore” but does substantially narrow the scope.  There are quite a lot of normative references to this subfield, so we need to add exclusions.  To help we will define some terminology.

The changes proposed below resolve the “may ignore” inconsistency,  in a way consistent with Brian’s original proposed change.   One additional change is made below highlighted in yellow.   Brian explains this additional change as follows:

“ECPAC Policy Enforced tells the PCP/AP to join a centralized cluster or find a new channel. And there always has to be a channel available for the PCP/AP to go to and not worry about centralized clustering. If ECPAC Policy Enforced is true on all channels, then that is a bug with the centralized cluster(s) at the venue and the PCP/AP is no longer bound by that ECPAC Policy Enforced mandate (“there always has to be a channel available for the PCP/AP to go to and not worry about centralized clustering”). But the PCP/AP is still free to join one of these buggy centralized clusters if it wants to (“may”).”

Status:  investigate meaning of “and if the PCP/AP is decentralized PCP/AP clustering,” below.

Proposed resolution:

Revised.  Make changes under CID 2094 in <this-document>.  These changes call out the exceptions resulting from the “may ignore”.

Change 1294.18 as follows:

A PCP/AP that receives at least one DMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 sent by an S-AP on every channel supported by the PCP/AP in the Operating Class within the most recent aMinChannelTime ignores the ECPAC Policy Enforced subfield (i.e., treats this subfield as though equal to 0) in DMG Beacon frames received from S-APs for 300×aMinChannelTime. During this period the PCP/AP is called an ECPAC policy blind PCP/AP.

Change 1293.12 as follows:

If a centralized clustering enabled STA receives at least one DMG Beacon frame that has the ECPAC Policy Enforced field set to1 from an SAP/member PCP/member AP of another ECPAC during the monitoring period and the STA is not an ECPAC policy blind PCP/AP, the STA — Shall cease its activity on this channel and, if desired, attempt operation on a different channel, or — If one of the received DMG Beacon frames was sentby an S-AP, may elect to unenroll from its current CCSS and join the cluster ofthe S-AP as a member PCP/AP. If a centralized clustering enabled STA receives at least one DMG Beacon frame that has the ECPAC Policy Enforced field set to1 from an S-AP from the same CCSS during the monitoring period and the STA is not an ECPAC policy blind PCP/AP, the STA may elect either to unenroll from its current CCSS and join the cluster ofthe S-AP as a member PCP/AP or to continue and become an S-AP in the CCSS.

Change 1293.60 as follows:

A PCP/AP that - receives a DMG Beacon frame that has the ECPAC PolicyEnforced subfield in the DMG Parameters field set to 1 from an S-AP on a channel and - does not receive at least one DMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 from an S-AP on every channel supported by the PCP/AP in the Operating Class within the next aMinChannelTime and - is not an ECPAC policy blind PCP/AP shall either join the cluster of the S-AP as a member PCP/AP if centralized clustering enabled or cease its activity on this channel and, if desired, attempt operation on a different channel. S-APs within a CCSS report the channels unused by the ECPAC via the Channel Usage procedures (see 10.24.15 (Channel usage procedures)). A PCP/AP within a decentralized PCP/AP cluster that - receives a DMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 from an S-AP and - does not receive at least one DMG Beacon frame that has the ECPAC Policy Enforced subfield in the DMG Parameters field set to 1 from an S-AP on every channel supported bythe PCP/AP in the Operating Class within the next aMinChannelTime (i.e., does not become a ECPAC policy blind PCP/AP ) and - is not already an ECPAC policy blind PCP/AP shall quit the decentralized PCP/AP cluster beforethe next TBTT + beacon interval length, then the PCP/AP shall either join the cluster of the S-AP as a member PCP/AP if centralized PCP/AP clustering enabled or cease its activity on this channel and, if desired, attempt operation on a different channel.

Change 1297.17 as follows:

Monitor the channel for DMG Beacon frames for an interval of length at least aMinChannelTime. During this period: - if one or more DMG Beacon frames are received with the ECPAC Policy Enforced field set to 1 in the DMG Parameters field and the Cluster Member Role set to 1 (S-PCP/SAP of cluster) from one or more S-APs and the PCP/AP is not an ECPAC policy blind PCP/AP, then the PCP/AP shall join a selected S-AP as a cluster member as described in 9.35.2.2 (Centralized PCP/AP cluster formation) - otherwise, if the PCP/AP is an ECPAC policy blind PCP/AP then it may join a selected S-AP as a cluster member; - otherwise, if, after the period elapses, no DMG Beacon frames are received with the ECPAC Policy Enforced field in the DMG Parameters field set to 1 and the Cluster Member Role set to 1 (S-PCP/S-AP of cluster) and if the PCP/AP is decentralized PCP/AP clustering, then the PCP/AP shall attempt to join a decentralized PCP/AP cluster if present as described in 9.35.2.1(Decentralized PCP/AP cluster formation). If the PCP/AP is not decentralized PCP/AP clustering capable or a decentralized PCP/AP cluster is not present, then the PCP/AP shall set its Cluster Member Role to 0 (not currently participating in a cluster). In all three cases, the PCP/AP…

| 2185 |  |  |  | There are ~30 "shall ignore" statements.The danger with these, is that they are used to create exceptions to rules, without recording the exception in the rule itself.Or they are clarifications that no such requirement exists.For example: "If A then do B. If A and C, Ignore A."This should instead be "If A and not C, do B." | Review all "shall ignore" statements and replace them either:1. With an informative note " ... ignores ..."2. By adding an exception to a separate rule tha the "shall ignore" attempts to "override". | GEN | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

A “shall ignore” either creates conflict with a “shall” statement elsewhere, or it doesn’t (in which case it has no effect).  In either case something is rotten in the state of Denmark.  I guess the question is what do we mean by “elsewhere”:  same sentence,  same para,  same page,  same standard,  or same instance of multiverse?

A strict interpretation would be that any normative statement (usually a sentence) that conflicts with a distinct “shall ignore” needs to be fixed.

The general solution is to except the “shall ignore” condition from a normative statement elsewhere.   The “shall ignore” statements are now redundant.  They can be turned into NOTEs or deleted.

And we must also consider the “may ignore” and “should ignore” statements.

Status:  Check “shall be ignored”,  “may be ignored”,  “should be ignored”

Changes (shown with tracked changes):

(Shall ignore)

1132.42:

(No need for “shall ignore” because there are no matching rules that this creates an exception to.)

A CF-Pollable STA that receives an individually addressed Data frame of any subtype that includes CF-Poll may transmit one Data frame a SIFS after receiving the CF-Poll. NOTE--A CF-Pollable STA ignores and does not reset its NAV when performing transmissions in response to a CF-Poll.

1157.30: - no change

(does not create an exception, because this is the only place that describes parsing a Country element)

When dot11OperatingClassesRequired is true, orwhere operating classes domain information is present and the STA parsing a Country element finds an invalid First Channel Number field or Operating Class field with a value that is reserved, the STA shall ignore the remainder of the Country element and shall parse any remaining Management frame body for additional elements.

1224.19:

(does not create an exception)

A STA that encounters an unknown or reserved element ID value in a Management frame received without error shall ignore that element and shall parse any remaining management frame body for additional elements with recognizable element ID values.

1224.30:

(does not create an exception)

A STA receiving a vendor-specific element that it does not support shall ignore the vendor-specific element.

Status:  Mark not entirely happy with this

1224.48:

(does not create an exception. But general case expanded to cover vendor specific case.)

A STA that encounters an unknown, unsupported, or reserved subelement ID value contained in an element or subelement shall ignore the subelement with that subelement ID value and shall continue to parse any remaining element or subelement body for additional subelements with recognizable subelement ID values. A STA that receives an element or subelement for which a vendor-specific subelement is defined and that contains a vendor-specific subelement that it does not support shall ignore this vendor-specific subelement and shall continue to parse any remaining remaining element or subelement body for additional subelements with recognizable subelement ID values.

1274.09:

(Difficulty knowing how to evaluate “expected to participate”.

When receiving an Extended Schedule element containing a new pseudo-static allocation in which it is expected to participate, a non-PCP/non-AP STA ignores the allocation if the value of the TSF at the time the frame containing the Extended Schedule element is received is greater than the value of the TSF at the start of the pseudo-static allocation; this allocation is called an obsolete allocation. The value of the TSF at the start of the pseudo-static allocation is constructed using the value of the Allocation Start Time field within the Allocation field for the pseudostatic allocation.

And these related changes at 1274.40:

9.34.6.2 Service period (SP) allocation The PCP/AP shall set the AllocationType subfield to 0 in an Allocation field within an Extended Schedule element to indicate an SP allocation. An SP is assigned to the source DMG STA identified in the Source AID subfield in an Allocation field that is not an obsolete allocation within the Extended Schedule element. The source DMG STA shall initiate the frame exchange sequence that takes place during the SP at the start of the SP, except when the source DMG STA intends to establish a DMG Protected Period in which case the rules described in 9.34.6.6 (DMG Protected Period) shall be followed before the source DMG STA initiates the frame exchange in the SP. The SP allocation identifies the TC or TS for which the allocation is made; however, the type of traffic transmitted is not restricted to the specified TC or TS (10.4.1 (Introduction)). … Any MAC entity coordinated by an MM-SME that belongs to an MMSL cluster identified by the Source AID and Destination AID that are equal to, respectively, the Source AID and Destination AID of the Allocation field that is not an obsolete allocation in the Extended Schedule element that allocates the SP may transmit during the SP, if the STA sent an MMS element to the peer STA and the BeamLink Cluster field within the MMS element is 1. The PCP/AP may create SPs in its beacon interval with the source and destination AID subfields within an Allocation field set to 255 to prevent transmissions during specific periods in the beacon interval.

Status:  got to here in review 2014-0228

1359.62:

(The following two paras were the only ones related to behavior on receipt of the BSS Type field.  However there is the question of whether the BSS Type is passed up through the MLME SAP and how “shall ignore” is represented there.  I don’t feel confident to delete the “shall ignore” statement because I’m not sure what other implied effects there are.)

DMG STAs in an IBSS shall use other information in any received DMG Beacon (excluding those with the Discovery Mode field equal to 1) and Announce frames for which the BSS Type subfield is 1, the content of the SSID element is equal to the SSID of the IBSS, and the TSF value is later than the receiving STA’s TSF timer. Use of this information is specified in 10.1.5 (Adjusting STA timers). A STA shall ignore the BSS Type field contained in a received DMG Beacon frame if the Discovery Mode field within the DMG Beacon is 1.

1410.56:

(Fair enough, this is the only place we describe parsing by classes).,

Class 2 and Class 3 frames are not allowed in an IBSS. If a STA in an IBSS receives a Class 2 or Class 3 frame, it shall ignore the frame.

1477.01:

The receiving STA shall ignore the channel and measurement duration specified in the Beacon request when Beacon Table mode is selected.

Related text:  1475.09

(I have highlighted things that the STA “shall ignore”.  “Beacon Table mode” is shorthand for “Measurement Mode field in the measurement request”.

… When more than one Beacon or Probe Response from a BSS is received in the measurement duration, the contents of the Beacon (#1294)report shall be based on the latest received. If only Measurement Pilot frames were received in the measurement duration, the contents of the Beacon (#1294)report shall be based on the latest Measurement Pilot frame received. - regulatory domain. … Except when the Measurement Mode field is equal to Beacon Table, measurements shall be made using the specified Measurement Duration with the time between each consecutive measurement as defined in Error: Reference source not found. Iterative measurements shall cease when all channels have been measured. While the STA is processing a Beacon (#1294)request for iterative channel measurements, the STA shall not begin processing the next measurement request in the Measurement (#99)Request frame. …

1488.33:

(doesn’t create an exception)

If dot11RMNeighborReportActivated is false in an AP receiving a neighbor report request, it shall ignore the request and return a Neighbor Report frame withthe Incapable bit in the Measurement Report Mode field set to 1.

Likewise at 1488.58.

1488.38:

(while not creating an exception,  these statements are partly redundant given 1224.48.  However there is no general statement relating to vendor specific subelements to match 1224.30.   We have the choice of creating the general statement and deleting the specific instances,  or leaving the specific instances alone.  I’m in two minds,  but I suggest we create a good general statement.  The changes below do this.)

1506.03:

(I show changes to related normative text that this creates exceptions to.  But I’m not confident this represents all implied behavior.  Certainly it doesn’t cover the actual channel switch itself!   So I’ll leave the “shall ignore” in place and work to minimise contradictions.)

An HT STA that receives a channel switch announcement through both the Extended Channel Switch Announcement element and the Channel Switch Announcement element shall ignore the received Channel Switch Announcement element; this is called a superseded Channel Switch Announcement element.

Related changes:

1461.36:

A STA that receives a valid Channel Switch Announcement element that is not a superseded Channel Switch Announcement element (see 10.16.3.3) shall repeat this element in all Beacon frames and Probe Response frames that it transmits.

1461.56:

If the STA receives a valid Channel Switch Announcement element element that is not a superseded Channel Switch Announcement element (see 10.16.3.3) from another member of the IBSS, the STA shall leave DFS owner recovery mode prior to the channel switch and adopt the received channel switch information.

1524.12:

- TDLS direct-link teardown To tear down a direct link, a TDLS peer STA shall send a TDLS Teardown frame to the respective TDLS peer STA. A TDLS peer STA shall disable the direct link and destroy the related security parameters after successfully transmitting TDLS Teardown frame. If the STA has security enabled on the link with the AP, then the FTE shall be included in the TDLS Teardown frame. … If the TPK Handshake was successful for this TDLS session, then a receiving STA shall validate the MIC in the TDLS Teardown frame prior to processing the TDLS Teardown frame; a TDLS Teardown frame in which the MIC validation fails is called an invalid TDLS Teardown frame. A TDLS peer STA that receives a TDLS Teardown frame that is not an invalid TDLS Teardown frame shall disable the direct link and destroy the related security parameters..

1531.51:

(redundant)

Likewise at 1532.40, 1533.06

1580.45:

(not redundant – specific to GAS)

A STA that encounters an unknown vendor-specific OIfield or subfield in a GAS frame (see 8-257 (Public Action field values)) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

1580.59:

(not redundant – specific to ANQP Info ID)

A responding STA that encounters an unknown or reserved ANQP Info ID value in an Query List ANQP element received without error shall ignore that ANQP Info ID and shall parse any remaining ANQP Info IDs.

1711.20:

(could not find any conflicting behaviour)

A STA shall ignore suite selectors that it does not recognize

1720.51:

(The “shall ignore” statement below is unqualified and too broad.  Even so, what Beacon frames are ignored and when.  For example,  does a STA with a different security policy filter Beacon frames according to security policy when updating its TSF.  Jouni and Dan suggest that its intent is to ensure that every STA in the same IBSS supports the same security policy.  This is presumably determined by the STA starting the BSS and inspected by STAs joining a BSS.   This is already achieved by the “shall support” statement below.   Reworded as a NOTE.  Also willing to delete.)

A STA joining an IBSS shall support and advertise in the Beacon frame the security configuration of the IBSS, which includes the group cipher suite, advertised pairwise cipher suite, AKMP, and if management frame protection is enabled, Group Management Cipher Suite (see 11.5.5 (RSNA policy selection in an IBSS and for DLS)). The STA may use the Probe Request frame to discover the security policy of a STA, including additional individual cipher suites the STA supports. If enabled, management frame protection shall only be used as a required feature (MFPR) in an IBSS. NOTE—Because of the requirement for a STA joining an IBSS to support the security configuration of the IBSS, all Beacon frames transmitted in an IBSS have the same security policy.

1723.18:

(doesn’t create a conflict.  Such protection would be covered by the “unknown element ID” rule.)

A STA with dot11RSNAProtectedManagementFramesActivated equal to false shall transmit group addressed robust Management frames unprotected and shall ignore the protection on received group addressed robust Management frames.

1739.63:

(The deleted text is unnecessary, given 8.1.  We’ve generally removed these statements elsewhere.)

Reserved (bits 4–5).

1740.61:

(What it’s saying is that this field is reserved.

SMK Message (bit 13) specifies whether this EAPOL-Key frame is part of an SMK Handshake. If the SMK Handshake is not supported, the SMK Message subfield is reserved.

1741.01:

(The deleted text is unnecessary, given 8.1.  We’ve generally removed these statements elsewhere.)

Reserved (bits 14–15).

1743.28:

(I couldn’t find any obvious conflict)

A STA shall ignore any elements and KDEs it does not understand

1743.34:

(I couldn’t find any obvious conflict)

The padding consists of appending a single octet 0xdd followed by zero or more 0x00 octets. When processing a received EAPOL-Key frame, the receiver shall ignore this trailing padding.

Likewise 1832.15.

1775.21:

(I couldn’t find any obvious conflict.  However,  I do note that the status of the “shall ignore” is ambiguous.  It follows an “if” sentence and precedes and “oetherwise” sentence.   Is it part of the “if” condition?)

(May ignore)

1026.30:

(The intended sense is “might”,  as it makes no sense for a STA to set a bit to give itself permission to do something.)

The Preferred Candidate List Included (bit 0) field indicates whether the BSS transition candidate list included in this frame is a preferred candidatelist or a list of known BSS transition candidates. The Preferred Candidate List Included bit set to 0 indicates that the receiving STA might ignore the BSS Transition Candidate List Entries field. The Preferred Candidate List Included bit set to 1 indicates that the sender expects the receiving STA to process this frame.

1235.63:

(The multi-tid BA frame can include more data than was asked for, and the originating can is allowed to ignore the extra information.   Creates no conflict, because I can find no description of how the originator uses the information in a Multi-TID BlockAck.)

A Multi-TID BlockAck frame shall include all the TIDs for which data werereceived with Ack Policy field equal to PSMP Ack and for the TIDs listed in any Multi-TID BlockAckReq frame received during the previous PSMP-DTT (STA) or PSMP-UTT (AP). The originator may ignore the bitmap for TIDs in the Multi-TID BlockAck frame for which the originator has not requested a Multi-TID BlockAck frame to be present either implicitly (by the transmission of Data MPDUs with the Ack Policy field set to PSMP Ack) or explicitly (by the transmission of a Multi-TID BlockAckReq frame)

1249.35:

(This is a difficult one because the conflicting conditions are not fully enumerated.  It seems to me that if a calibration is ongoing, then the onus should be on the peer not to start a conflicting process.  However, creating any such rules might make existing equipment non-compliant.  So we create a new class of error condition in those processes, in which the request is received and perhaps acknowledged, but which is never completed.  This is just too complex for me to propose any changes.

I asked for PHY expert input, but didn’t get concrete suggestions for a fix,  more an indication that this conflict wasn’t of practical concern.)

A STA that has started but not completed the calibration procedure and that receives some other request that requires the buffering of CSI (such asanother calibration initiation frame, MFB request, CSI feedback request for link adaptation, or feedback request for explicit Transmit Beamforming) may ignore the other request.

1294.18:

(covered by CID 2094)

1366.37:

(This touches on a philosophical area.  Should the standard describe in normative terms what a STA does when it receives a frame that does not comply with the standard?   That is what is being done here.  Generally, I think the standard does not and should not describe the million and one ways it could be misused by a malfeasant STA.  The onus is on the implementer to extract the most value from this situation.  IMHO the best solution is for the STA to not care about element ID ordering,  unless it is somehow significant (and I can’t think of a reason why it might be).  I suppose this normative text was written to protect an implementation from doing dumb things to itself.)

Requested Element IDs in the Request element shall be included inthe Probe Response if the responding STA supports it. In the Probe Response frame, the STA shall return the requested elements in the same order as requested in the Request element.

1458.56:

(Creates internal conflict in the para, which is easy to resolve.  If we make the leap that failing to parse all the Measurement Requst elements is the same as ignoring the frame,  then we are OK.)

A STA that receives a Measurement Request frame from a STA in its BSS shall (except as follows) parse the frame’s Measurement Request elements in order, with measurements starting at the times specified by the Measurement Request elements. The exception is that the STA may ignore any group addressed Measurement Request frames.

1459.32:

(This is back to front.  It appears to permit a STA to disable a mandatory report and the AP to ignore that disablement.  It should have been written to disallow a STA from requesting something mandatory be disabled,  because that makes the most sense.   “All other requests shall be honoured” is really bad specmanship and hints that the description of response to disabled requests is incomplete elsewhere.  I’m not going to open that can of worms.   And what about the “may”.   Surely the AP “shall” ignore a request to disable a mandatory feature,  otherwise just what is the meaning of “mandatory”?  Status: This is the pits and I’m not going to do anything with it.)

A STA may enable or disable measurement requests or autonomous measurement reports from another STA by transmitting Measurement Request elements with the Enable bit set to 1 and the Request bit and Report bit set to 0 or 1, as appropriate. These elements do not require a corresponding Measurement Report element in a Measurement Report frame. All measurement requests and reports are enabled by default. A PCP/AP may ignore a request to disable a mandatory measurement request. All others requests shall be honored.

1464.59:

(The statement “may ignore and either stop or carry on” seems to grant complete license!  However it doesn’t create a conflict once the non-DMG infrastructure case is excluded – which it already should have been.)

A PCP/AP shall inform associated STAs that the PCP/AP is changing to a new channel and maintain the association by advertising the switch using the Extended Channel Switch Announcement element in DMG Beacon frames, Announce frames, and Information Response frames until the intended channel switch time. The channel switch should be scheduled so that all non-PCP/non-AP STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Extended Channel Switch Announcement element before the switch. A STA may ignore the Channel Switch Mode field and either cease transmissions or attempt new transmissions in the operating channel until the channel change takes effect.

Related changes:

- Selecting and advertising a new channel in a non-DMG infrastructure BSS This subclause describes operation in a non-DMG infrastructure BSS. For a DMG infrastructure BSS, see 10.9.8.6. The decision to switch to a new operating channel in an infrastructure BSS shall be made only by the AP. An AP may make use of the information in Supported Channel elements and the results of measurements undertaken by the AP and other STAs in the BSS to assist the selection of the new channel. The algorithm to choose a new channel is beyond the scope of this standard, but shall satisfy applicable regulatory requirements, including uniform spreading rules and channel testing rules. The AP shall attempt to select a new channel that is supported by all associated STAs, although note(#1288) that this might not always be possible. An AP shall inform associated STAs that the AP is moving to a new channel and maintain the association by advertising the switch using Channel Switch Announcement elements in Beacon frames, Probe Response frames, and Channel Switch Announcement frames until the intended channel switch time. The AP may force STAs in the BSS to stop transmissions until the channel switch takes place by setting the Channel Switch Mode field in the Channel Switch Announcement element to 1. The channel switch should be scheduled so that all STAs in the BSS, including STAs in power save mode, have the opportunity to receive at least one Channel Switch Announcement element before the switch. The AP may send the Channel Switch Announcement frame in a BSS without performing a backoff, after determining the WM is idle for one PIFS (see 9.3.2.3 (IFS))(#164)(#156). …

1561.47:

(The text doesn’t create a conflict,  but it is badly worded)

Each DMS Status field includes a TCLAS element to identify the GCR group address, the DMSID corresponding to this GCR traffic flow, and other associated parameters. The Status subfield of this DMS Status field shall be set to “GCR Advertise.” A STA that receives this DMS Response frame from its AP may initiate a GCR agreement for one or more of the group addresses contained in the frame. NOTE—The STA might alternatively ignore this DMS Response frame.

(Should ignore)

1488.38:

(I don’t understand the distinction between “shall ignore” unknown subelements and “should ignore” unknown OIs.  The changes shown above for “shall ignore” delete this para.)

1756.59:

(I suspect this creates conflict with 11.6.6.3-11.6.6.5,  which internally describe various reasons to discard messages,  but don’t appear to include this condition)

The Authenticator should ignore EAPOL-Key frames it is not expecting in reply to messages it has sent or EAPOL-Key frames in which the Ack bit is 1. This stops an attacker from sending the first message to the Supplicant who responds to the Authenticator.

Jouni writes:

“It looks like we have number of informative sentences (e.g., the note in

11.6.6.1 or the penultimate paragraph of 11.6.6.8) indicating that Key Ack bit really needs to be verified. 11.6.11.3 has even a shall requirement on this in the description of EAPOLKeyReceived so that would likely come closest to being a normative requirement for the Authenticator. That said, I could not find a normative shall requirement for the Authenticator to ignore the frame.

The Authenticator side is easier in the sense that there is not even a single EAPOL-Key sent by the Supplicant that has Key Ack bit set to 1.

As such, it would be fine to add "The Authenticator shall silently ignore an EAPOL-Key frame with Key Ack bit set to 0." (or some similar language that allows logging of that even instead of "silently" ignoring this). This sentence could be added, e.g., in 11.6.6.1.

I don't know how to handle the implied need for Key Ack validation on Supplicant side, though, but anyway, that does not seem to be subject of this comment. :-)

The "not expecting in reply" is somewhat unclear.. That EAPOLKeyReceived description in 11.6.11.3 does have language related to this as well.

Interestingly, it first has a shall requirement on the frame having same Key Replay Counter value, but then following with _should_ be discarded if the value is different.”

Status:  Waiting for Jouni to respond with more definite changes.

Proposed resolution:

Revised.  Make changes in <this-document> under CID 2185.

| 2118 | 82.13 | 4.5.1 |  | " Some of the services are supported by MAC management messages and some by MAC data messages."I don't think it helpful to have yet another term to describe a unit of communication. Some uses of "message" apply to MSDUs, some to MPDUs. | Discuss whether this ambiguity is helpful or harmful, or so deeply ingrained we shouldn't fix it. If we decide it is harmful and we can fix it, I will volunteer to do the work. | GEN | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

When “message” clearly relates to an existing term, we should use that term.   If it is handwaving for some undefined entity, that is arguably OK.

Context and proposed changes: 89.36

- Overview of the services - General There are many services specified by IEEE Std 802.11. Six of the services are used to support medium access control (MAC) service data unit (MSDU) delivery between STAs. Three of the services are used to control IEEE Std(#130) 802.11 LAN access and confidentiality. Two of the services are used to provide spectrum management. One of the services provides support for LAN applications with QoS requirements. Another of the services provides support for higher layer timer ---synchronization. One of the services is used for radio measurement. This subclause presents the services, an overview of how each service is used, and a description of how each service relates to other services and the IEEE Std(#130) 802.11 architecture. The services are presented in an order designed to help build an understanding of the operation of an IEEE Std(#130) 802.11 (11ad)network. As a result, the services that comprise the SS and DSS are intermixed in order (rather than being grouped by category). The services that comprise the PCPS are a subset of the services provided by the SS and DSS.(11ad) Each of the services is supported by one or more MAC frame types. Some of the services are supported by MAC management PDUs, which are transported in MAC Management frames.The MSDU delivery service is supported by MAC Data frames. All of these frames gain access to the WM via the IEEE Std(#130) 802.11 MAC sublayer medium access method specified in Clause 9 (MAC sublayer functional description). The IEEE Std(#130) 802.11 MAC sublayer uses four types of frame—Data, Management, Extension,(11ad) and Control (see Clause 8 (Frame formats)). Data frames are handled by the MAC data plane (see Figure 5-1). MAC Management frames (see 8.3.3) and MAC Extension frames (see 8.3.4 (Extension frames))(11ad) are handled via the MAC management plane. MAC Control frames are used to support the delivery of IEEE Std(#130) 802.11 Data, Management, and Extension frames. The examples here assume an ESS network environment. The differences among the ESS, the PBSS,(11ad) and the IBSS network environments are discussed separately in Error: Reference source not found. - Distribution of messages within a DS - Distribution For the purpose of this subclause, a data message represents the parameters related to a MAC-UNITDATA primitive, such as the addresses and the MSDU. This is the primary service used by IEEE Std(#130) 802.11 STAs. It is conceptually invoked by every data message to or from an IEEE Std(#130) 802.11 STA operating in an ESS (when the frame is sent via the DS). Distribution is via the DSS. Refer to the ESS network in Error: Reference source not found and consider a data message being sent from STA 1 to STA 4. The message is sent from STA 1 and received by STA 2 (the “input” AP). The AP gives the message to the distribution service of the DS. It is the job of the distribution service to deliver the message within the DS in such a way that it arrives at the appropriate DS destination for the intended recipient. In this example, the message is distributed to STA 3 (the “output” AP) and STA 3 accesses the WM to send the message to STA 4 (the intended destination). How the message is distributed within the DS is not specified by IEEE Std 802.11. All IEEE Std 802.11 is required to do is to provide the DS with enough information for the DS to be able to determine the “output” point that corresponds to the intended recipient. The necessary information is provided to the DS by the three association related services (association, reassociation, and disassociation). The previous example was a case in which the AP that invoked the distribution service was different from the AP that received the distributed message. If the message had been intended for a STA that was a member of the same BSS as the sending STA, then the “input” and “output” APs for the message would have been the same. In either example, the distribution service was logically invoked. Whether the message actually had to traverse the physical DSM or not is a DS implementation matter and is not specified by this standard. While IEEE Std 802.11 does not specify DS implementations, it does recognize and support the use of the WM as one possible DSM. This is specifically supported by the IEEE Std(#130) 802.11 frame formats. (Refer to Clause 8 (Frame formats) for details.) A mesh BSS might form an entire DS or a part of a DS using the WM, as shown in Error: Reference source not found. Mesh services are used to form a mesh BSS and distribute messages. Clause 13 (MLME mesh procedures) defines how mesh BSSs are formed and how messages are distributed through a mesh BSS.

Proposed Resolution:

Revised.  Make changes in <this-document> under CID 2118.

| 2123 | 502.49 | 7.3.5.17 |  | "PHY-TxBusy.indication" - generally primitive names are upper case. | Review names of all MAC and PHY SAP primitives and upper case any that are not. | GEN | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

| Old Name | New Name |
| --- | --- |
| MLME-PeerKeySTART | MLME-PEERKEY-START |
| MLME-DLSTeardown | MLME-DLS-TEARDOWN |
| MLME-SAQuery | MLME-SA-QUERY |
| MLME-TIMING_ADVERTISEMENT | MLME-TIMING-ADVERTISEMENT |
| MLME-QoSMap | MLME-QOS-MAP |
| MLME-FSTSetup | MLME-FST-SETUP |
| MLME-FSTAck | MLME-FST-ACK |
| MLME-FSTTeardown | MLME-FST-TEARDOWN |
| MLME-FSTIncoming | MLME-FST-INCOMING |
| MLME-RELAYSearch | MLME-RELAY-SEARCH |
| MLME-RLSTeardow | MLME-RLS-TEARDOWN |
| MLME-PN-Exhaustion | MLME-PN-EXHAUSTION |
| MLME-PN-Warning | MLME-PN-WARNING |
| MSGCF* | Upper case version of same name |
| MSSME* | Upper case version of same name |
| PHY-TxBusy | PHY-TXBUSY |

Proposed resolution:
Revised.  Globally change the names of the primitives as shown under CID 2123 in <this-document>.

| 2049 | 1124.28 | 9.3.6 |  | (From Editor Panel Review of D1.1) Why is "transferred" used in 9.3.6 here and elsewhere to describe the transmission of a MPDU? It implies a different action than transmit. | Determine intent and reword "transferred" to "transmitted" if appropriate, otherwise document from where and to where the tranfer is occuring. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1124.28

A DMG STA may transmit a copy of the same group addressed MPDU using different antenna configurations. This might be needed to provide a quasi-omni coverage or to enable transmission by an MCS that is higher than MCS 0. If multiple copies of a group addressed MPDU with a To DS field equal to 0 are transferred, the DMG STA shall not transmit a different frame before the completion of the transmission of all copies of the group addressed MPDU.

Discussion:

I believe this is gratuitous variation in language.

Proposed resolution:

Revised.  Replace “transferred” with “transmitted” at cited location.

| 2051 | 1133.38 | 9.4.4.3 |  | Desire considered dangerous. We have 136 instances of desire(d¦s). But STAs don't have desire. Worse, when used in the passive voice (e.g. "the desired UP", "unless extra protection against PCF collisions is desired.") the identity of the entity suffering these pangs of desire is discretely hidden.I claim that this word is harmful and should be eliminated. "unless extra protection against PCF collisions is desired" could become "unless a STA determined that extra protection against PCF collisions is needed. How a STA makes this determination is implementation defined." | Review all uses of "desire" and replace with non-emotive language based on observables.e.g. "desired" BSSIS can become a target BSSID. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

A definition of this word is:

noun a strong feeling of wanting to have something or wishing for something to happen. verb strongly wish for or want (something).

In the context of “dot11DesiredSSID, dot11DesiredBSSType”,  I suppose the MIB variables are indirectly conveying the desires of an operator, so such language is reasonable.  However, more neutral language such as “target” would also work.

Changes:

109.13:

(assuming this represent’s its user’s desire,  no change needed)

GAS provides functionality that enables STAs to discover the availability of information related to desired network services, e.g., information about services such as provided in an IBSS, local access services, available subscription service providers (SSPs) and/or SSPNs or other external networks.

Likewise 109.26.

111.28:

Because operation of certain functions of the MAC may cause reordering of some MSDUs, as discussed in more detail below, in non-QoS STAs, there are two service classes within the data service. By selecting a service class, each LLC entity initiating the transfer of MSDUs is able to control whether MAC entities are or are not allowed to reorder those MSDUs.

114.22:

If a higher layer protocol using the data service cannot tolerate this possible reordering, the optional StrictlyOrdered service class might be used. MSDUs transferred between any pair of STAs using the StrictlyOrdered service class are not subject to the relative reordering that is possible when the ReorderableGroupAddressed service class is used. However, the ability to receive MSDUs sent using the StrictlyOrdered service class at a STA while preserving their order precludes simultaneous use of the MAC power management facilities at that STA. Note that the use of the StrictlyOrdered service class is obsolete and the StrictlyOrdered service class might be removed in a future revision of the standard.

117.52:

(How can a parameter specify any desires if it “shall be null”?   Should we attempt to describe something that is no interpreted in any way by 802.11?  Anyhow,  here’s a least-change fix.)

The routing information parameter specifies the route for the data transfer (a null value indicates source routing is not to be used). For IEEE Std 802.11, the routing information parameter shall be null.

117.59:

The priority parameter specifies the priority of the data unit transfer. The allowed values of priority are described in 5.1.1.4 (Interpretation of priority parameter in MAC service primitives). The service class parameter specifies the service class of the data unit transfer. The allowed values of service class are described in 5.1.1.5 (Interpretation of service class parameter in MAC service primitives …

127.28:

An enumerated type thatdescribes the requested power management mode of the STA.

Globally change all “desired SSID” to “target SSID”.

129.40:

Specifies the target specific access network type or the wildcard access network type. This field is present when dot11InterworkingServiceActivated is true Specifies the target specific HESSID network identifier or the wildcard network identifier. This field is present when dot11InterworkingServiceActivated is true. Only present if BSSType = MESH or BSSType = ANY_BSS. Specifies the target Mesh ID or wildcard Mesh ID

131.51:

The set of data rates that shall be supported by all STAs in order to to join this BSS.

131.55:

(I’ve always found this odd.  Who is the “peer STA” of a broadcast?  The MIB indicates this is related to the set of rates the AP wants to use to transmit at.  But this maps on to the Supported Rates element,  which encodes both the basic rates and the operational rates.  I think there is the implicit assumption that transmitted and received rate capabilities are the same,  otherwise it should be described in terms of what the STA transmitting the Beacon can receive.  The dot11OperationalRateSet is a read-only variable written by the SME when starting or joining a BSS.)

The set of data rates that the peer STA might use for communication within the BSS. This set is a superset of the rates contained in the BSSBasicRateSet parameter.

134.25:

(See also comment 2010,  which proposes deleting certain parameters of the START request.  If that resolution is accepted,  some of the changes below become moot,  as the text will be deleted.   Your editor can cope with this minor conflict, if it exists.)

The set of MCS values that the peer STA might use for communication within the BSS. See 10.15 (HT BSS Operation). This set is a superset of the MCS values contained in the BSSBasicMCSSet parameter.

Likewise “desires to” -> “might” at 140.35, 140.42, 140.45, 181.13, 181.20, 182.13.

473.24:

(I don’t see any obvious fix for this,  so I’m leaving it alone.)

The integer representing the desired peak modulation data rate used for Data frame transmission.

Ditto at line 30 and 36.

502.10

(Does this statement below add anything.  “When it desires” is undefined,  so adds no clarity as to the “when”.)

7.3.5.15.3 When generated This primitive is generated by the MAC sublayer for the local PHY entity prior to generating or receiving a primitive dependent on the PHYCONFIG_VECTOR.

Or

| 7.3.5.15.3 When generated Not specified. |  |
| --- | --- |

660.36:

Randomization Interval is set to the maximum random delay in the measurement start time, expressed in units of TUs.

664.45:

The Randomization Interval field is the upper limit of random delay before the measurement begins, expressed in TUs.

733.22:

The TS Info Ack Policy subfield is 2 bits in length and indicates whether MAC acknowledgments are required for MPDUs or A-MSDUs belonging tothis TID and the form of those acknowledgments. The encoding of the TS Info Ack Policy subfield is shown in Table 8-121 (TS Info Ack Policy subfield encoding).

736.1:

The Minimum PHY Rate field is 4 octets long indicates the minimum PHY rate for transport of MSDUs or A-MSDUs belonging to this TS within the bound of this TSPEC.

853.56:

(not sure “target” or “targeted” is any improvement.  Leaving alone.)

A non-AP STA uses this field to indicate the desired access network type in an active scan.

855.02:

(Contrary to 853.56,  I think this change does make sense)

A STA uses this field to indicate the target HESSID in an active scan per 10.1.4 (Acquiringsynchronization, scanning).

886.11:

The User Priority subfield indicates the UP of MSDUs or A-MSDUs of the stream to which this Intra-Access Category Priority element relates.

906.20:

— The Maximum Allocation field is set to the requested allocation in microseconds in each allocation period

948.19:

To obtain the number of TRN-R subfields, the value of the L-RX field is multiplied by 4. Possible values range from 0 to 16, corresponding to 0 to 64 TRN-R fields. Other values are reserved.

1117.41:

In general, a STA may transmit a pending MPDU when it is operating under the DCF access method, either in the absence of a PC, or in the CP of the PCF access method, when the STA determines that the medium is idle for greater than or equal to a DIFS, or an EIFS if the immediately preceding medium-busy event was caused by detection of a frame that was not received at this STA with a correct MAC FCS value. If, under these conditions, the medium is determined by the CS mechanism to be busy when a STA becomes ready to initiate the initial frame of a frame exchange sequence (described in Annex G), exclusive of the CF period, the random backoff procedure described in 9.3.4.3 (Backoff procedure for DCF) shall be followed. There are conditions, specified in 9.3.4.3 (Backoff procedure for DCF) and 9.3.4.5 (Control of the channel), where the random backoff procedure shall be followed even for the first attempt to initiate a frame exchange sequence.

1133.38:

To further reduce the susceptibility to inter-PC collisions, the PC shall require that the medium be determined as being idle for a DIFS plus a random (over a range of 1 to aCWmin) number of slot times once every dot11MediumOccupancyLimit TU during the CFP. This results in loss of control of the medium to overlapping BSS or hidden STA traffic, because the STAs in this BSS are prevented from transmitting by their NAV setting to CFPMaxDuration or CFPDurRemaining. dot11MediumOccupancyLimit may be set equal to CFPMaxDuration, which provides no protection against PCF collisions. The dot11MediumOccupancyLimit is also useful for compliance in regulatory domains that impose limits on continuous transmission time by a single STA as part of a spectrum etiquette.

1172.57:

In certain regulatory domains, channel sensing needs to be done at periodic intervals (for example, in Japan, this period is 4 ms). This means that the duration of a TXOP in these regulatory domains might not be more than this periodic interval. In order to extend the TXOP beyond the limit implied by this periodic interval, the TXOP holder needs to sense the channel at least once in the limit imposed in the regulatory domain, by waiting for at least for the duration of one PIFS during which it senses the channel.

1174.06:

(I know “can” has a special meaning,  and the following change is an exception to that meaning.  But I couldn’t find a workable alternative)

A QoS STA shall use QoS Data frames for all MSDU transfers to another QoS STA. The TID in the QoS Control fields of these frames shall indicate the TC or TS to which the MPDU belongs. Furthermore, either the Queue Size subfield shall indicate the amount of queued traffic present in the output queue that the STA uses for traffic belonging to this TC or TS, or the TXOP Duration Requested subfield shall indicate the duration that the STA can use for the next TXOP for traffic belonging to this TC or TS. The queue size value reflects the amount on the appropriate queue not including the present MPDU. In order to inform the HC of queue status, a STA may use the QoS Null frame indicating the TID and the queue size or TXOPduration request (also see 9.20.3.5.2 (TXOP requests)).

1175.31:

In order to provide improved NAV protection, a STA may send an RTS frame as the first frame of any frame exchange sequence, during either the CP or CFP, and without regard for dot11RTSThreshold.

1175.38:

In order to provide NAV protection for a transmission to the AP in response to a QoS Data frame with a subtype that includes CF-Poll, the polled STA may send a CTS frame with the RA containing its own MAC. NOTE—1 The CTS frame is shorter than a QoS Data frame and has a higher probability that it will be received by other STAs. NOTE—2 The transmission of the CTS sets the NAV in its own vicinity without the extra time required to send an RTS frame.

1175.49:

(Note the weasel words added “attempt to”.  Nothing is guaranteed in WLAN,  and admission control has no magic wand to provide any guarantees.)

An IEEE Std 802.11 network may use admission control to administer policy or regulate the available bandwidth resources. Admission control is also used to attempt to provide a guarantee of the amount of time that a STA has available to access the channel.

1176.31:

The STA may transmit unadmitted traffic for the ACs for which the AP does not require admission control. If dot11RejectUnadmittedTraffic is false, a STA may send data without admission control for an AC that mandates admission control by using the EDCA parameters that correspond to a lower priority that does not require admission control.

1245.01:

A STA acting as a beamformer should be calibrated to maximize performance. A STA acting only as a beamformee does not need to be calibrated. Calibration can be performed using the over-the-air calibration procedure described below

1285.62:

A STA that is neither source nor destination during a truncatable SP may participate in Dynamic allocation of service period by being in the receive state with its receive antenna configured in a quasi-omni antenna pattern for the duration of the truncatable SP.

1289.44:

A STA that has updated a NAV timer as a result of the reception of an an RTS may reset its NAV timer(s) as follows. After the NAV timer update for a duration of TXTIME(DMG CTS) +2×SIFS, the STA shall monitor the channel to determine if a preamble or carrier detect event has occurred. If such an event has not occurred during this time period, then the STA may reset to 0 any NAV Timer that has a value of true for its associated NAV_RTSCANCELABLE variable

1291.30:

The PCP/AP shall not become a member of the cluster if no Beacon SP is determinedto be empty during aMinChannelTime, in which case, subject to the requirements described in 9.35.2.2 (Centralized PCP/AP cluster formation), then the PCP/AP may become the S-PCP/S-AP of a new cluster, or may cease its activity on this channel and may attempt operation on a different channel.

Likewise 1293.16, 1294.02, 1294.14, 1294.57, 1296.22

1310.25:

The initiator may include transmit training as part of the beam refinement phase by setting the TX-TRN-REQ field to 1 in the SSW Feedback frame and setting the L-RX field to indicate the length of the training sequence it requests the responder to use in the beam refinement phase. The initiator may carry out the MIDC subphase as part of the beam refinement by setting the BC-REQ field to 1 (to request a BC subphase) and setting the MID-REQ field to 1 (to request an MID subphase); in this case, the LRX field shall be set to indicate the number of receive AWVs the initiator uses during the MID subphase.

1310.53:

The responder may include transmit training as part of the beam refinement phase by setting the TX-TRN-REQ field to 1 in the SSW Ack frame and setting the L-RX field to indicate the length of the training sequence it requests the initiator to use in the beam refinement phase, as described in 8.5.4 (BRP Request field). The responder may carry out a MID subphase by setting the MID-REQ bit to 1 in the BRP Request field of the SSW frame. In this case, it shall also set the L-RX field to indicate the number of receive AWVs it uses during the MID subphase. The responder may carry out a BC subphase by setting the BCREQ bit to 1. If the initiator has set either the MID-REQ or the BC-REQ fields to 1 in the SSW-Feedback frame, the responder may set the MID-Grant or the BC-Grant fields to 1, or both, to grant the requests

1313.14:

Upon receiving a BRP packet with the Capability Request subfield set to 1, the responder shall respond with a BRP packet with the subfields within the BRP Request field set to determine the presence ofan MID subphase, a BC subphase and a beam refinement subphase.

1330.26:

A STA requests transmit beam refinement training by sending a BRP frame as follows. In the BRP Request field, the TX-TRN-REQ field is set to 1, and the FBCK-REQ field determines the feedback type

1362.47:

If a STA’s scanning does not result in finding a BSS with the target SSID and of the target type, or does not result in finding any BSS, the STA may start an IBSS upon receipt of the MLME-START.request primitive. The MAC of a STA receiving an MLME-START.request primitive shall use the regulatory domain information it has to process the request and shall return a result code of NOT_SUPPORTED to the request if regulatory domain information indicates starting the IBSS is illegal.

1365.12 figure 10-5 replace “, if desired” with “(optional)”

1440.20:

A non-AP DMG STA may add TSs to an existing allocation with a peer non-AP DMG STA. To do this, the non-AP DMG STA transmits an ADDTS Request frame to the peer STA to include the additional TS. The ADDTS Request frame shall contain a PTP TSPEC with the Allocation ID field set to indicate the allocation for the additional TS.

1587.49:

(I found it difficult to find a replacement for this)

The procedures defined in 10.25.3.1 (GAS Protocol), transmit the Alert Identifier Hash of the desired message in the Query request field of a GAS Initial Request frame. The Advertisement Protocol ID field in the GAS Initial Request frame is set to the value for EAS (see Table 8-193 (Advertisement protocol ID definitions))

1607.26:

Any time after dot11BeamLinkMaintenanceTime has elapsed, the source DMG STA of the SP may initiate an ISS to restore the beamformed link with the destination DMG STA of the SP following the rules defined in 9.36.2 (Sector-level sweep (SLS) phase). The source DMG STA may initiate the ISS before expiration of dot11BeamLinkMaintenanceTime.

1607.37:

Any time after dot11BeamLinkMaintenanceTime has elapsed, the initiator DMG STA of the beamformed link in the CBAP may initiate an ISS to restore the beamformed link with the responder STA following the rules defined in 9.36.2 (Sector-level sweep (SLS) phase). The initiator STA may initiate the ISS before expiration of dot11BeamLinkMaintenanceTime.

1619.26:

An initiator or responder may change the FSTS ID of its existing FST session by tearing down the existing FST session (10.33.3 (FST teardown)) and setting up a new one with a different FSTS ID value.

1664.50:

A Commit Message shall consist of a Finite Cyclic Group field (8.4.1.42 (Finite Cyclic Group field)) indicating a group, a Scalarfield (8.4.1.39 (Scalar field)) containing the scalar, and an Element field containing the element (8.4.1.40 (Element field)). If the Commit Message is in response to an AntiClogging Token request (see 11.3.7.6 (Status codes)), the Anti-Clogging Token is present (see 8.4.1.38 (Anti-Clogging Token field)).

1940.26:

(“desires” in informative text.   Could turn into a NOTE to highlight this.  No change proposed.)

Individually addressed frames may be used to temporarily raise the activity level of the mesh STA for a mesh peering. This is useful in cases when a link temporarily requires efficient data transmission with the peer mesh STA and the mesh STA desires to be able to transit back to lower activity level without performing the mesh power mode transition signaling with all peer mesh STAs.

1975.33:

The optional short preamble and header provides maximum throughput i when interoperability with legacy and nonshort-preamble-capable equipment is not a consideration.

2011.43:

Encode the extended, scrambled data string with a convolutional encoder (R = 1/2). Omit (puncture) some of the encoder output string (chosen according to “puncturing pattern”) to reach the “coding rate.” corresponding to the DATARATE TXVECTOR parameter. Refer to 18.3.5.6 (Convolutional encoder) for details.

2011.47:

Divide the encoded bit string into groups of NCBPS bits. Within each group, perform an “interleaving” (reordering) of the bits according to a rule corresponding to the DATARATE TXVECTOR parameter. Refer to 18.3.5.7 (Data interleaving) for details.

2012.09:

Up-convert the resulting “complex baseband” waveform to an RF according to the center frequency of the operating channel and transmit. Refer to 18.3.2.5 (Mathematical conventions in the signal descriptions) and 18.3.8.2 (Outline description) for details.

Likewise 2080.57.

2015.57:

(Status:  needs PHY expertise)

After performing an IFFT, the output is cyclically extended to the desired length

2020.49:

The DATA field, composed of SERVICE, PSDU, tail, and pad parts, shall be coded with a convolutional encoder of coding rate R = 1/2, 2/3, or 3/4, corresponding to the DATARATE TXVECTOR parameter.

2036.03:

(I think desired signal is probably industry standard terminology,  so leaving alone.)

The adjacent channel rejection shall bemeasured by setting the desired signal’s strength 3 dB above the ratedependent sensitivity specified inTable 18-14 (Receiver performance requirements) and raising the power of the interfering signal until 10% PER is caused for a PSDU length of 1000 octets.

Likewise 2036.52, 2056.65, 2140.16, 2140.26, 2140.46, 2140.55

2036.06:

(I think desired signal is probably industry standard terminology,  so leaving alone.)

The power difference between the interfering and the desired channel is the correspondingadjacent channel rejection. The interfering signal in the adjacent channel shall be a conformant OFDM signal, unsynchronized with the signal in the channel under test. For a conformant OFDM PHY the corresponding rejection shall be no less than specified in Table 18-14 (Receiver performance requirements).

Likewise 2036.55, 2057.04, 2140.19, 2140.29, 2140.50, 2140.59

2079.11:

If BCC encoding is to be used, encode the extended, scrambled data string with a rate 1/2 convolutional encoder (see 18.3.5.6 (Convolutional encoder)). Omit (puncture) some of the encoder output string (chosen according to puncturing pattern) to reach the coding rate, R, corresponding to the MCS or L_DATARATE TXVECTOR parameters. Refer to 20.3.11.6 (Binary convolutional coding and puncturing) for details. If LDPC encoding is to be used, encode the scrambled data stream according to 20.3.11.7.5 (LDPC PPDU encoding process).

2410.08:

This value is reported in transmitted Beacon, Probe Request, Probe Response, Association Request, Association Response, Reassociation Request, and Reassociation Response frames, and is used to determine whether a BSS with which the station might synchronize is suitable.

2476.58:

This value is reported in transmitted Beacon, Probe Request, Probe Response, Association Request, Association Response, Reassociation Request, and Reassociation Response frames, and is used to determine whether a BSS with which the station might synchronize is suitable. It is also used when starting a BSS, as specified in 10.3 (STA authentication and association).

2504.09:

(Can I really be bothered to correct an informative statement in a comment in a MIB that nobody cares about?

Go on,  Guess.)

-- * equal to notInService. Once the dot11RMRequestEntry(s) have been -- * created for a desired measurement sequence the corresponding -- * dot11RMRqstRowStatus(s) objects are set to active to indicate that

2808.50:

This attribute defines the time interval over which the MAC State Generic convergence function will attempt to predict the failure of an 802.11 network in time units (TUs). The convergence function should issue predicted network failure events at least this time interval before the network failure is detected.

2989.07:

/* show that smaller tables can be used */

3007.18:

A pass-phrase is a sequence of between 8 and 63ASCII-encoded characters.The limit of 63 comes from the requirement to distinguish between a pass-phrase and a PSK displayed as 64 hexadecimal characters.

3011.35:

The output of the LFSR is read by software and concatenated until enough randomness is collected. As a rule of thumb, reading from the LFSR 8 to 16 times the number of bits as the output number of random bits is sufficient.

3044.20:

Medium Time used for EDCA Admission Control is based upon one second periods and HCCA Medium time, used to aggregate TSPECs (see X.2.3 (Calculation of potential traffic self))also uses the one second period. Hence, for each application, S is the number of packets that might be sent in each one second period.

3106.40:

A mesh STA may save power by selecting the light or deep sleep mode for a mesh peering as follows: — mesh STAs that are “mains powered” might apply light or deep sleep mode if they do not have MSDUs to forward — mesh STAs that are “battery powered” might freely use all mesh power modes

<dramatic pause while author exits stage left to bang his head vigorously against a brick wall…>

| 2058 | 1167.52 | 9.20.2.7 |  | The terminology "TXTIME(CF-End) is undefined.Are all the paramters of the Clause 21 transmission fixed so that this is determined? | Replace with an equation para and variable list. Substitute a short variable name for the cited term and then explain this term in the variable list.If all the parameters of the transmission are not already fixed, specify them here.Ditto as 1289.45 | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

I think we can separate out the issues of whether all the TXVECTOR parameters determing the duration are fixed with the presentation. Assuming that the CF-End is sent using the DMG control PHY,  this is the case.

Proposed Resolution:

Revised. Replace with an equation para and variable list.

Replace “TXTIME(CF-End)” with “T_CF-End” ( _ denoting subscript) and add “, where T_CF-End is duration of a CF-End frame”).

Make matching change at 12898.45.

| 2063 | 1178.19 | 9.20.4.3 |  | The word "normative" is generally not used in the standard, except when labelling Annexes. | Review each use of normative and replace with a declarative, "shall", "may" or "should" statement.For example, at the cited location: "The normative behavior of the scheduler is as follows:" can become "The scheduler follows these rules:". Declarative suffices because the entries themselves contain adequate normative statements. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  Make changes under CID 2063 in <this-document>.

At 1178.19:

The following are rules for the operation of the scheduler: — The scheduler shall be implemented so that, under controlled operating conditions, all STAs with admitted TS are offered TXOPs that satisfy the service schedule. — Specifically, if a TS is admitted by the HC, then the scheduler …

1225.18:

(Sentence deleted below is either unnecessary or creates a conflict.  In this case it is unnecessary.)

The RD protocol may be supported by an HT STA and by a DMG STA. For an HT STA, the RDG/More PPDU subfield and the AC Constraint subfield are present in the HTC field, and for a DMG STA, the RDG/More PPDU subfield and the AC Constraint subfield are present in the QoS Control field.

1339.42:

In addition to the behavior specified in9.22.7.5 (Generation and transmission of BlockAck frames by an HT STA or DMG STA) when responding with a BlockAck frame, the RBUFCAP field shall be updated with the value of WinCapacityB

1425.40:

(The more I look at this,  the odder this appears.  “no rules apart from in here” is weird,  and creates a maintenance timebomb.)

TSPECs and DMG TSPECs are constructed at the SME, from application requirements supplied via the SME, and with information specific tothe MAC layer The value of the Minimum PHY Rate in a TSPEC shall satisfy the following constraints: — it is in the AP’s operational rate set, for an uplink TS. — it is in the non-AP STA’s operational rate set, for a downlink TS. — it is in both the AP’s operational rate set and non-AP STA’s operational rate set, for a bidirectional TS. N.4 (TSPEC construction), describes parameter choice for the TSPEC. NOTE—This standard states no requirements on how a DMG TSPEC is to be generated.

2014.33:

(I don’t understand the intent of the following.  How can a specification utilize anything except paper?  Needs PHY expertise.)

In the case of vanishing TTR, the windowing function degenerates into a rectangular pulse of duration T. The normative specifications of generating the transmitted waveforms shall utilize the rectangular pulse shape.

2224.49:

(important to have no ambiguity.  Prefer not to modify.)

The sequences in the tables are normative, the description above is informative.

Proper use at 2234.55 and other “(normative)” in annex headers.

2738.18:

"The use of dot11TempType is deprecated, because no associated behavior is described in IEEE Std 802.11-<year>, and this entity may be removed in a later revision of the standard. This is a status variable. It is written by the PHY.

2900.39:

(I’d say leave this alone.  Alternatively could say “an example of a time windowing function”.

The time-windowing feature illustrated here is not part of the normative specifications.

Ditto at 2907.35

3040.27:

The schedule generated by the scheduler satisfies the requirements specified in 9.20.4.3 (Controlledaccess admission control). The schedule for an admitted stream is calculated in two steps. The first step is the calculation of the scheduled SI. In the second step, the TXOP duration for a given SI is calculated for the stream.

3042.44:

(Deleting sentence because it is not true. See 1425.40.)

TSPECs are constructed at the SME from application requirements supplied via the SME and with information specific to the MAC layer.

| 2079 | 1271.45 | 9.34.3 |  | Why is there a reference to 9.3.2.9 here? Makes no sense to me. | Remove reference to 9.3.2.9 or replace it with a more scrutable reference. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1271.45:

NOTE 2—STAs do not transmit a response frame to the PCP/AP when they receive a request frame from the PCP/AP with the RA equal to a group address (see 9.3.2.9 (Block Ack procedure) and 9.3.6 (Group addressed MPDU transfer procedure)).

Discussion:  I could see no relevance to the Block Ack procedure.

Proposed resolution:

Revised.  Remove reference to 9.3.2.9.

| 2086 | 1275.14 | 9.34.6.2 |  | " The frame sent by the STA may be an RTS or a DMG CTS-To-Self. The frame sent by the STA may be a SSW frame or a BRP packet if the STA is performing beamforming (9.7.7.5 (Rate selection for BRP packets)). " -- there are two STAs. So which one is meant? | Differentiate which STA (source or destination) is intended. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:  existing text is confusing and contains redundant shall.

Context: (and proposed changes) 1275.08

At the beginning of an SP, (except when the source DMG STA intends to establish a DMG Protected Period, see 9.34.6.6 (DMG Protected Period) ) before the source DMG STA initiates the frame exchange in the SP, a source DMG STA shall transmit a frame to the destination DMG STA using the DMG Control modulation class before it uses any other modulation class for transmission if the Heartbeat field in the destination DMG STA’s DMG Capabilities element is 1. The frame sent by the source STA may be an RTS or a DMG CTS-To-Self. The frame sent by the source STA may be a SSW frame or a BRP packet if the source STA is performing beamforming (9.7.7.5 (Rate selection for BRP packets)).

Resolution:

Revised.  Make changes under CID 2086 in <this-document>. This changes provide the differentiation sought.

| 2089 | 1282.31 | 9.34.7.1 |  | "If a PCP/AP wants to dynamically allocate Service Periods during a scheduled SP for which both the sourceand destination AID fields are set to the broadcast AID,the PCP/AP shall set the truncatable subfield to onewithin the Allocation field corresponding to the scheduled SP. "This normative statement is based on what a STA wants to do. That is surely wrong. | Reword so that the normative statement is based on observable conditions. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1282.31 and proposed changes

A PCP/AP may dynamically allocate Service Periods during a scheduled SP for which both the source and destination AID fields are set to the broadcast AID by setting the Truncatable subfield to one within the Allocation field corresponding to the scheduled SP.

Proposed resolution:

Revised.  Make changes under CID 2089 in <this-document>.

| 2153 | 1291.30 | 9.35.2.1 |  | "Beacon SPn" - is this a local variable? If so, needs a definition on first use. Or is it a field? If so, say so. Or is it a period of time? | Define the terminology. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1291.25 and proposed changes

A decentralized clusteringenabled PCP/AP that receives a DMG Beacon frame with the ECPAC Policy Enforced subfield in the DMG Parameters field set to 0 from an S-PCP/S-AP on the channel the PCP/AP selects to establish a BSS shall monitor the channel for DMG Beacon transmissions during each Beacon SP for an interval of length at least aMinChannelTime. A Beacon SP is empty if no DMG Beacon frame is received during the Beacon SP over an interval of length aMinChannelTime. The PCP/AP shall not become a member of the cluster if no Beacon SP is determinedto be empty during aMinChannelTime, in which case, subject to the requirements described in 9.35.2.2 (Centralized PCP/AP cluster formation), then the PCP/AP may become the S-PCP/S-AP of a new cluster, or may cease its activity on this channel and, if desired, attempt operation on a different channel.

Proposed resolution.

Revised.  Replace cited sentence with: “A Beacon SP is empty if no DMG Beacon frame is received during the Beacon SP over an interval of length aMinChannelTime.”

| 2093 | 1292.41 | 9.35.2.2 |  | "Annex E to be 56.16 GHz" - The whole purpose of Annex E is to avoid embedding magic numbers in the standard. | Find some way to avoid repeating magic numbers at this point by reference to entities defined in Annex E. | MAC | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

Either the Channel Starting Frequency of the intended operating class of the STA is defined in Annex E to be 56.16 GHz and channel 2 is an allowed channel in the current regulatory domain and the list of excluded channels includes channel 2, or the Channel Starting Frequency of the intended operating class of the STA is defined in Annex E to be 56.16 GHz and channel 2 is not an allowed channel in the current regulatory domain; and

Discussion:

Magic numbers are evil for two reasons:

- They do not document their meaning,  only a value

- Multiple copies of the same number have a habit of getting updated independently or inconsistently

I have consulted with Brian, Peter E and Carlos C.   There are a range of possibilities.  The changes below are one of many possible solutions.

The deletion is possible because the only operating classes in Annex E that can be used by a DMG STA are those with a starting frequency of 56.16 GHz,  and the behaviour cited is specific to a DMG STA.

Change 9.35.2.2 as follows: (1292.40)

Either the Channel Starting Frequency of the intended operating class of the STA is defined in

Annex E to be 56.16 GHz and channel 2 is an allowed channel in the current regulatory domain

and the list of excluded channels includes channel 2, or the Channel Starting Frequency of the

intended operating class of the STA is defined in Annex E to be 56.16 GHz and channel 2 is not

an allowed channel in the current regulatory domain; and

Proposed resolution:

Revised.  Delete “the Channel Starting Frequency of the intended operating class of the STA is defined in

Annex E to be 56.16 GHz and” at the cited location (twice).

| 2100 | 1303.22 | 9.36.2.1 |  | "A responder shall not begin transmitting the frames of an RSS before the ISS is successfully completed" -- what is the definition of success, visible to the responder? | Add reference to 9.36.1 as defining success. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1303.22 and proposed changes

A responder shall not begin transmitting the frames ofan RSS before the ISS is successfully completed as defined in 9.36.1, except when the ISS occurs in the BTI (9.36.5 (Beamforming in A-BFT)).

Proposed resolution:

Revised.  Add “as defined in 9.36.1” after “successfully completed”.

| 2101 | 1303.25 | 9.36.2.1 |  | "An initiator shall not begin an SSW-Feedback before the RSS phase issuccessfully completed," -- successful completion is not defined | Add reference to 9.36.1 as defining success. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.  Add “as defined in 9.36.1” after “successfully completed”.

| 2102 | 1303.30 | 9.36.2.1 |  | "A responder shall not begin an SSW-Ack with an initiator in the A-BFT. A responder shall begin an SSWAck with an initiator immediately following the successful completion of the SSW-Feedback" - success is not defined | Add reference to 9.36.1 as defining success. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Revised.  Add “, as defined in 9.36.1,” after “successful completion”.

| 2162 | 1326.07 | 9.36.6.3.2 |  | "using the Nbeam subfield in the FBCK-TYPE field"Some style errors here. Subfields are normal text (not italic). Variables are italic. | Review this and sibling subclauses and ensure quoted names of subfields are body text, and variables are italic. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Accepted.

| 2109 | 1336.01 | 9.37 |  | Subclause 9.37 is a specialization of block ack. It is in the wrong place. | Move to become a subclause of 9.22 | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed Resolution:

Revised.  Move 9.37 to become 9.22.11.

| 2129 | 1365.52 | 10.1.4.3.3 |  | "Only the following STAs respond to probe requests"Either this statement has no effect, or it creates an exception to the normative statement "STAs ... shall respond" at 1365.27. | Merge this list with the lettered list, or find a way to call out the exceptions from the "shall respond" statement. | MAC | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

This comment is considered in 11-14/0057.

| 2138 | 1405.11 | 10.2.6.2.4 |  | "Group addressed MSDUs, individually addressed MSDUs and MMPDUs that are to be transmitted to a STAthat is in PS mode are first announced through ATIM frames during the Awake window."Other parts of REVmc use "BU" terminology to describe these items, specifically addressing that A-MSDUs may be buffered, and that not all MMPDUs are. | Review all 10.2.6 references to the type of thing buffered and determine if the rules should be the same as for non-DMG. If that is so, replace such references with "BU". | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context & Proposed changes:

- Power management in a PBSS and DMG infrastructure BSS (11ad) - General(11ad) … An AP shall buffer BUs addressed to non-AP STAs in Doze state. The buffered BUs shall be transmitted only at designated times (Error: Reference source not found). A non-AP STA shall defer deliver BUs addressed to other non-AP STA in Doze state. The BUs shall be transmitted only at designated times (Error: Reference source not found). … - Non-PCP/non-AP STA power management mode(11ad) - Power management mode operation of a non-PCP/non-AP STA with or without a wakeup schedule (11ad) … BUs that are to be transmitted to a STA that is in PS mode are first announced through ATIM frames during the Awake window. A STA in PS mode that is awake during an Awake window shall listen for these announcements to determine if it needs to remain in the Awake state. If during the Awake window the STA does not receive or transmit an ATIM frame with BSSID equal to the BSSID of the BSS the STA is a member, then it may enter the Doze state at the end of the Awake window. If a STA receives an ATIM frame during the Awake Window, it shall acknowledge the ATIM frame. Any two STAs that successfully complete an ATIM frame exchange with each other during the Awake Window become peer STAs. If a STA receives or transmits an ATIM frame during the Awake Window, it shall be awake during the CBAP(s) within the current beacon interval that have the source AID or destination AID described by the ATIM frame to wait for the announced BU(s) to be received and/or to transmit announced BU(s). A STA that receives or transmits an ATIM frame during the Awake Window may enter the Doze state when it has successfully transmitted to and received from all corresponding peer STAs for this beacon interval a QoS data frame with the EOSP subfield set to 1. ATIM frame transmissions and MSDU transmissions follow the rules defined in Error: Reference source not found. …

Proposed Resolution:

Revised.  Make changes under CID 2138 in <this-document>.  These changes adopt “BU” terminology.

| 2158 | 1437.52 | 10.4.10 |  | "The case of uplink TS timeout in which the PCP/AP is the destination DMG STA of the TS(11ad) is shown in Figure 10-18 (TS timeout)" -- the qualification inserted by .11ad "hijacks" the figure so that it no longer describes the non-DMG case. | Add back the non-DMG case. | MAC | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

The changes to the figures were bogus.

For example: Figure 10-18 a)

The changes added “non-AP” to the right hand half.  But DMG STAs do not transmit QoS CF-Polls.  This is part of the HCCA mechanism, which is not included in a DMG STA (See Figure 9-1).  So the figures are actually inapplicable to the DMG case. Further, TS deletion in a PCP is more complex,  and may involve the PCP and both peers of a DMG or P2P TSPEC.  Comment 2156 resolves the TS deletion issues.

So I propose to “undo” the .11ad changes to figure 10-18 and the text that references it.

It is possible that other errors are present in the text of this subclause.  I have made no attempt to determine if this is the case.

Proposed Resolution:

Revised.

At 1437.52, delete “in which the PCP/AP is the destination DMG STA of the TS”.

In Figure 10-18 change “HC/non-AP” to “HC” (in four places)

| 2160 | 1440.10 | 10.4.13.3 |  | " that each allocation is at least Minimum Duration microseconds" - where is Minimum Duration? | Add the word "field" somewhere. | MAC | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: 1440.07

With an asynchronous DMG TSPEC, the DMG STA registers the minimum allocation it expects within the allocation period while an SP request is in effect that is greater than the minimum allocation specified. In addition, the STA expects that each allocation is at least of duration specified by the Minimum Duration field of the DMG TSPEC, provided the outstanding SP request is at least that long. In admitting a DMG TSPEC, the PCP/AP should check that there are sufficient resources available to meet the TSPEC requirements.

| 2167 | 1559.32 | 10.24.16.3.1 |  | The insertions by .11ad are unnecessary because the exclusions apply to features already not supported by DMG STAs. | Remove any unnecessary "non-DMG" qualifications | MAC | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context (.11ad p381):

Discussion:

DMG does not allow support of GCR.   The insertions support this position.   However the last two are redundant as “STA that implements advanced GCR” excludes DMG by 1556.42, and “In a mesh

BSS,” excludes DMG by 1853.29.

Proposed resolution:

Revised.  Delete “non-DMG” at 1559.35 and 1559.39.   These remove the only redundant qualifications in this para.

| 2182 | 2219.58 | 21.9 |  | "the PHY shall maintain (#1601)PHY-CCA.indication(BUSY) primitive for any signal 20 dB higher" -- Indications are discrete events, not continuous signals. "shall maintain" is incorrect. | Propagate changes for the same reason from the HT PHY. | GEN | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context: And proposed changes

In the case of signal loss before the decoding of the header or in the case of an invalid header, the PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the received level drops below a value that is 20 dB higher than the receive sensitivity of MCS 1. In the case of signal loss after decoding of a validheader, the PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the expected end of the packet, including AGC and TRN-T/R fields.

Part of HT PHY: 2147.40

The HT PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the received level drops below the CCA sensitivity level (for a missed preamble) specified in 20.3.20.5 (CCA sensitivity).

Proposed resolution:

Revised. Replace cited para with: “In the case of signal loss before the decoding of the header or in the case of an invalid header, the PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the received level drops below a value that is  20 dB higher than the receive sensitivity of MCS 1. In the case of signal loss after decoding of a valid header, the PHY shall not generate a PHY-CCA.indication(IDLE) primitive until the expected end of the packet, including AGC and TRN-T/R fields.”

| 2177 | 2184.15 | 21.3.10 |  | Either the changes made for Motion 34 (less than becomes less than or equal) are incorrect, in which case they should be unmade. Or they are correct, in which case matching changes need to be made in the other PHY clauses, and on the equation line below.Changes at lines 15 and 22. | Either unmake changes or propagate to other PHYs. | GEN | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Context:

Discussion:

I believe the changes to be correct. From the equation at 2184.27, when power is 0dBm, RCPI is 220,  and when power is -110dBm,  RCPI is 0.   So the equality signs are correct.   Note that the change made by 11-13/0937r2 failed to correct the inequality in the equation line itself.

Also note that the “rounded to nearest 0.5 db” does not correspond to the floor operator in the equation.  And note that 0 actually represents Power <  -109.5 dBm.

Also note at line 10: “RCPI shall be a monotonically increasing, logarithmic function of

the received power level defined in dBm.”   There is a missing comma that completely changes the meaning.   It is not a logarithm of the “received signal power defined in dBm”, in which case we’d have two logarithms going is.

OK, so I’ve found 4 errors in this text, which has been faithfully cut and paste into each PHY.

I propose to remove the duplicate (and inconsistent) specification as follows:

Change 2184 lines 10-27 to read: (note,  Editor to substitute single glyph forms of <=, >= and floor())

RCPI is a monotonically increasing, logarithmic function of the received power level. The allowed values for the Received Channel Power Indicator (RCPI) parameter are defined in Table 21-x, where P is the received power level in dBm. Table 21-x RCPI values RCPI Value; Description. 0; Represents P < -109.5 dBm. 1-219; Power levels in the range -109.5 <= P < 0 are represented by RCPI = floor((P+110) x 2). 220; Represents P >= 0 dBm. 221-254; Reserved. 255; Measurement not available.

Make matching changes at 1972.50 (Clause 16), 2003.55 (Clause 17), 2037.61(Clause 18), 2142.24 (Clause 20),  2184.10 (Clause 21).

(Note, Clause 19 references clauses 18 and 17 for definitions)

Proposed resolution:

Revised.  Make changes in <this-document> under CID 2177.  These changes change the form of expression of RCPI to avoid redundancy and correct errors.

| 2183 | 2383.00 | B.4.24.1 |  | Invalid or bogus references at:2386.10 (8.4.2.111.2), 2386.11 (11.3), 2386.15 (8.4.2.145),2386.23 (9.13a9.14),2387.31 (9.4.2.138), 2387.35 (8.4.2.140),2387.44 & 2387.57 (8.4.2.138),2392.55 (8.4.2.138), 2392.59 (8.4.2.140),2393.07 (8.4.2.138), 2393.11 (8.4.2.140) |  | GEN | 2 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Status:   Dorothy has proposed a resolution in 11-14/221. Carlos also has independently done some work on this and proposed a resolution.  I have merged these resolutions and made some tweaks of my own.

Merged changes:

In 2385.54, delete “8.4.2.145”

At 2383.10, change the DMG-M4.4 reference list to include “8.4.2.127.2, 9.12.2”

At 2386.23, change the DMG-M5 reference to “9.14”.

At 2387.27 insert 8.4.2.136 in the DMG-M7.3.1 reference list.

In 2387.31, replace “8.4.2.138” by “8.4.2.131”

In 2387.35, replace “8.4.2.140” by “8.4.2.133”

At 2387.44 insert 8.4.2.136 in the DMG-M7.3.2 reference list.
In 2387.44, replace “8.4.2.138” by “8.4.2.131”

At 2387.49, insert 8.4.2.136 in the DMG-M7.3.3 reference list and change the second 9.34.6.2 reference to 9.34.6.3.

In 2387.57, replace “8.4.2.138” by “8.4.2.131”

At 2392.55 insert 8.4.2.136 in the DMG-M7.12.1 reference list.

In 2392.55, replace “8.4.2.138” by “8.4.2.131”

In 2392.59, replace “8.4.2.140” by “8.4.2.133”

At 2393.07 insert 8.4.2.136 in the DMG-M7.12.2 reference list.
In 2393.07, replace “8.4.2.138” by “8.4.2.131”

In 2393.11, replace “8.4.2.140” by “8.4.2.133”

| 2060 | 3050.34 | O.2 |  | The changes indicated in the resolution of comment 234 on figure O-3 made no sense to me as editor. | Please review Figure O-3 versus the resolution of comment 234 and determine if any additional changes are necessary. | GEN | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

The resolution of CID 234 states:

REVISED (GEN: 2013-09-18 09:24:31Z) - - In 8.4.2.7, after the para which starts "When dot11MgmtOptionMultiBSSIDActivated is false" add a "NOTE---The bit numbered 0 in the traffic indication virtual bitmap need not be included in the Partial Virtual Bitmap field even if that bit is set." - In the same para, and in the "Method A" and "Method B" paras below, change "in the bitmap" to "in the traffic indication virtual bitmap" - In the next para, and in the para which ends "Otherwise, an AP uses Method A." below, change "in the virtual bitmap" to "in the traffic indication virtual bitmap" - In Figures O-2 and O-3 show the AID 0 bit in the PVB as 1 and split the arrow from AID 0 to point at both the Bitmap Control b0 and the PVB b0. Similarly, on O-5, show the AID 0 bit in the PVB as 1. - In Figures O-1 to O-7 change the captions to: - say "Partial" first - have "Bitmap" in caps - not have "Example" in caps - say "Bitmap" (for O-7) - Ditto for the title of Annex O - Change "bit map" (case-insensitively) to "Bitmap" - Change "bitmap control" (case-insensitively) to "Bitmap Control" - "Traffic Indicator bit" is used exactly once in the spec, despite the grandiose uppercase letters -- change to "traffic indication virtual bitmap bit

# Figure 0-3 from Std-2012 (prior to any changes)

The difficulty is “AID 0 bit in the PVB as 1”.   The PVB starts at AID 16,  so there is no AID 0 in the PVB.  Hence my enquiry.

The instruction “1 and split the arrow from AID 0 to point at both the Bitmap Control b0 and the PVB b0” results in B0 of the PVB having two arrows end at it.

The current 0-3 from REVmc D2 is: 3050.23

The following resolution undoes this obvious problem,  but it may miss a fix badly expressed from the original comment resolution.

Proposed resolution:

Revised.  Remove the diagonal arrow in Figure 0-3.

| 2157 | 1437.35 | 10.4.10 |  | Generally there's a lot of unnecessary qualification introduced by .11ad. For example in " PCP shall send a DELTS frame to the non-PCP DMG STA", it is unnecessary to include DMG in "non-PCP DMG STA" because the PCP cannot send a DELTS to a non-DMG STA. We went through a process in REVmb days of removing unnecessary "QoS" qualifications. Looks like we might need to do the same for DMG. | Review this subclause and remove any unnecessary qualification. | MAC | 4 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Discussion:

There are 941 instances of DMG STA and 194 instances of non-DMG.

We can remove a DMG qualification when:

- It is in a subclause specific to DMG

- It relates to a beviour specific to DMG

- The behaviour has already been qualified in the text in question to be specific to DMG.

An example of “Subclause specific to DMG”,  it includes 1302:25:

A DMG STA does not have to be capable to perform MID as described in 9.36.6.3 (MIDC (multiple sector ID capture) subphase)

An example of “already been qualified in the text” is at 28.23:

beacon transmission interval (BTI): The time interval between the start of the first Directional Multigigabit (DMG) Beacon frame transmission by a DMG station (STA) in a beacon interval to the end of the last DMG Beacon frame transmission by the DMG STA in the same beacon interval.

The second “DMG STA” refers to the first one by the use of “the”.

The first one is arguably unnecessary because the feature is DMG-specific.  However, no harm is done by leaving it in place.

There are arguably many locations that should be changed.   The challenge is to find an efficient way to allow them to be reviewed, and for the changes finally made to accurately reflect the proposal made.

Using the flag “(#DMG)”, I have identified where changes are to be made in D2.5 of the draft, which is also awaiting editorial review of technical changes.  There are ~140 such flags, which is a small fraction of the ~1000 terms involving DMG that are not names of fields or frames.

Proposed resolution:

Revised.  Remove DMG or non-DMG qualifiers at the locations indicated by a (#DMG) flag in P802.11REVmc-D2.5 and reword surrounding text as necessary for grammar.

# Comments for which no non-reject resolution is proposed

Others are welcome to find a non-reject resolution …

| 2039 | 1288.01 | 9.34.10 |  | The format of the pseudo-code is somewhat raggle-taggle. We use some C-like constructs (e.g. comments, logical operator "¦¦" and braces, but not others such as "!="). Some keywords are capitalized, others are not. Recommend we make the pseudo-code similar to that in Clause 11. | Reword pseudo-code to make it look like the clause 11 code, or alternatively express it in valid C. | EDITOR | 3 |
| --- | --- | --- | --- | --- | --- | --- | --- |

Proposed resolution:

Rejected.   The comment fails to identify changes in sufficient detail so that the specific wording of the changes that will satisfy the commenter can be determined.