IEEE P802.11
Wireless LANs

| D0.01 comment resolution (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-03-18 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Jiamin Chen | Huawei/HiSilicon |  |  | Jiamin.chen@mail01.huawei.com |

Abstract

This document proposes resolutions to TGaj D0.01 CIDs: 13, 59, 73, 76, 77, 79, 80, 81, 112, 117, 133, 149.

# MAC related Comments

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy |
| --- | --- | --- | --- | --- | --- | --- |
| 13 |  | 14 | 4 | TR | "The Measurement Request Mode subfield valid value and its meaning is defined in Table 8-221f" and "The Measurement Request Mode field is set to 2 to indicate". If the table defines the meaning, why do it in the text? Duplicate normative definition is a sin. | Remove one of these. If the table doesn't add enough explanation, move the explanation from the text into the "Meaning of value" column. |

Proposed resolution: Accept

Remove the redundant txt.   Move explanation from text into the "Meaning of value" column. Throughout the spec, do the same where applicable.

| 59 |  | 34 | 7 | TR | "in Beacon frames, " - but DMG devices don't transmit these frames | "in DMG Beacon frames" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

The CDMG PCP/AP send DMG Beacon. There is no definition for CDMG Beacon in the spec. Use “DMG Beacon” in the spec where applicable.

| 73 | 10.39.1 | 31 | 49 | TR | The statement of this paragraph doesn't make sense in a standard. | Delete this paragraph. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Remove the paragraph in P31L49.

| 76 | 10.39.1 | 31 | 64 | TR | The description is ambiguous, e.g. when will it be set to true besides requiring DCT procedure? | Remove "It may also be set to true in other circumstances", or clarify when this happens |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Remove "It may also be set to true in other circumstances" in P31L64

| 77 | 10.39.1 | 32 | 54 | TR | The statement is redundant. | Remove "and is not meant to be exhaustive of all possible uses case" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept.

Remove "and is not meant to be exhaustive of all possible uses case" in P32L54.

| 79 | 10.39.4 | 33 | 25 | TR | the description " without undue delay " is ambiguous | Please clarify how to understand "undue" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Revise

This paragraph is talking about the Radio Measurement procedure (defined in 10.11). Remove the paragraph in P33L25 from this subclause.

| 80 | 10.39.4 | 33 | 27 | TR | No signal is defined to indicate " the STA is unable to complete the measurement request." | please define it somewhere. |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Revise

See resolution to CID79.

| 81 | 10.39.4 | 33 | 46 | TR | To assess the DCT request, the responder should have received a DCT request. While the DCT request is never mentioned before in this sub-clause. | delete " the DCT request and" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Change the sentence in P33L46 as follows:

“The DCT responder PCP/AP shall assess the DCT request and DCT measurement request against its capabilities and the impact on its BSS’s performance.”

| 112 | All |  |  | TR | Once CDMG is defined, all the references to STA and PCP/AP in the 11aj draft should be "CDMG STA", "CDMG PCP/AP". | As noted. Please search for all occurrences of STA, PCP/AP and replace them by CDMG STA, CDMG PCP/AP respectively |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Search for all occurrences of STA, PCP/AP and replace them by CDMG STA, CDMG PCP/AP respectively where applicable.

| 117 | 8.5.8.1 |  |  | TR | Why are the new DCT frames Public Action? | Clarify |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

The Dynamic Channel Transfer (DCT) procedure is used to coordinate the operating channel between two CDMG BSSs. When the two CDMG BSSs perform the DCT procedure, all communication is inter-BSS, not within a same BSS. In 802.11 spec, the Public Action frame is defined to allow inter-BSS communication without performing association and authentication between two STAs. While the  Action frame is defined to be used within a same BSS. So, the frames used in DCT procedure are  defined as Public Action frames, rather than as Action frames. There are many Public Action frames defined in 802.11, such as GAS, DSE, etc. On the other hand, the DCT responder PCP/AP can decide to refuse the DCT request by itself for some reasons, such as its BSS is busy. So there might be no explicit security issues to use Public Action frame in DCT procedure. In order to disable a STA, a attacker can transmit any signal to cause the media busy.

| 133 | 1.1 | 3 | 7 | TR | This amendment modifies IEEE 802.11, not IEEE 802.11ad. | Change "IEEE 802.11ad" to "IEEE 802.11" |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

# PHY related Comments

| 149 |  | 63 | 59 | TR | In the equation, the variable "m" shall be "k". The variable "(q)" does not make sense here. | change "m" to "k". Remove"(q)". |
| --- | --- | --- | --- | --- | --- | --- |

Proposed resolution: Accept

Change the equation in P63L59

to：