IEEE P802.11
Wireless LANs

| PHY Simulations and Methodology |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-03-18 |  |  |  |  |
| Author(s)/Supporter(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| CHEN, Jiamin | Huawei /CWPAN |  |  | Jiamin.chen@mail01.huawei.com |
| CHEN, Qian | I2R / CWPAN |  |  | qchen@i2r.a-star.edu.sg |
| CHIN, Francois | I2R /CWPAN |  |  | chinfrancois@i2r.a-star.edu.sg |
| DU, Guanglong | BUPT/CWPAN |  |  | guanglong108@sina.com |
| GAO, Bo | Tsinghua /CWPAN |  |  | bgao_cug@126.com |
| HAO, Peng | JUST /CWPAN |  |  | haopeng@just.edu.cn |
| HUANG Shanguo | BUPT/CWPAN |  |  | shghuang@upt.edu.cn |
| JIN, Depeng | Tsinghua/CWPAN |  |  | jindp@tsinghua.edu.cn |
| LI, Dejian | Huawei/CWPAN |  |  | lidejian@huawei.com |
| LI, Zhiqiang | IMECAS /CWPAN |  |  | lizhiqiang@ime.ac.cn |
| LIU, Jianhan | Media Tek/CWPAN |  |  | Jianhan.liu@mediatek.com |
| LIU, Zongru | Hangzhou Millimeter Wave /CWPAN |  |  | jerry.liu@milliwave.cn |
| LIU, Pei | Hisilicon /CWPAN |  |  | liupei@hisilicon.com |
| LU, Su | Huawei /CWPAN |  |  | su.lu@huawei.com |
| PENG, Xiaoming | I2R/CWPAN |  |  | pengxm@i2r.a-star.edu.sg |
| PNG, Khiam Boon | I2R /CWPAN |  |  | kbpng@i2r.a-star.edu.sg |
| SUN, Bo | ZTE/CWPAN |  |  | sun.bo1@zte.com.cn |
| XIAO, Zhenyu | Beihang |  |  | xiaozy06@gmail.com |
| YEE, James | Media Tek/CWPAN |  |  | james.yee@mediatek.com |
| YUE, Guangrong | UESTC/CWPAN |  |  | yuegr@uestc.edu.cn |
| ZHANG, Changming | Tsinghua/CWPAN |  |  | zhangcm10@mails.tsinghua.edu.cn |
| ZHUO, Lan | CESI/CWPAN |  |  | this_z17812@163.com |
| ZOU, Weixia | BUPT/CWPAN |  |  | zwx0218@bupt.edu.cn |

Abstract

# This document is part and is in support of the complete proposal described in 802.11-13/1301r2 (slides) and 802.11-13/1302r1 (text).

# This document contains physical layer (PHY) simulation results for CDMG HR SC MCSs for the TGaj complete proposal (60GHz).

# Simulation Conditions

Except as noted below, the following general conditions were employed for the PHY simulations.

## Channel Models

- 802.11ad Channel Models

  - Conference room, directional TX, directional RX, LOS

  - Conference room, directional TX, directional RX, NLOS

## Hardware Impairments

### IQ Imbalance

At transmitter, the gain imbalance is 0.5%, and the phase imbalance is 0.5 degree.

At receiver, the gain imbalance is 5%, and the phase imbalance is 5 degree.

### PA Non-linearity

The power amplifier (PA) non-linearity model from 802.11-09/0296r16 was employed.  The proper back off from saturation was selected to meet the both the TX spectral mask and TX EVM requirements of the proposal.

# HR SC PHY Simulations

From the proposal, 802.11-13/0435r1.

## HR SC Modulation and Coding Sets

| CMCS Index | Modulation | Code Rate | Payload Rate，11ad (Mbps) | Payload Rate，1.08GHz (Mbps) | Transmit EVM (dB) |
| --- | --- | --- | --- | --- | --- |
| 10 | pi/2-16QAM | 1/2 | 3080 | 1540 | -19 |
| 11 | pi/2-16QAM | 5/8 | 3850 | 1925 | -20 |
| 12 | pi/2-16QAM | 3/4 | 4620 | 2310 | -21 |
| 13 | pi/2-16QAM | 13/16 | \ | 2502.5 | -23 |
| 14 | pi/2-64QAM | 5/8 | \ | 2887.5 | -25 |
| 15 | pi/2-64QAM | 3/4 | \ | 3465 | -26 |
| 16 | pi/2-64QAM | 13/16 | \ | 3753.75 | -28 |

## Simulation Results

### IQ Imbalance Compensation Evaluation

### PA Non-linearity Compensation Evaluation

### Joint IQ Imbalance and PA Non-linearity Compensation