IEEE P802.11
Wireless LANs

| TGaq – PADP(Pre-Association Discovery Protocol) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| RYU Cheol | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82-42-860-6604 | ryuch@etri.re.kr |

Abstract

This document is the text for proposed changes of the Pre-Association Discovery Protocol (PADP) D0.01

8.4.2.122a Pre-association Discovery Capabilities

Change the Figure 8-401aq and text as the following.

|  | Element ID | Length | Update Counter | Service Type mask | PADP Mode mask | ULP List count (optional) | ULP ID#1 (optional) | … | ULP ID#n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 1 | 1 | 1 | 1 | 1 |  | 1 |

Figure 8-401aq – PADP Capabilities element format

The Element ID field is set to the value given in Table 8-54 [this table requires a new value adding to it].

The value of the Length field is 2 plus the number of optional ULP ID elements.

The Update Counter is the not-decreasing number of the revision of the service information in AP.

The Service Type mask is a 1 octet bit mask, indicating the Service Types which are supported by an AP.  The format of the mask is shown in Figure 8-402aq.

After Figure 8-402aq – Service 3 Type mask format,  Change the text and Table 8-401aq and the following as the below

The PADP identifier (ID) field indicates the PADP supported by an AP. The PADP ID is one of the following

Table 8-401aq – PADP Identifiers

| PADP | ULP ID |
| --- | --- |
| PAD request/response using a Service Identifier Hash | 0 |
| PAD request/response using Service Identifier Number | 1 |
| Encapsulated ULP | 2 |
| Reserved | 3-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

The PADP Mode mask field is a bit mask for the PADPs supported by an AP. The format of the mask is shown in Figure 8-<TBD>

|  | PAD request/response using a Service Identifier Hash | PAD request/response using Service Identifier Number | Encapsulated ULP | PAD Broadcasting | Reserved | Vendor Specific | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4-5 | 6 | 7 |

Figure 8-<TBD> - PADP Mode mask

8.4.6.1 General

Change as the following including Figure 8-403aq

PADP-elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet Length field and a variable-length element-specific Information field. The PADP-element format is shown in Figure 8-403aq:

|  | Info ID |  | Length |  | PADP Protocol- specific Information |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 2 |  | 2 |  | Variable |  |

Figure 8-403aq – PADP-element format

Each PADP-element in 8.4.6 is assigned a value of 276.

The Length field is a 2-octet field that indicates the number of octets in the Information field and is encoded following the conventions given in 8.2.2 (Conventions).

The PADP Protocol-Specific Information field is a variable length field that contains a specific PADP element definition.

8.4.6.2 PADP Request/Response using a Service Identifier Hash

The PADP Request/Response using a Service Identifier Hash (SIH) is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a hash as explained in Annex AQ1.  The hash is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specfic Information fields are defined as follows:

8.4.6.2.1 Request PADP-element

The Request PADP-element is used to request service information between STAs using the PADP Request/Response protocol. The Request PADP-element is included in a GAS Query Request.

|  | Token | PADP Mode mask | SIH | Service Type Mask |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure 8-404aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

PADP Mode mask is a 1-octet field that is a bit mask for the PADP mode talked in by an STA and it is shown in Figure Figure 8-<TDB> PADP Mode mask. In the case of using a Service Identifier Hash, the bit for PAD request/response using a Service Identifier Hash is set to ‘1’.

The Service Identifier Hash (SIH) field is an 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information.

8.4.6.2.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs using the GAS protocol in response to a Request PADP-element. The Response PADP-element is used in a GAS Query Response.

|  | Token | PADP Mode mask | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |  | variable |

Figure 8-405aq – Response PADP-element format

PADP Mode mask is a 1-octet field that is a bit mask for the PADP mode talked in by an STA and it is shown in Figure Figure 8-<TDB> PADP Mode mask.

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-406aq

|  | Length | SIH | Additional Attributes Count | Additional Attribute#1 (optional) | … | Additional Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octet: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-406aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SIH sub-field is described in section 8.4.6.2.1.

The Additional Attributes Count is a 1 octet field indicating the number of Additional Attribute sub-fields.

The Additional Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SIH, for example, a service name string.

The ULP sub-field is an optional 1 octet field that identifies an ULP associated with the SIH and is defined in section 8.4.2.122a.

8.4.6.3 PAD Request/Response using Service Identifier Number

The PADP Request/Response using a Service Identifier Number (SIN) is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a number.  The number is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specfic Information fields are defined as follows:

[Correct sub-clause number as the following]

8.4.6.3.1 Request PADP-element

The Request element is used to request service information between STAs using the PADP Request/Response protocol. The Request PADP-element is included in a GAS Query Request.

|  | Token | PADP Mode mask | SIN | Service Type Mask |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |

Figure 8-406aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

PADP Mode mask is a 1-octet field that is a bit mask for the PADP mode talked in by an STA and it is shown in Figure Figure 8-<TDB> PADP Mode mask. In the case of using Service Identifier Number, the bit for PAD request/response using Service Identifier Number is set to ‘1’.

The SIN sub-field is a 6 octet free form number of a service which is within the Request or Response PADP-element. See Annex AQ1.3 for further information. [this requires some more detail]

[Correct sub-clause number as the following ]

8.4.6.3.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs using the GAS protocol in response to a Request PADP-element. The Response PADP-element is used in a GAS Query Response.

|  | Token | PADP Mode mask | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |  | variable |

Figure 8-407aq – Response PADP-element format

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-408aq

|  | Length | SIN | Additional Attributes Count | Additional Attribute#1 (optional) | … | Additional Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octet: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-408aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SIN sub-field is described in section 8.4.6.2.1

The Additional Attributes Count is a 1 octet field indicating the number of Additional Attribute sub-fields.

The Additional Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SPI, for example, a service name string.

The ULP sub-field is a 1 octet field that identifies an ULP associated with the SPI and is defined in section 8.4.2.122a.

Change the text as the following after 8.4.6.3.2 Response PADP-element

8.4.6.4 PAD Request/Response using ULP Encapsulation

The Encapsulation element is used to exchange upper layer protocol (ULP) frames between STAs using the GAS protocol. The Encapsulation PADP-element is used as a transaction request included in a GAS Query Request or returned as a transaction response in the GAS Query Response.

|  | Length | Token | <TBD> |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 8-409aq – Encapsulation PADP-element format

The Length and Token fields are defined in 8.4.6.1 (General).

The format of the <TBD> sub-field is  <TBD>.

Insert the following after 8.4.6.4 Response PADP-element

8.4.6.5 PAD Broadcasting of Service Information

The PAD Broadcasting of Service Information-element is used to broadcast service information. The PAD Broadcasting of Service Information-element is included in a GAS frame. The bit of Advertisement Broadcasting The field of PADP Mode masks mask in PADP-element format is on. Only one of PAD request/response using a Service Identifier Hash and PAD request/response using  Service Identifier Number bit could be set on. For the broadcasting of service information, the bit for ULP encapsulated can’t be set.

|  | Broadcasting Window | PADP Mode mask | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable |  | variable |

Figure 8-404aq – PAD Broadcasting of Service Information-element format

The Broadcasting Window specifies the window of the next broadcasting of service information for periodic broadcasting.

The PADP Mode mask determines the mode of SD sub-fields. In the cast of the bit for PAD request/response using a Service Identifier Hash is on, the service indentifiers in SD sub-fields is SIH. In the case of the bit for PAD request/response using  Service Identifier Number is on, the service indentifiers in SD sub-fields is SIN.

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-406aq for SIH and Figure 8-408aq for SIN respectifully.

10. MLME

10.24 WLAN interworking with external networks procedures

10.24.3 Interworking procedures: generic advertisement service (GAS)

Insert new subclause 10.24.3.4 after the end of 10.24.3.3 as follows:

10.24.3.4 Pre-Association Discovery Protocol (PADP) procedures

When dot11PADActivated is true, a STA shall include a PADP Capabilities element in Beacon and Probe Response frames.

<TBD> - text on how the PADP-elements are indicated by a STA

<TBD> - text on how the PADP-elements are interpreted by a non-AP STA.

When dot11PADActivated is true, a non-AP STA may use PADP to request information as defined in Table 8-175 (Advertisement protocol ID definitions) from a peer STA that has transmitted an Advertisement Protocol element indicating support for PADP (see 8.4.2.95 (Advertisement Protocol element)) in a Beacon or Probe Response frame. The receiving STA may respond to queries with or without proxying the query to a server in an external network. See Annex AQ2 for more information on use of a Proxy Entity

- <TBD> The PADP Capabilities element advertises service types as well as the PADP protocol supported by the STA.

- <TBD>The requesting STA shall use the protocol advertised  by the responding STA.

If information is not configured for a particular PADP-element, then a query for that element returns that element with no optional fields.

A STA may use PADP to exchange service information as defined in Table 8-190 from a peer STA.  PADP uses the Encapsulation PADP-element to transport a Service Identifier between STAs.

A STA that encounters an unknown or reserved PADP Info ID value in a GAS frame (see 8-210) received without error shall ignore that PADP Info ID and shall parse any remaining PADP Info IDs.

A STA that encounters an unknown vendor-specific PADP-element field or subfield in a GAS frame (see 8-210) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

10.24.3.4.1 PAD Request/Response  using Service Hash  procedures

<TBD> - Normative description of the protocol here.

10.24.3.4.2 PAD Request/Response  using Service Identifier Number  procedures

<TBD> - Normative description of the protocol here.

10.24.3.4.3 PAD Encapsulation Protcol  procedures

<TBD> - Normative description of the protocol here.

Insert the following after the end of 10.24.3.4.3 PAD Encapsulation Protocol procedure

10.24.3.4.4 PAD Broadcasting of Service Information

<TBD> - Normative description of the protocol here.