IEEE P802.11
Wireless LANs

| TGaq – PADP(Pre-Association Discovery Protocol) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-09-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| RYU Cheol | ETRI | 161 Gajeong-dong, Yuseong-gu, Daejeon, Korea | +82-42-860-6604 | ryuch@etri.re.kr |

Abstract

This document is the text for proposed changes of the Pre-Association Discovery Protocol (PADP) D0.01

8.4.2.122a Pre-association Discovery Capabilities

Change the Figure 8-401aq and text as the following.

|  | Element ID | Length | PADP version | PADP CAG version number | Service Hint Information | PADP Mode mask | ULP List count (optional) | ULP ID#1 (optional) | … | ULP ID#n (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Octets | 1 | 1 | 1 | variable | variable | 1 | 1 | 1 |  | 1 |

Figure 8-401aq – PADP Capabilities element format

The Element ID field is set to the value given in Table 8-54 [this table requires a new value adding to it].

The value of the Length field is 2 plus the number of optional ULP ID elements.

PADP version is a version number of PADP for future extensions,

The PADP CAG version number is the revision number of the service information in AP.

The Service Hint Information contains information identifying services which are supported by an AP.

After Figure 8-402aq – Service 3 Type mask format,  Change the text and Table 8-401aq and the following as the below

The PADP identifier (ID) field indicates the PADP supported by an AP. The PADP ID is one of the following

Table 8-401aq – PADP Identifiers

| PADP | ULP ID |
| --- | --- |
| PAD request/response using a Service Identifier Hash | 0 |
| PAD request/response using Service Identifier Number | 1 |
| Encapsulated ULP | 2 |
| Reserved | 3-220 |
| Vendor Specific | 221 |
| Reserved | 222-255 |

The PADP Mode mask field is a bit mask for the PADP modes supported by an AP. The format of the mask is shown in Figure 8-<TBD>

|  | PAD request/response using Service Identifier Hash | Encapsulated ULP | PAD Broadcasting | Reserved | Vendor Specific | Reserved | Reserved |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bit: | 0 | 1 | 2 | 3 | 4-5 | 6 | 7 |

Figure 8-<TBD> - PADP Mode mask

8.4.6.1 General

Change as the following including Figure 8-403aq

PADP-elements are defined to have a common format consisting of a 2-octet Info ID field, a 2-octet Length field and a variable-length element-specific Information field. The PADP-element format is shown in Figure 8-403aq:

|  | Info ID |  | Length |  | PADP Protocol- specific Information |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octets: |  | 2 |  | 2 |  | Variable |  |

Figure 8-403aq – PADP-element format

Each PADP-element in 8.4.6 is assigned a value of 276.

The Length field is a 2-octet field that indicates the number of octets in the Information field and is encoded following the conventions given in 8.2.2 (Conventions).

The PADP Protocol-Specific Information field is a variable length field that contains a specific PADP element definition.

For the PADP Protoco-Specific Information, PADP Sub-element IDs are defined in Table-<TBD> - PADP Sub-element IDs.

Table 8-<TBD> – PADP Sub-element IDs

| PAD sub-element | ID | Reference |
| --- | --- | --- |
| Request PADP-element | 0 | 8.4.6.2.1 |
| Response PADP-element | 1 | <TBD> |
| Encapsulation PADP-element | 2 | <TBD> |
| Service Announcement PADP-element | 3 | <TBD> |
| Service Hint Information PADP-element | 4 | <TBD> |
| Reserved | 5-255 |  |

8.4.6.2 PADP Request/Response using Service Identifier Hash

The PADP Request/Response using Service Identifier Hash is a transaction-oriented protocol for exchanging Service Discovery information, which identifies services by using a hash as explained in Annex AQ1.  The hash is used both as a identifier within a service query request and also as a response identification of services that are available within the AP.

The PADP Protocol-Specfic Information fields are defined as follows:

8.4.6.2.1 Request PADP-element

The Request PADP-element is used to request service information between STAs in a Probe Request and a GAS Query Request.

|  | Token | Number of SIDs | SID #1 | … | SID #n |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 |  | 6 |

Figure 8-404aq – Request PADP-element format

The token identifies a specific instance of the PADP request.

The Number of SIDs is the number of the following SIDs.

The SID fields are an 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information

8.4.6.2.2 Response PADP-element

The Response PADP-element is used to provide service information between STAs in a Probe Response and a GAS Query Response. The frame which has the Response PADP-element is broadcasted.

|  | Token | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-405aq – Response PADP-element format

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The format of the Service Descriptor sub-field is shown in Figure 8-406aq

|  | Length | SID | Additional Attributes Count | Additional Attribute#1 (optional) | … | Additional Attribute #n (optional) | ULP (optional) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Octet: | 2 | 6 | 1 | 4 |  | 4 | 1 |

Figure 8-406aq – Service Descriptor sub-field format

The Length field is a 1 octet field indicating the length of the Service Descriptor sub-field.

The SID sub-field is described in section 8.4.6.2.1.

The Additional Attributes Count is a 1 octet field indicating the number of Additional Attribute sub-fields.

The Additional Attribute sub-field is a 4 octet field each of which contain additional information about the service identified by the SIH, for example, a service name string.

The ULP sub-field is an optional 1 octet field that identifies an ULP associated with the SIH and is defined in section 8.4.2.122a.

Change the text as the following after 8.4.6.3.2 Response PADP-element

8.4.6.3 PADP Request/Response using ULP Encapsulation

The Encapsulation element is used to exchange upper layer protocol (ULP) frames between STAs using the GAS protocol. The Encapsulation PADP-element is used as a transaction request included in a GAS Query Request or returned as a transaction response in the GAS Query Response.

|  | Length | Token | <TBD> |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | variable |

Figure 8-409aq – Encapsulation PADP-element format

The Length and Token fields are defined in 8.4.6.1 (General).

The format of the <TBD> sub-field is  <TBD>.

Insert the following after 8.4.6.4 Response PADP-element

8.4.6.4 PADP Service Announcement

PADP Service Announcement is unsolicited service announcement which a STA provide to other STAs to know the available services.

8.4.6.4.1 Service Announcement PADP-element

The Service Announcment PADP-element is used to provide the available service information through an AP. The Service Announcement PADP-elements is transmitted in broadcasting frames.

|  | Broadcasting Window | Service Descriptor List Count | SD#1 (optional) | … | SD#n (optional) |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |  | variable |

Figure 8-404aq – PAD Broadcasting of Service Information-element format

The Broadcasting Window specifies the window of the next broadcasting of service information for periodic broadcasting.

The Service Descriptor List Count is a 1 octet field indicating the number of Service Descriptor sub-elements in the Response PADP-element.

The SID fields are an 6 octet version of a Unique Service Identifier (USID) for the service which is within the Request or Response PADP-element.  See Annex AQ1.2 for further information

10. MLME

10.24 WLAN interworking with external networks procedures

10.24.3 Interworking procedures: generic advertisement service (GAS)

Insert new subclause 10.24.3.4 after the end of 10.24.3.3 as follows:

10.24.3.4 Pre-Association Discovery Protocol (PADP) procedures

When dot11PADActivated is true, a STA shall include a PADP Capabilities element in Beacon and Probe Response frames.

<TBD> - text on how the PADP-elements are indicated by a STA

<TBD> - text on how the PADP-elements are interpreted by a non-AP STA.

When dot11PADActivated is true, a non-AP STA may use PADP to request information as defined in Table 8-175 (Advertisement protocol ID definitions) from a peer STA that has transmitted an Advertisement Protocol element indicating support for PADP (see 8.4.2.95 (Advertisement Protocol element)) in a Beacon or Probe Response frame. The receiving STA may respond to queries with or without proxying the query to a server in an external network. See Annex AQ2 for more information on use of a Proxy Entity

- <TBD> The PADP Capabilities element advertises service types as well as the PADP protocol supported by the STA.

- <TBD>The requesting STA shall use the protocol advertised  by the responding STA.

If information is not configured for a particular PADP-element, then a query for that element returns that element with no optional fields.

A STA may use PADP to exchange service information as defined in Table 8-190 from a peer STA.  PADP uses the Encapsulation PADP-element to transport a Service Identifier between STAs.

A STA that encounters an unknown or reserved PADP Info ID value in a GAS frame (see 8-210) received without error shall ignore that PADP Info ID and shall parse any remaining PADP Info IDs.

A STA that encounters an unknown vendor-specific PADP-element field or subfield in a GAS frame (see 8-210) received without error shall ignore that field or subfield respectively, and shall parse any remaining fields or subfields for additional information with recognizable field or subfield values.

10.24.3.4.1 PAD Request/Response  using Service Indendifiers  procedures

<TBD> - Normative description of the protocol here.

10.24.3.4.2 PAD Encapsulation Protcol  procedures

<TBD> - Normative description of the protocol here.

Insert the following after the end of 10.24.3.4.2 PAD Encapsulation Protocol procedure

10.24.3.4.3 PAD Service Announcement procedures

<TBD> - Normative description of the protocol here.