IEEE P802.11
Wireless LANs

| D0.01 PHY related comment resolutions (part 1) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-05-21 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | Email |
| Changming Zhang | Tsinghua University |  |  | zhangcm10@mails.tsinghua.edu.cn |

Abstract

This document proposes revised resolutions to TGaj D0.01 CID 83.

# Technical Comments

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 83 | 25.1.1 | 35 | 12 | TR | It's not necessary to split SC into HR and MR. The only difference is MR requested as mandatory and HR is optional. | Keep one single SC |  |

Proposed resolution: Accept

As the comment, we have merged MR SC PHY and HR SC PHY into SC PHY, as shown in 25.6.

## 25.6 CDMG SC PHY

### 25.6.1 Introduction

Transmission and reception of SC PHY PPDUs is mandatory for CMCS 1-9, and optional for CMCS 10-16.

### 25.6.2 Frame format

The SC frame is composed of the Short Training Field (STF), the channel estimation field (CE), the Header, SC blocks and optional training fields, as shown in Figure 25-10.

- F 1Figure 25-10—SC frame format

### 25.6.3 Transmission

#### 25.6.3.1 Header

##### 25.6.3.1.1 General

In the SC PHY, the preamble is followed by the header. The header consists of several fields which define the details of the PPDU to be transmitted. The encoding and modulation of the header is described in subclause .

- The header fields are described in Table 25-11.

- T 1Table 25-11—11 SC Header fields

| Field Name | Number of bits | Start Bit | Description |
| --- | --- | --- | --- |
| Scrambler Initialization | 7 | 0 | bits X1-X7 of the initial scrambler state. |
| CMCS | 6 | 7 | Index into the Modulation and Coding Scheme table |
| Length | 18 | 13 | Number of data octets in the PSDU. Range 1-262143. |
| Additional PPDU | 1 | 31 | Contains a copy of the parameter ADD-PPDU from the TXVECTOR. A value of 1 indicates that this PPDU is immediately followed by another PPDU with no IFS or preamble on the subsequent PPDU. A value of 0 indicates that no additional PPDU follows this PPDU.. |
| Packet type | 1 | 32 | Corresponds to the TXVECTOR parameter PACKET-TYPE. - Packet Type = 0 indicates either a packet whose data part is followed by one or more TRN-R subfields, or a packet that is requesting TRN-R subfields to be appended to a future response packet. - Packet Type = 1 indicates a packet whose data part is followed by one or more TRN-T subfields. The field is reserved when the Training Length field is 0.. |
| Training Length | 5 | 33 | Corresponds to the TXVECTOR parameter TRN-LEN. If the Beam Tracking Request field is 0, the Training Length field indicates the length of the training field. The use of this field is defined in 25.10.2.2.3. A value of 0 indicates that no training field is present in this PPDU. If the Beam Tracking Request field is 1 and the Packet Type field is 1, the Training Length field indicates the length of the training field. If the Packet Type field is 0, the Training Length field indicates the length of the training field requested for receive training.. |
| Aggregation | 1 | 38 | Set to 1 to indicate that the PPDU in the data portion of the packet contains an A-MPDU; otherwise, set to 0. |
| Beam Tracking Request | 1 | 39 | Corresponds to the TXVECTOR parameter BEAM_TRACKING_REQUEST. Set to 1 to indicate the need for beam tracking (9.35.7); otherwise, set to 0. The Beam Tracking Request field is reserved when the Training Length field is 0.. |
| Last RSSI | 4 | 40 | Contains a copy of the parameter LAST_RSSI from the TXVECTOR. When set to 0, this field reserved and ignored by the receiver. The value is an unsigned integer: - Values of 2-14 represent power levels (-71+value×2) dBm. - A value of 15 represents a power greater than or equal to -42 dBm. - A value of 1 represents a power less than or equal to -68 dBm. Value of 0 indicates that the previous packet was not received a SIFS period before the current transmission.. |
| Turnaround. | 1 | 44 | As defined in Table 25-1.. |
| Reserved | 3 | 45 | Set to 0, ignored by receiver |
| HCS | 16 | 48 | Header check sequence. Definition of this field calculation is in Error: Reference source not found. |

All the numeric fields are encoded in unsigned binary, least significant bit first.

Reserved bits are set to 0 by the transmitter and shall be ignored by the receiver.

If the Additional PPDU field is equal to 1, the Training Length field shall be set to 0.

##### 25.6.3.1.2 Modulation and Coding Scheme

The modulation and coding scheme defines the modulation and code rate that is used in the PPDU. The modulation and coding schemes for SC are defined in Table 25-12.

- T 2Table 25-12—Modulation and Coding Scheme for SC PHY

| CMCS Index | Modulation | NCBPS | Repetition | Code Rate | Data Rate (Mbps) |
| --- | --- | --- | --- | --- | --- |
| 1 | π/2-BPSK | 1 | 2 | 1/2 | 192.5 |
| 2 | π/2-BPSK | 1 | 1 | 1/2 | 385 |
| 3 | π/2-BPSK | 1 | 1 | 5/8 | 481.25 |
| 4 | π/2-BPSK | 1 | 1 | 3/4 | 577.5 |
| 5 | π/2-BPSK | 1 | 1 | 13/16 | 625.625 |
| 6 | π/2-QPSK | 2 | 1 | 1/2 | 770 |
| 7 | π/2-QPSK | 2 | 1 | 5/8 | 962.5 |
| 8 | π/2-QPSK | 2 | 1 | 3/4 | 1155 |
| 9 | π/2-QPSK | 2 | 1 | 13/16 | 1251.25 |
| 10 | π/2-16QAM | 4 | 1 | 1/2 | 1540 |
| 11 | π/2-16QAM | 4 | 1 | 5/8 | 1925 |
| 12 | π/2-16QAM | 4 | 1 | 3/4 | 2310 |
| 13 | π/2-16QAM | 4 | 1 | 13/16 | 2502.5 |
| 14 | π/2-64QAM | 6 | 1 | 5/8 | 2887.5 |
| 15 | π/2-64QAM | 6 | 1 | 3/4 | 3465 |
| 16 | π/2-64QAM | 6 | 1 | 13/16 | 3453.75 |

##### 25.6.3.1.3 Generation of the HCS bits

Calculation of the HCS for bits 0-47 of the header is defined in subclause Error: Reference source not found.

##### 25.6.3.1.4 Header encoding and modulation

See 21.6.3.1.4.

#### 25.6.3.2 The Data field

##### 25.6.3.2.1 General

The data field consists of the payload data of the PSDU and possible padding. The data are padded with zeros, scrambled, encoded and modulated as described in the following subclauses.

##### 25.6.3.2.2 Scrambler

See 21.6.3.2.2.

##### 25.6.3.2.3 Encoding

- See 21.6.3.2.3

##### 25.6.3.2.4 Modulation Mapping

###### 25.6.3.2.4.1 General

The coded and padded bit stream is converted into a stream of complex constellation points according to the modulation specified in the CMCS table.

###### 25.6.3.2.4.2 π/2-BPSK Modulation

In π/2-BPSK modulation, the input bit stream is mapped according to the following equation: sk=2⋅ck−1, where ck is the kth input coded (or scrambled pad) bit. Each output symbol is then rotated according to the following equation:.

- F 2Figure 25-11—BPSK constellation bit encoding

NOTE – With appropriate choice of transmit filtering, π/2-BPSK is equivalent to a precoded pulse-shaped MSK (e.g., GMSK). The precoder is simply bout,k=bin,k⊕bin,k−1,k=0,1,..., where bin,k is the scrambled input stream, bin,-1 is 0, and bout,k is the input to the (G)MSK modulator.

###### 25.6.3.2.4.3 π/2-QPSK Modulation

In π/2-QPSK modulation, the input bit stream is grouped into sets of 2 bits and mapped according to the following equation: , where k is the output symbol index, k = 0, 1, …. Each outputc symbol is then rotated according to the following equation: .

- F 3Figure 25-12—QPSK constellation bit encoding

###### 25.6.3.2.4.4 π/2-16QAM Modulation

In π/2-16QAM modulation, the input bit stream is grouped into sets of 4 bits and mapped according to the following equation: , where k is the output symbol index, k = 0, 1, …. Each output symbol is then rotated according to the following equation: .

- F 4Figure 25-13—16QAM constellation bit encoding

###### 22.7.3.2.4.5 π/2-64QAM Modulation

In π/2-64QAM modulation, the input bit stream is grouped into sets of 6 bits and mapped according to the following equation: , where k is the output symbol index, k = 0, 1, …. Each output symbol is then rotated according to the following equation: .

- F 5Figure 25-14—64QAM constellation bit encoding

##### 25.6.3.2.5 Symbol Blocking and Guard Insertion

Each group of NCBPB bits is pre-pended by π/2-BPSK symbols generated by the 64 point Golay sequence Ga64 defined in 21.11. NCBPB values are shown in Table 25-13. The starting index for the first symbol for π/2 rotation is 0.

- T 3Table 25-13—Values of NCBPB

| Symbol Mapping | NCBPB |
| --- | --- |
| π/2-BPSK | 448 |
| π/2-QPSK | 896 |

448 symbols

448 symbols

448 symbols

64

64

64

GI

DATA

GI

DATA

GI

DATA

64

GI

If the Additional PPDU field within the PLCP header is equal to 0, the final block transmitted is followed by the same Golay sequence guard interval. If the Additional PPDU field within the PLCP header is equal to 1, the final block transmitted of the last PPDU in an A-PPDU is followed by the same Golay sequence guard interval.

- F 6Figure 25-15—Block transmission

### 25.6.4 Performance requirements

#### 25.6.4.1 Transmit Requirements

##### 25.6.4.1.1 Transmit EVM

The transmit EVM accuracy test shall be performed by instrumentation capable of converting the transmitted signal into a stream of complex samples, with sufficient accuracy in terms of I/Q arm amplitude and phase balance, dc offsets, phase noise, etc.

The instrumentation shall perform carrier lock, symbol timing recovery and amplitude adjustment while making the measurements. The equalizer shall be trained using information in the SC preamble (STF and/or CEF). For the SC PHY EVM, measuring Ns samples at the sample rate, the measured symbols should not contain the first and the last hundred symbols of a given packet (ramp up/down). The EVM is calculated according to the formula below:

Where Ns is the number of samples to be measured and Ns shall be 1000, is the average power of the constellation, is the complex coordinates of the measured symbol i,  is the complex coordinates of the ideal constellation point for the measured symbol i, and (Io, Qo) is the complex DC term chosen to minimize EVM.

The test equipment should use a root-raised cosine filter with roll-off factor of 0.25 for the pulse shaping filter when conducting EVM measurement.

The transmit pulse shaping used is left to the implementer.

The relative constellation error (EVM) shall not exceed a CMCS dependent value according to the table below.

- T 4Table 25-14—Modulation and Coding Scheme for SC PHY

| CMCS Indexes | Modulation | Coding Rate | EVM Value [dB] |
| --- | --- | --- | --- |
| 1 | π/2-BPSK | 1/2 with repetition | -6 |
| 2 | π/2-BPSK | 1/2 | -7 |
| 3 | π/2-BPSK | 5/8 | -9 |
| 4 | π/2-BPSK | 3/4 | -10 |
| 5 | π/2-BPSK | 13/16 | -12 |
| 6 | π/2-QPSK | 1/2 | -11 |
| 7 | π/2-QPSK | 5/8 | -12 |
| 8 | π/2-QPSK | 3/4 | -13 |
| 9 | π/2-QPSK | 13/16 | -15 |
| 10 | π/2-16QAM | 1/2 | -19 |
| 11 | π/2-16QAM | 5/8 | -20 |
| 12 | π/2-16QAM | 3/4 | -21 |
| 13 | π/2-16QAM | 13/16 | -23 |
| 14 | π/2-64QAM | 5/8 | -25 |
| 15 | π/2-64QAM | 3/4 | -26 |
| 16 | π/2-64QAM | 13/16 | -28 |

##### 25.6.4.1.2 Time of Departure accuracy

The Time of Departure accuracy test evaluates TIME_OF_DEPARTURE against aTxPmdTxStartRMS and aTxPmdTxStartRMS against TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH as defined in Annex T with the following test parameters:

- MULTICHANNEL_SAMPLING_RATE is 880x106 sample/s

- FIRST_TRANSITION_FIELD is Short Training field

- SECOND_TRANSITION_FIELD is Channel Estimation field

- TRAINING_FIELD is Channel Estimation field

- TIME_OF_DEPARTURE_ACCURACY_TEST_THRESH is 80 ns

NOTE — The indicated windowing applies to the time of departure accuracy test equipment, and not the transmitter or receiver.

#### 25.6.4.2 Rx Requirements

##### 25.6.4.2.1 Introduction

This subclause describes the receiver requirements of the CDMG SC PHY.

##### 25.6.4.2.2 CCA

The start of a valid CDMG SC PHY transmission at a receive level greater than the minimum sensitivity for CMCS 1 (-68dBm) shall cause CCA to indicate busy with a probability > 90% within 1 us. The receiver shall hold the carrier sense signal busy for any signal 20 dB above the minimum sensitivity for CMCS 1.