IEEE P802.11
Wireless LANs

| 802.11 Location Related Corrections to Draft 2.7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-05-15 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Carlos Aldana | Qualcomm Corporation |  |  | caldana@qca.qualcomm.com |
| Jonathan Segev | Intel Corporation |  |  | jonathan.segev@intel.com |
| Qi Wang | Broadcom Corporation |  |  | qi.wang@broadcom.com |

Abstract

This document contains some corrections (in blue) to Revmc Draft 2.7.

Rev0 is initial submission.

Rev1 has 1/64 used throughout for range measurements.  Incorporate Mark Rison’s suggested edits.  To prevent having to use RFC 3396, added clause that Location Civic field be limited to 255 bytes.

Rev2 has additional edit of dot11MgmtOptionFineTimingMsmtImplemented and adding “initial” in front of Fine Timing Measurement frame.

Rev3 added Qi as co-author.

Rev4 changed the Location Civic field limit to a NOTE in Section 10.11.9.9

Rev5 changed Figure 8-586 in Section 8.4.4.12 to reflect the fact that LCI subelement can be 2 or 18 octets

# Pertinent comment

Please fix any errors related to Draft 2.7.

Add the following in abbreviation section 3.1:

fine timing measurement (FTM) procedure: A procedure that allows a STA to obtain its range to another STA.

MLME-FINETIMINGMSMT.request

- Function

This primitive requests the transmission of a Fine Timing Measurement frame to a peer entity.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.request(

Peer MAC Address,
Dialog Token,
Follow Up Dialog Token,
t1,
Max t1 Error,
t4,
Max t4 Error,
VendorSpecific
)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which the Fine Timing Measurement frame is sent. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates the end of the transaction. |
| Follow Up Dialog Token | Integer | 0–255 | The dialog token of a Fine Timing Measurement frame which the current frame follows. See 10.24.6 (fine timing measurement procedure(#46)).(#2164) |
| t1 | Integer | 0–(248–1) | Set to the value of t1 (see Error: Reference source not found(#2164)) expressed in 0.1 ns units. |
| Max t1 Error | Integer | 0–32 767 (#2164) | Maximum error in the t1 value expressed in 0.1 ns(#2164) units; see 8.6.8.35 8.6.15.3 (Fine Timing Measurement frame format). A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767 (#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs. |
| t4 | Integer | 0–(248–1) | Set to the value of t4 (see Error: Reference source not found(#2164)) expressed in 0.1 ns units. |
| Max t4 Error | Integer | 0–32 767 (#2164) | Maximum error in t4 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767 (#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs. |
| VendorSpecific | A set of information elements | As defined in 8.4.2.25 (Vendor Specific element) | Zero or more elements. |

- When generated

This primitive is generated by the SME to request that a Fine Timing Measurement frame be sent to a peer entity.

- Effect of receipt

On receipt of this primitive, the MLME constructs a Fine Timing Measurement frame with the specified parameters. This frame is then scheduled for transmission.

- MLME-FINETIMINGMSMT.confirm

- Function

This primitive indicates that a Fine Timing Measurement frame has been successfully received by the peer STA to which it was sent.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.confirm(

Peer MAC Address,

Dialog Token,

t1,

Max t1 Error,

t4,

Max t4 Error(#1015)

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity to which acknowledges the receipt of the Fine Timing Measurement frame. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates the end of the transaction. |
| t1 | 48-bit unsigned Integer | 0–(248–1) | Set to the value of t1 (see Error: Reference source not found(#2164)) expressed in 0.1 ns units. |
| Max t1 Error | Integer | 0–32 767 (#2164) | Maximum error in the t1 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767 (#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs. |
| t4 | 48-bit unsigned Integer | 0–(248–1) | Set to the value of t4 (see Error: Reference source not found) expressed in 0.1 ns units. |
| Max t4 Error (#1015) | Integer | 0–32 767 (#2164) | Maximum error in t4 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767 (#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs. |

- When generated

This primitive is generated by the MLME when an (#190)(#1198)Ack frame corresponding to the Fine Timing Measurement frame is received from the peer STA.

- Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

- MLME-FINETIMINGMSMT.indication

- Function

This primitive indicates that a Fine Timing Measurement frame has been received and the corresponding (#190)(#1198)Ack frame has been transmitted.

- Semantics of the service primitive

The primitive parameters are as follows:

MLME-FINETIMINGMSMT.indication(

Peer MAC Address,

Dialog Token,

Follow Up Dialog Token,

t1,

Max t1 Error,

t4,

Max t4 Error,

t2,

Max t2 Error,

t3,

Max t3 Error,

VendorSpecific

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| Peer MAC Address | MACAddress | Any valid individual addressed MAC Address | The address of the peer MAC entity from which the Fine Timing Measurement frame was sent. |
| Dialog Token | Integer | 0–255 | The dialog token to identify the Fine Timing Measurement transaction. A value of 0 indicates the end of the transaction. |
| Follow Up Dialog Token | Integer | 0–255 1-255 | The dialog token of a Fine Timing Measurement frame which the current frame follows. See 10.24.6 (fine timing measurement procedure(#46))(#2164). |
| t1 | 48-bit unsigned integer | 0–(248–1) | Set to the value of t1 (see Error: Reference source not found) expressed in 0.1 ns(#2164) units. |
| Max t1 Error | Integer | 0–32 767 (#2164) | Maximum error in the t1 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767(#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs(#2164). |
| t4 | 48-bit unsigned integer | 0–(248–1) | Set to the value of t4 (see Error: Reference source not found) expressed in 0.1 ns(#2164) units. |
| Max t4 Error | Integer | 0–32 767 (#2164) | Maximum error in t4 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767(#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs(#2164). |
| t2 | 48-bit unsigned Integer | 0–(248–1) | Set to the value of t2 (see Error: Reference source not found) expressed in 0.1 ns(#2164) units. |
| Max t2 Error | Integer | 0–32 767 (#2164) | Maximum error in t2 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767(#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs(#2164). |
| t3 | 48-bit unsigned integer | 0–(248–1) | Set to the value of t3 (see Error: Reference source not found) expressed in 0.1 ns(#2164) units. |
| Max t3 Error | Integer | 0–32 767 (#2164) | Maximum error in t3 value expressed in 0.1 ns(#2164) units. A value of 0 indicates that the upper bound on the error is unknown. A value of 32 767(#2164) indicates that the upper bound on the error is greater than or equal to 3.2767 µs(#2164). |
| VendorSpecific | A set of information elements | As defined in 8.4.2.25 (Vendor Specific element) | Zero or more elements. |

- When generated

This primitive is generated by the MLME when a valid Fine Timing Measurement frame is received.

- Effect of receipt

On receipt of this primitive, the SME uses the information contained within the notification.

- Location Configuration Information (#1294)report

The Originator Requesting STA MAC Address subelement contains the MAC address of the STA that requested the Location Information and it is present whenever the location subject definition field in the    corresponding LCI (#1294)request was set to 2. The format of the Originator Requesting STA MAC Address subelement is shown in Error: Reference source not found.

The Target MAC Address subelement contains the MAC address of the STA whose Location Information was requested and it is present whenever the location subject definition field in the corresponding LCI (#1294)request was set to 2. The format of the Target MAC Address subelement is shown in Error: Reference source not found.

The Z subelement is used to report the floor and location of the STA with respect to the floor level. The format of the Z subelement is shown in Z subelement format.

|  | Subelement ID | Length | STA Floor Information | STA Height Above Floor | STA Height Above Floor Uncertainty |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 2 | 2 | 1 |
| - Z subelement format(#2403) |  |  |  |  |  |

(#2403)

The Subelement ID field is equal to the value for Z in Error: Reference source not found.(#2403)

The Length field is defined in 8.4.3 ((#2041)Subelements).(#2403)

The format of the STA Floor Info field is defined in Z subelement format.

|  | B0 | B1 B14 | B15 |
| --- | --- | --- | --- |
|  | Expected to Move | STA Floor Number | Reserved |
| Bits: | 1 | 14 | 1 |
| - STA Floor Info field format(#2403) |  |  |  |

(#2403)

The Expected to Move field indicates whether the STA is expected to change its location. The value 1 indicates that the STA is expected to change its location. The value 0 indicates that the STA is not expected to change its location. (#2403)

NOTE—Examples of STAs that are expected to move include a) battery-powered STAs, b) STAs installed within trains/vehicles, c) STAs installed for temporary events.(#2403)

The STA Floor Number field indicates the floor number of the STA. A higher value indicates a higher floor, and the integer approximates the floor number labels used at the venue (e.g., in stairwells and elevators, if present). The field is encoded as a 2s complement integer with units of 1/16-th of a floor. The value –8192 indicates an unknown floor number. The value 8191 indicates the STA's floor is 8191/16 floors or more. The value –8191 indicates the STA’s floor is –8191/16 floors or less. (#2403)

NOTE—For example, a building with floors labelled as Basement 1, Ground, Mezzanine, 1, and 2 might have the floors identified by STA Floor Number values equal -16, 0, 8, 16 and 32 respectively.

The STA Height Above Floor field indicates the height of the STA above the floor. The field is coded as a 2s complement integer with units of 1/64 m. The value –32 768 indicates an unknown STA height above floor. The value –32 767 indicates the height of the STA above the floor is –32767/64 -32767/256 meters or less. The value 32 767 indicates the height of the STA above the floor is 32767/64 32767/256 meters or more.(#2403)

An STA Height Above Floor Uncertainty value of 0 indicates an unknown STA height above floor uncertainty. Values 19 or higher are reserved. A value from 1 to 18 indicates that the actual STA height above floor, a, is bounded according to:(#2403)

Editor : Please delete following figure and replace with Equation below.

h-211-u <= a <= h+211-u

where

h	is the value in units of 1/64 m of the STA Height Above Floor field

u	is the value of the STA Height Above Floor Uncertainty field

If the STA Height Above Floor field indicates an unknown STA height above floor, the STA Height Above Floor Uncertainty field is set to 0. (#2403)

The Relative Location Error subelement is used to report the location error of STAs with respect to a reference STA (rather than with respect to an absolute geographic location). The format of the Relative Location Error subelement is defined in Relative Location Error subelement format.

|  | Subelement ID | Length | Reference STA | Relative Location Error |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 6 | 1 |
| - Relative Location Error subelement format(#2403) |  |  |  |  |

(#2403)

The Subelement ID field is equal to the value for Relative Location Error in Error: Reference source not found.(#2403)

The Length field is defined in 8.4.3 ((#2041)Subelements).(#2403)

The Reference STA field contains the MAC address of the reference STA.(#2403)

The format of the Relative Location Error field is defined in Relative Location Error field format.

|  | B0 B3 | B4 B7 |
| --- | --- | --- |
|  | Power of Two Horizontal Error | Power of Two Vertical Error |
| Bits: | 4 | 4 |
| - Relative Location Error field format(#2403) |  |  |

(#2403)

The Power Of Two Horizontal Error field contains an upper bound on the error between the horizontal location of the Reference STA and the Latitude and Longitude fields in the LCI subelement. The Power Of Two Horizontal Error field indicates a relative horizontal error of  m, where p is the value of the Power of Two Horizontal Error field in the range 0 to 13. The value 14 indicates a relative horizontal error of greater than 32 m. The value 15 indicates an unknown relative horizontal error. (#2403)

The Power Of Two Vertical Error field contains an upper bound on the error between the vertical location of the Reference STA and the Altitude field in the LCI subelement. The Power Of Two Vertical Error field indicates a relative vertical error of  m, where p is the value of the Power Of Two Vertical Error field in the range 0 to 13. The value 14 indicates a relative vertical error of greater than 32 m. The value 15 indicates an unknown relative vertical error. (#2403)

The Usage Rules subelement is used to report the usage rules of the reporting STA. The format of the Usage-rules subelement is defined in Usage-rules subelement format.

|  | Subelement ID | Length | Usage Rules Parameters | Retention Expires Relative |
| --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 2 |
| - Usage-rules subelement format(#2403) |  |  |  |  |

(#2403)

The Subelement ID field is equal to the value for Usage Rules in Error: Reference source not found.(#2403)

The Length field is defined in 8.4.3 ((#2041)Subelements).(#2403)

The Usage Rules Parameters field is defined in Usage Rules Parameters field format.

|  | B0 | B1 | B2 B7 |
| --- | --- | --- | --- |
|  | Retransmission Allowed | Retention Expires Relative Present | reserved |
| Bits: | 1 | 1 | 6 |
| - Usage Rules Parameters field format(#2403) |  |  |  |

(#2403)

The Retransmission Allowed field definition is the same as the definition for the retransmission-allowed element in IETF RFC 4119, except that the “no” and “yes” text encoding specified in IETF RFC 4119 is replaced by 0 and 1 respectively.

The Retention Expires Relative Present field indicates whether Retention Expires Relative field is present. The value 1 indicates that the Retention Expires Relative field is present; otherwise the Retention Expires Relative field is not present. If the Retention Expires Relative field is not present, it indicates that the retention duration of the LCI report field is unbounded.(#2403)

The definition of the Retention Expires Relative field is the same as the definition for the retention-expires element in IETF RFC 4119, except that the absolute time text encoding specified in IETF RFC 4119 is replaced by a relative binary encoding: the Retention Expires Relative field is encoded as a number of hours in the future relative to the time of transmission of the Usage Rules subelement.

The Vendor Specific subelement has the same format as the Vendor Specific element (see Error: Reference source not found). Multiple Vendor Specific subelements are optionally present in the list of optional subelements.

- Location Civic (#1294)report

The Location Civic (#1294)report includes the location information defined in Civic format for the location subject provided in the Location Civic (#1294)request, as shown in Location Civic Report field format.

|  | Civic Location Type | Location Civic Subelement | Optional Subelements |
| --- | --- | --- | --- |
| Octets: | 1 | variable(#2403) | variable |
| - Location Civic Report field format |  |  |  |

The Civic Location Type field contains the format of location information in the Civic Location field, as indicated in Error: Reference source not found.

The subelement IDs of the Location Civic Report field are defined in Subelement.

| - Subelement(#2403) IDs for Location Civic report(#1294)(#1429) |  |  |
| --- | --- | --- |
| Subelement ID | Name | Extensible |
| 0 | Location Civic(#2403) | No |
| 1 | Originator Requesting STA MAC Address | No |
| 2 | Target MAC Address | No |
| 3 | Location Reference |  |
| 4 | Location Shape |  |
| 5 | Map Image |  |
| 6–220 | Reserved |  |
| 221 | Vendor Specific |  |
| 222–255 | Reserved |  |

(#2403)

The Location Civic subelement of the Location Civic Report (see Figure 8-194 (Location Civic Report field format)) is formatted according to Location Civic Report subelement format.

|  | Subelement ID | Length | Location Civic |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| - Location Civic Report subelement format(#2403) |  |  |  |

(#2403)

The Subelement ID is equal to Location Civic as defined in Subelement.(#2403)

The Location Civic field contains the location information in the format as indicated in the Civic Location Type field. When the Civic Location Type is IETF RFC 4776-2006:(#2403)

- Location Civic field is formatted according to IETF RFC 4776-2006 starting at the country code field (i.e., excluding the GEOCONF_CIVIC/ OPTION_GEOCONF_CIVIC, N/option-len and what fields)

- An unknown civic location is indicated by a subelement Length of 0 and a zero-length Location Civic field

- The Civic Location field follows the little-endian octet ordering

When the Civic Location Type is IETF RFC4776-2006, the Optional Subelements field optionally(#1677) includes the Location Reference, Location Shape, Map Image, and Vendor Specific subelements as defined in Subelement.

When the Civic Location Type value is Vendor Specific, a Vendor Specific subelement is included in the Optional Subelements field that identifies the Organization Identifier corresponding to the Civic Location Type.

The Optional Subelements field format contains zero or more subelements with subelement ID greater than or equal to 1,(#2403) each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable-length Data field, as shown in Figure 8-565 (Subelement format). The optional subelements are ordered by nondecreasing subelement ID.

The Subelement ID field values for the defined (#2403)subelements are shown in Subelement. A Yes in the Extensible column of a subelement listed in Subelement indicates that the length of the subelement might be extended in future revisions or amendments of this standard. When the Extensible column of an element is Subelement, then the subelement might be extended in future revisions or amendments of this standard by defining additional subelements within the subelement. See 9.26.9 (Extensible subelement parsing).

The Originator Requesting STA MAC Address subelement contains the MAC address of the STA that requested the Location Information and it is present whenever the location subject definition field in the corresponding Location Civic (#1294)request was set to 2. The format of the Originator Requesting STA MAC Address subelement is shown in Error: Reference source not found.

The Target MAC Address subelement contains the MAC address of the STA whose Location Information was requested and it is present whenever the location subject definition field in the corresponding Location Civic (#1294)request was set to 2. The format of the Target MAC Address subelement is shown in Error: Reference source not found.

The format of the Location Reference subelement is shown in Location Reference subelement format.

|  | Subelement ID | Length | Location Reference |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | variable |
| - Location Reference subelement format |  |  |  |

The Location Reference is an ASCII string that defines a position on a floor from which the relative location contained in the Location Shape subelement is offset. A Location Reference value of 0 length indicates that the position of the Location Shape is top north west corner (i.e., 0,0) of the floor plan that on which the Location Shape is defined.

- Fine Timing Measurement range report(#2403)

The format of the Measurement Report field corresponding to a Fine Timing Measurement range report is shown in Measurement Report field format for a Fine Timing Measurement Range re.

|  | Range Entry Count | Range Entry | Error Entry Count | Error Entry | Optional Subelements |
| --- | --- | --- | --- | --- | --- |
| Octets: | 1 | M x 14 | 1 | N x 11 | variable |
| - Measurement Report field format for a Fine Timing Measurement Range report(#2403) |  |  |  |  |  |

The Range Entry Count field indicates the number of Range Entry fields (i.e., M in Measurement Report field format for a Fine Timing Measurement Range re).

The Range Entry field indicates parameters relating to a successful range measurement with a single AP, and is formatted according to Error: Reference source not found.

|  | Measurement Start Time | BSSID | Range | Max Range Error |
| --- | --- | --- | --- | --- |
| Octets: | 4 | 6 | 2 3 | 2 |

The Measurement Start Time field contains the least significant 4 octets of the TSF (synchronized with the associated AP) at the time (± 32 µs) at which the first initial Fine Timing Measurement frame was transmitted where the timestamps of both the frame and response frame were successfully measured.

The BSSID field contains the BSSID of the AP whose range is being reported.

The Range field indicates the estimated range between the requested STA and the AP using the fine timing measurement procedure, in units of 1/64 1/256 m. A value of 216–1 224–1 indicates a range of (216–1)/64 m (224–1)/256 m or higher. See 10.11.9.11 (Fine Timing Measurement Range report(#2403)).

The Max Range Error field contains an upper bound for the error in the value specified in the Range field, in units of 1/64 m of 1/256m.   A value of zero indicates an unknown error. A value of 216–1 indicates a maximum range error of (216 -1)/64 (216 -1)/256 or higher. For instance, a value of  512 128 in the Max Range Error field indicates that the value in the Range field has a maximum error of ± 2 m.

The Error Entry Count field indicates the number of Error Entry fields (i.e., N in Measurement Report field format for a Fine Timing Measurement Range re).

The Error Entry field indicates parameters relating to a failed range measurement with a single AP, and is formatted according to Error Entry field format.

|  | Measurement Start Time | BSSID | Error Code |
| --- | --- | --- | --- |
| Octets: | 4 | 6 | 1 |
| - Error Entry field format(#2403) |  |  |  |

The Measurement Start Time field contains the least significant 4 octets of the TSF (synchronized with the associated AP) at the time (± 32 µs) at which the fine timing measurement failure was first detected.

The BSSID field contains the BSSID of the AP whose range was attempted to be measured.

The Error Code field is defined in Error Code field values.

| - Error Code field values(#2403) |  |
| --- | --- |
| Error Code | Meaning |
| 0–2 | Reserved |
| 3 | AP reported “Request incapable” |
| 4 | AP reported “Request failed. Do not send new request for a specified period” |
| 5–7 | Reserved |
| 8 | Unable to successfully transmit to AP |
| 9–255 | Reserved |

- No subelements are defined for this report, so the following is bogus.

The Optional Subelements field format contains zero or more subelements, each consisting of a 1-octet Subelement ID field, a 1-octet Length field, and a variable-length Data field, as shown in Figure 8-565 (Subelement format). Any optional subelements are ordered by nondecreasing Subelement ID.

- Extended Capabilities element

The Extended Capabilities element carries information about the capabilities of a (#1268)STA that augment the capabilities specified in(#1269) the Capability Information field.(Ed) The format of this element is shown in Extended Capabilities element format.

|  | Element ID | Length | Capabilities |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | n |
| - Extended Capabilities element format |  |  |  |

The Element ID and Length fields are defined in Error: Reference source not found.(#139)

The Capabilities field is a bit field indicating the capabilities being advertised by the STA transmitting the element. The length of the Capabilities field is a variable n. The Capabilities field is shown in Capabilities fieldinput.doc.

| - Capabilities fieldinput.doc |  |  |
| --- | --- | --- |
| Bit | Information | Notes |
| 70 (Ed) (#46) | Fine Timing Measurement Responder | The STA sets the Fine Timing Measurement Responder field to 1 when dot11(#1676)FineTimingMsmtRespActivated is true and sets it to 0 otherwise. See 10.24.6 (fine timing measurement procedure(#46)). |
| 71 (Ed) (#46) | Fine Timing Measurement Initiator | The STA sets the Fine Timing Measurement Initiator field to 1 when dot11(#1676)FineTimingMsmtInitActivated is true and sets it to 0 otherwise. See 10.24.6 (fine timing measurement procedure(#46)). |

If a STA does not support any of capabilities defined in the Extended Capabilities element, then the STA is not required to transmit the Extended Capabilities element.

NOTE—The fields of the Extended Capabilities element are not dynamic. They are determined by the parameters of the MLME-START.request or MLME-JOIN.request primitive that caused the STA to start or join its current BSS, and they remain unchanged until the next MLME-START.request or MLME-JOIN.request primitive.

- Fine Timing Measurement Parameter element(#2164)

The Fine Timing Measurement Parameter element contains a number of fields that are used to advertise the desired requested or allocated fine timing measurement configuration from one STA to another.    The Fine Timing Measurement Parameter element may be included in the initial Fine Timing Measurement Request frame Frame, as described in 8.6.8.25 (Fine Timing Measurement Request frame format(#46)), and the initial Fine Timing Measurement frame, as described in 8.6.8.26 (Fine Timing Measurement frame format(#46)).   The use of the Fine Timing Measurement Parameter element is described in 10.24.6 (fine timing measurement procedure(#46)).

The format of the Fine Timing Measurement Parameter element is shown in Fine Timing Measurement Parameter element format.

|  | Element ID | Length | Fine Timing Measurement Parameter Information |
| --- | --- | --- | --- |
| Octets: | 1 | 1 | 9 |
| - Fine Timing Measurement Parameter element format(#2164) |  |  |  |

- Reviewers, please carefully check the updated bit offsets below.

The format of the Fine Timing Measurement Parameter Information field  is shown in Fine Timing Measurement Parameter Information field format.

|  | B0 B2 | B3 B7 | B8 B11 | B12 B15 | B16 B23 | B24 B39 |
| --- | --- | --- | --- | --- | --- | --- |
|  | Status Indication | Value | Number of Bursts Exponent | Burst Timeout | Min Delta FTM | Partial TSF Timer |
| Bits: | 3 | 5 | 4 | 4 | 8 | 16 |

|  | B40 | B41 | B42 | B43 B47 | B48 B49 | B50 B55 | B56 B71 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ASAP | FTM_1 Timestamps Available | Reserved | FTMs per Burst | Reserved | FTM Channel Spacing/Format | Burst Period |
| Bits: | 1 | 1 | 1 | 5 | 2 | 6 | 16 |
| - Fine Timing Measurement Parameter Information field format |  |  |  |  |  |  |  |

The Element ID and Length fields are defined in 8.4.2.1 (General).

When the Fine Timing Measurement Parameter element is sent in the initial Fine Timing Measurement frame, the Status Indication and Value fields provide the result of an initial Fine Timing Measurement Request frame; otherwise, these fields are reserved.

The Status Indication field indicates the responding STA’s response to the Fine Timing Request.   The encoding of the Status Indication field is shown in Status Indication field values.

| - Status Indication field values(#2164) |  |
| --- | --- |
| Value | Description |
| 0 | Reserved |
| 1 | Successful. Indicates that FTM measurements Measurement exchanges are about to begin. All fields accepted. |
| 2 | Overridden. Indicates that FTM measurements Measurement exchanges are about to begin. One or more fields overridden. |
| 3 | Request incapable (do not send same request again). |
| 4 | Request failed. Do not send new request for Value seconds. |
| 5 | ASAP request incapable (do not send same ASAP request again). |
| 6 | ASAP request failed. Do not send new ASAP request for Value seconds. |
| 7 | Reserved |

When the Status Indication field equals 4 or 6, the Value field contains a duration in units of seconds; otherwise the Value field is reserved.

The Number of Bursts Exponent field indicates how many burst instances, defined in 10.24.6.4 (Measurement exchange(#2164)), are requested/allocated for the FTM session if included in an  initial Fine Timing Measurement Request/Fine Timing measurement frame, or allocated for the FTM session if included in an initial Fine Timing Measurement frame respectively, where the number of burst instances is 2Number of Bursts Exponent. The value 15 in an initial Fine Timing Measurement Request frame indicates no preference by the initiating STA and is valid, indicating 215 burst instances, when set by the responding STA.

The Burst Timeout field indicates the duration of a burst instance. defined in 10.24.6.4 (Measurement exchange(#2164)).    The value 15 in the initial Fine Timing Measurement Request indicates no preference by the initiating STA and is reserved not used when set by the responding STA. Burst Timeout field encoding shows the encoding of this field.

| - Burst Timeout field encoding(#2164) |  |
| --- | --- |
| Value | Represents |
| 0 | 250 µs |
| 1 | 500 µs |
| 2 | 1 ms |
| 3 | 2 ms |
| 4 | 4 ms |
| 5 | 8 ms |
| 6 | 16 ms |
| 7 | 32 ms |
| 8 | 64 ms |
| 9 | 128 ms |
| 10–14 | Reserved |
| 15 | No preference |

The Min Delta FTM field indicates the minimum time between consecutive Fine Timing Measurement frames. It is measured from the start of a Fine Timing Measurement frame to the start of following Fine Timing Measurement frame, in units of 100 µs. The value 0 indicates no preference by the initiating STA and is not used by the responding STA.

The Partial TSF Timer field in an initial Fine Timing Measurement frame indicates the partial value of the responding STA's TSF timer at the time start of the first burst instance of the FTM session. The Partial TSF Timer value is derived as follows, so as to have units of TUs: from the 64 TSF timer bits, remove the most significant 38 bits and the least significant 10 bits.

- STAs don’t have desires.

The ASAP field indicates the desire by the initiating STA’s request to start the first burst instance of the FTM session as soon quickly as possible. When the ASAP field is set to 0, the initiating STA requests the start of the first burst instance specified by the Partial TSF Timer field in the initial Fine Timing Measurement Request frame. When the ASAP field is set to 1 by an initiating STA, the Partial TSF Timer field in the initial Fine Timing Measurement Request frame is reserved.  When the ASAP field is set to 0 by the responding STA, the Partial TSF Timer field in the initial Fine Timing Measurement frame indicates the earliest time the Fine Timing Measurement Request trigger frame should be sent by the initiating STA. The responding STA sets the ASAP field to 1 to indicate the STA’s intent to send a Fine Timing Measurement frame as soon as possible.

- Considered by whom?

NOTE—10 ms is considered a reasonable upper bound on the time taken to respond to the initial Fine Timing Measurement Request frame.

The FTMs per Burst field indicates how many Fine Timing Measurement frames are requested/allocated per burst instance by the initial Fine Timing Measurement Request/Fine Timing Measurement frame, or allocated by the initial Fine Timing Measurement frame, respectively.  The maximum value is 31; values in the range 32–127 are reserved. The value 0 indicates no preference by the initiating STA and is not used by the responding STA.

The FTM_1 Timestamps Available field indicates that the timestamps for the initial Fine Timing Measurement frame have been captured and are valid.  If this is the case, it is set to 1. This field is reserved when transmitted in the initial Fine Timing Measurement Request frame.

- “/” is ambiguous. Is it: “and”, “or”, or “and/or”?

The FTM Channel Spacing/Format field indicates the desired requested or allocated packet bandwidth/format used by all the Fine Timing Measurement frames in an FTM session and is shown in FTM Channel Spacing/Format field. The value 0 indicates no preference by the initiating STA and is not used by the responding STA.

| - FTM Channel Spacing/Format field(#2164) |  |  |
| --- | --- | --- |
| Field Value | FORMAT | CH_BANDWIDTH Bandwidth (MHz) |
| 0 | No preference | No preference |
| 1 | NON_HT non-HT | 5 |
| 2 | NON_HT non-HT | 10 |
| 3 | NON_HT, non-HT excluding Clause 16 (DSSS PHY specification for the 2.4 GHz band designated for ISM -applications) and Clause 17 (High rate direct sequence spread spectrum (HR/DSSS) PHY -specification) | 20 |
| 4 | HT_MF HT-mixed | 20 |
| 5 | VHT | 20 |
| 6 | HT_MF HT-mixed | 40 |
| 7 | VHT | 40 |
| 8 | VHT | 80 |
| 9 | VHT | 80+80 |
| 10 | VHT | 160 |
| 11 | DMG | 2160 |
| 12–63 | Reserved | Reserved |

The Burst Period field indicates the interval between two consecutive burst instances, in units of 100 ms. The value 0 indicates no preference by the initiating STA and is not used by the responding STA.  This field is reserved when the Number of Bursts Exponent is set to 0.

- AP Geospatial Location ANQP-element

The AP Geospatial Location ANQP-element provides the AP’s location in LCI format; see 8.4.2.21.10 (Location Configuration Information (#1294)report).

The format of the AP Geospatial Location ANQP-element is provided in AP Geospatial Location ANQP-element format.

|  | Info ID | Length | Location Configuration Report |
| --- | --- | --- | --- |
| Octets: | 2 | 2 | 2 or 18 |
| - AP Geospatial Location ANQP-element format |  |  |  |

The Info ID and Length fields are defined in Error: Reference source not found.(#1430)

The Location Configuration Report is an 18-octet field and the format is provided in 8.4.2.21.10 (Location Configuration Information (#1294)report). There are no Optional Subelements field present in the Location Configuration Report when it is used in the AP Geospatial Location ANQP-element. This information is taken from the dot11APLCITable MIB object.

- Fine Timing Measurement Request frame format(#46)

The format of the Fine Timing Measurement Request Action field(#2042) is shown in Error: Reference source not found.

|  | Category | Public (#2164) Action | Trigger | LCI Request (optional) | Location Civic Request (optional) | Fine Timing Measurement Parameter (optional) |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | variable (#2403) | variable (#2403) | variable (#2164) |
| - Fine Timing Measurement Request Action field(#2042) format |  |  |  |  |  |  |

The Category(#2164) field is set to the value for Public, specified in Table 8-46 (Category values(#2019)).

The Public Action field is set to indicate a Fine Timing Measurement Request frame, as defined in Error: Reference source not found.

(#2164)The Trigger field set to the value 1 indicates that the initiating STA requests that the responding STA start or continue sending Fine Timing Measurement frames (see 10.24.6 (fine timing measurement procedure(#46))). The Trigger field set to the value 0 indicates that the initiating(#2164) STA requests that the responding(#2164) STA stops sending Fine Timing Measurement frames. Trigger field values 2–255 are reserved.

The LCI Request field is optionally present. If present, it contains a Measurement Request element with Measurement Type equal to LCI request (see Table 8-79 (Measurement Type definitions for measurement requests)), which indicates a request for a Measurement Report element with Measurement Type equal to LCI (see 10.24.6.7 (LCI and Location Civic retrieval using fine timing measurement procedure(#2403))). The Enable bit in the Measurement Request Mode field in the Measurement Request element is set to 0. The Location Subject field in the Measurement Request field of the Measurement Request element is set to Location Subject Remote (see Table 8-91 (Location subject definition)). The Request, Report and Duration Mandatory bits in the Measurement Request Mode field in the Measurement Request element are reserved (see 8.4.2.20.1 (General)).(#2403)

The Location Civic Request field is optionally present. If present, it contains a Measurement Request element with Measurement Type equal to Location Civic request (see Table 8-79 (Measurement Type definitions for measurement requests)), which indicates a request for a Measurement Report element with Measurement Type equal to Location Civic report (see 10.24.6.7 (LCI and Location Civic retrieval using fine timing measurement procedure(#2403))). The Enable bit in the Measurement Request Mode field in the Measurement Request element is set to 0. The Location Subject field in the Measurement Request field of the Measurement Request element is set to Location Subject Remote (see Table 8-91 (Location subject definition)). The Location Service Interval Units field in the Measurement Request field of the Measurement Request element is set to 0. The Request, Report and Duration Mandatory bits in the Measurement Request Mode field within the Measurement Request element are reserved (see 8.4.2.20.1 (General)). (#2403)

The Fine Timing Measurement Parameter is optionally present.  If present, it contains a Fine Timing Measurement Parameter element as defined in 8.4.2.166 (Fine Timing Measurement Parameter element(#2164)).(#2164)

- Fine Timing Measurement frame format(#46)

- Note, in merging the resolution of comments 2403 and 2164, I had to select an ordering for the FTM parameter element wrt the two report elements. I followed the hint in resolution to CID 2403, and placed the Report elements first.

The Fine Timing Measurement frame is used to support the fine timing measurement procedure described in 10.24.6 (fine timing measurement procedure(#46)). The format of the Timing Measurement Action field(#2042) is shown in Fine Timing Measurement Action field.

|  | Category | Public Action | Dialog Token | Follow Up Dialog Token | TOD | TOA |
| --- | --- | --- | --- | --- | --- | --- |
| Octets: | 1 | 1 | 1 | 1 | 6 | 6 |
|  | TOD Error | TOA Error | LCI Report (optional) | Location Civic Report (optional) | Fine Timing Measurement Parameter (optional) |  |
| Octets: | 2 | 2 | variable (#2403) | variable (#2403) | variable (#2164) |  |
| - Fine Timing Measurement Action field(#2042) format |  |  |  |  |  |  |

The Category field is set to the value for Public, specified in Table 8-46 (Category values(#2019)).

The Public Action field is set to indicate a Fine Timing Measurement Request frame, as defined in Error: Reference source not found.

- WG style doesn’t allow mixing octets and bits. So the following two structures defined here. Also defining before natural order in structure because the follow up dialog token field refers to these subfields.

The TOD Error field is structured as shown in Format of the TOD Error field.

|  | B0 B14 | B15 |
| --- | --- | --- |
|  | Max TOD Error TOD Not Continuous | TOD Not Continuous Max TOD Error |
| Bits: | 15 | 1 |
| - Format of the TOD Error field(#2164) |  |  |

The TOA Error field is structured as shown in Format of the TOAD Error field.

|  | B0 B14 | B15 |
| --- | --- | --- |
|  | Max TOA Error TOA Not Continuous | TOA Not Continuous Max TOA Error |
| Bits: | 15 | 1 |
| - Format of the TOAD Error field(#2164) |  |  |

The Dialog Token field is a nonzero value chosen by the responding(#2164) STA to identify the Fine Timing Measurement frame as the first of a pair, with the second or follow-up Fine Timing Measurement frame to be sent later. The Dialog Token field is set to 0 to indicate that the Fine Timing Measurement frame will not be followed by a subsequent follow-up Fine Timing Measurement frame.

The Follow Up Dialog Token is the nonzero value of the Dialog Token field of the last(#2164) transmitted Fine Timing Measurement frame to indicate that it is the follow up Fine Timing Measurement frame and that the TOD, TOA, Max TOD Error and Max TOA Error fields contain the values of the timestamps captured with the first Fine Timing Measurement frame of the pair. The Follow Up Dialog Token is 0 to indicate that the Fine Timing Measurement frame is not a follow up to a last(#2164) transmitted Fine Timing Measurement frame.  The value 0 in this field also indicates that TOD, TOA, Max TOD Error, and Max TOA Error fields are reserved. See 10.24.6 (fine timing measurement procedure(#46)).

The TOD, TOA, Max TOD Error, and Max TOA Error fields are expressed in units of 0.1 ns.

The TOD field contains a timestamp that represents the time at which the start of the preamble of the last(#2164) transmitted Fine Timing Measurement frame appeared at the transmit antenna connector(#1410).

The TOA field contains a timestamp that represents the time at which the start of the preamble of the (#190)(#1198)Ack frame to the last(#2164) transmitted Fine Timing Measurement frame arrived at the receive antenna connector(#1410).

NOTE—The values specified in the TOD and TOA fields are described in 6.3.70 (Fine timing measurement request(#46)).

- The following para inserted from 11-14/160r7, even though not marked as an insert. Edited to remove “may”. Likewise for the similar TOA field.

The TOD Not Continuous field indicates that the TOD value is with respect to a different underlying time base than the last transmitted TOD value. It is set to 1 when a discontinuity is present. It is optionally set to 1 when the TOD field wraps around the maximum counter value of 248–1.   Otherwise, it is set to 0.(#2164)

The Max TOD Error field contains an upper bound for the error in the value specified in the TOD field.

(#2164)NOTE—For instance, a value of 2 in the Max TOD Error field indicates that the value in the TOD field has a maximum error of ± 0.2 ns.

The TOA Not Continuous field indicates that the TOA value is with respect to a different underlying time base than the last transmitted TOA value. It is set to 1 when a discontinuity is present. It is optionally set to 1 when the TOA field wraps around the maximum counter value of 248–1.   Otherwise, it is set to 0.(#2164)

The Max TOA Error field contains an upper bound for the error in the value specified in the TOA field. (#2164)

A value of 0 for the Max TOD Error or the Max TOA Error field indicates that the upper bound on the error in the corresponding TOD or TOA value is unknown. A value of  65535 32767 indicates that the upper bound on the error is greater than or equal to 6.5535 3.2767 us.

The LCI Report field is optionally present. If present, it contains a Measurement Report element with Measurement Type equal to LCI report (see Table 8-103 (Measurement Type definitions for measurement reports)), which either indicates the LCI of the transmitting STA and includes the Z subelement or the element indicates an unknown LCI (see 10.24.6.7 (LCI and Location Civic retrieval using fine timing measurement procedure(#2403))). The Late, Incapable and Refused bits in the Measurement Report Mode field are set to 0.(#2403)

The Location Civic Report field is optionally present. If present, it contains a Measurement Report element with Measurement Type equal to Location Civic report (see Table 8-103 (Measurement Type definitions for measurement reports)), which either indicates the Civic address of the transmitting STA or an unknown Civic address (see 10.24.6.7 (LCI and Location Civic retrieval using fine timing measurement procedure(#2403))). The Late, Incapable and Refused bits in the Measurement Report Mode field are set to 0.(#2403)

The Fine Timing Measurement Parameter field is optionally present.  If present, it contains a Fine Timing Measurement Parameter element as defined in 8.4.2.166 (Fine Timing Measurement Parameter element(#2164)).(#2164)

- Location Civic report

If dot11RMCivicMeasurementActivated is true and civic location information is not available, the STA shall reject a Location Civic (#1294)request and shall respond with a Measurement Report frame including a Measurement Report element with the incapable bit set to 1. If dot11RMCivicMeasurementActivated is true and civic location information is available, the STA shall respond with a Measurement Report frame including one Location Civic Report element.

A Location Civic (#1294)request shall indicate a location request for the requesting STA, the reporting STA, or a third STA with the MAC address specified in the Target MAC address field, by setting the Location Civic request Location Subject field to indicate a Local, a Remote, or a third-party request respectively. Local Location Civic (#1294)request is used by requesting STA to obtain its own location by asking “Where am I?” Remote Location Civic (#1294)request is used by requesting STA to obtain location of the reporting STA by asking “Where are you?” Third-party Location request is used by requesting STA to obtain location of a STA with the MAC address specified in the Target MAC address field.

If dot11RMCivicMeasurementActivated is false, a STA shall reject the received Location Civic (#1294)request and shall respond with a Location Civic (#1294)report where the Incapable bit is set in the Measurement Report Mode field.

If dot11RM3rdPartyMeasurementActivated is false, a STA shall reject any LCI (#1294)request that includes an(#1294) LCI (#1294)request with the Location Subject field equal to 2 and shall respond with a Radio Measurement Report frame including an Radio Measurement Report element with the incapable bit set to 1.

It is optional for a STA to support a Location Civic (#1294)request and a Location Civic (#1294)report with the Location Subject field equal to 2. If dot11RM3rdPartyMeasurementActivated is true and a STA supports Location Civic (#1294)request and Location Civic (#1294)report, the following procedure shall be followed:

- When a non-AP STA requests the civic location of a STA with the MAC address specified in the Target MAC address field, it shall also include its own MAC address in the Originator Requesting STA MAC address field. When an AP receives a Location Civic (#1294)request with the Location Subject field value equal to 2, the AP shall generate a Location Civic (#1294)request to the STA with the MAC address specified in the Target MAC address field. If the AP does not have an association with the STA with the MAC address specified in the Target MAC address field, the AP shall reject the received Location Civic (#1294)request and shall respond with a Location Civic (#1294)report where the Incapable bit is set in the Measurement Report Mode field. The AP shall copy the Originator Requesting STA MAC address and Target MAC address fields into the request from the received Location Civic (#1294)request.

- When a STA receives a Location Civic (#1294)request with the Location Subject field value equal to 2, the STA shall only generate a Location Civic (#1294)report if the MAC address in the Target MAC address field is its own MAC address. When a Location Civic (#1294)report is generated, the reporting STA shall include its MAC address into the Target MAC address field and the MAC address present in the Originator Requesting STA MAC address field of the corresponding Location Civic (#1294)request into the Originator Requesting STA MAC address field. When an AP receives a Location Civic (#1294)report with an Originator Requesting STA MAC address field present, the AP shall generate a Location Civic (#1294)report to the associated STA with the MAC address specified in the Originator Requesting MAC address field. The AP shall copy the Originator Requesting STA MAC address and Target MAC address fields into the Location Civic (#1294)report being transmitted to the originating requesting STA.

When a STA receives a Location Civic (#1294)request with the subject field equal to 2, but does not support third-party location, it shall respond with a Radio Measurement Report frame including a Radio Measurement Report element with the incapable bit set to 1.

If dot11RMCivicMeasurementActivated is true and a STA has its own location configured in Civic format, it shall set the Civic Location field to 1 in the Extended Capabilities element.

If dot11RMCivicMeasurementActivated is true and a STA has its own location available in one or more location formats, as defined in Table 8-97 (Civic Location Type), and includes Civic Location Type Value 0, indicating “IETF RFC4776-2006,” it shall set the Civic Location field to 1 in the Extended Capabilities element.(#2363).  The Location Civic Field shall be less than or equal to 255 octets.

NOTE---The Location Civic field's size limit constrains the amount of information that can be carried.)

If the Location Civic (#1294)report contains the Location Reference and Location Shape subelements, the receiving STA may use the information specified in those subelements in combination with the Civic Location field value for additional granularity on the position reported in the Civic Location field.

If the Location Civic (#1294)report contains the Map Image subelement, the receiving STA’s SME can(#2364) retrieve the floor plan specified by the Map URL field. The method to retrieve the floor plan specified by the Map URL field is out of scope of this document.

- Fine timing measurement procedure(#46)

- Overview(#2164)

- Note to reviewers. Verbs below edited from submission 11-14/160. In particular “may” should only be used in the context “is allowed to”, i.e., where this specification grants permission. It should not be used when expressing an ability granted by this spec, or a possibility.

The fine timing measurement procedure allows a STA to obtain its range to another STA. A STA may perform this procedure with multiple STAs in order to obtain its location.

An FTM session is an instance of a fine timing measurement procedure between an initiating STA and a responding STA along with the associated scheduling and operational parameters of that instance (see 8.4.2.166 (Fine Timing Measurement Parameter element(#2164))). An FTM session is composed of a negotiation, measurement exchange and termination. A STA might have multiple concurrent FTM sessions. Concurrent FTM sessions might occur with responding STAs that are members of different BSSs and possibly different ESSs, or possibly outside of a BSS, each session using its own scheduling, channel and operational parameters.

A responding STA might be required to establish overlapping FTM sessions with a large number of initiating STAs (e.g. an AP providing measurements to STAs at a mall or a store). On the other hand, an initiating STA might have multiple ongoing FTM sessions on the same or different channels with different responding STAs, while being associated with an AP for the exchange of data or signaling.

To support the constraints of both the initiating and responding STAs, during the negotiation phase the initiating STA initially requests a preferred periodic time window allocation. The responding STA subsequently responds by accepting or overriding the allocation request based on its resource availability and capability.

Since some of the initiating STA's activities may be non-deterministic and might have higher precedence than the FTM session (e.g. data transfer interaction with an associated AP), a conflict might prevent the initiating STA from being available at the beginning of the burst instance determined by the responding STA.

- TODO- redraw figure in Visio

Error: Reference source not found shows an example of such scheduling conflicts.

The initiating STA in Error: Reference source not found establishes sessions with responding STA 1 and responding STA 2 on different channels. The sessions' burst instance periodicity might be different as well as the STAs' clock offsets and thus, over time, some temporal conflicts may occur. To overcome this, during each burst instance the initiating STA indicates its availability by transmitting a trigger frame in the form of a Fine Timing Measurement Request frame. During each burst instance, the responding STA transmits one or more Fine Timing Measurement frames as negotiated.

- FTM capabilities(#2164)

Implementation of fine timing measurement is optional for a WNM STA. A STA in which dot11MgmtOptionFineTimingMsmtRespActivated is true is defined as a STA that supports fine timing measurement as a responder. A STA in which dot11MgmtOptionFineTimingMsmtInitActivated is true is defined as a STA that supports fine timing measurement as an initiator.

When dot11MgmtOptionFineTimingMsmtImplemented is true, dot11WirelessManagementImplemented shall be true.

A STA in which dot11MgmtOptionFineTimingMsmtRespActivated is true shall set the Fine Timing Measurement Responder field of the Extended Capabilities element to 1.

A STA in which dot11MgmtOptionFineTimingMsmtInitActivated is true shall set the Fine Timing Measurement Initiator field of the Extended Capabilities element to 1.

A STA in which dot11MgmtOptionFineTimingMsmtRespActivated is false shall set the Fine Timing Measurement Responder field of the Extended Capabilities element to 0.

A STA in which dot11MgmtOptionFineTimingMsmtInitActivated is false shall set the Fine Timing Measurement Initiator field of the Extended Capabilities element to 0.

- Fine timing measurement procedure setup(#2164)

A STA that supports the fine timing measurement procedure as an initiator shall(#2164) transmit a Fine Timing Measurement Request frame to initiate a Ffine Ttiming Mmeasurement procedure. A STA that supports the fine timing measurement procedure as a responder may transmit Fine Timing Measurement frames addressed to a peer STA that also supports the fine timing measurement procedure as an initiator.

The initiating STA sends a Fine Timing Measurement Request frame which may include a set of scheduling parameters to describe the initiating STA’s availability for measurement exchange.   This frame is called the initial Fine Timing Measurement Request frame.

The first Fine Timing Measurement frame in the FTM session is called the initial Fine Timing Measurement frame. The responding STA should transmit an initial Fine Timing Measurement frame within 10 ms in response to the initial Fine Timing Measurement Request frame. This initial Fine Timing Measurement frame shall include the Fine Timing Measurement Parameter element. The value of the Status Indication field indicates whether the request was accepted, modifiedoverridden or rejected.

If the request was accepted or overridden:

---The responding STA’s selection of ASAP should be the same as that requested by the initiating STA.

---The responding STA’s selection of the Min Delta FTM value shall be greater than or equal to the corresponding value requested by the initiating STA.

---The responding STA’s selection of the Number of Bursts Exponent value shall be 0 when the initiating STA requests it to be 0.

When the responding STA is not capable of the initiator’s ASAP=1 request, the responding STA shall send the Status Indication field to ASAP request incapable.  When the initiator requests ASAP=1, and the responding STA is ASAP=1 capable, but is unable to fulfill the ASAP=1 request by the initiating STA, the responding STA shall set the Status Indication field to ASAP request failed.

When the responding STA is unable to meet the Min Delta FTM or Number of Bursts Exponent constraints, it shall send the Status Indication field to Request incapable.

NOTE—Apart from the Status Indication, Value, ASAP, Number of Bursts Exponent, and Min Delta FTM fields, the other fields in the Fine Timing Measurement Parameter element in the initial Fine Timing Measurement frame have no constraints.

- Measurement exchange(#2164)

Fine Timing Measurements frames are sent during each availability time windows called burst instances. The timing of the burst instances are defined by the following parameters:

- Partial TSF Timer – The partial TSF timer, as defined in 8.4.2.166 Fine Timing Measurement Parameter element,   at the beginning of the first burst instance.

- Burst Timeout – The time duration of each burst instance starting at the boundary of a burst period.

- Burst Period – The interval from the beginning of one burst instance to the beginning of the following burst instance.

The initiating STA shall transmit a Fine Timing Measurement Request frame, which serves as a trigger, as soon as it is available on channel to decode the medium. This trigger frame shall have the Trigger field set to 1 and shall not include a Fine Timing Measurement Parameter element. This indicates to the responding STA its availability for the remainder of the burst instance. Following this trigger frame the responding STA shall transmit an Ack frame and should transmit FTMs per Burst Fine Timing Measurement frames. These Fine Timing Measurement frames shall not include a Fine Timing Measurement Parameter element. Within each burst instance, consecutive Fine Timing Measurement frames shall be spaced at least Min Delta FTM apart. Within each burst instance the initiating STA shall perform fine timing measurement on each Fine Timing Measurement frame addressed to it.

The first burst instance shall start at the value indicated by the Partial TSF Timer field.

- TODO: redraw figures in Visio. Note, SWAGs taken at missing figure titles.

The timing parameters of an FTM session are illustrated in Error: Reference source not found.

The initiating STA can request the responding STA to start the burst instance “as soon as possible” by setting the ASAP field to 1. The initiating STA may also request a single burst of fine timing measurements to be taken, in which case it will set the Number of Bursts Exponent field to 0.  If the responding STA performs fine timing measurement on the initial Fine Timing Measurement frame it shall signal this by setting the FTM_1 Timestamps Available field to 1. The first burst instance shall start at the value indicated by the Partial TSF timer field.  The scheduling in this case is illustrated in Error: Reference source not found.

Both the responding STA and initiating STA shall transmit using a single RF chain.

The responding STA shall not transmit Fine Timing Measurement frames using Clause 16 or Clause 17 formats.

The initiating STA may request the Fine Timing Measurement to have a certain format and CH_BANDWIDTH bandwidth by setting the values in the FTM Channel Spacing/Format field of the Fine Timing Measurement Parameter element in the initial Fine Timing Measurement Request frame. The responding STA should transmit Fine Timing Measurement frames with the requested format and CH_BANDWIDTH bandwidth. The responding STA shall not use capabilities beyond those implied by the initial Fine Timing Measurement frame.  The responding STA shall not use a bandwidth wider than requested.  The responding STA shall not use a VHT format if HT-mixed or non-HT format was requested.  The responding STA shall not use an HT format if non-HT format was requested.

A responding STA transmits Fine Timing Measurement frames in overlapping pairs. The first Fine Timing Measurement frame of a pair contains a nonzero value in the Dialog Token field. The follow up Fine Timing Measurement frame contains a Follow Up Dialog Token field set to the value of the Dialog Token field in the first frame of the pair. Dialog Token field values of consecutive Fine Timing Measurement frames shall, excluding retries, be consecutive, except when the value wraps around to 1. The initial Fine Timing Measurement Fframe may or may not have captured timestamps. This is indicated by the FTM_1 Timestamps Available field in the Fine Timing Measurement Parameter element of this frame. With the first Fine Timing Measurement frame, both STAs capture timestamps. The responding STA captures the time at which the Fine Timing Measurement frame is transmitted (t1). The initiating STA captures the time at which the Fine Timing Measurement frame arrives (t2) and the time at which the Ack response is transmitted (t3). The responding STA captures the time at which the Ack frame arrives (t4). See Figure 6-17 (Fine timing measurement primitives and timestamps capture).(Ed) In the follow up Fine Timing Measurement frame, the responding STA transfers the timestamp values it captured (t1 and t4) to the initiating STA. In this follow up Fine Timing Measurement frame, the timestamp values (t1 and t4) shall be the measurement according to the responding STA’s clock (i.e., without correcting the clock offset).

See Error: Reference source not found for fine timing measurements when the ASAP field is set to 0 and Error: Reference source not found when the ASAP field is set to 1.

The (#1198)Ack frame shall(#2164) have the same CH_BANDWIDTH as the preceding action frame.

NOTE—A Fine Timing Measurement frame can contain nonzero values in both the Dialog Token and Follow Up Dialog Token fields, meaning that the Action frame contains follow up information from a previous measurement, and new Timestamp values are captured to be sent in a future follow up Fine Timing Measurement frame.

The round trip time (RTT) is defined by Error: Reference source not found.

- RTT = [(t4 – t1) – (t3 – t2)]

If the (#190)(#1198)Ack frame for a transmitted Fine Timing Measurement frame is not received, the responding(#2164) STA may retransmit the frame. The responding(#2164) STA shall capture a new set of timestamps for the retransmitted frame and its (#190)(#1198)Ack frame.

- Fine timing measurement parameter modification(#2164)

During an ongoing FTM session, an initiating STA may request a modification of the session parameters by transmitting a Fine Timing Measurement Request frame with Trigger field set to 1 and including a new Fine Timing Measurement Parameter element. The ongoing existing FTM session is terminated upon reception of such a Fine Timing Measurement Request frame. This Fine Timing Measurement Request frame becomes an initial Fine Timing Measurement Request frame for the new FTM session, which follows the behavior described in Fine timing measurement procedure setup.

- Fine timing measurement termination(#2164)

There are four(#2164) ways an FTM session is terminated:

- The responding STA can send a Fine Timing Measurement frame with Dialog Token field set to 0.

- The initiating STA can send a Fine Timing Measurement Request frame with Trigger field set to 0.

- The initiating STA requests a modification of the Fine Timing Measurement parameters (see Fine timing measurement parameter modification).

- When the number of burst instances agreed during the FTM session setup has been reached.

- After the number of burst instances indicated in the Number of Bursts Exponent field in the initial Fine Timing Measurement frame has been reached.

- This last para was not cited in doc 160. It has been left in position, although it does not seem to relate to termination.

On receiving a Fine Timing Measurement frame with a Dialog Token for which timestamps have previously been captured, the receiving STA shall discard previously captured timestamps and capture a new set of timestamps.

- Default QMF policy

The default QMF policy is as defined in Default QMF policy. It defines the access category of each (#100)Management frame based on management subtype value, category value, and action value. QMFs not included in this table shall be assigned an access category AC_BE.

| - Default QMF policyinput.doc(11ae) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Management Frame Subtype value from Table 8-1 (Valid type and subtype combinations) | Category value from Table 8-54 (Category values(#2019)) | Action field(#2164) | QMF access category |
| (Re)Association Request/Response | 0000–0011 | N/A | N/A | AC_VO |
| Probe Request (individually addressed | 0100 | N/A | N/A | AC_VO |
| Probe Request (group addressed) | 0100 | N/A | N/A | AC_BE |
| Probe Response | 0101 | N/A | N/A | AC_BE |
| Timing Advertisement | 0110 | N/A | N/A | AC_BE |
| Beacon, ATIM, Disassociation, Authentication, Deauthentication | 1000–1100 | N/A | N/A | AC_VO |
| Spectrum management | 1101 | 0 | 0–3 | AC_BE |
| Spectrum management— channel switch announcement | 1101 | 0 | 4 | AC_VO |
| QoS | 1101 | 1 | 0–3 | AC_VO |
| DLS | 1101 | 2 | 0–2 | AC_BE |
| Block Ack | 1101 | 3 | 0–2 | AC_VO |
| Public | 1101 | 4 | 0, 1, 3, 5–6, 8–9 | AC_BE |
| Public—DSE deenablement, extended channel switch announcement | 1101 | 4 | 2, 4 | AC_VO |
| Public—measurement pilot | 1101 | 4 | 7 | AC_VO |
| Public—TDLS Discovery Response | 1101 | 4 | 14 | AC_VO |
| Public—Fine Timing Measurement (#2164) Frame | 1101 | 4 | <ANA> | AC_VO |
| Public—Fine Timing Measurement Request(#2164) | 1101 | 4 | <ANA> | AC_VO |
| Radio measurement | 1101 | 5 | 0–5 | AC_BE |
| Fast BSS Transition | 1101 | 6 | 0–4 | AC_VO |
| HT | 1101 | 7 | 0–3 | AC_VO |
| HT | 1101, 1110 | 7 | 4–7 | AC_VO |
| SA Query | 1101 | 8 | 0–1 | AC_VO |
| Protected Dual of Public Action | 1101 | 9 | 1–2, 5–6, 8–9 | AC_BE |
| Protected Dual of Public Action—extended channel switch announcement | 1101 | 9 | 4 | AC_VO |
| WNM | 1101 | 10 | 0–24 | AC_BE |
| Unprotected WNM | 1101 | 11 | 0–1 | AC_BE |
| Mesh Action—HWMP Mesh Path Selection | 1101 | 13 | 1 | AC_VO |
| Mesh Action— Congestion Control | 1011 | 13 | 3 | AC_VO |
| Mesh Action | 1101 | 13 | 0, 2, 4–10 | AC_BE |
| Multihop Action | 1101 | 14 | 0–1 | AC_BE |
| Self Protected | 1101 | 15 | 0–5 | AC_VI |
| DMG(#2486) | 1101 | 16 | 0–22 | AC_BE |
| Reserved (used by WFA) | 1101 | 17 | All | AC_BE |
| Fast Session Transfer (#2486) | 1101 | 18 | 0–5 | AC_VO |
| Robust AV Streaming (#2486) | 1101 | 19 | 0–3 | AC_BE |
| Unprotected DMG (#2486) | 1101 | 20 | 0–1 | AC_VO |
| VHT(11ac) | 1101, 1110 | 21 | 0–2 | AC_VO |
| Vendor-specific Protected | 1101 | 126 | N/A | AC_BE |
| Vendor-specific | 1101 | 127 | N/A | AC_BE |

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

dot11(#1676)LocationActivated	TruthValue,

dot11(#1676)FMSImplemented	TruthValue,

dot11(#1676)FMSActivated	TruthValue,

dot11(#1676)EventsActivated	TruthValue,

dot11(#1676)DiagnosticsActivated	TruthValue,

dot11(#1676)MultiBSSIDImplemented	TruthValue,

dot11(#1676)MultiBSSIDActivated	TruthValue,

dot11(#1676)TFSImplemented	TruthValue,

dot11(#1676)TFSActivated	TruthValue,

dot11(#1676)WNMSleepModeImplemented	TruthValue,

dot11(#1676)WNMSleepModeActivated	TruthValue,

dot11(#1676)TIMBroadcastImplemented	TruthValue,

dot11(#1676)TIMBroadcastActivated	TruthValue,

dot11(#1676)ProxyARPImplemented	TruthValue,

dot11(#1676)ProxyARPActivated	TruthValue,

dot11(#1676)BSSTransitionImplemented	TruthValue,

dot11(#1676)BSSTransitionActivated	TruthValue,

dot11(#1676)QoSTrafficCapabilityImplemented	TruthValue,

dot11(#1676)QoSTrafficCapabilityActivated	TruthValue,

dot11(#1676)ACStationCountImplemented	TruthValue,

dot11(#1676)ACStationCountActivated	TruthValue,

dot11(#1676)CoLocIntfReportingImplemented	TruthValue,

dot11(#1676)CoLocIntfReportingActivated	TruthValue,

dot11(#1676)MotionDetectionImplemented	TruthValue,

dot11(#1676)MotionDetectionActivated	TruthValue,

dot11(#1676)TODImplemented	TruthValue,

dot11(#1676)TODActivated	TruthValue,

dot11(#1676)TimingMsmtImplemented	TruthValue,

dot11(#1676)TimingMsmtActivated	TruthValue,

dot11(#1676)ChannelUsageImplemented	TruthValue,

dot11(#1676)ChannelUsageActivated	TruthValue,

dot11(#1676)TriggerSTAStatisticsActivated	TruthValue,

dot11(#1676)SSIDListImplemented	TruthValue,

dot11(#1676)SSIDListActivated	TruthValue,

dot11(#1676)MulticastDiagnosticsActivated	TruthValue,

dot11(#1676)LocationTrackingImplemented	TruthValue,

dot11(#1676)LocationTrackingActivated	TruthValue,

dot11(#1676)DMSImplemented	TruthValue,

dot11(#1676)DMSActivated	TruthValue,

dot11(#1676)UAPSDCoexistenceImplemented	TruthValue,

dot11(#1676)UAPSDCoexistenceActivated 	TruthValue,

dot11(#1676)WNMNotificationImplemented	TruthValue,

dot11(#1676)WNMNotificationActivated	TruthValue,

dot11(#1676)UTCTSFOffsetImplemented	TruthValue,

dot11(#1676)UTCTSFOffsetActivated	TruthValue,

dot11(#1676)FineTimingMsmtImplemented	TruthValue,(#46)

dot11(#1676)FineTimingMsmtRespActivated	TruthValue,(#46)

dot11(#1676)FineTimingMsmtInitActivated	TruthValue,(#46)

dot11LciCivicInNeighborReport	TruthValue(#2403)

}

dot11(#1676)FineTimingMsmtImplemented OBJECT-TYPE(#46)

SYNTAX TruthValue

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute, when true, indicates that the station implementation is capable of supporting Fine Timing Measurement capability when the dot11WirelessManagementImplemented is set to true."

::= { dot11WirelessMgmtOptionsEntry 46 }

dot11(#1676)FineTimingMsmtRespActivated OBJECT-TYPE(#46)

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect at the next occurrence of an MLME-START.request or MLME-JOIN.request primitive.(#1066)

This attribute, when true, indicates that the station capability for Fine Timing Measurement as a responder is enabled. False indicates the station has no Fine Timing Measurement capability as a responder or that the capability is present but is disabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 47 }

dot11(#1676)FineTimingMsmtInitActivated OBJECT-TYPE(#46)

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect at the next occurrence of an MLME-START.request or MLME-JOIN.request primitive.(#1066)

This attribute, when true, indicates that the station capability for Fine Timing Measurement as an initiator is enabled. False indicates the station has no Fine Timing Measurement capability as an initiator or that the capability is present but is disabled."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry ZZ }

- This new variable appears in no groups or compliance statements.

dot11LciCivicInNeighborReport OBJECT-TYPE(#2403)

SYNTAX TruthValue

MAX-ACCESS read-write

STATUS current

DESCRIPTION

"This is a control variable.

It is written by an external management entity or the SME.

Changes take effect as soon as practical in the implementation.

This attribute, when true, indicates that the station includes LCI and Civic location subelements in the Neighbor Report without regard to either dot11FineTimingMsmtRespActivated or dot11FineTimingMsmtInitActivated."

DEFVAL { false}

::= { dot11WirelessMgmtOptionsEntry 48 }