IEEE P802.11
Wireless LANs

| Proposed 802.11ax Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-05-11 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Lei Wang Hui-Ling Lou Hongyuan Zhang Yakun Sun Liwen Chu | Marvell | 5488 Marvell Lane, Santa Clara, CA, 95054 | +1-858-205-7286 | leileiw@marvell.com |

### Abstract

This submission proposes the 802.11ax Functional Requirements as derived from the 802.11ax PAR [Ref-1] and CSD [Ref-2] and also from analyzing the 802.11ax use models [Ref-3].

- Introduction

  - Background

The IEEE Standards Association (IEEE-SA) Standards Board approved 802.11ax in March, 2014.

One of the next basic steps for the 11ax development is to clearly and unambiguously define the functional requirements.

This contribution continues the development effort of 802.11ax functional requirements as discussed in a previous contribution, 13/0306r0.

  - Overview

    - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the TGax amendment, referred to as the TGax Functional Requirements (FRs).

    - Scope

The scope for deriving functional requirements is set by the P802.11ax PAR [Ref-1] and CSD [Ref-2], as well as by the TGax usage models [Ref-3].

The functional requirements as stated in this document cover the following aspects of 802.11ax:

- System performance

- System Efficiency

- System Deployment

- Backward compatibility and Coexistence

- Compliance to PAR

    - Notation

Requirements are identified by a preceding unique number in the format of “TGax Rn” , where n is an integer number representing the ID of the requirements.

- Functional Requirements

  - System Performace

    - Average throughput per station measured at MAC SAP

- The TGax amendment shall provide at least one mode of operation capable of achieving at least four times improvement in the average throughput per station (measured at the MAC data service access point), compared  to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

    - the 5th percentile of per station throughput

- The TGax amendment shall provide a mechanism to improve the 5th percentile of per station throughput (measured at the MAC data service access point), compared  to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

    - User Experience

- The TGax amendment shall provide a mechanism to improve the performance in delivering a variety of applications such as video, cloud access, broadcast/multicast, and offloading.

    - System Robustness

- The TGax amendment shall provide a mechanism to increase robustness in outdoor propagation environments and uplink transmissions.

  - System Efficiency

    - Spectrum Reuse Efficiency

- The TGax amendment shall provide a mechanism to increase spectral frequency reuse and manage interference between neighboring overlapping BSS (OBSS) in scenarios with a high density of STAs per BSS or a high density of both STAs and BSSs.

    - Power Efficiency

- The TGax amendment shall  maintain or improve the power efficiency per station as measured as average power consumption of device, compared  to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

  - System Deployment

    - Operation Band

- The TGax amendment shall define operations in frequency bands between 1 GHz and 6 GHz.

    - Dense Deployment

The TGax amendment shall support at least one mode of operation that supports dense deployment environments characterized by the existence of a high density of STAs per BSS or a high density of both STAs and BSSs in geographically limited areas.

    - Outdoor Deployment

- The TGax amendment shall support at least one mode of operation that supports outdoor operations with stationary devices or devices at pedestrian speeds.

  - Backward Compatibility and Coexistence

- The TGax amendment shall enable backward compatibility with legacy IEEE 802.11 devices operating in the same band.

- The TGax amendment shall enable coexistence with legacy IEEE 802.11 devices operating in the same band.

  - Compliance to PAR

- The TGax amendment shall comply with the PAR [Ref-1] and the CSD [Ref-2].

- References

- 11-14-0165-01-0hew-802-11-hew-sg-proposed-par

- 11-14-0169-01-0hew-ieee-802-11-hew-sg-proposed-csd

- 11-13-0657-06-0hew-hew-sg-usage-models-and-requirements-liaison-with-wfa

- 11-12-0147-01-00ai-tgai-spec-development-process-update-proposal

- 11-14-0419-00-0hew-802-11ax-spec-development-process-proposal

- 11-14-0306-00-0hew-Discussions-on-HEW-Functional-Requirements