IEEE P802.11
Wireless LANs

| Proposed 802.11ax Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-14 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Lei Wang Hui-Ling Lou Hongyuan Zhang Yakun Sun Liwen Chu | Marvell | 5488 Marvell Lane, Santa Clara, CA, 95054 | +1-858-205-7286 | leileiw@marvell.com |
| Zhou Lan Jiayin Zhang Yingpei Lin | Huawei Technologies Co., Ltd |  |  | lanzhou1@huawei.com zhangjiayin@huawei.com |
| Hyunjeong Kang SangHyun Chang Rakesh Taori | Samsung Electronics |  |  | hyunjeong.kang@samsung.com |
| Yonggang Fang Bo Sun Ke Yao Kaiying Lv | ZTE Corporation |  |  | yfang@ztetx.com sun.bo1@zte.com.cn |
| Robert Stacey | Intel |  |  | robert.stacey@intel.com |
| Ron Porat Vinko Erceg | Broadcom |  |  | rporat@broadcom.com |
| Simone Merlin Bin Tian George Cherian Hemanth Sampath Rolf De Vegt | Qualcomm |  |  | smerlin@qti.qualcomm.com |
| Yasuhiko Inoue Yasushi Takatori Koichi Ishihara Akira Kishida | NTT |  |  | inoue.yasuhiko@lab.ntt.co.jp takatori.yasushi@lab.ntt.co.jp ishihara.koichi@lab.ntt.co.jp kishida.akira@lab.ntt.co.jp |
| Akira Yamada | NTT docomo |  |  | yamadaakira@nttdocomo.com |
| Laurent Cariou Thomas Derham | Orange |  |  | laurent.cariou@orange.com |
| Jinsoo Choi Wookbong Lee Kiseon Ryu HanGyu Cho | LGE |  |  | hg.cho@LGE.COM |
| William Carney Tomoya Yamaura | Sony |  |  | william.carney@am.sony.com yamauratomoya@yahoo.co.jp |

### Abstract

This submission proposes the 802.11ax Functional Requirements as derived from the 802.11ax PAR [Ref-1] and CSD [Ref-2] and also from analyzing the 802.11ax use models [Ref-3].

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the TGax amendment, referred to as the TGax Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is set by the P802.11ax PAR [Ref-1] and CSD [Ref-2], as well as by the TGax usage models [Ref-3].

The functional requirements as stated in this document cover the following aspects of 802.11ax:

- System performance

- System Efficiency

- System Deployment

- Backward compatibility and Coexistence

- Compliance to PAR

  - Notation

Requirements are identified by a preceding unique number in the format of “TGax Rn” , where n is an integer number representing the ID of the requirements.

- Functional Requirements

For the purpose of this document, “deployment scenario” is one of the scenarios defined in the TGax Simulation Scenario document [Ref-9].  Evaluation of performance (e.g., throughput, delay, spectra efficiency, power consumption,  etc..) is conducted according to the TGax Evaluation Methodology Document [Ref-8]. The TGax use cases are described in the TGax Usage Models document [Ref-3].

  - System Performace

    - Average throughput per station measured at MAC SAP

- The TGax amendment shall provide at least one mode of operation capable of achieving at least four times improvement in the average throughput per station (measured at the MAC data service access point), for at least one of the TGax use cases with TGax devices only, without degradation of other performance requirements including transmission latency and 5th percentile of per station throughput, compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same dense deployment scenario.

- The TGax amendment shall provide mode(s) of operation capable of achieving a significant improvement in the average throughput per station (measured at the MAC data service access point), for each of the TGax use cases without degradation of other performance requirements including transmission latency and 5th percentile of per station throughput, compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same dense deployment scenario.

- The TGax amendment shall provide at least one mode of operation capable of achieving a significant improvement in the average throughput per TGax station (measured at the MAC data service access point) in mixture network consisting of both TGax stations and legacy stations, without degradation of legacy IEEE 802.11 stations performance,  compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same dense deployment scenario.

    - The percentile of per station throughput

- The TGax amendment shall define a mechanism or mechanisms to improve the 5th percentile of per station throughput (measured at the MAC data service access point), without degradation of other performance requirements including transmission latency and average throughput per station, compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same dense deployment scenario.

- The TGax amendment shall also ensure the 95th percentile of per station throughput (measured at the MAC data service access point) comparable to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same dense deployment scenario.

    - Latency of transmission

- The mechanisms defined in the TGax amendment shall improve the average transmission latency to satisfy the QoS requirements of various applications, compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

Notes: The transmission latency is measured from the time that MAC receives a packet till the time that PHY starts transmitting. The transmission latency requirements satisfying the QoS of typical applications are defined in TGax Evaluation Methodology Document [Ref-8].

  - System Efficiency

    - Spectrum Efficiency

- The TGax amendment shall provide a mechanism to improve spectral efficiency of 20 MHz and larger channels in scenarios with high density of 11ax STAs with and without the presence of legacy STAs.

Notes: The TGax spectrum efficiency improvement are evaluated in both link level and system level as specified in the TGax Evaluation Methodology document [Ref-8]; in the simulation scenarios as defined in the TGax Simulation Scenario document [Ref-9].

- The TGax amendment shall provide a mechanism to improve frequency reuse efficiency and manage interference between neighboring overlapping BSS (OBSS) in scenarios with a high density of STAs per BSS or a high density of both STAs and BSSs.

- The 802.11ax amendment shall provide a mechanism to improve frequency reuse efficiency and manage interference in the deployment scenarios with high density of 11ax STAs and with presence of transmission between non-AP STAs .

    - Packet Transmission Robustness

- The TGax amendment shall provide a mechanism to increase packet transmission robustness in outdoor propagation environments and uplink transmissions, measured by packet loss ratio (at MAC Service Access Point (SAP)), compared to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

Notes: The packet loss ratio is defined as the number of MAC packet not delivered at all or not delivered in time to the receiver over the total number of offered MAC payloads, as given in TGax Evaluation Methodology Document [Ref-8].

    - Power Efficiency

- The mechanisms defined in the TGax amendment shall  maintain or improve the power efficiency per station as measured as average power consumption of device, compared  to the existing IEEE 802.11 standard and its amendments operating in the same band (IEEE 802.11n in 2.4 GHz and IEEE 802.11ac in 5 GHz) and in the same deployment scenario.

  - System Deployment

    - Operation Band

- The TGax amendment shall define operations in frequency bands between 1 GHz and 6 GHz.

    - Dense Deployment

- The TGax amendment shall support at least one mode of operation that supports dense deployment environments characterized by the existence of a high density of STAs per BSS or a high density of both STAs and BSSs in geographically limited areas.

    - Outdoor Deployment

- The TGax amendment shall support at least one mode of operation that supports outdoor operations with stationary devices or devices at pedestrian speeds.

  - Backward Compatibility and Coexistence

- The backward compatibility means the ability of one “new” system to interwork with another “old” system. In this case the different set of rules implies that the new set of rules is a modification of the old set of rules, i.e. a subset of interworking.

Coexistence means the ability of one system to perform a task in a given shared environment where other systems have an ability to perform their tasks and may or may not be using the same set of rules.

- The TGax amendment shall enable backward compatibility with legacy IEEE 802.11 devices operating in the same band.

- The TGax amendment shall enable coexistence with legacy IEEE 802.11 devices operating in the same band.

  - Compliance to PAR

- The TGax amendment shall comply with the PAR [Ref-1] and the CSD [Ref-2].

- References

- 11-14-0165-01-0hew-802-11-hew-sg-proposed-par

- 11-14-0169-01-0hew-ieee-802-11-hew-sg-proposed-csd

- 11-13-0657-06-0hew-hew-sg-usage-models-and-requirements-liaison-with-wfa

- 11-12-0147-01-00ai-tgai-spec-development-process-update-proposal

- 11-14-0419-00-0hew-802-11ax-spec-development-process-proposal

- 11-14-0306-00-0hew-Discussions-on-HEW-Functional-Requirements

- 11-14-0636-00-00ax-discussion-on-functional-requirement

- 11-14-0571-02-00ax-evaluation-methodology

- 11-14-0621-04-00ax-simulation-scenarios