IEEE P802.11
Wireless LANs

| Proposed 802.11ax Functional Requirements |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-17 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Lei Wang Hui-Ling Lou Hongyuan Zhang Yakun Sun Liwen Chu | Marvell | 5488 Marvell Lane, Santa Clara, CA, 95054 | +1-858-205-7286 | leileiw@marvell.com |
| Zhou Lan Jiayin Zhang Yingpei Lin | Huawei Technologies Co., Ltd |  |  | lanzhou1@huawei.com zhangjiayin@huawei.com |
| Hyunjeong Kang SangHyun Chang Rakesh Taori | Samsung Electronics |  |  | hyunjeong.kang@samsung.com |
| Yonggang Fang Bo Sun Ke Yao Kaiying Lv | ZTE Corporation |  |  | yfang@ztetx.com sun.bo1@zte.com.cn |
| Robert Stacey | Intel |  |  | robert.stacey@intel.com |
| Ron Porat Vinko Erceg | Broadcom |  |  | rporat@broadcom.com |
| Simone Merlin Bin Tian George Cherian Hemanth Sampath Rolf De Vegt | Qualcomm |  |  | smerlin@qti.qualcomm.com |
| Yasuhiko Inoue Yasushi Takatori Koichi Ishihara Akira Kishida | NTT |  |  | inoue.yasuhiko@lab.ntt.co.jp takatori.yasushi@lab.ntt.co.jp ishihara.koichi@lab.ntt.co.jp kishida.akira@lab.ntt.co.jp |
| Akira Yamada | NTT docomo |  |  | yamadaakira@nttdocomo.com |
| Laurent Cariou Thomas Derham | Orange |  |  | laurent.cariou@orange.com |
| Jinsoo Choi Wookbong Lee Kiseon Ryu HanGyu Cho | LGE |  |  | hg.cho@LGE.COM |
| William Carney Tomoya Yamaura Kazuyuki Sakoda | Sony |  |  | william.carney@am.sony.com yamauratomoya@yahoo.co.jp KazuyukiA.Sakoda@jp.sony.com |
| Jarkko Kneckt Sayantan Choudhury | Nokia |  |  | jarkko.kneckt@nokia.com |
| Guido R. Hiertz | Ericsson | Ericsson Allee 1 52134 Herzogenrath Germany |  | guido.hiertz@ericsson.com |
| Chuck Lukaszewski | Aruba Networks | 1344 Crossman Avenue Sunnyvale, CA 95035 |  | chuck@arubanetworks.com |
| Carol Ansley | Arris |  |  | Carol.ansley@arrisi.com |
| Jin Sam Kwak John Sohn | WILUS Institute |  |  | sam@wilusgroup.com |
| Young Doo Kim | SK Telecom |  |  | yd76.kim@sk.com |
| Tomo Adachi David Halls | Toshiba |  |  | tomo.adachi@toshiba.co.jp david.halls@toshiba-trel.com |

### Abstract

This submission proposes the 802.11ax Functional Requirements as derived from the 802.11ax PAR [Ref-1] and CSD [Ref-2] and also from analyzing the 802.11ax use models [Ref-3].

- Introduction

  - Purpose

This document proposes requirements for solutions addressing functionality to be provided by the 802.11ax amendment, referred to as the TGax Functional Requirements (FRs).

  - Scope

The scope for deriving functional requirements is set by the P802.11ax PAR [Ref-1] and CSD [Ref-2], as well as by the TGax usage models [Ref-3].

The functional requirements as stated in this document cover the following aspects of 802.11ax:

- System performance

- System Efficiency

- System Deployment

- Backward compatibility and Coexistence

- Compliance to PAR

  - Notation

Requirements are identified by a preceding unique number in the format of “TGax Rn” , where n is an integer number representing the ID of the requirements.

- Functional Requirements

For the purpose of this document, “dense deployment scenario” is one of the scenarios defined in the TGax Simulation Scenario document [Ref-9].  Evaluation of performance (e.g., throughput, delay, spectra efficiency, power consumption, etc..) is conducted according to the TGax Evaluation Methodology Document [Ref-8]. The TGax use cases are described in the TGax Usage Models document [Ref-3].

All throughput measurements shall occur at the MAC SAP. For the 2.4 GHz band 802.11ax performance improvements shall be compared with operation of 802.11n. For the 5 GHz band 802.11ax performance improvements shall be compared with operation of 802.11ac. The TGax simulation scenarios build the foundation of these comparisons.

  - System Performance

    - Average throughput per station measured at MAC SAP

- The 802.11ax amendment shall provide at least one mode of operation capable of achieving at least four times improvement in the average throughput per station without degradation of other performance requirements including transmission latency and 5th percentile of per station throughput, compared to the existing IEEE 802.11 standard and its amendments, in the same dense deployment scenario.

- The 802.11ax amendment shall provide at least one mode of operation capable of improving the average throughput per station for the 802.11ax STAs, compared to the existing IEEE 802.11 standard and its amendments, while limiting the average throughput degradation of the legacy IEEE 802.11 STAs to no more than TBD% in a mixed network consisting of both 802.11ax STAs and legacy STAs and in the same dense deployment scenario.

    - The percentile of per station throughput

- The 802.11ax amendment shall define a mechanism to improve the 5th percentile of per 802.11ax station throughput, without degradation of other performance requirements including transmission latency and average throughput per station, compared to the existing IEEE 802.11 standard and its amendments operating in the same dense deployment scenario.

- The 802.11ax amendment shall ensure a 95th percentile of per 802.11ax station throughput comparable to the existing IEEE 802.11 standard and its amendments operating in the same dense deployment scenario.

    - Latency of transmission

- The 802.11ax amendment shall define a mechanism to improve the average transmission latency, compared to the existing IEEE 802.11 standard and its amendments operating in the same dense deployment scenario.

Notes: The transmission latency requirements satisfying the QoS of typical applications are defined in TGax Evaluation Methodology Document [Ref-8].

  - System Efficiency

    - Spectrum Efficiency

- The 802.11ax amendment shall provide a mechanism to improve spectral efficiency of 20 MHz and larger frequency channels in scenarios with high density of 802.11ax STAs with and without the presence of legacy STAs.

Notes: The 802.11ax spectrum efficiency improvements are evaluated in both link level and system level as specified in the TGax Evaluation Methodology document [Ref-8]; in the simulation scenarios as defined in the TGax Simulation Scenario document [Ref-9].

- The 802.11ax amendment shall provide a mechanism to facilitate frequency reuse and manage interference between neighboring overlapping BSS (OBSS) in dense deployment scenarios with a high density of STAs per BSS or a high density of both STAs and BSSs.

- The 802.11ax amendment shall provide a mechanism to facilitate frequency reuse and manage interference in the deployment scenarios with high density of 802.11ax STAs and with presence of transmission between non-AP STAs.

    - Packet Transmission Robustness

- The 802.11ax amendment shall increase the probability of successful frame transmissions, compared to the existing IEEE 802.11 standard and its amendments operating in the same dense deployment scenarios.

    - Power Efficiency

- The mechanisms defined in the 802.11ax amendment shall maintain or improve the per STA power efficiency measured as average power consumption, compared to compared to the existing IEEE 802.11 standard and its amendments operating in the same dense deployment scenario.

  - System Deployment

    - Operation Band

- The 802.11ax amendment shall define operations in frequency bands between 1 GHz and 6 GHz.

    - Dense Deployment

- The 802.11ax amendment shall support at least one mode of operation that supports dense deployment environments characterized by the existence of a high density of STAs per BSS or a high density of both STAs and BSSs in geographically limited areas.

    - Outdoor Deployment

- The 802.11ax amendment shall support at least one mode of operation that supports outdoor operations with stationary devices or devices at pedestrian speeds.

  - Backward Compatibility and Coexistence

- The 802.11ax amendment shall enable backward compatibility with legacy IEEE 802.11 devices operating in the same band.

- The 802.11ax amendment shall enable coexistence with legacy IEEE 802.11 devices operating in the same band.

  - Compliance to PAR

- The 802.11ax amendment shall comply with the PAR [Ref-1] and the CSD [Ref-2].

- References

- 11-14-0165-01-0hew-802-11-hew-sg-proposed-par

- 11-14-0169-01-0hew-ieee-802-11-hew-sg-proposed-csd

- 11-13-0657-06-0hew-hew-sg-usage-models-and-requirements-liaison-with-wfa

- 11-12-0147-01-00ai-tgai-spec-development-process-update-proposal

- 11-14-0419-00-0hew-802-11ax-spec-development-process-proposal

- 11-14-0306-00-0hew-Discussions-on-HEW-Functional-Requirements

- 11-14-0636-00-00ax-discussion-on-functional-requirement

- 11-14-0571-02-00ax-evaluation-methodology

- 11-14-0621-04-00ax-simulation-scenarios