IEEE P802.11
Wireless LANs

| LB201-Comment resolution on AP-CSN related CIDs in 6.3.3.2.2. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-05-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Jeongki Kim | LG Electronics | LG Seocho R&D Campus, Yangjae-dong, Seocho-gu, Seoul, Korea |  | jeongki.kim@lge.com |

Abstract

This document provides comment resolutions on CIDs for AP-CSN in 6.3.3.2.2.

CIDs: 4001, 4407, 4408, 4409, 4686, 4699 5050

Comment and discussion on CID 4001, 4407, 4408, 4409, 4686, 5050

| CID | Page(C) | Clause | Comment | Proposed Change | Resolution |
| --- | --- | --- | --- | --- | --- |
| 4001 | 10 | 6.3.3.2.2 | There's nothing that really indicates how the AP Configuration ChangeCount parameter works. The point is that exists per AP, but the scan may enounter multiple APs, so there is a structure clash. | Remove parameter, or replace it by a list of AP address vs change count. Also add some description of how this parameter is used, there is currently no adequte decription. | REVISED. Adopt the Proposed Remedy #1 in contribution 14/0739r0. (https://mentor.ieee.org/802.11/dcn/14/11-14-0739-00-00ai-lb201-cids-for-ap-csn-in-6.3.3.2.2.doc ) MLME-SCAN.request already has BSSID field and if the BSSID is individual MAC address, AP-CNS is optionally present. Change "APConfigurationChangeCount" to "APConfigurationSequenceNumber" in line 26 on page 10. There is a description of APConfigurationSequenceNumber in subclause 8.4.2.178 |
| 4407 | 9 | 6.3.3.2.2 | Why is the name APConfigurationChangeCount different from the corresponding element name APConfigurationSequenceNumber? | change the parameter name to APConfigurationSequenceNumber | ACCEPTED |
| 4408 | 10 | 6.3.3.2.2 | Why is the name APConfigurationChangeCount different from the corresponding element name APConfigurationSequenceNumber? | change the parameter name to APConfigurationSequenceNumber | ACCEPTED |
| 4409 | 10 | 6.3.3.2.2 | APConfigurationChangeCount is used only when dot11FILDActivated is true. | Add the following description to APConfigurationChangeCount: "This parameter is optionally present when dot11FILSActivated is true." | REVISED Adopt the Proposed Remedy #1 in contribution 14/0739r0. (https://mentor.ieee.org/802.11/dcn/14/11-14-0739-00-00ai-lb201-cids-for-ap-csn-in-6.3.3.2.2.doc ) (4409, 5050) |
| 4686 | 9 | 6.3.3.2.2 | This parameter carries a CSN, not a CCC | Change "APConfigurationChangeCount" to "APConfigurationSequenceNumber" | ACCEPTED |
| 5050 | 10 | 6.3.3.2.2 | The APConfigurationChangeCount is only optionally present when dot11FILSActivated is true | Apend at the end of Description of APConfigurationChangeCount the following sentence: "This parameter is optionally present when dot11FILSActivated is true." | REVISED Adopt the Proposed Remedy #1 in contribution 14/0739r0. (https://mentor.ieee.org/802.11/dcn/14/11-14-0739-00-00ai-lb201-cids-for-ap-csn-in-6.3.3.2.2.doc ) (4409, 5050) |

Discussion:

First of all, we need to change "APConfigurationChangeCount" to "APConfigurationSequenceNumber" in in 6.3.3.2.2. MLME-SCAN.request already has BSSID field and if the BSSID is individual MAC address, AP-CNS is optionally present and it is for the BSS. So, we need to add the related description in APConfigurationSequenceNumber. And there is a detailed description of APConfigurationSequenceNumber in 8.4.2.178.

For CID 4409 and 5050 we need to add the description suggested by commenters because the AP-CSN is optional feature in 11ai.

Proposed Remedy#1:

Instructions to TGai Editor: Modify the subclause 6.3.3.2.2 as follows:

6.3.3.2.2 Semantics of the service primitive

Change the primitive parameter list and associated table (only rows with changes are shown):

The primitive parameters are as follows:

MLME-SCAN.request(

BSSType,

BSSID,

SSID,

ScanType,

ProbeDelay,

ChannelList,

MinChannelTime,

MaxChannelTime,

RequestInformation,

SSID List,

ChannelUsage,

AccessNetworkType,

HESSID,

MeshID,

DiscoveryMode,

FILSRequestParameters,

ReportingOption,

APConfigurationChangeCountChangeSequenceNumber

VendorSpecificInfo

)

| Name | Type | Valid range | Description |
| --- | --- | --- | --- |
| FILSRequestParameters | Octet string | As defined in 8.4.2.174 FILS Request Parameters element | Used in determining whether to transmit a Probe Response frame. This parameter is optionally present when dot11FILSActivated is true. |
| ReportingOption | Enumeration | IMMEDIATE, CHANNEL_ SPECIFIC, AT_END | Indicates the result reporting mode. When immediate reporting is requested, every STA that is discovered during the scanning process shall be immediately returned via MLME-SCAN.confirm using INTERMEDIATE_SCAN_R ESULT as the ResultCode. This parameter is optionally present when dot11FILSActivated is true. |
| APConfigurationChangeCountConfigurationSequenceNumber | Octet stringInteger | As defined in 8.4.2.178 AP Configuration Sequence Number element | This parameter is optionally present when dot11FILSActivated is true and BSSID is individual MAC address. Indicates the configuration sequence number for the static information fields and elements as described in the 10.1.4.3.5 AP Configuration Information Set |