IEEE P802.11
Wireless LANs

| Proposed text resolution to CID 144 in CC12 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date:2014-06-12 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Affiliation | Address | Phone | email |
| Bo Gao | Tsinghua University | Luomu Building 10-202, Tsinghua University, Hai-Dian District,Beijing, China | +86-10-62772387-312 | bgao_cug@126.com |
| Changming Zhang |  |  |  |  |
| Wei Feng |  |  |  |  |
| Depeng Jin |  |  |  |  |

Abstract

This document presents suggested proposal towards CID 144 in CC12

Modify the following definition into 10.3.1 as highlighted in red texts:

- STA authentication and association

| CID | Clause | Page | Line | Type | Comment | Suggestion Remedy | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 144 |  |  |  | TR | In 60-GHz systems, the power consumption of idle listening is considerable, due to power-hungery multi-Gbps ADC. The packet detection with low-power lower-sampling-precision ADC is useful method to remedy the power of idle listening. Thus, the preamble settings (e.g., the number of GA128/Gb128) need to consider the performance loss due to the reduction of sampling precision. | Mechanism for lower-sampling-precision packet detection and the corresponding settings of preamble should be illustrated. |  |

Proposed resolution: Accept

As the comments, we modify the preamble of TGaj for the lower-sampling-precision idle listening. Specifically, two Ga128/Gb128 is added in the beginning of preamble, which is shown as follows:

“25.3.6.2 The Short Training field

The Short Training field is composed of 18 16 repetitions of sequences Ga128(n) of length 128 defined in 21.11, a single frequency sequence (SFS) of length 256 that used for IQ imbalance estimation, followed by a single sequence –Ga128(n).

The waveform for the Short Training field is:

- where mod is the modulus operation.”

“25.4.3.1.2 Short Training field

The Short Training field is composed of 50 48 repetitions of sequences Gb128(n) of length 128, followed by a single -Gb128(n) sequence (for synchronization) and then a single -Ga128(n) sequence. The sequences Ga128(n) and Gb128(n) are defined in 21.11.

The waveform for the Short Training field is:

Where mod is the modulus operation. Note that sequences Ga128(n) and Gb128(n) are defined for 0≤n≤127. For other n they are set to zero.”

Insert the following text, Annex AC, after Annex AB:

“Annex AC

(informative)

Low-power 1-bit-sampling idle listening

Since the power consumption of an ADC is exponential to its bit depth, a low power idle listening mechanism using 1-bit ADC is summarized as follows:

A 1/M-bit-sampling receiver is proposed in Figure AC-1. The original multi-bit sampling-precision ADC is termed M-bit ADC. The receiver includes a receive radio-frequency (RF) chain, a 1-bit ADC, an M-bit ADC, and a digital baseband. The packet detection algorithm constitutes a small component of the digital baseband. Within the receiving period, the receive RF chain is always active. During the idle listening period, the receiver adopts the 1-bit ADC, and the corresponding packet detector is enabled to detect the arrival of burst packets. The M-bit ADC and the other components except the packet detector in the digital baseband are all disabled to save power. Upon detecting a packet, the M-bit ADC is enabled, and the receiver switches to the M-bit ADC. The 1-bit ADC and packet detector are disabled to save power. The other components of the digital baseband are enabled to demodulate the packet.

Figure AC-1. Diagram of 1/M-bit-sampling receiver.

The receiver opration is illustrated in Figure AC-2. Once the arrival of a packet is successfully detectedby using 1-bit ADC, the receiver switches to the original M-bit ADC to demodulate information data normally.  By using this receiver, the energy dissipation of IL is efficiently reduced, while the demodulation performance is maintained.

Figure AC-2. The 1-bit-sampling idle listening and receiving operation”