IEEE P802.11
Wireless LANs

| 802.11 IEEE P802.11REVmc D3.0 Mandatory Draft Review (MDR) Report |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: 2014-07-30 |  |  |  |  |
| Author(s): |  |  |  |  |
| Name | Company | Address | Phone | email |
| Adrian Stephens | Intel Corporation |  |  | Adrian.p.stephens@intel.com |
| Peter Ecclesine | Cisco Systems |  |  | pecclesi@cisco.com |

Abstract

This document contains the report of the 802.11REVmc Mandatory Draft Review.

R1:  added issues / other actions arising and Edward’s contribution.

R2: As discussed in TGmc. 2014-07-

R2: As discussed in TGmc 2014-07-15

R3: As discussed in TGmc 2014-07-15

R4: Added MEC findings from IEEE-SA

R5: Minor tweaks from Ed Reuss.   Plus status from editing of changes. (e.g. “- done”)

# Introduction

## Purpose of this document

This document is the report from the group of volunteers that participated in the P802.11REVmc Draft 3 mandatory draft review.

This document contains recommendations for changes to REVmc to bring it into improved compliance to IEEE-SA and WG11 style.

Those recommended changes need to be reviewed by TGmc and approved, or ownership of the issues taken by TGmc.

## Process / references

The MDR process is described in:

- 11-11/615r5 – Mandatory Draft Review process

And references:

- 11-09/1034r9 – 802.11 Editorial Style Guide

A setup meeting was held, and review topics identified and assigned to volunteers.  The volunteers provided their review comments, which have been compiled into this document, with some editorial changes.

## Acknowledgements

The 802.11 technical editors (Adrian Stephens and Peter Ecclesine) gratefully acknowledge the work and contribution of:

- Yongho Seok

- Dan Gal

- Peter Ecclesine

- Emily Qi

- Jiamin Chen

- Donald Eastlake

- Alfred Asterjadhi

- Edward Au

## Issues needing discussion in TGmc

See: 2.1.2.2 (true and false), 2.1.3 (is set to), 2.1.6 (use of problematic words,  including changes to normative verbs), 2.1.12 (sap primitive references)

## Other Actions arising

ANA has actions in 2.2.5.

WG technical editor to review outcome of discussions in TGmc and reflect them in an updated WG11 style guide.

# Findings

## Style

### Style Gude 2.1.1 Frame Format Figures + Review of Table format

This document contains some proposed resolutions to MDR comments.

| Page | Line | Clause/table/figure | Comment |
| --- | --- | --- | --- |
| 570 | 6 | Table 8-14 | The horizontal line at line 6 should be bold. – ruling reset to “from table” |
| 598 | 18 | Figure 8-36 | The vertical lines in the figure should be bold. – ruling set to thin |
| 599 | 47 | Figure 8-39, 8-40, 8-41, 8-42, 8-43, 8-44, 8-45, 8-46, 8-47 | The vertical lines in the figure should be bold. - done |
| 592 | 57 | Figure 27 | The vertical lines and horizatal lines should be bold. - done |
| 614 | 5 | Table 8-35 | The horizontal line at line 5 should be bold. - ruling reset |
| 617 | 41 | Table 8-36 | The horizontal line at line 42 should be bold. – heading moved to table heading row |
| 619 | 5 | Table 8-37 | The horizontal line at line 5 should be bold. - ruling reset |
| 627 | 5 | Table 8-42 | The horizontal line at line 5 should be bold. - ruling reset |
| 631, 632 | 3 | Table 8-43 | The table on page 631 and 632 should include the first line of the table on page 630 – heading moved to table heading row. |
| 633 | 7 | Table 8-44 | The horizontal line at lune 7 should be bold. - ruling reset |
| 634 | 3 | Table 8-46 | The table on page 634 should include the first line of the table on page 633 – heading moved to table heading row. |
| 644 | 51 | Table 8-52 | The line at line 51 should not be bold. – ruling reset |
| 645 | 50 | Table 8-52 | The line at line 50 should not be bold. - ditto |
| 661 | 29 | Table 8-58 | The line at line 29 should not be bold. – ditto |
| 622 | 29 | Table 8-59 | The line at line 29 should not be bold. – ditto |
| 666 | 49 | Table 8-64 | The horizontal line at line 49 should not be bold. – moved heading to table heading row, reset ruling |
| 667 | 45 | Table 8-66 | The line at line 46 should not be bold. – ruling reset |
| 673, 674 | 55 | Table 8-69 | The line at line 55 on both pages should not be bold. – ruling reset |
| 680 | 5 | Table 8-72 | The horizontal line should be bold. – ruling reset |
| 683 | 6 | Table 8-73 | The horizontal line should be bold. – ruling reset |
| 686, 687, 688, 689 | 7 | Table 8-76 | The horizontal lines should be bold. – ruling reset |
| 694, 695 | 6 | Table 8-79 | The horizontal lines should be bold – ruling reset |
| 710 | 49 | Figure 8-120, Figure 8-121, 8-122, 8-123, 8-124, | The vertical lines in the figures should be bold. – ruling set to thin |
| 717 | 6 | Figure 8-131 | Is this figure aligned with figure format rules? No change indicated, none made. |
| 718 | 47 | Figure 8-132 | The vertical line in the figures should be bold. – done |
| 719 | 17 | Figure 8-133, 8-134, 135, 136, 137, 138, 139, 140, 141, 142 | The vertical lines in the figures should be bold. - done |
| 726 | 7 | Table 8-89 | The hotizonal line should be bold. – ruling reset |
| 737 | 42 | Table 8-99 | The hotizonal line should be bold. – heading moved to table header row |
|  |  | Figure 8-155, 156, 157, 158, 160, 161, 162, 164, 165, 166, 168, 169, 170, 171, 172, 174 to 184, 187 to 190, 193 to 197, 201 to 208, 220 to 224, 226 to 237, 241 to 243, 247, 248, 250, 253, 255 to 257, 264 to 273, 284 to 293, 295 to 298, 301, 302, 304, 305, 314, 327, 329, 340, 341, 343, 344, 400, 434 to 436, 496 to 551, 611 to 618, 714, 716 | The vertical lines in the figures should be bold. - done |
| 764 | 41 | Table 8-117, Table 8-119 | The hotizonal line should be bold. – heading moved to table header row |
| 772 | 29 | Table 8-122 | The line at line 29 should not be bold. – ruling reset |
| 773 | 7, 24 | Table 8-122 | The line at line 7 should be bold. The line at line 24 should not be bold. – ruling reset |
| 775, 776 | 7 | Table 8-122 | The line at line 7 should be bold. – ruling reset |
| 795 | 13 | Table 8-130 | The line should be bold. – ruling reset |
| 799 | 5 | Table 8-131 | The line should be bold. – ruling reset |
| 819, 820 | 5 | Table 8-142 | The line should be bold. – ruling reset |
| 821 | 36 | Table 8-142 | The line should not be bold. – ruling reset |
| 834 | 21 | Table 8-151 | The frame lines of the table should be bold. – ruling reset |
| 871 | 9, 64 | Table 8-167 | The line at line 9 is too thick. The line at line 64 is too thin. ditto |
| 873 | 40 | Figure 8-328 | All lines in the figure should be bold. - nothing done. Ruling appears to be correct in source. |
| 875 | 10 | Table 8-170, | The second line in the table is too bold. - ruling reset |
| 877 | 9 | Table 8-171 | The second line in the table is too bold. The bottom line is too thin. – ruling reset, although I didn’t see anything wrong in the source. |
| 881 | 6 | Table 8-173 | The second line and third line in the table are too bold. – ruling reset |
| 882, 883 | 6 | Table 8-173 | The second line is too bold. - ditto |
| 916 | 43 | Table 8-192 | The line at ine 43 should not be bold. – ruling reset |
| 918 | 45 | Table 8-196 | The line at ine 45 should not be bold. – ruling reset |
| 931 | 65 | Table 8-203 | Need a bottom line. – seems to be a frame bug. No change made. |
| 932 | 5 | Table 8-203 | The line at line 5 should be bold. – ruling reset. |
| 948 | 24 | Table 8-216 | The line should be bold.- ruling reset |
| 953 | 20 | Table 8-219 | The line should NOT be bold. – ruling reset |
| 960 | 13 | Table 8-223 | The line should be bold. – ruling reset |
| 1017 | 17 | Table 8-246 | Should this table be formatted as a figure? – no change proposed. Yes, it might be better as a figure. |
| 1029 | 62 | Table 8-250 | The bottom line should be bold. – ruling reset |
| 1030, 1031 | 5 | Table 8-250 | The line should be bold. – ruling reset |
| 1038 | 10 | Figure 8-562 | Since these are optional fields. Should “variable” shuld be replaced by “0 or n”? – no change proposed. None made. I think “variable” includes “0 or n”. |
| 1043 | 11 | Table 8-245 | The line should be bold. – header moved to table header row and ruling reset |
| 1052 | 41 | Figure 8-584, Figure 8-585, 8-586, 8-591, 8-592, 8-594, 8-596, 8-600, 8-605 | Since these are optional fields. Should “variable” shuld be replaced by “0 or n”? – no change proposed. None made. I think “variable” includes “0 or n”. |
| 1055 | 6 | Figure 8-590 | Add spaces between B0 and B1. Add spaces between B2 and B7. - done |
| 1056 | 37 | Figure 8-593 | Add spaces between B1 and B7. – done |
| 1079 | 3 | Table 8-277 | The first row should be repeated on this page. – done |
| 1099 | 61 | Table 8-296 | The line should NOT be bold. – ruling reset |
| 1112 | 31 | Table 8-304 | The line should NOT be bold. – ruling reset |
| 1126 | 22 | Figure 8-664 | All lines should be bold. - done |
| 1130 | 1 | Figure 8-669 | The figure is split to two pages. Move it to one page. - done |
| 1131, 1132 | 3 | Table 8-316 | The first row should be repeated on continued pages. – moved to table header row and ruling reset |
| 1132 | 62 | Table 8-317 | The line should NOT be bold. – ruling reset |
| 1133 | 5 | Table 8-317 | The line is too thick. – ruling reset |
| 1137 | 43 | Table 8-326 | The line should be bold. – moved to table header row and ruling reset |
| 1141 | 44 | Table 8-329 | The line should be bold– moved to table header row and ruling reset |
| 1142 | 3 | Table 8-329 | The first row should be repeated on continued pages. – moved to table header row and ruling reset |
| 1148 | 4 | Table 8-338 | The line should be bold – ruling reset. |

Proposed change:  Editor to visit the locations and adjust style for consistency as indicated.

### Style Guide 2.2 – True and False - done

#### Changes proposed

Change True to true:

[001] Page  169, line 13.

[002] Page  176, line 35.

[003] Page  183, line 24.

[004] Page  190, line 22.

[005] Page  477, line 10.

[006] Page  528, line  6.

[007] Page  530, line 13.

[008] Page 2270, line 27.

[009] Page 2275, line 26.

[010] Page 2358, line 24.

[011] Page 2376, line 22.

[012] Page 2442, line 45.

[013] Page 2891, line  2.

[014] Page 2891, line 16.

[015] Page 2891, line 31.

Change "true" to true:

[001] Page 1853, line 51.

[002] Page 1854, line 54.

[003] Page 1995, line 37.

[004] Page 1995, line 46.

[005] Page 3323, line 28.

[006] Page 3325, line 23.

Change False to false:

[001] Page  169, line 13.

[002] Page  183, line 24.

[003] Page  528, line  6.

[004] Page  530, line 13.

[005] Page 2154, line 22.

[006] Page 2209, line  6.

[007] Page 2256, line 60.

[008] Page 2270, line 28.

[009] Page 2275, line 28.

[010] Page 2356, line 45.

[011] Page 2356, line 47.

[012] Page 2356, line 49.

[013] Page 2356, line 51.

[014] Page 2356, line 53.

[015] Page 2356, line 55.

[016] Page 2356, line 56.

[017] Page 2358, line 40.

[018] Page 2358, line 42.

[019] Page 2358, line 51.

[020] Page 2358, line 53.

[021] Page 2358, line 55.

[022] Page 2358, line 56.

[023] Page 2358, line 58.

[024] Page 2358, line 60.

[025] Page 2358, line 62.

[026] Page 2358, line 64.

[027] Page 2359, line  7.

[028] Page 2359, line  9.

[029] Page 2359, line 11.

[030] Page 2359, line 13.

[031] Page 2359, line 15.

[032] Page 2359, line 16.

[033] Page 2359, line 38.

[034] Page 2359, line 40.

[035] Page 2359, line 42.

[036] Page 2359, line 44.

[037] Page 2359, line 46.

[038] Page 2359, line 49.

[039] Page 2359, line 51.

[040] Page 2360, line 12.

[041] Page 2360, line 13.

[042] Page 2360, line 15.

[043] Page 2360, line 17.

[044] Page 2376, line 26.

[045] Page 2442, line 50.

[046] Page 2542, line 59.

[047] Page 2542, line 61.

[048] Page 2543, line 10.

[049] Page 2543, line 12.

[050] Page 2543, line 14.

[051] Page 2543, line 15.

[052] Page 2543, line 17.

[053] Page 2543, line 19.

[054] Page 2543, line 21.

[055] Page 2543, line 22.

[056] Page 2543, line 24.

[057] Page 2543, line 26.

[058] Page 2543, line 28.

[059] Page 2543, line 30.

[060] Page 2543, line 32.

[061] Page 2543, line 33.

[062] Page 2543, line 48.

[063] Page 2543, line 50.

[064] Page 2543, line 52.

[065] Page 2543, line 53.

[066] Page 2543, line 55.

[067] Page 2543, line 57.

[068] Page 2543, line 59.

[069] Page 2543, line 61.

[070] Page 2543, line 63.

[071] Page 2543, line 64.

[072] Page 2544, line 15.

[073] Page 2544, line 16.

[074] Page 2544, line 18.

[075] Page 2544, line 20.

[076] Page 2544, line 22.

[077] Page 2544, line 26.

[078] Page 2544, line 29.

[079] Page 2544, line 50.

[080] Page 2544, line 52.

[081] Page 2544, line 54.

[082] Page 2544, line 56.

[083] Page 2833, line 44.

[084] Page 2833, line 61.

[085] Page 2834, line 13.

[086] Page 2834, line 30.

[087] Page 2834, line 48.

[088] Page 2834, line 65.

[089] Page 2835, line 17.

[090] Page 2835, line 35.

[091] Page 2835, line 53.

[092] Page 2836, line  5.

[093] Page 2836, line 21.

[094] Page 2836, line 39.

[095] Page 2836, line 56.

[096] Page 2837, line  7.

[097] Page 2837, line 24.

[098] Page 2837, line 43.

[099] Page 2837, line 57.

[100] Page 2838, line 47.

[101] Page 2839, line 16.

[102] Page 2839, line 33.

[103] Page 2839, line 49.

[104] Page 2840, line  2.

[105] Page 2840, line 19.

[106] Page 2840, line 36.

[107] Page 2847, line  6.

[108] Page 2847, line 22.

[109] Page 2851, line 48.

[110] Page 2890, line 65.

[111] Page 2891, line 15.

[112] Page 2891, line 30.

[113] Page 2905, line  3.

[114] Page 2908, line 62.

[115] Page 2909, line 55.

Change "false" to false:

[001] Page 1853, line 29.

[002] Page 1994, line 49.

[003] Page 1994, line 55.

[004] Page 1995, line 49.

#### Changes for discussion

The discussion point here is whether the 802.1X state machine terminology is fixed by 802.1X,  or whether the usages listed below are a local matter.

Discussion in TGmc:

- State machines in 802.1X are somewhat variable, but all state machines in diagrams are uppercase.

- Many of these variables are locally defined in 802.11,  and can be changed.

- There is no objection to changing them all to lower case.

Until that discussion has taken place,  no change is proposed.

Change TRUE to true:

[001] Page 1921, line 32.

[002] Page 1984, line 14.

[003] Page 1984, line 23.

[004] Page 1984, line 38.

[005] Page 1985, line 49.

[006] Page 1985, line 51.

[007] Page 1985, line 55.

[008] Page 1985, line 59.

[009] Page 1985, line 62.

[010] Page 1986, line  5.

[011] Page 1992, line  1.

[012] Page 1992, line  2.

[013] Page 1992, line  5.

[014] Page 1992, line  7.

[015] Page 1992, line 10.

[016] Page 1992, line 12.

[017] Page 1992, line 38.

[018] Page 1992, line 48.

[019] Page 1992, line 51.

[020] Page 1992, line 56.

[021] Page 1992, line 59.

[022] Page 2036, line 57.

[023] Page 2037, line 55.

[024] Page 2037, line 57.

[025] Page 2037, line 59.

[026] Page 2041, line 36.

[027] Page 2041, line 38.

[028] Page 2041, line 42.

[029] Page 2041, line 46.

[030] Page 2041, line 48.

[031] Page 2041, line 51.

[032] Page 2041, line 56.

[033] Page 2041, line 59.

[034] Page 2042, line  1.

[035] Page 2043, line 27.

[036] Page 2047, line  6.

[037] Page 2047, line  8.

[038] Page 2047, line 10.

[039] Page 2047, line 15.

[040] Page 2047, line 20.

[041] Page 2047, line 21.

[042] Page 2047, line 23.

[043] Page 2047, line 24.

[044] Page 2047, line 25.

[045] Page 2047, line 26.

[046] Page 2047, line 29.

[047] Page 2047, line 32.

[048] Page 2047, line 38.

### Style Guide 2.3 – “is set to”

The WG11 style guide states:

“The verb “set” should only be used when describing how a field obtains a value, e.g. “The Measurement Duration field is set to the preferred or mandatory duration of the requested measurement, expressed in units of TUs.”

Where the value of the field is read or referenced, (e.g., in the context of a condition), “is set to” shall not be used.”

Edward reviewed usage of “is set” and produced a long list of proposed changes.

Adrian reviewed some of this list and did not agree with the proposed changes.

Discussion point: We ask TGmc to review a random selection of this list and determine what terminology it prefers,  we will then edit the WG11 style guide to reflect this.

According to that outcome, the a set of changes will be proposed.  No change is proposed until that discussion has taken place.

TGmc reviewed this.   Edward will review the list and highlight items that should be considered.

Consider changng “set to” to “equal to”:

[001] Page  146, line 56. - undecided

[002] Page  426, line 56. – ok as is

[003] Page  426, line 57.

[004] Page  549, line 46.

[005] Page  554, line 22.

[006] Page  554, line 33.

[007] Page  554, line 36.

[008] Page  555, line 42.

[009] Page  555, line 59.

[010] Page  555, line 61.

[011] Page  556, line  7.

[012] Page  556, line 10.

[013] Page  556, line 11.

[014] Page  556, line 22.

[015] Page  556, line 25.

[016] Page  556, line 26.

[017] Page  556, line 29.

[018] Page  556, line 40.

[019] Page  556, line 44.

[020] Page  556, line 49.

[021] Page  556, line 51.

[022] Page  556, line 63.

[023] Page  558, line 36.

[024] Page  562, line 42.

[025] Page  562, line 52.

[026] Page  563, line 39.

[027] Page  563, line 58.

[028] Page  565, line  8.

[029] Page  565, line  9.

[030] Page  565, line 12.

[031] Page  565, line 53.

[032] Page  565, line 55.

[033] Page  565, line 63.

[034] Page  565, line 65.

[035] Page  566, line  2.

[036] Page  566, line 52.

[037] Page  566, line 54.

[038] Page  570, line 17.

[039] Page  570, line 64. (two occurrences)

[040] Page  572, line 16.

[041] Page  572, line 31.

[042] Page  580, line 33.

[043] Page  590, line 12.

[044] Page  590, line 49.

[045] Page  592, line 50.

[046] Page  593, line 12.

[047] Page  593, line 25.  – don’t change

[048] Page  593, line 43.

[049] Page  593, line 62.

[050] Page  594, line 18.

[051] Page  596, line  6.

[052] Page  596, line 65. – don’t change

[053] Page  597, line 36.

[054] Page  598, line  7.

[055] Page  598, line 43.

[056] Page  600, line 21.

[057] Page  602, line 29.

[058] Page  605, line 46.

[059] Page  606, line 10.

[060] Page  606, line 12.

[061] Page  606, line 13.

[062] Page  606, line 16.

[063] Page  606, line 20.

[064] Page  608, line 12.

[065] Page  612, line  5.

[066] Page  612, line 34. (two occurrences)

[067] Page  612, line 39. (two occurrences)

[068] Page  636, line 50.

[069] Page  636, line 53.

[070] Page  636, line 54.

[071] Page  636, line 62.

[072] Page  637, line 11.

[073] Page  637, line 12.

[074] Page  637, line 13.

[075] Page  637, line 20. (two occurrences)

[076] Page  637, line 43.

[077] Page  637, line 44.

[078] Page  638, line 10.

[079] Page  638, line 11.

[080] Page  638, line 31.

[081] Page  640, line 64.

[082] Page  641, line 31.

[083] Page  642, line 24.

[084] Page  653, line 62.

[085] Page  653, line 63.

[086] Page  654, line 10.

[087] Page  654, line 27.

[088] Page  655, line 10.

[089] Page  655, line 11.

[090] Page  656, line  2.

[091] Page  659, line 19.

[092] Page  660, line 18.

[093] Page  660, line 40.

[094] Page  671, line 32. (two occurrences)

[095] Page  671, line 34.

[096] Page  672, line 10.

[097] Page  672, line 11.

[098] Page  679, line  6.

[099] Page  679, line  7.

[100] Page  679, line 12.

[101] Page  679, line 13.

[102] Page  679, line 15.

[103] Page  679, line 17.

[104] Page  679, line 21. (two occurrences)

[105] Page  680, line 57.

[106] Page  680, line 58.

[107] Page  697, line 63.

[108] Page  697, line 64.

[109] Page  700, line 14.

[110] Page  700, line 18.

[111] Page  723, line 26.

[112] Page  744, line 32.

[113] Page  744, line 47.

[114] Page  752, line 40.

[115] Page  752, line 45.

[116] Page  753, line 41.

[117] Page  753, line 47.

[118] Page  754, line 21.

[119] Page  754, line 22.

[120] Page  756, line 13.

[121] Page  756, line 14. (two occurrences)

[122] Page  756, line 57.

[123] Page  756, line 58. (two occurrences)

[124] Page  756, line 63.

[125] Page  756, line 64.

[126] Page  757, line  1.

[127] Page  757, line  3.

[128] Page  759, line 15.

[129] Page  782, line  7.

[130] Page  782, line  9.

[131] Page  782, line 10.

[132] Page  782, line 12.

[133] Page  782, line 13.

[134] Page  782, line 15.

[135] Page  782, line 36.

[136] Page  782, line 40.

[137] Page  783, line 44.

[138] Page  783, line 50.

[139] Page  784, line 56.

[140] Page  788, line 24.

[141] Page  790, line 57.

[142] Page  790, line 60.

[143] Page  791, line 27.

[144] Page  794, line 18.

[145] Page  794, line 25.

[146] Page  799, line 55.

[147] Page  801, line 23.

[148] Page  801, line 30.

[149] Page  802, line  2.

[150] Page  802, line  3.

[151] Page  803, line 27.

[152] Page  803, line 28.

[153] Page  803, line 29.

[154] Page  804, line 50.

[155] Page  804, line 51.

[156] Page  804, line 52.

[157] Page  805, line  2.

[158] Page  815, line 46.

[159] Page  816, line 30.

[160] Page  816, line 39.

[161] Page  816, line 44.

[162] Page  816, line 47.

[163] Page  816, line 48.

[164] Page  818, line 14.

[165] Page  818, line 17.

[166] Page  818, line 31.

[167] Page  820, line 26.

[168] Page  820, line 27.

[169] Page  820, line 32.

[170] Page  820, line 33.

[171] Page  820, line 39. (two occurrences)

[172] Page  820, line 42.

[173] Page  820, line 45.

[174] Page  820, line 48.

[175] Page  820, line 51.

[176] Page  820, line 53.

[177] Page  820, line 56.

[178] Page  820, line 58.

[179] Page  820, line 61.

[180] Page  821, line  6.

[181] Page  821, line  9.

[182] Page  821, line 14. (two occurrences)

[183] Page  821, line 18.

[184] Page  821, line 22.

[185] Page  821, line 38.

[186] Page  822, line 47.

[187] Page  822, line 52.

[188] Page  822, line 55.

[189] Page  824, line 62.

[190] Page  825, line 31.

[191] Page  827, line 45.

[192] Page  828, line 58. (two occurrences)

[193] Page  828, line 59.

[194] Page  828, line 62.

[195] Page  828, line 63.

[196] Page  830, line 42.

[197] Page  833, line 20.

[198] Page  833, line 21.

[199] Page  838, line  7.

[200] Page  842, line 13.

[201] Page  843, line 23.

[202] Page  843, line 24.

[203] Page  843, line 53. (two occurrences)

[204] Page  846, line 11.

[205] Page  846, line 12.

[206] Page  849, line 57.

[207] Page  866, line 58.

[208] Page  866, line 63.

[209] Page  867, line  1.

[210] Page  867, line  8.

[211] Page  868, line 59.

[212] Page  884, line 19.

[213] Page  886, line  5.

[214] Page  886, line 12.

[215] Page  886, line 13.

[216] Page  886, line 17.

[217] Page  886, line 18.

[218] Page  886, line 23.

[219] Page  886, line 24.

[220] Page  890, line 46.

[221] Page  890, line 47.

[222] Page  890, line 50.

[223] Page  890, line 51.

[224] Page  890, line 53.

[225] Page  890, line 54.

[226] Page  890, line 57.

[227] Page  890, line 58.

[228] Page  901, line 39.

[229] Page  902, line 29.

[230] Page  929, line 28.

[231] Page  934, line 61.

[232] Page  934, line 62.

[233] Page  942, line 25.

[234] Page  945, line  1.

[235] Page  945, line  6.

[236] Page  948, line 38.

[237] Page  948, line 49.

[238] Page  953, line 43.

[239] Page  953, line 44.

[240] Page  957, line 11.

[241] Page  962, line 16.

[242] Page  962, line 17.

[243] Page  962, line 23.

[244] Page  962, line 49.

[245] Page  962, line 50.

[246] Page  962, line 54.

[247] Page  962, line 56. (two occurrences)

[248] Page  962, line 60. (two occurrences)

[249] Page  962, line 64. (two occurrences)

[250] Page  963, line  1. (two occurrences)

[251] Page  963, line  6. (two occurrences)

[252] Page  963, line  9.

[253] Page  963, line 11.

[254] Page  963, line 12.

[255] Page  966, line 32.

[256] Page  966, line 35.

[257] Page  966, line 37.

[258] Page  966, line 38.

[259] Page  966, line 42.

[260] Page  966, line 43.

[261] Page  966, line 49.

[262] Page  968, line  1.

[263] Page  968, line  2.

[264] Page  971, line 15.

[265] Page  971, line 16.

[266] Page  972, line 48.

[267] Page  972, line 49.

[268] Page  972, line 52.

[269] Page  972, line 53.

[270] Page  972, line 56.

[271] Page  972, line 57.

[272] Page  976, line 61.

[273] Page  977, line 45.

[274] Page  978, line 55.

[275] Page  978, line 57.

[276] Page  985, line 16.

[277] Page  987, line 38.

[278] Page  987, line 51.

[279] Page  988, line 60. (two occurrences)

[280] Page  989, line  3. (two occurrences)

[281] Page  989, line  9.

[282] Page  989, line 11.

[283] Page  989, line 15.

[284] Page  989, line 16.

[285] Page  991, line 11.

[286] Page  991, line 12. (two occurrences)

[287] Page  991, line 13.

[288] Page  991, line 63.

[289] Page  991, line 64.

[290] Page  992, line 31.

[291] Page  992, line 32.

[292] Page  992, line 34. (two occurrences)

[293] Page  992, line 38.

[294] Page  992, line 39.

[295] Page  992, line 46.

[296] Page  992, line 47.

[297] Page  992, line 62.

[298] Page  992, line 63.

[299] Page  993, line 38.

[300] Page  993, line 39.

[301] Page  994, line  2.

[302] Page  994, line 13.

[303] Page  994, line 18.

[304] Page  994, line 19.

[305] Page  994, line 24.

[306] Page  994, line 25.

[307] Page  994, line 30.

[308] Page  994, line 31.

[309] Page  994, line 34.

[310] Page  994, line 35.

[311] Page  994, line 37.

[312] Page  994, line 41.

[313] Page  994, line 43.

[314] Page  994, line 47.

[315] Page  994, line 49.

[316] Page  994, line 50.

[317] Page  995, line  4.

[318] Page  995, line  5.

[319] Page  995, line  7.

[320] Page  995, line  8.

[321] Page  995, line 37.

[322] Page  995, line 39.

[323] Page  995, line 41.

[324] Page  995, line 43.

[325] Page  995, line 44.

[326] Page  995, line 45.

[327] Page  995, line 46.

[328] Page  995, line 49.

[329] Page  995, line 50.

[330] Page  995, line 57. (two occurrences)

[331] Page  995, line 60.

[332] Page  995, line 61.

[333] Page  996, line  1.

[334] Page  996, line  2.

[335] Page  996, line  6.

[336] Page  996, line  7.

[337] Page  996, line 40.

[338] Page  996, line 41.

[339] Page  996, line 44.

[340] Page  996, line 45. (two occurrences)

[341] Page  996, line 50.

[342] Page  996, line 51.

[343] Page  998, line 13.

[344] Page  998, line 17.

[345] Page  998, line 21.

[346] Page  998, line 24.

[347] Page  998, line 32.

[348] Page 1000, line  2.

[349] Page 1000, line  6.

[350] Page 1001, line 44.

[351] Page 1001, line 45.

[352] Page 1001, line 47.

[353] Page 1001, line 48.

[354] Page 1001, line 52.

[355] Page 1001, line 53.

[356] Page 1001, line 57.

[357] Page 1001, line 58. (two occurrences)

[358] Page 1001, line 59.

[359] Page 1002, line 14.

[360] Page 1002, line 26.

[361] Page 1002, line 29.

[362] Page 1002, line 30.

[363] Page 1002, line 60.

[364] Page 1002, line 61.

[365] Page 1003, line 11.

[366] Page 1003, line 12.

[367] Page 1004, line 23. (two occurrences)

[368] Page 1004, line 26.

[369] Page 1004, line 28.

[370] Page 1004, line 31.

[371] Page 1004, line 33.

[372] Page 1008, line 58. (two occurrences)

[373] Page 1008, line 59.

[374] Page 1008, line 61. (two occurrences)

[375] Page 1010, line 20.

[376] Page 1010, line 21.

[377] Page 1010, line 24.

[378] Page 1010, line 27.

[379] Page 1010, line 36.

[380] Page 1010, line 40.

[381] Page 1010, line 49.

[382] Page 1011, line 17. (two occurrences)

[383] Page 1011, line 20. (two occurrences)

[384] Page 1011, line 23.

[385] Page 1011, line 24.

[386] Page 1011, line 26.

[387] Page 1011, line 27.

[388] Page 1011, line 30.

[389] Page 1011, line 31.

[390] Page 1011, line 41.

[391] Page 1011, line 44.

[392] Page 1012, line 37.

[393] Page 1015, line  6.

[394] Page 1015, line  8.

[395] Page 1015, line 36.

[396] Page 1015, line 37.

[397] Page 1015, line 44.

[398] Page 1015, line 46.

[399] Page 1015, line 47.

[400] Page 1015, line 51.

[401] Page 1016, line 52.

[402] Page 1016, line 55.

[403] Page 1017, line  1.

[404] Page 1017, line  3.

[405] Page 1017, line  6.

[406] Page 1017, line  7.

[407] Page 1018, line 36.

[408] Page 1018, line 37.

[409] Page 1018, line 39.

[410] Page 1018, line 41.

[411] Page 1018, line 42.

[412] Page 1018, line 47.

[413] Page 1018, line 48.

[414] Page 1018, line 51.

[415] Page 1018, line 52.

[416] Page 1018, line 57.

[417] Page 1018, line 62.

[418] Page 1018, line 63.

[419] Page 1019, line 21.

[420] Page 1020, line  3. (two occurrences)

[421] Page 1020, line  7.

[422] Page 1020, line  8.

[423] Page 1020, line 12.

[424] Page 1020, line 13.

[425] Page 1020, line 15.

[426] Page 1020, line 16.

[427] Page 1020, line 19.

[428] Page 1020, line 20.

[429] Page 1020, line 24.

[430] Page 1020, line 26. (two occurrences)

[431] Page 1020, line 29.

[432] Page 1020, line 30.

[433] Page 1020, line 31.

[434] Page 1021, line  2. (two occurrences)

[435] Page 1021, line  8.

[436] Page 1021, line  9.

[437] Page 1021, line 14.

[438] Page 1021, line 15.

[439] Page 1024, line 37.

[440] Page 1024, line 42.

[441] Page 1024, line 49.

[442] Page 1024, line 50.

[443] Page 1025, line 18.

[444] Page 1025, line 19.

[445] Page 1026, line 46.

[446] Page 1027, line 34.

[447] Page 1038, line 55.

[448] Page 1038, line 56.

[449] Page 1039, line 55.

[450] Page 1039, line 56.

[451] Page 1042, line 37.

[452] Page 1042, line 40.

[453] Page 1042, line 42.

[454] Page 1042, line 58. (two occurrences)

[455] Page 1045, line 20.

[456] Page 1045, line 21.

[457] Page 1064, line 62.

[458] Page 1066, line 57.

[459] Page 1067, line 18. (two occurrences)

[460] Page 1067, line 24.

[461] Page 1069, line  7.

[462] Page 1069, line  8.

[463] Page 1069, line 29.

[464] Page 1069, line 33.

[465] Page 1069, line 34.

[466] Page 1069, line 39.

[467] Page 1069, line 40.

[468] Page 1069, line 57.

[469] Page 1069, line 58.

[470] Page 1070, line  3.

[471] Page 1070, line 43.

[472] Page 1070, line 44.

[473] Page 1070, line 45.

[474] Page 1070, line 49.

[475] Page 1070, line 50.

[476] Page 1070, line 54.

[477] Page 1070, line 55.

[478] Page 1070, line 60.

[479] Page 1071, line  7.

[480] Page 1071, line  8.

[481] Page 1072, line 19. (two occurrences)

[482] Page 1073, line 49.

[483] Page 1073, line 51.

[484] Page 1074, line 18.

[485] Page 1074, line 21.

[486] Page 1074, line 49.

[487] Page 1074, line 51.

[488] Page 1075, line 10.

[489] Page 1075, line 12.

[490] Page 1075, line 40.

[491] Page 1075, line 42.

[492] Page 1077, line 53.

[493] Page 1077, line 55.

[494] Page 1080, line 26.

[495] Page 1080, line 29.

[496] Page 1081, line 59.

[497] Page 1081, line 61.

[498] Page 1081, line 65.

[499] Page 1082, line 22.

[500] Page 1082, line 57.

[501] Page 1082, line 59.

[502] Page 1083, line 62.

[503] Page 1084, line 39.

[504] Page 1085, line 54.

[505] Page 1085, line 56.

[506] Page 1086, line 44.

[507] Page 1086, line 46.

[508] Page 1087, line 25.

[509] Page 1087, line 27.

[510] Page 1088, line 34.

[511] Page 1088, line 36.

[512] Page 1089, line 34.

[513] Page 1089, line 36.

[514] Page 1090, line 28.

[515] Page 1090, line 30.

[516] Page 1094, line 61.

[517] Page 1095, line 13.

[518] Page 1098, line 19.

[519] Page 1098, line 20.

[520] Page 1098, line 22.

[521] Page 1098, line 23.

[522] Page 1108, line  7.

[523] Page 1109, line 59.

[524] Page 1110, line  9.

[525] Page 1110, line 18.

[526] Page 1113, line 15.

[527] Page 1114, line 57.

[528] Page 1121, line  5.

[529] Page 1122, line 20.

[530] Page 1122, line 23.

[531] Page 1123, line 20.

[532] Page 1123, line 46. (two occurrences)

[533] Page 1123, line 47.

[534] Page 1123, line 57. (two occurrences)

[535] Page 1123, line 58.

[536] Page 1162, line 32.

[537] Page 1172, line 33.

[538] Page 1188, line 36.

[539] Page 1189, line 46.

[540] Page 1194, line  8. (two occurrences)

[541] Page 1194, line  9.

[542] Page 1194, line 11.

[543] Page 1194, line 50. (two occurrences)

[544] Page 1194, line 51.

[545] Page 1195, line 38.

[546] Page 1197, line 51.

[547] Page 1198, line 18.

[548] Page 1198, line 19.

[549] Page 1204, line 62. (two occurrences)

[550] Page 1205, line  1.

[551] Page 1205, line  2.

[552] Page 1205, line  3.

[553] Page 1209, line 31.

[554] Page 1338, line 31.

[555] Page 1338, line 32.

[556] Page 1338, line 34. (two occurrences)

[557] Page 1338, line 36. (two occurrences)

[558] Page 1338, line 49.

[559] Page 1382, line 20.

[560] Page 1382, line 24.

[561] Page 1388, line 32.

[562] Page 1398, line 21.

[563] Page 1398, line 24. (two occurrences)

[564] Page 1398, line 27. (two occurrences)

[565] Page 1401, line 32.

[566] Page 1415, line 32.

[567] Page 1463, line 53.

[568] Page 1496, line 13.

[569] Page 1496, line 14. (two occurrences)

[570] Page 1496, line 32.

[571] Page 1658, line 19.

[572] Page 1737, line 18.

[573] Page 1777, line 15.

[574] Page 1835, line 28.

[575] Page 1835, line 48.

[576] Page 1835, line 54.

[577] Page 2138, line 63.

[578] Page 2142, line 26.

[579] Page 2142, line 27.

[580] Page 2146, line 26.

[581] Page 3529, line 49.

[582] Page 3529, line 54.

[583] Page 3529, line 62.

### Style Guide 2.4.1 – Information Elements/subelements – Naming - done

“Elements should be called the “<Purpose> element”, where <Purpose> does not include the word “information”….”

- P1043 L55

“8.4.2.167 Device Location Information element

A  Device  Location  Information  element  includes  the  location  configuration  information  (LCI),  which contains latitude, longitude, and altitude information. The Device Location Information element format is shown in Figure 8-568 (Device Location Information element format).

The Element ID and Length fields are defined in 8.4.2.1 (General).

The format of the Device Location Information Body is defined in 8.4.1.56 (Device Location Information Body).”

Proposed Change:  Globally change “Device Location Information element” to “Device Location element” – done, plus changed the refernce in the table of Element IDs.

### Style Guide 2.4.2 – Definition Conventions

“…There is no need to specify any value for the Element ID field….”

- P827 L16

“8.4.2.29 TSPEC element

Add the following statement in this subclause:

” The Element ID and Length fields are defined in 8.4.2.1 (General).” “

- P 743 L63: - done

“The Subelement ID field is defined set to the value for Azimuth Request in Table 8-103 (Optional subelement IDs for LCI request).

The Length field is defined in 8.4.3 (Subelements).”

The following instances are similar with above:

P746 L50 - done

P 758 L14-15 - done

P 770 L34-35 - done

P 783 L12-13 - done

P 922 L34-35 - done

- P862 L40 - done

“The Subelement ID field is defined in contains one of the values from Table 8-164 (Subelement IDs):”

The following instances are similar with above:

P892 L47 L62 - done

P893 L28 - done

- P919 L40 - done

“The Subelement ID field is contains the value for Location Indication Parameters as defined in Table 8-196 (Location subelements).”

The following instances are similar with above:

P921 L12-13 - done

P922 L1-2 - done

P923 L18 - done

P924 L28, L57 - done

P925 L32-33 - done

Proposed changes:  make changes as shown above at cited locations.

### Style Guide 2.9 - Use of verbs & problematic words

(Note that the findings have been adjusted on review.  Some original findings are shown with strikeout where the review disagrees with the original findings.)

- “Will”

- P83 L32-33 - done

“…WNM-Sleep mode enables a non-AP STA to signal to an AP that it might sleep for a specified length of time….”

- P100 L26-27 - done

“It is expected that most other protocol exchanges use the IEEE Std 802.1X Controlled Ports.”

- – will is OK as this is a necessary statement of future fact

- – OK as is

- P414 L34	  will send -> sends - done

- P550 L60	  will be -> is - done

- P957 L32-33	  will first attempt -> first attempts - done

- P968 L15-16	  will be returned -> are returned - done

- P1050 L2-3	   will return -> returns - done

- P1053 L38-39	  will exchange -> exchanges - done

- P1109 L47 L49-50  will be present -> is present - done

- P1154 L46-47  will issue -> issues - done

- – OK as is

- P1552 L42 - done

“…The shorter receive time will reduces the power consumption for non-AP STAs in a power save mode….”

- P1719 L50 - done

“... in which case it will sets the Number of Bursts Exponent field to 0.”

- P1720 L50-51 will set -> sets - done

- P1855 L51-21 - done

“The parties involved will be are called STA-A and STA-B.”

- P1858 L21-22 - done

“…there will be are two possible solutions:…”

- P2489 L39-40 - done

“NOTE—Due to the limitations in the maximum A-MPDU length, B19-20 are will arealways be 0 for an 80 MHz, 160 MHz, and 80+80 MHz VHT SU PPDU.”

- P2496 L64 “will each be” -> “each is ” - done

“Must”:

- P1859 L62-63 – no action taken

“The values qr and qnr may be used for all loops in the hunting-and-pecking process but a new value for r shall must be generated each time a quadratic residue is checked.”

TGmc:  need to  with Dan Harkins – no need to take action because of the following para:

(Note this is the subject of an existing comment resolution with the same proposed change.  Any resolution to that comment will be applied.  No change proposed.)

- P2853 L12-13: “must wait-> waits” – ok - done

- P2869 L40-41:  “must use-> uses” – ok - done

- P2876 L13-14: -  - done

If a new SMK authentication is not completed successfully before the number of seconds indicated by this attribute elapse, from the prior authentication, the STA deletes the SMKSA.

- P2886 L49:   - done

- Replace: “The same value must be used for the NAS Client identifier and dot11FTR0KeyHolderID.” With:

- Subclause 12.2.2 requires that the same value is used for the NAS Client identifier and dot11FTR0KeyHolderID.

- P3441 L54-55 (The annex M is informative.): - done

*     The value {TA,IV32,IV16} for Phase1/Phase2 is must be unique

*     across all frames using the same key TK value….

TGmc: Ask Dan for opinion. Change made as indicated.  reverse if necessary.

- – leave it as is

- P3458 L22 “must have -> has” – ok - done

- – leave it as is

- P3490 L11 : (Annex N is informative, so…) - done

“Must be Specified if the delay bound is present”

- P3500 L58  “must be -> is” – ok - done

“May not”:

- P1721 L51-52 - done

“…The initial Fine Timing Measurement frame may or may might or might not have captured timestamps….”

- P1757 L5  “may or may not” ->”might or might not” - done

- - leave unachanged as this is deprecated text.

- “Only”

- Note,  there are about 193 “is only” in REVmc D3.  The vast majority of these appear to fail the WG11 style guide on proper use of “only”.  Only a smallish number of these were reported and addressed in the MDR.

- OK

- P84 L58-60 - done

“…; some mesh STAs might propagate traffic for other STAs, but do not act as a source or sink of traffic.” - ok

- P535 L5 - done

“This primitive  can  only  be  issued  only following  a  transmit  initialization  response  (a  PHY-TXSTART.confirm primitive) from the PHY.”

- P972 L6-7 - done

“This field is only present when the TX-RX Report Present subfield of the MCCAOP Advertisement Element Information field is equal to 1; it is not present otherwise.”

- P972 L13-14 L20-21  - done

Similar with above

- P973 L53-54

“This field is only included if the element is transmitted by the MCCAOP responder; it is not included otherwise.”

- P976 L32-33 – done,  but see comment.

“If equal to 0, it will only reply under certain conditions (see 13.10.4.2 (Proactive PREQ mechanism)); it will not reply otherwise.”

- P981 L44 - done

“It is only present if the Originator Is Proxy subfield of the Flags subfield is 0; it is not present otherwise.”

- P981 L49 - done

“It is only present if the Lifetime subfield of the Flags subfield is 1; it is not present otherwise.”

- P1155 L63 - done

“The BSS Transition Management Response frame is only transmitted in response to a BSS Transition Management Request frame; it is not transmitted otherwise.”

- P1239 L1-4 - done

“Figure 9-4  (RTS/CTS/data/Ack  and  NAV  setting)  indicates  the  NAV  for  STAs  that  may might receive  the  RTS frame, while other STAs may might receive only the CTS frame, resulting in the lower NAV bar as shown (with the exception of the STA to which the RTS frame was addressed).”

- P1485 L24 - done

“The SSW-Ack only occurs within the DTI of a beacon interval (9.38.6.2 (SLS phase execution)); it does not occur otherwise.”

- P1520 L52-53 - done

“ …In addition, the AP or PCPAP may choose to only include a partial list of nontransmitted BSSID profiles in the Beacon frame or DMG Beacon frame or to include different sets of nontransmitted BSSID profiles in different Beacon frames or DMG Beacon frames.”

- P1649 L55-56 - done

“When a STA receives an LCI request with the Location Subject field value equal to 2, the STA may only generate an LCI report if the MAC address in the Target MAC address field is its own MAC address; it shall not generate an LCI report otherwise.” - ok

- P1653 L18-19 – ok - done

Similar case as above

- P1654 L46 – ok - done

Similar case as above

- P1702 L8-11 - done

“A  STA  that  supports  event  reporting  may only send an Event  Request  or  Event  Report  frame to  a STA within the same infrastructure BSS or the same IBSS whose last received Extended Capabilities element contained a value of 1 for the Event bit in the Capabilities field; it shall not send an Event Request or Event Report frame otherwise.” ?

- P1702 L28-29 - done

“…the receiving STA may only respond to the most recent request frame; it shall not respond to a request frame that is not the most recent one.”

- P1705 L55-58 - done

“A  STA  that  supports  diagnostics  reporting  may  only send  a  Diagnostics  Request  or  Diagnostics  Report frame  to  a  STA  within  the  same  infrastructure  BSS  or  the  same  IBSS  whose  last  received  Extended Capabilities element contained a value of 1 for the Diagnostics bit in the Capabilities field; it shall not send  a  Diagnostics  Request  or  Diagnostics  Report frame otherwise..”

- P1706 L37-38 - done

“A  STA  shall transmit  both  the  Diagnostic  Request  frame  and  the  Diagnostic  Report  frame  with  an individually addressed destination address”

- P1706 L48-49 - done

Proposed change:  delete cited sentence.

- P1748 - done

“A  STA  that  supports  WNM-Notification  reporting  may  send a  WNM-Notification  Request  or Response  frame  to  a  STA  within  the  same  infrastructure  BSS  or  the  same  IBSS  whose  last  received Extended Capabilities element contained a value of 1 for the WNM-Notification bit in the Capabilities field; it shall not send a WNM-Notification Request or Response frame otherwise.”

- P2103 L56-57 - done

“This field is only present if Bit 6 of the Flags field (AE subfield) is 1; it is not present otherwise.”

- P2105 L11-12 - done

“This field is only present if Bit 6 of the Flags field (AE subfield) is 1; it is not present otherwise.”

- P2106 L26-27 - done

“This field is only present if Bit 6 of the Flags field (AE subfield) is 1; it is not present otherwise.”

- P2116 L47-48 –done’

“This field is only present if Bit 6 (AE subfield) of the Flags field #1 is 1; it is not present otherwise.”

- P2127 L39-40 - done

“MAC address of the proxy mesh gate. This field is only present if Bit 1 of the Flags field is 0; it is not present otherwise.”

The following instances of “shall only” were added in R5, during editing of the R4 changes shown above.  These need to be discussed in TGmc.

P169.17 and 183.28:

Specifies that the non-AP STA intends to associate for the purpose of unauthenticated access to emergency services. The parameter may be present if dot11InterworkingServiceActivated is true; the parameter shall not be present otherwise.

P1262.50:

The tolerances are specified in the physical layer management entity (PLME) SAP interface specification (see 6.5 (PLME SAP interface)) and apply only to the SIFS specification so that tolerances do not accumulate.

P1332.34:

However, MCCAOP reservations may be set up among mesh STAs that have dot11MCCAActivated true and that operate on the same channel; MCCAOP reservations shall not be set up otherwise . The performance of MCCA might be impacted by STAs that do not respect MCCAOP reservations.

P1641.50:

A number of triggered measurements may run concurrently at a non-AP STA. The number of simultaneous triggered measurements supported is outside the scope of the standard. Each triggered measurement is logically separate; reporting conditions such as Trigger Timeout periods apply only to the measurement for which they are established.

P1703.10:

If the number of available logged events of the requested type exceeds the Event Response Limit, the STA shall report an Event Response Limit number of the most recent events.

P1703.22:

On receipt of an Event Request frame with an Destination URI element, the reporting non-AP STA SME may send the Event Report to the AP using the Destination URI with another network interface (if available).

P1706.11:

Only a single Diagnostic Request frame from a STA is outstanding at a given STA at any time. If a STA receives a subsequent Diagnostic Request frame with a different Dialog Token before completing the Diagnostic Report for the previous request from the requesting STA, the STA may respond to the most recent Diagnostic Request frame; the STA shall not respond to a Diagostic Request frame that is not the most recent one. The STA need not send a Diagnostic Report frame for the previous Diagnostic Request frame.

P1709.24:

A STA may transmit the Location Configuration Request frame as a broadcast or individually addressed frame. A STA that supports location and receives a broadcast Location Configuration Request frame may send a Location Configuration Response frame if the STA does not accept the parameters included in the Location Configuration Request; the STA shall not send a Location Configuration Response frame if it accepts the parameters included in the Location Configuration Request.

P1709.43:

Upon receipt of a Channel Usage element in the Probe Request frame, the AP supporting Channel Usage shall send a Probe Response frame including one or more Channel Usage elements. Upon receiving a Channel Usage Request frame, the AP supporting Channel Usage shall send a Channel Usage Response frame including one or more Channel Usage elements. Channel Usage elements shall include channels that are valid for the regulatory domain in which the AP transmitting the element is operating and consistent with the Country element in the Beacon or Probe Response frame; the Channel Usage elements shall not include any other channels.

P1748.32:

A STA shall transmit both the WNM-Notification Request frame and the WNM-Notification Response frame with an individually addressed destination address.

P1759.53:

The Query List ANQP-element is used by a requesting STA to perform an ANQP Query using the procedures defined in Error: Reference source not found. The requesting STAymay include Info IDs in the Query List ANQP-element that have the sole ANQP-element type of S as shown in Error: Reference source not found; the STA shall not include other Info IDs. Info IDs that have an ANQP-element type of Q shall not be included in the Query List ANQP-element (e.g., the Info ID for Vendor Specific ANQP-element shall not be included).

P1760.07:

An AP may include an OI in the dot11RoamingConsortiumTable, if in conjunction with an AS, it is capable of successfully authenticating a non-AP STA having valid security credentials for the SSPN identified by that OI; the AP shall not include an OI otherwise. Methods used by the AP to authenticate the non-AP STA include, but are not limited to, RSNA algorithms and Open System authentication.

P1766.15:

When an AP is located in a regulatory domain that requires location capabilities, the ESR field may be set to 1 and the Network Type may be set to “Emergency services only network” (see Table 8-219 (Access network type)), if location capability is enabled on the AP; the ESR field shall not be set to 1 and the Network Type shall not be set to “Emergency services only network” otherwise.

P1767.47:

An AP may include an OI in the dot11RoamingConsortiumTable, if in conjunction with an AS, it is capable of successfully authenticating a non-AP STA having valid security credentials for the SSPN identified by that OI; the AP shall not include an OI otherwise..

P2144.51:

A mesh STA may transmit frames to a mesh STA operating in a light or deep sleep mode if the recipient mesh STA is in the Awake state as defined in Error: Reference source not found, Error: Reference source not found, and Error: Reference source not found; otherwise, the mesh STA shall not transmit frames and shall buffer frames.

- “Ensure”:

- P6 L43-44 - done

Replace: An algorithm to ensure that admittance of a new flow into a resource constrained network does not violate parameterized service commitments made by the network to admitted flows.

with:

An algorithm intended to prevent the violation of parameterized service commitments made by the network to admitted flows by controlling the admittance of a new flow into a resource constrained network.

- P43 L55 - done

The portion of a transient key used to validate the integrity of medium access control (MAC) service data units (MSDUs) or MAC protocol data units (MPDUs).

- – No obvious replacement that maintains the current meaning

- – “shall … ensure” is OK.

(note to editors,  need to update the WG11 style guide to call out this is OK”)

- – “shall … ensure” is OK.

- P1442 L31 - done

The AP or PCP shall insert a sufficient guard time between adjacent allocations so that transmissions in adjacent allocations do not overlap in time.

- – “shall … ensure” is OK.

- – “ensure” is ok in this context.

- P2331 L1: - done

Steps a) and b) occur over a short time interval so that the channel changes as little as possible

between measurements.

- P2475 L26-27 - done

“To ensure compatibility with…” -> “To maintain compatibility with”

- P3236 L45 - done

A road section identifier might be needed to make an address unique.

- P3454 L64 - done

void Michael::runTestPlan()

- P3496 L1: - done

9.22.3.3

Subclause 9.22.3.3 requires that the ACU complies with the dot11CAPlimit, i.e., the scheduler does not allocate TXOPs that exceed dot11CAPlimit. The ACU might also consider additional time to allow for retransmissions.

- P3532 L51-52 (The Annex S is informative)

“Some of the STAs that are associated with a 20/40 MHz BSS are required to perform monitoring in order to ensure so that  the  conditions  that  allowed  the  establishment  of  the  20/40  MHz  BSS  do  not  change  to conditions that would disallow the existence of the 20/40 MHz BSS.”

- P3553 L3-4 (The Annex S is informative)

If the EDCA Access Factor is greater than one, then there is a potential over-allocation of the WM. APs should avoid over-allocation in the channel selection process, but if over-allocation exists, then a sharing scheme should be used to provide each AP with a fair share of the bandwidth, but more importantly, so that any already admitted or scheduled QoS streams are not impaired by the addition of streams from any OBSS.

The following additional instances of “ensure” need to be discussed in TGmc.

P1828.09:

An AP should notify associated STAs of a change in the maximum number of spatial streams it is able toreceive through one or more of the following mechanisms:

— Using individually addressed Operating Mode Notification frames

— Including the Operating Mode Notification element in Beacon frames for a period of time that ensures that STAs in PS modewill receive the notification

P2499.09:

Note that this process results in the BCC tail bits being placed at the very end of the PPDU.

P3310 & P3312.44:

bIt is the responsibility of management layers outside the scope of this standard to ensure that channels in use at any location are nonoverlapping.

P3496.06:

The ACU ensures that all admitted streams have guaranteed access to the channel. Any modification can be implemented for the design of the ACU. For example, UP-based ACU is possible by examining the UP field in TSPEC to decide whether to admit, retain, or drop a stream. If the UP is not specified, a default value of 0 is used. If a higher UP stream needs to beserviced, an ACU mightdrop lower UP streams.

### Style Guide 2.9.1 – Which/that

- P1299 L29 - done

Missing comma:

“An implementation may reduce the A-MPDU_Length[n] by the amount of padding for user n that was added subsequent to the addition of a subframe for user n that contains 1 in the EOF field.”

### Style Guide 2.9.3 – Missing noun in noun phrase

Change BA Ack Policy to BA Ack Policy subfield – Page  595, line 31. - done

### Style Guide 2.10 Numbers: - done except as shown below

Values are shown as digits when representing the value of fields

- P27 line 56

- P503 line 19

- P596 line 7

- P637 line 25

- P700 line 63

- P835 line 30 2-bit – Ed: disagree, no change made.

- P1004 line 2

- P1113 line 15

- P1341 line 17

- P1447 lines 3, 18

- P1455 line 42

- P1464 lines 29, 34, 35, 37

- P1475 line 33

- P1482 lines 15, 19

- P1510 lines 62, 64

- P1511 line 12

- P1516 line 3

- P1568 line 53

- P1769 line 42

- P1772 line 63

- P1773 lines 3, 11, 13

- P1774 line 17

- P1789 line 45

- P1805 lines 4, 10

- P2065 line 13

- P2060 line 62

- P2572 line 39

- P3489 line 40

Bits are numbered using an upper-case B, e.g., B12 – done except as noted below

- P2183 lines 4, 5, 8, 9

- P2391 lines 32, 38 – no change made,  these are not bit numbering labels, but variables representing the value of a bit at that position.

- P2398 lines 10, 15 - ditto

- P2410 lines 34. 39, 46, 48, 56 – ditto

- P2417 lines 26, 28, 51 - ditto

Note that there is always a space between a number and its units (e.g., “20 MHz”).  - done

- P76 line 29

- P2339 line 35 (40MHz and 20MHz)

Long numbers have embedded spaces to group digits into threes (e.g., “65 635 octets”). - done

- P284 line 10

- P965 line 23

- P966 line 51

- P1049 lines 50, 56

- P1629 line 51

- P851 lines 23, 25, 27, 29

Proposed change:  adjust cited locations to match style

### Style Guide 2.11 Maths operators and relations:

The  symbolic forms for Floor() and Ceil() should be used wherever possible.

- P781 lines 54, 57

- P782 line 5

- P1149 lines 4, 10

- P1434 line 2 – no change made,  this is the 2-parameter form

- P1442 line 37 – ditto

- P1448 line 43 - ditto

- P3491 lines 3, 24

- P3566 line 20

Proposed change:  adjust cited locations to match style

### Style Guide 2.12 Hyphenation:

The editor closed up the following compound words in Std 802.11-2012:

- P34 line 47 nonbandwidth

- P132 Figure 5-1 Deaggregation

- P133 Figure 5-2 Deaggregation

- P141 line 17 unadmitted

- P589 line 16 nonbandwidth

- P821 lines 28, 32 unadmitted

- P1015 line 46 streambased

- P1161 line 48 nonzero

- P1202 lines 28, 31 predefined

- P1203 line 27 predetermined

- P1223 line 60 nonmesh

- P1241 lines 26, 59 nonbandwidth

- P1242 line 1 nonbandwidth

- P1249 line 57 nonmesh

- P1285 lines 15, 33 nonbandwidth

- P1302 line 59 nonunique

- P1310 line 60 nonbandwidth

- P1312 line 43 nonprimary

- P1315 line 43 nonzero

- P1351 Figure 9-34 Deaggregation

- P1363 Figure 9-36 Deaggregation

- P1359 line 33 nonprotected

- P1421 lines 29, 58 nonbandwidth

- P1531 line 28 Nonbufferable

- P1570 line 16 nonmesh

- P1571 lines 1, 45 nonmesh

- P1623 line 50 selfallocated

- P1715 line 46 nondeterministic

- P1725 line 57 web browser

- P1725 line 58 end user

- P1733 lines 27, 30, 35 keepalive

- P1734 line 61 noninfrastructure

- P1757 line 48 nondecreasing

- P1769 line 57 nondefault

- P1784 line 26 noninteger

- P2090 line 16 proactive

- P2141 Fig 13-5 has three non-peer

- P2442 line 17 nonzero

- P2499 line 19 rearranged

- P3492 line 56 nonzero

- P3504 Figure O-4 nontransmitted

- P3505 Figures O-5, O-6 nontransmitted

- P3506 Figure O-7 nontransmitted

- P3561 line 25 nonmonotonic

- P3565 lines 48, 53, 57, 63 overallocated

- P3566 line 59 overallocation

- P3573 lines 50, 51, 62 overallocation

- P3574 line 7 overallocation

- P3576 line 2 overallocated

Note that IETF RFC xxx references do not use a hyphen in IEEE 802.11.

- P3229 lines 49, 58

Proposed change:  adjust cited locations to remove hyphen - done

### Style Guide 2.13 – References to SAP primitives

- Editor: also adjusted terminology e.g., “message|event|procedure” -> “primitive”

- For TGn: recommend adding to word usage 1.5:

- ”In the expression ‘Generating an <name-of-primitive>.<type-of-primitive> primitive’, it should be understood that it is an instance of the primitive that is generated.”

- P143 L59 - done

“6.3 MLME SAP interface

6.3.1 Introduction

The  services  provided  by  the  MLME  to  the  SME  are  specified  in  this  subclause.  These  services  are described in an abstract way (following the model described in ITU-T Recommendation X.210 [B51]) and do not imply any particular implementation or exposed interface. MLME SAP primitives are of the general form  ACTION.request primitive followed  by  ACTION.confirm primitive (for  an  exchange  initiated  by  the  SAP  client)  and ACTION.indication primitive followed by ACTION.response primitive (for an exchange initiated by the MLME). The SME uses the services provided by the MLME through the MLME SAP.”

- P204 L44 - done

“6.3.12.2.3 When generated

(The 3rd Paragraph:)

The SME should notify associated non-AP STAs of imminent infrastructure BSS termination before issuing the  MLME-STOP.request primitive.  This  can  be  done  with  the  BSS  transition  management  procedure,  using  the Termination information.”

- P232 L23 – done + reviewed all “.indication.”, “.request.”, “.confirm.”, “.response.”

“As defined in the corresponding MLME-ADDTS.indication primitive.”

- P235 L34 – done

- P288 L51 – done + reviewed all “.indication request”

- P289 L59 - ok

“6.3.42.2.4 Effect of receipt

The MLME issues an MLME-GETTSFTIME.confirm primitive.”

- P290 L30 -ok

“6.3.42.3.3 When generated

This primitive is generated by the MLME to report to the SME the result of an MLME-GETTSFTIME.request primitive.”

- P306 L54 – done + reviewed all “.request and”

“6.3.47.3.3 When generated

This  primitive  is  generated  by  the  MLME  as  a  result  of  an MLME-TDLSPTI.request primitive and  indicates  the results of the request.”

- P310 L13 - ok

“6.3.48.3.3 When generated

This primitive is generated by the MLME as a result of an MLME-TDLSCHANNELSWITCH.request primitive and indicates the results of the request.”

- P313 L49 - ok

“6.3.49.3.3 When generated

This primitive is generated by the MLME as a result of an MLME-TDLSPEERPSM.request primitive and indicates the results of the request.”

- P345 L23 – done

- P350 L61 - ok

6.3.60.3.3 When generated

This primitive is generated by the MLME as a result of an MLME-FMS.request primitive and indicates the results of the request. This primitive is generated when the STA receives an FMS Response frame from the AP.

- P360 L46 – ok

- P361 L34 - ok

- P370 L48 - ok

- P371 L26 – ok

- P375 L41 – done + reviewed all “.request shall”

6.3.67.2.4 Effect of receipt

On receipt of this primitive, the MLME constructs a Channel Usage Request frame. The STA then attempts to transmit this to the BSS which is advertising the SSID included in this primitive request. When wild card SSID  is  passed  down,  the  MLME-CHANNELREQUEST.request primitive shall  be  transmitted  to  all  BSSs  in  the current scan list as determined by the most recent MLME-SCAN.request primitive.

- P377 L38 – ok

- P378 L23 – ok

- P382 L30 - ok

- P383 L4 - ok

- P400 L4 – done + reviewed all “.request with”

6.3.75.3.3 When generated P399 L4

This  primitive  is  generated  as  a  result  of  an  MLME-MESHPEERINGMANAGEMENT.request primitive with  a specified MAC peer.

- P402 L49 - ok

6.3.76.3.3 When generated

This primitive is generated as a result of an MLME-MESHPOWERMGT.request primitive.

- P403 L34 – done + reviewed all “.request that”

“6.3.77.2.4 Effect of receipt

On receipt of this primitive, the MLME commences the neighbor offset synchronization method and the calculation    of    the    TSF    timer    offset    value.    The    MLME    subsequently    issues    an    MLME-MESHNEIGHBOROFFSETSYNCSTART.confirm primitive that reflects the results of this request.”

- P404 L5 L45 L49 – done.  + reviewed all /.request [^po(]/

- P405 L17 L57 – done + reviewed all /.confirm [^p(]/, /.indication [^p(]/, /.response [^p(]/

- P406 L20 – ok

- P407 L32 – ok

- P409 L2-3 – ok

- P412 L18 L30 - ok

- P413 L4-?? L54 – ok

- P414 L61 ok

- P415 L23 L30 - ok

- P416 L52 - ok

- P420 L48 - ok

- P424 L35 - ok

- P425 L20 - ok

- P433 L26 - ok

- P466 L62 - ok

- P469 L64  6.3.93.7.2 - ok

- P482 L51  6.3.97.3.3 - ok

- P502 L20-21: - ok

“6.4.4.1.2 From ESS_DISENGAGING

To make this transition, the SME cancels a previous event that predicted an ESS link failure. This might be due to network parameters indicating renewed link strength or a successful renewal of an expiring RSN SA. When     this     transition     is     complete,     the     MSGCF     sends     an     MSGCF-ESS-LINK-EVENT-ROLLBACK.indication primitive event to indicate that a prior link failure predictive event is no longer valid. If the transition was due to network parameters crossing a threshold, the MSGCF also sends issues an MSGCF-ESS-LINK-THRESHOLD-REPORT.indication primitive to higher layers.”

- P502 L32-33 - ok

“… sends issues an MSGCF-ESS-LINK-DOWN.indication primitive event to higher layer protocols.”

- P502 L38-39 L55 - ok

- P504 L1  - ok

“…encryption. This event is generated upon receipt of an MLME-ASSOCIATE.confirm primitive message with a result code of success.”

- P504 L13 L14 - ok

- P507 L29 L30 - ok

- P512 L15 L24-25 L30 L35 - ok

- P518 L33 L51 - ok

- P519 L46-47 - ok

- P523 L57 - ok

- P526 L58 - ok

- P1013 L46 - ok

“The Reference Timestamp field contains the lower 4 octets of the TSF timer value sampled at the instant that the MAC received the PHY-CCA.indication(IDLE) primitive signal that corresponds to the end of the reception of the PPDU that was used to generate the feedback information contained in the Link Measurement Report frame.”

- P1014 L41-45 – ok

- P1035 L49 – ok, also reviewed all /.indication(*) [^p(]/

- P1233 L36 - ok

“…the value of aAirPropagationTime indicated in the LME-CHARACTERISTICS.confirm primitive.”

- P1260 L62 L63 - ok

“…, the value indicated in the PLME-CHARACTERISTICS.confirm primitive

aSlotTime and aSIFSTime are the values indicated in the PLME-CHARACTERISTICS.confirm primitive”

- P1308 L33 - ok

- P1317 L34 – ok – but note awkward resulting language “.indication of” almost makes sense,  but “.indication primitive of” makes little.

- P1427 L31: - ok

“MSDUs sent by a mesh STA (as a consequence of an MA-UNITDATA.request primitive with an individual destination address) and destined to another mesh STA in the MBSS shall be transmitted using a frame with the four-address MAC header format (with the Address Extension Mode subfield in the Mesh Control field set to 00 (binary)), where the four address fields are set as follows (see row “Mesh Data (individually addressed)” in Table 9-17 (Valid address field usage for Mesh Data and Multihop Action frames)):”

L45:

MSDUs that are sent by a mesh STA as a consequence of an MA-UNITDATA.request primitive with an individual destination address and are destined to an address) that is different from the mesh STA at the end of a mesh path  shall  be  transmitted  using  a  frame  with  the  four-address  MAC  header  format  [with  the  Address Extension Mode subfield in the Mesh Control field set to 10 (binary)], where the Mesh Address Extension subfield in the Mesh Control field carries the addresses of the end stations, as specified in row “Mesh Data (proxied, individually addressed)” of  Table 9-17  (Valid  address  field  usage  for  Mesh  Data  and  Multihop Action frames). The additional addresses 5 and 6 are defined as follows:

- P1429 L28 - ok

- P1432 L65 - ok

- P1450 L10-11 – ok (but awkward)

- P1451 L19 - ditto

- P1576 L15 – alternative: “generating an MLME-SETPROTECTION.request(None) (MDR)primitive”

“  e)  The  SME,  upon  receipt  of  an  MLME-DEAUTHENTICATE.confirm  primitive,  shall  delete  any PTKSA and temporal keys held for communication with the indicated STA by using the MLME-DELETEKEYS.request  primitive  (see  11.5.18  (RSNA  security  association  termination))  and  by invoking the LME-SETPROTECTION.request primitive (None).

- P1576 L44 - ditto

- P1577 L54 - ok

“If dot11InterworkingServiceActivated is true, the STA does not have credentials for the AP, and the STA is initiating   an   emergency   services   association   procedure,   the   SME   shall   submit   the   MLME-ASSOCIATE.request primitive with EmergencyServices parameter set to true.”

- P1578 L47 L49 - done

- P1580 L35 L55 L65 - done

- P1582 L35 - ok

- P1583 L49 - ok

- P1584 L25 - ok

- P1597 L34 - ok

- P1601 L39 L40 L58-59 – ok.  Also reviewed “.request)”

- P1602 L7 - ok

- P1708 L22 L24 L59 L61 L65 - ok

- P1725 L54 L56-57 - ok

- P1756 L34 L36 L48 L53 - ok

- P1768 L44 - ok

- P1773 L28 L32-33 L34-L35 L38-39 L39-40 - ok

- P1773 L33 L35 L37~40 as follows: - ok

“The     SME     of     the     peer     STA     evaluates     the     QMF     policy     and     uses     the     MLME-QMFPOLICYCHANGE.response primitive to transmit a QMF Policy frame to the STA including the result of the QMF policy change. The value of the dialog token in the MLME-QMFPOLICYCHANGE.response primitive shall    be    set    to    the    value    of    the    dialog    token    received    in    the    corresponding    MLME-QMFPOLICYCHANGE.indication primitive.

When    the    QMF    STA   SME    receives   an    MLME-QMFPOLICYCHANGE.confirm primitive or    MLME-QMFPOLICY.indication primitive with Result Code of SUCCESS, it shall set the new QMF policy using the MLME-QMFPOLICYSET.request primitive.”

- P1826 L35  - ok

- P2163 L32 - ok

- P2347 L45-46 L51-52 - ok

- P2348 L3-4 L7 L12 L15-16 - done

- P2355 L16-17 - ok

- P2419 L28 - done

- P2420 L20 - ok

- P2420 L31 L32 - ok

“The PHY continues with the encoding and transmission of the header according to the parameters of the PHY-TXSTART.request(TXVECTOR)  primitive.  The  PHY  proceeds  with  PSDU  transmission  through  a series  of  data  octet  transfers  from  the  MAC.  The  data  are  encoded  as  described  in  21.4.3.2.3  (Header encoding  and  modulation),  21.5.3.2.3  (Encoding),  and  21.6.3.2.3  (Encoding).  The  encoded  data  are  then modulated as described in 21.4 (DMG control PHY), 21.5 (DMG OFDM PHY), 21.6 (DMG SC PHY), and 21.7  (DMG  low-power  SC  PHY),  depending  on  the  MCS  requested  in  the  PHY-TXSTART.request primitive. Transmission  can  be  prematurely  terminated  by  the  MAC  through  the  primitive  PHY-TXEND.request primitive. PHY-TXSTART shall be disabled by receiving a PHY-TXEND.request primitive.”

- P2422 L44-45 L52-53 - ok

- P2448 L40 - ok

- P2532 L3-4 L37 L43-44: - done

“The receiver shall issue a PHY-CCA.indication(BUSY, {primary}) primitive for any signal that exceeds a threshold equal to 20 dB above the minimum modulation and coding rate sensitivity (–82 + 20 = –62 dBm) in the primary 20 MHz channel within a period of aCCATime after the signal arrives at the receiver's antenna(s); then the receiver shall not issue a      PHY-CCA.indication(BUSY,{secondary}), PHY-CCA.indication(BUSY,{secondary40}),       PHY-CCA.indication(BUSY,{secondary80}), or PHY-

CCA.indication(IDLE) primitive while the threshold continues to be exceeded.”

- P2532 L48 L49 L55 L62 L63 - ok

- P2533 L6-7 L17-18 L20 - ok

- P2535 L21 - ok

Transmission  can  be  prematurely  terminated  by  the  MAC  through  the  primitive  PHY-TXEND.request. PSDU primitive  transmission   is   terminated   by   receiving   a   PHY-TXEND.request  primitive.   Each   PHY- TXEND.request primitive is  acknowledged  with  a  PHY-TXEND.confirm  primitive  from  the  PHY.  In  an  SU transmission,  normal  termination  occurs  after  the  transmission  of  the  final  bit  of  the  last  PSDU  octet, according to the number of OFDM symbols indicated by NSYM (see 22.4.3 (TXTIME and PSDU_LENGTH calculation)).

- P2537 L12 L17 L27-28 L30 L39-40

- P2539 L40-41

- P2607 L53-54

- P2608 L16-17 L21 L27 L28-29 L37 L44-45 L45-46 L53-54

- P2877 L1

- P3084 L19

- P3085 L9 L26

- P3086 L45 L58

- P3088 L1 L17

- P3160 L44 L59

- P3161 L8 L23

- P3536 L7

- P3549 L11 L37-38

Proposed change:  Make changes as shown above.

### Style Guide 2.15 References to MIB variables/attributes:

In P74L49:

“from dot11RMNeighborReportTable concerning neighbor APs”

In P79L25:

“A STA whose MIB does not include dot11OCBActivated operates as if the attribute is false.”

In P160L58, P165L55:

“when dot11AuthenticationAlgorithm is simultaneousAuthEquals (4))”

In P506L44:

“and might configure dot11ESSLinkDownTimeInterval accordingly.”

In P508L59:

“in dot11MACStateESSLinkDetectedTable”

In P614L58, P615L45, P627L48, P628L61:

“at least one dot11GASAdvertisementID”

In P615L18, P615L20, P618L40, P620L14, P622L10, P623L56, P623L58, P625L47, P628L31, P628L34, P1085L44, P1086L34:

“when dot11HighThroughputOptionImplemented is true”

In P615L23, P618L42, P620L17, P622L12, P623L61, P625L49, P628L36:

“dot112040BSSCoexistenceManagementSupport is”

In P615L27, P620L20, P623L64, P628L39:

“dot11FortyMHzOptionImplementedis true.”

In P627L32:

“if dot11QosOptionImplemented”

In P630L50:

“when dot11MeshActiveAuthenticationProtocol is sae (1)”

In P716L2:

“dot11CountryStringbefore”

In P997L9:

“overrides the value of dot11PSRequestSuspensionIntervalwith the”

In P997L15:

“value of dot11MinBHIDuration with the value”

In P997L21:

“STA overrides the value of dot11BroadcastSTAInfoDuration with the value of this”

In P997L27:

“overrides the value of dot11AssocRespConfirmTime with the”

In P997L33:

“a STA overrides the value of dot11MinPPDuration with the value”

In P997L39:

“overrides the value of dot11SPIdleTimeout with the value”

In P997L42:

“the value of dot11MaxLostBeacons”

In P997L43:

“a STA overrides the value of dot11MaxLostBeacons with the”

In P1060L7:

“This information is taken from dot11APLCITable.”

In P1060L27:

“This information is taken from dot11ApCivicLocation.”

In P1066L54:

“the value contained in dot11CountryString.”

In P1232L21:

“control ofdot11RTSThreshold.”

In P1237L29:

“from dot11EDCATableAIFSN.”

In P1237L35:

“each AIFS[AC] from dot11QAPEDCATableAIFSN.”

In P1258L41:

“indicated by dot11RTSThreshold.”

In P1258L47:

If dot11RTSThreshold is 0, …”

In P1304L16:

“mechanism is active, then dot11MultiDomainCapabilityActivated shall be true”

In P1319L9:

“equal to dot11ShortRetryLimit.”

In P1342L55:

“shall be set to dot11EDCATable,”

In P1513L61:

“equal to dot11MaxLostBeacons”

In P1514L38:

“the interval between Beacon frames is defined by dot11BeaconPeriod”

In P1515L2:

“the STA sets its dot11BeaconPeriod to that beacon period”

In P1614L25:

“SSP if the dot11NonAPStationAuthDls in”

In P1621L57:

“dot11SpectrumManagementRequired shall be set to true”

In P1682L25:

“if the STA has dot11FortyMHzIntolerant and dot11FortyMHzIntolerant is true”

In P1690L23:

“If a STA does not include the dot11OCBActivated, the STA shall operate as if the attribute is false.”

In P1710L45:

“dot11LocationTrackingActivated for the STA is false.”

In P1748L55:

“STAs indicate their support for interworking service by setting dot11InterworkingServiceActivated to true.”

In P1748L62:

“the AP may provide its operating channel and operating class to an Interworked SSPN using the values from dot11OperatingClassesTable.”

In P1753L31:

“If the Advertisement Protocol ID in the Advertisement Protocol element does not equal the value contained in any dot11GASAdvertisementID, then…”

In P1753L53:

“If the Advertisement Protocol ID in the Advertisement Protocol element equals the value contained

in any dot11GASAdvertisementID,…”

In P1786L32:

(specified by the current value of dot11BeamLinkMaintenanceTime)

In P1797L35:

“capable if the value of dot11MultibandImplemented is true”

In P1831L21:

“The current dot11DSTALCITable allows management access to the..”

In P1833L24:

“by decrementing the dot11GDDEnablementValidityTimer. The…”

In P1833L62:

“domain sets its dot11GeolocationCapabilityActivated to true.”

In P1834L62:

“that has its dot11GeolocationCapabilityActivated true should”

In P1852L17:

“exchange unless its dot11PrivacyOptionImplemented is true.”

In P1855L1:

“dot11PrivacyInvoked shall not take the value of true if dot11PrivacyOption-

Implemented is false.”

In P1926L22:

“…if the values of both dot11MultibandImplemented and dot11RSNAEnabled are true.”

In P2148L64:

“period is the lesser of the Max Retry Limit and  dot11MeshSTAMissingAckRetryLimit.”

Proposed change:  make changes as shown above

### Style guide 3.2 - General Description (Clause 4)

- 4.3.22 Operation under geolocation database (GDB) control

“Regulators designate television broadcast bands for the deployment of dynamic sharing technologies.

Different schemes result in different times that elapse from the moment an authorized database is told to change access to a particular slice of spectrum and the time that sharing radios are required to change their operations.”

- 4.5.2.2 Integration

page 98, line 60:  
“When the distribution service determines that the intended recipient of an MSDU is a member of an integrated LAN, the “output” point of the DS is a portal instead of an AP.”

- 4.6 Multiple logical address spaces

page 108, line 6:  
“A multiple address space example is one in which the DS implementation uses network layer addressing. In this case, the WM address space and the DS address space are different.”

### Style guide 3.3 – Frame Formats (Clause 8)

- 8.4.2.24.3 AKM suites
page 814, line 35:

“The AKM suite selector value 00-0F-AC:11 is used only with cipher suite selector values 00-0F-AC:8(GCMP-128) and 00-0F-AC:11 (BIP-GMAC-128). The AKM suite selector value 00-0F-AC:12 is used only with cipher suite selector values 00-0F-AC:9 (GCMP-256), 00-0F-AC:10 (CCMP-256), 00-0FAC:13 (BIP-CMAC-256), and 00-0F-AC:12 (BIP-GMAC-256).”

- 8.4.1.48 VHT Compressed Beamforming Report field

page 684, line 53:

“A beamformer may choose to reduce Ns by using a method referred to as grouping, in which only a single Compressed Beamforming Feedback Matrix is reported for each group of Ng adjacent subcarriers.”

- 8.4.2.24.2 Cipher suites

page 811, line 27:

“The cipher suite selector 00-0F-AC:0 (Use group cipher suite) is only valid as the pairwise cipher suite. An AP may specify the selector 00-0F-AC:0 (Use group cipher suite) for a pairwise cipher suite when it does not support any pairwise cipher suites. When an AP specifies 00-0F-AC:0 (Use group cipher suite) as the pairwise cipher selection, this is the only pairwise cipher selection the AP advertises.”

- 8.4.2.30 TCLAS element

page 835, line 22:

“An incoming MSDU that is not classified to a particular TS may be classified to another active TS, based on the frame classifier for that TS.”

- 8.4.2.46 Mobility Domain element (MDE)

page 861, line 44:

“When the Resource Request Protocol Capability subfield value is 1, the STA may perform the FT Resource Request Protocol of 12.6 (FT Resource Request Protocol).”

Proposed change:  make changes as indicated above.

## ANA

The generic proposed response to these findings is to instruct the ANA to review the findings in this section and update the ANA names, references and descriptions as necessary.

Specific substantive (i.e., technical) instructions to change to draft or ANA database are highlighted.

### TGaa ANA Usage

AKMSuiteSelector

Ref Subcluase and Location columns say 8.4.2.27.3 and Table 8-101 but its 8.4.2.24.3 and Table 8-140.
Value 10 ANA has blank Name and Description of “AP Peer Key Suite” but D3.0 says “APPeerKey Authentication with SHA-256 or using PMKSA caching as defined in 11.5.10.3”.

Categories
Ref Location says Table 8-38 but it is Table 8-54.

ElementIDs

Ref Location column says Table 8-54 but it is 8-85.

ExtendedCapabilities

Ref Subclause and Location columns say 8.4.2.29 and Table 8-103 but it is 8.4.2.26 and Table 8-142.

MACAddresses

ANA gives no Ref Subclause but I think it should say C.3 since the default GCR address value is given in the MIB.

PublicActionFrames

Ref Subclause and Location columns say 8.5.8.1 and Table 8-201 but it is 8.6.8.1 and Table 8-210.

StatusCodes
Ref Location column says Table 8-37, but it is actually Table 8-53.

### TGac ANA Usage

AKMSuiteSelector

Ref Subcluase and Location columns say 8.4.2.27.3 and Table 8-101 but its 8.4.2.24.3 and Table 8-140.
For values 11 and 12 the ANA Description column contains the embedded Clause #11.5.9.3 but should be Clause #11.5.10.3.
For value 13 ANA says “FT authentication negotiated over IEEE 802.1X (HMAC-SHA384)” but D3.0 just says “FT authentication negotiated over IEEE 802.1X”.

Categories
Ref Location says Table 8-38 but it is Table 8-54.

CipherSuiteSelectors

Ref Subclause and Location columns say 8.4.2.27.2 and Table 8-99 but its 8.4.2.24.2 and Table 8-138

ElementIDs

Ref Location column says Table 8-54 but it is 8-85.

ExtendedCapabilities

Ref Subclause and Location columns say 8.4.2.29 and Table 8-103 but it is 8.4.2.26 and Table 8-142.

OperatingClassesGlobal and OperatinClassesJapan and OperatingClassesEurope and OperatingClassesUSA

ANA has blank Name and Description but D3.0 says:

Value 128 – UseEirpForVHTTxPowEnv

Value 129 – UseEirpForVHTTxPowEnv
Value 130 – 80+, UseEirpForVHTTxPowEnv

StatusCodes
Ref Location column says Table 8-37, but it is actually Table 8-53.

### TGad ANA Usage

Categories

Ref Location says Table 8-38 but its Table 8-54.

Entry for #16 says Name “mmWave” but D3.0 says “DMG”.

CipherSuiteSelectors

Ref Subclause and Location columns say 8.4.2.27.2 and Table 8-99 but its 8.4.2.24.2 and Table 8-138

dot11Compliances

ANA lists 9 for “dot11DbandCompliance” but I think it should be “dot11DMGCompliance”.

dot11Groups
ANA lists 64 through 67 for dot11DBandComplianceGroup, dot11DBandOperationsComplianceGroup, dot11DBandBeamformingComplianceGroup, dot11DBandCountersComplianceGroup. But I think for all four “DBand” should be replaced by “DMG”.

dot11phy

ANA lists 19 for “dot11DBandPHYConfigTable” but I believe it should be “dot11PHYDMGTable”.

ANA lists 22 for “dot11DBandSTABeamformingConfigTable but I believe it should be “dot11DMGSTABeamformingConfigTable”.

dot11smt

ANA lists 27 for “dot11DBandSTAConfigTable” but I believe it should be “dot11DMGSTAConfigTable”.

ElementIDs

Ref Location column says Table 8-54 but it is 8-85.

For 143 ANA says “Wakeup Schedule” but D3.0 says “DMG Wakeup Schedule”.

For 146 ANA says “Extended mmWave TSPEC” but D3.0 says “DMG TSPEC”.

For 147 ANA says “Next mmWave AT” but D3.0 says “Next DMG ATI”.

For 148 ANA says “mmWave Capabilities” but D3.0 says “DMG Capabilities”.

For 151 ANA says “mmWave Operation” but D3.0 says “DMG Operation”.

For 152 ANA says “mmWave BSS Parameter Change” but D3.0 says “DMG BSS Parameter Change”.

For 153 ANA says “mm mmWave Beam refinement” but D3.0 says “DMG Beam Refinement”.

For 162 ANA says “mmWave Link Margin” but D3.0 says “DMG Link Margin”.

For 170 ANA says “Multiple MAC Addresses” but D3.0 says “Multiple MAC Sublayers”.

For 172 ANA says “DBand Link Adaptation Acknowledgement” but D3.0 says “DMG Link Adaptation Acknowledgment”.

ExtendedCapabilities

Ref Subclause and Location columns say 8.4.2.29 and Table 8-103 but it is 8.4.2.26 and Table 8-142.

ExtendedControlSubTypes

Ref Location column says Table 8-1a but it is now 8-2.

For 5 ANA says “mmWaveCTS” but D3.0 says “DMG CTS”.
For 6 ANA says “mmWaveDTS” but D3.0 says “DMG DTS”.
For 7 ANA says “mmWaveCF-End” but D3.0 says “Grant Ack”.
For 8 ANA says “ScS” but D3.0 says “SSW”.
For 9 ANA says “SS-Feedback” but D3.0 says “SW-Feedback”.
For 10 ANA says “SS-ACK” but D3.0 says “SSW-Ack”.

ExtendedSubTypes
For 0 ANA says “mmWave Beacon” but D3.0 says “DMG Beacon”.

PublicActionFrames

Ref Subclause and Location columns say 8.5.8.1 and Table 8-201 but it is 8.6.8.1 and Table 8-210.

RSNCapabilities
Ref Subclause and Location columns say 8.4.2.27.4 and Figure 8-188 but it is 8.4.2.24.4 and Figure 8-252.

StatusCodes
Ref Location column says Table 8-37, but it is actually Table 8-53.

For 86 ANA has blank name and says “FST pending, in process of acquiring resources from AP/PCP”. D3.0 has name “PENDING_ADMITTING_FST_SESSION” and says “FST pending, in process of admitting FST session”.
For 87 ANA has blank name but D3.0 has name “PERFORMING_FST_NOW”.
For 88 ANA has blank name but D3.0 has name “PENDING_GAP_IN_BA_WINDOW”.
For 89 ANA has blank name but D3.0 has name “REJECT_U-PID_SETTING”.

### TGae ANA Usage

ElementIDs

Ref Location column says Table 8-54 but it is 8-85.

ExtendedCapabilities

Ref Subclause and Location columns say 8.4.2.29 and Table 8-103 but it is 8.4.2.26 and Table 8-142.

PublicActionFrames

Ref Subclause and Location columns say 8.5.8.1 and Table 8-201 but it is 8.6.8.1 and Table 8-210.

### TGaf ANA Usage

dot11Compliances

I can’t find dot11TVHTCompliance in D3.0.

At 3299.16, change dot11TVWSCompliance to dot11TVHTCompliance

ElementIDs

Ref Location column says Table 8-54 but it is 8-85.

ANA says #200 is ChannelScheduleManagement but D3.0 says Reserved.

ANA to mark this Reserved (was allocated) in database.

ExtendedCapabilities

Ref Subclause and Location columns say 8.4.2.29 and Table 8-103 but it is 8.4.2.26 and Table 8-142.

PublicActionFrames

Ref Subclause and Location columns say 8.5.8.1 and Table 8-201 but it is 8.6.8.1 and Table 8-210.

StatusCodes

Ref Location column says Table 8-37, but it is actually Table 8-53.

TLVencodings

“Table E-8” missing from Column E (Ref Location) for TLV encodings values 4 through 6.

I can’t find the “Model Identifier” or “Technology Identifier” TLV encodings, #7 and #8, in Draft 3.0.

Remove definition for “model identifier” at 46.48.  Mark it Reserved (was allocated) in ANA database.

Mark “Technology Identifer” Reserved (was allocated) in ANA database.

## MIB

### Summary of major issues discovered

- Both 11ac and 11af define the same MIB variable names, dot11CurrentChannelWidth, dot11CurrentChannelCenterFrequencyIndex0, dot11CurrentChannelCenterFrequencyIndex1.

- The proposal here is to change the MIB variable names of 11af as the following:

  - replaces “dot11CurrentChannelWidth” with “dot11TVHTCurrentChannelWidth”

  - replaces “dot11CurrentChannelCenterFrequencyIndex0” with “dot11TVHTCurrentChannelCenterFrequencyIndex0”

  - replaces “dot11CurrentChannelCenterFrequencyIndex1” with “dot11TVHTCurrentChannelCenterFrequencyIndex1”

- Regarding 11ad,

  - "dot11PhyDMGComplianceGroup" is not used in the REVmc D3.0. and, "dot11DMGComplianceGroup" is almost same with "dot11PhyDMGComplianceGroup" and is frequently used.

  - Compiling error of "dot11PhyDMGComplianceGroup" is that this MIB variable was not defined. So, my proposal is to remove the unused "dot11PhyDMGComplianceGroup".

### Detailed proposed changes

Editing Instruction: TGmc Editor change Annex C as follows:

(changes are highlighted yellow for visibility)

-- Editor Note: During publication, update the following datestamp to the date of publication using the format yyyymmdd0000Z.

REVISION    "201403260000Z201211270000Z"

DESCRIPTION "IEEE Std 802.11-<year>

Dot11WirelessMgmtOptionsEntry ::=

SEQUENCE {

…

dot11FineTimingMsmtImplemented	TruthValue,

dot11FineTimingMsmtRespActivated	TruthValue,

dot11FineTimingMsmtInitActivated	TruthValue,

dot11LciCivicInNeighborReport	TruthValue,

dot11RMFineTimingMsmtRangeRepImplemented 	TruthValue,

dot11RMFineTimingMsmtRangeRepActivated 	TruthValue

}

dot11ADDBAResponseTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"Deprecated as no longer used by 802.11-<year>"

This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the timeout in seconds for an ADDBA Response frame that is a response to an ADDBA Request frame."

DEFVAL { 1 }

::= { dot11OperationEntry 14 }

dot11ADDTSResponseTimeout OBJECT-TYPE

SYNTAX Unsigned32 (1..65535)

MAX-ACCESS read-write

STATUS deprecated

DESCRIPTION

"Deprecated as no longer used by 802.11-<year>"

This is a control variable.

It is written by an external management entity.

Changes take effect as soon as practical in the implementation.

This attribute specifies the maximum number of seconds an ADDTS request is to be responded."

DEFVAL { 1 }

::= { dot11OperationEntry 15 }

dot11BSSLoadAssocFailCount OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of unsuccessful association procedures at the AP (i.e., it is the number of association procedures started less those that completed successfully). It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request)."

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 4 }

dot11BSSLoadReassocFailCount OBJECT-TYPE

SYNTAX Unsigned32 (0..4294967295)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

It is written by the MLME.

This object contains the count of the number of unsuccessful reassociation procedures at the AP (i.e., it is the number of reassociation procedures started less those that completed successfully). It reports the count of such events since the AP started the operation of its BSS (i.e., since the last MLME-START.request)."

DEFVAL { 0 }

::= { dot11BSSStatisticsEntry 6 }

dot11PhyTVHTEntry OBJECT-TYPE

SYNTAX Dot11PhyTVHTEntry

MAX-ACCESS not-accessible

STATUS current

DESCRIPTION

"An entry in the dot11PhyTVHTEntry Table. ifIndex - Each IEEE 802.11 interface is represented by an ifEntry. Interface tables in this MIB module are indexed by ifIndex."

INDEX {ifIndex}

::= { dot11PhyTVHTTable 1 }

Dot11PhyTVHTEntry ::=

SEQUENCE {

dot11TVHTChannelWidthOptionImplemented	INTEGER,

dot11TVHTCurrentChannelWidth	INTEGER,

dot11TVHTCurrentChannelCenterFrequencyIndex0	Unsigned32,

dot11TVHTCurrentChannelCenterFrequencyIndex1	Unsigned32,

dot11TVHTShortGIOptionIn4WImplemented	TruthValue,

dot11TVHTShortGIOptionIn4WActivated	TruthValue,

dot11TVHTLDPCCodingOptionImplemented	TruthValue,

dot11TVHTLDPCCodingOptionActivated	TruthValue,

dot11TVHTTxSTBCOptionImplemented	TruthValue,

dot11TVHTTxSTBCOptionActivated	TruthValue,

dot11TVHTRxSTBCOptionImplemented	TruthValue,

dot11TVHTRxSTBCOptionActivated	TruthValue,

dot11TVHTMUMaxUsersImplemented	Unsigned32,

dot11TVHTMUMaxNSTSPerUserImplemented	Unsigned32,

dot11TVHTMUMaxNSTSTotalImplemented	Unsigned32,

dot11TVHTMaxNTxChainsImplemented	Unsigned32,

dot11TVHTMaxNTxChainsActivated	Unsigned32

dot11TVHTChannelWidthOptionImplemented OBJECT-TYPE

SYNTAX INTEGER { tvht_mode_1(0), tvht_mode_2c(1), tvht_mode_2n(2),

tvht_mode_4c(3), tvht_mode_4n(4) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a capability variable.

Its value is determined by device capabilities.

This attribute indicates the channel widths supported: W MHz, 2W MHz, W+W MHz, 4W MHz or 2W+2W MHz."

DEFVAL { tvht_mode_1 }

::= { dot11PhyTVHTEntry 1 }

dot11TVHTCurrentChannelWidth OBJECT-TYPE

SYNTAX INTEGER { tvht_w(0), tvht_2w(1), tvht_wpw(2), tvht_4w(3), tvht_2wp2w(4) }

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

This attribute indicates the operating channel width."

DEFVAL { tvht_w }

::= { dot11PhyTVHTEntry 2 }

dot11TVHTCurrentChannelCenterFrequencyIndex0 OBJECT-TYPE

SYNTAX Unsigned32 (0..200)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

For a W MHz, 2W MHz or 4W MHz channel, denotes the channel center frequency.

For an W+W or 2W+2W MHz channel, denotes the center frequency of frequency segment 0. See 23.3.14 (Channelization)."

DEFVAL { 0 }

::= { dot11PhyTVHTEntry 3 }

dot11TVHTCurrentChannelCenterFrequencyIndex1 OBJECT-TYPE

SYNTAX Unsigned32 (0..200)

MAX-ACCESS read-only

STATUS current

DESCRIPTION

"This is a status variable.

For an W+W or 2W+2W MHz channel, denotes the center frequency of frequency segment 1.

Set to 0 for a W MHz, 2W MHz or 4W MHz channel. See 23.3.14 (Channelization)."

DEFVAL { 0 }

::= { dot11PhyTVHTEntry 4 }

dot11SMTRMRequest OBJECT-GROUP

OBJECTS {

dot11RMRqstRowStatus,

dot11RMRqstToken,

dot11RMRqstRepetitions,

dot11RMRqstIfIndex,

dot11RMRqstType,

dot11RMRqstTargetAdd,

dot11RMRqstTimeStamp,

dot11RMRqstChanNumber,

dot11RMRqstOperatingClass,

dot11RMRqstRndInterval,

dot11RMRqstDuration,

dot11RMRqstParallel,

dot11RMRqstEnable,

dot11RMRqstRequest,

dot11RMRqstReport,

dot11RMRqstDurationMandatory,

dot11RMRqstBeaconRqstMode,

dot11RMRqstBeaconRqstDetail,

dot11RMRqstFrameRqstType,

dot11RMRqstBssid,

dot11RMRqstSSID,

dot11RMRqstBeaconReportingCondition,

dot11RMRqstBeaconThresholdOffset,

dot11RMRqstSTAStatRqstGroupID,

dot11RMRqstLCIRqstSubject,

dot11RMRqstLCILatitudeResolution,

dot11RMRqstLCILongitudeResolution,

dot11RMRqstLCIAltitudeResolution,

dot11RMRqstLCIAzimuthType,

dot11RMRqstLCIAzimuthResolution,

dot11RMRqstPauseTime,

dot11RMRqstTransmitStreamPeerQSTAAddress,

dot11RMRqstTransmitStreamTrafficIdentifier,

dot11RMRqstTransmitStreamBin0Range,

dot11RMRqstTrigdQoSAverageCondition,

dot11RMRqstTrigdQoSConsecutiveCondition,

dot11RMRqstTrigdQoSDelayCondition,

dot11RMRqstTrigdQoSAverageThreshold,

dot11RMRqstTrigdQoSConsecutiveThreshold,

dot11RMRqstTrigdQoSDelayThresholdRange,

dot11RMRqstTrigdQoSDelayThreshold,

dot11RMRqstTrigdQoSMeasurementCount,

dot11RMRqstTrigdQoSTimeout,

dot11RMRqstChannelLoadReportingCondition,

dot11RMRqstChannelLoadReference,

dot11RMRqstNoiseHistogramReportingCondition,

dot11RMRqstAnpiReference,

dot11RMRqstAPChannelReport,

dot11RMRqstSTAStatPeerSTAAddress,

dot11RMRqstFrameTransmitterAddress,

dot11RMRqstVendorSpecific }

STATUS current

DESCRIPTION

"The SMTRMRequest package is a set of attributes that are present if the STA supports the Radio Measurement service."

::= { dot11Groups 37 }

dot11MACbase3 OBJECT-GROUP

OBJECTS {

dot11MACAddress,

dot11GroupAddress,

dot11GroupAddressesStatus,

dot11RTSThreshold,

dot11ShortRetryLimit,

dot11LongRetryLimit,

dot11FragmentationThreshold,

dot11MaxTransmitMSDULifetime,

dot11MaxReceiveLifetime,

dot11ManufacturerID,

dot11ProductID,

dot11CAPLimit,

dot11HCCWmin,

dot11HCCWmax,

dot11HCCAIFSN,

dot11ADDBAResponseTimeout,

dot11ADDTSResponseTimeout,

dot11ChannelUtilizationBeaconInterval,

dot11ScheduleTimeout,

dot11DLSResponseTimeout,

dot11QAPMissingAckRetryLimit,

dot11EDCAAveragingPeriod,

dot11HTProtection,

dot11RIFSMode,

dot11PSMPControlledAccess,

dot11ServiceIntervalGranularity,

dot11DualCTSProtection,

dot11LSIGTXOPFullProtectionActivated,

dot11NonGFEntitiesPresent, dot11PCOActivated,

dot11PCOFortyMaxDuration,

dot11PCOTwentyMaxDuration,

dot11PCOFortyMinDuration,

dot11PCOTwentyMinDuration }

STATUS deprecated

DESCRIPTION

"Superseded by dot11MACbase4."

The MAC object class provides the necessary support for the access control, generation, and verification of frame check sequences (FCSs), and proper delivery of valid data to upper layers."

::= { dot11Groups 45 }

dot11HTMACAdditions2 OBJECT-GROUP

OBJECTS {

dot11MIMOPowerSave,

dot11NDelayedBlockAckOptionImplemented,

dot11MaxAMSDULength,

dot11STBCControlFrameOptionImplemented,

dot11LsigTxopProtectionOptionImplemented,

dot11MaxRxAMPDUFactor,

dot11MinimumMPDUStartSpacing,

dot11PCOOptionImplemented,

dot11TransitionTime,

dot11MCSFeedbackOptionImplemented,

dot11HTControlFieldSupported,

dot11RDResponderOptionImplemented,

dot11BSSWidthTriggerScanInterval,

dot11BSSWidthChannelTransitionDelayFactor,

dot11OBSSScanPassiveDwell,

dot11OBSSScanActiveDwell,

dot11OBSSScanPassiveTotalPerChannel,

dot11OBSSScanActiveTotalPerChannel,

dot112040BSSCoexistenceManagementSupport,

dot11OBSSScanActivityThreshold,

dot11SPPAMSDUCapable,

dot11SPPAMSDURequired }

STATUS current

DESCRIPTION

"Attributes that configure the HT for IEEE Std 802.11."

::= { dot11Groups 87 <ANA> }

(Note that this change to dot11HTMACAdditions2 was necessary to get the MIB to compile.  However, assignement of this number is a matter for the ANA (and the topic of a comment in the recent ballot).)

(Comment: the intent of having two DMG groups might be to reflect the MAC-specific and PHY-specific compliance requirements.  However, what .11ad provided didn’t do that.   See “Issues” above.   It is an open question as to how TGaj will respond to this.  They might need to pick apart the DMG compliance group into a MAC and PHY part.  But that is a task for the future.)

GROUP dot11PhyDMGComplianceGroup

DESCRIPTION

"Implementation of this group is required when object dot11PHYType has the value of dmg.

This group is mutually exclusive to the following groups:

dot11PhyIRComplianceGroup

dot11PhyFHSSComplianceGroup2

dot11PhyDSSSComplianceGroup

dot11PhyOFDMComplianceGroup3

dot11PhyHRDSSSComplianceGroup

dot11PhyERPComplianceGroup

dot11PhyHTComplianceGroup

dot11PhyVHTComplianceGroup

dot11PhyTVHTComplianceGroup"

GROUP dot11DMGComplianceGroup

DESCRIPTION

"Implementation of this group is required when the object

dot11PHYType has the value of DMG.

This group is mutually exclusive to the following groups:

dot11PhyIRComplianceGroup

dot11PhyFHSSComplianceGroup2

dot11PhyDSSSComplianceGroup

dot11PhyOFDMComplianceGroup3

dot11PhyHRDSSSComplianceGroup

dot11PhyERPComplianceGroup

dot11PhyHTComplianceGroup"

dot11PhyVHTComplianceGroup

dot11PhyTVHTComplianceGroup"

dot11TVWSCompliance MODULE-COMPLIANCE

STATUS current

DESCRIPTION

"The compliance statement for SNMPv2 entities that implement the IEEE 802.11 MIB for TVWS operation."

MODULE -- this module

MANDATORY-GROUPS {

dot11PhyTVHTTVWSComplianceGroup,

dot11PhyTxPowerComplianceGroup2,

dot11TVHTTransmitBeamformingGroup,

dot11VHTMACAdditions, dot11TVWSComplianceGroup }

::= { dot11Compliances 15 }

Editing Instruction: TGmc Editor replace “dot11CurrentChannelWidth” with “dot11TVHTCurrentChannelWidth” throughout Clause 23.

Editing Instruction: TGmc Editor replace “dot11CurrentChannelCenterFrequencyIndex0” with “dot11TVHTCurrentChannelCenterFrequencyIndex0” throughout Clause 23.

Editing Instruction: TGmc Editor replace “dot11CurrentChannelCenterFrequencyIndex1” with “dot11TVHTCurrentChannelCenterFrequencyIndex1” throughout Clause 23.

### Files

The old MIB text, corrected (new) MIB text, and difference files are embedded below.

# Collateral findings

In P549L57 broken reference: “Each field is defined in (The Frame Body field is of variable size, constrained as defined in 8.2.4.7.1 (General).)”

In P1249L65: There seems to be some formatting issue with the table.

- P1163 L13, L46 “The Subelement ID field is set to 0.”

Proposed change:  Editor to address these issues.

# IEEE-SA MEC

At the time of writing this report,   the IEEE-SA mandatory editorial coordination (MEC) is ongoing.  When complete, the findings will be added to this report.

From: Michelle Turner Subject: P802.11REVmc/D3 MEC … Attached is the MEC for P802.11REVmc/D3. As a quick summary the final items shall be addressed before the draft can proceed to ballot. If figures, tables, or text have been borrowed from another published source, please make sure all permission letters are sent to me as soon as possible. Before the draft can proceed to ballot, please make sure each page includes the following information in the footers Copyright © 2014 IEEE. All rights reserved. This is an unapproved IEEE Standards Draft, subject to change. The MEC document attached is reproduced below: Re: Pre-ballot Mandatory Editorial Coordination (Pre-ballot MEC) Dear Adrian Stephens: I have reviewed Draft 3 of IEEE P802.11™, and I have the following comments. Please note that this review has been organized into two sections and uses the “language of standards” to communicate necessary requirements (shall) of the IEEE-SA standards process versus those issues that are voluntary (should) in nature. Section I: Items/issues that shall be resolved before the ballot begins The draft cannot be balloted or recirculated until these issues are resolved. Your Staff Liaison will review the updated draft for compliance prior to upload of the PDF for ballot. Section II: Items/issues that shall be resolved before the final recirculation These issues have to be resolved and viewed by balloters. The items will be checked for completion by the Project Editor during the Sponsor ballot, then checked by the Review Committee (RevCom) of the IEEE-SA Standards Board (IEEE-SASB), and will impact approval unless rectified. Working groups who wish to have a draft that is very close to the published document may want to implement these changes. However, the comments should not affect the approval of the standard. Please note that professional editing takes place once the document has been approved and, as such, this MEC does not address all of the editorial items that will be reviewed then (i.e., punctuation, grammar, formatting). The following comments are derived from the IEEE Standards Style Manual. The complete IEEE Standards Style Manual, in viewable/downloadable format, can be found at: https://development.standards.ieee.org/myproject/Public/mytools/draft/styleman.pdf. SECTION I: Items/issues that shall be resolved before the ballot begins: Copyright - If applicable, all copyright permission for excerpted text, tables, and figures shall be submitted to the IEEE prior to the start of ballot. If there are missing permission response letters, please submit them immediately to me m.d.turner@ieee.org Sample permission letters can be found in Annex A of the IEEE Standards Style Manual or http://standards.ieee.org/develop/stdsreview.html. More information on the IEEE SA Copyright Policy can be found at: http://standards.ieee.org/ipr/copyright.html - The correct copyright line shall appear on the bottom of every page, including the body of the standard: Copyright © 20xx IEEE. All rights reserved. This is an unapproved IEEE Standards Draft, subject to change. SECTION II: Items/issues that shall be resolved before the final recirculation Trademarks or service marks - Please review the use of trademarks in the draft, if applicable. References to commercial equipment or products in a standard shall be generic and shall not include trademarks or other proprietary designations. Where a sole source exists for essential equipment or materials, it is permissible to supply the name of the trademark owner in a footnote. The proper use guidelines for trademarks shall be determined by the trademark owner. Trademark owners must grant written permission before their trademarks may be referenced in a standard. - Trademarks or other proprietary designations that are not commercial equipment or products should be avoided in standards. If used however, all trademarks shall be credited to the trademark owner in the front matter of the standard. The following text shall introduce any mention of specific trademark information: The following information is given for the convenience of users of this standard and does not constitute an endorsement by the IEEE of these products. Equivalent products may be used if they can be shown to lead to the same results. For more information on commercial terms and conditions see the IEEE-SA Policy on commercial terms set forth in 6.2 of the IEEE-SA Standards Board Operations Manual. Registration objects - If the draft contains a registration of objects (for additional information, visit the IEEE Standards Web site <http://standards.ieee.org/regauth/index.html>), the working group shall submit the document to the IEEE Registration Authority (IEEE-RA) for mandatory coordination (submit to a.n.thomas@ieee.org for review). The text containing the registration information should be highlighted in the draft and the clause should be noted in the email. If the working group believes that the draft may potentially contain a registration of objects or if the working group would like information about setting up a registration, contact the IEEE-RA as early as possible to prevent a delay in approval by the IEEE-SA Standards Board. Search on the following words: object identifier, unique identifier, and assignment of unique numbers. Normative references and bibliography - All citations in Clause 2 shall be normatively referenced within the body of the document. If the following documents are not needed for the implementation of the standard, please move to the bibliography: IEEE C95.1, ISO/IEC 8824-2, ISO/IEC 8824-4, ISO/IEC 8825-1, ISO 8825-2, ISO 15802-3, ITU-T Z.100, ITU-T Z.105. - IEEE Std 802.11-2011 is cited in the normative reference clause. Do you want to consider removing the date, so it is clear that this includes the base and all amendments? - 802.21-2008 is cited in the normative reference clause. Do you want to consider removing the date, so it is clear that this includes the base and all amendments? - 802-2001 is cited in the normative reference clause. This document was approved and published in 2014. Would you like to remove the date, so the latest version is used? Graphics - Separate electronic files of figures shall be supplied (unless created in Word or Framemaker) Please note that the following are next steps for this project. a) After you have implemented this review, create the pdf that will be used for ballot (remember that the draft number shall be rolled to reflect that changes have been made to this document, e.g., P1234™/Dx+1). b) Upon completion of the invitation to ballot please follow the next steps: Instructions: 1. Login to myProject and click the Balloting tab 2. Click Initiate Sponsor Ballot 3. Select your project from the PAR drop down list 4. Enter the Ballot Open Date 5. Enter the Ballot Close Date (should be minimum of 30 days) 6. Enter the Draft #: (must match the draft number in the draft ) 7. Select File for Uploading: Click the Browse button to find your draft file. The file must be in pdf. 8. Review the system generated text. If you would like to add additional instruction or information, use the Sponsor Text Area. 9. Click Initiate Ballot. c) Note that compliance with items in Section I will be reviewed by the Staff Liaison when you upload the pdf to the URL in item b). The Project Editor will not review your draft until the Ballot MEC, which occurs during the Sponsor ballot. d) The RevCom MEC will occur after you submit the final balloted draft to RevCom. At that time you will also be required to submit the document source file. If the figures are not native Word or Framemaker graphics, each graphic shall be submitted as a separate file following the requirements outlined in Clause 14 of the IEEE Standards Style Manual. http://standards.ieee.org/resources/development/writing/writinginfo.html Thank you for the opportunity to review this draft. If you have any queries about the comments in this mandatory editorial coordination, please contact Michelle Turner via email (). cc: Kathryn Bennett